Draft Annual Springtime O3 NAAQS Exceedances in the Northern Plaines

June 13, 2020

ISSUE: Late springtime O3 levels routinely exceed the level of the NAAQS in a sparsely populated agricultural area in the northern plains: eastern Dakotas, Minnesota, northern Iowa, and southwest Wisconsin.

In this region, the highest O3 levels occur during a small two-week timeframe of late May to early June (as shown in Figure 1 for Sioux Falls, SD). These levels tend to be the highest of the year which means these ares are not expected to be designated nonattainment since a traditional summer O3 season is not common in the region. At least 8 rural monitors in the region exceeded the level of the O3 NAAQS in 2018, 7 in 2019, and 1 in 2020 (none in 2017).

Figure 1. Daily maximum 8-hr O3 levels at Sioux Falls, SD from 2014 to 2020.

These O3 exceedances happen when warmer, summerlike temperatures first occur in the region and coincide with regional elevated NO2 detected from the TropOMI satellite (Figure 2). The monitors with O3 peaks are likely influenced by local sources since the local O3 enhancements have some spatial agreement with NO2 column measurements.

Most rural areas tend to be NOX limited for O3 formation. However, a large area of agriculture in southern Minnesota, northern Iowa, and eastern South Dakota my provide an area of O3 formation transition as abundant biogenic VOC is no longer available (Figure 3). The NO2 seen on satellite products may react with biogenic VOC in the northern portion of Minnesota and western Wisconsin then transporting to other areas in the region with O3 formed from other combinations of NOX and VOC sources. These other sources could include agricultural activity in the eastern Dakotas, southern Minnesota, and northern Iowa and urban/industrial sources in the region.

The area did have an O3 exceedance in 2020, in addition to several monitors being at or near the 8-hour ozone NAAQS, even though anthropogenic emissions are expected to be lower related to restrictions put in place to minimize the COVID-19 pandemic. The 2020 period may provide an opportunity to study the relative contribution of biogenic NO and VOC compared to anthropogenic sources. Years with larger flooding in the Red River region (Minnesota/North Dakota border) tend to have larger O3 levels during this spring period: 2019 had greater flooding and more exceedances than 2017 when flooding was minimal. Another example is the historic 2019 spring flooding in the Missouri River Basin portion of eastern SD, and the subsequent four ozone exceedances seen between the Brookings and Sioux Falls, SD monitors from May 30 and June 7th, 2019. The additional NO from biogenic activity in areas drying out from flood water may provide additional O3 to the region, especially when synoptic transport patterns combine these emissions with abundant VOC from vegetation in northern Minnesota and Wisconsin.

Figure 2. June 2, 2020 at 19/20 GMT. TROPOMI NO2 (left) and surface O3 (right).

Sources of regional NO2 are not totally clear. It is possible that the NO2 emissions could be related to agricultural fertilizer mobilization, crop burning, and field tilling. Biogenic NO from flooded rivers, streams, and farmland may also be a regional source of emissions. The amount of flooding varies by year but tends to happen in years with elevated O3.

Emissions from large metropolitan areas (Minneapolis, MN, Chicago, Ill) and smaller urban areas (Sioux Falls, SD and Des Moines, IA) may contribute to the O3 exceedances, especially when considering meteorology (surface winds) on the high ozone days. Subsequently, the yearly springtime increase in regional O3 in agricultural area can provide an enhancement of regional transported ozone into Midwest urban areas like the Chicago metro and may influence and contribute to early season ozone exceedances in the Chicago/Milwaukee area.

Sources of regional VOC are often biogenic, which are shown in Figure 3.

| Figure 3. May 2019 average biogenic (isoprene) emissions. NO2 tropospheric column density measured by TropOMI for June 2019. |  |
| --- | --- |

Flooding

MODIS images from May 2017 and May 2019 for a portion of this region is shown in Figure 4. The satellite image is from MODIS (reflectance from bands 1, 2, and 7). The green areas show vegetation and brown areas show farmland (before planting) and pasture (https://go.nasa.gov/3fi5amb). Flood water within the Red River flood plain along the ND/Minnesota border (near the Canadian border) is evident in 2019 compared to 2017.

Figure 4. MODIS landcover image for May 7th, 2017 (top) and May 7th, 2019 (bottom).

Figure 5 shows screenshot images from Planet Labs (www.planet.com). April 2019 and April 2020, with a May 2020 as a reference. Lots of flooding is apparent. These are monthly composites. Weekly composites and/or the daily images that make up the composites can be requested, although the images are proprietary, and costs are associated with them.

| Figure 5. Satellite image of Red river. Courtesy of David Williams (EPA ORD). |  |
| --- | --- |
| 2017 | 2018 |
| 2019 | 2020 (April) |
|  | 2020 (May) – no flooding |

Model Performance

Model applications looking specifically at the high O3 episodes in this region have not yet been investigated. However, model performance products are available for this region for an annual CAMx photochemical model application for 2016. A discussion of performance for that simulation follows.

The model baseline error in the springtime (Figure B) is almost certainly due to representation of some variety of some large-scale processes. The tropospheric ozone lifetime is very long in the winter and spring due to these seasons having minimum atmospheric water vapor relative to their corresponding photochemical seasons (e.g., January compared to December: similar UV, but less water vapor in January as ocean has large heat capacity and thus long memory effects). Any minor errors in NOx lifetime, NOx steady state chemistry or stratospheric-tropospheric exchange rates gets amplified in these seasons given the longer O3 lifetime.

Another big factor may be O3 dry deposition. There are big inflection points in the AQS mean vs the models are in June/July and October – December, when the areas go from fairly bare to fully vegetated or vice versa. Biogenic emissions of VOC (especially related to leaf-out timing) and NO are uncertain and also likely contribute to poor model representation of high O3 during this period.

Figure B. 2016 model performance for the upper midwest region.

APPENDIX A.

Monitored O3 in the region

2017 Ozone

|  | # of exceedances | Max MDA8 (ppb) | Date of Max MDA8 | 4th High (ppb) |
| --- | --- | --- | --- | --- |
| Brookings, SD | 0 | 67 | 2017-06-04 | 63 |
| Sioux Falls, SD | 0 | 69 | 2017-06-09 | 66 |
| Detroit Lakes, MN | 0 | 62 | 2017-06-02 | 57 |
| Rochester, MN | 0 | 66 | 2017-06-08 | 62 |
| Eu Claire, WI | 0 | 67 | 2017-06-08 | 56 |
| La Crosse, WI | 0 | 66 | 2017-06-04 | 62 |
| Emmitsburg, IA | 0 | 64 | 2017-06-04 | 61 |
| Waterloo, IA | 0 | 68 | 2017-06-04 | 59 |

2018 Ozone Information

|  | # of exceedances | Max MDA8 (ppb) | Date of Max MDA8 | 4th High (ppb) |
| --- | --- | --- | --- | --- |
| Brookings, SD | 1 | 71 | 2018-06-05 | 67 |
| Sioux Falls, SD | 3 | 75 | 2018-05-27 | 69 |
| Detroit Lakes, MN | 0 | 70 | 2018-05-24 | 63 |
| Rochester, MN | 0 | 66 | 2018-05-27 | 58 |
| Eu Claire, WI | 1 | 79 | 2018-05-27 | 66 |
| La Crosse, WI | 0 | 68 | 2018-05-29 | 62 |
| Emmitsburg, IA | 1 | 73 | 2018-05-27 | 67 |
| Waterloo, IA | 2 | 73 | 2018-05-29 | 68 |

2018 Summary: 8 Ozone exceedances in the “agriculture” area of eastern SD, Western and Southern MN, northern IA and SW Wisconsin occurring in a two timeframe in late May and early June.

2019 Ozone

|  | # of exceedances | Max MDA8 (ppb) | Date of Max MDA8 | 4th High (ppb) |
| --- | --- | --- | --- | --- |
| Brookings, SD | 5 | 78 | 2019-05-30 | 71 |
| Sioux Falls, SD | 1 | 72 | 2019-06-06 | 65 |
| Detroit Lakes, MN | 1 | 72 | 2019-05-30 | 59 |
| Rochester, MN | 0 | 59 | 2019-05-16 | 54 |
| Eu Claire, WI | 0 | 67 | 2019-05-31 | 56 |
| La Crosse, WI | 0 | 64 | 2019-06-08 | 58 |
| Emmitsburg, IA | 0 | 68 | 2019-06-06 | 65 |
| Waterloo, IA | 0 | 67 | 2019-06-06 | 61 |

2019 Summary:  7 Ozone exceedances in the “agriculture” area of eastern SD, Western MN, with 5 occurring at Brookings, SD (population – 22,000).

2020 Ozone Information

|  | # of exceedances | Max MDA8 (ppb) | Date of Max MDA8 | 4th High MDA8 (ppb) |
| --- | --- | --- | --- | --- |
| Brookings, SD | 0 | 66 | 2019-05-30 | - |
| Sioux Falls, SD | 0 | 67 | 2019-06-06 | - |
| Detroit Lakes, MN | 1 | 71 | 2020-06-02 | - |
| Rochester, MN | 0 | 70 | 2020-06-04 | - |
| Eu Claire, WI | 0 | 68 | 2019-06-08 | - |
| La Crosse, WI | 0 | 68 | 2020-06-04 | - |
| Emmitsburg, IA | 0 | 66 | 2020-06-02 | - |
| Waterloo, IA | 0 | 66 | 2020-06-08 | - |