Characterizing Air Quality Impacts Related to North Atlantic Offshore Wind Energy Projects

Kirk R. Baker, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, USA*

Byron Rice, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, USA

Neal Fann, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, USA

*Corresponding author contact: baker.kirk@epa.gov

ABSTRACT

Wind energy projects are being planned and constructed off the northern Atlantic coast to provide additional energy capacity to the eastern U.S. Emissions related to construction, operation, and routine maintenance of these offshore wind projects and the chemical transformation of these emissions in the atmosphere can result in pollutants that have known negative human health effects. However, the increased electrical capacity provided by these offshore wind projects could result in some reduction to onshore electrical generating (EGU) facilities. Here, multiple air quality models (a reduced complexity tool and a more sophisticated photochemical grid model) were applied to predict annual average PM2.5 and seasonal average maximum daily 8-hr average O3 impacts from offshore wind projects and resulting reductions in onshore EGUs. The reduced complexity tool reasonably replicated the magnitudes and spatial gradients of impacts predicted by the photochemical transport model. Air pollution impacts from the offshore wind energy projects tended to be highest nearest the projects. Air pollution impacts were much higher from the construction phase compared to post-construction (operation and maintenance). Predicted reductions to onshore EGUs due to increased offshore energy capacity resulted in regional decreases in PM2.5 and O3 that outpaced increases related to offshore wind projects. This effect was more pronounced for population influenced PM2.5 compared to O3. This is likely due to offshore wind energy capacity being highest in the winter which results in more onshore EGU emissions reductions outside of the summer season when precursor emissions would be most impactful on O3 production. Reductions in onshore EGU emissions were based on assumptions that the increased energy capacity would not simply meet increased demand over present-day levels or be balanced by non-fossil fuel based energy sources such as nuclear, solar, or onshore wind farms.

Keywords: offshore wind, electricity generation, PM2.5, O3, air quality

INTRODUCTION

Government initiatives along with improvements in technology and construction have resulted in new wind farm development offshore of the United States northern Atlantic coast (Costoya et al., 2020; DeCastro et al., 2019). Construction and maintenance of these offshore wind energy projects will result in emissions of gases and aerosols that can react in the atmosphere to form ozone (O3) and particulate matter less than 2.5 microns (PM2.5) in diameter. Both PM2.5 and O3 have known negative health impacts on humans (U.S. Environmental Protection Agency, 2019a, 2020, 2022) and are regulated by the U.S. Environmental Protection Agency with National Ambient Air Quality Standards (85 FR 87256 and 85 FR 82684). Research has shown that the wind turbines themselves can influence weather, most notably slowing wind speeds in the vicinity of the wind farm. However, studies suggest these changes to meteorology do not have a regional systematic impact on O3 or PM2.5 (Golbazi and Archer, 2024).

The construction phase of offshore wind energy projects results in substantially more emissions than post-construction operation and maintenance. Many areas off the U.S. Atlantic coast have been leased for the purpose of developing wind energy farms but not all are being constructed at the same time meaning over time emissions will vary as projects are constructed and energy capacity becomes available once construction is complete. It is expected that this increase in electrical generation capacity will result in a reduction to onshore electrical generating unit (EGU) generation in the eastern U.S. (Millstein et al., 2017; Wiser et al., 2016).

The dichotomous nature of increasing offshore emissions and potentially decreasing onshore emissions presents a complex and dynamic scenario with respect to human exposure as new wind farms are constructed over time off the northern U.S. Atlantic coast. It is important to understand the air quality impacts due to changing emissions related to these offshore wind energy projects and whether projected decreases in emissions from onshore EGUs offset these increases or provide greater benefit to the highly populated eastern U.S. region. Numerous studies have explored the air quality impact of onshore renewable energy sources such as wind power (Mailloux et al., 2022; McCubbin and Sovacool, 2013; Millstein et al., 2017; Rezaee et al., 2022) but none have quantified the anticipated change in PM2.5 and O3 due to offshore wind project related emissions and change in onshore EGU emissions.

Here, air quality models were used to calculate year specific PM2.5 and O3 impacts related to offshore and onshore emission changes resulting from the construction and operation of a subset of existing and planned offshore wind projects that have publicly available permit applications off the U.S. Atlantic coast. These offshore wind projects are anticipated to begin and complete construction at various times between 2024 and 2031. Emissions for these offshore wind energy projects are based on information provided in permit applications. Changes in onshore EGU emissions were estimated using a simple tool that relates offshore wind energy capacity to regional-specific historical relationships between EGU generation and demand. Even though the group of offshore wind farms modeled as part of this assessment is not reflective of all leased areas that could be developed given the staggered nature of construction of offshore wind projects, it nevertheless provides a lifecycle assessment that can be broadly applied to future development in the region.

Sophisticated models such as 3D photochemical grid models (Baker and Woody, 2017; Bergin et al., 2008) and reduced complexity (typically statistically based) tools (Baker et al., 2023) have been used extensively to quantify both PM2.5 and O3 air quality impacts due to emissions from specific sources or sectors. Both types of systems were used to estimate O3 and PM2.5 air quality impacts related to offshore wind energy project emissions and onshore EGU emissions. A photochemical grid model was applied for a year anticipated to have large construction phase emissions and a year where offshore wind projects were anticipated to be in both the operation and maintenance phase. A reduced complexity tool was used for these same scenarios and to quantify the annual changes in air quality anticipated due to scheduled project construction and anticipated new electrical generation capacity from completed projects. Human health impacts related to these changes in PM2.5 and O3 were quantified and monetized to provide an estimate of population exposure. This is important because reductions in emissions from EGUs happen near populated areas while increases offshore may impact populated areas due to certain wind patterns but are further from populated areas.

METHODS

This assessment makes use of multiple air quality models to estimate annual average PM2.5 and seasonal average (April through September) maximum daily 8-hr average (MDA8) O3: a photochemical grid model and a reduced complexity tool. Table S1 provides a list of emissions scenarios applied with each model. Both models were applied for the same scenarios and the reduced complexity tool was also used to generate year-specific air quality impacts. The photochemical grid model and reduced complexity tool were applied for a 2026 and multiple 2055 scenarios. The 2026 scenario represents a year where multiple projects were anticipated to be in the construction phase. The 2055 scenario represents a year at the end of the lifetime of the turbines installed at these projects to capture operation phase emissions. An additional simulation was done for 2055 that included adjusted onshore EGU emissions resulting from increased offshore energy capacity. The reduced complexity model was applied for each year of 2024 to 2031 using year specific offshore wind project emissions and onshore EGU emissions changes. The annual air quality estimates from the reduced complexity model were used to estimate and monetize human health impacts. More details about the tools and emissions follow in this section.

Offshore wind project emissions

Offshore wind project emissions were based on information provided as part of requirements for air quality permit programs (https://www.epa.gov/nsr). The emissions provided for these program requirements represent a maximum allowable level but are likely a reasonable approximation of typical operating conditions. Emissions were modeled to be the same for each hour of the day and day of the year. Commercial marine vessels have traditionally been included in modeling assessments and were treated in this assessment similarly in terms of emissions characteristics. Emissions associated with construction were assigned stack characteristics consistent with Class 3 commercial marine vessels and emissions associated with routine operation and maintenance were assigned stack characteristics consistent with Class 1 and 2 commercial marine vessels. Routine operation and maintenance emissions also include emissions from periodic maintenance and construction projects. Emissions for each lease area were spatially assigned to the center of the respective lease area (Figure S1).

Onshore EGU emissions adjustments

EGU emissions for 2026 and 2055 were projected from 2016 using economic conditions and anticipated emission controls with the Integrated Planning Model (IPM; https://www.epa.gov/power-sector-modeling). EGU emissions for 2016 were based on emissions reported to the Continuous Emissions Monitoring (CEM) database and National Emission Inventory (NEI) for 2016 (U.S. Environmental Protection Agency, 2023).

The Avoided Emissions and geneRation Tool (AVERT) v4.3 (epa.gov/avert) was applied using electrical generation estimates for offshore wind energy projects to estimate the change in electrical generation and resulting emissions for onshore EGUs (U.S. Environmental Protection Agency, 2024b). AVERT bins hours of the year into similar levels of generation then uses power plant specific operational characteristics including generation and emissions at these binned levels to create a new profile for each power plant under the new alternative regional load levels. Each offshore wind energy project was associated with a single onshore interconnect in AVERT based on proximity (Table 1). EGU emissions were adjusted using month and county specific percent changes predicted by AVERT applied to specific precursors: nitrogen oxides (NOX), sulfur dioxide (SO2), volatile organic compounds (VOC), primary PM2.5, and ammonia (NH3). The predicted emissions changes from AVERT based on offshore energy capacity for 2026 were applied to the 2026 IPM projected EGU emissions and those based on offshore energy capacity where all offshore projects were in the operation phase with no construction were applied to the 2055 IPM projected EGU emissions. The 2055 IPM scenario was selected to represent atmospheric conditions at the end of anticipated lifespan for these projects and to leverage existing model inputs generated as part of other projects.

Additional year specific EGU emissions for 2024 through 2031 were developed for application in the reduced complexity tool using anticipated year specific offshore wind energy capacity applied with the AVERT model. These year specific EGU emissions were estimated by applying month and county specific percent changes estimated by the AVERT model to the 2021 NEI/CEM emissions for the EGU sector since an IPM projection was not available for each year. The use of 2021 EGU emissions provided a recent reflection of the sector and a consistent starting point for each year so that air quality results would be reflective of changes due to increased offshore capacity without introducing other factors impacting year to year variability in onshore EGU emissions.

Photochemical grid model application

The Community Multiscale Air Quality (CMAQ) model version 5.4 (epa.gov/cmaq) was applied with 12 km sized grid cells for a domain covering the conterminous U.S. and areas offshore (Appel et al., 2021). The vertical atmosphere was resolved up to 50 mb with 35 layers. Vertical layers were thinner nearest the surface to best resolve diurnal fluctuation in the surface mixing layers. Lateral boundary inflow was extracted from a hemispheric scale simulation for the same year. Meteorological inputs were developed with the Weather Research and Forecasting model version 3.8.1 applied with the same grid domain as the photochemical model (U.S. Environmental Protection Agency, 2019b). CMAQ was applied with Carbon Bond 6 revision 5 gas phase chemistry (Emery et al., 2015) with halogen mediated first order O3 loss (Sarwar et al., 2015), aqueous phase reactions (Fahey et al., 2017), an inorganic thermodynamic model (Fountoukis and Nenes, 2007), and deposition processes (Clifton et al., 2023; Hogrefe et al., 2023). Primarily emitted organic aerosol was treated as nonvolatile.

All CMAQ simulations used the same 2016 meteorology. An annual simulation using 2016 emissions and 2016 lateral boundary inflow from a hemispheric model simulation was done to compare model predictions with ambient measurements to provide a model performance evaluation. Projected 2026 (from 2016) emissions were modeled with anticipated 2026 construction and operation emissions from offshore wind projects and also without any offshore wind project emissions. The difference between these simulations was intended to provide an estimate of the impacts of the offshore wind project emissions for 2026. Projected 2055 (from 2016) emissions were modeled as part of 3 different scenarios: 1) no offshore wind project emissions, 2) operation emissions at offshore wind projects, and 3) operation emissions at offshore wind projects with alternative onshore EGU emissions.

Anthropogenic emissions used for the 2026 and 2055 model simulations were based on 2026 (2026gf) (U.S. Environmental Protection Agency, 2023) and 2055 (2055gh_ref_16j) (U.S. Environmental Protection Agency, 2024a) anthropogenic emissions projected from 2016 (2016gf) conditions. Biogenic emissions were day and hour specific estimated with the Biogenic Emissions Inventory System (BEIS) version 3.6.1 (Bash et al., 2016). Wildland fire emissions were based on day specific satellite detections and activity information (Larkin et al., 2020). Sea spray emissions were day and hour specific based on meteorology (Gantt et al., 2015). Biogenic, wildfire, sea spray emissions, and lateral boundary inflow conditions were kept the same for projected 2026 and 2055 simulations.

Reduced complexity tool application

The Pattern Constructed Air Pollution Surfaces (PCAPS) model has been applied for complex sector-specific emissions scenarios for stationary and mobile sources and predicted air quality results consistent with more sophisticated models (Baker et al., 2023). PCAPS version 1.1 was applied for each year between 2024 and 2031 with year-specific offshore wind project and EGU emissions. PCAPS was also applied using the same offshore wind and onshore EGU emissions for the 2026 and 2055 scenarios simulated with CMAQ to allow for a direct comparison of results. The database of normalized air quality surfaces in PCAPS v1.1 was updated with new O3 and particulate source apportionment modeling for locations offshore where wind energy projects were anticipated. More details about changes between version 1.0 and version 1.1 of PCAPS are provided in the supporting information.

Human health impacts and monetization

Health impacts and monetization of air quality impacts were estimated using the Environmental Benefits Mapping and Analysis Program—Community Edition (BenMAP–CE; epa.gov/benmap) version 1.5.8 (Sacks et al., 2018). Annual average PM2.5 and seasonal average MDA8 O3 air quality scenario impacts estimated with PCAPS were used as input to BenMAP along with annual projections of population counts; baseline morbidity and mortality rates projected in 5-year increments for 2025 (assigned to 2024-2027) and for 2030 (assigned to 2028-2031); and a set of empirically-derived health impact functions that relate a change in pollutant concentration to incidence of specific morbidity effects and mortality. Present economic value was estimated using an $8.7 million (in 2015 dollars) value of a statistical life and a 3% annual discount rate. A comprehensive description of BenMAP health impact functions and valuation estimation methods applied is provided elsewhere (U.S. Environmental Protection Agency, 2024c).

RESULTS AND DISCUSSION

Emissions Overview

The offshore wind energy projects included in this assessment are listed in Table 1 and shown in Figure S1. Table 1 also provides expected energy capacity and onshore EGU grid interconnect assumption. Annual total NOX, VOC, primary PM2.5, SO2, and NH3 emissions for each offshore wind energy project are provided in Table S2. Figure 1 shows annual total emissions for construction phase, operation and maintenance phase, and energy generation capacity summed for all projects by year. Construction phase emissions for these projects are expected to peak in the mid-2020s while operation phase emissions and generation steadily increase as projects get completed. Total anticipated generation for these projects in 2030 is about 20 GW which is consistent with other projections for this area (DeCastro et al., 2019).

The distribution of AVERT predicted county level percent changes to onshore EGU NOX emissions by month are shown in Figure S2. The decreases in emissions are largest in the winter months when winds are strongest and steadiest resulting in more available energy capacity. The total change predicted by AVERT in county level emissions for NOX, primary PM2.5, SO2, NOX and NH3 based on all offshore projects being complete are shown spatially in Figure S3. Reductions are largest nearest the Atlantic coast in New England. Modest emission reductions for onshore EGUs extend into the Ohio Valley region due to the expansive nature of the mid-Atlantic electrical interconnect utilized by the AVERT tool. Occasional increases in emissions reflect situations where the Monte Carlo scenarios used to relate power and emissions in AVERT result in small load increases at some facilities to generate a balanced solution for the region.

Air Quality Impacts

PCAPS was applied for the same 2026 and 2055 emissions scenarios as CMAQ. Qualitatively, PCAPS generally captures the spatial gradients and magnitudes of the impacts on PM2.5 mass and chemical components predicted by CMAQ for the 2026 emissions scenario (Figure S4, S6, and S7) and 2055 EGU reduction scenario (Figure S4, S8, and S9). PCAPS also captures the spatial gradients of seasonal average MDA8 O3 impacts predicted by CMAQ but tends to underestimate the magnitude (Figure S5).

CMAQ and PCAPS prediction of annual average PM2.5 (Figure S4) and seasonal average MDA8 O3 (Figure S5) for 2026 and 2055 emissions scenarios show impacts from offshore wind project emissions tend to be highest nearest the project and decrease as distance from the project increases. Increases in annual average PM2.5 in 2026 are largely due to secondarily formed nitrate ion and primarily emitted aerosol (Figure S6). Both PM2.5 and O3 impacts were higher for the 2026 scenario compared to 2055 due to the larger (mostly related to construction phase) emissions anticipated for that year. Regional reductions in PM2.5 and O3 were evident related to decreased onshore EGU emissions in the 2055 simulation that included these effects. However, the impact on O3 may not be as large or regionally expansive as PM2.5 since EGU emission reductions were larger in the colder months that are not conducive to O3 formation but favor nitrate partitioning into the aerosol phase (Seinfeld and Pandis, 2008).

Table 2 shows regionally aggregated metrics (Baker et al., 2023) used to compare PCAPS with CMAQ predicted impacts on annual average PM2.5 and seasonal average MDA8 O3 for both the 2026 and 2055 scenarios. PCAPS was well correlated with CMAQ predicted annual average PM2.5 impacts (r=0.94 and 0.96) but predicted slightly lower annual average PM2.5 impacts (NMB=-66% and -8%) in the northeast U.S. PCAPS predictions of seasonal average MDA8 O3 impacts were well correlated (r=0.85 and 0.94) but again underestimated (NMB=-28% and -56%) compared to CMAQ. All the metrics comparing PCAPS and CMAQ were within the range presented for other types of emissions scenarios and reduced complexity tools (Baker et al., 2023).  These performance metrics suggest that PCAPS is reasonably replicating predictions of both PM2.5 and O3 impacts of these emission scenarios from the more sophisticated CMAQ model.

Figure 2 shows annual average PM2.5 impacts associated with year-specific offshore wind project emissions and changes to onshore EGUs for 2024 to 2031. The same is shown in Figure 3 for seasonal average MDA8 O3. These Figures illustrate the dynamic nature of air quality impacts related to changing offshore wind energy project emissions as these projects are constructed (highest emissions period) and then are completed providing additional onshore energy capacity that results in reductions to onshore EGU emissions. Both Figure 2 and Figure 3 show increased air pollution related to the construction of multiple offshore energy projects in the first few years of this time-period. These impacts are highest offshore.

Once these offshore wind energy projects become completed small increases in offshore air pollution are evident for both PM2.5 and O3 related to operation and maintenance related emissions. However, as these projects finish and begin providing additional energy capacity onshore the reduction in onshore EGU emissions result in areas with reduced air pollution. Once all the projects part of this cohort finish construction regional reductions in PM2.5 and O3 are larger and more spatially extensive than the increases near offshore wind energy project locations (Figures 2 and 3). The transition to broad regional reductions in air quality happens more quickly for PM2.5 than O3 since the largest onshore EGU reductions were predicted to happen in the winter when PM2.5 formation is favored meteorologically.

Human Exposure

An assessment of PM2.5 and O3 impacts on human health is a useful way to illustrate the relative importance of offshore wind project emissions increases compared to onshore EGU emission decreases since onshore sources have a closer proximity to populated areas. Figure 4 shows population influenced (the product of PM2.5 or O3 impacts and population) PM2.5 and O3 impacts aggregated by year to illustrate how much offshore construction and operation phase emissions impact populated areas compared to onshore EGU reductions. The same information is provided spatially by year for PM2.5 (Figure S10) and O3 (Figure S11). Figure 5 shows annual mortality as a percent of baseline incidence for respiratory causes related to O3 exposure, all cause related to PM2.5 exposure, and total monetized valuation of O3 and PM2.5 impacts. Annual total mortality for O3 (Table S3) and PM2.5 (Table S4) is provided using information for each of the epidemiological studies used in BenMAP.

Year to year variability in PM2.5 and O3 impacts were strongly related to the amount of offshore construction emissions and available electricity generation from completed projects leading to predicted reductions in onshore EGU emissions. Over the time cycle of projects in this assessment, total human exposure to PM2.5 became a net benefit more quickly than O3, which did not show a net benefit until construction emissions had been fully replaced by operation and maintenance phase emission levels (Figure 4). Mortality expressed as a percent of baseline mortality for O3 showed an increase during the initial years where construction was prevalent at offshore wind projects but once construction was complete and multiple projects were providing additional energy capacity resulting in onshore EGU emission reductions human health benefits were predicted (Figure 5).

Rates of mortality associated with PM2.5 compared to baseline was fairly small compared to O3 during the first years of this assessment (Figure 5). Once construction is complete and additional energy capacity is available leading to less onshore EGU emissions broad regional scale benefits to human health were shown for both O3 and PM2.5. Total monetized valuation followed similar trends to O3 and PM2.5 mortality (Figure 5). Total monetized damages related to mortality total 6.3B$ over the years included in this assessment with 92% of that total related to PM2.5 mortality (Table S5). Given the tendency of PCAPS to predict lower O3 and PM2.5 compared to CMAQ (Table 2) this estimate could be different depending on whether these differences were spatially coincident with large population centers. Annual total monetized damages (Figure 5 and Table S5) were much less than estimated for national rulemakings for the mobile and EGU sector and also several hypothetical scenarios applying emission reductions to specific industrial sectors (Simon et al., 2023).

Limitations

This assessment is based on a single year of meteorology. A single year of meteorology was used to isolate the effects of changing emissions, but the meteorology used may not capture the entirety of important meteorological conditions that result in offshore to shore air flow which could impact population exposure. Further, this study does not attempt to illustrate the impacts wind turbines have on meteorology and how that might impact air quality, although other studies suggest these impacts tend to be localized near turbines (Golbazi and Archer, 2024). The results of this assessment may not be representative of other offshore areas of the United States. Further, these results may not reflect air quality impacts of wind energy projects further from the U.S. shore since transport distances would be longer impacting pollutant dispersal.

New developments and understanding of marine environment chemistry may influence the chemical lifetime of NOX or other oxidants which could influence the amount of O3 and PM2.5 formed near offshore sources and further downwind (Bertram and Thornton, 2009; Kasibhatla et al., 2018; McDuffie et al., 2018). The need to characterize air quality impacts of offshore sources means more extensive model evaluation is needed to provide additional confidence that these modeling systems well represent the offshore chemical and physical environment. Given the paucity of routine offshore air pollutant measurements, these type of model evaluations will need to rely on special field study (McDuffie et al., 2018; Thompson et al., 2023) and space-based (Fedkin et al., 2024) measurements.

The 2026 and 2055 scenarios modeled with CMAQ included adjustments made to future year representations of the EGU sector predicted by the IPM model. However, since the addition of offshore energy capacity was not explicitly included as part of the IPM future year projection it is likely that offshore capacity would influence projected EGU emissions to some extent. The application of emission reductions estimated by AVERT to IPM predicted future years (e.g., 2025 and 2055 in this assessment) limits the full penetration of emission reductions as AVERT sometimes estimates changes to counties that IPM projects an EGU shutdown or significant control. The PCAPS predictions of EGU impacts for specific years was based on actual EGU locations and emissions in 2021 and does not include any representation of potential changes to EGU emissions in the future.

It is possible that increased offshore energy capacity could change emissions for other sectors such as transportation. Relationships between offshore energy capacity and changes to non-fossil fuel sources of energy and other emissions sectors were not included as part of this assessment.

Implications and Future Direction

This assessment of air quality and human health impacts from emissions related to new wind energy projects built off the northern U.S. Atlantic coast suggests negative impacts on human exposure during the construction phase eventually transitioning to positive impacts when construction is complete and increased energy capacity results in onshore EGU emissions reductions. More work is needed to corroborate or improve the estimates of onshore EGU emissions changes that would result from new capacity being available from offshore energy projects. Sophisticated energy dispatch models, EGU sector specific economic projection tools, and regional to national scale energy system emissions projection tools provide additional opportunity to corroborate these findings and potentially provide a more realistic representation of energy distribution related to increased offshore wind energy capacity (Wang et al., 2024).

Future work should consider the variability in air quality introduced using multiple years of meteorology to better understand the potential influence on these results. Further, additional work is needed to understand the change in emissions to other sectors (e.g., mobile sources) and more specificity within onshore energy options (e.g., solar, fossil-fuel facilities, etc.) that would result from the increased electrical capacity provided by offshore wind energy projects. Other types of modeling systems that consider broader relationships between sector emissions and energy (such as the Global Change Analysis Model) (Calvin et al., 2019) could be applied to provide insights on emissions changes in the transportation sector and competition with other types of non-fossil fuel based energy supply. For instance, onshore reductions in air pollution may be less if the increase in offshore capacity was balanced with a reduction in non-fossil fuel energy (e.g., nuclear, onshore wind farms, solar) or simply meeting an increase in future energy demand over the current baseline.

Acknowledgements

The authors very much appreciate the contribution many made to this manuscript: Christian Hogrefe, Golam Sarwar, Colby Tucker, Nancy Hwang, Kevin Talgo, Lara Reynolds, James Beidler, Tyler Fox, Neha Sareen, Jay McAlpine, Brian Marmo, Tim Leon-Guerrero, Eimy Bonilla, and Sarah Carroll.

The views expressed in this article are those of the authors and do not necessarily represent the views or policies of the U.S. Environmental Protection Agency. It has been subjected to the Agency’s review and has been approved for publication. Note that approval does not signify that the contents necessarily reflect the views of the Agency.

Disclaimer

The views expressed in this paper are those of the authors and do not necessarily represent the views or policies of the U.S. EPA. Mention of trade names or commercial products does not constitute endorsement or recommendation for use.

REFERENCES

Appel, K.W., Bash, J.O., Fahey, K.M., Foley, K.M., Gilliam, R.C., Hogrefe, C., Hutzell, W.T., Kang, D., Mathur, R., Murphy, B.N., 2021. The Community Multiscale Air Quality (CMAQ) model versions 5.3 and 5.3. 1: system updates and evaluation. J Geoscientific Model Development 14, 2867-2897.

Baker, K.R., Simon, H., Henderson, B., Tucker, C., Cooley, D., Zinsmeister, E., 2023. Source–Receptor Relationships Between Precursor Emissions and O3 and PM2. 5 Air Pollution Impacts. J Environmental Science Technology 57, 14626-14637.

Baker, K.R., Woody, M.C., 2017. Assessing model characterization of single source secondary pollutant impacts using 2013 SENEX field study measurements. Environmental science & technology 51, 3833-3842.

Bash, J.O., Baker, K.R., Beaver, M.R., 2016. Evaluation of improved land use and canopy representation in BEIS v3. 61 with biogenic VOC measurements in California. J Geoscientific Model Development 9, 2191-2207.

Bergin, M.S., Russell, A.G., Odman, M.T., Cohan, D.S., Chameldes, W.L., 2008. Single-Source Impact Analysis Using Three-Dimensional Air Quality Models. Journal of the Air & Waste Management Association 58, 1351-1359.

Bertram, T., Thornton, J., 2009. Toward a general parameterization of N 2 O 5 reactivity on aqueous particles: the competing effects of particle liquid water, nitrate and chloride. Atmos. Chem. Phys. 9, 8351-8363.

Calvin, K., Patel, P., Clarke, L., Asrar, G., Bond-Lamberty, B., Cui, R.Y., Di Vittorio, A., Dorheim, K., Edmonds, J., Hartin, C., Hejazi, M., Horowitz, R., Iyer, G., Kyle, P., Kim, S., Link, R., McJeon, H., Smith, S.J., Snyder, A., Waldhoff, S., Wise, M., 2019. GCAM v5.1: representing the linkages between energy, water, land, climate, and economic systems. Geosci. Model Dev. 12, 677-698.

Clifton, O.E., Schwede, D., Hogrefe, C., Bash, J.O., Bland, S., Cheung, P., Coyle, M., Emberson, L., Flemming, J., Fredj, E., 2023. A single-point modeling approach for the intercomparison and evaluation of ozone dry deposition across chemical transport models (Activity 2 of AQMEII4). J Atmospheric Chemistry Physics 23, 9911-9961.

Costoya, X., DeCastro, M., Carvalho, D., Gómez-Gesteira, M., 2020. On the suitability of offshore wind energy resource in the United States of America for the 21st century. J Applied Energy 262, 114537.

DeCastro, M., Salvador, S., Gómez-Gesteira, M., Costoya, X., Carvalho, D., Sanz-Larruga, F.J., Gimeno, L., 2019. Europe, China and the United States: Three different approaches to the development of offshore wind energy. J Renewable Sustainable Energy Reviews 109, 55-70.

Emery, C., Jung, J., Koo, B., Yarwood, G., 2015. Improvements to CAMx Snow Cover Treatments and Carbon Bond Chemical Mechanism for Winter Ozone. Prepared for the Utah Department of Environmental Quality, Division of Air Quality, Salt Lake City, UT. Available at: http://www.camx.com/files/udaq_snowchem_final_6aug15.pdf.

Fahey, K.M., Carlton, A.G., Pye, H.O., Baek, J., Hutzell, W.T., Stanier, C.O., Baker, K.R., Appel, K.W., Jaoui, M., Offenberg, J.H., 2017. A framework for expanding aqueous chemistry in the Community Multiscale Air Quality (CMAQ) model version 5.1. Geoscientific Model Development 10.

Fedkin, N.M., Stauffer, R.M., Thompson, A.M., Kollonige, D.E., Wecht, H.D., Elguindi, N., 2024. Satellite NO2 trends and hotspots over offshore oil and gas operations in the Gulf of Mexico. J Earth Space Science 11, e2023EA003165.

Fountoukis, C., Nenes, A., 2007. ISORROPIA II: a computationally efficient thermodynamic equilibrium model for K+-Ca2+-Mg2+-Nh(4)(+)-Na+-SO42--NO3--Cl--H2O aerosols. Atmos. Chem. Phys. 7, 4639-4659.

Gantt, B., Kelly, J., Bash, J., 2015. Updating sea spray aerosol emissions in the Community Multiscale Air Quality (CMAQ) model version 5.0. 2. J Geoscientific Model Development 8, 3733-3746.

Golbazi, M., Archer, C., 2024. Large offshore wind farms have minimal direct impacts on air quality. J Environmental Research Letters.

Hogrefe, C., Bash, J.O., Pleim, J.E., Schwede, D.B., Gilliam, R.C., Foley, K.M., Appel, K.W., Mathur, R., 2023. An analysis of CMAQ gas-phase dry deposition over North America through grid-scale and land-use-specific diagnostics in the context of AQMEII4. J Atmospheric Chemistry Physics 23, 8119-8147.

Kasibhatla, P., Sherwen, T., Evans, M.J., Carpenter, L.J., Reed, C., Alexander, B., Chen, Q., Sulprizio, M.P., Lee, J.D., Read, K.A., 2018. Global impact of nitrate photolysis in sea-salt aerosol on NO x, OH, and O 3 in the marine boundary layer. Atmos. Chem. Phys. 18, 11185-11203.

Larkin, N.K., Raffuse, S.M., Huang, S., Pavlovic, N., Lahm, P., Rao, V., 2020. The comprehensive fire information reconciled emissions (CFIRE) inventory: Wildland fire emissions developed for the 2011 and 2014 US National Emissions Inventory. J Journal of the Air Waste Management Association 70, 1165-1185.

Mailloux, N.A., Abel, D.W., Holloway, T., Patz, J.A., 2022. Nationwide and regional PM2. 5‐related air quality health benefits from the removal of energy‐related emissions in the United States. J GeoHealth 6, e2022GH000603.

McCubbin, D., Sovacool, B.K., 2013. Quantifying the health and environmental benefits of wind power to natural gas. J Energy policy 53, 429-441.

McDuffie, E.E., Fibiger, D.L., Dubé, W.P., Lopez Hilfiker, F., Lee, B.H., Jaeglé, L., Guo, H., Weber, R.J., Reeves, J.M., Weinheimer, A.J., 2018. ClNO2 yields from aircraft measurements during the 2015 WINTER campaign and critical evaluation of the current parameterization. Journal of Geophysical Research: Atmospheres 123, 12,994-913,015.

Millstein, D., Wiser, R., Bolinger, M., Barbose, G., 2017. The climate and air-quality benefits of wind and solar power in the United States. J Nature Energy 2, 1-10.

Rezaee, A., Chen, L.-W.A., Lin, G., Buttner, M.P., Gakh, M., Bloomfield, E.F., 2022. Air Quality Health Benefits of the Nevada Renewable Portfolio Standard. J Atmosphere 13, 1387.

Sacks, J.D., Lloyd, J.M., Zhu, Y., Anderton, J., Jang, C.J., Hubbell, B., Fann, N., 2018. The Environmental Benefits Mapping and Analysis Program–Community Edition (BenMAP–CE): A tool to estimate the health and economic benefits of reducing air pollution. J Environmental Modelling Software 104, 118-129.

Sarwar, G., Gantt, B., Schwede, D., Foley, K., Mathur, R., Saiz-Lopez, A., 2015. Impact of enhanced ozone deposition and halogen chemistry on tropospheric ozone over the Northern Hemisphere. J Environmental science technology 49, 9203-9211.

Seinfeld, J., Pandis, S., 2008. Atmospheric chemistry and physics. 1997. New York.

Simon, H., Baker, K.R., Sellers, J., Amend, M., Penn, S.L., Bankert, J., Chan, E.A., Fann, N., Jang, C., McKinley, G., Zawacki, M., Roman, H., 2023. Evaluating reduced-form modeling tools for simulating ozone and PM 2.5 monetized health impacts. J Environmental science: atmospheres 3, 1306-1318.

Thompson, A.M., Kollonige, D.E., Stauffer, R.M., Kotsakis, A.E., Abuhassan, N., Lamsal, L.N., Swap, R.J., Blake, D.R., Townsend‐Small, A., Wecht, H.D., 2023. Two air quality regimes in total column NO2 over the Gulf of Mexico in May 2019: Shipboard and satellite views. J Earth Space Science 10, e2022EA002473.

U.S. Environmental Protection Agency, 2019a. Integrated Science Assessment (ISA) for Particulate Matter (Final Report, Dec 2019). U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-19/188, 2019.

U.S. Environmental Protection Agency, 2019b. Meteorological Model Performance for Annual 2016 Simulation WRF v3.8. EPA-454/R-19-010.

U.S. Environmental Protection Agency, 2020. U.S. EPA. Integrated Science Assessment (ISA) for Ozone and Related Photochemical Oxidants (Final Report, Apr 2020). U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-20/012, 2020.

U.S. Environmental Protection Agency, 2022. Supplement to the 2019 Integrated Science Assessmenet for Particulate Matter. EPA/600/R-22/028. www.epa.gov/isa.

U.S. Environmental Protection Agency, 2023. Technical Support Document (TSD): Preparation of Emissions Inventories for the 2016v3 North American Emissions Modeling Platform. EPA-454/B-23-002

U.S. Environmental Protection Agency, 2024a. Air Quality Analysis for the Light- and Medium-Duty Vehicle Multipollutant Rule. EPA-420-R-24-008. https://nepis.epa.gov/Exe/ZyPDF.cgi?Dockey=P1019Z9X.pdf.

U.S. Environmental Protection Agency, 2024b. AVoided Emissions and geneRation Tool (AVERT) User Manual Version 4.3. https://www.epa.gov/system/files/documents/2024-04/avert-user-manual-v4.3.pdf.

U.S. Environmental Protection Agency, 2024c. Estimating PM2.5- and Ozone-Attributable Health Benefits: 2024 Update. https://www.epa.gov/system/files/documents/2024-06/estimating-pm2.5-and-ozone-attributable-health-benefits-tsd-2024.pdf.

Wang, S., Wang, H., Ellis, J.H., Hobbs, B.F., 2024. Linking Electricity and Air Quality Models by Downscaling: Weather-Informed Hourly Dispatch of Generation Accounting for Renewable and Load Temporal Variability Scenarios. J Environmental Science Technology.

Wiser, R., Bolinger, M., Heath, G., Keyser, D., Lantz, E., Macknick, J., Mai, T., Millstein, D., 2016. Long-term implications of sustained wind power growth in the United States: Potential benefits and secondary impacts. J Applied Energy 179, 146-158.

TOC ART

Table 1. Offshore wind energy projects, anticipated year of completed construction, anticipated total generation, and onshore interconnect assumption used in the AVERT tool application. Only projects with emissions information at the time of this assessment are included.

Table 2. Metrics aggregated for the northeast region of the U.S. comparing model predicted annual average PM2.5 and seasonal average MDA8 O3 by the PCAPS reduced complexity tool and CMAQ photochemical grid model. Results are provided by region and both the 2026 and 2055 emissions scenarios. The 2055 simulation includes both offshore wind emissions and changes to onshore EGUs.

Figure 1. Annual total NOX emissions for all offshore wind energy projects included in this assessment. Emissions are shown separately for the construction phase and operation and maintenance phase. Total anticipated generation is also shown.

Figure 2. Annual average PM2.5 predicted by PCAPS based on emissions related to offshore wind energy projects and onshore EGUs. Warm colors indicate an increase and cool colors indicated a decrease in PM2.5.

Figure 3. Seasonal (April to September) average MDA8 O3 predicted by PCAPS based on emissions related to offshore wind energy projects and onshore EGUs. Warm colors indicate an increase and cool colors indicated a decrease in MDA8 O3.

Figure 4. Population influenced annual average PM2.5 (left) and seasonal average MDA8 O3 (right) impacts aggregated over the entire model domain. The contribution from wind project construction phase, operation and maintenance phase, and EGU sector are shown.

Figure 5. Predicted mortality as a percent of baseline incidence related to O3 (respiratory) and PM2.5 (all cause) for each year modeled as part of this assessment. Monetized valuation that includes both O3 and PM2.5 is also shown. Positive numbers represent an increase in mortality and negative numbers represent a decrease in mortality.

Supporting Information

Characterizing Air Quality Impacts Related to North Atlantic Offshore Wind Energy Projects

Kirk R. Baker, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, USA*

Byron Rice, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, USA

Neal Fann, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, USA

*Corresponding author contact: baker.kirk@epa.gov

The supporting information is organized to first show Tables and Figures specifically referenced in the manuscript followed by a section providing more details of changes to the PCAPS tool since the original formulation was published. These changes are largely minor (e.g., bug fixes) with a substantive update related to an improved representation of primarily emitted PM2.5 which is relevant for the EGU sources modeled as part of this assessment.

Table S1. Model simulations done using CMAQ (simulations 1 to 6) and PCAPS (simulations 7 to 17). Rows colored in blue indicate where emissions were the same for each model to support a direct quantitative comparison of results. The PCAPS model is applied only with emissions changes so some components (such as anthropogenic emissions) were not included in the model application. All CMAQ simulations used 2016 meteorology and the same lateral boundary inflow conditions. PCAPS does not have meteorology or lateral boundary inflow as input options.

Table S2. Annual total emissions for each offshore wind energy project.

Table S2 (continued). Annual total emissions for each offshore wind energy project.

Table S2 (continued). Annual total emissions for each offshore wind energy project.

Table S3. Health endpoints predicted for each annual simulation for O3 (respiratory) mortality.

Table S4. Health endpoints predicted for each annual simulation for PM2.5 (all cause) mortality.

Table S5. Valuation of health endpoints predicted for each annual simulation for O3 (respiratory) and PM2.5 (all cause) mortality.

Figure S1. The location of offshore areas leased for wind energy projects. Areas included in this assessment are indicated with a triangle symbol.

Figure S2. County and month specific percent change in EGU emissions predicted by AVERT based on increased energy capacity available from the offshore wind energy projects included in this assessment.

Figure S3. Total emissions changes by county and pollutant predicted by AVERT based on increased energy capacity available from the offshore wind energy projects included in this assessment.

Figure S4. CMAQ (top row) and PCAPS (bottom row) predicted annual average PM2.5 impacts for multiple emissions scenarios. Warm colors indicate an increase in PM2.5 and cool colors represent a decrease in PM2.5. The first column represents the 2026 scenario with only the offshore emission effects, the second column represents the 2055 scenario with only the offshore emissions effect, and the last column represents the 2055 scenario with both the offshore and onshore emission effects.

Figure S5. CMAQ (top row) and PCAPS (bottom row) predicted seasonal average MDA8 O3 impacts for multiple emissions scenarios. Warm colors indicate an increase in O3 and cool colors represent a decrease in O3. The first column represents the 2026 scenario with only the offshore emission effects, the second column represents the 2055 scenario with only the offshore emissions effect, and the last column represents the 2055 scenario with both the offshore and onshore emission effects.

Figure S6. CMAQ predicted annual average PM2.5 (top left) and chemical components: nitrate ion (top right), organic and elemental carbon (bottom left), and sulfate ion (bottom right) for the 2026 scenario.

Figure S7. PCAPS predicted annual average PM2.5 (top left) and chemical components: nitrate ion (top right), organic and elemental carbon (bottom left), and sulfate ion (bottom right) for the 2026 scenario.

Figure S8. CMAQ predicted annual average PM2.5 (top left) and chemical components: nitrate ion (top right), organic and elemental carbon (bottom left), and sulfate ion (bottom right) for EGU impacts related to the 2055 scenario.

Figure S9. PCAPS predicted annual average PM2.5 (top left) and chemical components: nitrate ion (top right), organic and elemental carbon (bottom left), and sulfate ion (bottom right) for EGU impacts related to the 2055 scenario.

Figure S10. PCAPS predicted population-influenced annual average PM2.5 impacts. The population-influenced metric is the annual average PM2.5 model predicted impact multiplied by population in the same grid cell. Warm colors indicate an increase in exposure and cool colors indicate a decrease in exposure.

Figure S11. PCAPS predicted population-influenced seasonal average MDA8 O3 impacts. The population-influenced metric is the seasonal average MDA8 O3 model predicted impact multiplied by population in the same grid cell. Warm colors indicate an increase in exposure and cool colors indicate a decrease in exposure.

This section is intended to provide information about updates to the original version of the Pattern Constructed Air Pollution Surfaces (PCAPS) reduced complexity tool (Baker et al., 2023). The PCAPS tool provides estimates of annual average PM2.5 and seasonal average maximum daily average 8-hr (MDA8) ozone (O3) using photochemical model source apportionment developed relationships between emission rates and spatial pollution gradients (Baker et al., 2023). Here, updates to the tool are discussed that address some limitations related to the original training dataset for primary PM2.5. The updated tool is compared against the original set of emissions control scenarios used for evaluation and additional scenarios intended to further challenge the tool.

The training database of hypothetical sources is important to capture regional meteorology and orographic influence. The number of generic sources (N=36) that made up the training database for primary PM2.5 was smaller compared to NOX and SO2 and did not capture large areas of the western and southern U.S. (Figure PCAPS-1). New photochemical model source apportionment simulations were done for 168 sources using 2016 meteorology to provide better spatial coverage of the U.S.

Figure PCAPS-1. Location of generic sources tracked with photochemical model source apportionment making up the primary PM2.5 training database for PCAPS.

In addition to providing broader spatial coverage, each of these new sources were tracked for contribution at 4 different stack heights: surface level, 90 m, 150 m, and 300 m. The original training database for primary PM2.5 only had air pollution spatial surfaces for surface level and 90 m release heights.

Numerous minor bug fixes and development updates made minor changes to prediction of all the pollutants predicted by PCAPS. The downwind extent used for the receptor grid was a square box sized 3,600 km 3,600 km in the original iteration of this tool and was extended in v1.1 to include the entirety of the grid domain used for the generic source modeling. These changes and the updates to the primary PM2.5 training dataset were integrated into PCAPS version 1.1. PCAPs version 1.1 was applied for the same evaluation datasets as shown in Baker et al, 2023. Aggregate performance metrics for the conterminous U.S. are shown in Table PCAPS-1 for seasonal average MDA8 O3 and annual average PM2.5. Mean bias (MB), normalized mean bias (NMB), fractional bias (FB), and fractional error (FE) are best when the metrics approach 0. The correlation coefficient (r) is best when the metric approaches 1. Performance for PCAPS v1.1 was consistent or better than PCAPS v1.0 for these evaluation emissions scenarios. Further, performance was consistent or better than similar tools (Baker et al., 2023).

Table PCAPS-1. Performance metrics comparing PCAPS with photochemical model (PGM) predicted change in seasonal MDA8 O3 and annual average PM2.5 with two complex emission control scenarios for each grid cell in multiple regions and the entire U.S. Modeled means and mean bias (MB) expressed in ppb for O3 and ug/m3 for PM2.5. Normalized mean bias (NMB), fractional bias (FB), and fractional error (FE) expressed as a percent.

Figures PCAPS-2 and PCAPS-3 show PCAPS v1.1 predicted spatial gradients of seasonal MDA8 O3 and annual average PM2.5. These compare well with the photochemical model predicted spatial gradients and are consistent with predictions using version 1.0 of PCAPS. PCAPS tended to underestimate the impacts of the mobile and EGU scenario while overestimating the impacts of the scenarios for the other three sectors.

Figure PCAPS-2. Spatial representation of photochemical (left panels) and PCAPS (center panels) model prediction of change in seasonal average MDA8 O3 for each of the emission scenarios. The difference between predicted change in air quality between these models is also shown (right panels).

Figure PCAPS-3. Spatial representation of photochemical (left panels) and PCAPS (center panels) model prediction of change in annual average PM2.5 for each of the emission scenarios. The difference between predicted change in air quality between these models is also shown (right panels).