File descriptions

- In all AERMOD input files, coordinates for sources and receptors are in UTM coordinates, UTM zone 11 (see TITLETWO of AERMOD input files for UTM zone)

- All sources with run a reference emission rate of 10,000 tons/year

- MMIF_AERMET_2020_33.881N_118.220W.SFC and MMIF_AERMET_2020_33.881N_118.220W.PFL are meteorological inputs for all sources listed here.

  - From 12-km WRF runs processed in the MMIF pre-processor and then through AERMET

Folder heavy_duty_onroad

aermod_G040R098.inp

- AERMOD input file for 4-km heavy duty on-network onroad sources

- 4 km sources are run as AREAPOLY sources

- Receptors are gridded 1-km receptors

- Hourly emissions are in file G040R098_hourly.dat

- Outputs are monthly POSTFILES by source group

  - Annual concentrations calculated in post-processing

Folder nonroad

aermod_G040R098.inp

- AERMOD input file for 12-km nonroad emissions

- 12 km source are run as AREAPOLY source

- Receptors are gridded 1-km receptors

- Emissions vary by hour of day

- Outputs are monthly POSTFILES

  - Annual concentrations calculated in post-processing

Folder point

aermod_2121911.inp

- AERMOD input file for Independent Ink Inc

- One 10x10 m area source and one point source

- Emissions vary by month

- 3,729 census block receptors modeled in the AERMOD run (out to 5 km from the facility)

- 9,890 gridded 1-km receptors modeled in the AERMOD run

- 0 monitors modeled in the AERMOD run (out to 5 km from facility)

- 183,553 census block receptors and 28 monitors will be interpolated from 5-50 km from the source

  - Contained in interpolate_recs_2121911.txt

    - File format

      - Receptor type (2=census block receptor, 3=monitor)

      - Receptor ID (if block, census block ID, if monitor, monitor ID)

      - Receptor counter 1

      - Receptor counter 2

      - CMAQ column and row of receptor

      - X-coordinate in Lambert Conformal projection (CMAQ projection)

      - y-coordinate in Lambert Conformal projection (CMAQ projection)

      - UTM x-coordinate

      - UTM y-coordinate

      - Integer flag indicating if receptor within 10 km of facility