Quality Criteria Evaluation (QSCORE)—Alvarez Sugar Cane Paper:  20

The Quality Criteria Factors provide an evaluation framework to easily recognize and assign value points to indicators of a strong, well planned and executed study, which is presented in a complete and logical manner.  The presentation of air emission profile data can be in the form of a peer-reviewed publication, or report.  The QSCORE evaluation can be applied to three varieties of research studies based on data derived from; 1) original measurements, 2) non-original composite measurement data, and, 3) synthesized measurement data resulting from model generation.

The evaluation framework is meant to guide the reviewer to assign quality value points to the areas of the study deemed most important for use in SPECIATE.  The framework is meant to be comprehensive, but should also be easy to understand and apply, not rigid and overly detailed.  A point to each question adds-up to an evaluation score.  An ideal point score would have 30 (Data from Measurements) or 26 (Data from other Methods) desired criteria (points).  Each point or points is additive, influencing, but not necessarily distinguishing the study.  The publication or report should be ranked as high as possible for inclusion into the SPECIATE database.  The evaluation score check points rank as follows:

20-30 = excellent

12-19 = good

5-11 = fair

<4 = poor

Each ranking will be added to the SPECIATE Literature Database.  Only the excellent, good, fair, poor ratings will go into SPECIATE.  The rankings can go on SharePoint.

DATA FROM MEASUREMENTS - (Ideal score of 30)

1.	Are data from a peer-reviewed publication?	1	(1 Point)

2.	Is the source U.S. based and does it relate to an NEI source?	0.5	(1 Point)

3.	Is the author well known or affiliated with a well-known institute (i.e., EPA)?  0.5(1 Point)

4.	Is the emission source current, are up-to-date technologies employed (collection, measurement, analysis)?  		1					(1 Point)

5.	Is subject source identified from “PRIORITY NEEDS” review? 1		(1 Point)

6.	Were data collected under an established quality system or sufficiently addressed /are QA/QC activities associated with the data collection/measurements included in the publication or supplementary information? 	1				(1 Point)

(If this isn’t stated in the paper or supplementary information, the author may need to be contacted.  If it is an EPA paper or from a Star Grant, it gets a point.)

7.	Sampling Design

a.	Is the sampling design discussed logically (logic behind the experiments)?1	(1 Point)

b.	Are the Data limitations clear (i.e., can the reviewer easily figure them out or are they explicitly stated)?		0						(1 Point)

c.	Are assumptions clearly stated?  (e.g., fireplace is representative of typical fireplace found throughout the country)	1						(1 Point)

d.	Are samples capturing the natural variability of the sources?	0		(1 Point)

8.	Measurement Methodologies (Has the author discussed the method used to take measurements?  Did that discussion include the preparation and use of instrumentation used; were considerations given to QA/QC restrictions or limitations; was it stated, inferred or did it appear that QA/QC guidance was used to structure data collection and measurement; were the measurements taken using equipment following standard methods [i.e., EPA, NIST]?)  1

a.	Is measurement instrumentation presented or referenced? 		1	(1 Point)

b.	Are the Data limitations clear?		1				(1 Point)

c.	Were measurements taken using standard methods (EPA, NIST), and applicable/up-to-date technologies, methods, and instrumentation?	1				(1 Point)

d.	Are measurements done in duplicates or triplicates?	1			(1 Point)

(Measurement methods using duplicate or triplicate collection implies that the study payed attention to data accuracy, representation and reproducibility.  This attention should be viewed as an advantage.)

9.	Data work up (statistics)

a.	Are standard deviations presented in the paper?  (SDs are needed in the profile or we would contact the PI to get it.) 	0						(1 Point)

b.	Are Standard deviations acceptable for the type of source and pollutants measured? 0

NOTES:  there were none. Probably because only 2 measurements.											(1 Point)

c.	Is the data ready for listing?  	1						(1 Point)

(i.e., Data is already in emission factor form, not in need of conversion or clarification; units consistently used throughout the publication; appropriate number of significant figures reported?)

d.	Is there complete speciation data of PM or VOC provided?	5		(1-10 Points)

(I.e., For VOC, does the profile include a total amount of gaseous organic compounds (TOG), TOG should include 	(1) methane;

(2) alkanes, alkenes and aromatic VOC;

(3) alcohols;

(4) aldehydes;

PM2.5 should include critical pollutants such as

(1) EC and OC1) EC/OC;

(2) sulfate/nitrate/NH4+ ions;

(3) metals/inorganics.

It would be a plus if PAHs and SVOCs are available.  Are there missing measurable components that need to be included to SPECIATE?

10.	The overall evaluation should ask; is the paper transparent with regards to describing sampling, test methods and data manipulation? Did the clarity and purpose of this paper leave a positive impression. 				2			(1-3 Points)

(This point is meant to be based on the reviewer’s impression of the paper, not a hard-fast scale.  Granted my impression may differ from another reviewer, but that variance seems not necessarily bad.)

DATA FROM OTHER METHODS (Based on non-original composite measurement data) (Ideal score of 29)

OTHER METHODS: Any paper where they didn’t directly measure what they report in the paper.   Examples of other methods:  Urbanski paper (putting together others’ work), profile for flares (FLR99) that estimated the composition from the TCEQ test of propylene.

1.	Are data from a peer-reviewed publication?					(1 Point)

2.	Is the source U.S. based and does it relate to an NEI source?			(1 Point)

3.  	Is the author well known or affiliated with a well-known institute (i.e., EPA)?													(1 Point)

4.	Is the source a relatively new dataset, specifically, is the paper from year 2000 or later?  												(1 Point)

5.	Is subject source identified as “priority” from Casey review?			(1 Point)

6. 	Composite Data Development

a.	Are data based on an established, acceptable methodology?  			(2 Points)

b.	If any of the values or data are based on assumptions or calculations are they clearly documented? 									(2 Points)

c.	Was post-processing used for the data?  If so, is it novel, reasonable or widely accepted?												(2 Points)

7. 	Is there complete speciation data of PM or VOC provided?			(1 -10 Points)

(I.e., For VOC, does the profile include a total amount of gaseous organic compounds (TOG), TOG should include 	(1) methane;

(2) alkanes, alkenes and aromatic VOC;

(3) alcohols;

(4) aldehydes;

PM2.5 should include critical pollutants such as

(1) EC and OC1) EC/OC;

(2) sulfate/nitrate/NH4+ ions;

(3) metals/inorganics.

It would be a plus if PAHs and SVOCs are available.  Are there missing measurable components that need to be included to SPECIATE?

8.	Are assumptions clearly stated?  (i.e., fireplace is representative of typical fireplace found throughout the country)							(2 Points)

9.  	Data work up (statistics)

a.	Are standard deviations presented in the paper?  (SDs are needed in the profile or we would contact the PI to get it.) 							(1 Point)

b.	Are Standard deviations acceptable for the type of source and pollutants measured? 												(1 Point)

c.	Is the data ready for listing?  							(1 Point)

(i.e., Data is already in emission factor form, not in need of conversion or clarification; units consistently used throughout the publication; appropriate number of significant figures reported?)

10.	The overall evaluation should ask; is the paper transparent with regards to describing data sources and data manipulation? Did the clarity and purpose of this paper leave a positive impression. 									(1-3 points)

(This point is meant to be based on the reviewer’s impression of the paper, not a hard-fast scale.  Granted my impression may differ from another reviewer, but that variance seems not necessarily bad.)

PROFILE DATA FROM OTHER METHODS (Based on synthesized measurement data resulting from model generation.) (Ideal score of 29)

OTHER METHODS: Profile data that estimated the composition.

1.	Is profile data from a peer-reviewed publication?				(1 Point)

2.	Is the source U.S. based and does it relate to an NEI source?			(1 Point)

3.  	Is the author well known or affiliated with a well-known institute (i.e., EPA)?													(1 Point)

4.	Is the source a relatively new dataset, specifically, is the paper from year 2000 or later?  												(1 Point)

5.	Is subject source identified as “priority” from Casey review?			(1 Point)

6. 	Composite Data Development

a.	Are data based on an established, acceptable methodology?  			(2 Points)

b.	If any of the values or data are based on assumptions or calculations are they clearly documented? 									(2 Points)

c.	Was post-processing used for the data?  If so, is it novel, reasonable or widely accepted?												(2 Points)

7. 	Is there complete speciation data of PM or VOC provided?			(1 -10 Points)

(I.e., For VOC, does the profile include a total amount of gaseous organic compounds (TOG), TOG should include 	(1) methane;

(2) alkanes, alkenes and aromatic VOC;

(3) alcohols;

(4) aldehydes;

PM2.5 should include critical pollutants such as

(1) EC and OC1) EC/OC;

(2) sulfate/nitrate/NH4+ ions;

(3) metals/inorganics.

It would be a plus if PAHs and SVOCs are available.  Are there missing measurable components that need to be included to SPECIATE?  Do weight percents add to 100?  Are fields complete?

8.	Are assumptions clearly stated?  (i.e., fireplace is representative of typical fireplace found throughout the country)							(2 Points)

9.  	Data work up (statistics)

a.	Are standard deviations presented in the paper?  (SDs are needed in the profile or we would contact the PI to get it.) 								(1 Point)

b.	Are Standard deviations acceptable for the type of source and pollutants measured? 												(1 Point)

c.	Is the data ready for listing?  							(1 Point)

(i.e., Data is already in emission factor form, not in need of conversion or clarification; units consistently used throughout the publication; appropriate number of significant figures reported?)

10.	The overall evaluation should ask; is the paper transparent with regards to describing data sources and data manipulation? Did the clarity and purpose of this paper leave a positive impression. 									(1-3 points)

(This point is meant to be based on the reviewer’s impression of the paper, not a hard-fast scale.  Granted my impression may differ from another reviewer, but that variance seems not necessarily bad.)