NAME OF REFERENCE:  STRUM2020c (Profile Workbook (WB_DJCONDENSATEFLARE.xlsx),  November 19, 2020. See README and DOCUMENTATION TAB.)

PROFILE: DJTFLR95

DATE: November 30, 2020

PANEL: Marc Menetrez, Madeleine Strum, George Pouliot, Casey Bray, Tesh Rao

22-30 = excellent

16-21 = good

8-15 = fair

≤7 = poor

DATA FROM OTHER METHODS (Blended) (Ideal score of 29)

OTHER METHODS: Any paper where the researches did not directly measure what they report in the paper. Examples of other methods: Urbanski 2014 (putting together others’ work), profile for flares (FLR99) that estimated the composition from a test of propylene.

Total = 20

| No. | Question | Total Points | This Reference |
| --- | --- | --- | --- |
| 1 | Are data from a peer-reviewed publication? | 1 | 0 |
| 2 | Is the source U.S. based or does it relate to a National Emissions Inventory (NEI) source? | 1 | 1 |
| 3 | Is the author well known or affiliated with a well-known research organization in conducting speciated source measurements or analyses? (Ramboll’s approach) | 1 | 1 |
| 4 | Is the emission source current, are up-to-date technologies employed (collection, measurement, analysis)? -underlying data fairly current (Eban Thoma and Texas Flare study) | 1 | 1 |
| 5 | Is subject source identified as “priority” source (see, for example, the study: Bray, et. al.1) | 1 | 1 |
| 6 | - Composite Data Development |  |  |
| 6a | Are data based on an established, acceptable methodology? (used in SPECIATE 4.5) | 2 | 1 |
| 6b | If any of the values or data are based on assumptions or calculations are they clearly documented? | 2 | 2 |
| 6c | Was post-processing used for the data? If so, is it novel, reasonable or widely accepted? | 2 | 1 |
| 7 | Is there complete speciation data of PM or organic gas provided? For organic gas, does the profile include a total amount of gaseous organic compounds (TOG), TOG should include (1) methane; (2) alkanes, alkenes and aromatic VOC; (3) alcohols; (4) aldehydes. PM2.5 should include critical pollutants such as (1) EC and OC; (2) sulfate/nitrate/NH4+ ions; (3) metals/inorganics. Higher scores are given if PAHs and SVOCs are also available. Hg should include: (1) Elemental mercury (Hg0) (2) Reactive Gas mercury (a.k.a. ionic) (3) Particulate form Scoring guidance for Hg profiles: One species=2, Two species=6, all three species=10 | 1-10 | 8 |
| 8 | Are assumptions clearly stated? (i.e., fireplace is representative of typical fireplace found throughout the country) | 2 | 1 |
| 9 | Data reduction procedures (statistics) |  |  |
| 9a | Are standard deviations (SDs) presented in the paper? (SDs are needed in the profile or we would contact the PI to get it.) | 1 | 0 |
| 9b | Are SDs acceptable for the type of source and pollutants measured? | 1 | 0 |
| 9c | Are the data ready for listing? (i.e., data are already in emission factor form, not in need of conversion or clarification; units consistently used throughout the publication; appropriate number of significant figures reported?) | 1 | 1 |
| 10 | The overall evaluation should ask; is the paper transparent with regards to describing sampling, test methods and data manipulation? Did the clarity and purpose of this paper leave a positive impression? (This element is meant to be based on the EPA reviewer’s impression of the paper, not a hard-fast scale, and may vary from one reviewer to another.) | 1-3 | 2 |

- Bray, et. al. 2019. Bray, C.D., Strum, M., Simon, H., Riddick, L., Kosusko, M., Menetrez, M., Hays, M.D., Rao, V., 2019. An Assessment of Important SPECIATE Profiles in the EPA Emissions Modeling Platform and Current Data Gaps. Atmospheric Environment 207, 93-104. DOI: 10.1016/j.atmosenv.2019.03.013