FUGITIVE DUST FROM UNPAVED ROADS

a. Source Category Description

Fugitive dust emissions from unpaved road traffic were estimated for PM10-PRI, PM10-FIL, PM25-PRI, and PM25-FIL. Since there are no PM-CON emissions for this category, PM10-PRI emissions are equal to PM10-FIL emissions and PM25-PRI emissions are equal to PM25-FIL emissions.

For this source category, the following SCC was assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2296000000 | Mobile Sources | Unpaved Roads | All unpaved Roads | Total: Fugitives |

Uncontrolled unpaved road emissions were calculated at the county level by roadway type for the year 2014. This was done by multiplying the county/roadway class unpaved road vehicle miles traveled (VMT) by the appropriate unpaved road emission factor. Next, control factors were applied to the unpaved road emissions in PM10 nonattainment and maintenance area counties. Emissions by roadway class were then totaled to the county level for reporting in the NEI. The following provides further details on the emission factor equation, determination of unpaved road VMT, and controls.

b. Emission Factor Equation

Re-entrained road dust emissions for unpaved roads were estimated using paved road VMT and the emission factor equation from AP-421:

E = [k × (s/12)1 × (SPD/30)0.5] / (M/0.5)0.2 - C

Where k and C are empirical constants given in Table 1, with:

E = unpaved road dust emission factor (lb/VMT)

k = particle size multiplier (lb/VMT)

s = surface material silt content (%)

SPD = mean vehicle speed (mph)

M = surface material moisture content (%)

C = emission factor for 1980’s vehicle fleet exhaust, brake wear, and tire wear (lb/VMT)

The uncontrolled emission factors without precipitation corrections are in the worksheet “Emission Factor Calculations” by county and roadway class.

Values used for the particle size multiplier and the 1980’s vehicle fleet exhaust, brake wear, and tire wear are provided in Table 1, and come from AP-421 defaults.

Average State-level unpaved road silt content values, developed as part of the 1985 NAPAP Inventory, were obtained from the Illinois State Water Survey2. Silt contents of over 200 unpaved roads from over 30 States were obtained. Average silt contents of unpaved roads were calculated for each sate that had three or more samples for that State. For States that did not have three or more samples, the average for all samples from all States was used as a default value. The silt content values are by State, and identifies if the values were based on a sample average or default value.

Table 1: Constants for Unpaved Roads re-entrained dust emission factor equation1

| Constant | PM25-PRI/PM25-FIL | PM10-PRI/PM10-FIL |
| --- | --- | --- |
| k (lb/VMT) | 0.18 | 1.8 |
| C | 0.00036 | 0.00047 |

Table 2 lists the speeds modeled on the unpaved roads by roadway class. These speeds were determined based on the average speeds modeled for onroad emission calculations and weighted to determine a single average speed for each of the roadway classes.  The roadway class “Urban collector” with an average speed of 20 mph was split into two sub-categories, “Urban major collector” and “Urban minor collector”, to correspond to the roadway types found in the 2014 VMT data.

Table 2: Speeds modeled by roadway type on Unpaved Roads3

| Unpaved Roadway Type | Speed (mph) |
| --- | --- |
| Rural Minor Arterial | 39 |
| Rural Major Collector | 34 |
| Rural Minor Collector | 30 |
| Rural Local | 30 |
| Urban Other Principal Arterial | 20 |
| Urban Minor Arterial | 20 |
| Urban Major Collector | 20 |
| Urban Minor Collector | 20 |
| Urban Local | 20 |

The value of 0.5 percent for M was chosen as the national default as sufficient resources were not available at the time the emissions were calculated to determine more locally-specific values for this variable.

c. Activity

Total annual VMT estimates by county and roadway class were derived from a 2011 EPA Motor Vehicle Emission Simulator (MOVES) modelling run. To estimate the portion of the total VMT occurring on unpaved roads, 2013 state-level FHWA data on length of unpaved roads by road type4 was used with 1996 ratios from FHWA (the last year these data were available) on average daily traffic volume per mile of unpaved road by road type.5

d. Allocation

State-level estimates of unpaved road VMT were allocated to the county level based on the proportion of rural population in the county, according to the 2010 Census.

e. Controls

The controls assumed for unpaved roads varied by PM10 nonattainment area classification and by urban and rural areas. On urban unpaved roads in moderate PM10 nonattainment areas, paving of the unpaved road was assumed and a control efficiency of 96 percent and a rule penetration of 50 percent were applied. Controls were not applied to rural unpaved roads in moderate nonattainment areas.  Chemical stabilization, with a control efficiency of 75 percent and a rule penetration of 50 percent, was assumed for rural areas in serious PM10 nonattainment areas. A combination of paving and chemical stabilization, with a control efficiency of 90 percent and a rule penetration of 75 percent, was assumed for urban unpaved roads in serious PM10 nonattainment areas.  In counties currently at maintenance status, controls were assumed based on the severity (moderate or serious) of their prior nonattainment status.  Some counties had multiple partial areas with differing levels of nonattainment.  In these cases, controls were assumed to be applied based on the most serious level of nonattainment found within a given county.

Note that the controls were applied at the county level, and the controls differ by urban vs. rural roadway class. In the final emissions table, the emissions for all roadway classes were summed to the county level. Therefore, the emissions at the county level can represent several different control effectiveness and rule penetration levels. However, the control efficiency and rule penetration values were reported in the Controlled Emissions worksheet at the county level for urban and rural roadways separately.

f. Changes from 2011 Methodology

The methodology described above contains several adjustments from the methodology used to compose the 2011 version.  This is due in part to differences in data sources used to compile the inventory. In 2014, the factors used to adjust for precipitation were removed from the 2011 emission factor equation, and precipitation was not accounted for in the final inventory.

The VMT data used in 2014 was based on EPA’s MOVES model, whereas 2011 VMT data was based on its precursor NMIM model.  In 2011 state-level VMT data was broken down by month and corrected for temporal variation, while in 2014 VMT data was at the county level and no temporal adjustments were made. Because VMT was only broken down by state in 2011, calculating ADTV required using state-level volume categories for urban and rural roads that included an assumed ADTV for a given range of vehicles per day per mile.  In 2014, ADTV was not calculated for unpaved roads as it wasn’t needed for the activity data or emission factor equation.

g. Puerto Rico and US Virgin Islands Emissions Calculations

Since insufficient data exists to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emission factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emission factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

h. References

- United States Environmental Protection Agency, Office of Air Quality Planning and Standards. “Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, Volume I: Stationary Point and Area Sources, Section 13.2.2, Unpaved Roads.” Research Triangle Park, NC. January 2011.

- W. Barnard, G. Stensland, and D. Gatz, Illinois State Water Survey, “Evaluation of Potential Improvements in the Estimation of Unpaved Road Fugitive Emission Inventories,” paper 87-58.1, presented at the 80th Annual Meeting of the APCA. New York, New York. June 21-26, 1987

- United States Environmental Protection Agency, Clearinghouse for Inventories & Emissions Factors. “2011 National Emissions Inventory, version 2 Technical Support Document.” Research Triangle Park, NC. August 2015. Available at http://www.epa.gov/ttn/chief/net/2011inventory.html#inventorydoc. (accessed September 2015)

- U.S. Department of Transportation, Federal Highway Administration. Highway Statistics 2013. Office of Highway Policy Information. Washington, DC. 2013. Available at http://www.fhwa.dot.gov/policyinformation/statistics/2013/. (accessed September 2015)

- Federal Highway Administration, “Highway Statistics 1996, Table HM-67.” 1996. Available at: https://www.fhwa.dot.gov/ohim/1996/text/roads.html.