Industrial, Commercial, and Institutional (ICI) Fuel Combustion Tool

User’s Guide

Verision 1.6

May 2017

Developed by:

Abt Associates

5001 South Miami Blvd., Suite 210

Durham, NC 27703

1.	Getting Started	1

2.	Using the ICI Combustion Tool	4

2.1	Home Screen	4

2.2	Select States	5

2.3	Select Point Source Subtraction Method	6

2.4	Point Source ACTIVITY Data	7

2.5	Point Source EMISSIONS Data	8

2.6	Edit Assumptions	9

2.6.1	State-Level Energy Consumption	10

2.6.2	State-Level Distillate Consumption	11

2.6.3	Coal Distribution Assumptions	12

2.6.4	Distillate Stationary Source Assumptions (part 1)	13

2.6.5	Distillate Stationary Source Assumptions (part 2)	14

2.6.6	Distillate Fuel Boiler and Engine Assumptions	15

2.6.7	LPG Stationary Source Assumptions	16

2.6.8	Nonfuel Use Assumptions	17

2.6.9	Sulfur and Ash Content of Fuels	18

2.6.10	Control Efficiencies for Nonpoint SCCs	19

2.7	Document Header	20

2.8	Run the ICI Tool	22

3.	Outputs	23

# Getting Started

The ICI Combustion Tool is designed to run in Microsoft Access 2007 or later. When you first open the ICI Combustion Tool, you may see a yellow ribbon at the top of the document. Click the button that reads “Enable Content.”

Next, make sure your version of Microsoft Access is set up to run macros.

Click “File” at the top left of the screen. Then click “Options”.

On the window that opens, click “Trust Center” and then “Trust Center Settings…”

On the next window that opens, click “Macro Settings.” Then we recommend that you select “Disable all macros with notification.” Then click OK.

# Using the ICI Combustion Tool

## Home Screen

Click this button to get started with the ICI Tool.

Click this button for a table with the tool change log.

Click this button for a copy of this User’s Guide.

## Select States

On this screen, select the state(s) for which to estimate ICI nonpoint emissions.

Click this button to move to the next screen to input point source data.

## Select Point Source Subtraction Method

In this window, select the method for point source subtraction. The tool can perform point source subtraction using either activity data or emissions data. The user must supply the point source data.

Note: if you do not wish to subtract point sources, or you do not have point source data, it does not matter which method you choose.

Click one of these two buttons, depending on the method of point source subtraction you wish to use.

Click this button if you want to go back to the State Selection window.

## Point Source ACTIVITY Data

If you selected “Use Point Source ACTIVITY Data” on the previous screen, you will see the screen below.

Use this screen to export a template of point source activity data to Excel, and also to re-import that template into the tool, once it has been filled out.

Click this button to export a blank template of county-level point source activity data to Excel. Fill this template out, but you must not change the filename or the names of the column headings.

Optional: If county-level point source data are unavailable, state-level data can be used.

Click this button to return to the previous screen to change the method of point source subtraction.

The tool can perform point source subtraction using county- or state-level data. If county-level data are available, they are used first.

Once you have filled out the template of point source activity data, click this button to import the Excel template into the tool. It must be saved in the same folder as the tool.

Click this button to reset the county-level point source activity data. This will set all county-level activity data to 0.

Once all point source data have been imported into the tool, click this button to edit the assumptions used in the tool.

## Point Source EMISSIONS Data

If you selected “Use Point Source EMISSIONS Data” on the previous screen, you will see the screen below.

Use this screen to export a template of point source emissions data to Excel, and also to re-import that template into the tool, once it has been filled out.

Click this button to export a blank template of county-level point source activity data to Excel. Fill this template out, but you must not change the filename or the names of the column headings.

Optional: If county-level point source data are unavailable, state-level data can be used.

Click this button to return to the previous screen to change the method of point source subtraction.

The tool can perform point source subtraction using county- or state-level data. If county-level data are available, they are used first.

Once you have filled out the template of point source activity data, click this button to import the Excel template into the tool. It must be saved in the same folder as the tool.

Click this button to reset the county-level point source emissions data. This will set all county-level emissions data to 0.

WARNING: If you choose to use emissions for point source subtraction, you should ensure that you submit emissions data for all PM species. If, for example, you submit emissions only for PM10-FIL, but not PM25-FIL, this could lead to situations where the PM25-FIL emissions are greater than the PM10-FIL emissions, leading to errors when submitting the outputs to CDX.

Once all point source data have been imported into the tool, click this button to edit the assumptions used in the tool.

## Edit Assumptions

Click these buttons to edit the assumptions used in the tool. Each button is discussed below.

Click this button to enter the document header information for the output file.

Click this button to return to choose the point source subtraction method.

### State-Level Energy Consumption

For the selected state(s), use this screen to update the underlying assumptions about state-level energy use in the industrial and commercial sectors for all fuels except distillate (which is handled in a different window). The default values come from the EIA State Energy Data System (SEDS).

Click this button to reset the assumptions to the default values from SEDS.

### State-Level Distillate Consumption

Use this screen to update the underlying assumptions about state-level distillate fuel use in the industrial, commercial, farm, off-highway construction, and oil company sectors in the selected state(s). Distillate fuel use must be entered in units of 1,000 gallons and must be total fuel use (i.e. not just stationary source use).

Click this button to reset the assumptions to the default values from EIA distillate sales data.

### Coal Distribution Assumptions

Use this screen to input the assumptions about the percentage of coal used in the industrial and commercial sectors in the selected state(s) that is anthracite and bituminous or subbituminous.

Click this button to reset the default assumption values, which are based on EIA data on state-level coal distribution.

### Distillate Stationary Source Assumptions (part 1)

Use this screen to update assumptions about the proportion of distillate fuel used in the selected state(s) by stationary and mobile sources for the industrial and commercial sectors.

Click this button to reset the default assumption values, which are based on MOVES modeling.

### Distillate Stationary Source Assumptions (part 2)

Use this screen to update assumptions about the proportion of distillate fuel used in the selected state(s) by mobile and stationary sources for the off-highway construction, oil company, and farm sectors.

Click this button to reset the default values.

### Distillate Fuel Boiler and Engine Assumptions

The ICI tool uses different emissions factors for distillate boilers and engines. However, the EIA State Energy Data System data only provide total distillate. For this reason, the user must enter an assumption based on the percentage of distillate fuel used by boilers and engines.

Click this button to reset the assumptions to the default values. Due to a lack of data, the default value for the boiler/engine split is 50/50.

### LPG Stationary Source Assumptions

Click this button to reset the assumptions to the default values, which are based on MOVES modeling.

Use this screen to update assumptions about the proportion of LPG used by stationary and mobile sources in the selected state(s).

### Nonfuel Use Assumptions

Use this screen to update assumptions about the percentage of each fuel source in the selected state(s) that is used for feedstock purposes in industrial processes, rather than for combustion.

Click this button to reset the assumptions to the default values from EIA data.

### Sulfur and Ash Content of Fuels

Use this form to update the assumptions about the sulfur and ash content of coal and residual oil combusted in the selected state(s). These values are used to calculate the emissions factors for particulate matter and sulfur dioxide for these fuels.

Click this button to edit the county-level assumptions on distillate sulfur content. The county-level data will only be used if the checkbox is checked.

When adding county-level, make sure to add data to the columns labeled “User-Supplied”

Check this box to use county-level assumptions on distillate sulfur content.

Click this button to reset the assumptions to the default values from EIA data.

### Control Efficiencies for Nonpoint SCCs

Use this table to add assumptions about control efficiencies that will be applied to emissions from the nonpoint SCCs. The control efficiencies can be added for each SCC in each county. Control efficiencies entered into this table will be used to adjust the final reported emissions using the following equation:

Controlled Nonpoint Emissionss,c= Uncontrolled Nonpoint Emissionss,c ×(1-Control Efficiencys,c)

Where Controlled Nonpoint Emissions are the final reported nonpoint emissions, Uncontrolled Nonpoint Emissions are the emissions estimated after point source subtraction but before the application of the control efficiency, Control Efficiency is the user-supplied control efficiency, s is SCC, and c is county.

Note that the control efficiency must be a number between 0 and 1. The default control efficiencies in the tool are 0 for all counties and SCCs.

Add county- and SCC-specific control efficiencies here. Control efficiencies must be between 0 and 1.

## Document Header

The ICI Combustion Tool will output the calculated inventory in EIS format into a separate Microsoft Access database called “ICI Tool Output - EIS Format.mdb”. Use this form to enter the document header information for the staging tables. See the table on the next page for definitions of the fields in the document header table.

Once the document header information has been entered, click this button to run the ICI Combustion Tool.

| Element | Required for Schema Validation | Data Type in EIS | Definition |
| --- | --- | --- | --- |
| Author Name | Yes | Character (20) | Your name, not your user ID |
| Organization Name | Yes | Character (20) | The name of the organization which you are representing |
| Document Title | Yes | Character (3) | Must always be "EIS" |
| Keywords | No | Character (100) | Words that best describe the payload. Multiple keywords should be separated by commas. This is for transaction categorization and searching. |
| Comment | No | Character (400) | Additional comments for processors |
| Data Flow Name | Yes | Character (20) | Must always be "EIS_v1_0 |
| Property-Submission Type | Yes | Character (20) | Either "QA" or "Production" |
| Property-Data Category | Yes | Character (20) | Either "FacilityInventory", "Point", "Nonpoint", "Onroad", "Nonroad", or "Event" |
| NCD Data File | (Yes) | Character (20) | The name of the NCD zipped file which is being attached. Only required when reporting onroad/nonroad activity input data. |
| User Identifier | Yes | Character (255) | User ID recognized by EIS. Usually your email address |
| Program System Code | Yes | Character (20) | The code that represents the information management system which has responsibility for the data in a linked or interrelated information management system. |
| Emissions Year | Yes | Year | The year of the submitted emissions. |
| Model | No | Character (80) | The name of the model or the conversion tool used for generating the emissions data. |
| Model Version | No | Character (20) | The version of the model or conversion tool. |
| Emissions Creation Date | No | Date YYYY-MM-DD | Date that the data being submitted were created, or the date when the model generating the data was run. |
| Submittal Comment | No | Character (400) | Any comments regarding the file submission. |

## Run the ICI Tool

Click “Run ICI Combustion Tool” to run the tool.

You will see this window while the tool is running.

# Outputs

The output of the ICI Combustion Tool is the calculated inventory for the selected state(s). When the tool finishes running, the calculated inventory will open in the tool.

The calculated inventory is also automatically exported to a separate database file called “ICI Tool Output - EIS Format.mdb”. This database file, which contains the populated EIS Staging Tables, will be located in the same folder as the ICI Combustion Tool.