Industrial, Commercial, and Institutional Fuel Combustion:

Documentation for EPA’s Nonpoint Emissions Estimation Tool

ICI Tool Version 1.6

May 2017

Prepared for:

U.S. Environmental Protection Agency

Emissions Inventory and Analysis Group

Office of Air Quality Planning and Standards

Research Triangle Park, NC, USA

Submitted by:

Abt Associates

5001 South Miami Blvd., Suite 210

Durham, NC, USA

Table of Contents

1. Introduction	1

2. Source Classification Codes and Pollutants Included in ICI Tool	2

3. Overview of Calculations	4

4. Activity Data Adjustments	5

4.1 Fuel Specific Data Adjustments	5

4.2 Non-fuel Use Energy Adjustment	7

4.3 Point Source Energy Adjustment	7

5. County Allocation of State Activity	9

6. Emission Factors	11

7. Running the ICI Tool	12

8. References	15

Appendix A. SCC Crosswalks and Emission Factors	17

Figures

Figure 1. Snapshot of state selection interface	12

Figure 2. Snapshot of point source activity data import interface	12

Figure 3. Snapshot of point source emissions data import interface	13

Figure 4. Snapshot of assumptions interface	13

Figure 5. Snapshot of Document Header interface	14

Tables

Table 1. Source Categories Included in ICI Tool	2

Table 2. Pollutants Included in ICI Tool	3

Table 3. Stationary Source Adjustments for Industrial Sector Distillate Fuel Consumption	5

Table 4. Stationary Source Adjustments for Commercial Sector Distillate Fuel Consumption	6

Table 5. Default assumptions for distillate boiler/engine split	7

Table 6. Industrial Sector Non-Fuel Use Estimates	7

Table 7. NAICS Code 31-33 (Manufacturing) Employment Data for Maine	10

Table A-1. Industrial Fuel Combustion Crosswalk for Point Source Subtractions	17

Table A-2. Commercial/Institutional Fuel Combustion Crosswalk for Point Source Subtractions	32

Table A-3. Criteria Pollutant Emission Factors for ICI Area Source Categories	37

Table A-4. Speciation factors for HAP VOCs. These factors are multiplied by VOC emissions to estimate emissions for each HAP.	38

Table A-5. Speciation factors for non-VOC HAPs. Factors are multiplied by PM25-PRI emissions to estimate emissions for each HAP.	48

1. Introduction

Emissions from Industrial, Commercial, and Institutional (ICI) fuel combustion are often a significant portion of most areas’ total emissions inventory. Unless all ICI combustion emission sources are covered in a geographic area’s point source inventory, it is necessary for inventory preparers to estimate ICI combustion nonpoint source emissions. Because there are specific challenges associated with estimating ICI nonpoint source emissions, the U.S. EPA in conjunction with Abt Associates developed a Microsoft® Access-based ICI Combustion Tool to assist State, Local, and Tribal agencies in estimating nonpoint emissions from ICI fuel combustion for the 2014 National Emission Inventory.

The primary data source behind the ICI Combustion Tool is total state-level ICI energy consumption data released annually as part of the Energy Information Administration’s State Energy Data System (SEDS).  The ICI Combustion Tool processes the SEDS data and adjusts the data to account for the fraction of fuel consumed by nonroad mobile sources whose emissions are included in the nonroad inventory and by non-fuel combustion uses of energy, such as product feedstocks. Through a user-friendly interface, users can update the underlying assumptions in the adjustment methodology. The ICI Combustion Tool also includes a nonpoint source to point source crosswalk and allows the user to perform point source activity subtractions to avoid double counting of emissions between their point and nonpoint inventories. The ICI Combustion Tool generates outputs in EPA’s Emissions Inventory System (EIS) Staging Table format, ready for conversion to XML and submission to the EIS. This documentation provides an overview of the ICI Combustion Tool and the procedures for developing a credible estimate of nonpoint emissions from ICI fuel-combustion sources.

2. Source Classification Codes and Pollutants Included in ICI Tool

Table 1 displays the source categories included in the ICI Combustion Tool and Table 2 displays the pollutant types covered.

Table 1. Source Categories Included in ICI Tool

| SCC | Description |
| --- | --- |
| Industrial |  |
| 2102001000 | Anthracite Coal /Total: All Boiler Types |
| 2102002000 | Bituminous/Subbituminous Coal /Total: All Boiler Types |
| 2102004001 | Distillate Oil /Boilers |
| 2102004002 | Distillate Oil /IC Engines |
| 2102005000 | Residual Oil /Total: All Boiler Types |
| 2102006000 | Natural Gas /Total: Boilers and IC Engines |
| 2102007000 | Liquified Petroleum Gas /Total: All Boiler Types |
| 2102008000 | Wood /Total: All Boiler Types |
| 2102011000 | Kerosene /Total: All Boiler Types |
| Commercial/Institutional |  |
| 2103001000 | Anthracite Coal /Total: All Boiler Types |
| 2103002000 | Bituminous/Subbituminous Coal /Total: All Boiler Types |
| 2103004001 | Distillate Oil /Boilers |
| 2103004002 | Distillate Oil /IC Engines |
| 2103005000 | Residual Oil /Total: All Boiler Types |
| 2103006000 | Natural Gas /Total: Boilers and IC Engines |
| 2103007000 | Liquified Petroleum Gas /Total: All Combustor Types |
| 2103008000 | Wood /Total: All Boiler Types |
| 2103011000 | Kerosene /Total: All Combustor Types |

Table 2. Pollutants Included in ICI Tool

| Category | Pollutant(s) |
| --- | --- |
| Criteria | Carbon monoxide |
|  | Nitrogen oxides |
|  | PM10 |
|  | PM2.5 |
|  | Sulfur dioxide |
|  | Volatile organic compounds |
| Hazardous | Polycyclic organic matter |
|  | Metals |
|  | Dioxins/furans |
|  | Other HAPs emitted from fuel combustion |
| PM Precursor | Ammonia |

3. Overview of Calculations

ICI combustion nonpoint source emissions are calculated using Equation 1.  The sector reference represents Industrial or Commercial/Institutional, and fuel types include: coal, natural gas, distillate oil, residual oil, liquefied petroleum gas, kerosene and wood.

Es,f=As,f×Fs,f	(1)

where E is emissions, A	 is activity data (fuel combusted), F is the emission factor, s is sector, and f is fuel type.

The key emissions activity data inputs in the emissions estimation methodology are:

- Total Industrial and total Commercial/Institutional energy consumption by fuel type and state for a given year;

- Industrial energy consumed for non-fuel purposes by fuel type and state in that year;

- ICI distillate oil and liquefied petroleum gas (LPG) consumption by state from nonroad mobile sources for the year of interest;

- ICI energy consumption by sector, state, and fuel type for point sources for the given year; and

- County-level employment by ICI sector and state for the year of interest.

The ICI Tool also relies on emission factors relating emission rates to the volume of fuel burned by sector/fuel type, and the sulfur content of coal consumed in each sector by state for the given year.

ICI combustion emissions are directly related to the sector, type, and volume of fuel burned. The EIA is responsible for developing official federal government estimates of energy consumption.  The EIA estimates annual energy consumption at the state-level as part of the State Energy Data System (SEDS) (EIA, 2015a).  The SEDS reports energy consumption estimates by state, sector, fuel type, and year.  The SEDS provides data for each of five consuming sectors, including Industrial and Commercial (note that the SEDS’ definition of “Commercial” includes Institutional sector use).  The EIA also publishes additional detailed estimates of state-level fuel oil and kerosene consumption estimates in their Fuel Oil and Kerosene Sales publication (EIA, 2013a).  This publication provides state-level annual end use sales of No.1, No. 2, and No. 4 distillate fuel oil for commercial, industrial, oil company, farm, off-highway construction, and other uses – these data are used to differentiate stationary from mobile source distillate fuel consumption.

4. Activity Data Adjustments

## 4.1 Fuel Specific Data Adjustments

Coal – For coal combustion, it is necessary to compile data representing a subset of total sector coal consumption. Data representing non-coke plant consumption are compiled from EIA because coal consumed by coke plants is accounted for in the point source inventory. The SEDS data do not provide coal consumption estimates by type of coal (i.e., anthracite versus bituminous/subbituminous). Therefore, state-level ICI coal distribution data for 2013 from the EIA’s Annual Coal Distribution Report 2013 are used to allocate coal consumption between the two types of coal (EIA, 2015b). The 2013 ratio of anthracite coal consumption to total coal consumption is used for this allocation procedure.

Distillate Oil and LPG – The SEDS ICI distillate oil and LPG consumption data include consumption estimates for equipment that are typically included in the nonroad sector inventory. In particular, SEDS considers the following nonroad source category activities to be part of the industrial sector: farming, logging, mining, and construction.

In order to avoid double-counting of distillate oil consumption between the nonpoint and nonroad sector emission inventories, the more detailed distillate oil consumption estimates reported in EIA’s Fuel Oil and Kerosene Sales are combined with assumptions used in the regulatory impact analysis (RIA) for EPA’s nonroad diesel emissions rulemaking (EIA, 2015b; EPA, 2003).

For distillate fuel, Table 3 presents the assumptions that are applied to the state-level Commercial sector distillate oil consumption data published in Fuel Oil and Kerosene Sales to estimate Commercial sector stationary source consumption.

Table 3. Stationary Source Adjustments for Industrial Sector Distillate Fuel Consumption

| Sector | Distillate Fuel Type | % of Total Consumption from Stationary Sources |
| --- | --- | --- |
| Industrial | No. 1 Distillate Fuel Oil | 60 |
|  | No. 2 Distillate Fuel Oil | 100 |
|  | No. 2 Distillate/Low and High Sulfur Diesel | 15a |
|  | No. 4 Distillate Fuel Oil | 100 |
| Farm | Diesel | 0 |
|  | Other Distillate Fuel Oil | 100 |
| Off-Highway (Construction and Other) | Distillate Fuel Oil | 5 |
| Oil Company | Distillate Fuel Oil | 50 |

a This value differs from the 0% assumption adopted in EPA’s nonroad diesel emissions rulemaking because it is known that some diesel fuel is used by stationary sources (a 15 percent value was selected for use as an approximate mid-point of a potential range of 8 to 24% stationary source use computed from a review of data from the EIA’s Manufacturing Energy Consumption Survey and Fuel Oil and Kerosene Sales).

Table 4 presents the assumptions that are applied to the state-level Commercial sector distillate oil consumption data published in Fuel Oil and Kerosene Sales to estimate Commercial sector stationary source consumption.

Table 4. Stationary Source Adjustments for Commercial Sector Distillate Fuel Consumption

| Sector | Distillate Fuel Type | % of Total Consumption from Stationary Sources |
| --- | --- | --- |
| Commercial | No. 1 Distillate Fuel Oil | 80 |
|  | No. 2 Distillate Fuel Oil | 100 |
|  | No. 2 Distillate/Ultra-Low, Low, and High Sulfur Diesel | 0a |
|  | No. 4 Distillate Fuel Oil | 100 |

a A very small portion of total commercial/institutional diesel is consumed by point sources (SCC 203001xx).

In order to avoid double-counting of LPG consumption, the ICI Tool uses data from the EPA National Mobile Inventory Model (NMIM) for 2006 to calculate the national volume of nonroad LPG consumption from agriculture, logging, mining, and construction source categories. This estimate is then divided into the SEDS total LPG consumption estimate to yield the proportion of total ICI LPG consumption attributable to the nonroad sector in that year (8.72% for industrial sources and 17.72% for commercial/institutional sources). It is assumed that these proportions are appropriate for future inventory years. This estimate of the nonroad portion of LPG consumption is subtracted from each state’s ICI LPG consumption estimate reported in SEDS.

Distillate oil is reported by EIA as the total consumption of distillate. Therefore, assumptions must be made to determine the amount of distillate consumed by boilers and internal combustion engines (Table 5). The default assumptions were calculated using data from the EIA, but S/L/T agencies are encouraged to update the default assumptions with better state-level data, if available.

The default boiler/engine split assumptions for industrial distillate consumption were calculated using data from EIA’s 2010 Manufacturing Energy Consumption Survey (MECS), Table 5.5 (EIA, 2013b), which provides data on distillate consumption by end use for the industrial sector. The boiler/engine split was calculated at the national level, because data was withheld for too many end uses at the regional level. The following end uses from MECS are assumed to be associated with engines: electricity generation (which assumes the electricity is generated using internal combustion engine generators) and machine drive (which includes use by motors, pumps, etc.). All other end uses are assumed to be associated with boilers. The total national-level distillate consumption for engine-based and boiler-based end uses is 6 million barrels and 9 million barrels, respectively. Therefore, we assume that the boiler/engine split for industrial distillate is 60% boilers and 40% engines.

The default boiler/engine split assumptions for commercial distillate consumption were calculated using data from EIA’s Commercial Building Energy Consumption Survey (CBECS), Table E9 (EIA, 2013c), which provides data on distillate consumption by end use for the commercial sector. It is assumed that space heating and water heating are associated with boilers (211 trillion Btu) and “other” is associated with engines (10 trillion Btu). The result is a default boiler/engine split for commercial distillate of 95% boilers and 5% engines. Note that this approach may overestimate the number of engines in the commercial sector, since the “other” end use category could also include boilers. Nevertheless, the data show that the vast majority of distillate consumption in the commercial sector is for space and water heating.

Table 5. Default assumptions for distillate boiler/engine split

|  | Industrial | Commercial |
| --- | --- | --- |
| Boiler | 60% | 95% |
| Engine | 40% | 5% |

## 4.2 Non-fuel Use Energy Adjustment

Some industrial sector energy is consumed for non-fuel purposes, such as natural gas that is used as a feedstock in chemical manufacturing plants and to make nitrogenous fertilizer, and LPG that is used to create intermediate products that are ultimately made into plastics. In order to estimate the volume of fuel that is associated with industrial combustion, it is necessary to subtract the volume of fuel consumption for non-energy uses from the volume of total fuel consumption.

The identification of feedstock usage was initially based upon the non-fuel use assumptions incorporated into the EIA’s GHG emissions inventory for 2005 (EIA, 2007). The following fuels are assumed to be used entirely for non-fuel purposes: asphalt and road oil, feedstocks (naphtha <401 °F), feedstocks (other oils >401 °F), lubricants, miscellaneous petroleum products, pentanes plus, special naphthas, and waxes. In addition, it is also assumed that kerosene and motor gasoline are used entirely as fuel without any non-fuel purposes. The remaining fuels (i.e., coal [non-coke], distillate oil, LPG, natural gas, and residual oil) are used both for fuel and non-fuel purposes. The regional non-fuel fractions for distillate oil, LPG, natural gas, non-coke coal and residual oil are derived from non-fuel (feedstock) and total energy use statistics contained in EIA’s 2010 Manufacturing Energy Consumption Survey (MECS) (EIA, 2013b) and are presented in Table 6.

Table 6. Industrial Sector Non-Fuel Use Estimates

| Fuel | % of Total Energy Consumption from Non-Fuel Use |  |  |  | Source |
| --- | --- | --- | --- | --- | --- |
|  | Northeast | Midwest | South | West |  |
| Non-Coke Coal | 63 | 38 | 26 | 4b | 2010 MECS |
| Natural Gas | 1 | 5 | 14 | 2 | 2010 MECS |
| LPG | 33 | 88 | 99 | 6b | 2010 MECS |
| Distillate Oil | 4a | 4a | 4a | 4a | 2010 MECS |
| Residual Oil | 5b | 50 | 68 | 20b | 2010 MECS |

a Nonfuel use of distillate fuel oil was not reported at the regional level therefore, the default nonfuel use fractions are based on national nonfuel use of distillate fuel oil. 
b Nonfuel use was reported in EIA data as "less than 0.5". In these cases a value of 0.25 was used to estimate the default nonfuel use fractions.

## 4.3 Point Source Energy Adjustment

To ensure that fuel consumption is not double-counted in the point source inventory, it is also necessary to subtract point source inventory fuel use from the fuel consumption estimates developed from the above steps.  Equation 2 illustrates the approach to performing point source subtractions.

Ns,f=Ts,f-Ps,f	(2)

where N is nonpoint fuel consumption, T is total fuel consumption, P is point source fuel consumption, s is sector, and f is fuel type.

The first step in the point source subtraction procedure is to identify how each ICI combustion nonpoint source classification code (SCC) links to associated ICI combustion point SCCs.  The ICI Combustion Tool includes two such crosswalks:  one between each Industrial fuel combustion nonpoint SCC and related point SCCs, and an analogous crosswalk developed for Commercial/Institutional fuel combustion SCCs.  One issue to note is that natural gas consumed as pipeline fuel is not included by the SEDS within the Industrial sector.  Therefore, it is necessary to exclude pipeline natural gas consumption in performing natural gas combustion subtraction.  This consumption may be included within industrial sector natural gas internal combustion engine records (SCC 202002xx).

An issue that must be considered is the geographic resolution at which point source subtractions should be performed.  While the locations of point sources is accurately known at (and below) the county-level, total ICI combustion activity is much less clear.  Because of the level of uncertainty associated with the county distribution of total ICI fuel consumption, states may wish to perform the ICI combustion point source subtractions at the state-level, and then allocate the resulting nonpoint source fuel consumption to counties. On the contrary, if states have more accurate county-level fuel consumption values then point source subtraction can be performed at the county-level. The ICI Tool is designed to prioritize county-level data over state-level data, so where county-level data exists, the ICI Tool will perform county-level subtractions before using state-level data.

If an agency does not have county- or state-level point source activity data, emissions data can be used in the place of activity data in the point source subtraction procedure. The procedure follows the same steps, except that the emissions are calculated first, and then the point source activity data are subtracted from the total emissions.

5. County Allocation of State Activity

Because the EIA only reports energy consumption down to the state-level, it is necessary to develop a procedure to allocate EIA’s fuel consumption estimates (after adjustments noted in sections above) to counties.  For the NEI, the procedure relies on the use of allocation factors developed from the county-level number of employees in the Industrial sector and the county number of employees in the Commercial/Institutional sector.  Because EIA fuel consumption data originate from fuel sector-specific surveys of energy suppliers, we reviewed these survey forms/instructions for further details on what individual economic sectors EIA considers to comprise the Industrial and Commercial sector.  Based on this review, we compiled employment data for manufacturing sector North American Industrial Classification System (NAICS) codes (i.e., NAICS 31-33) for use in allocating Industrial fuel combustion.  The only source of NAICS-code based EIA definitions of the Commercial energy sector is a “rough crosswalk” between Commercial building types and NAICS codes developed for EIA’s CBECS (EIA, 2013c).  With the exception of NAICS code 814 (Private Households), this crosswalk links all NAICS codes between 42 and 92 with Commercial building energy consumption.

The ICI Combustion Tool compiles employment data for these NAICS codes from two Bureau of the Census publications  ̶  County Business Patterns (for private sectors) and Census of Governments (for public administration sectors) (Bureau of the Census, 2015a; Bureau of the Census, 2015b).  For NAICS code 92, county-level employment is estimated from local government employment data in the Census of Governments.  Employment estimates from each source are then combined to estimate total Commercial/ Institutional sector employment by county.  The state-level fuel combustion by fuel type estimates in each sector are then allocated to each county using the ratio of the number of Industrial or Commercial/Institutional employees in each county in a given state.

Due to concerns with releasing confidential business information, County Business Patterns (CBP) withholds values for a given county/NAICS code if it would be possible to identify data for individual facilities. In such cases, the Census reports a letter code, representing a particular employment size range. We used the following procedure to estimate data for withheld counties/NAICS codes.

- County-level employment for counties with reported values are totaled by state for the applicable NAICS code.

- The value from step 1 is subtracted from the state employment value for the NAICS code.

- Each of the withheld counties is assigned an initial employment estimate reflecting the midpoint of the CBP range code (e.g., code A, which reflects 1-19 employees, is assigned an estimate of 10 employees).

- The initial employment estimates from step 3 are then summed to the state level.

- The value from step 2 is divided by the value from step 4 to yield an adjustment factor to apply to the initial employment estimates to yield employment values that will sum to the state employment total for the applicable NAICS code.

- The final county-level employment values are estimated by multiplying the initial employment estimates from step 3 by the step 5 adjustment factors.

Table 7 illustrates the employment estimation procedure with an example of CBP data reported for Maine.

Table 7. NAICS Code 31-33 (Manufacturing) Employment Data for Maine

| FIPSSTATE | FIPSCTY | NAICS | EMPFLAG | EMP |
| --- | --- | --- | --- | --- |
| 23 | 001 | 31---- |  | 6,774 |
| 23 | 003 | 31---- |  | 3,124 |
| 23 | 005 | 31---- |  | 10,333 |
| 23 | 007 | 31---- |  | 1,786 |
| 23 | 009 | 31---- |  | 1,954 |
| 23 | 011 | 31---- |  | 2,535 |
| 23 | 013 | 31---- |  | 1,418 |
| 23 | 015 | 31---- | F | 0 |
| 23 | 017 | 31---- |  | 2,888 |
| 23 | 019 | 31---- |  | 4,522 |
| 23 | 021 | 31---- |  | 948 |
| 23 | 023 | 31---- | I | 0 |
| 23 | 025 | 31---- |  | 4,322 |
| 23 | 027 | 31---- |  | 1,434 |
| 23 | 029 | 31---- |  | 1,014 |
| 23 | 031 | 31---- |  | 9,749 |

- The total of employees not including counties 015 and 023 is 52,801.

- County Business Patterns reports 59,322 state employees in NAICS 31—the difference is 6,521.

- County 015 is given a midpoint of 1,750 (since range code F is 1,000-2,499) and County 023 is given a midpoint of 17,500.

- State total for these two counties is 19,250.

- 6,521/19,250 = 0.33875.

The final employment estimate for county 015 is 1,750 x 0.33875 = 593. The county 023 final employment estimate is computed as 17,500 x 0.33875 = 5,928.

6. Emission Factors

Table A-3 lists the criteria pollutant emissions factors used in the ICI Combustion Tool. The criteria and hazardous air pollutant emission factors for each nonpoint source fuel combustion category included in the ICI Combustion Tool are primarily EPA emission factors. The majority of the emission factors are from the EPA/ERTAC2 database and EPA’s AP-42 report, Compilation of Air Pollutant Emission Factors (Huntley, 2009; EPA, 2010). The ammonia emission factors for wood combustion are from an Emission Inventory Improvement Program (EIIP) guidance document (EPA, 2004).

For coal combustion, the SO2 emission factors are based on the sulfur content of the coal burned, and some of the PM emission factors for anthracite coal require information on the ash content of the coal. For the industrial and commercial/institutional sectors, state-specific coal sulfur contents for bituminous coal are obtained from the EIA’s quarterly coal report (EIA, 2012). For anthracite coal, an ash content value of 13.38% and a sulfur content of 0.89% are applied to all states.

In the ICI Tool, users may edit the assumptions about the sulfur and ash content of fuels, using the form “Sulfur and Ash Content of Fuels” from the “Edit Assumptions” form. Assumptions about sulfur content can be adjusted at the state level for bituminous/subbituminous coal, anthracite coal, residual oil, and distillate oil. Sulfur content assumptions can also be adjusted at the county level for distillate oil. Assumptions about ash content can be adjusted at the state level for anthracite coal.

In addition, users may also add user defined control efficiencies for each SCC in each county, using the “Control Efficiencies for Nonpoint SCCs” table, which can be accessed from the “Edit Assumptions” form. Control efficiencies entered into this table will be used to adjust the final reported emissions using the following equation:

Controlled Nonpoint Emissionss,c= Uncontrolled Nonpoint Emissionss,c ×(1-Control Efficiencys,c)

Where Controlled Nonpoint Emissions are the final reported nonpoint emissions, Uncontrolled Nonpoint Emissions are the emissions estimated after point source subtraction but before the application of the control efficiency, Control Efficiency is the user-supplied control efficiency, s is SCC, and c is county.

Note that the control efficiency must be a number between 0 and 1. The default control efficiencies in the tool are 0 for all counties and SCCs.

7. Running the ICI Tool

The ICI Combustion Tool includes a user-friendly graphical interface that walks the user through the development of their nonpoint ICI combustion emissions inventory. The ICI Combustion Tool allows the user to select the state(s) of interest, import state- or county-level point source inventory data, modify the default assumptions used in the Tool, and export their final nonpoint ICI emissions inventory in EIS staging table format. Figures 1 through 5 below show snapshots of the user interface.

Figure 1. Snapshot of state selection interface

Figure 2. Snapshot of point source activity data import interface

Figure 3. Snapshot of point source emissions data import interface

Figure 4. Snapshot of assumptions interface

Figure 5. Snapshot of Document Header interface

8. References

Bureau of the Census, 2015a: County Business Patterns 2013, U.S. Department of Commerce, Washington DC, accessed May 2019.

Bureau of the Census, 2015b: “Local Government Employment and Payroll, March 2012,” 2012 Census of Governments, U.S. Department of Commerce, Washington DC, accessed May 2019.

EIA, 2015a: Energy Information Administration, “State Energy Data System, Consumption Estimates, 2013” U.S. Department of Energy, Washington DC, annual data released July 24, 2015.

EIA, 2015b: Energy Information Administration, “Annual Coal Distribution Archive, Domestic Distribution of U.S. Coal by Destination State, Consumer, Destination and Method of Transportation” U.S. Department of Energy, Washington DC, 2013 data file release date April 16, 2015.

EIA, 2013a: Energy Information Administration, Fuel Oil and Kerosene Sales, 2013, U.S. Department of Energy, Washington DC, accessed from http://www.eia.gov/dnav/pet/pet_cons_821dst_dcu_nus_a.htm

EIA, 2013b. Energy Information Administration, “2010 MECS Survey Data”. U.S. Department of Energy, Energy Information Administration, data files  release date 2013.

EIA, 2013c: Energy Information Administration, “Appendix Table A-51. EIA's Commercial Sector: Building Activities and NAICS Industries,” Commercial Building Energy Consumption Survey, U.S. Department of Energy, Washington DC, available from http://www.eia.gov/consumption/commercial/index.cfm, accessed July 2013.

EIA, 2012. Quarterly Coal Report, January – March 2012. U.S. Department of Energy, Energy Information Administration.

EIA, 2007: Energy Information Administration, Documentation for Emissions of Greenhouse Gases in the United States 2005, U.S. Department of Energy, Washington, DC, October 2007; DOE/EIA-0638 (2005).

EPA, 2010. U.S. Environmental Protection Agency, AP-42, Compilation of Air Pollutant Emission Factors, Volume 1, Stationary Point and Area Sources, accessed May 2019.

EPA, 2004. Estimating Ammonia Emissions from Anthropogenic Sources. Draft Final Report. Emission Inventory Improvement Program, U.S. Environmental Protection Agency, Research Triangle Park, North Carolina, April 2004.

EPA, 2003: Draft Regulatory Impact Analysis: Control of Emissions from Nonroad Diesel Engines. EPA 420-R-03-008. U.S. Environmental Protection Agency, Office of Transportation and Air Quality, April.

Huntley, R., 2009. U.S. Environmental Protection Agency, Eastern Regional Technical Advisory Committee (ERTAC), Excel file: state_comparison_ERTAC_SS_version7.2_23nov2009.xls

Appendix A. SCC Crosswalks and Emission Factors

Table A-1. Industrial Fuel Combustion Crosswalk for Point Source Subtractions

| Point SCC | SCC1 DESC | SCC3 DESC | SCC6 DESC | SCC8 DESC |
| --- | --- | --- | --- | --- |
| 2102001000 - Stationary Source Fuel Combustion; Industrial; Anthracite Coal; Total: All Boiler Types |  |  |  |  |
| 10200101 | External Combustion Boilers | Industrial | Anthracite Coal | Pulverized Coal |
| 10200104 | External Combustion Boilers | Industrial | Anthracite Coal | Traveling Grate (Overfeed) Stoker |
| 10200107 | External Combustion Boilers | Industrial | Anthracite Coal | Hand-fired |
| 10200117 | External Combustion Boilers | Industrial | Anthracite Coal | Fluidized Bed Boiler Burning Anthracite-Culm Fuel |
| 39000199 | Industrial Processes | In-process Fuel Use | Anthracite Coal | General |
| 2102002000 - Stationary Source Fuel Combustion; Industrial; Bituminous/Subbituminous Coal; Total: All Boiler Types |  |  |  |  |
| 10200201 | External Combustion Boilers | Industrial | Bituminous Coal | Pulverized Coal: Wet Bottom |
| 10200202 | External Combustion Boilers | Industrial | Bituminous Coal | Pulverized Coal: Dry Bottom |
| 10200203 | External Combustion Boilers | Industrial | Bituminous Coal | Cyclone Furnace |
| 10200204 | External Combustion Boilers | Industrial | Bituminous Coal | Spreader Stoker |
| 10200205 | External Combustion Boilers | Industrial | Bituminous Coal | Overfeed Stoker |
| 10200206 | External Combustion Boilers | Industrial | Bituminous Coal | Underfeed Stoker |
| 10200212 | External Combustion Boilers | Industrial | Bituminous Coal | Pulverized Coal: Dry Bottom (Tangential) |
| 10200213 | External Combustion Boilers | Industrial | Bituminous Coal | Wet Slurry |
| 10200217 | External Combustion Boilers | Industrial | Bituminous Coal | Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| 10200218 | External Combustion Boilers | Industrial | Bituminous Coal | Atmospheric Fluidized Bed Combustion: Circulating Bed |
| 10200219 | External Combustion Boilers | Industrial | Bituminous Coal | Cogeneration |
| 10200221 | External Combustion Boilers | Industrial | Subbituminous Coal | Pulverized Coal: Wet Bottom |
| 10200222 | External Combustion Boilers | Industrial | Subbituminous Coal | Pulverized Coal: Dry Bottom |
| 10200223 | External Combustion Boilers | Industrial | Subbituminous Coal | Cyclone Furnace |
| 10200224 | External Combustion Boilers | Industrial | Subbituminous Coal | Spreader Stoker |
| 10200225 | External Combustion Boilers | Industrial | Subbituminous Coal | Traveling Grate (Overfeed) Stoker |
| 10200226 | External Combustion Boilers | Industrial | Subbituminous Coal | Pulverized Coal: Dry Bottom Tangential |
| 10200229 | External Combustion Boilers | Industrial | Subbituminous Coal | Cogeneration |
| 39000201 | Industrial Processes | In-process Fuel Use | Bituminous Coal | Cement Kiln/Dryer (Bituminous Coal) |
| 39000203 | Industrial Processes | In-process Fuel Use | Bituminous Coal | Lime Kiln (Bituminous) |
| 39000288 | Industrial Processes | In-process Fuel Use | Bituminous Coal | General (Subbituminous) |
| 39000289 | Industrial Processes | In-process Fuel Use | Bituminous Coal | General (Bituminous) |
| 50390002 | Waste Disposal | Solid Waste Disposal - Industrial | Auxillary Fuel/No Emissions | Coal |
| 2102004001 - Stationary Source Fuel Combustion; Industrial; Distillate Oil; Boilers |  |  |  |  |
| 10200501 | External Combustion Boilers | Industrial | Distillate Oil | Grades 1 and 2 Oil |
| 10200502 | External Combustion Boilers | Industrial | Distillate Oil | 10-100 Million BTU/hr ** |
| 10200503 | External Combustion Boilers | Industrial | Distillate Oil | < 10 Million BTU/hr ** |
| 10200504 | External Combustion Boilers | Industrial | Distillate Oil | Grade 4 Oil |
| 10200505 | External Combustion Boilers | Industrial | Distillate Oil | Cogeneration |
| 10201403 | External Combustion Boilers | Industrial | CO Boiler | Distillate Oil |
| 10500105 | External Combustion | Space Heaters | Industrial | Distillate Oil |
| 30190001 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30190011 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 30190021 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Distillate Oil (No. 2): Flares |
| 30290001 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30390001 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30390011 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 30390021 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Flares |
| 30400406 | Industrial Processes | Secondary Metal Production | Lead | Pot Furnace Heater: Distillate Oil |
| 30490001 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30490011 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 30490021 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Flares |
| 30490031 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Distillate Oil (No. 2): Furnaces |
| 30500208 | Industrial Processes | Mineral Products | Asphalt Concrete | Asphalt Heater: Distillate Oil |
| 30505022 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) | Asphalt Heater: Distillate Oil |
| 30590001 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30590011 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 30590021 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Distillate Oil (No. 2): Flares |
| 30600901 | Industrial Processes | Petroleum Industry | Flares | Distillate Oil |
| 30609901 | Industrial Processes | Petroleum Industry | Incinerators | Distillate Oil (No. 2) |
| 30890001 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30890011 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 30890021 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Distillate Oil (No. 2): Flares |
| 30990001 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Distillate Oil (No. 2): Process Heaters |
| 30990011 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 31390001 | Industrial Processes | Electrical Equipment | Process Heaters | Distillate Oil (No. 2) |
| 39000501 | Industrial Processes | In-process Fuel Use | Distillate Oil | Asphalt Dryer ** |
| 39000502 | Industrial Processes | In-process Fuel Use | Distillate Oil | Cement Kiln/Dryer |
| 39000503 | Industrial Processes | In-process Fuel Use | Distillate Oil | Lime Kiln |
| 39000598 | Industrial Processes | In-process Fuel Use | Distillate Oil | Grade 4 Oil: General |
| 39000599 | Industrial Processes | In-process Fuel Use | Distillate Oil | General |
| 39900501 | Industrial Processes | Miscellaneous Manufacturing Industries | Process Heater/Furnace | Distillate Oil |
| 39990001 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Distillate Oil (No. 2): Process Heaters |
| 39990011 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Distillate Oil (No. 2): Incinerators |
| 39990021 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Distillate Oil (No. 2 Oil): Flares |
| 40201002 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater | Distillate Oil |
| 40290011 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment | Distillate Oil: Incinerator/Afterburner |
| 49090011 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment | Distillate Oil (No. 2): Incinerators |
| 49090021 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment | Distillate Oil (No. 2): Flares |
| 50390005 | Waste Disposal | Solid Waste Disposal - Industrial | Auxiliary Fuel/No Emissions | Distillate Oil |
| 2102004002 - Stationary Source Fuel Combustion; Industrial; Distillate Oil; IC Engines |  |  |  |  |
| 20200101 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Turbine |
| 20200102 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Reciprocating |
| 20200103 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Turbine: Cogeneration |
| 20200104 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Reciprocating: Cogeneration |
| 20200105 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Reciprocating: Crankcase Blowby |
| 20200106 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20200107 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Reciprocating: Exhaust |
| 20200108 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20200109 | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) | Turbine: Exhaust |
| 20200401 | Internal Combustion Engines | Industrial | Large Bore Engine | Diesel |
| 20200405 | Internal Combustion Engines | Industrial | Large Bore Engine | Crankcase Blowby |
| 20200406 | Internal Combustion Engines | Industrial | Large Bore Engine | Evaporative Losses (Fuel Storage and Delivery System) |
| 20200407 | Internal Combustion Engines | Industrial | Large Bore Engine | Exhaust |
| 27000320 | Internal Combustion Engines | Off-highway Diesel Engines | Industrial Equipment | Industrial Fork Lift: Diesel |
| 2102005000 - Stationary Source Fuel Combustion; Industrial; Residual Oil; Total: All Boiler Types |  |  |  |  |
| 10200401 | External Combustion Boilers | Industrial | Residual Oil | Grade 6 Oil |
| 10200402 | External Combustion Boilers | Industrial | Residual Oil | 10-100 Million BTU/hr ** |
| 10200403 | External Combustion Boilers | Industrial | Residual Oil | < 10 Million BTU/hr ** |
| 10200404 | External Combustion Boilers | Industrial | Residual Oil | Grade 5 Oil |
| 10200405 | External Combustion Boilers | Industrial | Residual Oil | Cogeneration |
| 10201404 | External Combustion Boilers | Industrial | CO Boiler | Residual Oil |
| 20200501 | Internal Combustion Engines | Industrial | Residual/Crude Oil | Reciprocating |
| 20200505 | Internal Combustion Engines | Industrial | Residual/Crude Oil | Reciprocating: Crankcase Blowby |
| 20200506 | Internal Combustion Engines | Industrial | Residual/Crude Oil | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20200507 | Internal Combustion Engines | Industrial | Residual/Crude Oil | Reciprocating: Exhaust |
| 30190002 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30190012 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Residual Oil: Incinerators |
| 30190022 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Residual Oil: Flares |
| 30290002 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30390002 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30390012 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Residual Oil: Incinerators |
| 30390022 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Residual Oil: Flares |
| 30490002 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30490012 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Residual Oil: Incinerators |
| 30490022 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Residual Oil: Flares |
| 30490032 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Residual Oil: Furnaces |
| 30500207 | Industrial Processes | Mineral Products | Asphalt Concrete | Asphalt Heater: Residual Oil |
| 30505021 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) | Asphalt Heater: Residual Oil |
| 30590002 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30590012 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Residual Oil: Incinerators |
| 30600111 | Industrial Processes | Petroleum Industry | Process Heaters | Oil-fired (No. 6 Oil) : 100 Million Btu Capacity |
| 30600902 | Industrial Processes | Petroleum Industry | Flares | Residual Oil |
| 30609902 | Industrial Processes | Petroleum Industry | Incinerators | Residual Oil |
| 30890002 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30890012 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Residual Oil: Incinerators |
| 30890022 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Residual Oil: Flares |
| 30990002 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Residual Oil: Process Heaters |
| 30990012 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Residual Oil: Incinerators |
| 31000402 | Industrial Processes | Oil and Gas Production | Process Heaters | Residual Oil |
| 31000412 | Industrial Processes | Oil and Gas Production | Process Heaters | Residual Oil: Steam Generators |
| 31390002 | Industrial Processes | Electrical Equipment | Process Heaters | Residual Oil |
| 39000402 | Industrial Processes | In-process Fuel Use | Residual Oil | Cement Kiln/Dryer |
| 39000403 | Industrial Processes | In-process Fuel Use | Residual Oil | Lime Kiln |
| 39000499 | Industrial Processes | In-process Fuel Use | Residual Oil | General |
| 39990002 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Residual Oil: Process Heaters |
| 39990012 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Residual Oil: Incinerators |
| 39990022 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Residual Oil: Flares |
| 40201003 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater | Residual Oil |
| 40290012 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment | Residual Oil: Incinerator/Afterburner |
| 49090012 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment | Residual Oil: Incinerators |
| 49090022 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment | Residual Oil: Flares |
| 2102006000 - Stationary Source Fuel Combustion; Industrial; Natural Gas; Total: Boilers and IC Engines |  |  |  |  |
| 10200601 | External Combustion Boilers | Industrial | Natural Gas | > 100 Million BTU/hr |
| 10200602 | External Combustion Boilers | Industrial | Natural Gas | 10-100 Million BTU/hr |
| 10200603 | External Combustion Boilers | Industrial | Natural Gas | < 10 Million BTU/hr |
| 10200604 | External Combustion Boilers | Industrial | Natural Gas | Cogeneration |
| 10201401 | External Combustion Boilers | Industrial | CO Boiler | Natural Gas |
| 10500106 | External Combustion | Space Heaters | Industrial | Natural Gas |
| 20200201 | Internal Combustion Engines | Industrial | Natural Gas | Turbine |
| 20200202 | Internal Combustion Engines | Industrial | Natural Gas | Reciprocating |
| 20200203 | Internal Combustion Engines | Industrial | Natural Gas | Turbine: Cogeneration |
| 20200204 | Internal Combustion Engines | Industrial | Natural Gas | Reciprocating: Cogeneration |
| 20200205 | Internal Combustion Engines | Industrial | Natural Gas | Reciprocating: Crankcase Blowby |
| 20200206 | Internal Combustion Engines | Industrial | Natural Gas | Reciprocating: Evaporative Losses (Fuel Delivery System) |
| 20200207 | Internal Combustion Engines | Industrial | Natural Gas | Reciprocating: Exhaust |
| 20200208 | Internal Combustion Engines | Industrial | Natural Gas | Turbine: Evaporative Losses (Fuel Delivery System) |
| 20200209 | Internal Combustion Engines | Industrial | Natural Gas | Turbine: Exhaust |
| 20200251 | Internal Combustion Engines | Industrial | Natural Gas | 2-cycle Rich Burn |
| 20200252 | Internal Combustion Engines | Industrial | Natural Gas | 2-cycle Lean Burn |
| 20200253 | Internal Combustion Engines | Industrial | Natural Gas | 4-cycle Rich Burn |
| 20200254 | Internal Combustion Engines | Industrial | Natural Gas | 4-cycle Lean Burn |
| 20200255 | Internal Combustion Engines | Industrial | Natural Gas | 2-cycle Clean Burn |
| 20200256 | Internal Combustion Engines | Industrial | Natural Gas | 4-cycle Clean Burn |
| 30190003 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30190013 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Natural Gas: Incinerators |
| 30190023 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment | Natural Gas: Flares |
| 30290003 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30291001 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment | Broiling Food: Natural Gas |
| 30390003 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30390013 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Natural Gas: Incinerators |
| 30390023 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment | Natural Gas: Flares |
| 30400407 | Industrial Processes | Secondary Metal Production | Lead | Pot Furnace Heater: Natural Gas |
| 30490003 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30490013 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Natural Gas: Incinerators |
| 30490023 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Natural Gas: Flares |
| 30490033 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Natural Gas: Furnaces |
| 30500206 | Industrial Processes | Mineral Products | Asphalt Concrete | Asphalt Heater: Natural Gas |
| 30505020 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) | Asphalt Heater: Natural Gas |
| 30590003 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30590013 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Natural Gas: Incinerators |
| 30590023 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Natural Gas: Flares |
| 30600105 | Industrial Processes | Petroleum Industry | Process Heaters | Natural Gas-fired |
| 30600903 | Industrial Processes | Petroleum Industry | Flares | Natural Gas |
| 30609903 | Industrial Processes | Petroleum Industry | Incinerators | Natural Gas |
| 30890003 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30890013 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Natural Gas: Incinerators |
| 30890023 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Natural Gas: Flares |
| 30990003 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Natural Gas: Process Heaters |
| 30990013 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Natural Gas: Incinerators |
| 30990023 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment | Natural Gas: Flares |
| 31000404 | Industrial Processes | Oil and Gas Production | Process Heaters | Natural Gas |
| 31390003 | Industrial Processes | Electrical Equipment | Process Heaters | Natural Gas |
| 39000602 | Industrial Processes | In-process Fuel Use | Natural Gas | Cement Kiln/Dryer |
| 39000603 | Industrial Processes | In-process Fuel Use | Natural Gas | Lime Kiln |
| 39000605 | Industrial Processes | In-process Fuel Use | Natural Gas | Metal Melting ** |
| 39000699 | Industrial Processes | In-process Fuel Use | Natural Gas | General |
| 39900601 | Industrial Processes | Miscellaneous Manufacturing Industries | Process Heater/Furnace | Natural Gas |
| 39990003 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Natural Gas: Process Heaters |
| 39990013 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Natural Gas: Incinerators |
| 39990023 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries | Natural Gas: Flares |
| 40201001 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater | Natural Gas |
| 40290013 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment | Natural Gas: Incinerator/Afterburner |
| 40290023 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment | Natural Gas: Flares |
| 49090013 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment | Natural Gas: Incinerators |
| 49090023 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment | Natural Gas: Flares |
| 50390006 | Waste Disposal | Solid Waste Disposal - Industrial | Auxillary Fuel/No Emissions | Natural Gas |
| 2102007000 - Stationary Source Fuel Combustion; Industrial; Liquified Petroleum Gas (LPG); Total: All Boiler Types |  |  |  |  |
| 10201001 | External Combustion Boilers | Industrial | Liquified Petroleum Gas (LPG) | Butane |
| 10201002 | External Combustion Boilers | Industrial | Liquified Petroleum Gas (LPG) | Propane |
| 10201003 | External Combustion Boilers | Industrial | Liquified Petroleum Gas (LPG) | Butane/Propane Mixture: Specify Percent Butane in Comments |
| 10500110 | External Combustion | Space Heaters | Industrial | Liquified Petroleum Gas (LPG) |
| 20201001 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Propane: Reciprocating |
| 20201002 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Butane: Reciprocating |
| 20201005 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Reciprocating: Crankcase Blowby |
| 20201006 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20201007 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Reciprocating: Exhaust |
| 20201008 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20201009 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Turbine: Exhaust |
| 20201011 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Turbine |
| 20201012 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Reciprocating Engine |
| 20201013 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Turbine: Cogeneration |
| 20201014 | Internal Combustion Engines | Industrial | Liquified Petroleum Gas (LPG) | Reciprocating Engine: Cogeneration |
| 27300320 | Internal Combustion Engines | Off-highway LPG-fueled Engines | Industrial Equipment | Industrial Fork Lift: Liquified Petroleum Gas (LPG) |
| 30290005 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment | Liquified Petroleum Gas (LPG): Process Heaters |
| 30490035 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment | Propane: Furnaces |
| 30500209 | Industrial Processes | Mineral Products | Asphalt Concrete | Asphalt Heater: LPG |
| 30505023 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) | Asphalt Heater: LP Gas |
| 30590005 | Industrial Processes | Mineral Products | Fuel Fired Equipment | Liquified Petroleum Gas (LPG): Process Heaters |
| 30600107 | Industrial Processes | Petroleum Industry | Process Heaters | LPG-fired |
| 30600905 | Industrial Processes | Petroleum Industry | Flares | Liquified Petroleum Gas |
| 30609905 | Industrial Processes | Petroleum Industry | Incinerators | Liquified Petroleum Gas |
| 30890004 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment | Liquified Petroleum Gas (LPG): Process Heaters |
| 31000406 | Industrial Processes | Oil and Gas Production | Process Heaters | Propane/Butane |
| 39001099 | Industrial Processes | In-process Fuel Use | Liquified Petroleum Gas | General |
| 39901001 | Industrial Processes | Miscellaneous Manufacturing Industries | Process Heater/Furnace | LPG |
| 40201004 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater | Liquified Petroleum Gas (LPG) |
| 50390010 | Waste Disposal | Solid Waste Disposal - Industrial | Auxillary Fuel/No Emissions | Liquified Petroleum Gas (LPG) |
| 2102008000 - Stationary Source Fuel Combustion; Industrial; Wood; Total: All Boiler Types |  |  |  |  |
| 10200901 | External Combustion Boilers | Industrial | Wood/Bark Waste | Bark-fired Boiler |
| 10200902 | External Combustion Boilers | Industrial | Wood/Bark Waste | Wood/Bark-fired Boiler |
| 10200903 | External Combustion Boilers | Industrial | Wood/Bark Waste | Wood-fired Boiler - Wet Wood (>=20% moisture) |
| 10200904 | External Combustion Boilers | Industrial | Wood/Bark Waste | Bark-fired Boiler (< 50,000 Lb Steam) ** |
| 10200905 | External Combustion Boilers | Industrial | Wood/Bark Waste | Wood/Bark-fired Boiler (< 50,000 Lb Steam) ** |
| 10200906 | External Combustion Boilers | Industrial | Wood/Bark Waste | Wood-fired Boiler (< 50,000 Lb Steam) ** |
| 10200907 | External Combustion Boilers | Industrial | Wood/Bark Waste | Wood Cogeneration |
| 10200908 | External Combustion Boilers | Industrial | Wood/Bark Waste | Wood-fired Boiler - Dry Wood (<20% moisture) |
| 10200910 | External Combustion Boilers | Industrial | Wood/Bark Waste | Fuel cell/Dutch oven boilers ** |
| 10200911 | External Combustion Boilers | Industrial | Wood/Bark Waste | Stoker boilers ** |
| 10200912 | External Combustion Boilers | Industrial | Wood/Bark Waste | Fluidized bed combustion boiler |
| 39000989 | Industrial Processes | In-process Fuel Use | Wood | General |
| 39000999 | Industrial Processes | In-process Fuel Use | Wood | General: Wood |
| 2102011000 - Stationary Source Fuel Combustion; Industrial; Kerosene; Total: All Boiler Types |  |  |  |  |
| 20200901 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Turbine |
| 20200902 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Reciprocating |
| 20200905 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Reciprocating: Crankcase Blowby |
| 20200906 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20200907 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Reciprocating: Exhaust |
| 20200908 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20200909 | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) | Turbine: Exhaust |
| 20400110 | Internal Combustion Engines | Engine Testing | Aircraft Engine Testing | Jet A Fuel |
| 20400111 | Internal Combustion Engines | Engine Testing | Aircraft Engine Testing | JP-5 Fuel |
| 20400112 | Internal Combustion Engines | Engine Testing | Aircraft Engine Testing | JP-4 Fuel |
| 20400406 | Internal Combustion Engines | Engine Testing | Reciprocating Engine | Kerosene/Naphtha (Jet Fuel) |

Table A-2. Commercial/Institutional Fuel Combustion Crosswalk for Point Source Subtraction

| Point SCC | SCC1 DESC | SCC3 DESC | SCC6 DESC | SCC8 DESC |
| --- | --- | --- | --- | --- |
| 2103001000 - Stationary Source Fuel Combustion; Commercial/Institutional; Anthracite Coal; Total: All Boiler Types |  |  |  |  |
| 10300101 | External Combustion Boilers | Commercial/Institutional | Anthracite Coal | Pulverized Coal |
| 10300102 | External Combustion Boilers | Commercial/Institutional | Anthracite Coal | Traveling Grate (Overfeed) Stoker |
| 10300103 | External Combustion Boilers | Commercial/Institutional | Anthracite Coal | Hand-fired |
| 2103002000 - Stationary Source Fuel Combustion; Commercial/Institutional; Bituminous/Subbituminous Coal; Total: All Boiler Types |  |  |  |  |
| 10300203 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Cyclone Furnace |
| 10300205 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Pulverized Coal: Wet Bottom |
| 10300206 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Pulverized Coal: Dry Bottom |
| 10300207 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Overfeed Stoker |
| 10300208 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Underfeed Stoker |
| 10300209 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Spreader Stoker |
| 10300214 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Hand-fired |
| 10300216 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Pulverized Coal: Dry Bottom (Tangential) |
| 10300217 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| 10300218 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Atmospheric Fluidized Bed Combustion: Circulating Bed |
| 10300221 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Pulverized Coal: Wet Bottom |
| 10300222 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Pulverized Coal: Dry Bottom |
| 10300223 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Cyclone Furnace |
| 10300224 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Spreader Stoker |
| 10300225 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Traveling Grate (Overfeed) Stoker |
| 10300226 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Pulverized Coal: Dry Bottom Tangential |
| 50190002 | Waste Disposal | Solid Waste Disposal - Government | Auxillary Fuel/No Emissions | Coal |
| 50290002 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxillary Fuel/No Emissions | Coal |
| 2103004001 - Stationary Source Fuel Combustion; Commercial/Institutional; Distillate Oil; Boilers |  |  |  |  |
| 10300501 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | Grades 1 and 2 Oil |
| 10300502 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | 10-100 Million BTU/hr ** |
| 10300503 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | < 10 Million BTU/hr ** |
| 10300504 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | Grade 4 Oil |
| 10500205 | External Combustion | Space Heaters | Commercial/Institutional | Distillate Oil |
| 50100602 | Waste Disposal | Solid Waste Disposal - Government | Fire Fighting | Structure: Distillate Oil |
| 50190005 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions | Distillate Oil |
| 50290005 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions | Distillate Oil |
| 2103004002 - Stationary Source Fuel Combustion; Commercial/Institutional; Distillate Oil; IC Engines |  |  |  |  |
| 20300101 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating |
| 20300102 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Turbine |
| 20300105 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating: Crankcase Blowby |
| 20300106 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20300107 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating: Exhaust |
| 20300108 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20300109 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Turbine: Exhaust |
| 2103005000 - Stationary Source Fuel Combustion; Commercial/Institutional; Residual Oil; Total: All Boiler Types |  |  |  |  |
| 10300401 | External Combustion Boilers | Commercial/Institutional | Residual Oil | Grade 6 Oil |
| 10300402 | External Combustion Boilers | Commercial/Institutional | Residual Oil | 10-100 Million BTU/hr ** |
| 10300403 | External Combustion Boilers | Commercial/Institutional | Residual Oil | < 10 Million BTU/hr ** |
| 10300404 | External Combustion Boilers | Commercial/Institutional | Residual Oil | Grade 5 Oil |
| 2103006000 - Stationary Source Fuel Combustion; Commercial/Institutional; Natural Gas; Total: Boilers and IC Engines |  |  |  |  |
| 10300601 | External Combustion Boilers | Commercial/Institutional | Natural Gas | > 100 Million BTU/hr |
| 10300602 | External Combustion Boilers | Commercial/Institutional | Natural Gas | 10-100 Million BTU/hr |
| 10300603 | External Combustion Boilers | Commercial/Institutional | Natural Gas | < 10 Million BTU/hr |
| 10500206 | External Combustion | Space Heaters | Commercial/Institutional | Natural Gas |
| 20300201 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating |
| 20300202 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine |
| 20300203 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine: Cogeneration |
| 20300204 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Cogeneration |
| 20300205 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Crankcase Blowby |
| 20300206 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Evaporative Losses (Fuel Delivery System) |
| 20300207 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Exhaust |
| 20300208 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine: Evaporative Losses (Fuel Delivery System) |
| 20300209 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine: Exhaust |
| 50190006 | Waste Disposal | Solid Waste Disposal - Government | Auxillary Fuel/No Emissions | Natural Gas |
| 50290006 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxillary Fuel/No Emissions | Natural Gas |
| 2103007000 - Stationary Source Fuel Combustion; Commercial/Institutional; Liquified Petroleum Gas (LPG); Total: All Combustor Types |  |  |  |  |
| 10301001 | External Combustion Boilers | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Butane |
| 10301002 | External Combustion Boilers | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Propane |
| 10301003 | External Combustion Boilers | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Butane/Propane Mixture: Specify Percent Butane in Comments |
| 10500210 | External Combustion | Space Heaters | Commercial/Institutional | Liquified Petroleum Gas (LPG) |
| 20301001 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Propane: Reciprocating |
| 20301002 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Butane: Reciprocating |
| 20301005 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Reciprocating: Crankcase Blowby |
| 20301006 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20301007 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Reciprocating: Exhaust |
| 50190010 | Waste Disposal | Solid Waste Disposal - Government | Auxillary Fuel/No Emissions | Liquified Petroleum Gas (LPG) |
| 50290010 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxillary Fuel/No Emissions | Liquified Petroleum Gas (LPG) |
| 2103008000 - Stationary Source Fuel Combustion; Commercial/Institutional; Wood; Total: All Boiler Types |  |  |  |  |
| 10300901 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Bark-fired Boiler |
| 10300902 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Wood/Bark-fired Boiler |
| 10300903 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Wood-fired Boiler - Wet Wood (>=20% moisture) |
| 10300908 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Wood-fired Boiler - Dry Wood (<20% moisture) |
| 10300910 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Fuel cell/Dutch oven boilers ** |
| 10300911 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Stoker boilers ** |
| 10300912 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Fluidized bed combustion boilers |
| 10500209 | External Combustion | Space Heaters | Commercial/Institutional | Wood |
| 2103011000 - Stationary Source Fuel Combustion; Commercial/Institutional; Kerosene; Total: All Combustor Types |  |  |  |  |
| 20300901 | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) | Turbine: JP-4 |
| 20300908 | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20300909 | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) | Turbine: Exhaust |
| 50100603 | Waste Disposal | Solid Waste Disposal - Government | Fire Fighting | Structure: Kerosene |

Table A-2. Commercial/Institutional Fuel Combustion Crosswalk for Point Source Subtractions

| Point SCC | SCC1 DESC | SCC3 DESC | SCC6 DESC | SCC8 DESC |
| --- | --- | --- | --- | --- |
| 2103001000 - Stationary Source Fuel Combustion; Commercial/Institutional; Anthracite Coal; Total: All Boiler Types |  |  |  |  |
| 10300101 | External Combustion Boilers | Commercial/Institutional | Anthracite Coal | Pulverized Coal |
| 10300102 | External Combustion Boilers | Commercial/Institutional | Anthracite Coal | Traveling Grate (Overfeed) Stoker |
| 10300103 | External Combustion Boilers | Commercial/Institutional | Anthracite Coal | Hand-fired |
| 2103002000 - Stationary Source Fuel Combustion; Commercial/Institutional; Bituminous/Subbituminous Coal; Total: All Boiler Types |  |  |  |  |
| 10300203 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Cyclone Furnace |
| 10300205 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Pulverized Coal: Wet Bottom |
| 10300206 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Pulverized Coal: Dry Bottom |
| 10300207 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Overfeed Stoker |
| 10300208 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Underfeed Stoker |
| 10300209 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Spreader Stoker |
| 10300211 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Overfeed Stoker ** |
| 10300214 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Hand-fired |
| 10300216 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Pulverized Coal: Dry Bottom (Tangential) |
| 10300217 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| 10300218 | External Combustion Boilers | Commercial/Institutional | Bituminous Coal | Atmospheric Fluidized Bed Combustion: Circulating Bed |
| 10300221 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Pulverized Coal: Wet Bottom |
| 10300222 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Pulverized Coal: Dry Bottom |
| 10300223 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Cyclone Furnace |
| 10300224 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Spreader Stoker |
| 10300225 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Traveling Grate (Overfeed) Stoker |
| 10300226 | External Combustion Boilers | Commercial/Institutional | Subbituminous Coal | Pulverized Coal: Dry Bottom Tangential |

Table A-2. Commercial/Institutional Fuel Combustion Crosswalk for Point Source Subtractions (Cont.)

| Point SCC | SCC1 DESC | SCC3 DESC | SCC6 DESC | SCC8 DESC |
| --- | --- | --- | --- | --- |
| 10500202 | External Combustion | Space Heaters | Commercial/Institutional | Coal ** |
| 50190002 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions | Coal |
| 50290002 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions | Coal |
| 2103004000 - Stationary Source Fuel Combustion; Commercial/Institutional; Distillate Oil; Total: Boilers and IC Engines |  |  |  |  |
| 10300501 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | Grades 1 and 2 Oil |
| 10300502 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | 10-100 Million BTU/hr ** |
| 10300503 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | < 10 Million BTU/hr ** |
| 10300504 | External Combustion Boilers | Commercial/Institutional | Distillate Oil | Grade 4 Oil |
| 10500205 | External Combustion | Space Heaters | Commercial/Institutional | Distillate Oil |
| 20300101 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating |
| 20300102 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Turbine |
| 20300105 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating: Crankcase Blowby |
| 20300106 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20300107 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Reciprocating: Exhaust |
| 20300108 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20300109 | Internal Combustion Engines | Commercial/Institutional | Distillate Oil (Diesel) | Turbine: Exhaust |
| 50100602 | Waste Disposal | Solid Waste Disposal - Government | Fire Fighting | Structure: Distillate Oil |
| 50190005 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions | Distillate Oil |
| 50290005 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions | Distillate Oil |
| 2103005000 - Stationary Source Fuel Combustion; Commercial/Institutional; Residual Oil; Total: All Boiler Types |  |  |  |  |
| 10300401 | External Combustion Boilers | Commercial/Institutional | Residual Oil | Grade 6 Oil |
| 10300402 | External Combustion Boilers | Commercial/Institutional | Residual Oil | 10-100 Million BTU/hr ** |

Table A-2. Commercial/Institutional Fuel Combustion Crosswalk for Point Source Subtractions (Cont.)

| Point SCC | SCC1 DESC | SCC3 DESC | SCC6 DESC | SCC8 DESC |
| --- | --- | --- | --- | --- |
| 10300403 | External Combustion Boilers | Commercial/Institutional | Residual Oil | < 10 Million BTU/hr ** |
| 10300404 | External Combustion Boilers | Commercial/Institutional | Residual Oil | Grade 5 Oil |
| 2103006000 - Stationary Source Fuel Combustion; Commercial/Institutional; Natural Gas; Total: Boilers and IC Engines |  |  |  |  |
| 10300601 | External Combustion Boilers | Commercial/Institutional | Natural Gas | > 100 Million BTU/hr |
| 10300602 | External Combustion Boilers | Commercial/Institutional | Natural Gas | 10-100 Million BTU/hr |
| 10300603 | External Combustion Boilers | Commercial/Institutional | Natural Gas | < 10 Million BTU/hr |
| 10500206 | External Combustion | Space Heaters | Commercial/Institutional | Natural Gas |
| 20300201 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating |
| 20300202 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine |
| 20300203 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine: Cogeneration |
| 20300204 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Cogeneration |
| 20300205 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Crankcase Blowby |
| 20300206 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Evaporative Losses (Fuel Delivery System) |
| 20300207 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Reciprocating: Exhaust |
| 20300208 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine: Evaporative Losses (Fuel Delivery System) |
| 20300209 | Internal Combustion Engines | Commercial/Institutional | Natural Gas | Turbine: Exhaust |
| 50190006 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions | Natural Gas |
| 50290006 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions | Natural Gas |
| 2103007000 - Stationary Source Fuel Combustion; Commercial/Institutional; Liquified Petroleum Gas (LPG); Total: All Combustor Types |  |  |  |  |
| 10301001 | External Combustion Boilers | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Butane |
| 10301002 | External Combustion Boilers | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Propane |

Table A-2. Commercial/Institutional Fuel Combustion Crosswalk for Point Source Subtractions (Cont.)

| Point SCC | SCC1 DESC | SCC3 DESC | SCC6 DESC | SCC8 DESC |
| --- | --- | --- | --- | --- |
| 10301003 | External Combustion Boilers | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Butane/Propane Mixture: Specify Percent Butane in Comments |
| 10500210 | External Combustion | Space Heaters | Commercial/Institutional | Liquified Petroleum Gas (LPG) |
| 20301001 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Propane: Reciprocating |
| 20301002 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Butane: Reciprocating |
| 20301005 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Reciprocating: Crankcase Blowby |
| 20301006 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| 20301007 | Internal Combustion Engines | Commercial/Institutional | Liquified Petroleum Gas (LPG) | Reciprocating: Exhaust |
| 50190010 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions | Liquified Petroleum Gas (LPG) |
| 50290010 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions | Liquified Petroleum Gas (LPG) |
| 2103008000 - Stationary Source Fuel Combustion; Commercial/Institutional; Wood; Total: All Boiler Types |  |  |  |  |
| 10300901 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Bark-fired Boiler |
| 10300902 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Wood/Bark-fired Boiler |
| 10300903 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Wood-fired Boiler - Wet Wood (>=20% moisture) |
| 10300908 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Wood-fired Boiler - Dry Wood (<20% moisture) |
| 10300910 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Fuel cell/Dutch oven boilers ** |
| 10300911 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Stoker boilers ** |
| 10300912 | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste | Fluidized bed combustion boilers |
| 10500209 | External Combustion | Space Heaters | Commercial/Institutional | Wood |
| 2103011000 - Stationary Source Fuel Combustion; Commercial/Institutional; Kerosene; Total: All Combustor Types |  |  |  |  |
| 20300901 | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) | Turbine: JP-4 |
| 20300908 | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) | Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| 20300909 | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) | Turbine: Exhaust |
| 50100603 | Waste Disposal | Solid Waste Disposal - Government | Fire Fighting | Structure: Kerosene |

Table A-3. Criteria Pollutant Emission Factors for ICI Area Source Categories

| SCC | Description | Emission Factor Units1 | VOC | NOx | CO | SO2 | PM2.5-FIL | PM10-FIL | PM-CON | NH3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2102001000 | Industrial Anthracite Coal | lb/ton | 0.3 | 9 | 0.6 | 39 * S% | 0.48 * A% | 1.1 * A% | 0.08*A% | 0.03 |
| 2102002000 | Industrial Bitum/Subbitum Coal | lb/ton | 0.05 | 11 | 5 | 38 * S% | 1.4 | 12 | 1.04 | 0.03 |
| 2102004001 | Industrial Distillate Boilers | lb/1000 gal | 0.2 | 20 | 5 | 142 * S% | 0.25 | 1 | 1.3 | 0.8 |
| 2102004002 | Industrial Distillate IC Engines | lb/1000 gal | 42 | 604 | 130 | 39.8 | 39.8 | 42.5 | 1 | 0.8 |
| 2102005000 | Industrial Residual Oil | lb/1000 gal | 0.28 | 55 | 5 | 157 * S% | 4.67 * (1.12 * S% + 0.37) | 7.17 * (1.12 * S% + 0.37) | 1.5 | 0.8 |
| 2102006000 | Industrial Natural Gas | lb/MMcf | 5.5 | 100 | 84 | 0.6 | 0.11 | 0.2 | 0.322 | 3.2 |
| 2102007000 | Industrial LPG 3 | lb/1000 gal | 0.52 | 14.23 | 7.97 | 0.06 | 0.01 | 0.02 | 0.03 | 0.34 |
| 2102008000 | Industrial Wood 5 | lb/MMBtu | 0.017 | 0.22 | 0.6 | 0.025 | 0.43 | 0.5 | 0.017 | 0.0076 |
| 2102011000 | Industrial Kerosene | lb/1000 gal | 0.19 | 19.29 | 4.82 | 142 * S%7 | 0.24 | 0.96 | 1.25 | 0.77 |
| 2103001000 | Comm/Inst Anthracite Coal | lb/ton | 0.3 | 9 | 0.6 | 39 * S% | 0.48 * A% | 1.1 * A% | 0.08 * A% | 0.03 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | lb/ton | 0.05 | 11 | 5 | 38 * S% | 1.4 | 12 | 1.04 | 0.03 |
| 2103004001 | Comm/Inst Distillate Boilers | lb/1000 gal | 0.34 | 20 | 5 | 142 * S% | 0.83 | 1.08 | 1.3 | 0.8 |
| 2103004002 | Comm/Inst Distillate IC Engines | lb/1000 gal | 42 | 604 | 130 | 39.8 | 42.5 | 42.5 |  | 0.8 |
| 2103005000 | Comm/Inst Residual Oil | lb/1000 gal | 1.13 | 55 | 5 | 157 * S% | 1.92 * (1.12 * S% + 0.37) | 5.17 * (1.12 * S% + 0.37) | 1.5 | 0.8 |
| 2103006000 | Comm/Inst Natural Gas | lb/MMcf | 5.5 | 100 | 84 | 0.6 | 0.11 | 0.2 | 0.32 | 0.49 |
| 2103007000 | Comm/Inst LPG | lb/1000 gal | 0.52 | 14.23 | 7.97 | 0.06 | 0.01 | 0.02 | 0.03 | 0.05 |
| 2103008000 | Comm/Inst Wood 5 | lb/MMBtu | 0.017 | 0.22 | 0.6 | 0.025 | 0.43 | 0.5 | 0.017 | 0.0056 |
| 2103011000 | Comm/Inst Kerosene | lb/1000 gal | 0.33 | 19.3 | 4.8 | 142 * S% | 0.8 | 1.04 | 1.3 | 0.8 |

Source:	Unless otherwise noted, ERTAC emission factors used to support the 2011 NEI (Huntley, 2009).

Notes:	1 lb = pound; ton = short ton; gal = gallon; MMcf = million cubic feet; MMBtu = million British thermal units; bbl = barrels; S% = percent sulfur content; A% = percent

ash content

2 The EPA ERTAC emission factor workbook (Huntley, 2009) for this emission factors (EF) contains an error. The change log in the ERTAC workbook conflicts with the actual changes made to the emission factors spreadsheet. The PM-CON EF should be 0.32 lb/MMcf for 2102006000 instead of the 0.49 lb/MMcf value reported in the ERTAC workbook.

3 Emission factors from Commercial/Institutional LPG.

4 The EPA ERTAC emission factor workbook (Huntley, 2009) for this emission factors (EF) contains an error. The change log in the ERTAC workbook conflicts with the actual changes made to the emission factors spreadsheet. The NH3 EF should be 0.3 lb/1000 gal for 2102007000 instead of the 0.05 lb/1000 gal value reported in the ERTAC workbook.

5 Emission factors from AP-42, Section 1.6, Wood Residue Combustion in Boilers (EPA, 2003).

6 Emission factor from Pechan, 2004 (converted from lb/ton using 0.08 ton/MMBtu for Industrial sector and 0.0625 ton/MMBtu for Commercial sector).

7 The EPA ERTAC emission factor workbook (Huntley, 2009) for this emission factors (EF) contains an error. The ERTAC workbook uses the equation 157*S%. The correct EF equation is 142*S%.

Table A-4. Speciation factors for HAP VOCs. These factors are multiplied by VOC emissions to estimate emissions for each HAP.

| SCC | Description | Pollutant Code | Pollutant Code Description | Speciation Ratio |
| --- | --- | --- | --- | --- |
| 2102001000 | Industrial Anthracite Coal | 85018 | Phenanthrene | 2.27E-02 |
| 2102001000 | Industrial Anthracite Coal | 92524 | Biphenyl | 8.33E-02 |
| 2102001000 | Industrial Anthracite Coal | 91203 | Naphthalene | 4.33E-01 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 60344 | Methylhydrazine | 3.40E-03 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 67663 | Chloroform | 1.18E-03 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 75003 | Ethyl Chloride | 8.40E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 75252 | Bromoform | 7.80E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 56832736 | Benzofluoranthenes | 2.20E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 91203 | Naphthalene | 2.60E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 77781 | Dimethyl Sulfate | 9.60E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 80626 | Methyl Methacrylate | 4.00E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 83329 | Acenaphthene | 1.02E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 85018 | Phenanthrene | 5.40E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 1634044 | Methyl Tert-Butyl Ether | 7.00E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 92524 | Biphenyl | 3.40E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 100447 | Benzyl Chloride | 1.40E-02 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 98828 | Cumene | 1.06E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 3697243 | 5-Methylchrysene | 4.40E-07 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 1330207 | Xylenes (Mixed Isomers) | 7.40E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 532274 | 2-Chloroacetophenone | 1.40E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 208968 | Acenaphthylene | 5.00E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 206440 | Fluoranthene | 1.42E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 121142 | 2,4-Dinitrotoluene | 5.60E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 120127 | Anthracene | 4.20E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 108054 | Vinyl Acetate | 1.52E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 106934 | Ethylene Dibromide | 2.40E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 86737 | Fluorene | 1.82E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 50000 | Formaldehyde | 4.80E-03 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 191242 | Benzo[g,h,i,]Perylene | 5.40E-07 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 193395 | Indeno[1,2,3-c,d]Pyrene | 1.22E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 129000 | Pyrene | 6.60E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 218019 | Chrysene | 2.00E-06 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 50328 | Benzo[a]Pyrene | 7.60E-07 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 56553 | Benz[a]Anthracene | 1.60E-06 |
| 2102004001 | Industrial Distillate Boilers | 50000 | Formaldehyde | 2.40E-01 |
| 2102004002 | Industrial Distillate IC Engines | 120127 | Anthracene | 6.10E-06 |
| 2102004002 | Industrial Distillate IC Engines | 53703 | Dibenzo[a,h]Anthracene | 1.90E-06 |
| 2102004002 | Industrial Distillate IC Engines | 50328 | Benzo[a]Pyrene | 6.13E-07 |
| 2102004002 | Industrial Distillate IC Engines | 50000 | Formaldehyde | 3.85E-03 |
| 2102004002 | Industrial Distillate IC Engines | 208968 | Acenaphthylene | 1.65E-05 |
| 2102004002 | Industrial Distillate IC Engines | 206440 | Fluoranthene | 2.48E-05 |
| 2102004002 | Industrial Distillate IC Engines | 205992 | Benzo[b]Fluoranthene | 3.23E-07 |
| 2102004002 | Industrial Distillate IC Engines | 193395 | Indeno[1,2,3-c,d]Pyrene | 1.22E-06 |
| 2102004002 | Industrial Distillate IC Engines | 191242 | Benzo[g,h,i,]Perylene | 1.60E-06 |
| 2102004002 | Industrial Distillate IC Engines | 56553 | Benz[a]Anthracene | 5.48E-06 |
| 2102004002 | Industrial Distillate IC Engines | 129000 | Pyrene | 1.56E-05 |
| 2102004002 | Industrial Distillate IC Engines | 218019 | Chrysene | 1.15E-06 |
| 2102004002 | Industrial Distillate IC Engines | 108883 | Toluene | 1.33E-03 |
| 2102004002 | Industrial Distillate IC Engines | 107028 | Acrolein | 3.02E-04 |
| 2102004002 | Industrial Distillate IC Engines | 106990 | 1,3-Butadiene | 1.28E-04 |
| 2102004002 | Industrial Distillate IC Engines | 1330207 | Xylenes (Mixed Isomers) | 9.30E-04 |
| 2102004002 | Industrial Distillate IC Engines | 75070 | Acetaldehyde | 2.50E-03 |
| 2102004002 | Industrial Distillate IC Engines | 83329 | Acenaphthene | 4.63E-06 |
| 2102004002 | Industrial Distillate IC Engines | 85018 | Phenanthrene | 9.59E-05 |
| 2102004002 | Industrial Distillate IC Engines | 86737 | Fluorene | 9.52E-05 |
| 2102004002 | Industrial Distillate IC Engines | 91203 | Naphthalene | 2.77E-04 |
| 2102004002 | Industrial Distillate IC Engines | 207089 | Benzo[k]Fluoranthene | 5.06E-07 |
| 2102004002 | Industrial Distillate IC Engines | 71432 | Benzene | 3.04E-03 |
| 2102005000 | Industrial Residual Oil | 83329 | Acenaphthene | 8.04E-05 |
| 2102005000 | Industrial Residual Oil | 53703 | Dibenzo[a,h]Anthracene | 6.43E-06 |
| 2102005000 | Industrial Residual Oil | 56553 | Benz[a]Anthracene | 1.55E-05 |
| 2102005000 | Industrial Residual Oil | 218019 | Chrysene | 9.11E-06 |
| 2102005000 | Industrial Residual Oil | 71432 | Benzene | 8.04E-04 |
| 2102005000 | Industrial Residual Oil | 50000 | Formaldehyde | 1.29E-01 |
| 2102005000 | Industrial Residual Oil | 208968 | Acenaphthylene | 9.64E-07 |
| 2102005000 | Industrial Residual Oil | 85018 | Phenanthrene | 4.02E-05 |
| 2102005000 | Industrial Residual Oil | 91203 | Naphthalene | 4.34E-03 |
| 2102005000 | Industrial Residual Oil | 120127 | Anthracene | 4.66E-06 |
| 2102005000 | Industrial Residual Oil | 129000 | Pyrene | 1.61E-05 |
| 2102005000 | Industrial Residual Oil | 191242 | Benzo[g,h,i,]Perylene | 8.57E-06 |
| 2102005000 | Industrial Residual Oil | 193395 | Indeno[1,2,3-c,d]Pyrene | 8.04E-06 |
| 2102005000 | Industrial Residual Oil | 206440 | Fluoranthene | 1.88E-05 |
| 2102005000 | Industrial Residual Oil | 86737 | Fluorene | 1.71E-05 |
| 2102005000 | Industrial Residual Oil | 75070 | Acetaldehyde | 1.88E-02 |
| 2102006000 | Industrial Natural Gas | 71432 | Benzene | 3.82E-04 |
| 2102006000 | Industrial Natural Gas | 129000 | Pyrene | 9.55E-07 |
| 2102006000 | Industrial Natural Gas | 206440 | Fluoranthene | 5.73E-07 |
| 2102006000 | Industrial Natural Gas | 91203 | Naphthalene | 1.16E-04 |
| 2102006000 | Industrial Natural Gas | 86737 | Fluorene | 5.35E-07 |
| 2102006000 | Industrial Natural Gas | 75070 | Acetaldehyde | 2.48E-06 |
| 2102006000 | Industrial Natural Gas | 50000 | Formaldehyde | 1.43E-02 |
| 2102006000 | Industrial Natural Gas | 85018 | Phenanthrene | 3.25E-06 |
| 2102008000 | Industrial Wood | 83329 | Acenaphthene | 1.47E-04 |
| 2102008000 | Industrial Wood | 91576 | 2-methylnaphthalene | 1.09E-03 |
| 2102008000 | Industrial Wood | 75070 | Acetaldehyde | 1.24E-01 |
| 2102008000 | Industrial Wood | 106990 | 1,3-butadiene | 8.51E-03 |
| 2102008000 | Industrial Wood | 832699 | 1-methylphenanthrene | 1.61E-04 |
| 2102008000 | Industrial Wood | 1330207 | Xylenes (Mixed Isomers) | 5.68E-03 |
| 2102008000 | Industrial Wood | 108883 | Toluene | 1.15E-02 |
| 2102008000 | Industrial Wood | 129000 | Pyrene | 1.36E-04 |
| 2102008000 | Industrial Wood | 123386 | Propionaldehyde (or Propanal; 1-Propanone; 1-Propanal) | 1.85E-02 |
| 2102008000 | Industrial Wood | 108952 | Phenol (carbolic acid) | 3.82E-02 |
| 2102008000 | Industrial Wood | 85018 | Phenanthrene | 1.14E-03 |
| 2102008000 | Industrial Wood | 91203 | Naphthalene | 1.65E-02 |
| 2102008000 | Industrial Wood | 123319 | Hydroquinone (or p-benzenediol; 1,4-Benzenediol) | 1.25E-03 |
| 2102008000 | Industrial Wood | 50000 | Formaldehyde | 8.47E-02 |
| 2102008000 | Industrial Wood | 2531842 | 2-methylphenanthrene | 1.35E-04 |
| 2102008000 | Industrial Wood | 90120 | 1-Methylnaphthalene | 7.71E-04 |
| 2102008000 | Industrial Wood | 208968 | Acenaphthylene | 1.35E-03 |
| 2102008000 | Industrial Wood | 98862 | Acetophenone (or 1-phenylethanone; Methyl phenyl ketone) | 2.84E-04 |
| 2102008000 | Industrial Wood | 1319773 | Cresol/Cresylic Acid (Mixed Isomers) | 3.42E-02 |
| 2102008000 | Industrial Wood | 100414 | Ethylbenzene | 1.67E-03 |
| 2102008000 | Industrial Wood | 206440 | Fluoranthene | 2.22E-04 |
| 2102008000 | Industrial Wood | 86737 | Fluorene | 3.23E-04 |
| 2102008000 | Industrial Wood | 107028 | Acrolein (2-propenal) | 4.58E-03 |
| 2102008000 | Industrial Wood | 120127 | Anthracene | 2.50E-04 |
| 2102008000 | Industrial Wood | 71432 | Benzene | 2.79E-02 |
| 2102008000 | Industrial Wood | 203123 | Benzo[ghi]fluoranthene | 5.96E-06 |
| 2102008000 | Industrial Wood | 120809 | 1,2-Benzenediol | 2.27E-02 |
| 2102011000 | Industrial Kerosene | 56832736 | Benzofluoranthenes | 7.82E-06 |
| 2102011000 | Industrial Kerosene | 193395 | Indeno[1,2,3-c,d]Pyrene | 1.07E-05 |
| 2102011000 | Industrial Kerosene | 56553 | Benz[a]Anthracene | 2.06E-05 |
| 2102011000 | Industrial Kerosene | 218019 | Chrysene | 1.21E-05 |
| 2102011000 | Industrial Kerosene | 208968 | Acenaphthylene | 1.28E-06 |
| 2102011000 | Industrial Kerosene | 53703 | Dibenzo[a,h]Anthracene | 8.53E-06 |
| 2102011000 | Industrial Kerosene | 71432 | Benzene | 1.07E-03 |
| 2102011000 | Industrial Kerosene | 75070 | Acetaldehyde | 2.49E-02 |
| 2102011000 | Industrial Kerosene | 83329 | Acenaphthene | 1.07E-04 |
| 2102011000 | Industrial Kerosene | 85018 | Phenanthrene | 5.33E-05 |
| 2102011000 | Industrial Kerosene | 206440 | Fluoranthene | 2.49E-05 |
| 2102011000 | Industrial Kerosene | 91203 | Naphthalene | 5.76E-03 |
| 2102011000 | Industrial Kerosene | 120127 | Anthracene | 6.18E-06 |
| 2102011000 | Industrial Kerosene | 129000 | Pyrene | 2.13E-05 |
| 2102011000 | Industrial Kerosene | 50000 | Formaldehyde | 1.71E-01 |
| 2102011000 | Industrial Kerosene | 191242 | Benzo[g,h,i,]Perylene | 1.14E-05 |
| 2102011000 | Industrial Kerosene | 86737 | Fluorene | 2.27E-05 |
| 2103001000 | Comm/Inst Anthracite Coal | 91203 | Naphthalene | 4.33E-01 |
| 2103001000 | Comm/Inst Anthracite Coal | 85018 | Phenanthrene | 2.27E-02 |
| 2103001000 | Comm/Inst Anthracite Coal | 92524 | Biphenyl | 8.33E-02 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 85018 | Phenanthrene | 5.40E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 120127 | Anthracene | 4.20E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 100447 | Benzyl Chloride | 1.40E-02 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 98828 | Cumene | 1.06E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 92524 | Biphenyl | 3.40E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 91203 | Naphthalene | 2.60E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 86737 | Fluorene | 1.82E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 106934 | Ethylene Dibromide | 2.40E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 83329 | Acenaphthene | 1.02E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 80626 | Methyl Methacrylate | 4.00E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 77781 | Dimethyl Sulfate | 9.60E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 75252 | Bromoform | 7.80E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 75003 | Ethyl Chloride | 8.40E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 67663 | Chloroform | 1.18E-03 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 60344 | Methylhydrazine | 3.40E-03 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 50000 | Formaldehyde | 4.80E-03 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 121142 | 2,4-Dinitrotoluene | 5.60E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 193395 | Indeno[1,2,3-c,d]Pyrene | 1.22E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 50328 | Benzo[a]Pyrene | 7.60E-07 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 191242 | Benzo[g,h,i,]Perylene | 5.40E-07 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 208968 | Acenaphthylene | 5.00E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 532274 | 2-Chloroacetophenone | 1.40E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 1330207 | Xylenes (Mixed Isomers) | 7.40E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 1634044 | Methyl Tert-Butyl Ether | 7.00E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 3697243 | 5-Methylchrysene | 4.40E-07 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 56832736 | Benzofluoranthenes | 2.20E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 56553 | Benz[a]Anthracene | 1.60E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 218019 | Chrysene | 2.00E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 129000 | Pyrene | 6.60E-06 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 108054 | Vinyl Acetate | 1.52E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 206440 | Fluoranthene | 1.42E-05 |
| 2103004001 | Comm/Inst Distillate Boilers | 50000 | Formaldehyde | 1.41E-01 |
| 2103004001 | Comm/Inst Distillate Boilers | 71432 | Benzene | 8.09E-03 |
| 2103004001 | Comm/Inst Distillate Boilers | 206440 | Fluoranthene | 9.26E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 50328 | Benzo[a]Pyrene | 6.13E-07 |
| 2103004002 | Comm/Inst Distillate IC Engines | 86737 | Fluorene | 9.52E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 85018 | Phenanthrene | 9.59E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 83329 | Acenaphthene | 4.63E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 75070 | Acetaldehyde | 2.50E-03 |
| 2103004002 | Comm/Inst Distillate IC Engines | 108883 | Toluene | 1.33E-03 |
| 2103004002 | Comm/Inst Distillate IC Engines | 71432 | Benzene | 3.04E-03 |
| 2103004002 | Comm/Inst Distillate IC Engines | 56553 | Benz[a]Anthracene | 5.48E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 53703 | Dibenzo[a,h]Anthracene | 1.90E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 91203 | Naphthalene | 2.77E-04 |
| 2103004002 | Comm/Inst Distillate IC Engines | 50000 | Formaldehyde | 3.85E-03 |
| 2103004002 | Comm/Inst Distillate IC Engines | 218019 | Chrysene | 1.15E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 208968 | Acenaphthylene | 1.65E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 107028 | Acrolein | 3.02E-04 |
| 2103004002 | Comm/Inst Distillate IC Engines | 120127 | Anthracene | 6.10E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 129000 | Pyrene | 1.56E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 1330207 | Xylenes (Mixed Isomers) | 9.30E-04 |
| 2103004002 | Comm/Inst Distillate IC Engines | 191242 | Benzo[g,h,i,]Perylene | 1.60E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 193395 | Indeno[1,2,3-c,d]Pyrene | 1.22E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 205992 | Benzo[b]Fluoranthene | 3.23E-07 |
| 2103004002 | Comm/Inst Distillate IC Engines | 206440 | Fluoranthene | 2.48E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 207089 | Benzo[k]Fluoranthene | 5.06E-07 |
| 2103004002 | Comm/Inst Distillate IC Engines | 106990 | 1,3-Butadiene | 1.28E-04 |
| 2103005000 | Comm/Inst Residual Oil | 129000 | Pyrene | 3.98E-06 |
| 2103005000 | Comm/Inst Residual Oil | 208968 | Acenaphthylene | 2.39E-07 |
| 2103005000 | Comm/Inst Residual Oil | 50000 | Formaldehyde | 3.19E-02 |
| 2103005000 | Comm/Inst Residual Oil | 91203 | Naphthalene | 1.08E-03 |
| 2103005000 | Comm/Inst Residual Oil | 218019 | Chrysene | 2.26E-06 |
| 2103005000 | Comm/Inst Residual Oil | 206440 | Fluoranthene | 4.65E-06 |
| 2103005000 | Comm/Inst Residual Oil | 193395 | Indeno[1,2,3-c,d]Pyrene | 1.99E-06 |
| 2103005000 | Comm/Inst Residual Oil | 191242 | Benzo[g,h,i,]Perylene | 2.12E-06 |
| 2103005000 | Comm/Inst Residual Oil | 120127 | Anthracene | 1.15E-06 |
| 2103005000 | Comm/Inst Residual Oil | 85018 | Phenanthrene | 9.96E-06 |
| 2103005000 | Comm/Inst Residual Oil | 83329 | Acenaphthene | 1.99E-05 |
| 2103005000 | Comm/Inst Residual Oil | 75070 | Acetaldehyde | 4.65E-03 |
| 2103005000 | Comm/Inst Residual Oil | 53703 | Dibenzo[a,h]Anthracene | 1.59E-06 |
| 2103005000 | Comm/Inst Residual Oil | 71432 | Benzene | 1.99E-04 |
| 2103005000 | Comm/Inst Residual Oil | 56553 | Benz[a]Anthracene | 3.85E-06 |
| 2103005000 | Comm/Inst Residual Oil | 86737 | Fluorene | 4.25E-06 |
| 2103005000 | Comm/Inst Residual Oil | 56832736 | Benzofluoranthenes | 1.46E-06 |
| 2103006000 | Comm/Inst Natural Gas | 71432 | Benzene | 3.82E-04 |
| 2103006000 | Comm/Inst Natural Gas | 206440 | Fluoranthene | 5.73E-07 |
| 2103006000 | Comm/Inst Natural Gas | 129000 | Pyrene | 9.55E-07 |
| 2103006000 | Comm/Inst Natural Gas | 91203 | Naphthalene | 1.16E-04 |
| 2103006000 | Comm/Inst Natural Gas | 86737 | Fluorene | 5.35E-07 |
| 2103006000 | Comm/Inst Natural Gas | 75070 | Acetaldehyde | 2.48E-06 |
| 2103006000 | Comm/Inst Natural Gas | 50000 | Formaldehyde | 1.43E-02 |
| 2103006000 | Comm/Inst Natural Gas | 85018 | Phenanthrene | 3.25E-06 |
| 2103007000 | Comm/Inst LPG | 91203 | Naphthalene | 1.07E-04 |
| 2103007000 | Comm/Inst LPG | 129000 | Pyrene | 8.78E-07 |
| 2103007000 | Comm/Inst LPG | 206440 | Fluoranthene | 5.27E-07 |
| 2103007000 | Comm/Inst LPG | 71432 | Benzene | 3.51E-04 |
| 2103007000 | Comm/Inst LPG | 85018 | Phenanthrene | 2.99E-06 |
| 2103007000 | Comm/Inst LPG | 75070 | Acetaldehyde | 2.28E-06 |
| 2103007000 | Comm/Inst LPG | 86737 | Fluorene | 4.92E-07 |
| 2103007000 | Comm/Inst LPG | 50000 | Formaldehyde | 1.32E-02 |
| 2103008000 | Comm/Inst Wood | 203123 | Benzo[ghi]fluoranthene | 5.96E-06 |
| 2103008000 | Comm/Inst Wood | 100414 | Ethylbenzene | 1.67E-03 |
| 2103008000 | Comm/Inst Wood | 106990 | 1,3-butadiene | 8.51E-03 |
| 2103008000 | Comm/Inst Wood | 107028 | Acrolein (2-propenal) | 4.58E-03 |
| 2103008000 | Comm/Inst Wood | 108883 | Toluene | 1.15E-02 |
| 2103008000 | Comm/Inst Wood | 206440 | Fluoranthene | 2.22E-04 |
| 2103008000 | Comm/Inst Wood | 120127 | Anthracene | 2.50E-04 |
| 2103008000 | Comm/Inst Wood | 71432 | Benzene | 2.79E-02 |
| 2103008000 | Comm/Inst Wood | 120809 | 1,2-Benzenediol | 2.27E-02 |
| 2103008000 | Comm/Inst Wood | 123319 | Hydroquinone (or p-benzenediol; 1,4-Benzenediol) | 1.25E-03 |
| 2103008000 | Comm/Inst Wood | 123386 | Propionaldehyde (or Propanal; 1-Propanone; 1-Propanal) | 1.85E-02 |
| 2103008000 | Comm/Inst Wood | 129000 | Pyrene | 1.36E-04 |
| 2103008000 | Comm/Inst Wood | 1319773 | Cresol/Cresylic Acid (Mixed Isomers) | 3.42E-02 |
| 2103008000 | Comm/Inst Wood | 108952 | Phenol (carbolic acid) | 3.82E-02 |
| 2103008000 | Comm/Inst Wood | 208968 | Acenaphthylene | 1.35E-03 |
| 2103008000 | Comm/Inst Wood | 98862 | Acetophenone (or 1-phenylethanone; Methyl phenyl ketone) | 2.84E-04 |
| 2103008000 | Comm/Inst Wood | 50000 | Formaldehyde | 8.47E-02 |
| 2103008000 | Comm/Inst Wood | 75070 | Acetaldehyde | 1.24E-01 |
| 2103008000 | Comm/Inst Wood | 832699 | 1-methylphenanthrene | 1.61E-04 |
| 2103008000 | Comm/Inst Wood | 83329 | Acenaphthene | 1.47E-04 |
| 2103008000 | Comm/Inst Wood | 85018 | Phenanthrene | 1.14E-03 |
| 2103008000 | Comm/Inst Wood | 86737 | Fluorene | 3.23E-04 |
| 2103008000 | Comm/Inst Wood | 90120 | 1-Methylnaphthalene | 7.71E-04 |
| 2103008000 | Comm/Inst Wood | 91203 | Naphthalene | 1.65E-02 |
| 2103008000 | Comm/Inst Wood | 91576 | 2-methylnaphthalene | 1.09E-03 |
| 2103008000 | Comm/Inst Wood | 1330207 | Xylenes (Mixed Isomers) | 5.68E-03 |
| 2103008000 | Comm/Inst Wood | 2531842 | 2-methylphenanthrene | 1.35E-04 |
| 2103011000 | Comm/Inst Kerosene | 85018 | Phenanthrene | 3.07E-05 |
| 2103011000 | Comm/Inst Kerosene | 120127 | Anthracene | 3.56E-06 |
| 2103011000 | Comm/Inst Kerosene | 50000 | Formaldehyde | 9.82E-02 |
| 2103011000 | Comm/Inst Kerosene | 53703 | Dibenzo[a,h]Anthracene | 4.91E-06 |
| 2103011000 | Comm/Inst Kerosene | 56553 | Benz[a]Anthracene | 1.19E-05 |
| 2103011000 | Comm/Inst Kerosene | 71432 | Benzene | 6.14E-04 |
| 2103011000 | Comm/Inst Kerosene | 75070 | Acetaldehyde | 1.43E-02 |
| 2103011000 | Comm/Inst Kerosene | 91203 | Naphthalene | 3.31E-03 |
| 2103011000 | Comm/Inst Kerosene | 86737 | Fluorene | 1.31E-05 |
| 2103011000 | Comm/Inst Kerosene | 56832736 | Benzofluoranthenes | 4.50E-06 |
| 2103011000 | Comm/Inst Kerosene | 129000 | Pyrene | 1.23E-05 |
| 2103011000 | Comm/Inst Kerosene | 191242 | Benzo[g,h,i,]Perylene | 6.55E-06 |
| 2103011000 | Comm/Inst Kerosene | 193395 | Indeno[1,2,3-c,d]Pyrene | 6.14E-06 |
| 2103011000 | Comm/Inst Kerosene | 206440 | Fluoranthene | 1.43E-05 |
| 2103011000 | Comm/Inst Kerosene | 208968 | Acenaphthylene | 7.36E-07 |
| 2103011000 | Comm/Inst Kerosene | 218019 | Chrysene | 6.95E-06 |
| 2103011000 | Comm/Inst Kerosene | 83329 | Acenaphthene | 6.14E-05 |

Note: Factors are developed by dividing the HAP emission factor (from AP-42) by the emission factor for VOC.

Table A-5. Speciation factors for non-VOC HAPs. Factors are multiplied by PM25-PRI emissions to estimate emissions for each HAP.

| SCC | Description | Pollutant Code | Pollutant Description | Speciation Factor |
| --- | --- | --- | --- | --- |
| 2102001000 | Industrial Anthracite Coal | 7440382 | Arsenic | 5.47E-05 |
| 2102001000 | Industrial Anthracite Coal | 7440417 | Beryllium | 2.80E-06 |
| 2102001000 | Industrial Anthracite Coal | 7440439 | Cadmium | 6.81E-06 |
| 2102001000 | Industrial Anthracite Coal | 18540299 | Chromium (VI) | 4.23E-06 |
| 2102001000 | Industrial Anthracite Coal | 16065831 | Chromium III | 3.10E-05 |
| 2102001000 | Industrial Anthracite Coal | 7439965 | Manganese | 6.54E-05 |
| 2102001000 | Industrial Anthracite Coal | 7439976 | Mercury | 1.11E-05 |
| 2102001000 | Industrial Anthracite Coal | 7440020 | Nickel | 3.74E-05 |
| 2102001000 | Industrial Anthracite Coal | 7782492 | Selenium | 1.73E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 7440382 | Arsenic | 1.68E-04 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 7440439 | Cadmium | 2.09E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 18540299 | Chromium (VI) | 1.28E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 16065831 | Chromium III | 9.38E-05 |
| 2102002000 | Industrial Bitum/Sibbitum Coal | 57125 | Cyanide | 1.02E-03 |
| 2102004001 | Industrial Distillate Boilers | 7440382 | Arsenic | 3.55E-04 |
| 2102004001 | Industrial Distillate Boilers | 7440417 | Beryllium | 2.66E-04 |
| 2102004001 | Industrial Distillate Boilers | 7440439 | Cadmium | 2.66E-04 |
| 2102004001 | Industrial Distillate Boilers | 7440473 | Chromium | 2.66E-04 |
| 2102004001 | Industrial Distillate Boilers | 7439965 | Manganese | 5.32E-04 |
| 2102004001 | Industrial Distillate Boilers | 7439976 | Mercury | 2.66E-04 |
| 2102004001 | Industrial Distillate Boilers | 7440020 | Nickel | 2.66E-04 |
| 2102004001 | Industrial Distillate Boilers | 7782492 | Selenium | 1.33E-03 |
| 2102004002 | Industrial Distillate IC Engines | 7439976 | Mercury | 1.02E-06 |
| 2102004002 | Industrial Distillate IC Engines | 7440382 | Arsenic | 3.70E-05 |
| 2102004002 | Industrial Distillate IC Engines | 7440417 | Beryllium | 1.04E-06 |
| 2102004002 | Industrial Distillate IC Engines | 7440439 | Cadmium | 1.62E-05 |
| 2102004002 | Industrial Distillate IC Engines | 7440473 | Chromium | 3.70E-05 |
| 2102004002 | Industrial Distillate IC Engines | 7439965 | Manganese | 2.66E-03 |
| 2102004002 | Industrial Distillate IC Engines | 7440020 | Nickel | 1.55E-05 |
| 2102004002 | Industrial Distillate IC Engines | 7782492 | Selenium | 8.42E-05 |
| 2102005000 | Industrial Residual Oil | 7440382 | Arsenic | 9.40E-05 |
| 2102005000 | Industrial Residual Oil | 7440417 | Beryllium | 2.00E-06 |
| 2102005000 | Industrial Residual Oil | 7440439 | Cadmium | 2.80E-05 |
| 2102005000 | Industrial Residual Oil | 18540299 | Chromium (VI) | 1.08E-05 |
| 2102005000 | Industrial Residual Oil | 16065831 | Chromium III | 4.92E-05 |
| 2102005000 | Industrial Residual Oil | 7439965 | Manganese | 2.10E-04 |
| 2102005000 | Industrial Residual Oil | 7439976 | Mercury | 8.10E-06 |
| 2102005000 | Industrial Residual Oil | 7440020 | Nickel | 6.00E-03 |
| 2102005000 | Industrial Residual Oil | 7782492 | Selenium | 4.90E-05 |
| 2102011000 | Industrial Kerosene | 7440382 | Arsenic | 3.62E-04 |
| 2102011000 | Industrial Kerosene | 7440417 | Beryllium | 2.72E-04 |
| 2102011000 | Industrial Kerosene | 7440439 | Cadmium | 2.72E-04 |
| 2102011000 | Industrial Kerosene | 18540299 | Chromium (VI) | 4.89E-05 |
| 2102011000 | Industrial Kerosene | 16065831 | Chromium III | 2.23E-04 |
| 2102011000 | Industrial Kerosene | 7439965 | Manganese | 5.44E-04 |
| 2102011000 | Industrial Kerosene | 7439976 | Mercury | 2.72E-04 |
| 2102011000 | Industrial Kerosene | 7440020 | Nickel | 2.72E-04 |
| 2102011000 | Industrial Kerosene | 7782492 | Selenium | 1.36E-03 |
| 2103001000 | Comm/Inst Anthracite Coal | 7440382 | Arsenic | 5.47E-05 |
| 2103001000 | Comm/Inst Anthracite Coal | 7440417 | Beryllium | 2.80E-06 |
| 2103001000 | Comm/Inst Anthracite Coal | 7440439 | Cadmium | 6.81E-06 |
| 2103001000 | Comm/Inst Anthracite Coal | 18540299 | Chromium (VI) | 4.23E-06 |
| 2103001000 | Comm/Inst Anthracite Coal | 16065831 | Chromium III | 3.10E-05 |
| 2103001000 | Comm/Inst Anthracite Coal | 7439965 | Manganese | 6.54E-05 |
| 2103001000 | Comm/Inst Anthracite Coal | 7439976 | Mercury | 1.11E-05 |
| 2103001000 | Comm/Inst Anthracite Coal | 7440020 | Nickel | 3.74E-05 |
| 2103001000 | Comm/Inst Anthracite Coal | 7782492 | Selenium | 1.73E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 7440382 | Arsenic | 1.68E-04 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 7440439 | Cadmium | 2.09E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 18540299 | Chromium (VI) | 1.28E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 16065831 | Chromium III | 9.38E-05 |
| 2103002000 | Comm/Inst Bitum/Subbitum Coal | 57125 | Cyanide | 1.02E-03 |
| 2103004001 | Comm/Inst Distillate Boilers | 7440382 | Arsenic | 2.63E-03 |
| 2103004001 | Comm/Inst Distillate Boilers | 7440417 | Beryllium | 1.97E-03 |
| 2103004001 | Comm/Inst Distillate Boilers | 7440439 | Cadmium | 1.97E-03 |
| 2103004001 | Comm/Inst Distillate Boilers | 18540299 | Chromium (VI) | 3.55E-05 |
| 2103004001 | Comm/Inst Distillate Boilers | 16065831 | Chromium (III) | 1.62E-04 |
| 2103004001 | Comm/Inst Distillate Boilers | 7439965 | Manganese | 3.94E-03 |
| 2103004001 | Comm/Inst Distillate Boilers | 7439976 | Mercury | 1.97E-04 |
| 2103004001 | Comm/Inst Distillate Boilers | 7440020 | Nickel | 1.97E-04 |
| 2103004001 | Comm/Inst Distillate Boilers | 7782492 | Selenium | 9.86E-04 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7440382 | Arsenic | 3.47E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7440417 | Beryllium | 7.13E-09 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7440439 | Cadmium | 1.52E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7440473 | Chromium | 3.47E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7439965 | Manganese | 2.49E-03 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7439976 | Mercury | 3.79E-06 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7440020 | Nickel | 1.45E-05 |
| 2103004002 | Comm/Inst Distillate IC Engines | 7782492 | Selenium | 7.90E-05 |
| 2103005000 | Comm/Inst Residual Oil | 7440382 | Arsenic | 2.00E-04 |
| 2103005000 | Comm/Inst Residual Oil | 7440417 | Beryllium | 4.26E-06 |
| 2103005000 | Comm/Inst Residual Oil | 7440439 | Cadmium | 5.96E-05 |
| 2103005000 | Comm/Inst Residual Oil | 18540299 | Chromium (VI) | 2.30E-05 |
| 2103005000 | Comm/Inst Residual Oil | 16065831 | Chromium III | 1.05E-04 |
| 2103005000 | Comm/Inst Residual Oil | 7439965 | Manganese | 4.47E-04 |
| 2103005000 | Comm/Inst Residual Oil | 7439976 | Mercury | 1.72E-05 |
| 2103005000 | Comm/Inst Residual Oil | 7440020 | Nickel | 1.28E-02 |
| 2103005000 | Comm/Inst Residual Oil | 7782492 | Selenium | 1.04E-04 |
| 2103011000 | Comm/Inst Kerosene | 7440382 | Arsenic | 2.57E-04 |
| 2103011000 | Comm/Inst Kerosene | 7440417 | Beryllium | 1.93E-04 |
| 2103011000 | Comm/Inst Kerosene | 7440439 | Cadmium | 1.93E-04 |
| 2103011000 | Comm/Inst Kerosene | 18540299 | Chromium (VI) | 3.47E-05 |
| 2103011000 | Comm/Inst Kerosene | 16065831 | Chromium III | 1.58E-04 |
| 2103011000 | Comm/Inst Kerosene | 7439965 | Manganese | 3.86E-04 |
| 2103011000 | Comm/Inst Kerosene | 7439976 | Mercury | 1.93E-04 |
| 2103011000 | Comm/Inst Kerosene | 7440020 | Nickel | 1.93E-04 |
| 2103011000 | Comm/Inst Kerosene | 7782492 | Selenium | 9.64E-04 |

Note: Factors are developed by dividing the HAP emission factor (from AP-42) by the emission factor for PM25-PRI.

For natural gas, for example – EIA-176 “Annual Report of Natural and Supplemental Gas Supply and Disposition.”

County-level federal and state government employment data are not available from the Bureau of the Census.