MEMORANDUM

TO:		Roy Huntley, US EPA

Jennifer Snyder, US EPA

FROM:	Mike Pring, ERG

Regi Oommen, ERG

DATE:	November 21, 2014

SUBJECT:	Updates to default factors in the EPA OAQPS Nonpoint Oil and Gas Emissions Estimation Tool

The purpose of this memo is to summarize changes made to the EPA Office of Air Quality Planning and Standards (OAQPS) Nonpoint Oil and Gas Emission Estimation Tool (tool) activity and emission factor data as a result of inclusion of data from the EPA Office of Atmospheric Programs (OAP) Inventory of Greenhouse Gas Emissions and Sinks (U.S. GHG Inventory) for the Natural Gas and Petroleum Systems sectors, and Subpart W of the Greenhouse Gas Reporting Rule.

Where available, the tool currently utilizes activity and emission factor data directly from state and local agencies to calculate emissions.  Where state input data are not available, nationally-averaged default data is used.  In most cases, this data was derived from work done in 2012 by the Central States Air Resource Agencies (CenSARA) to develop an improved area source (nonpoint) oil and gas emissions inventory for their region.  In order to supplement this data and provide more nationally averaged data, ERG and EPA analyzed information used in the U.S. GHG Inventory and Subpart W of the Greenhouse Gas Reporting Rule to ascertain if any additional sources of input data could be used in the tool.  As a result of this analysis, changes were made to input data used to estimate emissions for several source categories.  Specifically, changes have been made to the following source categories:

- Condensate Tanks;

- Liquids Unloading;

- Pneumatic Devices; and

- Well Completions.

A description of the revisions made to the input data for each of these categories is provided below.  Note that these changes have only been implemented for counties not covered under the CenSARA inventory efforts, or for those counties where states have not provided updated activity and emission factors to EPA.

Condensate Tanks

The following control parameters used in the U.S. GHG Inventory have been used in the tool as default data to estimate emissions from condensate tanks:

- 50% of condensate tanks are controlled; and

- For controlled tanks, the control efficiency is 80% (represents the combined overall control efficiency, combining capture and control).

Liquids Unloading

The U.S. GHG Inventory was recently updated to reflect current information on emissions from liquids unloading.  Specifically, EPA analyzed a report issued in September of 2012 by the American Petroleum Institute (API) and America’s Natural Gas Alliance (ANGA) entitled “Characterizing Pivotal Sources of Methane Emissions from Natural Gas Production”.  Using data presented in the report, EPA developed updated vent rates for liquids unloading activities based on U.S. Energy Information Administration (EIA) Supply Regions.  Figure 1 below shows the six EIA Supply Regions used in the U.S. GHG Inventory.

Figure 1. EIA Supply Region Map

Table 1 below shows the vent rates by EIA Supply Region for each venting scenario used in the U.S. GHG Inventory.

Table 1. Liquids Unloading Vent Rates from the U.S. GHG Inventory

| EIA Supply Region | Wells venting with plunger lift (%) | Wells venting without plunger lift (%) | Vent Rate for Wells with Plunger Lift (scf/yr/well) a | Vent Rate for Wells without Plunger Lift (scf/yr/well) a |
| --- | --- | --- | --- | --- |
| North East | 4.3 | 11.26 | 314,626 | 166,174 |
| Midcontinent | 2.33 | 4.14 | 1,379,958 | 230,199 |
| Rocky Mountain | 12.88 | 1.52 | 154,300 | 2,579,444 |
| South West | 3.32 | 19.47 | 3,547 | 96,748 |
| West Coast | 7.59 | 6.80 | 345,343 | 304,048 |
| Gulf Coast | 2.32 | 7.08 | 70,021 | 300,592 |

a Whole gas vent rates.

In order to utilize this information under the current structure and methodology used in the tool, a weighted vent rate is required for all wells in a county.  This is needed as the tool does not contain distinct counts of wells with plunger lifts, and wells without.  Calculation of a weighted vent rate is accomplished using the data in Table 1.  For example, the updated default liquids unloading vent rate for the North East EIA Supply Region is calculated as follows:

= 32,421 (scf/yr/well)

Table 2 shows the resultant updated default vent rates used in the tool (data from the West Coast Region has been used for the State of Alaska). As there are annual vent rates, where this information is used in the tool, the frequency of liquids unloading venting has been set equal to one event per year.  Additionally, as these rates reflect some level of control (through the use of plunger lifts), where this information is used in the tool, a value of “NA” is used for the control method, and no additional reduction from use of controls has been applied.  See the tool documentation for detailed information on the methodology used to estimate emissions for liquids unloading.

Table 2. Default Liquids Unloading Vent Rates for the Tool

| EIA Supply Region | Gas Well Count | Default Vent Rate for all Wells (scf/yr/well) a |
| --- | --- | --- |
| North East | 153,773 | 32,421 |
| Midcontinent | 87,193 | 41,659 |
| Rocky Mountain | 58,285 | 59,047 |
| South West | 41,919 | 18,956 |
| West Coast | 1,516 | 46,884 |
| Gulf Coast | 71,629 | 22,913 |

a Whole gas vent rates.

Pneumatic Devices

Subpart W of the Greenhouse Gas Reporting Rule prescribes bleed rates for low bleed, high bleed, and intermittent bleed devices that are to be used by reporters to estimate emissions.  These rates, shown in Table 3 below, have been incorporated into the tool as default bleed rates for pneumatic devices used at oil and gas wells.

Table 3. Whole Gas Bleed Rates for Pneumatic Devices from Subpart W of the Greenhouse Gas Reporting Rule

| Onshore petroleum and natural gas production | Bleed Rate (scf/hour/component) |
| --- | --- |
| Low Bleed Pneumatic Devices | 1.39 |
| High Bleed Pneumatic Devices | 37.3 |
| Intermittent Bleed Pneumatic Devices | 13.5 |

In addition to updated bleed rates, the U.S. GHG Inventory utilizes per-well pneumatic device  counts that may be used in the tool.  For gas wells, the total device counts in the U.S. GHG Inventory were used to derive default device counts by device type to be used in the tool, based on the total number of devices from the U.S. GHG Inventory and a distribution between low, intermittent, and high bleed devices from the CenSARA inventory and survey effort.

The updated default device counts are shown in Table 4 below for each EIA Supply Region. (Note that for oil wells, the total device counts by device type will be updated in future inventories as EPA has identified an error for the oil well device counts shown in Table 4.)

Table 4. Pneumatic Device Counts for Oil and Gas Wells from the U.S. GHG Inventory

| EIA Supply Region | Oil Well Device Counts |  |  | Gas Well Device Counts |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Bleed | High Bleed | Intermittent Bleed | Low Bleed | High Bleed | Intermittent Bleed |
| North East | 0.762 | 0.762 | 0 | 0.144 | 0.222 | 0.120 |
| Midcontinent | 0.762 | 0.762 | 0 | 0.460 | 0.709 | 0.382 |
| Rocky Mountain | 0.762 | 0.762 | 0 | 0.434 | 0.669 | 0.360 |
| South West | 0.762 | 0.762 | 0 | 0.394 | 0.607 | 0.327 |
| West Coast | 0.762 | 0.762 | 0 | 0.297 | 0.458 | 0.247 |
| Gulf Coast | 0.762 | 0.762 | 0 | 0.206 | 0.318 | 0.171 |

Well Completions

Limited data is available in the literature for estimating emissions from oil well completions.  At this time, and until such information becomes available, EPA has determined not to develop default estimate emissions from oil well completions in the tool.  However, where state data on oil well completion emissions has been provided, this data has been used in the tool.

Emissions from unconventional gas well completions are currently estimated using a default vent rate of 9,000 (MCF/completion).  This data has not been revised.  However, the default vent rate for conventional gas well completions has been updated to reflect an uncontrolled vent rate of 1,884 (ft3/completion).  This factor is based on a controlled (flared) vent rate of 733 (ft3/completion) used in the U.S. GHG Inventory, back-calculated to reflect the following default control assumptions used in the tool:

- 28.43% of completions are flared;

- 98% flare control efficiency;

- 90% flare capture efficiency; and

- 36% of completions are “Green Completions” and have no emissions.

Therefore, the default uncontrolled vent rate is:

= 1,884 (ft3/completion)