# AGRICULTURAL PESTICIDES

# Source Category Description

Pesticides are substances used to control nuisance species and can be classified by targeted pest group: weeds (herbicides), insects (insecticides), fungi (fungicides), and rodents (rodenticides). They can be further described by their chemical characteristics: synthetics, non-synthetics (petroleum products), and inorganics. Different pesticides are made through various combinations of the pest-killing material, also called the active ingredient, and various solvents (which serve as carriers for the active ingredient). Both types of ingredients contain volatile organic compounds (VOC) that may be emitted to the air during application or after application as a result of evaporation.

Approximately 68 to 75 percent of pesticides used in the United States are applied to agricultural lands, both cropland and pasture. Agricultural pesticides continue to be a cost-effective means of controlling weeds, insects, and other threats to the quality and yield of food production. Since application rates for a particular pesticide may vary from region to region, the regional application rates should be considered when estimating potential VOC emissions. In 2014, agricultural pesticides resulted in approximately 205,448.4 tons of VOC emissions.

For this source category, the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2461850000 | Solvent Utilization | Miscellaneous Non-industrial: Commercial | Pesticide Application: Agricultural | All Processes |

# Overview of Calculations

The USGS provides county-level estimates of pesticide application, in its preliminary county-level pesticide use estimates. These data provide information about the total application of each active ingredient in a pesticide product (e.g. 2,4-D, atrazine, captan). There are often many different pesticide products with the same active ingredient. For example, the California Department of Pesticide Regulation’s (CA DPR) database lists 49 registered pesticide products with atrazine as the active ingredient, each with slightly different formulations, including different proportions of active ingredient and solvents. The CA DPR database includes information on the mass fraction of active ingredient in each pesticide product. EPA uses this information to calculate an average VOC emissions factor for each active ingredient listed in the CA DPR database. This VOC emissions factor is multiplied by the amount of active ingredient applied in each county, from the USGS report, to estimate VOC emissions in each county. For active ingredients not in the CA DPR database, a weighted emissions factor is calculated by weighting the emissions factors from the CA DPR database with total pounds of active ingredient reported in the USGS report. HAP emissions are calculated by multiplying the total pounds of active ingredients applied in each county by an emissions factor. Sources of data and calculations for the total pounds of active ingredients applied per pesticide are discussed in section C. Emissions factors calculations are discussed in section E. The estimation of emissions from agricultural pesticides is discussed in section G.

# Activity Data

The activity for pesticide application is the pounds of active ingredient applied per pesticide at the county level for the years 2016 and 2017, from the USGS preliminary county-level pesticide use estimates2 which gives county-level pesticide data in terms of kg of active ingredient applied. The data estimate preliminary annual county-level pesticide use for 387 herbicides, insecticides, and fungicides applied to agricultural crops grown in the conterminous United States during 2016 and 2017. For all states except California, pesticide-use data are compiled from proprietary surveys of farm operations located within U.S. Department of Agriculture Crop Reporting Districts (CRDs). Surveyed pesticide-use data are used in conjunction with county annual harvested-crop acres reported by the U.S. Department of Agriculture 2012 Censuses of Agriculture and the 2013 County Agricultural Production Survey to calculate use rates per harvested-crop acre, or an “estimated pesticide use” (EPest) rate, for each crop by year. County-use estimates are then calculated by USGS by multiplying EPest rates by harvested-crop acres for each pesticide crop combination. Use estimates for California in the USGS data are obtained from annual CA DPR Pesticide Use Reports.

The USGS report calculates both EPest-low and EPest-high rates. The EPest-high rates are used here to estimate VOC emissions. Both methods incorporated surveyed and extrapolated rates to estimate pesticide use for counties, but EPest-low and EPest-high estimations differ in how situations are treated when a CRD was surveyed and pesticide use was not reported for a particular pesticide-by-crop combination. If use of a pesticide on a crop is not reported in a surveyed CRD, EPest-low reports zero use in the CRD for that pesticide-by-crop combination. EPest-high, however, treats the unreported use for that pesticide-by-crop combination in the CRD as unsurveyed, and pesticide-by-crop use rates from neighboring CRDs and, in some cases, CRDs within the same Farm Resources Region are used to calculate the pesticide-by-crop EPest-high rate for that CRD.

Due to data limitations, the USGS report does not contain active ingredient usages for Alaska and Hawaii .However, the Census of Agriculture contains acres treated with pesticide by county for Alaska and Hawaii and these values are used to estimate emissions.

# Allocation Procedure

The activity data are reported at the county level and do not need to be allocated.

# Emissions Factors

The VOC emissions factors are derived for each active ingredient based on the pesticide profiles database maintained by the CA DPR.2 This database contains the chemical formulation for pesticide products registered in the State of California and provides key inputs for the development of VOC emissions factors, including the mass fraction of the active ingredient and the emission potential (EP) of registered pesticide products. The EP value represents the VOC content of the pesticide product and it is determined empirically through thermogravimetric analysis. Since the CA DPR database lists both agricultural and non-agricultural pesticide products, it is necessary to screen out entries that were likely formulated as a consumer product. Pesticide products that contained terms suggesting non-agricultural applications are excluded. Terms used to screen out likely consumer products are listed in Table 1.

Table 1. Terms Used to Screen Out Consumer Products

| ALGAE | DEODORIZING | GERM | MRSA | STAIN |
| --- | --- | --- | --- | --- |
| ANT | DETERGENT | HAMSTER | ORNAMENTAL | SWIM |
| BATHROOM | DISHWASHER | HOME | POND | TICK |
| BEDBUG | DISINFECT | HORNET | POTTY | TURF |
| BEE | DOG | HORSE | PRESCRIPTION | WASP |
| CAT | DRAIN | HOUSE | RAT | WIPES |
| CATTLE | EQUINE | INDOOR | ROACH | YARD |
| CLEANER | FLEA | KLEEN | RODENTICIDE |  |
| DECK | FLY | LANDSCAPE | ROOF |  |
| DEGREASER | FOGGER | LAWN | SANI |  |
| DEODORIZER | GERBIL | MOUSE | SPA |  |

Each record in the CA DPR database is for a specific pesticide product, and provides the product name, primary active ingredient, mass percent of active ingredient, emission potential (EP), registration number, and method used to estimate the EP. The pesticide-specific EP of reactive organic gases (i.e., the mass percentage of product that contributes to VOC emissions) and the mass percent of active ingredient are used to calculate pesticide-specific VOC emissions factors.

The CA DPR emission potential database2 provides the pesticide-specific emissions potential of reactive organic gases (i.e., the mass percentage of each pesticide product that contributes to VOC emissions) and the mass percent of active ingredient. To determine the total amount of pesticide product applied (i.e. both the active ingredient and the solvent) the amount of active ingredient applied (from USGS data)3  is divided by the mass percent of active ingredient, which is divided by 100 to convert from percent to fraction.

| TPpest,US=AIpest,USMPpest,US100 |  |
| --- | --- |

Where:

TPpest,US	= 	Total pesticide applied for each active ingredient in the United States, in lbs.

AIpest,US	=	Total active ingredient applied of each pesticide type in the United States, in lbs.

MPpest,US	=	Average mass percent of active ingredient in each pesticide type in the United States, in percent

Next, the total national-level VOC emissions from each pesticide type are estimated by multiplying the total pesticide applied by the pesticide-specific emissions potential of reactive organic gases (i.e., the mass percentage of each pesticide that contributes to VOC emissions), from the CA DPR database.2

| EVOC,US,pest=TPpest,US×EProg,pest100 |  |
| --- | --- |

Where:

EVOC,US,pest	= 	Total national-level VOC emissions for each pesticide type, in lbs.

TPpest,US	= 	Total pesticide applied of each pesticide type in the United States, in lbs.

EProg,pest	=	Emissions potential of reactive organic gases for each pesticide, expressed as % of pesticide mass

The VOC emissions factor for each pesticide type is calculated by dividing the total national-level VOC emissions for each pesticide type by the total active ingredient applied for each pesticide type.

| EFpest=EVOC,US,pestAIpest,US |  |
| --- | --- |

Where:

EFpest		= 	  Pesticide-specific emissions factor, in pounds VOC / pound active ingredient

EVOC,US,pest	=   Total national-level VOC emissions for each pesticide type, in lbs.

AIpest,US		=	  Total active ingredient applied of each pesticide type in the United States, in lbs.

For active ingredients not in the CA DPR database, a weighted average emissions factor (EFavg) is calculated. This weighted average is estimated by weighting the emissions factors from the CA DPR database using the total pounds of active ingredient reported in the USGS report “Preliminary Estimates of Annual Agricultural Pesticide Use for Counties of the Conterminous United States, 2013.”3 A crosswalk between compound name in the USGS database and the chemical name in the CA DPR database is provided in Table 4. Note that any pesticide compound from the USGS database that is not in the CA DPR data are marked with the word “AVERAGE,” to denote that the weighted average VOC emissions factors of 0.4 pounds of VOC per pound of active ingredient is used to estimate VOC emissions for the application of that pesticide. The pesticide-specific VOC emissions factors for all pesticides from the CA DPR database are shown in Table 5.

The emissions factor is calculated for these active ingredients based on calculating a weighted average emissions factor for all active ingredients. The weights are determined by dividing the active ingredient applied for each pesticide type by the total active ingredients applied for all pesticides types. These weights are multiplied by the emissions factor for each pesticide (calculated in step 3), and then summed to determine the weighted average emissions factor.

| EFavg=pest=1PESTAIpest,USpest=1PESTAIpest,US×EFpest |  |
| --- | --- |

Where:

EFavg	=	Weighted average emissions factor, in pounds VOC / pound active ingredient

AIpest,US	=	Total active ingredient applied of each pesticide type in the United States, in lbs.

EFpest	=   Pesticide-specific emissions factor, in pounds VOC / pound active ingredient

The HAP emissions factors are from EIIP and are based on vapor pressure of the active ingredient.1 Compounds with a vapor pressure between 1 × 10-4 and 1 × 10-6 mm Hg at 20°C to 25°C have an emissions factor of 700 lbs./ton (or 0.35 lbs./lb.). Compounds with a vapor pressure greater than 1 × 10-4 mm Hg at 20°C to 25°C have an emissions factor of 1,160 lbs./ton (or 0.58 lbs./lb.). The subset of HAPs is extracted from the list of active ingredients and is shown in Table 2 along with the HAP emissions factors. If the calculated emissions factor for any HAP is greater than the VOC emissions factor for that active ingredient, calculated in step 3 of section B, then the HAP emissions factor is set equal to the VOC emissions factor.

Table 2. HAP Emissions Factors

| Compound | Pollutant Code | Vapor Pressure (mm Hg at 20°C to 25°C) | Emissions Factor (lbs. per lb. active ingredient) | Source |
| --- | --- | --- | --- | --- |
| 2,4-D | 94757 | 8 × 10-6 | 0.35 | Reference 1, Tables 9.4-2 and 9.4-4 |
| CAPTAN | 133062 | 8 × 10-8 | 0.1441 | Set equal to VOC emissions factor calculated from the CA DPR. See Table 4 of this document. |
| CARBARYL | 63252 | 1.2 × 10-6 | 0.3208 | Set equal to VOC emissions factor calculated from the CA DPR. See Table 4 of this document. |
| METHYL BROMIDE | 74839 | 1,420 | 0.58 | Vapor pressure: Reference Emissions factor: Reference 1, Table 9.4-4 |
| METHYL IODIDE | 74884 | 400 | 0.58 | Vapor pressure: Reference 4 Emissions factor: Reference 1, Table 9.4-4 |
| PARATHION | 56382 | 5 × 10-6 | 0.35 | Reference 1, Tables 9.4-2 and 9.4-4 |
| TRIFLURALIN | 1582098 | 1.1 × 10-4 | 0.58 | Reference 1, Tables 9.4-2 and 9.4-4 |

For Alaska and Hawaii, data from the conterminous United States is used to develop average emissions factors by pollutant in terms of emissions per acre treated with pesticides. This is calculated by summing the total emissions by pollutant for the conterminous United States and dividing by the total acres treated in the conterminous United States.

# Controls

There are no controls assumed for this category.

# Emissions

VOC and HAP emissions are calculated by multiplying the amount of active ingredient applied in each county, from the USGS database, by the appropriate emissions factor. The emissions factor for VOC is calculated using equations 1-4. The emissions factors for the HAPs are listed in Table 2.

The VOC emissions are calculated by multiplying the active ingredients applied in each county per year by the corresponding emissions factor.

| EVOC,c=AIpest, c ×EFpest×1 ton2000 lb |  |
| --- | --- |

Where:

EVOC,c	= 	Annual emissions of VOC from pesticide active ingredient applications in county c, in tons

AIpest,c	= 	Active ingredient of each pesticide type applied in county c, in pounds

EFpest	= 	Pesticide-specific emissions factor, in pounds VOC / pound active ingredient

Note that if the active ingredient (AIpest) is included in the CA DPR database, then the pesticide-specific emissions factor is used (EFpest); for all other active ingredients, the weighted average emissions factor is used (EFavg).

The HAP emissions are calculated by multiplying the active ingredients applied in each county per year by the corresponding emissions factor. The HAPs listed in Table 2 correspond to the active ingredients in the USGS database. For example, emissions of the HAP captan only occur from applications of the active ingredient captan. Emissions are then summed across pesticide types to estimate the total county-level emissions for each HAP.

| Ep,c= pest=1PESTAIpest,c×EFp,pest |  |
| --- | --- |

Where:

Ep,c	= 	Emissions of pollutant p from pesticide applications in county c, in lbs.

EFp,pest	= 	Emissions factor for pollutant p, in pounds emissions / pound active ingredient

AIpest,c	= 	Active ingredient of each pesticide type applied in county c, in pounds

Note that the HAP emissions factors are from the EIIP.1 If the HAP emissions factor for a certain pesticide type exceeds the VOC emissions factor calculated for that pesticide type as calculated in equations 1 and 2, then the HAP emissions factor is set equal to the VOC emissions factor.

For Alaska and Hawaii, emissions are estimated by multiplying the acres treated with pesticides by pollutant-specific emissions per acre emissions factors.

# Point Source Subtraction

Point source subtraction is not performed for this category.

# Sample Calculations

Table 3 lists sample calculations to determine the VOC and 2,4-D emissions from 2,4-Dichlorophenoxy Acetic Acid (2,4-D). The sample calculations show the emission calculations for the pesticide 2,4-D only. To estimate the total county-level emissions, the process would need to be repeated for each pesticide.

Table 3. Sample calculations for VOC/HAP emissions from 2,4-D agricultural pesticide application in Autauga County, AL

| Eq. # | Equation | Values for Autauga County, AL | Result |
| --- | --- | --- | --- |
| 1 | TPpest,US=AIpest,USMPpest,US100 | 41,912,210 lbs 2,4-D active ingredient4.84 mass percent100 | 865,954,752 lbs total 2,4-D pesticide applied in the United States |
| 2 | EVOC,US,pest=TPpest,US×EProg,pest100 | 865,954,752 lbs total 2,4-D pesticide ×4.0100 | 34,638,190 lbs VOC emissions from 2,4-D in the United States |
| 3 | EFpest=EVOC,US,pestAIpest,US | 34,638,190 lbs VOC41,912,210 lbs 2,4-D active ingredient | 0.826 lbs. VOC/ lb. 2,4-D active ingredient |
| 4 | EFavg=pest=1PESTAIpestpest=1PESTAIpest×EFpest | N/A | This calculation is not needed, as 2,4-D is included in the CA DPR database. |
| 5 | EVOC,c=AIpest, c ×EFpest×1 ton2000 lb | 8020 lbs. 2,4-D active ingredient×0.826 ×1 ton2000 lb | 3.31 tons VOC emissions from 2,4-D in Autauga County, AL |
| 6 | Ep,c= pest=1PESTAIpest,c×EFp,pest | 8020 lbs. 2,4-D active ingredient×0.35 | 2,807 pounds 2,4-D emissions from 2,4-D in Autauga County, AL |

# Changes from 2014 Methodology

As discussed in section C above, EPA developed an emissions estimation methodology for Alaska and Hawaii counties that is not used for the 2014 v2 NEI.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Since insufficient data exist to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emissions factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emissions factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

Table 4. Crosswalk between USGS Compound Name and CA DPR Chemical Name

| USGS Compound Name | CA DPR Compound Name |
| --- | --- |
| 2,4-D | 2,4-D |
| 2,4-DB | 2,4-DB ACID |
| 6-BENZYLADENINE | AVERAGE |
| ABAMECTIN | ABAMECTIN |
| ACEPHATE | ACEPHATE |
| ACEQUINOCYL | ACEQUINOCYL |
| ACETAMIPRID | ACETAMIPRID |
| ACETOCHLOR | AVERAGE |
| ACIBENZOLAR | ACIBENZOLAR-S-METHYL |
| ACIFLUORFEN | ACIFLUORFEN, SODIUM SALT |
| ALACHLOR | ALACHLOR |
| ALDICARB | ALDICARB |
| ALUMINUM PHOSPHIDE | ALUMINUM PHOSPHIDE |
| AMECTOCTRADIN | AMETOCTRADIN |
| AMETRYN | AMETRYNE |
| AMINOPYRALID | AMINOPYRALID, TRIISOPROPANOLAMINE SALT |
| ASULAM | ASULAM, SODIUM SALT |
| ATRAZINE | ATRAZINE |
| AVIGLYCINE | AVERAGE |
| AZADIRACHTIN | AZADIRACHTIN |
| AZINPHOS-METHYL | AZINPHOS-METHYL |
| AZOXYSTROBIN | AZOXYSTROBIN |
| BACILLUS AMYLOLIQUIFACIEN | BACILLUS AMYLOLIQUEFACIENS STRAIN D747 |
| BACILLUS CEREUS | BACILLUS CEREUS, STRAIN BP01 |
| BACILLUS FIRMUS | BACILLUS FIRMUS (STRAIN I-1582) |
| BACILLUS PUMILIS | BACILLUS PUMILUS GHA 180 |
| BACILLUS SUBTILIS | BACILLUS SUBTILIS GB03 |
| BACILLUS THURINGIENSIS | BACILLUS THURINGIENSIS (BERLINER) |
| BENFLURALIN | AVERAGE |
| BENOMYL | BENOMYL |
| BENSULFURON | BENSULFURON METHYL |
| BENSULIDE | BENSULIDE |
| BENTAZONE | BENTAZON, SODIUM SALT |
| BIFENAZATE | BIFENAZATE |
| BIFENTHRIN | BIFENTHRIN |
| BISPYRIBAC | BISPYRIBAC-SODIUM |
| BOSCALID | BOSCALID |
| BROMACIL | BROMACIL |
| BROMOXYNIL | BROMOXYNIL BUTYRATE |
| BUPROFEZIN | BUPROFEZIN |
| BUTRALIN | AVERAGE |
| CALCIUM POLYSULFIDE | AVERAGE |
| CAPTAN | CAPTAN |
| CARBARYL | CARBARYL |
| CARBOPHENOTHION | CARBOPHENOTHION |
| CARBOXIN | CARBOXIN |
| CARFENTRAZONE-ETHYL | CARFENTRAZONE-ETHYL |
| CHINOMETHIONAT | AVERAGE |
| CHLORANTRANILIPROLE | CHLORANTRANILIPROLE |
| CHLORETHOXYFOS | AVERAGE |
| CHLORFENAPYR | CHLORFENAPYR |
| CHLORIMURON | AVERAGE |
| CHLORMEQUAT | CHLORMEQUAT CHLORIDE |
| CHLORONEB | CHLORONEB |
| CHLOROPICRIN | CHLOROPICRIN |
| CHLOROTHALONIL | CHLOROTHALONIL |
| CHLORPROPHAM | CHLORPROPHAM |
| CHLORPYRIFOS | CHLORPYRIFOS |
| CHLORSULFURON | CHLORSULFURON |
| CLETHODIM | CLETHODIM |
| CLODINAFOP | AVERAGE |
| CLOFENTEZINE | CLOFENTEZINE |
| CLOMAZONE | CLOMAZONE |
| CLOPYRALID | CLOPYRALID |
| CLORANSULAM-METHYL | AVERAGE |
| CLOTHIANIDIN | CLOTHIANIDIN |
| CONIOTHYRIUM MINITANS | CONIOTHYRIUM MINITANS STRAIN CON/M/91-08 |
| COPPER | COPPER |
| COPPER HYDROXIDE | COPPER HYDROXIDE |
| COPPER OCTANOATE | COPPER OCTANOATE |
| COPPER OXYCHLORIDE | COPPER OXYCHLORIDE |
| COPPER OXYCHLORIDE S | COPPER OXYCHLORIDE SULFATE |
| COPPER SULF TRIBASIC | COPPER SULFATE (BASIC) |
| COPPER SULFATE | COPPER SULFATE (PENTAHYDRATE) |
| CPPU | AVERAGE |
| CRYOLITE | CRYOLITE |
| CUPROUS OXIDE | COPPER OXIDE (OUS) |
| CYANAMIDE | AVERAGE |
| CYAZOFAMID | CYAZOFAMID |
| CYCLANILIDE | CYCLANILIDE |
| CYCLOATE | CYCLOATE |
| CYDIA POMONELLA | AVERAGE |
| CYFLUFENAMID | CYFLUFENAMID |
| CYFLUTHRIN | CYFLUTHRIN |
| CYHALOFOP | CYHALOFOP-BUTYL |
| CYHALOTHRIN-GAMMA | AVERAGE |
| CYHALOTHRIN-LAMBDA | AVERAGE |
| CYMOXANIL | CYMOXANIL |
| CYPERMETHRIN | CYPERMETHRIN |
| CYPROCONAZOLE | AVERAGE |
| CYPRODINIL | CYPRODINIL |
| CYROMAZINE | CYROMAZINE |
| CYTOKININ | CYTOKININ |
| DAMINOZIDE | DAMINOZIDE |
| DAZOMET | DAZOMET |
| DCPA | AVERAGE |
| DECAN-1-OL | AVERAGE |
| DELTAMETHRIN | DELTAMETHRIN |
| DESMEDIPHAM | DESMEDIPHAM |
| DIAZINON | DIAZINON |
| DICAMBA | DICAMBA |
| DICHLOBENIL | DICHLOBENIL |
| DICHLOROPROPENE | AVERAGE |
| DICHLORPROP | DICHLORPROP, BUTOXYETHANOL ESTER |
| DICLOFOP | DICLOFOP-METHYL |
| DICLORAN | DICLORAN |
| DICLOSULAM | AVERAGE |
| DICOFOL | DICOFOL |
| DICROTOPHOS | DICROTOPHOS |
| DIENOCHLOR | DIENOCHLOR |
| DIETHATYL | DIETHATYL-ETHYL |
| DIFENOCONAZOLE | DIFENOCONAZOLE |
| DIFLUBENZURON | DIFLUBENZURON |
| DIFLUFENZOPYR | DIFLUBENZURON |
| DIMETHENAMID | DIMETHENAMID-P |
| DIMETHENAMID-P | DIMETHENAMID-P |
| DIMETHIPIN | DIMETHIPIN |
| DIMETHOATE | DIMETHOATE |
| DIMETHOMORPH | DIMETHOMORPH |
| DIMETHYL DISULFIDE | AVERAGE |
| DINOSEB | DINOSEB |
| DINOTEFURAN | DINOTEFURAN |
| DIQUAT | DIQUAT DIBROMIDE |
| DISULFOTON | DISULFOTON |
| DITHIOPYR | DITHIOPYR |
| DIURON | DIURON |
| DODINE | DODINE |
| EMAMECTIN | EMAMECTIN BENZOATE |
| ENDOSULFAN | ENDOSULFAN |
| ENDOTHAL | ENDOTHALL, DISODIUM SALT |
| EPTC | EPTC |
| ESFENVALERATE | ESFENVALERATE |
| ETHALFLURALIN | ETHALFLURALIN |
| ETHEPHON | ETHEPHON |
| ETHION | ETHION |
| ETHOFUMESATE | ETHOFUMESATE |
| ETHOPROPHOS | ETHOPROP |
| ETOXAZOLE | ETOXAZOLE |
| ETRIDIAZOLE | AVERAGE |
| FAMOXADONE | AVERAGE |
| FATTY ALCOHOLS | AVERAGE |
| FENAMIDONE | FENAMIDONE |
| FENAMIPHOS | FENAMIPHOS |
| FENARIMOL | FENARIMOL |
| FENBUCONAZOLE | FENBUCONAZOLE |
| FENBUTATIN OXIDE | FENBUTATIN-OXIDE |
| FENHEXAMID | FENHEXAMID |
| FENOXAPROP | FENOXAPROP-ETHYL |
| FENOXYCARB | FENOXYCARB |
| FENPROPATHRIN | FENPROPATHRIN |
| FENPYROXIMATE | FENPYROXIMATE |
| FENTIN | FENTIN HYDROXIDE |
| FERBAM | FERBAM |
| FIPRONIL | FIPRONIL |
| FLAZASULFURON | FLAZASULFURON |
| FLONICAMID | FLONICAMID |
| FLORASULAM | FLORASULAM |
| FLUAZIFOP | FLUAZIFOP-BUTYL |
| FLUAZINAM | FLUAZINAM |
| FLUBENDIAMIDE | FLUBENDIAMIDE |
| FLUCARBAZONE | AVERAGE |
| FLUDIOXONIL | FLUDIOXONIL |
| FLUFENACET | AVERAGE |
| FLUMETRALIN | FLUOMETURON |
| FLUMETSULAM | AVERAGE |
| FLUMICLORAC | FLUMICLORAC-PENTYL |
| FLUMIOXAZIN | FLUMIOXAZIN |
| FLUOMETURON | FLUOMETURON |
| FLUOPICOLIDE | FLUOPICOLIDE |
| FLUOPYRAM | FLUOPYRAM |
| FLUOXASTROBIN | FLUOXASTROBIN |
| FLURIDONE | FLURIDONE |
| FLUROXYPYR | FLUROXYPYR |
| FLUTHIACET-METHYL | AVERAGE |
| FLUTOLANIL | FLUTOLANIL |
| FLUTRIAFOL | FLUTRIAFOL |
| FLUVALINATE-TAU | AVERAGE |
| FLUXAPYROXAD | FLUXAPYROXAD |
| FOMESAFEN | AVERAGE |
| FORAMSULFURON | FORAMSULFURON |
| FORMETANATE | FORMETANATE HYDROCHLORIDE |
| FOSETYL | FOSETYL-AL |
| GALLEX | META-CRESOL |
| GAMMA AMINOBUTYRIC ACID | AVERAGE |
| GIBBERELLIC ACID | GIBBERELLINS |
| GLUFOSINATE | GLUFOSINATE-AMMONIUM |
| GLYPHOSATE | GLYPHOSATE |
| HALOSULFURON | HALOSULFURON-METHYL |
| HARPIN PROTEIN | HARPIN PROTEIN |
| HEXAZINONE | HEXAZINONE |
| HEXYTHIAZOX | HEXYTHIAZOX |
| HYDRAMETHYLNON | HYDRAMETHYLNON |
| HYDRATED LIME | CALCIUM HYDROXIDE |
| HYDROGEN PEROXIDE | HYDROGEN PEROXIDE |
| HYMEXAZOL | AVERAGE |
| IBA | IBA |
| IMAZALIL | IMAZALIL |
| IMAZAMETHABENZ | IMAZAMETHABENZ |
| IMAZAMOX | IMAZAMOX |
| IMAZAPIC | IMAZAPIC |
| IMAZAPYR | IMAZAPYR |
| IMAZAQUIN | AVERAGE |
| IMAZETHAPYR | IMAZETHAPYR |
| IMAZOSULFURON | IMAZOSULFURON |
| IMIDACLOPRID | IMIDACLOPRID |
| INDAZIFLAM | INDAZIFLAM |
| INDOXACARB | INDOXACARB |
| IODOSULFURON | AVERAGE |
| IPCONAZOLE | IPCONAZOLE |
| IPRODIONE | IPRODIONE |
| ISOXABEN | ISOXABEN |
| ISOXAFLUTOLE | AVERAGE |
| KAOLIN CLAY | KAOLIN |
| KINOPRENE | KINOPRENE |
| KRESOXIM-METHYL | KRESOXIM-METHYL |
| LACTOFEN | AVERAGE |
| L-GLUTAMIC ACID | GLUTAMIC ACID |
| LINURON | LINURON |
| MALATHION | MALATHION |
| MALEIC HYDRAZIDE | MALEIC HYDRAZIDE |
| MANCOZEB | MANCOZEB |
| MANDIPROPAMID | MANDIPROPAMID |
| MANEB | MANEB |
| MCPA | MCPA |
| MCPB | MCPB, SODIUM SALT |
| MECOPROP | MECOPROP-P |
| MEFENOXAM | MEFENOXAM |
| MEPIQUAT | MEPIQUAT CHLORIDE |
| MESOSULFURON | MESOSULFURON-METHYL |
| MESOTRIONE | MESOTRIONE |
| METALAXYL | METALAXYL |
| METALDEHYDE | METALDEHYDE |
| METAM | METAM-SODIUM |
| METAM POTASSIUM | METAM-SODIUM |
| METCONAZOLE | METCONAZOLE |
| METHAMIDOPHOS | METHAMIDOPHOS |
| METHIDATHION | METHIDATHION |
| METHIOCARB | METHIOCARB |
| METHOMYL | METHOMYL |
| METHOXYFENOZIDE | METHOXYFENOZIDE |
| METHYL BROMIDE | METHYL BROMIDE |
| METHYL BROMIDE | METHYL BROMIDE |
| METHYL IODIDE | METHYL IODIDE |
| METHYL PARATHION | METHYL PARATHION |
| METIRAM | METIRAM |
| METOLACHLOR | METOLACHLOR |
| METOLACHLOR-S | METOLACHLOR |
| METRAFENONE | METRAFENONE |
| METRIBUZIN | METRIBUZIN |
| METSULFURON | METSULFURON-METHYL |
| MEVINPHOS | MEVINPHOS |
| MSMA | MSMA |
| MYCLOBUTANIL | MYCLOBUTANIL |
| MYROTHECIUM VERRUCARIA | MYROTHECIUM VERRUCARIA, DRIED FERMENTATION SOLIDS |
| NALED | NALED |
| NAPHTHYLACETAMIDE | AVERAGE |
| NAPHTHYLACETIC ACID | AVERAGE |
| NAPROPAMIDE | NAPROPAMIDE |
| NAPTALAM | NAPTALAM, SODIUM SALT |
| NEEM OIL | AVERAGE |
| NICOSULFURON | NICOSULFURON |
| NORFLURAZON | NORFLURAZON |
| NOSEMA LOCUSTAE CANN | NOSEMA LOCUSTAE SPORES |
| NOVALURON | NOVALURON |
| ORTHOSULFAMURON | ORTHOSULFAMURON |
| ORYZALIN | ORYZALIN |
| OXADIAZON | OXADIAZON |
| OXAMYL | OXAMYL |
| OXYDEMETON-METHYL | OXYDEMETON-METHYL |
| OXYFLUORFEN | OXYFLUORFEN |
| OXYTETRACYCLINE | OXYTETRACYCLINE HYDROCHLORIDE |
| PACLOBUTRAZOL | PACLOBUTRAZOL |
| PARAQUAT | PARAQUAT DICHLORIDE |
| PARATHION | PARATHION |
| PELARGONIC ACID | AVERAGE |
| PENDIMETHALIN | PENDIMETHALIN |
| PENOXSULAM | PENOXSULAM |
| PENTHIOPYRAD | PENTHIOPYRAD |
| PERMETHRIN | PERMETHRIN |
| PETROLEUM DISTILLATE | PETROLEUM DISTILLATES |
| PETROLEUM OIL | PETROLEUM NAPHTHENIC OILS |
| PHENMEDIPHAM | PHENMEDIPHAM |
| PHORATE | PHORATE |
| PHOSMET | PHOSMET |
| PHOSPHORIC ACID | PHOSPHORIC ACID |
| PICLORAM | PICLORAM |
| PINOXADEN | PINOXADEN |
| PIPERONYL BUTOXIDE | PIPERONYL BUTOXIDE |
| POLYHEDROSIS VIRUS | POLYHEDRAL OCCLUSION BODIES (OB'S) OF THE NUCLEAR |
| POLYOXORIM | AVERAGE |
| POTASSIUM BICARBONATE | POTASSIUM BICARBONATE |
| POTASSIUM OLEATE | AVERAGE |
| PRIMISULFURON | AVERAGE |
| PRODIAMINE | PRODIAMINE |
| PROFENOFOS | PROFENOFOS |
| PROHEXADIONE | PROHEXADIONE CALCIUM |
| PROMETRYN | PROMETRYN |
| PROPAMOCARB HCL | PROPAMOCARB HYDROCHLORIDE |
| PROPANIL | PROPANIL |
| PROPARGITE | PROPARGITE |
| PROPAZINE | PROPAZINE |
| PROPICONAZOLE | PROPICONAZOLE |
| PROPOXYCARBAZONE | AVERAGE |
| PROPYZAMIDE | PROPYZAMIDE |
| PROSULFURON | AVERAGE |
| PROTHIOCONAZOLE | PROTHIOCONAZOLE |
| PSEUDOMONAS FLUORESCENS | PSEUDOMONAS FLUORESCENS, STRAIN A506 |
| PYMETROZINE | PYMETROZINE |
| PYRACLOSTROBIN | PYRACLOSTROBIN |
| PYRAFLUFEN ETHYL | PYRAFLUFEN-ETHYL |
| PYRASULFOTOLE | AVERAGE |
| PYRETHRINS | PYRETHRINS |
| PYRIDABEN | PYRIDABEN |
| PYRIMETHANIL | PYRIMETHANIL |
| PYRIPROXYFEN | PYRIPROXYFEN |
| PYRITHIOBAC-SODIUM | PYRITHIOBAC-SODIUM |
| PYROXASULFONE | AVERAGE |
| PYROXSULAM | PYROXSULAM |
| QUINCLORAC | QUINCLORAC |
| QUINOXYFEN | QUINOXYFEN |
| QUINTOZENE | AVERAGE |
| QUIZALOFOP | QUIZALOFOP-ETHYL |
| RIMSULFURON | RIMSULFURON |
| ROTENONE | ROTENONE |
| SABADILLA | SABADILLA ALKALOIDS |
| SAFLUFENACIL | SAFLUFENACIL |
| SETHOXYDIM | SETHOXYDIM |
| SILICATES | SILICA AEROGEL |
| SIMAZINE | SIMAZINE |
| SODIUM CHLORATE | SODIUM CHLORATE |
| SODIUM CHLORATE | SODIUM CHLORATE |
| SPINETORAM | SPINETORAM |
| SPINOSYN | SPINOSAD |
| SPIRODICLOFEN | SPIRODICLOFEN |
| SPIROMESIFEN | SPIROMESIFEN |
| SPIROTETRAMAT | SPIROTETRAMAT |
| STREPTOMYCIN | STREPTOMYCIN |
| SULFCARBAMIDE | AVERAGE |
| SULFENTRAZONE | SULFENTRAZONE |
| SULFOMETURON | SULFOMETURON-METHYL |
| SULFOSATE | AVERAGE |
| SULFOSULFURON | SULFOSULFURON |
| SULFOXAFLOR | SULFOXAFLOR |
| SULFUR | SULFUR |
| SULFURIC ACID | SULFURIC ACID |
| TCMTB | TCMTB |
| TEBUCONAZOLE | TEBUCONAZOLE |
| TEBUFENOZIDE | TEBUFENOZIDE |
| TEBUPIRIMPHOS | AVERAGE |
| TEBUTHIURON | TEBUTHIURON |
| TEFLUTHRIN | AVERAGE |
| TEMBOTRIONE | TEMBOTRIONE |
| TERBACIL | TERBACIL |
| TERBUFOS | AVERAGE |
| TETRABOROHYDRATE | AVERAGE |
| TETRACONAZOLE | TETRACONAZOLE |
| TETRATHIOCARBONATE | AVERAGE |
| THIABENDAZOLE | THIABENDAZOLE |
| THIACLOPRID | THIACLOPRID |
| THIAMETHOXAM | THIAMETHOXAM |
| THIAZOPYR | THIAZOPYR |
| THIDIAZURON | THIDIAZURON |
| THIENCARBAZONE-METHYL | AVERAGE |
| THIFENSULFURON | THIFENSULFURON-METHYL |
| THIOBENCARB | THIOBENCARB |
| THIODICARB | THIODICARB |
| THIOPHANATE-METHYL | THIOPHANATE-METHYL |
| THIRAM | THIRAM |
| TOPRAMEZONE | AVERAGE |
| TRALKOXYDIM | TRALKOXYDIM |
| TRIADIMEFON | TRIADIMEFON |
| TRIADIMENOL | TRIADIMENOL |
| TRI-ALLATE | TRIALLATE |
| TRIASULFURON | AVERAGE |
| TRIBENURON METHYL | TRIBENURON-METHYL |
| TRIBUFOS | AVERAGE |
| TRICLOPYR | TRICLOPYR, BUTOXYETHYL ESTER |
| TRIFLOXYSTROBIN | TRIFLOXYSTROBIN |
| TRIFLOXYSULFURON | TRIFLOXYSULFURON-SODIUM |
| TRIFLUMIZOLE | TRIFLUMIZOLE |
| TRIFLURALIN | TRIFLURALIN |
| TRIFLUSULFURON | AVERAGE |
| TRINEXAPAC | TRINEXAPAC-ETHYL |
| TRITICONAZOLE | TRITICONAZOLE |
| UNICONAZOLE | UNICONIZOLE-P |
| VINCLOZOLIN | VINCLOZOLIN |
| ZETA-CYPERMETHRIN | AVERAGE |
| ZINC | ZINC CHLORIDE |
| ZINEB | ZINEB |
| ZIRAM | ZIRAM |
| ZOXAMIDE | AVERAGE |

Table 5. VOC Emissions Factors

| Pesticide | Emission Factor (lbs. VOC per lb. active ingredient) |
| --- | --- |
| 2,4-D | 0.827 |
| 2,4-DB ACID | 0.067 |
| ABAMECTIN | 15.236 |
| ACEPHATE | 0.275 |
| ACEQUINOCYL | 0.135 |
| ACETAMIPRID | 0.207 |
| ACIBENZOLAR-S-METHYL | 0.063 |
| ACIFLUORFEN, SODIUM SALT | 1.887 |
| ALACHLOR | 0.513 |
| ALDICARB | 0.064 |
| ALUMINUM PHOSPHIDE | 0.055 |
| AMETOCTRADIN | 0.041 |
| AMETRYNE | 0.024 |
| AMINOPYRALID, TRIISOPROPANOLAMINE SALT | 0.160 |
| ASULAM, SODIUM SALT | 0.202 |
| ATRAZINE | 0.148 |
| AZADIRACHTIN | 10.092 |
| AZINPHOS-METHYL | 0.464 |
| AZOXYSTROBIN | 0.344 |
| BACILLUS AMYLOLIQUEFACIENS STRAIN D747 | 0.076 |
| BACILLUS CEREUS, STRAIN BP01 | 0.106 |
| BACILLUS FIRMUS (STRAIN I-1582) | 0.052 |
| BACILLUS PUMILUS GHA 180 | 2,050.000 |
| BACILLUS SUBTILIS GB03 | 190.333 |
| BACILLUS THURINGIENSIS (BERLINER) | 0.487 |
| BENOMYL | 0.074 |
| BENSULFURON METHYL | 0.031 |
| BENSULIDE | 0.553 |
| BENTAZON, SODIUM SALT | 0.053 |
| BIFENAZATE | 0.084 |
| BIFENTHRIN | 1.566 |
| BISPYRIBAC-SODIUM | 0.038 |
| BOSCALID | 0.229 |
| BROMACIL | 0.850 |
| BUPROFEZIN | 0.164 |
| CALCIUM HYDROXIDE | 0.003 |
| CAPTAN | 0.144 |
| CARBARYL | 0.321 |
| CARBOPHENOTHION | 0.446 |
| CARBOXIN | 0.437 |
| CARFENTRAZONE-ETHYL | 0.653 |
| CHLORANTRANILIPROLE | 0.364 |
| CHLORFENAPYR | 0.137 |
| CHLORMEQUAT CHLORIDE | 0.586 |
| CHLORONEB | 0.074 |
| CHLOROPICRIN | 1.272 |
| CHLOROTHALONIL | 0.113 |
| CHLORPROPHAM | 0.325 |
| CHLORPYRIFOS | 1.538 |
| CHLORSULFURON | 0.028 |
| CLETHODIM | 1.840 |
| CLOFENTEZINE | 0.147 |
| CLOMAZONE | 0.149 |
| CLOPYRALID | 0.050 |
| CLOTHIANIDIN | 0.153 |
| CONIOTHYRIUM MINITANS STRAIN CON/M/91-08 | 0.698 |
| COPPER | 0.218 |
| COPPER HYDROXIDE | 0.060 |
| COPPER OCTANOATE | 2.198 |
| COPPER OXIDE (OUS) | 0.029 |
| COPPER OXYCHLORIDE | 0.023 |
| COPPER OXYCHLORIDE SULFATE | 0.026 |
| COPPER SULFATE (BASIC) | 0.048 |
| COPPER SULFATE (PENTAHYDRATE) | 0.062 |
| CRYOLITE | 0.025 |
| CYAZOFAMID | 0.166 |
| CYCLANILIDE | 2.468 |
| CYCLOATE | 0.507 |
| CYFLUFENAMID | 0.175 |
| CYFLUTHRIN | 1.736 |
| CYHALOFOP-BUTYL | 0.452 |
| CYMOXANIL | 0.044 |
| CYPERMETHRIN | 1.521 |
| CYPRODINIL | 0.049 |
| CYROMAZINE | 0.228 |
| CYTOKININ | 0.254 |
| DAMINOZIDE | 0.045 |
| DAZOMET | 1.000 |
| DELTAMETHRIN | 3.949 |
| DESMEDIPHAM | 3.668 |
| DIAZINON | 0.760 |
| DICAMBA | 0.084 |
| DICHLOBENIL | 0.434 |
| DICLOFOP-METHYL | 1.042 |
| DICLORAN | 0.087 |
| DICOFOL | 0.424 |
| DICROTOPHOS | 0.258 |
| DIENOCHLOR | 0.182 |
| DIFENOCONAZOLE | 1.120 |
| DIFLUBENZURON | 0.159 |
| DIMETHENAMID-P | 0.135 |
| DIMETHIPIN | 0.367 |
| DIMETHOATE | 0.830 |
| DIMETHOMORPH | 0.038 |
| DINOSEB | 0.455 |
| DINOTEFURAN | 0.191 |
| DIQUAT DIBROMIDE | 1.456 |
| DISULFOTON | 1.186 |
| DITHIOPYR | 0.955 |
| DIURON | 0.072 |
| DODINE | 0.049 |
| EMAMECTIN BENZOATE | 3.055 |
| ENDOSULFAN | 0.492 |
| EPTC | 0.517 |
| ESFENVALERATE | 8.919 |
| ETHALFLURALIN | 1.554 |
| ETHEPHON | 0.302 |
| ETHION | 0.397 |
| ETHOFUMESATE | 0.691 |
| ETHOPROP | 0.416 |
| ETOXAZOLE | 0.059 |
| FENAMIDONE | 0.101 |
| FENAMIPHOS | 1.043 |
| FENARIMOL | 1.404 |
| FENBUCONAZOLE | 0.049 |
| FENBUTATIN-OXIDE | 0.058 |
| FENHEXAMID | 0.037 |
| FENOXAPROP-ETHYL | 3.132 |
| FENOXYCARB | 0.655 |
| FENPROPATHRIN | 1.469 |
| FENPYROXIMATE | 8.721 |
| FENTIN HYDROXIDE | 0.039 |
| FERBAM | 0.045 |
| FIPRONIL | 6.463 |
| FLAZASULFURON | 0.148 |
| FLONICAMID | 0.060 |
| FLORASULAM | 0.052 |
| FLUAZIFOP-BUTYL | 1.464 |
| FLUAZINAM | 0.406 |
| FLUBENDIAMIDE | 0.102 |
| FLUDIOXONIL | 0.308 |
| FLUMICLORAC-PENTYL | 0.565 |
| FLUMIOXAZIN | 0.075 |
| FLUOMETURON | 0.046 |
| FLUOPICOLIDE | 0.136 |
| FLUOPYRAM | 0.291 |
| FLUOXASTROBIN | 0.172 |
| FLURIDONE | 0.629 |
| FLUROXYPYR | 0.279 |
| FLUTOLANIL | 0.031 |
| FLUTRIAFOL | 0.331 |
| FLUXAPYROXAD | 0.020 |
| FORAMSULFURON | 0.252 |
| FORMETANATE HYDROCHLORIDE | 0.011 |
| FOSETYL-AL | 0.049 |
| GIBBERELLINS | 2.819 |
| GLUFOSINATE-AMMONIUM | 0.442 |
| GLUTAMIC ACID | 0.063 |
| GLYPHOSATE | 0.159 |
| HALOSULFURON-METHYL | 0.032 |
| HARPIN PROTEIN | 1.233 |
| HEXAZINONE | 0.142 |
| HEXYTHIAZOX | 0.423 |
| HYDRAMETHYLNON | 0.614 |
| HYDROGEN PEROXIDE | 0.356 |
| IBA | 0.559 |
| IMAZALIL | 0.794 |
| IMAZAMETHABENZ | 0.504 |
| IMAZAMOX | 0.016 |
| IMAZAPIC | 0.016 |
| IMAZAPYR | 0.025 |
| IMAZETHAPYR | 0.019 |
| IMAZOSULFURON | 0.049 |
| IMIDACLOPRID | 0.305 |
| INDAZIFLAM | 0.416 |
| INDOXACARB | 0.453 |
| IPCONAZOLE | 0.122 |
| IPRODIONE | 0.203 |
| ISOXABEN | 0.103 |
| KAOLIN | 0.015 |
| KINOPRENE | 0.466 |
| KRESOXIM-METHYL | 0.034 |
| LINURON | 0.077 |
| MALATHION | 0.409 |
| MALEIC HYDRAZIDE | 0.015 |
| MANCOZEB | 0.047 |
| MANDIPROPAMID | 0.209 |
| MANEB | 0.071 |
| MCPA | 0.470 |
| MCPB, SODIUM SALT | 1.206 |
| MECOPROP-P | 0.622 |
| MEFENOXAM | 0.587 |
| MEPIQUAT CHLORIDE | 0.661 |
| MESOSULFURON-METHYL | 0.822 |
| MESOTRIONE | 0.236 |
| META-CRESOL | 73.605 |
| METALAXYL | 0.506 |
| METALDEHYDE | 0.691 |
| METAM-SODIUM | 0.566 |
| METCONAZOLE | 0.369 |
| METHAMIDOPHOS | 0.710 |
| METHIDATHION | 1.068 |
| METHIOCARB | 0.220 |
| METHOMYL | 0.115 |
| METHOXYFENOZIDE | 0.223 |
| METHYL BROMIDE | 1.159 |
| METHYL IODIDE | 1.212 |
| METHYL PARATHION | 0.502 |
| METIRAM | 0.110 |
| METOLACHLOR | 0.198 |
| METRAFENONE | 0.074 |
| METRIBUZIN | 0.087 |
| METSULFURON-METHYL | 0.037 |
| MEVINPHOS | 0.534 |
| MSMA | 0.315 |
| MYCLOBUTANIL | 0.451 |
| MYROTHECIUM VERRUCARIA, DRIED FERMENTATION SOLIDS | 0.127 |
| NALED | 0.494 |
| NAPROPAMIDE | 0.385 |
| NAPTALAM, SODIUM SALT | 0.588 |
| NICOSULFURON | 0.037 |
| NORFLURAZON | 0.031 |
| NOSEMA LOCUSTAE SPORES | 7.085 |
| NOVALURON | 2.273 |
| ORTHOSULFAMURON | 0.097 |
| ORYZALIN | 0.212 |
| OXADIAZON | 0.182 |
| OXAMYL | 0.721 |
| OXYDEMETON-METHYL | 0.928 |
| OXYFLUORFEN | 1.012 |
| OXYTETRACYCLINE HYDROCHLORIDE | 0.199 |
| PACLOBUTRAZOL | 0.983 |
| PARAQUAT DICHLORIDE | 0.311 |
| PARATHION | 0.357 |
| PENDIMETHALIN | 0.559 |
| PENOXSULAM | 0.208 |
| PENTHIOPYRAD | 0.054 |
| PERMETHRIN | 3.345 |
| PETROLEUM DISTILLATES | 1.142 |
| PETROLEUM NAPHTHENIC OILS | 0.884 |
| PHENMEDIPHAM | 3.129 |
| PHORATE | 0.448 |
| PHOSMET | 1.162 |
| PHOSPHORIC ACID | 0.434 |
| PICLORAM | 0.398 |
| PINOXADEN | 10.388 |
| PIPERONYL BUTOXIDE | 4.504 |
| POLYHEDRAL OCCLUSION BODIES (OB'S) OF THE NUCLEAR | 8.922 |
| POTASSIUM BICARBONATE | 0.027 |
| PRODIAMINE | 0.126 |
| PROFENOFOS | 0.367 |
| PROMETRYN | 0.184 |
| PROPAMOCARB HYDROCHLORIDE | 0.180 |
| PROPANIL | 0.099 |
| PROPARGITE | 0.196 |
| PROPAZINE | 0.200 |
| PROPICONAZOLE | 1.052 |
| PROPYZAMIDE | 0.055 |
| PROTHIOCONAZOLE | 0.139 |
| PSEUDOMONAS FLUORESCENS, STRAIN A506 | 0.022 |
| PYMETROZINE | 0.020 |
| PYRACLOSTROBIN | 0.549 |
| PYRAFLUFEN-ETHYL | 5.343 |
| PYRETHRINS | 6.737 |
| PYRIDABEN | 0.019 |
| PYRIMETHANIL | 0.188 |
| PYRIPROXYFEN | 1.387 |
| PYRITHIOBAC-SODIUM | 0.193 |
| PYROXSULAM | 0.135 |
| QUINCLORAC | 0.121 |
| QUINOXYFEN | 0.060 |
| QUIZALOFOP-ETHYL | 4.121 |
| RIMSULFURON | 0.070 |
| ROTENONE | 0.808 |
| SABADILLA ALKALOIDS | 2.018 |
| SAFLUFENACIL | 0.015 |
| SETHOXYDIM | 3.751 |
| SILICA AEROGEL | 0.381 |
| SIMAZINE | 0.089 |
| SODIUM CHLORATE | 0.025 |
| SPINETORAM | 0.138 |
| SPINOSAD | 0.483 |
| SPIRODICLOFEN | 0.229 |
| SPIROMESIFEN | 0.119 |
| SPIROTETRAMAT | 0.101 |
| STREPTOMYCIN | 0.133 |
| SULFENTRAZONE | 0.128 |
| SULFOMETURON-METHYL | 0.076 |
| SULFOSULFURON | 0.027 |
| SULFOXAFLOR | 0.060 |
| SULFUR | 0.013 |
| SULFURIC ACID | 0.088 |
| TCMTB | 0.995 |
| TEBUCONAZOLE | 0.178 |
| TEBUFENOZIDE | 0.163 |
| TEBUTHIURON | 0.075 |
| TEMBOTRIONE | 0.096 |
| TERBACIL | 0.023 |
| TETRACONAZOLE | 0.492 |
| THIABENDAZOLE | 0.117 |
| THIACLOPRID | 0.119 |
| THIAMETHOXAM | 0.178 |
| THIAZOPYR | 1.756 |
| THIDIAZURON | 0.396 |
| THIFENSULFURON-METHYL | 0.049 |
| THIOBENCARB | 0.158 |
| THIODICARB | 0.133 |
| THIOPHANATE-METHYL | 0.118 |
| THIRAM | 0.219 |
| TRALKOXYDIM | 0.141 |
| TRIADIMEFON | 0.162 |
| TRIADIMENOL | 0.243 |
| TRIALLATE | 0.573 |
| TRIBENURON-METHYL | 0.030 |
| TRICLOPYR, BUTOXYETHYL ESTER | 0.433 |
| TRIFLOXYSTROBIN | 0.083 |
| TRIFLOXYSULFURON-SODIUM | 0.014 |
| TRIFLUMIZOLE | 0.067 |
| TRIFLURALIN | 0.737 |
| TRINEXAPAC-ETHYL | 2.386 |
| TRITICONAZOLE | 0.240 |
| UNICONIZOLE-P | 125.636 |
| VINCLOZOLIN | 0.055 |
| ZINC CHLORIDE | 0.329 |
| ZINEB | 0.082 |
| ZIRAM | 0.031 |

U.S. Environmental Protection Agency. 2001. Emissions Inventory Improvement Program, Vol. 3, Ch. 9, “Pesticides - Agricultural and Nonagricultural”, Section 5.1, p. 9.5-4, https://www.epa.gov/sites/production/files/2015-08/documents/iii09_jun2001.pdf

United States Geological Survey. 2017. “Archived preliminary county-level pesticide use estimates”, https://water.usgs.gov/nawqa/pnsp/usage/maps/county-level/

Personal communication from Pam Wofford, California Department of Pesticide Regulation to Jonathan Dorn, Abt Associates, “CDPR_Emission_Potential_Database_10_2015.xlsx”, January 2016

U.S. Environmental Protection Agency. 2000. Health Effects Notebook for Hazardous Air Pollutants. https://www.epa.gov/haps/health-effects-notebook-hazardous-air-pollutants