# Asphalt Paving

# Source Category Description

Asphalt paving is the process of applying asphalt concrete to seal or repair the surface of roads, parking lots, driveways, walkways, or airport runways. Asphalt concrete is a composite material comprised of a binder and a mineral aggregate. The binder, referred to as asphalt cement, is a byproduct of petroleum refining and contains the semi-solid residual material left after the more volatile chemical fractions have been distilled off.

Asphalt cements thinned with water and an emulsifying agent are known as emulsified asphalts. Asphalt cements thinned with petroleum distillates are known as cutback asphalts; cut-back asphalt is produced by thinning the binder in diluent containing 25 to 45 percent petroleum distillates by volume prior to mixing with the aggregate. Thinning reduces the viscosity of the asphalt making it easier to work with the mixture. The primary uses of asphalt cements include tack and seal operations, priming roadbeds, and paving operations for pavements up to several inches thick.

Estimates of emissions of volatile organic compounds (VOC), and hazardous air pollutants (HAPs) from asphalt paving are based on the amount of cutback and emulsified asphalt used. In 2014, cutback asphalt use in the US, Puerto Rico, and US Virgin Islands resulted in approximately 77,609 tons of VOC and emulsified asphalt use in the US, Puerto Rico, and US Virgin Islands resulted in 134,706 tons of VOC emissions.

For this source category, the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2461021000 | Solvent Utilization | Misc Non-industrial: Commercial | Cutback Asphalt | Total: All Solvent Types |
| 2461022000 | Solvent Utilization | Misc Non-industrial: Commercial | Emulsified Asphalt | Total: All Solvent Types |

Note that these source categories do not include emissions from the use of hot mix asphalt (HMA) or warm mix asphalt (WMA).

# Overview of Calculations

The calculations for estimating the emissions from asphalt use involve first estimating the amount of cutback and emulsified asphalt used in each county. The amount of state-level cutback and emulsified asphalt used in 2008 is available from an Asphalt Institute report. Asphalt use is adjusted to 2017 using a ratio of the vehicle miles traveled (VMT) in the US in 2017 to US VMT in 2008. The amount of state-level asphalt used is then distributed to the counties based on the county-level utilization of paved roads. The total amount of asphalt used is multiplied by emissions factors for VOC and HAPS to estimate emissions of these pollutants from asphalt usage. Sources of data and calculations for the amount of asphalt used are discussed in section C. The process of allocating asphalt activity data to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from asphalt use is discussed in section G.

# Activity Data

| The activity data for this source category is the amount of cutback and emulsified asphalt used, which is from a 2008 survey from the Asphalt Institute. The 2008 data are used for the 2008, 2011, and 2014 NEI, as research suggests that more recent data are not readily available. The 2008 asphalt data are adjusted to account for changing use of roads, parking lots, driveways, walkways, or airport runways, using ratio of US VMT in 2017 to US VMT in 2008. State-level VMT data are obtained from the Federal Highway Administration (FHWA) report: State-level annual vehicle miles traveled (VMT) by FHWA road class, 2017. VMTFrac=VMTUS,yVMTUS,2008 |  |
| --- | --- |
| AUs,t=VMTFrac×UAUs,t |  |

Where:

VMTFrac	=	 The fraction of US VMT in 2008 to US VMT in 2017

VMTUS,2008	=	 Total VMT in the US in 2008

VMTUS,y		=  Total VMT in the US in 2017

AUs,t		=	 The amount of asphalt type t used in state s, in tons of asphalt per year, from equation 2

UAUs,t		=  The amount of unadjusted asphalt type t used in state s, in tons of asphalt per year, from Table    1

Table 1 shows the total state-level amount of cutback and emulsified asphalt used in the U.S in 2008.

The process used to distribute the state-level amount of asphalt used to the counties is discussed in section D.

Table 1. State-level Asphalt Usage in 2008

| State | Cutback Asphalt Usage (Tons) | Emulsified Asphalt Usage (Tons) |
| --- | --- | --- |
| Alabama | 1,728 | 18,988 |
| Alaska | 0 | 1,108 |
| Arizona | 7,917 | 62,416 |
| Arkansas | 1,442 | 9,201 |
| California | 30,657 | 151,767 |
| Colorado | 331 | 837 |
| Connecticut | 0 | 0 |
| Delaware | 0 | 0 |
| District of Columbia | 0 | 150 |
| Florida | 809 | 19,459 |
| Georgia | 1,136 | 7,848 |
| Hawaii | 0 | 0 |
| Idaho | 2,880 | 41,805 |
| Illinois | 18,889 | 146,873 |
| Indiana | 290 | 17,427 |
| Iowa | 4,874 | 13,570 |
| Kansas | 3,641 | 0 |
| Kentucky | 456 | 16,137 |
| Louisiana | 175 | 6,418 |
| Maine | 0 | 0 |
| Maryland | 0 | 2,080 |
| Massachusetts | 0 | 805 |
| Michigan | 52 | 31,250 |
| Minnesota | 1,604 | 67,082 |
| Mississippi | 259 | 45,035 |
| Missouri | 7,385 | 36,933 |
| Montana | 1,614 | 17,880 |
| Nebraska | 2,997 | 35,376 |
| Nevada | 948 | 15,971 |
| New Hampshire | 0 | 0 |
| New Jersey | 0 | 0 |
| New Mexico | 320 | 58,048 |
| New York | 0 | 32,954 |
| North Carolina | 0 | 143 |
| North Dakota | 7,323 | 22,701 |
| Ohio | 3,214 | 22,777 |
| Oklahoma | 8,724 | 9,157 |
| Oregon | 865 | 34,918 |
| Pennsylvania | 26,844 | 69,671 |
| Rhode Island | 0 | 0 |
| South Carolina | 0 | 0 |
| South Dakota | 19,034 | 44,691 |
| Tennessee | 894 | 34,561 |
| Texas | 14,618 | 154,613 |
| Utah | 549 | 7,039 |
| Vermont | 0 | 0 |
| Virginia | 670 | 41,249 |
| Washington | 5,774 | 24,263 |
| West Virginia | 0 | 3,581 |
| Wisconsin | 8,188 | 18,925 |
| Wyoming | 227 | 5,292 |

Source: Asphalt Institute

# Allocation Procedure

Asphalt usage data are not available at the county-level, therefore state –level data are allocated to the county based on road utilization numbers calculated from FHWA data.

State-level VMT data are obtained from the FHWA report: State-level annual vehicle miles traveled (VMT) by FHWA road class, 2017.2   EPA used the state-level data and 2011 MOVES data to allocate VMT to the county-level.

| VMTc,r=MOVESc,r×VMTs,rMOVESs,r |  |
| --- | --- |

Where:

VMTc,r	= 	The amount of VMT on road type r  in county c from EPA, in millions of miles

MOVESc,r	=	The amount of VMT on road type r in county c from the 2011 MOVES run

VMTs,r	=	The amount of VMT on road type r in state s from FHWA, in millions of miles

MOVESs,r	=	The amount of VMT on road type r in state s from the 2011 MOVES run

The county-level VMT is used to calculate the fraction of VMT in each county.

| VMTFrc,r=VMTc,rVMTs,r |  |
| --- | --- |

Where:

VMTFrc,r	=	The fraction of VMT on road type r  in county c

VMTc,r	= 	The amount of VMT on road type r  in county c from EPA, in millions of miles

VMTs,r	=	The amount of VMT on road type r in state s from FHWA, in millions of miles

State-level lane-miles and paved road miles from FHWA are used to calculate an estimate of state lane-miles that are paved by road type.

| PLMs,r=PMs,rPUMs,r×LMs,r |  |
| --- | --- |

Where:

PLMs,r	=	The amount of paved lane-miles of road type r in state s, in miles

PMs,r	=	The amount of paved road miles of road type r in state s from FHWA, in miles

PUMs,r	=	The amount of paved and unpaved miles of road type r in state s from FHWA, in miles

LMs,r	=	The amount of lanes miles of road type r in state s from FHWA, in miles

State-level VMT from FHWA and paved lane-miles (from equation 3) are used to calculate a state-level utilization measure for paved roads by road type.

| Us,r=VMTs,rPLMs,r |  |
| --- | --- |

Where:

Us,r	=	Utilization of paved road type r in state s

VMTs,r	=	The amount of VMT on road type r in state s from FHWA

PLMs,r	=	The amount of paved lane-miles of road type r in state s

County-level utilization of paved roads by road type is calculated based on the fraction of county-level VMT (from equation 2).

| Uc,r=VMTFrc,r×Us,r |  |
| --- | --- |

Where:

Uc,r	=	Utilization of paved road type r in state s

VMTFrc,r	=	The fraction of VMT on road type r  in county

Us,r	=	Utilization of paved road type r in state s

County-level utilization values are summed across all road types and then summed to the state level.

| Uc=rUc,r |  |
| --- | --- |
| Us=cUc |  |

Where:

Us	=	The total utilization of paved roads in state s

Uc	=	The total utilization of paved roads in county c

Uc,r	=	Utilization of paved road type r in state s

The fraction of county-level utilization is calculated based on the ratio of total utilization at the county level to state level.

| UFrc=UcUs |  |
| --- | --- |

Where:

UFrc	=	The fraction of paved road utilization in county c

Us	=	The total utilization of paved roads in state s

Uc	=	The total utilization of paved roads in county c

County-level asphalt usage is the calculated by multiplying the fraction of county-level paved road utilization by the amount of cutback and emulsified asphalt used from Table 1.

| AUc,t=UFrc×AUs,t |  |
| --- | --- |

Where:

AUc,t	=	The amount of asphalt type t used in county c, in tons of asphalt per year

UFrc	=	The fraction of paved road utilization in county c

AUs,t	=	The amount of asphalt type t used in state s, in tons of asphalt per year, from Table 1

# Emissions Factors

Emissions factors for asphalt paving are reported in Table 4 and Table 5. The emissions factors for VOC and HAPs are developed based on information from material safety and data sheets (MSDS) for cutback and emulsified asphalt.

Table 2. Chemical Composition Assumptions for Cutback Asphalt

| Pollutant | Average % by Weight | % Weight Volatilized |
| --- | --- | --- |
| Naphtha | 40 | 95 |
| Naphthalene & PAH | 0.58 | 95 |
| Toluene | 0.59 | 95 |
| Xylene | 0.99 | 95 |
| Benzene | 0.19 | 95 |
| Ethylbenzene | 0.49 | 95 |
| Hydrogen Sulfide | 0.09 | 95 |

Source: Average of MSDS values, reference 5

Table 3. Chemical Composition Assumptions for Emulsified Asphalt

| Pollutant | Average % by Weight | % Weight Volatilized |
| --- | --- | --- |
| Naphtha | 10 | 95 |
| Naphthalene & PAH | 0.29 | 95 |
| Hydrogen Sulfide | 0.09 | 95 |

Source: Average of MSDS values, reference 6

Emissions factors for HAPs are calculated using the assumptions found in Table 2 and Table 3. The total amount of cutback asphalt used nationally is 190,613 tons and the amount of emulsified asphalt used is 1,374,693 tons.

| ZUS,p,t=AUUS,t×2000lbs.ton×%Wp,t×%Vp |  |
| --- | --- |
| EFp,t=ZUS,p,tAUUS,t |  |

Where:

ZUS,p,t	=	The amount of pollutant p emitted from use of asphalt type t in the United States, in lbs. of pollutant per year

EFp,t	=	Emissions factor for pollutant p from asphalt type t, in lbs. of pollutant per ton of asphalt

AUUS,t	=	Total usage of asphalt type t, in tons of asphalt per year

%Wp,t	=	Average percent by weight of pollutant p from asphalt type t

%Vp	=	Average percent weight of pollutant p volatilized

Emissions factors for VOC are calculated by summing the amount of pollutant emitted each year for all HAPs, except hydrogen sulfide.

| EFvoc,t=pZtAUUS,t |  |
| --- | --- |

Where:

EFvoc,t	=	VOC emissions factor for asphalt type t, in lbs. of  VOC per ton of asphalt

Zp,t	=	The amount of pollutant emitted from use of asphalt type t, where p is equal to all pollutants except hydrogen sulfide, in lbs. of pollutant per year

AUUS,t	=	Total usage of asphalt type t, in tons of asphalt per year

Table 4. Emissions Factors for Cutback Asphalt Usage

| Pollutant | Pollutant Code | Emissions Factor | Emissions Factor Units |
| --- | --- | --- | --- |
| Volatile Organic Compounds | VOC | 813.96 | lbs./ton asphalt |
| Benzene | 71432 | 3.6 | lbs./ton asphalt |
| Ethylbenzene | 100414 | 9.3 | lbs./ton asphalt |
| Naphthalene | 91203 | 11.0 | lbs./ton asphalt |
| Toluene | 108883 | 11.2 | lbs./ton asphalt |
| Xylenes (mixed isomers) | 1330207 | 18.8 | lbs./ton asphalt |
| Hydrogen Sulfide | 7783064 | 1.7 | lbs./ton asphalt |

Source: Based on MSDS values from Table 2

Table 5. Emissions Factors for Emulsified Asphalt Usage

| Pollutant | Pollutant Code | Emissions Factor | Emissions Factor Units |
| --- | --- | --- | --- |
| Volatile Organic Compounds | VOC | 195.5 | lbs./ton asphalt |
| Naphthalene | 91203 | 5.5 | lbs./ton asphalt |
| Hydrogen Sulfide | 7783064 | 1.7 | lbs./ton asphalt |

Source: Based on MSDS values from Table 3

# Controls

There are no controls assumed for this category.

# Emissions

The total asphalt usage in each county is multiplied by the emissions factors in Table 4 and Table 5 to estimate emissions.

| Ep,c,t= EFp,t×AUc,t |  |
| --- | --- |

Where:

Ep,c,t 	=	Annual emissions of pollutant p in county c from use of asphalt type t, in lbs. of pollutant

EFp,t	=	Emissions factor for pollutant p from asphalt type t, in lbs. of pollutant per ton of asphalt	AUc,t	=	The amount of asphalt type t used in county c, in tons of asphalt per year

# Point Source Subtraction

There are no point source-specific SCCs for asphalt paving; therefore point source subtraction is not performed for this category. There are point source SCCs for asphalt manufacturing, but these do not apply to asphalt paving.

# Sample Calculations

Table 6 lists sample calculations to determine the VOC emissions from emulsified asphalt used in Barnstable County, Massachusetts. The equations 2 through 7 use asphalt use on rural interstates as an example; however, these calculations would need to be repeated for all 14 FHWA road types.

Table 6. Sample calculations for VOC emissions from emulsified asphalt use in Barnstable County, Massachusetts.

| Eq. # | Equation | Values for Barnstable County, MA | Result |
| --- | --- | --- | --- |
| 1 | VMTFr=VMTUS,yVMTUS,2008 | 3,025,659 Million Miles2,973,509 Million Miles | 1.02 VMT fraction between 2008 and 2017 |
| 2 | AUs,t=VMTFrac×UAUs,t | 1.02×805 tons of emulsified asphalt in MA | 819 tons of adjusted emulsified asphalt used in MA |
| 3 | VMTc,r=MOVESc,r×VMTs,rMOVESs,r | Barnstable County VMT on rural interstates from EPA | 153,721,475.26 vehicle miles traveled on rural interstates in Barnstable County, MA |
| 4 | VMTFrc,r=VMTc,rVMTs,r | 153.72 million vehicle mi. in Barnstable County778.15 million vehicle mi. in MA | 0.198 fraction of rural interstate VMT in Barnstable County, MA |
| 5 | PLMs,r=PMs,rPUMs,r×LMs,r | 63.65 paved mi.63.65 total mi.×275.25 lane mi. | 275.25 rural interstate paved lane miles in MA |
| 6 | Us,r=VMTs,rPLMs,r | 778.15 million vehicle mi. in MA275.25 paved lane mi. in MA | 2.83 utilization factor of paved rural interstates in MA |
| 7 | Uc,r=VMTFrc,r×Us,r | 0.198 VMT fraction×2.83 utilization factor | 0.558 utilization factor of paved rural interstates in Barnstable County, MA |
| 8 | Uc=rUc,r | Utilization of all paved roads in Barnstable (This is based on repeating calculations for equations 1-5 for all 14 FHWA road types.) | 2.18 Barnstable County utilization of paved roads in MA |
| 9 | Us=cUc | Utilization of paved roads in all counties in MA | 46.20 utilization of paved roads in MA |
| 10 | UFrc=UcUs | 2.18 utilization in Barnstable County46.20 utilization in MA | 0.05 fraction of utilization of paved roads in Barnstable County, MA |
| 11 | AUc,t=UFrc×AUs,t | 0.05 fraction utilized in Barnstable County×819 tons emulsified asphalt in MA | 37.91 tons of emulsified asphalt used in Barnstable County, MA |
| 12 | Pp,t=AUUS,t×2000lbs.ton×%Wp,t×%Vp | 1,350,999 tons emulsified asphalt per year×2000lbs.ton×0.10×0.95 | 256,689,810 lbs. naphtha emitted per year from emulsified asphalt |
|  |  | 1,350,999 tons emulsified asphalt per year×2000lbs.ton×0.0029×0.95 | 7,444,004 lbs. naphthalene emitted per year from emulsified asphalt |
| 13 | EFp=PpAUUS,t | N/A | Emissions factors for HAPs are not used to calculate the emissions factor for VOC |
| 14 | EFvoc,t=pZtAUUS,t | 256,689,810 lbs. naptha+7,444,004 lbs. naphthalene 1,350,999 tons of emulsified asphalt | 195.51 lbs. VOC emitted per ton of emulsified asphalt used |
| 15 | Ep,c,t= EFp,t×AUc,t | 195.51 lbs. VOC per top emulsified asphalt ×37.91 tons emulsified asphalt | 7,411.78 tons VOC emitted from emulsified asphalt use in Barnstable County, MA |

# Changes from 2014 Methodology

State-level asphalt use is adjusted in the 2017 methodology using a ratio of VMT in the inventory year to VMT in 2008, the year of the original asphalt data..

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Insufficient data exists to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, so emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emission factor.  For each Puerto Rico and US Virgin Island County, the tons per capita emission factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

Asphalt Institute, 2008. 2008 Asphalt Usage Survey for the United States and Canada. http://www.asphaltinstitute.org/

FHWA, 2017. Functional System Travel-2017, Annual Vehicle Miles (Table VM-2). https://www.fhwa.dot.gov/policyinformation/statistics/2017/vm2.cfm

FHWA, 2017. Functional System Lane-Length-2017, Lane-Miles (Table HM-60). https://www.fhwa.dot.gov/policyinformation/statistics/2017/hm60.cfm

FHWA, 2017. Functional System Length-2017, Miles by Type of Surface - Rural (Table HM-51). https://www.fhwa.dot.gov/policyinformation/statistics/2017/hm51.cfm

Cutback Asphalt MSDS

| Product Supplier | MSDS/SDS ID |
| --- | --- |
| Valero | 2013V04 |
| Asphalt Emulsion Industries | CUT-SDS-1 |
| Martin Asphalt Company | Jan 2007 |
| Mohawk Asphalt Emulsions | UN1999 |
| Asphalt & Fuel Supply | 211 |
| Valero | 211 |
| Valero | 210 |

Emulsified Asphalt MSDS

| Product Supplier | MSDS/SDS ID |
| --- | --- |
| Marathon | 0137MAR019 |
| Marathon | 0138MAR019 |
| Asphalt Emulsion Industries | EMU-SDS-1 |
| U.S. Oil & Refining Co. | 951 |