# Commercial Cooking

# Source Category Description

Commercial cooking refers to the cooking of meat, including steak, hamburger, poultry, pork, and seafood, and french fries on five different cooking devices: chain driven (conveyorized) char-broilers, underfired char-broilers, deep-fat fryers, flat griddles, and clamshell griddles. Estimates of emissions of criteria air pollutants (CAPs) and hazardous air pollutants (HAPs) are based on the average amount of meat cooked on the different equipment types per week. Emissions from french fries are based on the amount of frozen potatoes sold in the US. In 2014, commercial cooking in the US, Puerto Rico, and US Virgin Islands resulted in approximately 15,200 tons of VOC, 34,916 tons of CO, 101,853 tons of primary PM10, and 94,511 tons of primary PM25 emissions.

For this source category, the following SCCs are assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2302002100 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking – Charbroiling | Conveyorized Charbroiling |
| 2302002200 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking – Charbroiling | Under-fired Charbroiling |
| 2302003000 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking – Frying | Deep Fat Frying |
| 2302003100 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking – Frying | Flat Griddle Frying |
| 2302003200 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking – Frying | Clamshell Griddle Frying |

# Overview of Calculations

The calculations for estimating the emissions from commercial cooking involve first estimating the amount of meat and french fries cooked on various cooking devices in each county. These data are estimated using the number of restaurants, by specific restaurant type, from the Dun & Bradstreet (D&B) Hoovers Database1 and assumptions concerning the percent of those restaurants with specific cooking devices, the number of devices per restaurant, and the amount of meat cooked per device from a California Air Resources Board (CARB) sponsored survey.2 The amount of french fries cooked by the foodservice industry is from a report prepared for Potatoes USA.3 The total amount of meat or french fries cooked on each device is multiplied by emissions factors for CAPS including, VOC, CO, PM10 and PM25, and various HAPs to estimate emissions of these pollutants from commercial cooking. Sources of data and calculations for the amount of meat cooked on each device are discussed in section C. The process of allocating data on the amount of french fries cooked to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from commercial cooking is discussed in section G.

# Activity Data

The activity data for this source category is the amount of meat and potatoes cooked on each type of cooking device in each county. These amounts are estimated based on the number of restaurants in a county that use commercial cooking equipment, the percent of restaurants with each type of cooking device, the average number of cooking devices per restaurant, and the average amount of meat or potatoes cooked on each device.

Data concerning the number of restaurants in each county are from the Dun & Bradstreet (D&B) Hoovers Database. Hoovers data are proprietary and were purchased by EPA for use in the NEI; EPA provides users with aggregated data on county level restaurants by type. The relevant restaurants pulled from the Hoovers Database and their primary SIC codes are listed in Table 1. The first 4 digits of the SIC Code refer to eating places, the last 2 digits are added by D&B to further classify the restaurants.

Table 1. Hoovers Database Restaurant Types

| Restaurant Type | Primary SIC Code |
| --- | --- |
| Ethnic Food | 5812-01 |
| Fast Food | 5812-03 |
| Family | 5812-05 |
| Seafood | 5812-07 |
| Steak & BBQ | 5812-08 |

The number of restaurants by type in each county, pulled from the Hoovers database, is then multiplied by the fraction of restaurants by type with commercial cooking equipment in order to calculate the number of restaurants with the specific cooking devices in each county (Table 2). The data on cooking devices and meat cooked are from a survey on charbroiling activity in the state of California.

Table 2. Percent of Restaurants with Each Type of Cooking Device

| Restaurant Type | Conveyorized Char-broilers | Underfired Char-broilers | Deep-Fat Fryers | Flat Griddles | Clamshell Griddles |
| --- | --- | --- | --- | --- | --- |
| Ethnic | 3.5 | 47.5 | 81.9 | 62.7 | 4.0 |
| Fast Food | 18.6 | 30.8 | 96.8 | 51.9 | 14.7 |
| Family | 10.1 | 60.9 | 91.4 | 82.9 | 1.4 |
| Seafood | 0.0 | 52.6 | 100.0 | 36.8 | 10.5 |
| Steak & BBQ | 6.9 | 55.2 | 82.8 | 89.7 | 0.0 |

Source: Reference 2, Table 4

| Rt,c,d=Rt,c×Fract,d |  |
| --- | --- |

Where:

Rt,c,e	=	Number of type t restaurants in county c with cooking device d

Rt,c	=	Number of type t restaurants in county c

Fract,e	=	Fraction of type t restaurants with cooking device d

The number of restaurants in each county with cooking devices are then multiplied by the average number of cooking devices by restaurant type (Table 3), from the same California Survey dataset, to calculate the total number of cooking devices.

Table 3. Average Number of Devices by Restaurant Type*

| Restaurant Type | Conveyorized Char-broilers | Underfired Char-broilers | Deep-Fat Fryers | Flat Griddles | Clamshell Griddles |
| --- | --- | --- | --- | --- | --- |
| Ethnic | 1.62 | 1.54 | 1.63 | 1.88 | 1.80 |
| Fast Food | 1.07 | 1.58 | 3.10 | 1.43 | 2.09 |
| Family | 1.71 | 1.29 | 2.34 | 2.03 | - |
| Seafood | - | 1.10 | 2.47 | 1.11 | 1.50 |
| Steak & BBQ | - | 1.63 | 2.42 | 1.35 | - |

*Only includes restaurants with at least one piece of the equipment. Source: Reference 2, Table 5.

| Dt,c,d=Rt,c,d×Et,d |  |
| --- | --- |

Where:

Dt,c,d	=	Total number of cooking device d  in county c from type t restaurants

Rt,c,d	=	Number of type t restaurants in county c with cooking device d

Et,d	=	Average number of cooking device d at type t restaurants

The number of cooking devices in each restaurant type from equation 2 are summed across restaurant types to estimate the total number of cooking devices in each county.

| Dc,d=tDt,c,d |  |
| --- | --- |

Where:

Dc,d	=	Total number of cooking devices d from all restaurants in county c

Dt,c,d	=	Total number of cooking device d in restaurant type t in county c

The total number of cooking devices in each county is used to determine the amount of meat cooked in that county. The average amount of meat cooked on each cooking device is listed in Table 4.

Table 4. Average Amount of Meat Cooked per Year on Each Cooking Device (tons)

| Meat Type | Conveyorized Char-broilers | Underfired Char-broilers | Deep-Fat Fryers | Flat Griddles | Clamshell Griddles |
| --- | --- | --- | --- | --- | --- |
| Steak | 6.1 | 4.7 | 4.7 | 4.3 | 2.4 |
| Hamburger | 20.7 | 7.0 | 7.1 | 9.4 | 34.2 |
| Poultry | 10.7 | 8.4 | 14.9 | 5.2 | 5.7 |
| Pork | 1.5 | 3.8 | 1.5 | 2.9 | 3.1 |
| Seafood | 3.1 | 3.7 | 4.1 | 2.4 | 16.4 |
| Other | - | 1.1 | 7.1 | 1.5 | - |

Source: Reference 2, Table 13

| Mi,d,c=Dc,d×mi,d |  |
| --- | --- |

Where:

Mi,d,c	=	Total amount of meat type i cooked on device d in county c, in tons

Dc,d	=	Total number of cooking device d from all restaurants in county c

mi,d	=	Average amount of meat type i cooked on device d, in tons

The amount of french fries cooked in each county is calculated based on the amount of frozen potatoes used in the foodservice industry. According to a report prepared for Potatoes USA, 5,977 million pounds of frozen potatoes were used in the food service industry in 2017. Frozen potatoes used in limited service restaurants account for approximately 74% of the total, and those used in full service restaurants account for the remaining 26%.

The process used to distribute the national amount of french fries cooked to the county-level is discussed in section D.

# Allocation Procedure

In 2017, 5,977 million pounds of frozen potatoes were used in limited and full service restaurants in the U.S.3 In order to allocate this value to the county-level, fractions of the number of limited and full service restaurants in each county are used. To create these fractions, it is assumed that limited service restaurants are D&B classified fast food restaurants and full services restaurants are represented by all other D&B restaurant codes. County-level fast food and other restaurants are summed, and then divided by the national number of fast food or other restaurants in order to develop the county-level fractions.

| RFraclim,c=Rlim,cRlim,US |  |
| --- | --- |
| RFracfull,c=Rfull,cRfull,US |  |

Where:

RFraclim,c	=	Fraction of limited service restaurants in county c

RFracfull,c	=	Fraction of full service restaurants in county c

Rlim,c	=	The number of limited service restaurants in county c

Rfull,c	=	The number of full service restaurants in county c

Rlim,US	=	The number of limited service restaurants in the U.S.

Rfull,US	=	The number of full service restaurants in the U.S.

The fraction of limited and full service restaurants in each county is then used to distribute the amount of frozen potatoes cooked. Approximately 4,414 million pounds of frozen potatoes were used in limited service restaurants in the US in 2017 and 1,563 million pounds were used in full service restaurants.3

| Flim,c=RFraclim,c×flim,US÷2000 lbs per ton |  |
| --- | --- |
| Ffull,c=RFracfull,c×ffull,US÷2000 lbs per ton |  |

Where:

Flim,c	=	Amount of french fries cooked in limited service restaurants in county c, in tons

Ffull,c	=	Amount of french fries cooked in full service restaurants in county c, in ton

RFraclim,c	=	Fraction of limited service restaurants in county c

RFracfull,c	=	Fraction of full service restaurants in county c

flim,US	=	Amount of french fries cooked in limited service restaurants in the U.S., in lbs.

ffull,US	=	Amount of french fries cooked in full service restaurants in the U.S., in lbs.

The amount of french fries cooked in limited and full service restaurants are then summed to the county level.

| Fall,c=Flim,c+Ffull,c |  |
| --- | --- |

Where:

Fall,c	=	Amount of french fries cooked in county c, in tons

Flim,c	=	Amount of french fries cooked in limited service restaurants in county c, in tons

Ffull,c	=	Amount of french fries cooked in full service restaurants in county c, in tons

# Emissions Factors

Emissions factors for CAPs from commercial cooking are reported in Table 6. CAP emissions factors are taken from the article Emissions from Charbroiling and Grilling of Chicken and Beef, and a South Coast Air Quality Management District Report (SCAQMD).   According to the most recent PM Augmentation tool, Primary PM is equal to Filterable PM and there are assumed to be no condensible PM emissions from commercial cooking. Emissions factors for HAPs from commercial cooking are reported in Table 7. HAP emissions factors are also from Emissions from Charbroiling and Grilling of Chicken and Beef,4 and an EPA report on emissions from street vendor cooking devices.

# Controls

There are no controls assumed for this category.

# Emissions

To calculate emissions of CAPs, the total amount of meat and potatoes cooked on each cooking device in each county is multiplied by the appropriate emissions factor (listed in Table 6). The amount of french fries cooked is converted from pounds to tons, and all emissions are converted to tons.

| Ep, i,d,c= Mi,d,c×EFp,i,d÷2000 lbs per ton |  |
| --- | --- |
| Ep,f,d,c= Fall,c×EFp,f,d÷2000 lbs per ton |  |

Where:

Ep,i,d,c	=	Annual emissions of pollutant p from cooking meat type i on device d in county c, in tons

Ep,f,d,c	=	Annual emissions of pollutant p from cooking french fries, f, on device d in county c, in tons

Mi,d,c	=	Total amount of meat type i cooked on device d in county c, in tons

Fall,c	=	Total amount of french fries cooked in county c, in tons

EFp,i,d	=	Emissions factor for pollutant p, in lbs. of pollutant per ton of meat type i cooked on device d

EFp,f,d	=	Emissions factor for pollutant p, in lbs. of pollutant per ton of french fries cooked on device d

Emissions of HAPs are also calculated by multiplying an emissions factor (Table 7) by the amount of meat cooked on each cooking device. Note that cooking of french fries does not result in HAP emissions. For HAPs, no conversion factor is needed and emissions are reported in pounds.

| Ep, i,d,c= Mi,d,c×EFp,i,d |  |
| --- | --- |

Where:

Ep,i,d,c	=	Annual emissions of pollutant p from cooking meat type i on device d in county c, in pounds

Mi,d,c	=	Total amount of meat type i cooked on device d in county c, in tons

EFp,i,d	=	Emissions factor for pollutant p, in lbs. of pollutant per ton of meat type i cooked on device d

The emissions are summed for all types of meat and french fries to estimate the total emissions from each cooking device type in each county.

| Ep,d,c=iEp,i,d,c+Ep,f,d,c |  |
| --- | --- |

Where:

Ep,d,c	=	Total annual emissions of pollutant p from cooking device d in county c

Ep,i,d,c	=	Annual emissions of pollutant p from cooking meat type i on device d in county c

Ep,f,d,c	=	Annual emissions of pollutant p from cooking french fries, f, on device d in county c

# Point Source Subtraction

There are no point source-specific SCCs for commercial cooking; therefore point source subtraction is not performed for this category.

# Sample Calculations

Table 5 lists sample calculations to determine the VOC emissions from commercial cooking on flat griddles in Apache County, Arizona. The first two equations use fast food restaurants as an example, and equations 4 and 10 use hamburgers as an example. However, these calculations would need to be repeated to calculate values for all restaurant and meat types.

Table 5. Sample calculations for VOC emissions from commercial cooking on Flat Griddles in Apache County, AZ.

| Eq. # | Equation | Values for Apache County, AZ | Result |
| --- | --- | --- | --- |
| 1 | Rt,c,d=Rt,c×Fract,d | 6 fast food rest. ×51.9% with flat griddles | 3.114 fast food restaurants in Apache County, AZ with flat griddles |
| 2 | Dt,c,d=Rt,c,d×Et,d | 3.114 fast food rest. with flat griddles ×1.43 flat griddles per rest. | 4.45 flat griddles in fast food restaurants in Apache County, AZ |
| 3 | Dc,d=tDt,c,d | Flat griddles in Apache County, AZ | 9.5 flat griddles in all restaurants in Apache County, AZ |
| 4 | Mi,d,c=Dc,d×mi,d | 9.5 flat griddles in Apache ×9.4 tons of hamburger cooked on flat griddles | 89.3 tons of hamburger cooked on flat griddles in Apache County, AZ |
| 5 | RFraclim,c=Rlim,cRlim,US | N/A | Equation is for deep-fat fryers; example is for flat griddles |
| 6 | RFracfull,c=Rfull,cRfull,US | N/A | Equation is for deep-fat fryers; example is for flat griddles |
| 7 | Flim,c=RFraclim,c×flim,US÷2000 lbs per ton | N/A | Equation is for deep-fat fryers; example is for flat griddles |
| 8 | Ffull,c=RFracfull,c×ffull,US÷2000 lbs per ton | N/A | Equation is for deep-fat fryers; example is for flat griddles |
| 9 | Fall,c=Flim,c+Ffull,c | N/A | Equation is for deep-fat fryers; example is for flat griddles |
| 10 | Ep, i,d,c= Mi,d,c×EFp,i,d÷2000 lbs per ton | 89.3 tons of hamburger cooked×0.14 lbs. VOC per ton hamburger÷2000 lbs. per ton | 0.00625 tons VOC emissions from cooking hamburgers on flat griddles in Apache County, AZ |
| 11 | Ep,f,d,c= Fall,c×EFp,f,d÷2000 lbs per ton | N/A | Equation is for deep-fat fryers; example is for flat griddles |
| 12 | Ep, i,d,c= Mi,d,c×EFp,i,d | NA | Equation is for HAPs; example is for VOC |
| 13 | Ep,d,c=iEp,i,d,c+ Ep,f,d,c | VOC emissions in Apache County | 0.04 tons VOC emissions from flat griddles in Apache County, AZ |

# Changes from 2014 Methodology

The methodology used to calculate commercial cooking emissions for the 2014 v2 NEI used data on the number of restaurants in each county, according to US NAICS codes, to grow emissions data from the 2002 NEI commercial cooking category. This was completed as EPA did not have access to the more specific D&B data on restaurants in each county. For the 2017 NEI, EPA has access to the D&B data and is therefore using the 2002 NEI methodology (which is also used by the state of California).

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Emissions from Puerto Rico and the Virgin Islands are calculated using the same methodology described above.

# References

Table 6. Criteria Air Pollutant Emissions Factors for Commercial Cooking

| Cooking Device | Meat | Pollutant | Pollutant Code | Emission Factor (original) | Emission Factor Units (original) | Emission Factor (converted) | Emission Factor Units (converted) | Emissions Factor Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conveyorized Char-broilers | Steak | Carbon Monoxide | CO | 8.29 | mg/kg | 16.58 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |  |
|  |  | Volatile Organic Compounds | VOC | 2.27 | lb./1000 lb. | 4.55 | lbs./ton | Ref. 5; Table XIII; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 7.40 | lb./1000 lb. | 14.80 | lbs./ton | Ref. 5; Table XIII |  |
|  |  | Primary PM2.5 | PM25-PRI | 7.30 | lb./1000 lb. | 14.60 | lbs./ton | Ref. 5; Table XIII |  |
|  | Hamburger | Carbon Monoxide | CO | 8.29 | mg/kg | 16.58 | lbs./ton | Ref. 4; Table 2 |  |
|  |  | Volatile Organic Compounds | VOC | 2.27 | lb./1000 lb. | 4.55 | lbs./ton | Ref. 5; Table XIII; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 7.40 | lb./1000 lb. | 14.80 | lbs./ton | Ref. 5; Table XIII |  |
|  |  | Primary PM2.5 | PM25-PRI | 7.30 | lb./1000 lb. | 14.60 | lbs./ton | Ref. 5; Table XIII |  |
|  | Poultry | Carbon Monoxide | CO | 4.84 | mg/kg | 9.68 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |  |
|  |  | Volatile Organic Compounds | VOC | 1.82 | lb./1000 lb. | 3.65 | lbs./ton | Ref. 5; Table V; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 10.47 | lb./1000 lb. | 20.93 | lbs./ton | Ref. 5; Table V |  |
|  |  | Primary PM2.5 | PM25-PRI | 9.90 | lb./1000 lb. | 19.80 | lbs./ton | Ref. 5; Table V |  |
|  | Pork | Carbon Monoxide | CO | 4.84 | mg/kg | 9.68 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |  |
|  |  | Volatile Organic Compounds | VOC | 1.82 | lb./1000 lb. | 3.65 | lbs./ton | Ref. 5; Table V; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 10.47 | lb./1000 lb. | 20.93 | lbs./ton | Ref. 5; Table V |  |
|  |  | Primary PM2.5 | PM25-PRI | 9.90 | lb./1000 lb. | 19.80 | lbs./ton | Ref. 5; Table V |  |
|  | Seafood | Volatile Organic Compounds | VOC | 0.38 | lb./1000 lb. | 0.76 | lbs./ton | Ref. 5; Table VI; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 3.27 | lb./1000 lb. | 6.53 | lbs./ton | Ref. 5; Table VI |  |
|  |  | Primary PM2.5 | PM25-PRI | 3.20 | lb./1000 lb. | 6.40 | lbs./ton | Ref. 5; Table VI |  |
| Underfired Char-broilers | Steak | Carbon Monoxide | CO | 4.97 | mg/kg | 9.94 | lbs./ton | Ref. 4; Table 2 |  |
|  |  | Volatile Organic Compounds | VOC | 0.86 | lb./1000 lb. | 1.71 | lbs./ton | Ref. 5; Table XVII; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 17.20 | lb./1000 lb. | 34.40 | lbs./ton | Ref. 5; Table XVII |  |
|  |  | Primary PM2.5 | PM25-PRI | 16.80 | lb./1000 lb. | 33.60 | lbs./ton | Ref. 5; Table XVII |  |
|  | Hamburger | Carbon Monoxide | CO | 13.72 | mg/kg | 27.44 | lbs./ton | Ref. 4; Table 2 |  |
|  |  | Volatile Organic Compounds | VOC | 3.94 | lb./1000 lb. | 7.89 | lbs./ton | Ref. 5; Table IV; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 32.67 | lb./1000 lb. | 65.33 | lbs./ton | Ref. 5; Table IV |  |
|  |  | Primary PM2.5 | PM25-PRI | 31.90 | lb./1000 lb. | 63.80 | lbs./ton | Ref. 5; Table IV |  |
|  | Poultry | Carbon Monoxide | CO | 4.84 | mg/kg | 9.68 | lbs./ton | Ref. 4; Table 2 |  |
|  |  | Volatile Organic Compounds | VOC | 1.82 | lb./1000 lb. | 3.65 | lbs./ton | Ref. 5; Table V; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 10.47 | lb./1000 lb. | 20.93 | lbs./ton | Ref. 5; Table V |  |
|  |  | Primary PM2.5 | PM25-PRI | 9.90 | lb./1000 lb. | 19.80 | lbs./ton | Ref. 5; Table V |  |
|  | Pork | Carbon Monoxide | CO | 4.84 | mg/kg | 9.68 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |  |
|  |  | Volatile Organic Compounds | VOC | 1.82 | lb./1000 lb. | 3.65 | lbs./ton | Ref. 5; Table V; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 10.47 | lb./1000 lb. | 20.93 | lbs./ton | Ref. 5; Table V |  |
|  |  | Primary PM2.5 | PM25-PRI | 9.90 | lb./1000 lb. | 19.80 | lbs./ton | Ref. 5; Table V |  |
|  | Seafood | Volatile Organic Compounds | VOC | 0.38 | lb./1000 lb. | 0.76 | lbs./ton | Ref. 5; Table VI; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 3.27 | lb./1000 lb. | 6.53 | lbs./ton | Ref. 5; Table VI |  |
|  |  | Primary PM2.5 | PM25-PRI | 3.20 | lb./1000 lb. | 6.40 | lbs./ton | Ref. 5; Table VI |  |
|  | Other | Carbon Monoxide | CO | 4.97 | mg/kg | 9.94 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |  |
|  |  | Volatile Organic Compounds | VOC | 0.86 | lb./1000 lb. | 1.71 | lbs./ton | Ref. 5; Table XVII; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 17.20 | lb./1000 lb. | 34.40 | lbs./ton | Ref. 5; Table XVII |  |
|  |  | Primary PM2.5 | PM25-PRI | 16.80 | lb./1000 lb. | 33.60 | lbs./ton | Ref. 5; Table XVII |  |
| Deep-Fat Fryers | Poultry | Volatile Organic Compounds | VOC | 0.12 | lb./1000 lb. | 0.25 | lbs./ton | Ref. 5; Table VIII; VOC measured as ROG |  |
|  | Pork | Volatile Organic Compounds | VOC | 0.12 | lb./1000 lb. | 0.25 | lbs./ton | Ref. 5; Table VIII; VOC measured as ROG |  |
|  | Seafood | Volatile Organic Compounds | VOC | 0.14 | lb./1000 lb. | 0.28 | lbs./ton | Ref. 5; Table IX; VOC measured as ROG |  |
|  | Potatoes | Volatile Organic Compounds | VOC | 0.21 | lb./1000 lb. | 0.42 | lbs./ton | Ref. 5; Table VII; VOC measured as ROG |  |
| Flat Griddles | Steak | Carbon Monoxide | CO | 0.38 | mg/kg | 0.76 | lbs./ton | Ref. 4; Table 2; Hamburger flat griddle EFs |  |
|  |  | Volatile Organic Compounds | VOC | 0.07 | lb./1000 lb. | 0.14 | lbs./ton | Ref. 5; Table X; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 5.00 | lb./1000 lb. | 10.00 | lbs./ton | Ref. 5; Table X |  |
|  |  | Primary PM2.5 | PM25-PRI | 3.80 | lb./1000 lb. | 7.60 | lbs./ton | Ref. 5; Table X |  |
|  | Hamburger | Carbon Monoxide | CO | 0.38 | mg/kg | 0.76 | lbs./ton | Ref. 4; Table 2 |  |
|  |  | Volatile Organic Compounds | VOC | 0.07 | lb./1000 lb. | 0.14 | lbs./ton | Ref. 5; Table X; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 5.00 | lb./1000 lb. | 10.00 | lbs./ton | Ref. 5; Table X |  |
|  |  | Primary PM2.5 | PM25-PRI | 3.80 | lb./1000 lb. | 7.60 | lbs./ton | Ref. 5; Table X |  |
|  | Poultry | Carbon Monoxide | CO | 0.45 | mg/kg | 0.90 | lbs./ton | Ref. 4; Table 2 |  |
|  |  | Volatile Organic Compounds | VOC | 0.40 | lb./1000 lb. | 0.79 | lbs./ton | Ref. 5; Table XI; VOC measured as ROG |  |
|  | Pork | Carbon Monoxide | CO | 0.45 | mg/kg | 0.90 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |  |
|  |  | Volatile Organic Compounds | VOC | 0.40 | lb./1000 lb. | 0.79 | lbs./ton | Ref. 5; Table XI; VOC measured as ROG |  |
|  | Seafood | Volatile Organic Compounds | VOC | 0.11 | lb./1000 lb. | 0.21 | lbs./ton | Ref. 5; Table XII; VOC measured as ROG |  |
|  | Other | Carbon Monoxide | CO | 0.38 | mg/kg | 0.76 | lbs./ton | Ref. 4; Table 2; Hamburger flat griddle EFs |  |
|  |  | Volatile Organic Compounds | VOC | 0.07 | lb./1000 lb. | 0.14 | lbs./ton | Ref. 5; Table X; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 5.00 | lb./1000 lb. | 10.00 | lbs./ton | Ref. 5; Table X |  |
|  |  | Primary PM2.5 | PM25-PRI | 3.80 | lb./1000 lb. | 7.60 | lbs./ton | Ref. 5; Table X |  |
| Clamshell Griddles | Steak | Volatile Organic Compounds | VOC | 0.01 | lb./1000 lb. | 0.03 | lbs./ton | Ref. 5; Table XVI; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 0.85 | lb./1000 lb. | 1.70 | lbs./ton | Ref. 5; Table XVI |  |
|  |  | Primary PM2.5 | PM25-PRI | 0.72 | lb./1000 lb. | 1.44 | lbs./ton | Ref. 5; Table XVI |  |
|  | Hamburger | Volatile Organic Compounds | VOC | 0.01 | lb./1000 lb. | 0.03 | lbs./ton | Ref. 5; Table XVI; VOC measured as ROG |  |
|  |  | Primary PM10 | PM10-PRI | 0.85 | lb./1000 lb. | 1.70 | lbs./ton | Ref. 5; Table XVI |  |
|  |  | Primary PM2.5 | PM25-PRI | 0.72 | lb./1000 lb. | 1.44 | lbs./ton | Ref. 5; Table XVI |  |
|  | Poultry | Volatile Organic Compounds | VOC | 0.08 | lb./1000 lb. | 0.15 | lbs./ton | Calculated by dividing the Flat Griddle Poultry VOC EF by the ratio of the Flat Griddle to Clamshell Griddle Hamburger VOC EFs |  |
|  | Pork | Volatile Organic Compounds | VOC | 0.08 | lb./1000 lb. | 0.15 | lbs./ton | Calculated by dividing the Flat Griddle Poultry VOC EF by the ratio of the Flat Griddle to Clamshell Griddle Hamburger VOC EFs |  |
|  | Seafood | Volatile Organic Compounds | VOC | 0.02 | lb./1000 lb. | 0.04 | lbs./ton | Calculated by dividing the Flat Griddle Seafood VOC EF by the ratio of the Flat Griddle to Clamshell Griddle Hamburger VOC EFs |  |
| ROG = Reactive Organic Gases |  |  |  |  |  |  |  |  |

Table 7. Hazardous Air Pollutant Emissions Factors for Commercial Cooking

| Cooking Device | Meat | Pollutant | Pollutant Code | Emission Factor (original) | Emission Factor Units (original) | Emission Factor (converted) | Emission Factor Units (converted) | Reference & Table No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conveyorized Char-broilers | Steak | Acenaphthene | 83329 | 0.28 | mg/kg | 0.00056 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Acenaphthylene | 208968 | 4.89 | mg/kg | 0.00978 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Anthracene | 120127 | 0.91 | mg/kg | 0.00182 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Benz[a]anthracene | 56553 | 0.22 | mg/kg | 0.00044 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Benzo[a]Pyrene | 50328 | 0.17 | mg/kg | 0.00034 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.16 | mg/kg | 0.00032 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Biphenyl | 92524 | 2.43 | mg/kg | 0.00486 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Fluoranthene | 206440 | 0.88 | mg/kg | 0.00176 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Fluorene | 86737 | 1.09 | mg/kg | 0.00218 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Naphthalene | 91203 | 23.04 | mg/kg | 0.04608 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Phenanthrene | 85018 | 4.88 | mg/kg | 0.00976 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | Pyrene | 129000 | 1.15 | mg/kg | 0.0023 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  |  | PAH, Total | 130498292 | 49.05 | mg/kg | 0.0981 | lbs./ton | Ref. 4; Table 2; Hamburger auto-char EFs |
|  | Hamburger | Acenaphthene | 83329 | 0.28 | mg/kg | 0.00056 | lbs./ton | Ref. 4; Table 2 |
|  |  | Acenaphthylene | 208968 | 4.89 | mg/kg | 0.00978 | lbs./ton | Ref. 4; Table 2 |
|  |  | Anthracene | 120127 | 0.91 | mg/kg | 0.00182 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benz[a]anthracene | 56553 | 0.22 | mg/kg | 0.00044 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[a]Pyrene | 50328 | 0.17 | mg/kg | 0.00034 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.16 | mg/kg | 0.00032 | lbs./ton | Ref. 4; Table 2 |
|  |  | Biphenyl | 92524 | 2.43 | mg/kg | 0.00486 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluoranthene | 206440 | 0.88 | mg/kg | 0.00176 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluorene | 86737 | 1.09 | mg/kg | 0.00218 | lbs./ton | Ref. 4; Table 2 |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2 |
|  |  | Naphthalene | 91203 | 23.04 | mg/kg | 0.04608 | lbs./ton | Ref. 4; Table 2 |
|  |  | Phenanthrene | 85018 | 4.88 | mg/kg | 0.00976 | lbs./ton | Ref. 4; Table 2 |
|  |  | Pyrene | 129000 | 1.15 | mg/kg | 0.0023 | lbs./ton | Ref. 4; Table 2 |
|  |  | PAH, Total | 130498292 | 49.05 | mg/kg | 0.0981 | lbs./ton | Ref. 4; Table 2 |
|  | Poultry | 4-nitrophenol | 100027 | 0.0066 | g/kg | 0.0132 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acenaphthene | 83329 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acenaphthylene | 208968 | 2.06 | mg/kg | 0.00412 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acetaldehyde | 75070 | 0.282 | g/kg | 0.564 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acetophenone | 98862 | 0.002425 | g/kg | 0.00485 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Anthracene | 120127 | 0.88 | mg/kg | 0.00176 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benz[a]anthracene | 56553 | 0.34 | mg/kg | 0.00068 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzene | 71432 | 0.504 | g/kg | 1.008 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Benzo[a]Pyrene | 50328 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Biphenyl | 92524 | 0.91 | mg/kg | 0.00182 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Cresol/Cresylic Acid (Mixed Isomers) | 1319773 | 0.00511 | g/kg | 0.01022 | lbs./ton | Ref. 6; Table 3-3; Sum of cresol EFs for marinated chicken test conditions, estimated emissions of meat only |
|  |  | Dibutyl Phthalate | 84742 | 0.00128 | g/kg | 0.00256 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethyl Benzene | 100414 | 0.04 | g/kg | 0.08 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethylene Dichloride | 107062 | 0.014 | g/kg | 0.028 | lbs./ton | REF. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Fluoranthene | 206440 | 1.28 | mg/kg | 0.00256 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Fluorene | 86737 | 0.72 | mg/kg | 0.00144 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Formaldehyde | 50000 | 0.393 | g/kg | 0.786 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Naphthalene | 91203 | 8.75 | mg/kg | 0.0175 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenanthrene | 85018 | 3.46 | mg/kg | 0.00692 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenol | 108952 | 0.02255 | g/kg | 0.0451 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Propionaldehyde | 123386 | 0.0755 | g/kg | 0.151 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Pyrene | 129000 | 1.8 | mg/kg | 0.0036 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Styrene | 100425 | 0.1895 | g/kg | 0.379 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Toluene | 108883 | 0.1995 | g/kg | 0.399 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | PAH, Total | 130498292 | 28.35 | mg/kg | 0.0567 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Xylenes (Mixed Isomers) | 1330207 | 0.059 | g/kg | 0.118 | lbs./ton | Ref. 6; Table 3-1; Sum of xylene EFs for marinated chicken test conditions, estimated emissions of meat only |
|  | Pork | 4-nitrophenol | 100027 | 0.0066 | g/kg | 0.0132 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acenaphthene | 83329 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acenaphthylene | 208968 | 2.06 | mg/kg | 0.00412 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acetaldehyde | 75070 | 0.282 | g/kg | 0.564 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acetophenone | 98862 | 0.002425 | g/kg | 0.00485 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Anthracene | 120127 | 0.88 | mg/kg | 0.00176 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benz[a]anthracene | 56553 | 0.34 | mg/kg | 0.00068 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzene | 71432 | 0.504 | g/kg | 1.008 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Benzo[a]Pyrene | 50328 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Biphenyl | 92524 | 0.91 | mg/kg | 0.00182 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Cresol/Cresylic Acid (Mixed Isomers) | 1319773 | 0.00511 | g/kg | 0.01022 | lbs./ton | Ref. 6; Table 3-3; Sum of cresol EFs for marinated chicken test conditions, estimated emissions of meat only |
|  |  | Dibutyl Phthalate | 84742 | 0.00128 | g/kg | 0.00256 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethyl Benzene | 100414 | 0.04 | g/kg | 0.08 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethylene Dichloride | 107062 | 0.014 | g/kg | 0.028 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Fluoranthene | 206440 | 1.28 | mg/kg | 0.00256 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Fluorene | 86737 | 0.72 | mg/kg | 0.00144 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Formaldehyde | 50000 | 0.393 | g/kg | 0.786 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Naphthalene | 91203 | 8.75 | mg/kg | 0.0175 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenanthrene | 85018 | 3.46 | mg/kg | 0.00692 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenol | 108952 | 0.02255 | g/kg | 0.0451 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Propionaldehyde | 123386 | 0.0755 | g/kg | 0.151 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Pyrene | 129000 | 1.8 | mg/kg | 0.0036 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Styrene | 100425 | 0.1895 | g/kg | 0.379 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Toluene | 108883 | 0.1995 | g/kg | 0.399 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | PAH, Total | 130498292 | 28.35 | mg/kg | 0.0567 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Xylenes (Mixed Isomers) | 1330207 | 0.059 | g/kg | 0.118 | lbs./ton | Ref. 6; Table 3-1; Sum of xylene EFs for marinated chicken test conditions, estimated emissions of meat only |
| Underfired Char-broilers | Steak | Acenaphthene | 83329 | 0.15 | mg/kg | 0.0003 | lbs./ton | Ref. 4; Table 2 |
|  |  | Acenaphthylene | 208968 | 4.28 | mg/kg | 0.00856 | lbs./ton | Ref. 4; Table 2 |
|  |  | Acetaldehyde | 75070 | 0.251 | g/kg | 0.502 | lbs./ton | Ref. 6; Table 3-4; Average of beef test conditions, estimated emissions of meat only |
|  |  | Acetophenone | 98862 | 0.001829 | g/kg | 0.003658 | lbs./ton | Ref. 6; Table 3-3; Average of beef test conditions, estimated emissions of meat only |
|  |  | Anthracene | 120127 | 1.03 | mg/kg | 0.00206 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzene | 71432 | 0.3915 | g/kg | 0.783 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Benz[a]anthracene | 56553 | 0.11 | mg/kg | 0.00022 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[a]Pyrene | 50328 | 0.07 | mg/kg | 0.00014 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2 |
|  |  | Biphenyl | 92524 | 1.54 | mg/kg | 0.00308 | lbs./ton | Ref. 4; Table 2 |
|  |  | Cresol/Cresylic Acid (Mixed Isomers) | 1319773 | 0.0026825 | g/kg | 0.005365 | lbs./ton | Ref. 6; Table 3-3; Sum of cresol EFs for beef test conditions, estimated emissions of meat only |
|  |  | Dibutyl Phthalate | 84742 | 0.0010255 | g/kg | 0.002051 | lbs./ton | Ref. 6; Table 3-3; Average of beef test conditions, estimated emissions of meat only |
|  |  | Ethyl Benzene | 100414 | 0.026 | g/kg | 0.052 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Ethylene Dichloride | 107062 | 0.0165 | g/kg | 0.033 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Fluoranthene | 206440 | 1.28 | mg/kg | 0.00256 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluorene | 86737 | 1.17 | mg/kg | 0.00234 | lbs./ton | Ref. 4; Table 2 |
|  |  | Formaldehyde | 50000 | 0.337 | g/kg | 0.674 | lbs./ton | Ref. 6; Table 3-4; Average of beef test conditions, estimated emissions of meat only |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.05 | mg/kg | 0.0001 | lbs./ton | Ref. 4; Table 2 |
|  |  | Naphthalene | 91203 | 14.8 | mg/kg | 0.0296 | lbs./ton | Ref. 4; Table 2 |
|  |  | Phenanthrene | 85018 | 5.31 | mg/kg | 0.01062 | lbs./ton | Ref. 4; Table 2 |
|  |  | Phenol | 108952 | 0.01605 | g/kg | 0.0321 | lbs./ton | Ref. 6; Table 3-3; Average of beef test conditions, estimated emissions of meat only |
|  |  | Propionaldehyde | 123386 | 0.0675 | g/kg | 0.135 | lbs./ton | Ref. 6; Table 3-4; Average of beef test conditions, estimated emissions of meat only |
|  |  | Pyrene | 129000 | 1.56 | mg/kg | 0.00312 | lbs./ton | Ref. 4; Table 2 |
|  |  | Styrene | 100425 | 0.151 | g/kg | 0.302 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Toluene | 108883 | 0.1535 | g/kg | 0.307 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | PAH, Total | 130498292 | 39.12 | mg/kg | 0.07824 | lbs./ton | Ref. 4; Table 2 |
|  |  | Xylenes (Mixed Isomers) | 1330207 | 0.045 | g/kg | 0.09 | lbs./ton | Ref. 6; Table 3-3; Sum of xylene EFs for beef test conditions, estimated emissions of meat only |
|  | Hamburger | Acenaphthene | 83329 | 0.15 | mg/kg | 0.0003 | lbs./ton | Ref. 4; Table 2 |
|  |  | Acenaphthylene | 208968 | 4.24 | mg/kg | 0.00848 | lbs./ton | Ref. 4; Table 2 |
|  |  | Anthracene | 120127 | 0.94 | mg/kg | 0.00188 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benz[a]anthracene | 56553 | 0.22 | mg/kg | 0.00044 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[a]Pyrene | 50328 | 0.15 | mg/kg | 0.0003 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.17 | mg/kg | 0.00034 | lbs./ton | Ref. 4; Table 2 |
|  |  | Biphenyl | 92524 | 1.72 | mg/kg | 0.00344 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluoranthene | 206440 | 1.4 | mg/kg | 0.0028 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluorene | 86737 | 1.26 | mg/kg | 0.00252 | lbs./ton | Ref. 4; Table 2 |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2 |
|  |  | Naphthalene | 91203 | 19.11 | mg/kg | 0.03822 | lbs./ton | Ref. 4; Table 2 |
|  |  | Phenanthrene | 85018 | 4.88 | mg/kg | 0.00976 | lbs./ton | Ref. 4; Table 2 |
|  |  | PAH, Total | 130498292 | 45.6 | mg/kg | 0.0912 | lbs./ton | Ref. 4; Table 2 |
|  |  | Pyrene | 129000 | 1.9 | mg/kg | 0.0038 | lbs./ton | Ref. 4; Table 2 |
|  | Poultry | 4-nitrophenol | 100027 | 0.0066 | g/kg | 0.0132 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acenaphthene | 83329 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acenaphthylene | 208968 | 2.06 | mg/kg | 0.00412 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acetaldehyde | 75070 | 0.282 | g/kg | 0.564 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acetophenone | 98862 | 0.002425 | g/kg | 0.00485 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Anthracene | 120127 | 0.88 | mg/kg | 0.00176 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benz[a]anthracene | 56553 | 0.34 | mg/kg | 0.00068 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzene | 71432 | 0.504 | g/kg | 1.008 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Benzo[a]Pyrene | 50328 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Biphenyl | 92524 | 0.91 | mg/kg | 0.00182 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Cresol/Cresylic Acid (Mixed Isomers) | 1319773 | 0.00511 | g/kg | 0.01022 | lbs./ton | Ref. 6; Table 3-3; Sum of cresol EFs for marinated chicken test conditions, estimated emissions of meat only |
|  |  | Dibutyl Phthalate | 84742 | 0.00128 | g/kg | 0.00256 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethyl Benzene | 100414 | 0.04 | g/kg | 0.08 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethylene Dichloride | 107062 | 0.014 | g/kg | 0.028 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Fluoranthene | 206440 | 1.28 | mg/kg | 0.00256 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Fluorene | 86737 | 0.72 | mg/kg | 0.00144 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Formaldehyde | 50000 | 0.393 | g/kg | 0.786 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Naphthalene | 91203 | 8.75 | mg/kg | 0.0175 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenanthrene | 85018 | 3.46 | mg/kg | 0.00692 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenol | 108952 | 0.02255 | g/kg | 0.0451 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Propionaldehyde | 123386 | 0.0755 | g/kg | 0.151 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Pyrene | 129000 | 1.8 | mg/kg | 0.0036 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Styrene | 100425 | 0.1895 | g/kg | 0.379 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Toluene | 108883 | 0.1995 | g/kg | 0.399 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | PAH, Total | 130498292 | 28.35 | mg/kg | 0.0567 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Xylenes (Mixed Isomers) | 1330207 | 0.059 | g/kg | 0.118 | lbs./ton | Ref. 6; Table 3-1; Sum of xylene EFs for marinated chicken test conditions, estimated emissions of meat only |
|  | Pork | 4-nitrophenol | 100027 | 0.0066 | g/kg | 0.0132 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acenaphthene | 83329 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acenaphthylene | 208968 | 2.06 | mg/kg | 0.00412 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Acetaldehyde | 75070 | 0.282 | g/kg | 0.564 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Acetophenone | 98862 | 0.002425 | g/kg | 0.00485 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Anthracene | 120127 | 0.88 | mg/kg | 0.00176 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benz[a]anthracene | 56553 | 0.34 | mg/kg | 0.00068 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzene | 71432 | 0.504 | g/kg | 1.008 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Benzo[a]Pyrene | 50328 | 0.1 | mg/kg | 0.0002 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Biphenyl | 92524 | 0.91 | mg/kg | 0.00182 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Cresol/Cresylic Acid (Mixed Isomers) | 1319773 | 0.00511 | g/kg | 0.01022 | lbs./ton | Ref. 6; Table 3-3; Sum of cresol EFs for marinated chicken test conditions, estimated emissions of meat only |
|  |  | Dibutyl Phthalate | 84742 | 0.00128 | g/kg | 0.00256 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethyl Benzene | 100414 | 0.04 | g/kg | 0.08 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Ethylene Dichloride | 107062 | 0.014 | g/kg | 0.028 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Fluoranthene | 206440 | 1.28 | mg/kg | 0.00256 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Fluorene | 86737 | 0.72 | mg/kg | 0.00144 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Formaldehyde | 50000 | 0.393 | g/kg | 0.786 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Naphthalene | 91203 | 8.75 | mg/kg | 0.0175 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenanthrene | 85018 | 3.46 | mg/kg | 0.00692 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Phenol | 108952 | 0.02255 | g/kg | 0.0451 | lbs./ton | Ref. 6; Table 3-3; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Propionaldehyde | 123386 | 0.0755 | g/kg | 0.151 | lbs./ton | Ref. 6; Table 3-4; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Pyrene | 129000 | 1.8 | mg/kg | 0.0036 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Styrene | 100425 | 0.1895 | g/kg | 0.379 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | Toluene | 108883 | 0.1995 | g/kg | 0.399 | lbs./ton | Ref. 6; Table 3-1; Average of marinated chicken test conditions, estimated emissions of meat only |
|  |  | PAH, Total | 130498292 | 28.35 | mg/kg | 0.0567 | lbs./ton | Ref. 4; Table 2; Chicken under-char EFs |
|  |  | Xylenes (Mixed Isomers) | 1330207 | 0.059 | g/kg | 0.118 | lbs./ton | Ref. 6; Table 3-1; Sum of xylene EFs for marinated chicken test conditions, estimated emissions of meat only |
|  | Other | Acenaphthene | 83329 | 0.15 | mg/kg | 0.0003 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Acenaphthylene | 208968 | 4.28 | mg/kg | 0.00856 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Acetaldehyde | 75070 | 0.251 | g/kg | 0.502 | lbs./ton | Ref. 6; Table 3-4; Average of beef test conditions, estimated emissions of meat only |
|  |  | Acetophenone | 98862 | 0.001829 | g/kg | 0.003658 | lbs./ton | Ref. 6; Table 3-3; Average of beef test conditions, estimated emissions of meat only |
|  |  | Anthracene | 120127 | 1.03 | mg/kg | 0.00206 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Benzene | 71432 | 0.3915 | g/kg | 0.783 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Benz[a]anthracene | 56553 | 0.11 | mg/kg | 0.00022 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Benzo[a]Pyrene | 50328 | 0.07 | mg/kg | 0.00014 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0.09 | mg/kg | 0.00018 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Biphenyl | 92524 | 1.54 | mg/kg | 0.00308 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Cresol/Cresylic Acid (Mixed Isomers) | 1319773 | 0.0026825 | g/kg | 0.005365 | lbs./ton | Ref. 6; Table 3-3; Sum of cresol EFs for beef test conditions, estimated emissions of meat only |
|  |  | Dibutyl Phthalate | 84742 | 0.0010255 | g/kg | 0.002051 | lbs./ton | Ref. 6; Table 3-3; Average of beef test conditions, estimated emissions of meat only |
|  |  | Ethyl Benzene | 100414 | 0.026 | g/kg | 0.052 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Ethylene Dichloride | 107062 | 0.0165 | g/kg | 0.033 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Fluoranthene | 206440 | 1.28 | mg/kg | 0.00256 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Fluorene | 86737 | 1.17 | mg/kg | 0.00234 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Formaldehyde | 50000 | 0.337 | g/kg | 0.674 | lbs./ton | Ref. 6; Table 3-4; Average of beef test conditions, estimated emissions of meat only |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0.05 | mg/kg | 0.0001 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Naphthalene | 91203 | 14.8 | mg/kg | 0.0296 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Phenanthrene | 85018 | 5.31 | mg/kg | 0.01062 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Phenol | 108952 | 0.01605 | g/kg | 0.0321 | lbs./ton | Ref. 6; Table 3-3; Average of beef test conditions, estimated emissions of meat only |
|  |  | Propionaldehyde | 123386 | 0.0675 | g/kg | 0.135 | lbs./ton | Ref. 6; Table 3-4; Average of beef test conditions, estimated emissions of meat only |
|  |  | Pyrene | 129000 | 1.56 | mg/kg | 0.00312 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Styrene | 100425 | 0.151 | g/kg | 0.302 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | Toluene | 108883 | 0.1535 | g/kg | 0.307 | lbs./ton | Ref. 6; Table 3-1; Average of beef test conditions, estimated emissions of meat only |
|  |  | PAH, Total | 130498292 | 39.12 | mg/kg | 0.07824 | lbs./ton | Ref. 4; Table 2; Steak under-char EFs |
|  |  | Xylenes (Mixed Isomers) | 1330207 | 0.045 | g/kg | 0.09 | lbs./ton | Ref. 6; Table 3-3; Sum of xylene EFs for beef test conditions, estimated emissions of meat only |
| Flat Griddles | Steak | Acenaphthene | 83329 | 0.02 | mg/kg | 0.00004 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Acenaphthylene | 208968 | 0.16 | mg/kg | 0.00032 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Anthracene | 120127 | 0.17 | mg/kg | 0.00034 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Benz[a]anthracene | 56553 | 0.07 | mg/kg | 0.00014 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Benzo[a]Pyrene | 50328 | 0.02 | mg/kg | 0.00004 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Biphenyl | 92524 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Fluoranthene | 206440 | 0.86 | mg/kg | 0.00172 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Fluorene | 86737 | 0.21 | mg/kg | 0.00042 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Naphthalene | 91203 | 0.61 | mg/kg | 0.00122 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Phenanthrene | 85018 | 2.07 | mg/kg | 0.00414 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Pyrene | 129000 | 1.15 | mg/kg | 0.0023 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | PAH, Total | 130498292 | 7.96 | mg/kg | 0.01592 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  | Hamburger | Acenaphthene | 83329 | 0.02 | mg/kg | 0.00004 | lbs./ton | Ref. 4; Table 2 |
|  |  | Acenaphthylene | 208968 | 0.16 | mg/kg | 0.00032 | lbs./ton | Ref. 4; Table 2 |
|  |  | Anthracene | 120127 | 0.17 | mg/kg | 0.00034 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benz[a]anthracene | 56553 | 0.07 | mg/kg | 0.00014 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[a]Pyrene | 50328 | 0.02 | mg/kg | 0.00004 | lbs./ton | Ref. 4; Table 2 |
|  |  | Biphenyl | 92524 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluoranthene | 206440 | 0.86 | mg/kg | 0.00172 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluorene | 86737 | 0.21 | mg/kg | 0.00042 | lbs./ton | Ref. 4; Table 2 |
|  |  | Naphthalene | 91203 | 0.61 | mg/kg | 0.00122 | lbs./ton | Ref. 4; Table 2 |
|  |  | Phenanthrene | 85018 | 2.07 | mg/kg | 0.00414 | lbs./ton | Ref. 4; Table 2 |
|  |  | Pyrene | 129000 | 1.15 | mg/kg | 0.0023 | lbs./ton | Ref. 4; Table 2 |
|  |  | PAH, Total | 130498292 | 7.96 | mg/kg | 0.01592 | lbs./ton | Ref. 4; Table 2 |
|  | Poultry | Acenaphthene | 83329 | 0.05 | mg/kg | 0.0001 | lbs./ton | Ref. 4; Table 2 |
|  |  | Acenaphthylene | 208968 | 0.13 | mg/kg | 0.00026 | lbs./ton | Ref. 4; Table 2 |
|  |  | Anthracene | 120127 | 0.44 | mg/kg | 0.00088 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benz[a]anthracene | 56553 | 0.12 | mg/kg | 0.00024 | lbs./ton | Ref. 4; Table 2 |
|  |  | Benzo[a]Pyrene | 50328 | 0.01 | mg/kg | 0.00002 | lbs./ton | Ref. 4; Table 2 |
|  |  | Biphenyl | 92524 | 0.13 | mg/kg | 0.00026 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluoranthene | 206440 | 0.62 | mg/kg | 0.00124 | lbs./ton | Ref. 4; Table 2 |
|  |  | Fluorene | 86737 | 0.18 | mg/kg | 0.00036 | lbs./ton | Ref. 4; Table 2 |
|  |  | Naphthalene | 91203 | 1 | mg/kg | 0.002 | lbs./ton | Ref. 4; Table 2 |
|  |  | Phenanthrene | 85018 | 1.87 | mg/kg | 0.00374 | lbs./ton | Ref. 4; Table 2 |
|  |  | Pyrene | 129000 | 0.82 | mg/kg | 0.00164 | lbs./ton | Ref. 4; Table 2 |
|  |  | PAH, Total | 130498292 | 9.51 | mg/kg | 0.01902 | lbs./ton | Ref. 4; Table 2 |
|  | Pork | Acenaphthene | 83329 | 0.05 | mg/kg | 0.0001 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Acenaphthylene | 208968 | 0.13 | mg/kg | 0.00026 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Anthracene | 120127 | 0.44 | mg/kg | 0.00088 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Benz[a]anthracene | 56553 | 0.12 | mg/kg | 0.00024 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Benzo[a]Pyrene | 50328 | 0.01 | mg/kg | 0.00002 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Benzo[g,h,i]Perylene | 191242 | 0 | mg/kg | 0 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Biphenyl | 92524 | 0.13 | mg/kg | 0.00026 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Fluoranthene | 206440 | 0.62 | mg/kg | 0.00124 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Fluorene | 86737 | 0.18 | mg/kg | 0.00036 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Indeno[1,2,3-c,d]Pyrene | 193395 | 0 | mg/kg | 0 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Naphthalene | 91203 | 1 | mg/kg | 0.002 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Phenanthrene | 85018 | 1.87 | mg/kg | 0.00374 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | Pyrene | 129000 | 0.82 | mg/kg | 0.00164 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  |  | PAH, Total | 130498292 | 9.51 | mg/kg | 0.01902 | lbs./ton | Ref. 4; Table 2; Chicken griddle EFs |
|  | Other | Acenaphthene | 83329 | 0.02 | mg/kg | 0.00004 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Acenaphthylene | 208968 | 0.16 | mg/kg | 0.00032 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Anthracene | 120127 | 0.17 | mg/kg | 0.00034 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Benz[a]anthracene | 56553 | 0.07 | mg/kg | 0.00014 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Benzo[a]Pyrene | 50328 | 0.02 | mg/kg | 0.00004 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Biphenyl | 92524 | 0.06 | mg/kg | 0.00012 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Fluoranthene | 206440 | 0.86 | mg/kg | 0.00172 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Fluorene | 86737 | 0.21 | mg/kg | 0.00042 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Naphthalene | 91203 | 0.61 | mg/kg | 0.00122 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Phenanthrene | 85018 | 2.07 | mg/kg | 0.00414 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | Pyrene | 129000 | 1.15 | mg/kg | 0.0023 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |
|  |  | PAH, Total | 130498292 | 7.96 | mg/kg | 0.01592 | lbs./ton | Ref. 4; Table 2; Hamburger griddle EFs |

Dun and Bradstreet Hoovers. 2018. http://www.hoovers.com/

Public Research Institute. 2001. Charbroiling Activity Estimation. Prepared for the California Air Resources Board and California EPA.  https://www.arb.ca.gov/research/apr/reports/l943.pdf

Technomic. 2017. Volumetric Assessment of the Foodservice Potato Market. Prepared for Potatoes USA.

McDonald, J., B. Zielinska, E. Fujita, J. Sagebiel, J. Chow, and J. Watson. 2003. “Emissions from Charbroiling and Grilling of Chicken and Beef.” Journal of Air & Waste Management Association. 53:185-194.

Norbeck, Joseph. 1997. Further Development of Emission Test Methods and Development of Emission Factors for Various Commercial Cooking Operations. Prepared for the South Coast Air Quality Management District.

US EPA. 1999. Emissions from Street Vendor Cooking Devices (Charcoal Grilling). Prepared by ARCADIS Geraghty & Miller.