DUST KICKED UP BY ANIMALS

# Source Category Description

Dust kicked up by animals refers to the dust emitted from different types of livestock feet. These emissions are primarily considered to be made by cattle and swine, but poultry emissions of dust are also examined. Fugitive dust emissions from animals are estimated for PM10-PRI, PM10-FIL, PM25-PRI, and PM25-FIL. In 2014, dust from animals in the US, Puerto Rico, and US Virgin Islands resulted in 1,078,49.73 tons of PM10-PRI and 204,225.20 tons of PM25-PRI emissions.

For this source category, the following SCCs were assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2805001000 | Miscellaneous Area Sources | Agriculture Production – Livestock | Beef cattle - finishing operations on feedlots (drylots) | Dust Kicked-up by Hooves |
| 2805001010 | Miscellaneous Area Sources | Agriculture Production – Livestock | Dairy Cattle | Dust Kicked-up by Hooves |
| 2805001020 | Miscellaneous Area Sources | Agriculture Production – Livestock | Broilers | Dust Kicked-up by Feet |
| 2805001030 | Miscellaneous Area Sources | Agriculture Production – Livestock | Layers | Dust Kicked-up by Feet |
| 2805001040 | Miscellaneous Area Sources | Agriculture Production – Livestock | Swine | Dust Kicked-up by Hooves |
| 2805001050 | Miscellaneous Area Sources | Agriculture Production – Livestock | Turkeys | Dust Kicked-up by Feet |

# Overview of Calculations

The calculations for estimating the emissions from dust kicked up by animals involves multiplying the livestock counts by an emission factor for PM10-PRI, PM10-FIL, PM25-PRI, and PM25-FIL. Sources of data and calculations for the livestock counts are discussed in section C. The process of allocating activity data to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from dust from animals is discussed in section G.

# Activity Data

The activity data for this source category is based on livestock counts (average annual number of standing head) and population information by state and county used to develop U.S. EPA’s Greenhouse Gas Inventory. This data set is derived from multiple data sets from the United States Department of Agriculture (USDA), particularly the National Agricultural Statistics Service (NASS) survey and census. The USDA NASS survey dataset, which represents latest available, 2017 national livestock data, is used to obtain the livestock counts for as many counties as possible across the United States.  For a full description of the GHG livestock population estimation methodology, refer to the above referenced citation for the EPA’s GHG inventory document.

Generally, counties not specifically included in the NASS survey data set (e.g., due to business confidentially reasons) were gap-filled based on the difference in the reported state total animal counts and the sum of all county-level reported animal counts. State-level data on animal counts from the GHG inventory were distributed to counties based on the proportion of animal counts in those counties from the 2017 NASS census.

| Pa,c,2017=Pa,s,2017×ra,c,2012 |  |
| --- | --- |

Where:

Pa,c,2017	=	Estimated population of animal type a in county c

Pa,s,2017	=	NASS survey reported state-level population of animal type a in state s

ra,c,2012	=	Ratio of animal county- to state-level animal counts from the 2012 NASS census for animals type a in county c

# Allocation Procedure

The USDA survey reports the livestock counts at the county level for many counties, so no allocation is necessary. The procedure for gap-filling missing county-level data using state-level data is described in section C.

# Emissions Factors

Emission factors for dust from animals are reported in Table 1. Dust emission factors are from different literature articles for each livestock type. No references for PM25 emission factors were found in the literature for Beef Cattle. To calculate PM25 for Beef Cattle, the Dairy Cattle PM10 to PM25 ratio of 4.81 from this tool was used and is based on ratios in the PM Augmentation tool. In general, if the study calculated an emission factor, it was converted to units of ton/year/head and is used in this tool. If the study did not calculate an emission factor, then it was calculated by dividing the emission rate in tons per year by animal units according to an equation used by the NRC’s Scientific Basis for Estimating Air Emissions from Animal Feeding Operations: Interim Report. Animal units are calculated by multiplying an equivalent factor by the livestock population according to an equation from the Wisconsin Department of Natural Resources. Equivalent factors are listed in Table 2. After converting the AU to number of animals, assuming that 1 AU is equivalent to 500 kilograms, the emission factor is calculated in units of tons per year per head.

Table 1. Emissions Factors for Dust Kicked-up by Animals

| Animal Type | Pollutant | EF (ton/year/1000head) | EF (ton/year/head) | References |
| --- | --- | --- | --- | --- |
| Dairy Cattle | PM10-PRI | 3.86685175 | 0.003866852 | Ref 5; Ref 6; Ref 7; Ref 8 |
| Dairy Cattle | PM10-FIL | 3.86685175 | 0.003866852 | Ref 5; Ref 6; Ref 7; Ref 8 |
| Dairy Cattle | PM25-PRI | 0.803721667 | 0.000803722 | Ref 5; Ref 6; Ref 8 |
| Dairy Cattle | PM25-FIL | 0.803721667 | 0.000803722 | Ref 5; Ref 6; Ref 8 |
| Beef Cattle | PM10-PRI | 11.46679018 | 0.01146679 | Ref 10 |
| Beef Cattle | PM10-FIL | 11.46679018 | 0.01146679 | Ref 10 |
| Beef Cattle | PM25-PRI | 0.803722 | 0.000803722 | Beef cattle EF adjusted by PM10 to PM25 ratio of 4.81 |
| Beef Cattle | PM25-FIL | 0.803722 | 0.000803722 | Beef cattle EF adjusted by PM10 to PM25 ratio of 4.81 |
| Swine | PM10-PRI | 0.803607373 | 0.000803607 | Ref 11; Ref 8 |
| Swine | PM10-FIL | 0.803607373 | 0.000803607 | Ref 11; Ref 8 |
| Swine | PM25-PRI | 0.008562274 | 8.56227E-06 | Ref 8 |
| Swine | PM25-FIL | 0.008562274 | 8.56227E-06 | Ref 8 |
| Laying Hens | PM10-PRI | 0.027138046 | 2.7138E-05 | Ref 12; Ref 13; Ref 14; Ref 15; Ref 16; Ref 17; Ref 8; Ref 18 |
| Laying Hens | PM10-FIL | 0.027138046 | 2.7138E-05 | Ref 12; Ref 13; Ref 14; Ref 15; Ref 16; Ref 17; Ref 8; Ref 18 |
| Laying Hens | PM25-PRI | 0.003368297 | 3.3683E-06 | Ref 14; Ref 15; Ref 16; Ref 17; Ref 8; Ref 18 |
| Laying Hens | PM25-FIL | 0.003368297 | 3.3683E-06 | Ref 14; Ref 15; Ref 16; Ref 17; Ref 8; Ref 18 |
| Broiler | PM10-PRI | 0.023119233 | 2.31192E-05 | Ref 20; Ref 21; Ref 22; Ref 23; Ref 18; Ref 8 |
| Broiler | PM10-FIL | 0.023119233 | 2.31192E-05 | Ref 20; Ref 21; Ref 22; Ref 23; Ref 18; Ref 8 |
| Broiler | PM25-PRI | 0.002004275 | 2.00427E-06 | Ref 20; Ref 22; Ref 23; Ref 18; Ref 8 |
| Broiler | PM25-FIL | 0.002004275 | 2.00427E-06 | Ref 20; Ref 22; Ref 23; Ref 18; Ref 8 |
| Turkey | PM10-PRI | 0.32615159 | 0.000326152 | Ref 24; Ref 7; Ref 8 |
| Turkey | PM10-FIL | 0.32615159 | 0.000326152 | Ref 24; Ref 7; Ref 8 |
| Turkey | PM25-PRI | 0.02623985 | 2.62399E-05 | Ref 24; Ref 8 |
| Turkey | PM25-FIL | 0.02623985 | 2.62399E-05 | Ref 24; Ref 8 |

Table 2. AU Equivalent Factors for each Animal Type

| Animal Type | Specification | AU Equivalent Factor |
| --- | --- | --- |
| Cattle | Dairy/Beef Calves (under 400 lbs) | 0.20 |
| Dairy Cattle | Milking & Dry Cows | 1.40 |
| Dairy Cattle | Heifers (800-1200 lbs) | 1.10 |
| Dairy Cattle | Heifers (400-800 lbs) | 0.60 |
| Beef Cattle | Steers or Cows (400 lbs to market) | 1.00 |
| Beet Cattle | Bulls | 1.40 |
| Cattle | Veal Calves | 0.50 |
| Swine | Pigs (up to 55 lbs) | 0.10 |
| Swine | Pigs (55 lbs to market) | 0.40 |
| Swine | Sows | 0.40 |
| Swine | Boars | 0.50 |
| Chicken | Layers – non-liquid manure system | 0.01 |
| Chicken | Broilers/pullets – non-liquid manure system | 0.005 |
| Chicken | Bird – liquid manure system | 0.033 |
| Ducks | Liquid manure system | 0.2 |
| Ducks | Non-liquid manure system | 0.01 |
| Turkeys | Turkey | 0.018 |
| Sheep | Sheep | 0.1 |
| Horses | Horses | 2 |

# Controls

There are no controls assumed for this category.

# Emissions

For each animal type and pollutant the livestock count is multiplied by the emissions factors in Table 1 to estimate emissions.

| Ep,c,a= Pa,c×EFp,a |  |
| --- | --- |

Where:

Ep,c,a	=	Annual emissions of pollutant p in county c for animal type a, in tons per year

Pa,c	=	Population of livestock for animal type a in county c

EFp,a	=	Emissions factor for pollutant p and animal type a, in tons per year per head

# Point Source Subtraction

There are no point source-specific SCCs for composting; therefore point source subtraction is not performed for this category.

# Sample Calculations

Table 3 lists sample calculations to determine PM10-PRI emissions from dust kicked up by animals in Apache County, Arizona. The sample calculations use swine as an example, but the calculations would need to be repeated to calculate values for all livestock types.

Table 3. Sample calculations for PM10-PRI emissions from dust kicked up by animals in Apache County, AZ.

| Eq. # | Equation | Values for Apache County, AZ | Result |
| --- | --- | --- | --- |
| 1 | Pa,c,2017=Pa,s,2017×ra,c,2012 | N/A | 2017 swine population is available by county and does not need to be calculated using 2012 NASS Census ratios. |
| 2 | Ep,c,a= Pa,c×EFp,a | 5,813 swine in Apache × 0.000803607 tons PM10 per head of swine | 4.67 tons PM10-PRI emissions from swine in Apache County, AZ |

# Changes from 2014 Methodology

When calculating livestock populations, heifers and calves are now included in population totals for dairy cattle in addition to mature dairy cows. For poultry-layers, pullets (young hens) are now included in population totals.

In addition, in the 2014 methodology, emissions were summed across all animal types and reported as a single SCC. In 2017, emissions are reported in separate SCCs for each individual animal type.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Emissions from Puerto Rico are calculated using the same method described above

# References

U.S. EPA. 2018. Inventory of Greenhouse Gas Emissions and Sinks, 1990-2016. Chapter 5.2, Manure Management. EPA 430-R-18-003.

United States Department of Agriculture. 2020. National Agricultural Statistics Service Quick Stats. https://quickstats.nass.usda.gov/

U.S. EPA. 2017. Air Emissions Inventories, PM Augmentation. https://19january2017snapshot.epa.gov/air-emissions-inventories/pm-augmentation_.html

National Research Council. 2002. The Scientific Basis for Estimating Air Emissions from Animal Feeding Operations: Interim Report. Washington, DC: The National Academies Press. https://doi.org/10.17226/10391.

State of Wisconsin Department of Natural Resources. 2012. Animal Unit Calculation Worksheet Form 3400-025A. https://dnr.wi.gov/files/PDF/forms/3400/3400-025A.pdf

5 Joo, H.S., Ndegwa, P.M., Heber, A.J., Ni, J.Q., Bogan, B.W., Ramirez-Dorronsoro, J.C. and Cortus, E.L., 2013. Particulate matter dynamics in naturally ventilated freestall dairy barns. Atmospheric Environment, 69, pp.182-190.

6 Marchant, C.C., Moore, K.D., Wojcik, M.D., Martin, R.S., Pfeiffer, R.L., Prueger, J.H. and Hatfield, J.L., 2011. Estimation of dairy particulate matter emission rates by lidar and inverse modeling. Transactions of the ASABE, 54(4), pp.1453-1463.

7 Hinz, T., Linke, S., Karlowski, J., Myczko, R., Kuczynski, T. and Berk, J., 2007. PM emissions in and from force-ventilated turkey and dairy cattle houses as factor of health and the environment. Gert-Jan Monteny, p.305.

8 Winkel, A., Mosquera, J., Koerkamp, P.W.G., Ogink, N.W. and Aarnink, A.J., 2015. Emissions of particulate matter from animal houses in the Netherlands. Atmospheric Environment, 111, pp.202-212.

9 Takai, H., Pedersen, S., Johnsen, J.O., Metz, J.H.M., Koerkamp, P.G., Uenk, G.H., Phillips, V.R., Holden, M.R., Sneath, R.W., Short, J.L. and White, R.P., 1998. Concentrations and emissions of airborne dust in livestock buildings in Northern Europe. Journal of agricultural engineering research, 70(1), pp.59-77.

10 Bonifacio, H.F., Maghirang, R.G., Auvermann, B.W., Razote, E.B., Murphy, J.P. and Harner III, J.P., 2012. Particulate matter emission rates from beef cattle feedlots in Kansas—Reverse dispersion modeling. Journal of the Air & Waste Management Association, 62(3), pp.350-361.

11Costa, A. and Guarino, M., 2009b. Definition of yearly emission factor of dust and greenhouse gases through continuous measurements in swine husbandry. Atmospheric Environment, 43(8), pp.1548-1556.

12 Costa, A. and Guarino, M., 2009a. Particulate matter concentration and emission factor in three different laying hen housing systems. Journal of Agricultural Engineering, 40(3), pp.15-24.

13 Zhao, L., Lim, T.T., Sun, H. and Diehl, C.A., 2005. Particulate matter emissions from a Ohio belt-battery layer barn. In 2005 ASAE Annual Meeting (p. 1). American Society of Agricultural and Biological Engineers.

14 Fabbri, C., L. Valli, M. Guarina, A. Costa, and V. Mazzotta. 2007. Ammonia, methane, nitrous oxide, and particulate matter emissions from two different buildings for laying hens. Biosystems Eng. 97(4): 441‐455.

Zhao, L., Lim, T.T., Sun, H. and Diehl, C.A., 2005. Particulate matter emissions from a Ohio belt-battery layer barn. In 2005 ASAE Annual Meeting (p. 1). American Society of Agricultural and Biological Engineers.

15 Lim, T.T., Heber, A.J., Ni, J.Q., Gallien, J.X. and Xin, H., 2003. Air quality measurements at a laying hen house: Particulate matter concentrations and emissions. In Air Pollution from Agricultural Operations-III (p. 249). American Society of Agricultural and Biological Engineers.

16 Li, S., Li, H., Xin, H. and Burns, R.T., 2011a. Particulate matter concentrations and emissions of a high-rise layer house in Iowa. Transactions of the ASABE, 54(3), pp.1093-1101.

17 Li, S., Li, H., Xin, H. and Burns, R.T., 2009. Particulate matter emissions from a high-rise layer house in Iowa. In 2009 Reno, Nevada, June 21-June 24, 2009 (p. 1). American Society of Agricultural and Biological Engineers.

18 Demmers, T.G.M., Saponja, A., Thomas, R., Phillips, G.J., McDonald, A.G., Stagg, S., Bowry, A. and Nemitz, E., 2010. Dust and ammonia emissions from UK poultry houses. In XVII World Congress of the International Commission of Agricultural and Biosystems Engineering (CIGR) CIGR, Québec city, Canada.

19 Qi, R., H. B. Manbeck, and R. G. Maghirang. 1992. Dust net generation rate in a poultry layer house. Trans. ASAE 35(5): 1639‐1645.

20 Hayes, M.D., Xin, H., Li, H., Shepherd, T., Zhao, Y. and Stinn, J.P., 2012. Ammonia, greenhouse gas, and particulate matter concentrations and emissions of aviary layer houses in the Midwestern USA. In 2012 IX International Livestock Environment Symposium (ILES IX) (p. 3). American Society of Agricultural and Biological Engineers

21 Lacey, R.E., Redwine, J.S. and Parnell, C.B., 2003. Particulate matter and ammonia emission factors for tunnel–ventilated broiler production houses in the Southern US. Transactions of the ASAE, 46(4), p.1203.

22 Roumeliotis, T.S. and Van Heyst, B.J., 2007. Size fractionated particulate matter emissions from a broiler house in Southern Ontario, Canada. Science of the Total Environment, 383(1), pp.174-182.

23 Burns, R.T., Li, H., Moody, L., Xin, H., Gates, R., Overhults, D. and Earnest, J., 2008. Quantification of particulate emissions from broiler houses in the southeastern United States. In Livestock Environment VIII, 31 August–4 September 2008, Iguassu Falls, Brazil (p. 15). American Society of Agricultural and Biological Engineers.

24 Li, H., Xin, H., Burns, R.T., Jacobson, L.D., Noll, S., Hoff, S.J., Harmon, J.D., Koziel, J.A. and Hetchler, B.P., 2011b. Air emissions from tom and hen turkey houses in the US Midwest. Transactions of the ASABE, 54(1), pp.305-314.