INDUSTRIAL, COMMERCIAL, AND INSTITUTIONAL FUEL CONSUMPTION

# Source Category Description

Industrial, Commercial, and Institutional (ICI) fuel consumption includes emissions from boilers, engines, and other combustion sources from the industrial, commercial, and institutional sectors that are not reported as point sources. This source category includes emissions from combustion of coal, distillate fuel oil, residual fuel oil, kerosene, liquefied petroleum gas (LPG), natural gas, and wood. In 2014, ICI emissions in the US, Puerto Rico, and US Virgin Islands resulted in approximately 160,000 tons of PM25-PRI emissions.

For this source category, the following SCCs are assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2102001000 | Stationary Source Fuel Combustion | Industrial | Anthracite Coal | Total: All Boiler Types |
| 2102002000 | Stationary Source Fuel Combustion | Industrial | Bituminous/Subbituminous Coal | Total: All Boiler Types |
| 2102004001 | Stationary Source Fuel Combustion | Industrial | Distillate Oil | All Boiler Types |
| 2102004002 | Stationary Source Fuel Combustion | Industrial | Distillate Oil | All IC Engine Types |
| 2102005000 | Stationary Source Fuel Combustion | Industrial | Residual Oil | Total: All Boiler Types |
| 2102006000 | Stationary Source Fuel Combustion | Industrial | Natural Gas | Total: Boilers and IC Engines |
| 2102007000 | Stationary Source Fuel Combustion | Industrial | Liquified Petroleum Gas (LPG) | Total: All Boiler Types |
| 2102008000 | Stationary Source Fuel Combustion | Industrial | Wood | Total: All Boiler Types |
| 2102011000 | Stationary Source Fuel Combustion | Industrial | Kerosene | Total: All Boiler Types |
| 2103001000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Anthracite Coal | Total: All Boiler Types |
| 2103002000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Bituminous/Subbituminous Coal | Total: All Boiler Types |
| 2103004001 | Stationary Source Fuel Combustion | Commercial/ Institutional | Distillate Oil | Boilers |
| 2103004002 | Stationary Source Fuel Combustion | Commercial/ Institutional | Distillate Oil | IC Engines |
| 2103005000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Residual Oil | Total: All Boiler Types |
| 2103006000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Natural Gas | Total: Boilers and IC Engines |
| 2103007000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Liquified Petroleum Gas (LPG) | Total: All Combustor Types |
| 2103008000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Wood | Total: All Boiler Types |
| 2103011000 | Stationary Source Fuel Combustion | Commercial/ Institutional | Kerosene | Total: All Combustor Types |

# Overview of Calculations

The calculations for estimating emissions from the ICI sectors include estimating the total fuel consumption by sector in each state, using data from the Energy Information Administration (EIA) State Energy Data System (SEDS). Total fuel consumption is adjusted to account for fuel consumed by mobile sources in each sector and fuel used as an input to industrial processes but is not combusted. Fuel consumption from nonpoint sources in each state is determined by subtracting fuel consumption from point sources from total fuel consumption. Estimated nonpoint source fuel consumption in each state is distributed to the county level based on the proportion of employment in the industrial and commercial sectors.

Sources of data and adjustment calculations for fuel consumption are discussed in section C. The process of allocating activity data to the county level is discussed in section 0. Emissions factors are discussed in section E. The estimation of emissions from ICI fuel consumption is discussed in section G. The process of subtracting point source data is discussed in section H.

# Activity Data

The activity data for this source category is total fuel consumption in the industrial and commercial/institutional sectors. The default data for this category are obtained from the total 2017 state-level fuel consumption in each sector from EIA SEDS1 for all fuel types except distillate. Distillate fuel consumption is taken from EIA’s Form 821 data, which reports distillate sales by state and sector for 2016. State, local, and tribal (SLT) agencies are expected to submit state-level fuel consumption data from point sources in these sectors. The state-level point source fuel consumption is subtracted from the total fuel consumption to estimate the fuel consumption from nonpoint sources. The point source subtraction method is described in more detail in section H.

Total fuel consumption is adjusted to account for the fraction of fuel consumed by nonroad mobile sources, whose emissions are included in the nonroad inventory. This fraction is based on results from the National Mobile Inventory Model (NMIM), a precursor to EPA’s Motor Vehicle Emission Simulator (MOVES). This adjustment is particularly important for distillate fuel oil consumption. The ICI tool uses distillate consumption data from Form 821 rather than SEDS because Form 821 reports more detailed data by sector, and the ICI tool uses different stationary source fuel consumption assumptions by sector, including the industrial, commercial, farm, off-highway, and oil company sectors. Note that fuel consumption in the farm, off-highway, and oil company sectors are mapped to the industrial sector in the ICI tool. Assumptions about the fraction of fuel consumed by stationary sources are shown in an appendix.

The total fuel consumption is also adjusted to account for fuel used as an input to industrial processes where it is not combusted. These assumptions are based on the EIA Manufacturing Energy Consumption Survey (MECS), which reports both total fuel consumption and non-combustion use of fuel by Census region. Assumptions about non-combustion use of fuel are shown in Table 1. In some cases EIA withholds the regional-level data on non-combustion use of fuel because it is less than 0.5 million barrels. In these cases, a value of 0.25 million barrels is used as the amount of regional-level non-combustion use of fuels.

Note that the stationary source adjustment is performed for fuel consumption from both the industrial and commercial/institutional sectors, while the non-combustion use of fuel adjustment is performed only for fuel consumption in the industrial sector.

| AFf,s,x=TFf,s, x×SSf,s,x×(1-ncf,s,industrial) |  |
| --- | --- |

Where:

AFf,s,x	=	Consumption of fuel f by stationary sources in state s in sector x

TFf,s,x	=	Total consumption of fuel f in state s in sector x, from EIA SEDS

SSf,s,x	=	Fraction of fuel f consumed by stationary sources in state s in sector x

ncf,s,x	=	Fraction of fuel f used as an industrial input and is not combusted in state s in the industrial sector, from Table 1

Table 1. Assumptions about non-combustion use of fuel by fuel type and state.

| State | Coal | Distillate | LPG | Natural Gas | Residual Oil | Kerosene |
| --- | --- | --- | --- | --- | --- | --- |
| AK | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| AL | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| AR | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| AZ | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| CA | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| CO | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| CT | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| DC | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| DE | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| FL | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| GA | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| HI | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| IA | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| ID | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| IL | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| IN | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| KS | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| KY | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| LA | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| MA | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| MD | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| ME | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| MI | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| MN | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| MO | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| MS | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| MT | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| NC | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| ND | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| NE | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| NH | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| NJ | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| NM | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| NV | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| NY | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| OH | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| OK | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| OR | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| PA | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| RI | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| SC | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| SD | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| TN | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| TX | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| UT | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| VA | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| VT | 75.0% | 8.3% | 91.3% | 1.0% | 0.0% | 0.0% |
| WA | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |
| WI | 44.0% | 6.3% | 80.0% | 4.3% | 100.0% | 0.0% |
| WV | 29.4% | 11.1% | 98.9% | 13.3% | 81.8% | 0.0% |
| WY | 0.0% | 8.3% | 6.3% | 1.6% | 0.0% | 0.0% |

The SEDS data do not distinguish between anthracite and bituminous/subbituminous coal consumption estimates. The EIA table “Domestic Distribution of U.S. Coal by Destination State, Consumer, Origin and Method of Transportation” provides state-level coal distribution data for 2006 that is used to estimate the fraction of coal consumption that is anthracite and bituminous/subbituminous. Table 2 presents the anthracite and bituminous coal ratios for each state.

Table 2. Anthracite and Bituminous Coal Distribution for the Residential and Commercial Sectors

| State | Ratio of Bituminous | Ratio of Anthracite | State | Ratio of Bituminous | Ratio of Anthracite |
| --- | --- | --- | --- | --- | --- |
| Alabama | 1.000 | 0.000 | Montana | 1.000 | 0.000 |
| Alaska | 1.000 | 0.000 | Nebraska | 1.000 | 0.000 |
| Arizona | 0.814 | 0.186 | Nevada | 1.000 | 0.000 |
| Arkansas | 0.814 | 0.186 | New Hampshire | 0.000 | 1.000 |
| California | 1.000 | 0.000 | New Jersey | 0.000 | 1.000 |
| Colorado | 0.996 | 0.004 | New Mexico | 1.000 | 0.000 |
| Connecticut | 0.000 | 1.000 | New York | 0.600 | 0.400 |
| Delaware | 0.814 | 0.186 | North Carolina | 1.000 | 0.000 |
| Dist. Columbia | 1.000 | 0.000 | North Dakota | 1.000 | 0.000 |
| Florida | 0.814 | 0.186 | Ohio | 0.873 | 0.127 |
| Georgia | 1.000 | 0.000 | Oklahoma | 0.917 | 0.083 |
| Hawaii | 1.000 | 0.000 | Oregon | 1.000 | 0.000 |
| Idaho | 0.979 | 0.021 | Pennsylvania | 0.194 | 0.806 |
| Illinois | 0.998 | 0.002 | Rhode Island | 0.000 | 1.000 |
| Indiana | 0.947 | 0.053 | South Carolina | 0.997 | 0.003 |
| Iowa | 0.999 | 0.001 | South Dakota | 1.000 | 0.000 |
| Kansas | 1.000 | 0.000 | Tennessee | 0.994 | 0.006 |
| Kentucky | 0.998 | 0.002 | Texas | 0.814 | 0.186 |
| Louisiana | 1.000 | 0.000 | Utah | 1.000 | 0.000 |
| Maine | 0.000 | 1.000 | Vermont | 0.000 | 1.000 |
| Maryland | 0.929 | 0.071 | Virginia | 0.963 | 0.037 |
| Massachusetts | 0.500 | 0.500 | Washington | 1.000 | 0.000 |
| Michigan | 0.667 | 0.333 | West Virginia | 0.905 | 0.095 |
| Minnesota | 0.997 | 0.003 | Wisconsin | 0.991 | 0.009 |
| Mississippi | 1.000 | 0.000 | Wyoming | 1.000 | 0.000 |
| Missouri | 1.000 | 0.000 |  |  |  |

Source: Reference 4

The SEDS data on industrial and commercial coal consumption are split into consumption of anthracite and bituminous/subbituminous coal based on the ratios in Table 2.

| AFant/bit,s,x=AFcoal,s,x×Rant/bit,s |  |
| --- | --- |

Where:

AFant/bit,s,x	= 	Adjusted anthracite or bituminous coal consumption in state s in sector x

AFcoal,s,x		= 	Total adjusted coal consumption in state s in sector x, from equation 1

Rant/bit,s		= 	Ratio of anthracite or bituminous coal to total coal in state s, from Table 2

The EIA Form 821 data report total distillate consumption, but the NEI requires data separately on consumption by boilers and engines, because there are substantially different emissions factors for distillate boilers and engines. The ICI tool uses assumptions based on the EIA MECS3 and the EIA Commercial Building Energy Consumption Survey (CBECS). These data sources suggest that in the industrial sector, 60 percent of distillate consumption is by boilers and 40 percent by engines, and in the commercial sector, 95 percent is by boilers and 5 percent is by engines.

| AFboiler/engine,s,x=AFdistillate,s,x×Rboiler/engine,s,x |  |
| --- | --- |

Where:

AFboiler/engine,s,x		= 	Adjusted distillate consumption in boilers or engines state s in sector x

AFdistillate	s,x			= 	Total adjusted distillate consumption in state s in sector x, from equation 1

Rboiler/engine,s,x			= 	Ratio of distillate consumption by boilers or engines in state s in sector x

Following the adjustments to the total fuel consumption, the total fuel consumption data is also adjusted to subtract fuel consumption from point sources, which is accounted for in the point source inventory. Point source fuel consumption data by fuel type and sector is submitted by SLT agencies. This point source subtraction procedure is described in more detail in section H. The point source subtraction step is performed at the state level, and it is done before the allocation procedure discussed in section D and before the emissions calculations discussed in section G.

# Allocation Procedure

SEDS data are reported at the state level. Following the adjustments to the state level fuel consumption discussed in section C and the point source subtraction discussed below in section H, the estimated state-level nonpoint source activity data in each state is distributed to the county level based on employment in the industrial or commercial sector from the Census Bureau’s County Business Patterns. The adjusted nonpoint fuel consumption in each state is distributed to the county based on the proportion of employment in each county in each sector to the total employment at the state level in each sector.

| NPFf,c,x=NPFf,s,x×empc,xemps,x |  |
| --- | --- |

Where:

NPFf,c,x	= 	Adjusted nonpoint consumption of fuel f in county c in sector x

NPFf,s,x	= 	Adjusted nonpoint consumption of fuel f in state s in sector x, from equation 6

empc,x	=	Employment in county c in sector x

emps,x	=	Employment in state s in sector x

Employment in each sector is determined based on the crosswalk between North American Industrial Classification System (NAICS) codes and sectors, as shown in Table 6 in the appendix.

# Emissions Factors

The emissions factors for ICI sectors are from AP-42 and a spreadsheet developed in 2010 by EPA and the Eastern Regional Technical Advisory Committee. The emissions factors for ammonia are taken from one of two reports from EPA on ammonia emissions in the ICI sectors., The emissions factors for hazardous air pollutants from wood combustion in the ICI sectors are taken from EPA’s SPECIATE database.

Table 5 in the appendix lists all emissions factors for the ICI sectors.

# Controls

There are no controls assumed for this category. However, the ICI tool includes options for SLT agencies to submit pollutant-, SCC-, and county-specific control factors if needed. These control factors are a number between 0 and 1 that is multiplied by the emissions for that pollutant, SCC, and county. These factors allow SLT agencies to “fine tune” emissions estimates based on their understanding of how specific national and local rules combined with their penetration/effectiveness could lead to “composite-rule” emission factors for specific counties and pollutants. The relative difference between these “composite-rule” and default ICI tool emission factors can then be used to compute SCC-, county-, and pollutant-specific “controls.”

Alternatively, SLT agencies can adjust the emissions factors; however, this would affect the calculation of emissions for all counties in the state.

# Emissions

Emissions in each ICI sector are estimated by multiplying the county-level nonpoint source fuel consumption by the emission factors from Table 5.

| Ep,f,c,x= NPFf,c,x×EFp,f,x |  |
| --- | --- |

Where:

Ep,f,c,x	=	Annual emissions of pollutant p from fuel type f in county c in sector x

NPFf,c,x	=	Nonpoint source consumption of fuel type f in county c in sector x

EFp,f,x	=	Emissions factor for pollutant p, fuel type f, and sector x

# Point Source Subtraction

The adjusted fuel consumption discussed in section C is an estimate of the state-level total fuel combusted for all sources, including point and nonpoint sources. To estimate the fuel consumption from only nonpoint sources, the fuel consumption from point sources is subtracted from the total adjusted fuel consumption. The fuel consumption from point sources is provided to EPA by SLT agencies.

The starting point for computing state-level point fuel consumption (PFf,s,x) begins by matching NEI (EIS/state) facility identifier codes with EIA facilities in EIA-923 data to identify facilities that are in the industrial, commercial, or electric utility sectors. NEI facilities that match EIA-923 facilities with EIA sector assignments of 4 (Commercial NAICS Non-Co-gen) or 5 (Commercial NAICS Cogen) are assigned as “Commercial/Institutional” whose point source throughput activity data (consumption) are subject to Point subtraction from EIA SEDS. Similarly, NEI facilities that match EIA-923 facilities with EIA sector assignments of 6 (Industrial NAICS Non-Co-gen) or 7 (Industrial NAICS Cogen) are assigned as “Commercial/Institutional” whose point source throughput activity data (consumption) are subject to Point subtraction from EIA SEDS. NEI facilities that match EIA-923 facilities with EIA sector assignments of 1, 2 or 3 (Electric Utility, NAICS-22 Non-Cogen, and NAICS-22 Cogen, respectively) are assigned as “EGU” and thus not subject to Point “ICI” subtraction. An existing EIA 923 to NEI (EIS/state) facility ID cross-reference to EIA ICI sectors is available for each state “Proposed_facility_to_ICI_sector_assignments_2016NEI_14dec18_<state>.csv” on the 2017 NEI Supporting Data site.

The remaining facilities that are not matched to EIA-923 facilities are then assigned to “Industrial”, “Commercial/Institutional” or “N/A” based on facility NAICS codes provided in Table 6.

Once all point facilities have been mapped to the appropriate sector via either the EIA-923 or the NAICS assignments, the point inventory fuel consumption data are then aggregated by fuels using one of four different options to identify the fuel:

- Option A: By NAICS and SCC. In this option, SLT agencies submit state-level point source data aggregated by NAICS code and SCC. NAICS codes are used to map the point source fuel consumption to the appropriate ICI sector according to the mapping in Table 6 in the appendix. SCCs are used to identify the type of fuel consumed, according to the mapping in Table 7 in the appendix.

- Option B: By NAICS and Fuel Type. If the SLT agency knows the type of fuel consumed at each facility, the agency can submit fuel consumption by fuel type and NAICS. As with option A, the NAICS code will be used to map the fuel consumption to the appropriate sector.

- Option C: Point Source Fuel Consumption By Sector and Fuel Type. If the SLT agency has an alternative approach for determining the state-level fuel consumption by point sources in the industrial and commercial/institutional sectors by fuel type, the agency can submit this data directly.

- Option D: Nonpoint Source Fuel Consumption By Sector and Fuel Type. If the SLT agency has an alternative approach for determining the state-level fuel consumption by nonpoint sources in the industrial and commercial/institutional sectors by fuel type, the agency can submit this data directly. If the SLT agency chooses this option, point source subtraction is not needed, and the nonpoint source fuel consumption will be used directly to estimate emissions without further adjustment.

| NPFf,s,x=AFf,s, x-PFf,s,x |  |
| --- | --- |

Where:

NPFf,s,x	= 	Adjusted nonpoint consumption of fuel f in state s in sector x

AFf,s,x	=	Total consumption of fuel f in state s in sector x, adjusted as discussed in section C

PFf,s,x	=	Consumption of fuel f by points sources in state s in sector x

Following point source subtraction at the state level, the estimated state-level nonpoint source fuel consumption is distributed to the states based on employment in the industrial and commercial sectors. This allocation procedure is discussed in section D.

# Sample Calculations

Table 3 lists sample calculations to determine PM25-PRI emissions from nonpoint source bituminous/subbituminous coal combustion in the industrial sector in Alamance County, North Carolina. Note that the equations in the table are listed in the order of the calculations, not in the order in which they are presented in this NEMO. Note also that the point source fuel consumption used in in equation 6 is just shown as an example and is not actual point source fuel consumption data submitted by an SLT agency.

Table 3. Sample calculations for PM25-PRI emissions from nonpoint source bituminous/subbituminous coal combustion in the industrial sector in Alamance County, NC.

| Eq. # | Equation | Values for Alamance County, NC | Result |
| --- | --- | --- | --- |
| 1 | AFf,s,x=TFf,s, x×SSf,s,x×(1-ncf,s,industrial) | 454 thousand tons coal consumption in the industrial sector in NC ×1 fraction of coal used by stationary sources×(1-0.2632 fraction of coal in NC used as input to industrial process] | 334.5 thousand tons adjusted industrial coal consumption in NC |
| 2 | AFant/bit,s,x=AFcoal,s,x×Rant/bit,s | 334.5 thousand tons coal ×1 fraction of bit/subbit coal consumption | 334.5 thousand tons industrial bituminous/ subbituminous coal consumption in NC |
| 3 | AFboiler/engine,s,x=AFdistillate,s,x×Rboiler/engine,s,x | N/A | Not needed for coal consumption |
| 6 | NPFf,s,x=AFf,s, x-PFf,s,x | 334.5 thousand tons bit/subbit coal-300 tons point source bit/subbit coal consumption | 34.5 thousand tons industrial nonpoint source bituminous/ subbituminous coal consumption |
| 4 | NPFf,c,x=NPFf,s,x×empc,xemps,x | 34.5 thousand tons ×17,733 industrial employees in Alamance861,292 industrial employees in NC | 0.71 thousand tons industrial nonpoint source bituminous/ subbituminous coal consumption in Alamance County, NC |
| 5 | Ep,f,c,x= NPFf,c,x×EFp,f,x | 0.71 thousand tons ×2.44 lbs PM25-PRI/ton | 1,732 lbs. (0.866 tons) PM25-PRI emissions from industrial nonpoint source bituminous/ subbituminous coal consumption in Alamance County |

# Changes from 2014 Methodology

The current method uses a different approach to point source subtraction compared to the 2014 method. The 2014 method used point source SCCs to identify both the sector and fuel type for fuel consumption by point sources. In the current method, the EIA-923 data is first used to assign point inventory facilities as Industrial, Commercial/Institutional, or neither, and then facilities’ NAICS codes are used to determine which of the remaining facilities are in the ICI sector. The current method also now allows four different options for submitting state-level fuel consumption data.

In addition, in the current method, point source subtraction is conducted at the state level, rather than the county level. In the 2014 method, total fuel consumption was distributed to the county level before point source subtraction, and then point source subtraction was conducted using county-level point source data.

Finally, the 2014 method allowed point source subtraction using either point source emissions or point source fuel consumption. In the current method, point source subtraction is conducted using only point source fuel consumption; point source subtraction using emissions is no longer allowed.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Since insufficient data exists to calculate emissions from the ICI sectors for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emissions factor. For each Puerto Rico and US Virgin Island county, the tons per capita emissions factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# Instructions for Submitting Point Activity Data to Input Template

The ICI Input Template has four options for submitting point inventory activity (fuel) throughput data, separately for Industrial vs Commercial/Institutional sources, which are described in Section H. When compiling the point inventory activity data, the SLT agency should first use the EIA-923 data12 to run an initial sweep on their point inventory to classify facilities as Industrial, Commercial/Institutional, neither, or non-matched to EIA. In the EIA-923 data, in the "Sector" column of the tab "Page 1 Generation and Fuel Data," facilities listed as Electric Utility or NAICS 22 are considered part of the Electric Utility sector and are not included in the ICI tool. All other facilities are considered Industrial or Commercial/Institutional, as labeled in the EIA-923 data. A preliminary set of these ICI facility assignments is available for each state in the spreadsheets called: “Proposed_facility_to_ICI_sector_assignments_2016NEI_14dec18_<state>.csv” on the 2017 NEI Supporting Data site.

Then for facilities that are not able to be matched to the EIA-923, the SLT agency should use the NAICS codes in Table 6 to assign the remaining facilities as Industrial, Commercial/Institutional, or neither. Next, the SLT agency should use the SCC-to-fuel cross-reference in Table 7 to determine the fuel type associated with the point source activity. Fuel consumption data needs to be estimated by SCC for all processes, separately for Industrial vs Commercial/Institutional facilities. Once this is complete, SLTs then aggregate this consumption data, separately for Industrial vs Commercial/Institutional, to state total NAICS/SCC (Option A), NAICS/fuel (Option B), or point inventory total for Sector- (I vs C/I) fuel type (Option C). The templates also include a fourth option (D), which is nonpoint inventory total by sector/fuel type. We will perform quality assurance on any SLT-submitted consumption data, comparing to SEDS data. EPA cannot populate this point source fuel consumption data if SLTs do not submit it.

# References

# Appendix

Table 4. Stationary source fuel consumption assumptions by sector

| State | .Industrial |  |  |  |  | Commercial |  |  |  |  | Farm |  | Off-Highway | Oil Company |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. 1 Distillate | No. 2 Fuel Oil | No. 2 Low Sulfur Distillate | No. 4 Distillate | LPG | No. 1 Distillate | No. 2 Fuel Oil | No. 2 Low Sulfur Distillate | No. 4 Distillate | LPG | Diesel | Other Distillate | Total Distillate | Total Distillate |
| AL | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| AK | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| AZ | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| AR | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| CA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| CO | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| CT | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| DE | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| DC | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| FL | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| GA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| HI | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| ID | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| IL | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| IN | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| IA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| KS | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| KY | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| LA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| ME | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MD | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MI | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MN | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MS | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MO | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| MT | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NE | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NV | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NH | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NJ | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NM | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NY | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| NC | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| ND | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| OH | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| OK | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| OR | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| PA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| RI | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| SC | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| SD | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| TN | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| TX | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| UT | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| VT | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| VA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| WA | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| WV | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| WI | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |
| WY | 60% | 100% | 15% | 100% | 91.28% | 80% | 100% | 0% | 100% | 82.28% | 0% | 100% | 5% | 50% |

Note: Farm, off-highway, and oil company sectors are mapped to the industrial sector for the ICI tool.

Table 5. Emissions factors for fuel consumption in the Industrial, Commercial, and Institutional sectors. Note: the original emissions factor is included in the table if it has different units in the original source and has been converted for use in the ICI tool

| SCC | Pollutant Code | Name | Original Factor Numeric Value | Original Unit Numerator | Original Denominator | Factor Numeric Value | Factor Unit Numerator | Factor Unit Denominator | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2102001000 | 85018 | Phenanthrene |  |  |  | 0.0068 | LB | TON | 7; Table 1.2-5 |
| 2102001000 | 91203 | Naphthalene |  |  |  | 0.13 | LB | TON | 7; Table 1.2-5 |
| 2102001000 | 92524 | Biphenyl |  |  |  | 0.025 | LB | TON | 7; Table 1.2-5 |
| 2102001000 | 7439921 | Lead |  |  |  | 0.00042 | LB | TON | 7; Table 1.2-3 |
| 2102001000 | CO | Carbon Monoxide |  |  |  | 0.6 | LB | TON | 8 |
| 2102001000 | NH3 | Ammonia |  |  |  | 0.03 | LB | TON | 8 |
| 2102001000 | NOX | Nitrogen Oxides |  |  |  | 9 | LB | TON | 8 |
| 2102001000 | PM10-FIL | PM10 Filterable |  |  |  | 1.1A | LB | TON | 8 |
| 2102001000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.08A + 1.1A | LB | TON | 8 |
| 2102001000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.48A | LB | TON | 8 |
| 2102001000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.08A + 0.48A | LB | TON | 8 |
| 2102001000 | PM-CON | PM Condensible |  |  |  | 0.08A | LB | TON | 8 |
| 2102001000 | SO2 | Sulfur Dioxide |  |  |  | 38S | LB | TON | 8 |
| 2102001000 | VOC | Volatile Organic Compounds |  |  |  | 0.3 | LB | TON | 8 |
| 2102002000 | 50000 | Formaldehyde |  |  |  | 0.00024 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 50328 | Benzo[a]Pyrene |  |  |  | 0.000000038 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 56553 | Benz[a]Anthracene |  |  |  | 0.00000008 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 60344 | Methylhydrazine |  |  |  | 0.00017 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 67663 | Chloroform |  |  |  | 0.000059 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 75003 | Ethyl Chloride |  |  |  | 0.000042 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 75252 | Bromoform |  |  |  | 0.000039 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 77781 | Dimethyl Sulfate |  |  |  | 0.000048 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 80626 | Methyl Methacrylate |  |  |  | 0.00002 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 83329 | Acenaphthene |  |  |  | 0.00000051 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 85018 | Phenanthrene |  |  |  | 0.0000027 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 86737 | Fluorene |  |  |  | 0.00000091 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 91203 | Naphthalene |  |  |  | 0.000013 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 92524 | Biphenyl |  |  |  | 0.0000017 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 98828 | Cumene |  |  |  | 0.0000053 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 100447 | Benzyl Chloride |  |  |  | 0.0007 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 106934 | Ethylene Dibromide |  |  |  | 0.0000012 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 108054 | Vinyl Acetate |  |  |  | 0.0000076 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 120127 | Anthracene |  |  |  | 0.00000021 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 121142 | 2,4-Dinitrotoluene |  |  |  | 0.00000028 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 129000 | Pyrene |  |  |  | 0.00000033 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 191242 | Benzo[g,h,i,]Perylene |  |  |  | 0.000000027 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 193395 | Indeno[1,2,3-c,d]Pyrene |  |  |  | 0.000000061 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 206440 | Fluoranthene |  |  |  | 0.00000071 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 208968 | Acenaphthylene |  |  |  | 0.00000025 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 218019 | Chrysene |  |  |  | 0.0000001 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 532274 | 2-Chloroacetophenone |  |  |  | 0.000007 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 1330207 | Xylenes (Mixed Isomers) |  |  |  | 0.000037 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 1634044 | Methyl Tert-Butyl Ether |  |  |  | 0.000035 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | 3697243 | 5-Methylchrysene |  |  |  | 0.000000022 | LB | TON | 7; Table 1.1-13 |
| 2102002000 | 7439921 | Lead |  |  |  | 0.00042 | LB | TON | 7; Table 1.2-3 |
| 2102002000 | 56832736 | Benzofluoranthenes |  |  |  | 0.00000011 | LB | TON | 7; Table 1.1-14 |
| 2102002000 | CO | Carbon Monoxide |  |  |  | 5 | LB | TON | 7; Table 1.1-3 |
| 2102002000 | NH3 | Ammonia |  |  |  | 0.03 | LB | TON | 9 |
| 2102002000 | NOX | Nitrogen Oxides |  |  |  | 11 | LB | TON | 7; Table 1.1-3 |
| 2102002000 | PM10-FIL | PM10 Filterable |  |  |  | 12 | LB | TON | 8 |
| 2102002000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 13.04 | LB | TON | 8 |
| 2102002000 | PM25-FIL | PM2.5 Filterable |  |  |  | 1.4 | LB | TON | 8 |
| 2102002000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 2.44 | LB | TON | 8 |
| 2102002000 | PM-CON | PM Condensible |  |  |  | 1.04 | LB | TON | 8 |
| 2102002000 | SO2 | Sulfur Dioxide |  |  |  | 38S | LB | TON | 8 |
| 2102002000 | VOC | Volatile Organic Compounds |  |  |  | 0.05 | LB | TON | 8 |
| 2102004001 | 50000 | Formaldehyde |  |  |  | 0.048 | LB | E3GAL | 7; Table 1.3-8 |
| 2102004001 | 7439921 | Lead |  |  |  | 1.51E-03 | LB | E3GAL | 7; Table 1.3-11 |
| 2102004001 | CO | Carbon Monoxide |  |  |  | 5 | LB | E3GAL | 7; Table 1.3-1 |
| 2102004001 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 9 |
| 2102004001 | NOX | Nitrogen Oxides |  |  |  | 20 | LB | E3GAL | 7; Table 1.3-1 |
| 2102004001 | PM10-FIL | PM10 Filterable |  |  |  | 5.9A | LB | E3GAL | 7; Table 1.3-1 |
| 2102004001 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 5.9A + 1.3 | LB | E3GAL | 7; Tables 1.3-1 1.3-2 |
| 2102004001 | PM25-FIL | PM2.5 Filterable |  |  |  | 4.3A | LB | E3GAL | 7; Table 1.3-1 |
| 2102004001 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 4.3A + 1.3 | LB | E3GAL | 7; Tables 1.3-1 1.3-2 |
| 2102004001 | PM-CON | PM Condensible |  |  |  | 1.3 | LB | E3GAL | 7; Table 1.3-2 |
| 2102004001 | SO2 | Sulfur Dioxide |  |  |  | 2S | LB | E3GAL | 7; Table 1.3-1 |
| 2102004001 | VOC | Volatile Organic Compounds |  |  |  | 0.2 | LB | E3GAL | 7; Table 1.3-3 |
| 2102004002 | 50000 | Formaldehyde | 1.18E-03 | LB | MMBTU | 0.16166 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 50328 | Benzo[a]Pyrene | 1.88E-07 | LB | MMBTU | 0.000025756 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 53703 | Dibenzo[a,h]Anthracene | 5.83E-07 | LB | MMBTU | 0.000079871 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 56553 | Benz[a]Anthracene | 1.68E-06 | LB | MMBTU | 0.00023016 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 71432 | Benzene | 9.33E-04 | LB | MMBTU | 0.127821 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 75070 | Acetaldehyde | 7.67E-04 | LB | MMBTU | 0.105079 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 83329 | Acenaphthene | 5.06E-06 | LB | MMBTU | 0.00019454 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 85018 | Phenanthrene | 2.94E-05 | LB | MMBTU | 0.0040278 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 86737 | Fluorene | 2.92E-05 | LB | MMBTU | 0.0040004 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 91203 | Naphthalene | 8.48E-05 | LB | MMBTU | 0.0116176 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 106990 | 1,3-Butadiene | 3.91E-05 | LB | MMBTU | 0.0053567 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 107028 | Acrolein | 9.25E-05 | LB | MMBTU | 0.0126725 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 108883 | Toluene | 4.09E-04 | LB | MMBTU | 0.056033 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 120127 | Anthracene | 1.87E-06 | LB | MMBTU | 0.00025619 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 129000 | Pyrene | 4.78E-06 | LB | MMBTU | 0.00065486 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 191242 | Benzo[g,h,i,]Perylene | 4.89E-07 | LB | MMBTU | 0.000066993 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 193395 | Indeno[1,2,3-c,d]Pyrene | 3.75E-07 | LB | MMBTU | 0.000051375 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 205992 | Benzo[b]Fluoranthene | 9.91E-08 | LB | MMBTU | 1.35767E-05 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 206440 | Fluoranthene | 7.61E-06 | LB | MMBTU | 0.00104257 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 207089 | Benzo[k]Fluoranthene | 1.55E-07 | LB | MMBTU | 0.000021235 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 208968 | Acenaphthylene | 5.06E-06 | LB | MMBTU | 0.00069322 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 218019 | Chrysene | 3.53E-07 | LB | MMBTU | 0.000048361 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | 1330207 | Xylenes (Mixed Isomers) | 2.85E-04 | LB | MMBTU | 0.039045 | LB | E3GAL | 7; Table 3.3-2 |
| 2102004002 | CO | Carbon Monoxide | 0.95 | LB | MMBTU | 130 | LB | E3GAL | 7; Table 3.3-1 |
| 2102004002 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 9 |
| 2102004002 | NOX | Nitrogen Oxides | 4.41 | LB | MMBTU | 604 | LB | E3GAL | 7; Table 3.3-1 |
| 2102004002 | PM10-FIL | PM10 Filterable | 0.31 | LB | MMBTU | 42.5 | LB | E3GAL | 7; Table 3.3-1 |
| 2102004002 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 43.5 | LB | E3GAL | Sum of PM10-FIL and PM-CON |
| 2102004002 | PM25-FIL | PM2.5 Filterable |  |  |  | 39.8 | LB | E3GAL | Determined used PM Augmentation Tool |
| 2102004002 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 40.8 | LB | E3GAL | Sum of PM25-FIL and PM-CON |
| 2102004002 | PM-CON | PM Condensible |  |  |  | 1 | LB | E3GAL | Determined used PM Augmentation Tool |
| 2102004002 | SO2 | Sulfur Dioxide | 0.29 | LB | MMBTU | 39.8 | LB | E3GAL | 7; Table 3.3-1 |
| 2102004002 | VOC | Volatile Organic Compounds | 0.36 | LB | MMBTU | 42 | LB | E3GAL | 7; Table 3.3-1 |
| 2102005000 | 50000 | Formaldehyde |  |  |  | 0.033 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 53703 | Dibenzo[a,h]Anthracene |  |  |  | 0.00000167 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 56553 | Benz[a]Anthracene |  |  |  | 0.00000401 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 71432 | Benzene |  |  |  | 0.000214 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 75070 | Acetaldehyde |  |  |  | 0.00525 | LB | E3GAL | 2002 NEI Documentation |
| 2102005000 | 83329 | Acenaphthene |  |  |  | 0.0000211 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 85018 | Phenanthrene |  |  |  | 0.0000105 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 86737 | Fluorene |  |  |  | 0.0000047 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 91203 | Naphthalene |  |  |  | 0.0013 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 120127 | Anthracene |  |  |  | 1.22E-06 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 129000 | Pyrene |  |  |  | 0.00000425 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 191242 | Benzo[g,h,i,]Perylene |  |  |  | 0.000002426 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 193395 | Indeno[1,2,3-c,d]Pyrene |  |  |  | 0.00000214 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 206440 | Fluoranthene |  |  |  | 0.00000484 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 208968 | Acenaphthylene |  |  |  | 0.000000211 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 218019 | Chrysene |  |  |  | 0.00000238 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | 7439921 | Lead | 9 | LB | E12BTU | 0.001233 | LB | E3GAL | 7; Table 1.3-10 |
| 2102005000 | 56832736 | Benzofluoranthenes |  |  |  | 0.00000148 | LB | E3GAL | 7; Table 1.3-9 |
| 2102005000 | CO | Carbon Monoxide |  |  |  | 5 | LB | E3GAL | 8 |
| 2102005000 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 8 |
| 2102005000 | NOX | Nitrogen Oxides |  |  |  | 55 | LB | E3GAL | 8 |
| 2102005000 | PM10-FIL | PM10 Filterable |  |  |  | 7.17(1.12*S+0.37) | LB | E3GAL | 8 |
| 2102005000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  |  | LB | E3GAL | 8 |
| 2102005000 | PM25-FIL | PM2.5 Filterable |  |  |  | 4.67(1.12*S+0.37) | LB | E3GAL | 8 |
| 2102005000 | PM25-PRI | PM25 Primary (Filt + Cond) |  |  |  |  | LB | E3GAL | 8 |
| 2102005000 | PM-CON | PM Condensible |  |  |  | 1.5 | LB | E3GAL | 8 |
| 2102005000 | SO2 | Sulfur Dioxide |  |  |  | 157S | LB | E3GAL | 8 |
| 2102005000 | VOC | Volatile Organic Compounds |  |  |  | 0.28 | LB | E3GAL | 8 |
| 2102006000 | 50000 | Formaldehyde |  |  |  | 0.075 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 71432 | Benzene |  |  |  | 0.0021 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 75070 | Acetaldehyde |  |  |  | 0.00001365 | LB | E6FT3 | 2002 NEI Documentation |
| 2102006000 | 85018 | Phenanthrene |  |  |  | 0.0000175 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 86737 | Fluorene |  |  |  | 2.80E-06 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 91203 | Naphthalene |  |  |  | 6.10E-04 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 129000 | Pyrene |  |  |  | 5.00E-06 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 206440 | Fluoranthene |  |  |  | 3.00E-06 | LB | E6FT3 | 7; Table 1.4-3 |
| 2102006000 | 7439921 | Lead |  |  |  | 0.0005 | LB | E6FT3 | 7; Table 1.4-2 |
| 2102006000 | CO | Carbon Monoxide |  |  |  | 84 | LB | E6FT3 | 8 |
| 2102006000 | NH3 | Ammonia |  |  |  | 3.2 | LB | E6FT3 | 8 |
| 2102006000 | NOX | Nitrogen Oxides |  |  |  | 100 | LB | E6FT3 | 8 |
| 2102006000 | PM10-FIL | PM10 Filterable |  |  |  | 0.2 | LB | E6FT3 | 8 |
| 2102006000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.54 | LB | E6FT3 | 8 |
| 2102006000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.11 | LB | E6FT3 | 8 |
| 2102006000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.43 | LB | E6FT3 | 8 |
| 2102006000 | PM-CON | PM Condensible |  |  |  | 0.32 | LB | E6FT3 | 8 |
| 2102006000 | SO2 | Sulfur Dioxide |  |  |  | 0.6 | LB | E6FT3 | 8 |
| 2102006000 | VOC | Volatile Organic Compounds |  |  |  | 5.5 | LB | E6FT3 | 8 |
| 2102007000 | CO | Carbon Monoxide |  |  |  | 7.97 | LB | E3GAL | 8 |
| 2102007000 | NH3 | Ammonia |  |  |  | 0.3 | LB | E3GAL | 8 |
| 2102007000 | NOX | Nitrogen Oxides |  |  |  | 14.23 | LB | E3GAL | 8 |
| 2102007000 | PM10-FIL | PM10 Filterable |  |  |  | 0.02 | LB | E3GAL | 8 |
| 2102007000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.05 | LB | E3GAL | 8 |
| 2102007000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.01 | LB | E3GAL | 8 |
| 2102007000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.04 | LB | E3GAL | 8 |
| 2102007000 | PM-CON | PM Condensible |  |  |  | 0.03 | LB | E3GAL | 8 |
| 2102007000 | SO2 | Sulfur Dioxide |  |  |  | 0.06 | LB | E3GAL | 8 |
| 2102007000 | VOC | Volatile Organic Compounds |  |  |  | 0.52 | LB | E3GAL | 8 |
| 2102008000 | 50000 | Formaldehyde |  |  |  | 0.00144053 | LB | E6BTU | 11 |
| 2102008000 | 71432 | Benzene |  |  |  | 0.000473582 | LB | E6BTU | 11 |
| 2102008000 | 75070 | Acetaldehyde |  |  |  | 0.002107007 | LB | E6BTU | 11 |
| 2102008000 | 83329 | Acenaphthene |  |  |  | 2.49774E-06 | LB | E6BTU | 11 |
| 2102008000 | 85018 | Phenanthrene |  |  |  | 1.94131E-05 | LB | E6BTU | 11 |
| 2102008000 | 86737 | Fluorene |  |  |  | 5.49009E-06 | LB | E6BTU | 11 |
| 2102008000 | 90120 | 1-Methylnaphthalene |  |  |  | 1.3107E-05 | LB | E6BTU | 11 |
| 2102008000 | 91203 | Naphthalene |  |  |  | 0.000280687 | LB | E6BTU | 11 |
| 2102008000 | 91576 | 2-Methylnaphthalene |  |  |  | 1.85476E-05 | LB | E6BTU | 11 |
| 2102008000 | 98862 | Acetophenone |  |  |  | 4.82237E-06 | LB | E6BTU | 11 |
| 2102008000 | 100414 | Ethyl Benzene |  |  |  | 2.8316E-05 | LB | E6BTU | 11 |
| 2102008000 | 106990 | 1,3-Butadiene |  |  |  | 0.000144671 | LB | E6BTU | 11 |
| 2102008000 | 107028 | Acrolein |  |  |  | 7.78999E-05 | LB | E6BTU | 11 |
| 2102008000 | 108883 | Toluene |  |  |  | 0.000195368 | LB | E6BTU | 11 |
| 2102008000 | 108952 | Phenol |  |  |  | 0.000649166 | LB | E6BTU | 11 |
| 2102008000 | 120127 | Anthracene |  |  |  | 4.25358E-06 | LB | E6BTU | 11 |
| 2102008000 | 120809 | Catechol |  |  |  | 0.00038579 | LB | E6BTU | 11 |
| 2102008000 | 123319 | Hydroquinone |  |  |  | 2.12679E-05 | LB | E6BTU | 11 |
| 2102008000 | 123386 | Propionaldehyde |  |  |  | 0.000315309 | LB | E6BTU | 11 |
| 2102008000 | 129000 | Pyrene |  |  |  | 2.31227E-06 | LB | E6BTU | 11 |
| 2102008000 | 203123 | Benzo(g,h,i)Fluoranthene |  |  |  | 1.01394E-07 | LB | E6BTU | 11 |
| 2102008000 | 206440 | Fluoranthene |  |  |  | 3.77134E-06 | LB | E6BTU | 11 |
| 2102008000 | 208968 | Acenaphthylene |  |  |  | 2.2999E-05 | LB | E6BTU | 11 |
| 2102008000 | 832699 | 1-Methylphenanthrene |  |  |  | 2.74504E-06 | LB | E6BTU | 11 |
| 2102008000 | 1319773 | Cresol/Cresylic Acid (Mixed Isomers) |  |  |  | 0.000580663 | LB | E6BTU | 11 |
| 2102008000 | 1330207 | Xylenes (Mixed Isomers) |  |  |  | 9.65711E-05 | LB | E6BTU | 11 |
| 2102008000 | CO | Carbon Monoxide |  |  |  | 0.6 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | NH3 | Ammonia |  |  |  | 0.007 | LB | E6BTU | 10 |
| 2102008000 | NOX | Nitrogen Oxides |  |  |  | 0.22 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | PM10-FIL | PM10 Filterable |  |  |  | 0.5 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.517 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.43 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.447 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | PM-CON | PM Condensible |  |  |  | 0.017 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | SO2 | Sulfur Dioxide |  |  |  | 0.025 | LB | E6BTU | 7; Section 1.6 |
| 2102008000 | VOC | Volatile Organic Compounds |  |  |  | 0.017 | LB | E6BTU | 7; Section 1.6 |
| 2102011000 | 50000 | Formaldehyde |  |  |  | 0.0324 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 53703 | Dibenzo[a,h]Anthracene |  |  |  | 0.00000162 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 56553 | Benz[a]Anthracene |  |  |  | 0.000003915 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 71432 | Benzene |  |  |  | 0.0002025 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 75070 | Acetaldehyde |  |  |  | 0.004725 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 83329 | Acenaphthene |  |  |  | 0.00002025 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 85018 | Phenanthrene |  |  |  | 0.000010125 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 86737 | Fluorene |  |  |  | 0.00000432 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 91203 | Naphthalene |  |  |  | 0.0010935 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 120127 | Anthracene |  |  |  | 1.1745E-06 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 129000 | Pyrene |  |  |  | 0.00000405 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 191242 | Benzo[g,h,i,]Perylene |  |  |  | 0.00000216 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 193395 | Indeno[1,2,3-c,d]Pyrene |  |  |  | 0.000002025 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 206440 | Fluoranthene |  |  |  | 0.000004725 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 208968 | Acenaphthylene |  |  |  | 0.000000243 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 218019 | Chrysene |  |  |  | 0.000002295 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 7439921 | Lead |  |  |  | 0.001215 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | 56832736 | Benzofluoranthenes |  |  |  | 0.000001485 | LB | E3GAL | 7; Table 1.3-9 |
| 2102011000 | CO | Carbon Monoxide |  |  |  | 4.82 | LB | E3GAL | 8 |
| 2102011000 | NH3 | Ammonia |  |  |  | 0.77 | LB | E3GAL | 8 |
| 2102011000 | NOX | Nitrogen Oxides |  |  |  | 19.29 | LB | E3GAL | 8 |
| 2102011000 | PM10-FIL | PM10 Filterable |  |  |  | 0.96 | LB | E3GAL | 8 |
| 2102011000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 2.21 | LB | E3GAL | 8 |
| 2102011000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.24 | LB | E3GAL | 8 |
| 2102011000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 1.49 | LB | E3GAL | 8 |
| 2102011000 | PM-CON | PM Condensible |  |  |  | 1.25 | LB | E3GAL | 8 |
| 2102011000 | SO2 | Sulfur Dioxide |  |  |  | 142S | LB | E3GAL | 8 |
| 2102011000 | VOC | Volatile Organic Compounds |  |  |  | 0.19 | LB | E3GAL | 8 |
| 2103001000 | 85018 | Phenanthrene |  |  |  | 0.0068 | LB | TON | 7; Table 1.2-5 |
| 2103001000 | 91203 | Naphthalene |  |  |  | 0.13 | LB | TON | 7; Table 1.2-5 |
| 2103001000 | 92524 | Biphenyl |  |  |  | 0.025 | LB | TON | 7; Table 1.2-5 |
| 2103001000 | 7439921 | Lead |  |  |  | 0.00042 | LB | TON | 7; Table 1.2-3 |
| 2103001000 | CO | Carbon Monoxide |  |  |  | 0.6 | LB | TON | 8 |
| 2103001000 | NH3 | Ammonia |  |  |  | 0.03 | LB | TON | 8 |
| 2103001000 | NOX | Nitrogen Oxides |  |  |  | 9 | LB | TON | 8 |
| 2103001000 | PM10-FIL | PM10 Filterable |  |  |  | 14.718 | LB | TON | 8 |
| 2103001000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 15.7884 | LB | TON | 8 |
| 2103001000 | PM25-FIL | PM2.5 Filterable |  |  |  | 6.4224 | LB | TON | 8 |
| 2103001000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 7.4928 | LB | TON | 8 |
| 2103001000 | PM-CON | PM Condensible |  |  |  | 1.0704 | LB | TON | 8 |
| 2103001000 | SO2 | Sulfur Dioxide |  |  |  | 142S | LB | TON | 8 |
| 2103001000 | VOC | Volatile Organic Compounds |  |  |  | 0.3 | LB | TON | 8 |
| 2103002000 | 50000 | Formaldehyde |  |  |  | 0.00024 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 50328 | Benzo[a]Pyrene |  |  |  | 0.000000038 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 56553 | Benz[a]Anthracene |  |  |  | 0.00000008 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 60344 | Methylhydrazine |  |  |  | 0.00017 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 67663 | Chloroform |  |  |  | 0.000059 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 75003 | Ethyl Chloride |  |  |  | 0.000042 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 75252 | Bromoform |  |  |  | 0.000039 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 77781 | Dimethyl Sulfate |  |  |  | 0.000048 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 80626 | Methyl Methacrylate |  |  |  | 0.00002 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 83329 | Acenaphthene |  |  |  | 0.00000051 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 85018 | Phenanthrene |  |  |  | 0.0000027 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 86737 | Fluorene |  |  |  | 0.00000091 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 91203 | Naphthalene |  |  |  | 0.000013 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 92524 | Biphenyl |  |  |  | 0.0000017 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 98828 | Cumene |  |  |  | 0.0000053 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 100447 | Benzyl Chloride |  |  |  | 0.0007 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 106934 | Ethylene Dibromide |  |  |  | 0.0000012 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 108054 | Vinyl Acetate |  |  |  | 0.0000076 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 120127 | Anthracene |  |  |  | 0.00000021 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 121142 | 2,4-Dinitrotoluene |  |  |  | 0.00000028 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 129000 | Pyrene |  |  |  | 0.00000033 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 191242 | Benzo[g,h,i,]Perylene |  |  |  | 0.000000027 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 193395 | Indeno[1,2,3-c,d]Pyrene |  |  |  | 0.000000061 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 206440 | Fluoranthene |  |  |  | 0.00000071 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 208968 | Acenaphthylene |  |  |  | 0.00000025 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 218019 | Chrysene |  |  |  | 0.0000001 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 532274 | 2-Chloroacetophenone |  |  |  | 0.000007 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 1330207 | Xylenes (Mixed Isomers) |  |  |  | 0.000037 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 1634044 | Methyl Tert-Butyl Ether |  |  |  | 0.000035 | LB | TON | 7; Table 1.1-14 |
| 2103002000 | 3697243 | 5-Methylchrysene |  |  |  | 0.000000022 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | 7439921 | Lead |  |  |  | 0.00042 | LB | TON | 7; Table 1.2-3 |
| 2103002000 | 56832736 | Benzofluoranthenes |  |  |  | 0.00000011 | LB | TON | 7; Table 1.1-13 |
| 2103002000 | CO | Carbon Monoxide |  |  |  | 5 | LB | TON | 8 |
| 2103002000 | NH3 | Ammonia |  |  |  | 0.03 | LB | TON | 8 |
| 2103002000 | NOX | Nitrogen Oxides |  |  |  | 11 | LB | TON | 8 |
| 2103002000 | PM10-FIL | PM10 Filterable |  |  |  | 1.1A | LB | TON | 8 |
| 2103002000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 1.1A + 1.04 | LB | TON | 8 |
| 2103002000 | PM25-FIL | PM2.5 Filterable |  |  |  | 1.4 | LB | TON | 8 |
| 2103002000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 2.44 | LB | TON | 8 |
| 2103002000 | PM-CON | PM Condensible |  |  |  | 1.04 | LB | TON | 8 |
| 2103002000 | SO2 | Sulfur Dioxide |  |  |  | 38S | LB | TON | 8 |
| 2103002000 | VOC | Volatile Organic Compounds |  |  |  | 0.05 | LB | TON | 8 |
| 2103004001 | 50000 | Formaldehyde |  |  |  | 0.048 | LB | E3GAL | 7; Table 1.3-8 |
| 2103004001 | 71432 | Benzene |  |  |  | 2.14E-04 | LB | E3GAL | 7; Table 1.3-9 |
| 2103004001 | 206440 | Fluoranthene |  |  |  | 4.84E-06 | LB | E3GAL | 7; Table 1.3-9 |
| 2103004001 | 7439921 | Lead | 9 | LB | E12BTU | 0.00123 | LB | E3GAL | 7; Table 1.3-10 |
| 2103004001 | CO | Carbon Monoxide |  |  |  | 5 | LB | E3GAL | 7; Table 1.3-1 |
| 2103004001 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 9 |
| 2103004001 | NOX | Nitrogen Oxides |  |  |  | 20 | LB | E3GAL | 7; Table 1.3-1 |
| 2103004001 | PM10-FIL | PM10 Filterable |  |  |  | 1 | LB | E3GAL | 7; Table 1.3-4 |
| 2103004001 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 2.38 | LB | E3GAL | Sum of PM10-FIL and PM-CON emission factors |
| 2103004001 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.25 | LB | E3GAL | Sum of PM25-FIL and PM-CON emission factors |
| 2103004001 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 2.13 | LB | E3GAL | Sum of PM25-FIL and PM-CON emission factors |
| 2103004001 | PM-CON | PM Condensible |  |  |  | 1.3 | LB | E3GAL | 7; Table 1.3-2 |
| 2103004001 | SO2 | Sulfur Dioxide |  |  |  | 2S | LB | E3GAL | 7; Table 1.3-1 |
| 2103004001 | VOC | Volatile Organic Compounds |  |  |  | 0.34 | LB | E3GAL | 7; Table 1.3-2 |
| 2103004002 | 50000 | Formaldehyde | 1.18E-03 | LB | MMBTU | 0.16166 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 50328 | Benzo[a]Pyrene | 1.88E-07 | LB | MMBTU | 0.000025756 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 53703 | Dibenzo[a,h]Anthracene | 5.83E-07 | LB | MMBTU | 0.000079871 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 56553 | Benz[a]Anthracene | 1.68E-06 | LB | MMBTU | 0.00023016 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 71432 | Benzene | 9.33E-04 | LB | MMBTU | 0.127821 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 75070 | Acetaldehyde | 7.67E-04 | LB | MMBTU | 0.105079 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 83329 | Acenaphthene | 5.06E-06 | LB | MMBTU | 0.00019454 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 85018 | Phenanthrene | 2.94E-05 | LB | MMBTU | 0.0040278 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 86737 | Fluorene | 2.92E-05 | LB | MMBTU | 0.0040004 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 91203 | Naphthalene | 8.48E-05 | LB | MMBTU | 0.0116176 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 106990 | 1,3-Butadiene | 3.91E-05 | LB | MMBTU | 0.0053567 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 107028 | Acrolein | 9.25E-05 | LB | MMBTU | 0.0126725 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 108883 | Toluene | 4.09E-04 | LB | MMBTU | 0.056033 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 120127 | Anthracene | 1.87E-06 | LB | MMBTU | 0.00025619 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 129000 | Pyrene | 4.78E-06 | LB | MMBTU | 0.00065486 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 191242 | Benzo[g,h,i,]Perylene | 4.89E-07 | LB | MMBTU | 0.000066993 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 193395 | Indeno[1,2,3-c,d]Pyrene | 3.75E-07 | LB | MMBTU | 0.000051375 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 205992 | Benzo[b]Fluoranthene | 9.91E-08 | LB | MMBTU | 1.35767E-05 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 206440 | Fluoranthene | 7.61E-06 | LB | MMBTU | 0.00104257 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 207089 | Benzo[k]Fluoranthene | 1.55E-07 | LB | MMBTU | 0.000021235 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 208968 | Acenaphthylene | 5.06E-06 | LB | MMBTU | 0.00069322 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 218019 | Chrysene | 3.53E-07 | LB | MMBTU | 0.000048361 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | 1330207 | Xylenes (Mixed Isomers) | 2.85E-04 | LB | MMBTU | 0.039045 | LB | E3GAL | 7; Table 3.3-2 |
| 2103004002 | CO | Carbon Monoxide | 0.95 | LB | MMBTU | 130 | LB | E3GAL | 7; Table 3.3-1 |
| 2103004002 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 9 |
| 2103004002 | NOX | Nitrogen Oxides | 4.41 | LB | MMBTU | 604 | LB | E3GAL | 7; Table 3.3-1 |
| 2103004002 | PM10-FIL | PM10 Filterable | 0.31 | LB | MMBTU | 42.5 | LB | E3GAL | 7; Table 3.3-1 |
| 2103004002 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 43.5 | LB | E3GAL | Sum of PM10-FIL and PM-CON |
| 2103004002 | PM25-FIL | PM2.5 Filterable | 0.31 | LB | MMBTU | 42.5 | LB | E3GAL | 7; Table 3.3-1 |
| 2103004002 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 43.5 | LB | E3GAL | Sum of PM25-FIL and PM-CON |
| 2103004002 | PM-CON | PM Condensible |  |  |  | 1 | LB | E3GAL | Determined used PM Augmentation Tool |
| 2103004002 | SO2 | Sulfur Dioxide | 0.29 | LB | MMBTU | 39.8 | LB | E3GAL | 7; Table 3.3-1 |
| 2103004002 | VOC | Volatile Organic Compounds | 0.36 | LB | MMBTU | 42 | LB | E3GAL | 7; Table 3.3-1 |
| 2103005000 | 50000 | Formaldehyde |  |  |  | 0.033 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 53703 | Dibenzo[a,h]Anthracene |  |  |  | 0.00000167 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 56553 | Benz[a]Anthracene |  |  |  | 0.00000401 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 71432 | Benzene |  |  |  | 0.000214 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 75070 | Acetaldehyde |  |  |  | 0.00525 | LB | E3GAL | 2002 NEI Documentation |
| 2103005000 | 83329 | Acenaphthene |  |  |  | 0.0000211 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 85018 | Phenanthrene |  |  |  | 0.0000105 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 86737 | Fluorene |  |  |  | 0.0000047 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 91203 | Naphthalene |  |  |  | 0.0013 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 120127 | Anthracene |  |  |  | 1.22E-06 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 129000 | Pyrene |  |  |  | 0.00000425 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 191242 | Benzo[g,h,i,]Perylene |  |  |  | 0.000002426 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 193395 | Indeno[1,2,3-c,d]Pyrene |  |  |  | 0.00000214 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 206440 | Fluoranthene |  |  |  | 0.00000484 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 208968 | Acenaphthylene |  |  |  | 0.000000211 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 218019 | Chrysene |  |  |  | 0.00000238 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | 7439921 | Lead |  |  |  | 0.001233 | LB | E3GAL | 7; Table 1.3-10 |
| 2103005000 | 56832736 | Benzofluoranthenes |  |  |  | 0.00000148 | LB | E3GAL | 7; Table 1.3-9 |
| 2103005000 | CO | Carbon Monoxide |  |  |  | 5 | LB | E3GAL | 8 |
| 2103005000 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 8 |
| 2103005000 | NOX | Nitrogen Oxides |  |  |  | 55 | LB | E3GAL | 8 |
| 2103005000 | PM10-FIL | PM10 Filterable |  |  |  | 1.92(1.12*S+0.37) | LB | E3GAL | 8 |
| 2103005000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  |  | LB | E3GAL | 8 |
| 2103005000 | PM25-FIL | PM2.5 Filterable |  |  |  | 5.17(1.12*S+0.37) | LB | E3GAL | 8 |
| 2103005000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  |  | LB | E3GAL | 8 |
| 2103005000 | PM-CON | PM Condensible |  |  |  | 1.5 | LB | E3GAL | 8 |
| 2103005000 | SO2 | Sulfur Dioxide |  |  |  | 157S | LB | E3GAL | 8 |
| 2103005000 | VOC | Volatile Organic Compounds |  |  |  | 1.13 | LB | E3GAL | 8 |
| 2103006000 | 50000 | Formaldehyde |  |  |  | 7.50E-02 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 71432 | Benzene |  |  |  | 0.0021 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 75070 | Acetaldehyde |  |  |  | 0.00001365 | LB | E6FT3 | 2002 NEI Documentation |
| 2103006000 | 85018 | Phenanthrene |  |  |  | 1.70E-05 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 86737 | Fluorene |  |  |  | 2.80E-06 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 91203 | Naphthalene |  |  |  | 6.10E-04 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 129000 | Pyrene |  |  |  | 5.00E-06 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 206440 | Fluoranthene |  |  |  | 3.00E-06 | LB | E6FT3 | 7; Table 1.4-3 |
| 2103006000 | 7439921 | Lead |  |  |  | 0.0005 | LB | E6FT3 | 7; Table 1.4-2 |
| 2103006000 | CO | Carbon Monoxide |  |  |  | 84 | LB | E6FT3 | 8 |
| 2103006000 | NH3 | Ammonia |  |  |  | 0.49 | LB | E6FT3 | 8 |
| 2103006000 | NOX | Nitrogen Oxides |  |  |  | 100 | LB | E6FT3 | 8 |
| 2103006000 | PM10-FIL | PM10 Filterable |  |  |  | 0.2 | LB | E6FT3 | 8 |
| 2103006000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.52 | LB | E6FT3 | 8 |
| 2103006000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.11 | LB | E6FT3 | 8 |
| 2103006000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.43 | LB | E6FT3 | 8 |
| 2103006000 | PM-CON | PM Condensible |  |  |  | 0.32 | LB | E6FT3 | 8 |
| 2103006000 | SO2 | Sulfur Dioxide |  |  |  | 0.6 | LB | E6FT3 | 8 |
| 2103006000 | VOC | Volatile Organic Compounds |  |  |  | 5.5 | LB | E6FT3 | 8 |
| 2103007000 | 50000 | Formaldehyde | 0.2877 | LB | E3BBL | 0.00684975 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 71432 | Benzene | 0.007672 | LB | E3BBL | 0.00018266 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 75070 | Acetaldehyde | 4.99E-05 | LB | E3BBL | 1.18729E-06 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 85018 | Phenanthrene | 6.52E-06 | LB | E3BBL | 1.55261E-06 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 86737 | Fluorene | 1.07E-05 | LB | E3BBL | 2.55724E-07 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 91203 | Naphthalene | 0.0023399 | LB | E3BBL | 5.57113E-05 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 129000 | Pyrene | 1.92E-05 | LB | E3BBL | 4.5665E-07 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 206440 | Fluoranthene | 1.15E-05 | LB | E3BBL | 2.7399E-07 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | 7439921 | Lead | 0.0018266 | LB | E3BBL | 4.34905E-05 | LB | E3GAL | 2002 NEI Documentation |
| 2103007000 | CO | Carbon Monoxide |  |  |  | 7.97 | LB | E3GAL | 8 |
| 2103007000 | NH3 | Ammonia |  |  |  | 0.05 | LB | E3GAL | 8 |
| 2103007000 | NOX | Nitrogen Oxides |  |  |  | 14.23 | LB | E3GAL | 8 |
| 2103007000 | PM10-FIL | PM10 Filterable |  |  |  | 0.02 | LB | E3GAL | 8 |
| 2103007000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.05 | LB | E3GAL | 8 |
| 2103007000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.01 | LB | E3GAL | 8 |
| 2103007000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.04 | LB | E3GAL | 8 |
| 2103007000 | PM-CON | PM Condensible |  |  |  | 0.03 | LB | E3GAL | 8 |
| 2103007000 | SO2 | Sulfur Dioxide |  |  |  | 0.06 | LB | E3GAL | 8 |
| 2103007000 | VOC | Volatile Organic Compounds |  |  |  | 0.52 | LB | E3GAL | 8 |
| 2103008000 | 50000 | Formaldehyde |  |  |  | 0.00144053 | LB | E6BTU | 11 |
| 2103008000 | 71432 | Benzene |  |  |  | 0.000473582 | LB | E6BTU | 11 |
| 2103008000 | 75070 | Acetaldehyde |  |  |  | 0.002107007 | LB | E6BTU | 11 |
| 2103008000 | 83329 | Acenaphthene |  |  |  | 2.49774E-06 | LB | E6BTU | 11 |
| 2103008000 | 85018 | Phenanthrene |  |  |  | 1.94131E-05 | LB | E6BTU | 11 |
| 2103008000 | 86737 | Fluorene |  |  |  | 5.49009E-06 | LB | E6BTU | 11 |
| 2103008000 | 90120 | 1-Methylnaphthalene |  |  |  | 1.3107E-05 | LB | E6BTU | 11 |
| 2103008000 | 91203 | Naphthalene |  |  |  | 0.000280687 | LB | E6BTU | 11 |
| 2103008000 | 91576 | 2-Methylnaphthalene |  |  |  | 1.85476E-05 | LB | E6BTU | 11 |
| 2103008000 | 98862 | Acetophenone |  |  |  | 4.82237E-06 | LB | E6BTU | 11 |
| 2103008000 | 100414 | Ethyl Benzene |  |  |  | 2.8316E-05 | LB | E6BTU | 11 |
| 2103008000 | 106990 | 1,3-Butadiene |  |  |  | 0.000144671 | LB | E6BTU | 11 |
| 2103008000 | 107028 | Acrolein |  |  |  | 7.78999E-05 | LB | E6BTU | 11 |
| 2103008000 | 108883 | Toluene |  |  |  | 0.000195368 | LB | E6BTU | 11 |
| 2103008000 | 108952 | Phenol |  |  |  | 0.000649166 | LB | E6BTU | 11 |
| 2103008000 | 120127 | Anthracene |  |  |  | 4.25358E-06 | LB | E6BTU | 11 |
| 2103008000 | 120809 | Catechol |  |  |  | 0.00038579 | LB | E6BTU | 11 |
| 2103008000 | 123319 | Hydroquinone |  |  |  | 2.12679E-05 | LB | E6BTU | 11 |
| 2103008000 | 123386 | Propionaldehyde |  |  |  | 0.000315309 | LB | E6BTU | 11 |
| 2103008000 | 129000 | Pyrene |  |  |  | 2.31227E-06 | LB | E6BTU | 11 |
| 2103008000 | 203123 | Benzo(g,h,i)Fluoranthene |  |  |  | 1.01394E-07 | LB | E6BTU | 11 |
| 2103008000 | 206440 | Fluoranthene |  |  |  | 3.77134E-06 | LB | E6BTU | 11 |
| 2103008000 | 208968 | Acenaphthylene |  |  |  | 2.2999E-05 | LB | E6BTU | 11 |
| 2103008000 | 832699 | 1-Methylphenanthrene |  |  |  | 2.74504E-06 | LB | E6BTU | 11 |
| 2103008000 | 1319773 | Cresol/Cresylic Acid (Mixed Isomers) |  |  |  | 0.000580663 | LB | E6BTU | 11 |
| 2103008000 | 1330207 | Xylenes (Mixed Isomers) |  |  |  | 9.65711E-05 | LB | E6BTU | 11 |
| 2103008000 | CO | Carbon Monoxide |  |  |  | 0.6 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | NH3 | Ammonia |  |  |  | 0.005 | LB | E6BTU | 10 |
| 2103008000 | NOX | Nitrogen Oxides |  |  |  | 0.22 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | PM10-FIL | PM10 Filterable |  |  |  | 0.5 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 0.517 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.43 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 0.447 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | PM-CON | PM Condensible |  |  |  | 0.017 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | SO2 | Sulfur Dioxide |  |  |  | 0.025 | LB | E6BTU | 7; Section 1.6 |
| 2103008000 | VOC | Volatile Organic Compounds |  |  |  | 0.017 | LB | E6BTU | 7; Section 1.6 |
| 2103011000 | 50000 | Formaldehyde |  |  |  | 0.0324 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 53703 | Dibenzo[a,h]Anthracene |  |  |  | 0.00000162 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 56553 | Benz[a]Anthracene |  |  |  | 0.000003915 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 71432 | Benzene |  |  |  | 0.0002025 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 75070 | Acetaldehyde |  |  |  | 0.004725 | LB | E3GAL | 2002 NEI Documentation |
| 2103011000 | 83329 | Acenaphthene |  |  |  | 0.00002025 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 85018 | Phenanthrene |  |  |  | 0.000010125 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 86737 | Fluorene |  |  |  | 0.00000432 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 91203 | Naphthalene |  |  |  | 0.0010935 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 120127 | Anthracene |  |  |  | 1.1745E-06 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 129000 | Pyrene |  |  |  | 0.00000405 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 191242 | Benzo[g,h,i,]Perylene |  |  |  | 0.00000216 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 193395 | Indeno[1,2,3-c,d]Pyrene |  |  |  | 0.000002025 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 206440 | Fluoranthene |  |  |  | 0.000004725 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 208968 | Acenaphthylene |  |  |  | 0.000000243 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 218019 | Chrysene |  |  |  | 0.000002295 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | 7439921 | Lead |  |  |  | 0.001215 | LB | E3GAL | 7; Table 1.3-11 |
| 2103011000 | 56832736 | Benzofluoranthenes |  |  |  | 0.000001485 | LB | E3GAL | 7; Table 1.3-9 |
| 2103011000 | CO | Carbon Monoxide |  |  |  | 5 | LB | E3GAL | 7; Table 1.3-1 |
| 2103011000 | NH3 | Ammonia |  |  |  | 0.8 | LB | E3GAL | 10 |
| 2103011000 | NOX | Nitrogen Oxides |  |  |  | 20 | LB | E3GAL | 7; Table 1.3-1 |
| 2103011000 | PM10-FIL | PM10 Filterable |  |  |  | 1.08 | LB | E3GAL | 7; Table 1.3-7 |
| 2103011000 | PM10-PRI | PM10 Primary (Filt + Cond) |  |  |  | 2.34 | LB | E3GAL | Sum of PM10-FIL and PM-CON |
| 2103011000 | PM25-FIL | PM2.5 Filterable |  |  |  | 0.83 | LB | E3GAL | 7; Table 1.3-7 |
| 2103011000 | PM25-PRI | PM2.5 Primary (Filt + Cond) |  |  |  | 2.1 | LB | E3GAL | Sum of PM25-FIL and PM-CON |
| 2103011000 | PM-CON | PM Condensible |  |  |  | 1.3 | LB | E3GAL | 7; Table 1.3-2 |
| 2103011000 | SO2 | Sulfur Dioxide |  |  |  | 142S | LB | E3GAL | 7; Table 1.3-1 |
| 2103011000 | VOC | Volatile Organic Compounds |  |  |  | 0.34 | LB | E3GAL | 7; Table 1.3-3 |

Table 6. Mapping of NAICS code to ICI sector

| NAICS | Sector |
| --- | --- |
| 11 | Industrial |
| 21 | Industrial |
| 2212 | Commercial |
| 2213 | Commercial |
| 23 | Industrial |
| 31 | Industrial |
| 32 | Industrial |
| 33 | Industrial |
| 42 | Commercial |
| 44 | Commercial |
| 45 | Commercial |
| 48 (except 4862) | Commercial |
| 49 | Commercial |
| 51 | Commercial |
| 52 | Commercial |
| 53 | Commercial |
| 54 | Commercial |
| 55 | Commercial |
| 56 | Commercial |
| 61 | Commercial |
| 62 | Commercial |
| 71 | Commercial |
| 72 | Commercial |
| 81 | Commercial |
| 92 | Commercial |

Table 7. Mapping of point source SCCs to ICI fuel

| Fuel | SCC | Description |
| --- | --- | --- |
| Coal | 10100101 | External Combustion Boilers; Electric Generation; Anthracite Coal, Pulverized; Boiler |
| Coal | 10100102 | External Combustion Boilers; Electric Generation; Anthracite Coal; Boiler, Traveling Grate (Overfeed) Stoker |
| Coal | 10100201 | External Combustion Boilers; Electric Generation; Bituminous Coal, Pulverized; Boiler, Wet Bottom |
| Coal | 10100202 | External Combustion Boilers; Electric Generation; Bituminous Coal, Pulverized; Boiler, Dry Bottom |
| Coal | 10100203 | External Combustion Boilers; Electric Generation; Bituminous Coal; Boiler, Cyclone Furnace |
| Coal | 10100204 | External Combustion Boilers; Electric Generation; Bituminous Coal; Boiler, Spreader Stoker |
| Coal | 10100205 | External Combustion Boilers; Electric Generation; Bituminous Coal; Boiler, Traveling Grate (Overfeed) Stoker |
| Coal | 10100211 | External Combustion Boilers; Electric Generation; Bituminous Coal; Boiler, Wet Bottom Tangential-fired |
| Coal | 10100212 | External Combustion Boilers; Electric Generation; Bituminous Coal, Pulverized; Boiler, Dry Bottom Tangential-fired |
| Coal | 10100215 | External Combustion Boilers; Electric Generation; Bituminous Coal; Cell Burner |
| Coal | 10100217 | External Combustion Boilers; Electric Generation; Bituminous Coal; Boiler, Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| Coal | 10100218 | External Combustion Boilers; Electric Generation; Bituminous Coal; Boiler, Atmospheric Fluidized Bed Combustion: Circulating Bed |
| Coal | 10100221 | External Combustion Boilers; Electric Generation; Subbituminous Coal, Pulverized; Boiler, Wet Bottom |
| Coal | 10100222 | External Combustion Boilers; Electric Generation; Subbituminous Coal, Pulverized; Boiler, Dry Bottom |
| Coal | 10100223 | External Combustion Boilers; Electric Generation; Subbituminous Coal; Cyclone Furnace |
| Coal | 10100224 | External Combustion Boilers; Electric Generation; Subbituminous Coal; Boiler, Spreader Stoker |
| Coal | 10100225 | External Combustion Boilers; Electric Generation; Subbituminous Coal; Boiler, Traveling Grate (Overfeed) Stoker |
| Coal | 10100226 | External Combustion Boilers; Electric Generation; Subbituminous Coal, Pulverized; Boiler, Dry Bottom Tangential-fired |
| Coal | 10100235 | External Combustion Boilers; Electric Generation; Subbituminous Coal; Cell Burner |
| Coal | 10100237 | External Combustion Boilers; Electric Generation; Subbituminous Coal; Boiler, Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| Coal | 10100238 | External Combustion Boilers; Electric Generation; Subbituminous Coal; Boiler, Atmospheric Fluidized Bed Combustion: Circulating Bed |
| Coal | 10100300 | External Combustion Boilers; Electric Generation; Pulverized Lignite; Boiler, Wet Bottom |
| Coal | 10100301 | External Combustion Boilers; Electric Generation; Pulverized Lignite; Boiler, Dry Bottom Wall-fired |
| Coal | 10100302 | External Combustion Boilers; Electric Generation; Pulverized Lignite; Boiler, Dry Bottom Tangential-fired |
| Coal | 10100303 | External Combustion Boilers; Electric Generation; Lignite; Cyclone Furnace |
| Coal | 10100304 | External Combustion Boilers; Electric Generation; Lignite; Boiler, Traveling Grate (Overfeed) Stoker |
| Coal | 10100306 | External Combustion Boilers; Electric Generation; Lignite; Boiler, Spreader Stoker |
| Coal | 10100317 | External Combustion Boilers; Electric Generation; Lignite; Boiler, Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| Coal | 10100318 | External Combustion Boilers; Electric Generation; Lignite; Boiler, Atmospheric Fluidized Bed Combustion: Circulating Bed |
| Residual Oil | 10100401 | External Combustion Boilers; Electric Generation; Residual Oil - Grade 6; Boiler, Normal Firing |
| Residual Oil | 10100404 | External Combustion Boilers; Electric Generation; Residual Oil - Grade 6; Boiler, Tangential-fired |
| Residual Oil | 10100405 | External Combustion Boilers; Electric Generation; Residual Oil; Grade 5 Oil: Normal Firing |
| Residual Oil | 10100406 | External Combustion Boilers; Electric Generation; Residual Oil; Grade 5 Oil: Tangential Firing |
| Distillate Oil | 10100501 | External Combustion Boilers; Electric Generation; Distillate Oil - Grades 1 and 2; Boiler |
| Distillate Oil | 10100504 | External Combustion Boilers; Electric Generation; Distillate Oil - Grade 4; Boiler, Normal Firing |
| Distillate Oil | 10100505 | External Combustion Boilers; Electric Generation; Distillate Oil - Grade 4; Boiler, Tangential-fired |
| Natural Gas | 10100601 | External Combustion Boilers; Electric Generation; Natural Gas; Boiler, >= 100 Million BTU/hr |
| Natural Gas | 10100602 | External Combustion Boilers; Electric Generation; Natural Gas; Boiler < 100 Million BTU, except tangential |
| Natural Gas | 10100604 | External Combustion Boilers; Electric Generation; Natural Gas; Boiler, Tangential-fired |
| Wood | 10100901 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Bark-fired Boiler |
| Wood | 10100902 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Wood/Bark-Fired Boiler |
| Wood | 10100903 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Wood-fired Boiler - Wet Wood (>=20% moisture) |
| Wood | 10100908 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Wood-fired Boiler - Dry Wood (<20% moisture) |
| Wood | 10100910 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Fuel cell/Dutch oven boilers |
| Wood | 10100911 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Stoker boilers |
| Wood | 10100912 | External Combustion Boilers; Electric Generation; Wood/Bark Waste; Fluidized bed combustion boiler |
| LPG | 10101001 | External Combustion Boilers; Electric Generation; Liquified Petroleum Gas (LPG); Butane |
| LPG | 10101002 | External Combustion Boilers; Electric Generation; Liquified Petroleum Gas (LPG); Propane |
| LPG | 10101003 | External Combustion Boilers; Electric Generation; Liquified Petroleum Gas (LPG); Butane/Propane Mixture: Specify Percent Butane in Comments |
| Wood | 10101207 | External Combustion Boilers; Electric Generation; Biomass Solids; Biomass Solids, Boiler type unknown (use 10101209 or -10) |
| Wood | 10101209 | External Combustion Boilers; Electric Generation; Biomass Solids; Boiler, Stoker |
| Wood | 10101210 | External Combustion Boilers; Electric Generation; Biomass Solids; Boiler, Non-stoker |
| Residual Oil | 10101302 | External Combustion Boilers; Electric Generation; Liquid Waste; Waste Oil |
| Wood | 10101304 | External Combustion Boilers; Electric Generation; Liquid Waste; Black Liquor |
| Wood | 10101305 | External Combustion Boilers; Electric Generation; Liquid Waste; Red Liquor |
| Wood | 10101306 | External Combustion Boilers; Electric Generation; Liquid Waste; Spent Sulfite Liquor |
| Wood | 10101307 | External Combustion Boilers; Electric Generation; Liquid Waste; Tall Oil |
| Wood | 10101308 | External Combustion Boilers; Electric Generation; Liquid Waste; Wood/Wood Waste Liquid |
| Wood | 10101309 | External Combustion Boilers; Electric Generation; Biomass Liquids; Boiler |
| Coal | 10101901 | External Combustion Boilers; Electric Generation; Coal-based Synfuel; All |
| Coal | 10102001 | External Combustion Boilers; Electric Generation; Waste Coal; All |
| Coal | 10102018 | External Combustion Boilers; Electric Generation; Waste Coal; Circulating Fluidized Bed Combustion |
| Residual Oil | 10102101 | External Combustion Boilers; Electric Generation; Other Oil; All |
| Coal | 10200101 | External Combustion Boilers; Industrial; Anthracite Coal; Pulverized Coal |
| Coal | 10200104 | External Combustion Boilers; Industrial; Anthracite Coal; Traveling Grate (Overfeed) Stoker |
| Coal | 10200107 | External Combustion Boilers; Industrial; Anthracite Coal; Hand-fired |
| Coal | 10200117 | External Combustion Boilers; Industrial; Anthracite Coal; Fluidized Bed Boiler Burning Anthracite-Culm Fuel |
| Coal | 10200201 | External Combustion Boilers; Industrial; Bituminous Coal; Pulverized Coal: Wet Bottom |
| Coal | 10200202 | External Combustion Boilers; Industrial; Bituminous Coal; Pulverized Coal: Dry Bottom |
| Coal | 10200203 | External Combustion Boilers; Industrial; Bituminous Coal; Cyclone Furnace |
| Coal | 10200204 | External Combustion Boilers; Industrial; Bituminous Coal; Spreader Stoker |
| Coal | 10200205 | External Combustion Boilers; Industrial; Bituminous Coal; Overfeed Stoker |
| Coal | 10200206 | External Combustion Boilers; Industrial; Bituminous Coal; Underfeed Stoker |
| Coal | 10200212 | External Combustion Boilers; Industrial; Bituminous Coal; Pulverized Coal: Dry Bottom (Tangential) |
| Coal | 10200213 | External Combustion Boilers; Industrial; Bituminous Coal; Wet Slurry |
| Coal | 10200217 | External Combustion Boilers; Industrial; Bituminous Coal; Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| Coal | 10200218 | External Combustion Boilers; Industrial; Bituminous Coal; Atmospheric Fluidized Bed Combustion: Circulating Bed |
| Coal | 10200219 | External Combustion Boilers; Industrial; Bituminous Coal; Cogeneration |
| Coal | 10200221 | External Combustion Boilers; Industrial; Subbituminous Coal; Pulverized Coal: Wet Bottom |
| Coal | 10200222 | External Combustion Boilers; Industrial; Subbituminous Coal; Pulverized Coal: Dry Bottom |
| Coal | 10200223 | External Combustion Boilers; Industrial; Subbituminous Coal; Cyclone Furnace |
| Coal | 10200224 | External Combustion Boilers; Industrial; Subbituminous Coal; Spreader Stoker |
| Coal | 10200225 | External Combustion Boilers; Industrial; Subbituminous Coal; Traveling Grate (Overfeed) Stoker |
| Coal | 10200226 | External Combustion Boilers; Industrial; Subbituminous Coal; Pulverized Coal: Dry Bottom (Tangential) |
| Coal | 10200229 | External Combustion Boilers; Industrial; Subbituminous Coal; Cogeneration |
| Coal | 10200300 | External Combustion Boilers; Industrial; Lignite; Pulverized Coal: Wet Bottom |
| Coal | 10200301 | External Combustion Boilers; Industrial; Lignite; Pulverized Coal: Dry Bottom, Wall Fired |
| Coal | 10200302 | External Combustion Boilers; Industrial; Lignite; Pulverized Coal: Dry Bottom, Tangential Fired |
| Coal | 10200303 | External Combustion Boilers; Industrial; Lignite; Cyclone Furnace |
| Coal | 10200304 | External Combustion Boilers; Industrial; Lignite; Traveling Grate (Overfeed) Stoker |
| Coal | 10200306 | External Combustion Boilers; Industrial; Lignite; Spreader Stoker |
| Coal | 10200307 | External Combustion Boilers; Industrial; Lignite; Cogeneration |
| Residual Oil | 10200401 | External Combustion Boilers; Industrial; Residual Oil; Grade 6 oil |
| Residual Oil | 10200402 | External Combustion Boilers; Industrial; Residual Oil; 10-100 Million BTU/hr |
| Residual Oil | 10200403 | External Combustion Boilers; Industrial; Residual Oil; < 10 Million BTU/hr |
| Residual Oil | 10200404 | External Combustion Boilers; Industrial; Residual Oil; Grade 5 Oil |
| Residual Oil | 10200405 | External Combustion Boilers; Industrial; Residual Oil; Cogeneration |
| Residual Oil | 10200406 | External Combustion Boilers; Industrial; Residual Oil; Boiler > 100 Million BTU/hr |
| Distillate Oil | 10200501 | External Combustion Boilers; Industrial; Distillate Oil - Grades 1 and 2; Boiler |
| Distillate Oil | 10200502 | External Combustion Boilers; Industrial; Distillate Oil; 10-100 Million BTU/hr |
| Distillate Oil | 10200503 | External Combustion Boilers; Industrial; Distillate Oil; < 10 Million BTU/hr |
| Distillate Oil | 10200504 | External Combustion Boilers; Industrial; Distillate Oil; Grade 4 Oil |
| Distillate Oil | 10200505 | External Combustion Boilers; Industrial; Distillate Oil; Cogeneration |
| Distillate Oil | 10200506 | External Combustion Boilers; Industrial; Distillate Oil; Boiler > 100 Million BTU/hr |
| Natural Gas | 10200601 | External Combustion Boilers; Industrial; Natural Gas; > 100 Million BTU/hr |
| Natural Gas | 10200602 | External Combustion Boilers; Industrial; Natural Gas; 10-100 Million BTU/hr |
| Natural Gas | 10200603 | External Combustion Boilers; Industrial; Natural Gas; < 10 Million BTU/hr |
| Natural Gas | 10200604 | External Combustion Boilers; Industrial; Natural Gas; Cogeneration |
| Wood | 10200901 | External Combustion Boilers; Industrial; Wood/Bark Waste; Bark-fired Boiler |
| Wood | 10200902 | External Combustion Boilers; Industrial; Wood/Bark Waste; Wood/Bark-fired Boiler |
| Wood | 10200903 | External Combustion Boilers; Industrial; Wood/Bark Waste; Wood-fired Boiler - Wet Wood (>=20% moisture) |
| Wood | 10200904 | External Combustion Boilers; Industrial; Wood/Bark Waste; Bark-fired Boiler (< 50,000 Lb Steam) |
| Wood | 10200905 | External Combustion Boilers; Industrial; Wood/Bark Waste; Wood/Bark-fired Boiler (< 50,000 Lb Steam) |
| Wood | 10200906 | External Combustion Boilers; Industrial; Wood/Bark Waste; Wood-fired Boiler (< 50,000 Lb Steam) |
| Wood | 10200907 | External Combustion Boilers; Industrial; Wood/Bark Waste; Wood Cogeneration |
| Wood | 10200908 | External Combustion Boilers; Industrial; Wood/Bark Waste; Wood-fired Boiler - Dry Wood (<20% moisture) |
| Wood | 10200910 | External Combustion Boilers; Industrial; Wood/Bark Waste; Fuel cell/Dutch oven boilers |
| Wood | 10200911 | External Combustion Boilers; Industrial; Wood/Bark Waste; Stoker boilers |
| Wood | 10200912 | External Combustion Boilers; Industrial; Wood/Bark Waste; Fluidized bed combustion boiler |
| LPG | 10201001 | External Combustion Boilers; Industrial; Liquified Petroleum Gas (LPG); Butane |
| LPG | 10201002 | External Combustion Boilers; Industrial; Liquified Petroleum Gas (LPG); Propane |
| LPG | 10201003 | External Combustion Boilers; Industrial; Liquified Petroleum Gas (LPG); Butane/Propane Mixture: Specify Percent Butane in Comments |
| Residual Oil | 10201302 | External Combustion Boilers; Industrial; Liquid Waste; Waste Oil |
| Natural Gas | 10201401 | External Combustion Boilers; Industrial; CO Boiler; Natural Gas |
| Distillate Oil | 10201403 | External Combustion Boilers; Industrial; CO Boiler; Distillate Oil |
| Residual Oil | 10201404 | External Combustion Boilers; Industrial; CO Boiler; Residual Oil |
| Wood | 10201801 | External Combustion Boilers; Industrial; Kiln-dried biomass; Boiler, Stoker |
| Wood | 10201802 | External Combustion Boilers; Industrial; Kiln-dried biomass; Boiler, Non-stoker |
| Wood | 10201901 | External Combustion Boilers; Industrial; Wood Residuals; Boiler, Stoker |
| Wood | 10201902 | External Combustion Boilers; Industrial; Wood Residuals; Boiler, non-stoker |
| Coal | 10300101 | External Combustion Boilers; Commercial/Institutional; Anthracite Coal; Pulverized Coal |
| Coal | 10300102 | External Combustion Boilers; Commercial/Institutional; Anthracite Coal; Traveling Grate (Overfeed) Stoker |
| Coal | 10300103 | External Combustion Boilers; Commercial/Institutional; Anthracite Coal; Hand-fired |
| Coal | 10300203 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Cyclone Furnace |
| Coal | 10300205 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Pulverized Coal: Wet Bottom |
| Coal | 10300206 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Pulverized Coal: Dry Bottom |
| Coal | 10300207 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Overfeed Stoker |
| Coal | 10300208 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Underfeed Stoker |
| Coal | 10300209 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Spreader Stoker |
| Coal | 10300214 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Hand-fired |
| Coal | 10300216 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Pulverized Coal: Dry Bottom (Tangential) |
| Coal | 10300217 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Atmospheric Fluidized Bed Combustion: Bubbling Bed |
| Coal | 10300218 | External Combustion Boilers; Commercial/Institutional; Bituminous Coal; Atmospheric Fluidized Bed Combustion: Circulating Bed |
| Coal | 10300221 | External Combustion Boilers; Commercial/Institutional; Subbituminous Coal; Pulverized Coal: Wet Bottom |
| Coal | 10300222 | External Combustion Boilers; Commercial/Institutional; Subbituminous Coal; Pulverized Coal: Dry Bottom |
| Coal | 10300223 | External Combustion Boilers; Commercial/Institutional; Subbituminous Coal; Cyclone Furnace |
| Coal | 10300224 | External Combustion Boilers; Commercial/Institutional; Subbituminous Coal; Spreader Stoker |
| Coal | 10300225 | External Combustion Boilers; Commercial/Institutional; Subbituminous Coal; Traveling Grate (Overfeed) Stoker |
| Coal | 10300226 | External Combustion Boilers; Commercial/Institutional; Subbituminous Coal; Pulverized Coal: Dry Bottom (Tangential) |
| Coal | 10300300 | External Combustion Boilers; Commercial/Institutional; Lignite; Pulverized Coal: Wet Bottom |
| Coal | 10300305 | External Combustion Boilers; Commercial/Institutional; Lignite; Pulverized Coal: Dry Bottom, Wall Fired |
| Coal | 10300306 | External Combustion Boilers; Commercial/Institutional; Lignite; Pulverized Coal: Dry Bottom, Tangential Fired |
| Coal | 10300307 | External Combustion Boilers; Commercial/Institutional; Lignite; Traveling Grate (Overfeed) Stoker |
| Coal | 10300309 | External Combustion Boilers; Commercial/Institutional; Lignite; Spreader Stoker |
| Residual Oil | 10300401 | External Combustion Boilers; Commercial/Institutional; Residual Oil - Grade 6; Boiler |
| Residual Oil | 10300402 | External Combustion Boilers; Commercial/Institutional; Residual Oil; 10-100 Million BTU/hr |
| Residual Oil | 10300403 | External Combustion Boilers; Commercial/Institutional; Residual Oil; < 10 Million BTU/hr |
| Residual Oil | 10300404 | External Combustion Boilers; Commercial/Institutional; Residual Oil; Grade 5 Oil |
| Residual Oil | 10300405 | External Combustion Boilers; Commercial/Institutional; Residual Oil; Boiler > 100 Million BTU/hr |
| Distillate Oil | 10300501 | External Combustion Boilers; Commercial/Institutional; Distillate Oil - Grades 1 and 2; Boiler |
| Distillate Oil | 10300502 | External Combustion Boilers; Commercial/Institutional; Distillate Oil; 10-100 Million BTU/hr |
| Distillate Oil | 10300503 | External Combustion Boilers; Commercial/Institutional; Distillate Oil; < 10 Million BTU/hr |
| Distillate Oil | 10300504 | External Combustion Boilers; Commercial/Institutional; Distillate Oil; Grade 4 Oil |
| Distillate Oil | 10300505 | External Combustion Boilers; Commercial/Institutional; Distillate Oil; Boiler > 100 Million BTU/hr |
| Natural Gas | 10300601 | External Combustion Boilers; Commercial/Institutional; Natural Gas; > 100 Million BTU/hr |
| Natural Gas | 10300602 | External Combustion Boilers; Commercial/Institutional; Natural Gas; 10-100 Million BTU/hr |
| Natural Gas | 10300603 | External Combustion Boilers; Commercial/Institutional; Natural Gas; < 10 Million BTU/hr |
| Wood | 10300901 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Bark-fired Boiler |
| Wood | 10300902 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Wood/Bark-fired Boiler |
| Wood | 10300903 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Wood-fired Boiler - Wet Wood (>=20% moisture) |
| Wood | 10300908 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Wood-fired Boiler - Dry Wood (<20% moisture) |
| Wood | 10300910 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Fuel cell/Dutch oven boilers |
| Wood | 10300911 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Stoker boilers |
| Wood | 10300912 | External Combustion Boilers; Commercial/Institutional; Wood/Bark Waste; Fluidized bed combustion boiler |
| LPG | 10301001 | External Combustion Boilers; Commercial/Institutional; Liquified Petroleum Gas (LPG); Butane |
| LPG | 10301002 | External Combustion Boilers; Commercial/Institutional; Liquified Petroleum Gas (LPG); Propane |
| LPG | 10301003 | External Combustion Boilers; Commercial/Institutional; Liquified Petroleum Gas (LPG); Butane/Propane Mixture: Specify Percent Butane in Comments |
| Wood | 10301101 | External Combustion Boilers; Commercial/Institutional; Biomass; Boiler, Stoker |
| Wood | 10301102 | External Combustion Boilers; Commercial/Institutional; Biomass; Boiler, Non-stoker |
| Residual Oil | 10301302 | External Combustion Boilers; Commercial/Institutional; Liquid Waste; Waste Oil |
| Distillate Oil | 10500105 | External Combustion; Space Heaters; Industrial; Distillate Oil |
| Natural Gas | 10500106 | External Combustion; Space Heaters; Industrial; Natural Gas |
| LPG | 10500110 | External Combustion; Space Heaters; Industrial; Liquified Petroleum Gas (LPG) |
| Residual Oil | 10500113 | External Combustion; Space Heaters;Industrial; Waste Oil: Air Atomized Burner |
| Residual Oil | 10500114 | External Combustion; Space Heaters;Industrial; Waste Oil: Vaporizing Burner |
| Distillate Oil | 10500205 | External Combustion; Space Heaters; Commercial/Institutional; Distillate Oil |
| Natural Gas | 10500206 | External Combustion; Space Heaters; Commercial/Institutional; Natural Gas |
| Wood | 10500209 | External Combustion; Space Heaters; Commercial/Institutional; Wood |
| LPG | 10500210 | External Combustion; Space Heaters; Commercial/Institutional; Liquified Petroleum Gas (LPG) |
| Residual Oil | 10500213 | External Combustion; Space Heaters; Commercial/Institutional; Waste Oil: Air Atomized Burner |
| Residual Oil | 10500214 | External Combustion; Space Heaters; Commercial/Institutional; Waste Oil: Vaporizing Burner |
| Distillate Oil | 20100101 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Turbine |
| Distillate Oil | 20100102 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Reciprocating |
| Distillate Oil | 20100105 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Reciprocating: Crankcase Blowby |
| Distillate Oil | 20100106 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| Distillate Oil | 20100107 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Reciprocating: Exhaust |
| Distillate Oil | 20100108 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| Distillate Oil | 20100109 | Internal Combustion Engines; Electric Generation; Distillate Oil (Diesel); Turbine: Exhaust |
| Natural Gas | 20100201 | Internal Combustion Engines; Electric Generation; Natural Gas; Turbine |
| Natural Gas | 20100202 | Internal Combustion Engines; Electric Generation; Natural Gas; Reciprocating |
| Natural Gas | 20100205 | Internal Combustion Engines; Electric Generation; Natural Gas; Reciprocating: Crankcase Blowby |
| Natural Gas | 20100206 | Internal Combustion Engines; Electric Generation; Natural Gas; Reciprocating: Evaporative Losses (Fuel Delivery System) |
| Natural Gas | 20100207 | Internal Combustion Engines; Electric Generation; Natural Gas; Reciprocating: Exhaust |
| Natural Gas | 20100208 | Internal Combustion Engines; Electric Generation; Natural Gas; Turbine: Evaporative Losses (Fuel Delivery System) |
| Natural Gas | 20100209 | Internal Combustion Engines; Electric Generation; Natural Gas; Turbine: Exhaust |
| Coal | 20100301 | Internal Combustion Engines; Electric Generation; Gasified Coal; Turbine |
| Kerosene | 20100901 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Turbine |
| Kerosene | 20100902 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Reciprocating |
| Kerosene | 20100905 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Reciprocating: Crankcase Blowby |
| Kerosene | 20100906 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Reciprocating: Evaporative Losses (Fuel Delivery System) |
| Kerosene | 20100907 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Reciprocating: Exhaust |
| Kerosene | 20100908 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| Kerosene | 20100909 | Internal Combustion Engines; Electric Generation; Kerosene/Naphtha (Jet Fuel); Turbine: Exhaust |
| Residual Oil | 20101302 | Internal Combustion Engines; Electric Generation; Liquid Waste; Waste Oil - Turbine |
| Distillate Oil | 20200101 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Turbine |
| Distillate Oil | 20200102 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Reciprocating |
| Distillate Oil | 20200103 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Turbine: Cogeneration |
| Distillate Oil | 20200104 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Reciprocating: Cogeneration |
| Distillate Oil | 20200105 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Reciprocating: Crankcase Blowby |
| Distillate Oil | 20200106 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| Distillate Oil | 20200107 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Reciprocating: Exhaust |
| Distillate Oil | 20200108 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| Distillate Oil | 20200109 | Internal Combustion Engines; Industrial; Distillate Oil (Diesel); Turbine: Exhaust |
| Natural Gas | 20200201 | Internal Combustion Engines; Industrial; Natural Gas; Turbine |
| Natural Gas | 20200202 | Internal Combustion Engines; Industrial; Natural Gas; Reciprocating |
| Natural Gas | 20200203 | Internal Combustion Engines; Industrial; Natural Gas; Turbine: Cogeneration |
| Natural Gas | 20200204 | Internal Combustion Engines; Industrial; Natural Gas; Reciprocating: Cogeneration |
| Natural Gas | 20200205 | Internal Combustion Engines; Industrial; Natural Gas; Reciprocating: Crankcase Blowby |
| Natural Gas | 20200206 | Internal Combustion Engines; Industrial; Natural Gas; Reciprocating: Evaporative Losses (Fuel Delivery System) |
| Natural Gas | 20200207 | Internal Combustion Engines; Industrial; Natural Gas; Reciprocating: Exhaust |
| Natural Gas | 20200208 | Internal Combustion Engines; Industrial; Natural Gas; Turbine: Evaporative Losses (Fuel Delivery System) |
| Natural Gas | 20200209 | Internal Combustion Engines; Industrial; Natural Gas; Turbine: Exhaust |
| Natural Gas | 20200251 | Internal Combustion Engines; Industrial; Natural Gas; 2-cycle Rich Burn |
| Natural Gas | 20200252 | Internal Combustion Engines; Industrial; Natural Gas; 2-cycle Lean Burn |
| Natural Gas | 20200253 | Internal Combustion Engines; Industrial; Natural Gas; 4-cycle Rich Burn |
| Natural Gas | 20200254 | Internal Combustion Engines; Industrial; Natural Gas; 4-cycle Lean Burn |
| Natural Gas | 20200255 | Internal Combustion Engines; Industrial; Natural Gas; 2-cycle Clean Burn |
| Natural Gas | 20200256 | Internal Combustion Engines; Industrial; Natural Gas; 4-cycle Clean Burn |
| Distillate Oil | 20200401 | Internal Combustion Engines; Industrial; Diesel; Large Bore Engine |
| Residual Oil | 20200501 | Internal Combustion Engines; Industrial; Residual/Crude Oil; Reciprocating |
| Residual Oil | 20200505 | Internal Combustion Engines; Industrial; Residual/Crude Oil; Reciprocating: Crankcase Blowby |
| Residual Oil | 20200506 | Internal Combustion Engines; Industrial; Residual/Crude Oil; Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| Residual Oil | 20200507 | Internal Combustion Engines; Industrial; Residual/Crude Oil; Reciprocating: Exhaust |
| Kerosene | 20200901 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Turbine |
| Kerosene | 20200902 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Reciprocating |
| Kerosene | 20200905 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Reciprocating: Crankcase Blowby |
| Kerosene | 20200906 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| Kerosene | 20200907 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Reciprocating: Exhaust |
| Kerosene | 20200908 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| Kerosene | 20200909 | Internal Combustion Engines; Industrial; Kerosene/Naphtha (Jet Fuel); Turbine: Exhaust |
| LPG | 20201001 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Propane: Reciprocating |
| LPG | 20201002 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Butane: Reciprocating |
| LPG | 20201005 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Reciprocating: Crankcase Blowby |
| LPG | 20201006 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| LPG | 20201007 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Reciprocating: Exhaust |
| LPG | 20201008 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| LPG | 20201009 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Turbine: Exhaust |
| LPG | 20201011 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Turbine |
| LPG | 20201012 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Reciprocating Engine |
| LPG | 20201013 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Turbine: Cogeneration |
| LPG | 20201014 | Internal Combustion Engines; Industrial; Liquified Petroleum Gas (LPG); Reciprocating Engine: Cogeneration |
| Distillate Oil | 20300101 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Reciprocating |
| Distillate Oil | 20300102 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Turbine |
| Distillate Oil | 20300105 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Reciprocating: Crankcase Blowby |
| Distillate Oil | 20300106 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| Distillate Oil | 20300107 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Reciprocating: Exhaust |
| Distillate Oil | 20300108 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| Distillate Oil | 20300109 | Internal Combustion Engines; Commercial/Institutional; Distillate Oil (Diesel); Turbine: Exhaust |
| Natural Gas | 20300201 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Reciprocating |
| Natural Gas | 20300202 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Turbine |
| Natural Gas | 20300203 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Turbine: Cogeneration |
| Natural Gas | 20300204 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Reciprocating: Cogeneration |
| Natural Gas | 20300205 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Reciprocating: Crankcase Blowby |
| Natural Gas | 20300206 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Reciprocating: Evaporative Losses (Fuel Delivery System) |
| Natural Gas | 20300207 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Reciprocating: Exhaust |
| Natural Gas | 20300208 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Turbine: Evaporative Losses (Fuel Delivery System) |
| Natural Gas | 20300209 | Internal Combustion Engines; Commercial/Institutional; Natural Gas; Turbine: Exhaust |
| Distillate Oil | 20300401 | Internal Combustion Engines; Commercial/Institutional; Diesel; Large Bore Engine |
| Kerosene | 20300901 | Internal Combustion Engines; Commercial/Institutional; Kerosene/Naphtha (Jet Fuel); Turbine: JP-4 |
| Kerosene | 20300908 | Internal Combustion Engines; Commercial/Institutional; Kerosene/Naphtha (Jet Fuel); Turbine: Evaporative Losses (Fuel Storage and Delivery System) |
| Kerosene | 20300909 | Internal Combustion Engines; Commercial/Institutional; Kerosene/Naphtha (Jet Fuel); Turbine: Exhaust |
| LPG | 20301001 | Internal Combustion Engines; Commercial/Institutional; Liquified Petroleum Gas (LPG); Propane: Reciprocating |
| LPG | 20301002 | Internal Combustion Engines; Commercial/Institutional; Liquified Petroleum Gas (LPG); Butane: Reciprocating |
| LPG | 20301005 | Internal Combustion Engines; Commercial/Institutional; Liquified Petroleum Gas (LPG); Reciprocating: Crankcase Blowby |
| LPG | 20301006 | Internal Combustion Engines; Commercial/Institutional; Liquified Petroleum Gas (LPG); Reciprocating: Evaporative Losses (Fuel Storage and Delivery System) |
| LPG | 20301007 | Internal Combustion Engines; Commercial/Institutional; Liquified Petroleum Gas (LPG); Reciprocating: Exhaust |
| Kerosene | 20400101 | Internal Combustion Engines; Engine Testing; Aircraft Engine Testing; Turbojet |
| Kerosene | 20400102 | Internal Combustion Engines; Engine Testing; Aircraft Engine Testing; Turboshaft |
| Kerosene | 20400110 | Internal Combustion Engines; Engine Testing; Aircraft Engine Testing; Jet A Fuel |
| Kerosene | 20400111 | Internal Combustion Engines; Engine Testing; Aircraft Engine Testing; JP-5 Fuel |
| Kerosene | 20400112 | Internal Combustion Engines; Engine Testing; Aircraft Engine Testing; JP-4 Fuel |
| Natural Gas | 20400301 | Internal Combustion Engines; Engine Testing; Turbine; Natural Gas |
| Distillate Oil | 20400302 | Internal Combustion Engines; Engine Testing; Turbine; Diesel/Kerosene |
| Distillate Oil | 20400303 | Internal Combustion Engines; Engine Testing; Turbine; Distillate Oil |
| Kerosene | 20400305 | Internal Combustion Engines; Engine Testing; Turbine; Kerosene/Naphtha |
| Distillate Oil | 20400402 | Internal Combustion Engines; Engine Testing; Reciprocating Engine; Diesel/Kerosene |
| Distillate Oil | 20400403 | Internal Combustion Engines; Engine Testing; Reciprocating Engine; Distillate Oil |
| Kerosene | 20400406 | Internal Combustion Engines; Engine Testing; Reciprocating Engine; Kerosene/Naphtha (Jet Fuel) |
| Residual Oil | 20400408 | Internal Combustion Engines; Engine Testing; Reciprocating Engine; Residual Oil/Crude Oil |
| LPG | 20400409 | Internal Combustion Engines; Engine Testing; Reciprocating Engine; Liquified Petroleum Gas (LPG) |
| Distillate Oil | 27000320 | Internal Combustion Engines; Off-highway Diesel Engines; Industrial Equipment; Industrial Fork Lift: Diesel |
| LPG | 27300320 | Internal Combustion Engines; Off-highway LPG-fueled Engines; Industrial Equipment; Industrial Fork Lift: Liquified Petroleum Gas (LPG) |
| Distillate Oil | 30190001 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Process Heater: Distillate Oil (No. 2) |
| Residual Oil | 30190002 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Process Heater: Residual Oil |
| Natural Gas | 30190003 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Process Heater: Natural Gas |
| Distillate Oil | 30190011 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Incinerator: Distillate Oil (No. 2) |
| Residual Oil | 30190012 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Incinerator: Residual Oil |
| Natural Gas | 30190013 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Incinerator: Natural Gas |
| Distillate Oil | 30190021 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Flare: Distillate Oil (No. 2) |
| Residual Oil | 30190022 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Flare: Residual Oil |
| Natural Gas | 30190023 | Industrial Processes; Chemical Manufacturing; Fuel Fired Equipment; Flare: Natural Gas |
| Coal | 30200112 | Industrial Processes; Food and Agriculture; Alfalfa Dehydration; Coal-fired, Triple-Pass Dryer Cyclone |
| Wood | 30200117 | Industrial Processes; Food and Agriculture; Alfalfa Dehydration; Wood-fired, Single-Pass Dryer Cyclone |
| Natural Gas | 30200220 | Industrial Processes; Food and Agriculture; Coffee Roasting; Indirect-fired Batch Roaster -Natural Gas (incl combustion emiss) |
| Natural Gas | 30200221 | Industrial Processes; Food and Agriculture; Coffee Roasting; Indirect-fired Continuous Roaster -Natural Gas (incl combustion emiss) |
| Natural Gas | 30200224 | Industrial Processes; Food and Agriculture; Coffee Roasting; Direct-fired Batch Roaster - Natural Gas |
| Natural Gas | 30200225 | Industrial Processes; Food and Agriculture; Coffee Roasting; Direct-fired Continuous Roaster - Natural Gas |
| Coal | 30201601 | Industrial Processes; Food and Agriculture; Sugar Beet Processing; Pulp Dryer : Coal-fired |
| Distillate Oil | 30201605 | Industrial Processes; Food and Agriculture; Sugar Beet Processing; Pulp Dryer : Oil-fired |
| Natural Gas | 30201608 | Industrial Processes; Food and Agriculture; Sugar Beet Processing; Pulp Dryer : Natural Gas-fired |
| Coal | 30201684 | Industrial Processes; Food and Agriculture; Sugar Beet Processing; Lime Kiln : Coal-fired |
| Natural Gas | 30201686 | Industrial Processes; Food and Agriculture; Sugar Beet Processing; Lime Kiln : Natural Gas-fired |
| Natural Gas | 30203811 | Industrial Processes; Food and Agriculture; Animal/Poultry Rendering; Blood Dryer: Natural Gas Direct Fired |
| Natural Gas | 30205020 | Industrial Processes; Food and Agriculture; Ethanol Production; Natural Gas Combustion from Dryer |
| Natural Gas | 30205021 | Industrial Processes; Food and Agriculture; Ethanol Production; Natural Gas Combustion from Thermal Oxidizer |
| Distillate Oil | 30290001 | Industrial Processes; Food and Agriculture; Fuel Fired Equipment; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 30290002 | Industrial Processes; Food and Agriculture; Fuel Fired Equipment; Residual Oil: Process Heaters |
| Natural Gas | 30290003 | Industrial Processes; Food and Agriculture; Fuel Fired Equipment; Natural Gas: Process Heaters |
| LPG | 30290005 | Industrial Processes; Food and Agriculture; Fuel Fired Equipment; Liquified Petroleum Gas (LPG): Process Heaters |
| Natural Gas | 30291001 | Industrial Processes; Food and Agriculture; Fuel Fired Equipment; Broiling Food: Natural Gas |
| Natural Gas | 30302312 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Indurating Furnace: Gas Fired (see 3-03-023-51 thru -88) |
| Distillate Oil | 30302313 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Indurating Furnace: Oil Fired (see 3-03-023-51 thru -88) |
| Coal | 30302314 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Indurating Furnace: Coal Fired (see 3-03-023-51 thru -88) |
| Natural Gas | 30302351 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Grate/Kiln, Gas-fired, Acid Pellets |
| Natural Gas | 30302352 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Grate/Kiln, Gas-fired, Flux Pellets |
| Coal | 30302359 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Grate/Kiln, Coal-fired, Acid Pellets |
| Coal | 30302360 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Grate/Kiln, Coal-fired, Flux Pellets |
| Natural Gas | 30302381 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Straight Grate, Gas-fired, Acid Pellets |
| Natural Gas | 30302382 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Straight Grate, Gas-fired, Flux Pellets |
| Distillate Oil | 30302383 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Straight Grate, Oil-fired, Acid Pellets |
| Distillate Oil | 30302384 | Industrial Processes; Primary Metal Production; Taconite Iron Ore Processing; Induration: Straight Grate, Oil-fired, Flux Pellets |
| Distillate Oil | 30390001 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 30390002 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Residual Oil: Process Heaters |
| Natural Gas | 30390003 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Natural Gas: Process Heaters |
| Distillate Oil | 30390011 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Incinerators |
| Residual Oil | 30390012 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Residual Oil: Incinerators |
| Natural Gas | 30390013 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Natural Gas: Incinerators |
| Distillate Oil | 30390021 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Flares |
| Residual Oil | 30390022 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Residual Oil: Flares |
| Natural Gas | 30390023 | Industrial Processes; Primary Metal Production; Fuel Fired Equipment; Natural Gas: Flares |
| Distillate Oil | 30400406 | Industrial Processes; Secondary Metal Production; Lead; Refining Kettle: Pot Furnace Heater: Distillate Oil |
| Natural Gas | 30400407 | Industrial Processes; Secondary Metal Production; Lead; Refining Kettle: Pot Furnace Heater: Natural Gas |
| Natural Gas | 30400740 | Industrial Processes; Secondary Metal Production; Steel Foundries; Reheating Furnace: Natural Gas |
| Distillate Oil | 30490001 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 30490002 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Residual Oil: Process Heaters |
| Natural Gas | 30490003 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Natural Gas: Process Heaters |
| Distillate Oil | 30490011 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Incinerators |
| Residual Oil | 30490012 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Residual Oil: Incinerators |
| Natural Gas | 30490013 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Natural Gas: Incinerators |
| Distillate Oil | 30490021 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Flares |
| Residual Oil | 30490022 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Residual Oil: Flares |
| Natural Gas | 30490023 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Natural Gas: Flares |
| Distillate Oil | 30490031 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Distillate Oil (No. 2): Furnaces |
| Residual Oil | 30490032 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Residual Oil: Furnaces |
| Natural Gas | 30490033 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Natural Gas: Furnaces |
| LPG | 30490035 | Industrial Processes; Secondary Metal Production; Fuel Fired Equipment; Propane: Furnaces |
| Natural Gas | 30500206 | Industrial Processes; Mineral Products; Asphalt Concrete; Asphalt Heater: Natural Gas |
| Residual Oil | 30500207 | Industrial Processes; Mineral Products; Asphalt Concrete; Asphalt Heater: Residual Oil |
| Distillate Oil | 30500208 | Industrial Processes; Mineral Products; Asphalt Concrete; Asphalt Heater: Distillate Oil |
| LPG | 30500209 | Industrial Processes; Mineral Products; Asphalt Concrete; Asphalt Heater: LPG |
| Natural Gas | 30500251 | Industrial Processes; Mineral Products; Asphalt Concrete; Batch Mix Plant: Rotary Dryer, Natural Gas-Fired (also see -45) |
| Distillate Oil | 30500252 | Industrial Processes; Mineral Products; Asphalt Concrete; Batch Mix Plant: Rotary Dryer, Oil-Fired (also see -46) |
| Residual Oil | 30500253 | Industrial Processes; Mineral Products; Asphalt Concrete; Batch Mix Plant: Rotary Dryer, Waste/Drain/# 6 Oil-Fired (also see -47 |
| Natural Gas | 30500255 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer / Mixer, Natural Gas-Fired |
| Natural Gas | 30500256 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer / Mixer, Natural Gas, Parallel Flow |
| Natural Gas | 30500257 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer / Mixer, Natural Gas, Counterflow |
| Distillate Oil | 30500258 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer / Mixer, #2 Oil-Fired |
| Distillate Oil | 30500259 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer / Mixer, #2 Oil-Fired, Parallel Flow |
| Distillate Oil | 30500260 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer / Mixer, #2 Oil-Fired, Counterflow |
| Residual Oil | 30500261 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Plant: Rotary Drum Dryer/Mixer, Waste/Drain/#6 Oil-Fired |
| Residual Oil | 30500262 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Pl: Rotary Drum Dryer/Mixer, Waste/Drain/#6 Oil, Parallel Flo |
| Residual Oil | 30500263 | Industrial Processes; Mineral Products; Asphalt Concrete; Drum Mix Pl: Rotary Drum Dryer/Mixer, Waste/Drain/#6 Oil, Counterflow |
| Natural Gas | 30500311 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Tunnel Kiln: Gas-fired |
| Distillate Oil | 30500312 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Tunnel Kiln: Oil-fired |
| Coal | 30500313 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Tunnel Kiln: Coal-fired |
| Natural Gas | 30500314 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Periodic Kiln: Gas-fired |
| Distillate Oil | 30500315 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Periodic Kiln: Oil-fired |
| Coal | 30500316 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Periodic Kiln: Coal-fired |
| Wood | 30500318 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Tunnel Kiln: Wood-fired |
| Natural Gas | 30500322 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Tunnel Kiln: Gas-fired: High Sulfur Material |
| Natural Gas | 30500332 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Other Kiln: Gas-fired |
| Distillate Oil | 30500333 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Other Kiln: Oil Fired |
| Coal | 30500334 | Industrial Processes; Mineral Products; Brick and Structural Clay Products Manufacture; Other Kiln: Coal-Fired |
| Natural Gas | 30500810 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Spray Dryer: Natural Gas-fired |
| Natural Gas | 30500821 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Rotary Calciner: Natural Gas-fired |
| Natural Gas | 30500823 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Fluidized Bed Calciner: Natural Gas-fired |
| Natural Gas | 30500840 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Presinter Thermal Processing: Natural Gas-fired Kiln |
| Natural Gas | 30500849 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Roller Kiln: Natural Gas-fired |
| Natural Gas | 30500850 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Tunnel Kiln: Natural Gas-fired |
| Natural Gas | 30500851 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Shuttle Kiln: Natural Gas-fired |
| Natural Gas | 30500856 | Industrial Processes; Mineral Products; Clay Ceramics Manufacture; Refiring: Natural Gas-fired Kiln |
| Coal | 30501618 | Industrial Processes; Mineral Products; Lime Manufacture; Calcining: Coal-fired Rotary Kiln |
| Natural Gas | 30501619 | Industrial Processes; Mineral Products; Lime Manufacture; Calcining: Gas-fired Rotary Kiln |
| Coal | 30501620 | Industrial Processes; Mineral Products; Lime Manufacture; Calcining: Coal- and Gas-fired Rotary Kiln |
| Coal | 30501621 | Industrial Processes; Mineral Products; Lime Manufacture; Calcining: Coal- and Coke-fired Rotary Kiln |
| Coal | 30501622 | Industrial Processes; Mineral Products; Lime Manufacture; Calcining: Coal-fired Rotary Preheater Kiln |
| Natural Gas | 30501623 | Industrial Processes; Mineral Products; Lime Manufacture; Calcining: Gas-fired Parallel Flow Regenerative Kiln |
| Natural Gas | 30502720 | Industrial Processes; Mineral Products; Industrial Sand and Gravel; Sand Drying: Gas- or Oil-fired Rotary or Fluidized Bed Dryer |
| Natural Gas | 30502721 | Industrial Processes; Mineral Products; Industrial Sand and Gravel; Sand Drying: Gas-fired Rotary Dryer |
| Distillate Oil | 30502722 | Industrial Processes; Mineral Products; Industrial Sand and Gravel; Sand Drying: Oil-fired Rotary Dryer |
| Natural Gas | 30502723 | Industrial Processes; Mineral Products; Industrial Sand and Gravel; Sand Drying: Gas-fired Fluidized Bed Dryer |
| Distillate Oil | 30502724 | Industrial Processes; Mineral Products; Industrial Sand and Gravel; Sand Drying: Oil-fired Fluidized Bed Dryer |
| Natural Gas | 30505020 | Industrial Processes; Mineral Products; Asphalt Processing (Blowing); Asphalt Heater: Natural Gas |
| Residual Oil | 30505021 | Industrial Processes; Mineral Products; Asphalt Processing (Blowing); Asphalt Heater: Residual Oil |
| Distillate Oil | 30505022 | Industrial Processes; Mineral Products; Asphalt Processing (Blowing); Asphalt Heater: Distillate Oil |
| LPG | 30505023 | Industrial Processes; Mineral Products; Asphalt Processing (Blowing); Asphalt Heater: LPG |
| Natural Gas | 30508909 | Industrial Processes; Mineral Products; Talc Processing; Natural Gas Fired Crude Ore Dryer |
| Natural Gas | 30508921 | Industrial Processes; Mineral Products; Talc Processing; Natural Gas-fired Rotary Dryer |
| Natural Gas | 30508931 | Industrial Processes; Mineral Products; Talc Processing; Natural Gas-fired Rotary Calciner |
| Natural Gas | 30508971 | Industrial Processes; Mineral Products; Talc Processing; Natural Gas-fired Flash Drying of Slurry after Flotation |
| Distillate Oil | 30590001 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 30590002 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Residual Oil: Process Heaters |
| Natural Gas | 30590003 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Natural Gas: Process Heaters |
| LPG | 30590005 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Liquified Petroleum Gas (LPG): Process Heaters |
| Distillate Oil | 30590011 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Distillate Oil (No. 2): Incinerators |
| Residual Oil | 30590012 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Residual Oil: Incinerators |
| Natural Gas | 30590013 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Natural Gas: Incinerators |
| Distillate Oil | 30590021 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Distillate Oil (No. 2): Flares |
| Natural Gas | 30590023 | Industrial Processes; Mineral Products; Fuel Fired Equipment; Natural Gas: Flares |
| Distillate Oil | 30600101 | Industrial Processes; Petroleum Industry; Process Heaters; Oil-fired |
| Natural Gas | 30600102 | Industrial Processes; Petroleum Industry; Process Heaters; Gas-fired |
| Distillate Oil | 30600103 | Industrial Processes; Petroleum Industry; Process Heaters; Oil |
| Natural Gas | 30600104 | Industrial Processes; Petroleum Industry; Process Heaters; Gas |
| Natural Gas | 30600105 | Industrial Processes; Petroleum Industry; Process Heaters; Natural Gas |
| LPG | 30600107 | Industrial Processes; Petroleum Industry; Process Heaters; Liquified Petroleum Gas (LPG) |
| Residual Oil | 30600111 | Industrial Processes; Petroleum Industry; Process Heaters; No. 6 Oil |
| Natural Gas | 30600203 | Industrial Processes; Petroleum Industry; Catalytic Cracking Unit; Fluid Catalytic Cracking Unit with CO Boiler: Natural Gas |
| Distillate Oil | 30600205 | Industrial Processes; Petroleum Industry; Catalytic Cracking Unit; Fluid Catalytic Cracking Unit with CO Boiler: Oil |
| Distillate Oil | 30600901 | Industrial Processes; Petroleum Industry; Flares; Distillate Oil |
| Residual Oil | 30600902 | Industrial Processes; Petroleum Industry; Flares; Residual Oil |
| Natural Gas | 30600903 | Industrial Processes; Petroleum Industry; Flares; Natural Gas |
| LPG | 30600905 | Industrial Processes; Petroleum Industry; Flares; Liquified Petroleum Gas |
| Distillate Oil | 30609901 | Industrial Processes; Petroleum Industry; Incinerators; Distillate Oil (No. 2) |
| Residual Oil | 30609902 | Industrial Processes; Petroleum Industry; Incinerators; Residual Oil |
| Natural Gas | 30609903 | Industrial Processes; Petroleum Industry; Incinerators; Natural Gas |
| LPG | 30609905 | Industrial Processes; Petroleum Industry; Incinerators; Liquified Petroleum Gas (LPG) |
| Wood | 30700670 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Dry Rotary Dryer: Direct Wood-fired: <600F Inlet air, <30%MC: Hardwood |
| Wood | 30700671 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Dry Rotary Dryer: Direct Wood-fired: <600F Inlet air, <30%MC: Softwood |
| Wood | 30700672 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Dry Rotary Dryer: Direct Wood-fired: <600F Inlet air, <30%MC: Mixed Softwood/Hardwood |
| Natural Gas | 30700673 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Dry Rotary Dryer: Direct Natural Gas-fired: <600F Inlet air, <30%MC: Hardwood |
| Natural Gas | 30700674 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Dry Rotary Dryer: Direct Natural Gas-fired: <600F Inlet air, <30%MC: Softwood |
| Natural Gas | 30700675 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Dry Rotary Dryer: Direct Natural Gas-fired: <600F Inlet air, <30%MC: Mixed Softwood/Hardwood |
| Wood | 30700676 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Green Rotary Dryer: Direct Wood-fired: Hardwood |
| Wood | 30700677 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Green Rotary Dryer: Direct Wood-fired: Softwood |
| Wood | 30700678 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Green Rotary Dryer: Direct Wood-fired: Mixed Softwood/Hardwood |
| Natural Gas | 30700679 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Green Rotary Dryer: Direct Natural Gas-fired: Hardwood |
| Natural Gas | 30700680 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Green Rotary Dryer: Direct Natural Gas-fired: Softwood |
| Natural Gas | 30700681 | Industrial Processes; Pulp and Paper and Wood Products; Particleboard Manufacture; Green Rotary Dryer: Direct Natural Gas-fired: Mixed Softwood/Hardwood |
| Wood | 30700734 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Hardwood Veneer Dryer: Direct Wood-fired: Heated Zones |
| Wood | 30700735 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Hardwood Veneer Dryer: Direct Wood-fired: Cooling Section |
| Wood | 30700736 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Softwood Veneer Dryer: Direct Wood-fired: Heated Zones |
| Wood | 30700737 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Softwood Veneer Dryer: Direct Wood-fired: Cooling Section |
| Natural Gas | 30700752 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Softwood Veneer Dryer: Direct Natural Gas-fired: Heated Zones |
| Natural Gas | 30700753 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Softwood Veneer Dryer: Direct Natural Gas-fired: Cooling Section |
| Natural Gas | 30700754 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Hardwood Veneer Dryer: Direct Natural Gas-fired: Heated Zones |
| Natural Gas | 30700755 | Industrial Processes; Pulp and Paper and Wood Products; Plywood Operations; Hardwood Veneer Dryer: Direct Natural Gas-fired: Cooling Section |
| Wood | 30700844 | Industrial Processes; Pulp and Paper and Wood Products; Sawmill Operations; Lumber Kiln: Direct-fired: Softwood: Pine Species |
| Wood | 30700845 | Industrial Processes; Pulp and Paper and Wood Products; Sawmill Operations; Lumber Kiln: Direct-fired: Softwood: Non-Pine Species |
| Wood | 30700846 | Industrial Processes; Pulp and Paper and Wood Products; Sawmill Operations; Lumber Kiln: Direct-fired: Hardwood |
| Wood | 30700893 | Industrial Processes; Pulp and Paper and Wood Products; Sawmill Operations; Log Storage: Softwood |
| Natural Gas | 30700909 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Blowline Blend: Non-Urea Formaldeh |
| Natural Gas | 30700910 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Blowline Blend: Non-Urea Formaldeh |
| Natural Gas | 30700911 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Blowline Blend: Non-Urea Formaldeh |
| Natural Gas | 30700912 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Blowline Blend: Urea Formaldehyde |
| Natural Gas | 30700913 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Blowline Blend: Urea Formaldehyde |
| Natural Gas | 30700914 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Blowline Blend: Urea Formaldehyde |
| Wood | 30700915 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Non-Blowline Blend: Softwood |
| Wood | 30700916 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Non-Blowline Blend: Hardwood |
| Wood | 30700917 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Non-Blowline Blend: Mixed Softwood/Hardwo |
| Wood | 30700918 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Non-Urea Formaldehyde Res |
| Wood | 30700919 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Non-Urea Formaldehyde Res |
| Wood | 30700920 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Non-Urea Formaldehyde Res |
| Wood | 30700924 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Urea Formaldehyde Resin: |
| Natural Gas | 30700926 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Non-Blowline Blend: Softwood |
| Natural Gas | 30700927 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Non-Blowline Blend: Hardwood |
| Natural Gas | 30700928 | Industrial Processes; Pulp and Paper and Wood Products; Medium Density Fiberboard (MDF) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Natural Gas-fired: Non-Blowline Blend: Mixed Softwood |
| Wood | 30701009 | Industrial Processes; Pulp and Paper and Wood Products; Oriented Strandboard (OSB) Manufacture; Rotary Strand Dryer: Direct Wood-fired: Softwood |
| Wood | 30701010 | Industrial Processes; Pulp and Paper and Wood Products; Oriented Strandboard (OSB) Manufacture; Rotary Strand Dryer: Direct Wood-fired: Hardwood |
| Wood | 30701015 | Industrial Processes; Pulp and Paper and Wood Products; Oriented Strandboard (OSB) Manufacture; Rotary Strand Dryer: Direct Wood-fired: Mixed Softwood/Hardwood |
| Natural Gas | 30701020 | Industrial Processes; Pulp and Paper and Wood Products; Oriented Strandboard (OSB) Manufacture; Rotary Strand Dryer: Direct Natural Gas-fired: Hardwood |
| Natural Gas | 30701021 | Industrial Processes; Pulp and Paper and Wood Products; Oriented Strandboard (OSB) Manufacture; Rotary Strand Dryer: Direct Natural Gas-fired: Softwood |
| Natural Gas | 30701022 | Industrial Processes; Pulp and Paper and Wood Products; Oriented Strandboard (OSB) Manufacture; Rotary Strand Dryer: Direct Natural Gas-fired: Mixed Softwood/Hardwood |
| Wood | 30701401 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Non-Phenol Formaldehyde R |
| Wood | 30701402 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Non-Phenol Formaldehyde R |
| Wood | 30701403 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Non-Phenol Formaldehyde R |
| Natural Gas | 30701404 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Natural Gas-fired: Blowline Blend: Non-Phenol Formaldehyde R |
| Natural Gas | 30701405 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Natural Gas-fired: Blowline Blend: Non-Phenol Formaldehyde R |
| Natural Gas | 30701406 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Natural Gas-fired: Blowline Blend: Non-Phenol Formaldehyde R |
| Wood | 30701410 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Phenol Formaldehyde Resin |
| Wood | 30701411 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Phenol Formaldehyde Resin |
| Wood | 30701412 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Direct Wood-fired: Blowline Blend: Phenol Formaldehyde Resin |
| Natural Gas | 30701413 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Natural Gas-fired: Blowline Blend: Phenol Formaldehyde Resin |
| Natural Gas | 30701414 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Natural Gas-fired: Blowline Blend: Phenol Formaldehyde Resin |
| Natural Gas | 30701415 | Industrial Processes; Pulp and Paper and Wood Products; Hardboard (HB) Manufacture; Pressurized Refiner/Primary Tube Dryer: Natural Gas-fired: Blowline Blend: Phenol Formaldehyde Resin |
| Wood | 30701605 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Hardwood Veneer Dryer: Direct Wood-fired: Heated Zones |
| Wood | 30701606 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Hardwood Veneer Dryer: Direct Wood-fired: Cooling Section |
| Wood | 30701607 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Softwood Veneer Dryer: Direct Wood-fired: Heated Zones |
| Wood | 30701608 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Softwood Veneer Dryer: Direct Wood-fired: Cooling Section |
| Natural Gas | 30701609 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Hardwood Veneer Dryer: Direct Natural Gas-fired: Heated Zones |
| Natural Gas | 30701610 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Hardwood Veneer Dryer: Direct Natural Gas-fired: Cooling Section |
| Natural Gas | 30701611 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Softwood Veneer Dryer: Direct Natural Gas-fired: Heated Zones |
| Natural Gas | 30701613 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Veneer Lumber (LVL) Manufacture; Softwood Veneer Dryer: Direct Natural Gas-fired: Cooling Section |
| Wood | 30701701 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Strand Lumber (LSL) Manufacture; Rotary Strand Dryer: Direct Wood-fired: Hardwood |
| Wood | 30701702 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Strand Lumber (LSL) Manufacture; Rotary Strand Dryer: Direct Wood-fired: Softwood |
| Wood | 30701703 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Strand Lumber (LSL) Manufacture; Rotary Strand Dryer: Direct Wood-fired: Mixed Softwood/Hardwood |
| Natural Gas | 30701704 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Strand Lumber (LSL) Manufacture; Rotary Strand Dryer: Direct Natural Gas-fired: Softwood |
| Natural Gas | 30701705 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Strand Lumber (LSL) Manufacture; Rotary Strand Dryer: Direct Natural Gas-fired: Hardwood |
| Natural Gas | 30701706 | Industrial Processes; Pulp and Paper and Wood Products; Laminated Strand Lumber (LSL) Manufacture; Rotary Strand Dryer: Direct Natural Gas-fired: Mixed Softwood/Hardwood |
| Wood | 30701744 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Hardwood Veneer Dryer: Direct Wood-fired: Heated Zones |
| Wood | 30701745 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Hardwood Veneer Dryer: Direct Wood-fired: Cooling Section |
| Wood | 30701746 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Softwood Veneer Dryer: Direct Wood-fired: Heated Zones |
| Wood | 30701747 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Softwood Veneer Dryer: Direct Wood-fired: Cooling Section |
| Natural Gas | 30701748 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Hardwood Veneer Dryer: Direct Natural Gas-fired: Heated Zones |
| Natural Gas | 30701749 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Hardwood Veneer Dryer: Direct Natural Gas-fired: Cooling Section |
| Natural Gas | 30701750 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Softwood Veneer Dryer: Direct Natural Gas-fired: Heated Zones |
| Natural Gas | 30701751 | Industrial Processes; Pulp and Paper and Wood Products; Parallel Strand Lumber (PSL) Manufacture; Softwood Veneer Dryer: Direct Natural Gas-fired: Cooling Section |
| Distillate Oil | 30890001 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 30890002 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Residual Oil: Process Heaters |
| Natural Gas | 30890003 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Natural Gas: Process Heaters |
| LPG | 30890004 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Liquified Petroleum Gas (LPG): Process Heaters |
| Distillate Oil | 30890011 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Distillate Oil (No. 2): Incinerators |
| Residual Oil | 30890012 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Residual Oil: Incinerators |
| Natural Gas | 30890013 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Natural Gas: Incinerators |
| Distillate Oil | 30890021 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Distillate Oil (No. 2): Flares |
| Residual Oil | 30890022 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Residual Oil: Flares |
| Natural Gas | 30890023 | Industrial Processes; Rubber and Miscellaneous Plastics Products; Fuel Fired Equipment; Natural Gas: Flares |
| Distillate Oil | 30990001 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 30990002 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Residual Oil: Process Heaters |
| Natural Gas | 30990003 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Natural Gas: Process Heaters |
| Distillate Oil | 30990011 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Distillate Oil (No. 2): Incinerators |
| Residual Oil | 30990012 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Residual Oil: Incinerators |
| Natural Gas | 30990013 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Natural Gas: Incinerators |
| Natural Gas | 30990023 | Industrial Processes; Fabricated Metal Products; Fuel Fired Equipment; Natural Gas: Flares |
| Distillate Oil | 31000401 | Industrial Processes; Oil and Gas Production; Process Heaters; Distillate Oil (No. 2) |
| Residual Oil | 31000402 | Industrial Processes; Oil and Gas Production; Process Heaters; Residual Oil |
| Natural Gas | 31000404 | Industrial Processes; Oil and Gas Production; Process Heaters; Natural Gas |
| LPG | 31000406 | Industrial Processes; Oil and Gas Production; Process Heaters; Propane/Butane |
| Distillate Oil | 31000411 | Industrial Processes; Oil and Gas Production; Process Heaters; Distillate Oil (No. 2): Steam Generators |
| Residual Oil | 31000412 | Industrial Processes; Oil and Gas Production; Process Heaters; Residual Oil: Steam Generators |
| Natural Gas | 31000414 | Industrial Processes; Oil and Gas Production; Process Heaters; Natural Gas: Steam Generators |
| Distillate Oil | 31390001 | Industrial Processes; Electrical Equipment; Process Heaters; Distillate Oil (No. 2) |
| Residual Oil | 31390002 | Industrial Processes; Electrical Equipment; Process Heaters; Residual Oil |
| Natural Gas | 31390003 | Industrial Processes; Electrical Equipment; Process Heaters; Natural Gas |
| Coal | 39000199 | Industrial Processes; In-process Fuel Use; Anthracite Coal; General |
| Coal | 39000201 | Industrial Processes; In-process Fuel Use; Bituminous Coal; Cement Kiln/Dryer |
| Coal | 39000203 | Industrial Processes; In-process Fuel Use; Bituminous Coal; Lime Kiln (Bituminous) |
| Coal | 39000288 | Industrial Processes; In-process Fuel Use; Bituminous Coal; General (Subbituminous) |
| Coal | 39000289 | Industrial Processes; In-process Fuel Use; Bituminous Coal; General (Bituminous) |
| Coal | 39000399 | Industrial Processes; In-process Fuel Use; Lignite; General |
| Residual Oil | 39000402 | Industrial Processes; In-process Fuel Use; Residual Oil; Cement Kiln/Dryer |
| Residual Oil | 39000403 | Industrial Processes; In-process Fuel Use; Residual Oil; Lime Kiln |
| Residual Oil | 39000499 | Industrial Processes; In-process Fuel Use; Residual Oil; General |
| Distillate Oil | 39000501 | Industrial Processes; In-process Fuel Use; Distillate Oil; Asphalt Dryer |
| Distillate Oil | 39000502 | Industrial Processes; In-process Fuel Use; Distillate Oil; Cement Kiln/Dryer |
| Distillate Oil | 39000503 | Industrial Processes; In-process Fuel Use; Distillate Oil; Lime Kiln |
| Distillate Oil | 39000598 | Industrial Processes; In-process Fuel Use; Distillate Oil; Grade 4 Oil: General |
| Distillate Oil | 39000599 | Industrial Processes; In-process Fuel Use; Distillate Oil; General |
| Natural Gas | 39000602 | Industrial Processes; In-process Fuel Use; Natural Gas; Cement Kiln/Dryer |
| Natural Gas | 39000603 | Industrial Processes; In-process Fuel Use; Natural Gas; Lime Kiln |
| Natural Gas | 39000605 | Industrial Processes; In-process Fuel Use; Natural Gas; Metal Melting |
| Natural Gas | 39000699 | Industrial Processes; In-process Fuel Use; Natural Gas; General |
| Wood | 39000988 | Industrial Processes; In-process Fuel Use; Wood; Wood Building Products: Flatwood and others |
| Wood | 39000989 | Industrial Processes; In-process Fuel Use; Wood; General |
| Wood | 39000999 | Industrial Processes; In-process Fuel Use; Wood; General: Wood |
| LPG | 39001088 | Industrial Processes; In-process Fuel Use; Liquified Petroleum Gas; Wood Building Products: Flatwood and others |
| LPG | 39001099 | Industrial Processes; In-process Fuel Use; Liquified Petroleum Gas; General |
| Distillate Oil | 39900501 | Industrial Processes; Miscellaneous Manufacturing Industries; Process Heater/Furnace; Distillate Oil |
| Natural Gas | 39900601 | Industrial Processes; Miscellaneous Manufacturing Industries; Process Heater/Furnace; Natural Gas |
| LPG | 39901001 | Industrial Processes; Miscellaneous Manufacturing Industries; Process Heater/Furnace; LPG |
| Distillate Oil | 39990001 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Distillate Oil (No. 2): Process Heaters |
| Residual Oil | 39990002 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Residual Oil: Process Heaters |
| Natural Gas | 39990003 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Natural Gas: Process Heaters |
| Distillate Oil | 39990011 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Distillate Oil (No. 2): Incinerators |
| Residual Oil | 39990012 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Residual Oil: Incinerators |
| Natural Gas | 39990013 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Natural Gas: Incinerators |
| Distillate Oil | 39990021 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Distillate Oil (No. 2): Flares |
| Residual Oil | 39990022 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Residual Oil: Flares |
| Natural Gas | 39990023 | Industrial Processes; Miscellaneous Manufacturing Industries; Miscellaneous Manufacturing Industries; Natural Gas: Flares |
| Natural Gas | 40201001 | Chemical Evaporation; Surface Coating Operations; Coating Oven Heater; Natural Gas |
| Distillate Oil | 40201002 | Chemical Evaporation; Surface Coating Operations; Coating Oven Heater; Distillate Oil |
| Residual Oil | 40201003 | Chemical Evaporation; Surface Coating Operations; Coating Oven Heater; Residual Oil |
| LPG | 40201004 | Chemical Evaporation; Surface Coating Operations; Coating Oven Heater; Liquified Petroleum Gas (LPG) |
| Distillate Oil | 40290011 | Chemical Evaporation; Surface Coating Operations; Fuel Fired Equipment; Distillate Oil: Incinerator/Afterburner |
| Residual Oil | 40290012 | Chemical Evaporation; Surface Coating Operations; Fuel Fired Equipment; Residual Oil: Incinerator/Afterburner |
| Natural Gas | 40290013 | Chemical Evaporation; Surface Coating Operations; Fuel Fired Equipment; Natural Gas: Incinerator/Afterburner |
| Natural Gas | 40290023 | Chemical Evaporation; Surface Coating Operations; Fuel Fired Equipment; Natural Gas: Flares |
| Distillate Oil | 49090011 | Chemical Evaporation; Organic Solvent Evaporation; Fuel Fired Equipment; Incinerator: Distillate Oil (No. 2) |
| Residual Oil | 49090012 | Chemical Evaporation; Organic Solvent Evaporation; Fuel Fired Equipment; Incinerator: Residual Oil |
| Natural Gas | 49090013 | Chemical Evaporation; Organic Solvent Evaporation; Fuel Fired Equipment; Incinerator: Natural Gas |
| Distillate Oil | 49090021 | Chemical Evaporation; Organic Solvent Evaporation; Fuel Fired Equipment; Flare: Distillate Oil (No. 2) |
| Residual Oil | 49090022 | Chemical Evaporation; Organic Solvent Evaporation; Fuel Fired Equipment; Flare: Residual Oil |
| Natural Gas | 49090023 | Chemical Evaporation; Organic Solvent Evaporation; Fuel Fired Equipment; Flare: Natural Gas |
| Distillate Oil | 50100602 | Waste Disposal; Solid Waste Disposal - Government; Fire Fighting; Structure: Distillate Oil |
| Kerosene | 50100603 | Waste Disposal; Solid Waste Disposal - Government; Fire Fighting; Structure: Kerosene |
| Coal | 50190002 | Waste Disposal; Solid Waste Disposal - Government; Auxiliary Fuel/No Emissions; Coal |
| Distillate Oil | 50190005 | Waste Disposal; Solid Waste Disposal - Government; Auxiliary Fuel/No Emissions; Distillate Oil |
| Natural Gas | 50190006 | Waste Disposal; Solid Waste Disposal - Government; Auxiliary Fuel/No Emissions; Natural Gas |
| LPG | 50190010 | Waste Disposal; Solid Waste Disposal - Government; Auxiliary Fuel/No Emissions; Liquified Petroleum Gas (LPG) |
| Coal | 50290002 | Waste Disposal; Solid Waste Disposal - Commercial/Institutional; Auxiliary Fuel/No Emissions; Coal |
| Distillate Oil | 50290005 | Waste Disposal; Solid Waste Disposal - Commercial/Institutional; Auxiliary Fuel/No Emissions; Distillate Oil |
| Natural Gas | 50290006 | Waste Disposal; Solid Waste Disposal - Commercial/Institutional; Auxiliary Fuel/No Emissions; Natural Gas |
| LPG | 50290010 | Waste Disposal; Solid Waste Disposal - Commercial/Institutional; Auxiliary Fuel/No Emissions; Liquified Petroleum Gas (LPG) |
| Coal | 50390002 | Waste Disposal; Solid Waste Disposal - Industrial; Auxiliary Fuel/No Emissions; Coal |
| Distillate Oil | 50390005 | Waste Disposal; Solid Waste Disposal - Industrial; Auxiliary Fuel/No Emissions; Distillate Oil |
| Natural Gas | 50390006 | Waste Disposal; Solid Waste Disposal - Industrial; Auxiliary Fuel/No Emissions; Natural Gas |
| LPG | 50390010 | Waste Disposal; Solid Waste Disposal - Industrial; Auxiliary Fuel/No Emissions; Liquified Petroleum Gas (LPG) |

Energy Information Administration. 2018. State Energy Data System, 2017 data. https://www.eia.gov/state/seds/

Energy Information Administration. 2018. Form 821: Sales of Distillate Fuel Oil by End Use, 2016 data. https://www.eia.gov/dnav/pet/pet_cons_821use_dcu_nus_a.htm

Energy Information Administration. 2018. Manufacturing Energy Consumption Survey, 2014 data. https://www.eia.gov/consumption/manufacturing/

Energy Information Administration. 2008. “Domestic Distribution of U.S. Coal by Destination State, Consumer, Origin and Method of Transportation” http://www.eia.doe.gov/cneaf/coal/page/coaldistrib/coal_distributions.html.

Energy Information Administration. 2015. Commercial Building Energy Consumption Survey, 2012 data. https://www.eia.gov/consumption/commercial/

U.S. Census Bureau. 2018. 2016 County Business Patterns. https://www.census.gov/data/datasets/2016/econ/cbp/2016-cbp.html

U.S. Environmental Protection Agency.  1996. Compilation of Air Pollutant Emission Factors, 5th Edition, AP-42, Volume I:  Stationary Point and Area Sources.  Research Triangle Park, North Carolina. https://www.epa.gov/air-emissions-factors-and-quantification/ap-42-compilation-air-emission-factors#5thed

EPA and Eastern Regional Technical Advisory Committee. 2010. Excel file: state_comparison_ERTAC_SS_version7_5_Mar 16 2010.xls

R. Battye, W. Battye, C. Overcash, and S. Fudge. 1994. Development and Selection of Ammonia Emission Factors: Final Report.  Durham, NC: EC/R Incorporated.  Prepared for USEPA Office of Research and Development.

E.H. Pechan and Associates, Inc. 2003. Estimating Ammonia Emissions from Anthropogenic Sources, Draft Report. Durham, NC. Prepared for USEPA Emission Factor and Inventory Group.

EPA. 2016. SPECIATE v4.5. Fireplace wood combustion – pine wood. https://www.epa.gov/air-emissions-modeling/speciate

Energy Information Administration. 2018. Form 923 Electricity Sector Data, 2017 data. https://www.eia.gov/electricity/data/eia923/