# mining and quarrying

# Source Category Description

Mining and quarrying activities produce particulate matter (PM) emissions due to the variety of processes used to extract the ore and associated overburden, including drilling and blasting, loading and unloading, and overburden replacement. Fugitive dust emissions for mining and quarrying operations are the sum of emissions from the mining of metallic and nonmetallic ores and coal. Each of these mining operations has specific emissions factors accounting for the different means by which the resources are extracted. In 2014, mining and quarrying in the US, Puerto Rico, and US Virgin Islands resulted in approximately 585,991 tons of PM10-PRI emissions.

For this source category the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2325000000 | Industrial Processes | Mining and Quarrying: SIC 14 | All Processes | Total |

# Overview of Calculations

Four specific activities are included in the emissions estimate for mining and quarrying operations: overburden removal, drilling and blasting, loading and unloading, and overburden replacement. Not included are the transfer and conveyance operations, crushing and screening operations, and storage since the dust emissions from these activities are assumed to be well controlled. Fugitive dust emissions for mining and quarrying operations are the sum of emissions from the mining of metallic and nonmetallic ores and coal. Emissions for each activity are calculated by multiplying the emissions factors by the activity data. The sources of activity data are described in section C, the allocation procedure is described in section D, the sources of the emissions factors are described in section E, and the sources of data and emissions calculations are described in section G.

# Activity Data

Activity data for this source category include state-level metallic and non-metallic (a.k.a. mineral) crude ore handled at surface mines from the U.S. Geologic Survey (USGS)  and mine-specific coal production data for surface mines from the Energy Information Administration (EIA). Emissions are not estimated for underground mining given that emissions factors are calculated exclusively for surface activity.

In some cases the amount of mining waste is withheld for some states to avoid disclosing company proprietary data. To estimate state-level withheld waste data the fraction of crude ore production in the state is multiplied by the amount of waste data withheld at the national level. The national-level amount of waste withheld is calculated by subtracting all known state-level waste values (i.e. those that are not withheld) from the national-level waste value. Note that this calculation only needs to be completed for states where state-level mining waste data are withheld.

| Ws=OsOUS×WUS |  |
| --- | --- |

Where:

Ws	=	Amount of metallic and non-metallic mining waste for state s, in metric tons

WUS	=	Amount of metallic and non-metallic mining waste withheld at the national-level, in metric tons

Os	=	Amount of crude ore produced in state s, in metric tons

OUS	=	Amount of crude ore produced at the national-level, in metric tons

The data on state-level mining production and waste is split into production and waste for metallic and non-metallic ores using the fraction of national-level metallic and non-metallic ore production. Values are also converted from metric tons to short tons. Throughout the remainder of this document references to “ton(s)” refer to short tons, while metric tons will be explicitly labeled.

| MPt,s=Ws+Os×MPt,USMPUS×1.1023tonmetric ton |  |
| --- | --- |

Where:

MPt,s	=	Amount of mining material type t (i.e. either metallic or non-metallic ore) produced in state s, in tons

Ws	=	Amount of total metallic and non-metallic mining waste for state s, in metric tons

Os	=	Amount of crude ore produced in state s, in metric tons

MPt,US	=	Amount of mining material type t produced at the national-level, in metric tons

MPUS	=	Total metallic and non-metallic ore production at the national-level, in metric tons

# Allocation Procedure

The state-level data on metallic and non-metallic mining materials (from equation 2) is distributed to the county level based on the proportion of employees in the metallic and non-metallic ore sectors (see Table 1 for a list of NAICS codes), from the U.S. Census Bureau County Business Patterns. Separate fractions are determined for metallic ore mining employees and non-metallic ore mining employees in each county.

| EmpFract,c=Empt,cEmpt,s |  |
| --- | --- |

Where:

EmpFract,c	= The fraction of mining employees for material type t in county c

Empt,c		=	The number of mining employees for material type t in county c

Empt,s		=	The number of mining employees for material type t in state s

Table 1. NAICS Codes for Metallic and Non-Metallic Mining

| NAICS Code | Description |
| --- | --- |
| 2122 | Metal Ore Mining |
| 212210 | Iron Ore Mining |
| 21222 | Gold Ore and Silver Ore Mining |
| 212221 | Gold Ore Mining |
| 212222 | Silver Ore Mining |
| 21223 | Copper, Nickel, Lead, and Zinc Mining |
| 212231 | Lead Ore and Zinc Ore Mining |
| 212234 | Copper Ore and Nickel Ore Mining |
| 21229 | Other Metal Ore Mining |
| 212291 | Uranium-Radium-Vanadium Ore Mining |
| 212299 | All Other Metal Ore Mining |
| 2123 | Nonmetallic Mineral Mining and Quarrying |
| 21231 | Stone Mining and Quarrying |
| 212311 | Dimension Stone Mining and Quarrying |
| 212312 | Crushed and Broken Limestone Mining and Quarrying |
| 212313 | Crushed and Broken Granite Mining and Quarrying |
| 212319 | Other Crushed and Broken Stone Mining and Quarrying |
| 21232 | Sand, Gravel, Clay, and Ceramic and Refractory Minerals Mining and Quarrying |
| 212321 | Construction Sand and Gravel Mining |
| 212322 | Industrial Sand Mining |
| 212324 | Kaolin and Ball Clay Mining |
| 212325 | Clay and Ceramic and Refractory Minerals Mining |
| 21239 | Other Nonmetallic Mineral Mining and Quarrying |
| 212391 | Potash, Soda, and Borate Mineral Mining |
| 212392 | Phosphate Rock Mining |
| 212393 | Other Chemical and Fertilizer Mineral Mining |
| 212399 | All Other Nonmetallic Mineral Mining |

Due to concerns with releasing confidential business information, the CBP does not release exact numbers for a given NAICS code if the data can be traced to an individual business.  Instead, a series of range codes is used.  To estimate employment in counties and states with withheld data, the following procedure is used for NAICS code being computed.

To gap-fill withheld state-level employment data:

- State-level data for states with known employment in each NAICS are summed to the national level.

- The total sum of state-level known employment from step a is subtracted from the national total reported employment for each NAICS in the national-level CBP to determine the employment total for the withheld states.

- Each of the withheld states is assigned the midpoint of the range code reported for that state. Table 2 lists the range codes and midpoints.

- The midpoints for the states with withheld data are summed to the national-level.

- An adjustment factor is created by dividing the number of withheld employees (calculated in step b of this section) by the sum of the midpoints (step d).

- For the states with withheld employment data, the midpoint of the range for that state (step c) is multiplied by the adjustment factor (step e) to calculate the adjusted state-level employment for landfills.

These same steps are then followed to fill in withheld data in the county-level business patterns.

- County-level data for counties with known employment are summed by state.

- County-level known employment is subtracted from the state total reported in state-level CBP (or, if the state-level data are withheld, from the state total estimated using the procedure discussed above).

- Each of the withheld counties is assigned the midpoint of the range code (Table 2).

- The midpoints for the counties with withheld data are summed to the state level.

- An adjustment factor is created by dividing the number of withheld employees (step h) by the sum of the midpoints (step j).

- For counties with withheld employment data, the midpoints (step i) are multiplied by the adjustment factor (step k) to calculate the adjusted county-level employment for landfills.

Table 2. Withheld data ranges and midpoints

| Employment Code | Employment Range | Midpoint |
| --- | --- | --- |
| A | 0-19 | 10 |
| B | 20-99 | 60 |
| C | 100-249 | 175 |
| E | 250-499 | 375 |
| F | 500-999 | 750 |
| G | 1,000-2,499 | 1,750 |
| H | 2,500-4,999 | 3,750 |
| I | 5,000-9,999 | 7,500 |
| J | 10,000-24,999 | 17,500 |
| K | 25,000-49,999 | 37,500 |
| L | 50,000-99,999 | 75,000 |
| M | 100,000+ |  |

For example, take the 2016 CBP data for NAICS 2123 (Nonmetallic Mineral Mining and Quarrying) in Arizona provided in Table 3.

Table 3. 2016 County Business Pattern for NAICS 2123 in Arizona

| State FIPS | County FIPS | County Name | NAICS | Employment Code | Employment |
| --- | --- | --- | --- | --- | --- |
| 04 | 001 | Apache | 2123 | B | withheld |
| 04 | 003 | Cochise | 2123 |  | 16 |
| 04 | 005 | Coconino | 2123 | A | withheld |
| 04 | 007 | Gila | 2123 |  | 10 |
| 04 | 009 | Graham | 2123 | B | withheld |
| 04 | 012 | La Paz | 2123 | A | withheld |
| 04 | 013 | Maricopa | 2123 |  | 563 |
| 04 | 015 | Mohave | 2123 |  | 69 |
| 04 | 017 | Navajo | 2123 |  | 65 |
| 04 | 019 | Pima | 2123 |  | 121 |
| 04 | 021 | Pinal | 2123 |  | 201 |
| 04 | 023 | Santa Cruz | 2123 | A | withheld |
| 04 | 025 | Yavapai | 2123 |  | 133 |
| 04 | 027 | Yuma | 2123 |  | 51 |

Note: Counties in Arizona that do not have employment in mining and quarrying are excluded from this table.

- The total number of employees reported at the county level is 1,229.

- The state-level CBP reports 1,363 employees for NAICS 2123.  This means that there are 134 employees withheld at the county level.

- The counties with withheld data are assigned midpoints according to the employment codes in Table 2. For example, County 001 is given a midpoint of 60 employees (since employment code B is 20-99).

- The sum of the midpoints for all withheld counties is 150 employees.

- The adjustment factor is 134/150 = 0.8933.

- The adjusted employment for county 001 is 60 × 0.8933 = 54 employees.

Once county- and state-level metal and non-metal employment are known for each county, the ratio of county to state employees (from equation 3) is multiplied by the state-level metal and non-metal production (from equation 2) to calculate county-level production.

| MPt,c=MPt,s×EmpFract,c |  |
| --- | --- |

Where:

MPt,c	=	Amount of mining material type t produced in county c, in tons

MPt,s	=	Amount of mining material type t (i.e. either metallic or non-metallic ore) produced in state s, in tons

EmpFract,c	=	The fraction of mining employees for material type t in county c

# Emissions Factors

Emissions factors are calculated separately for metallic ore mining, non-metallic ore mining, and coal mining. This section describes those calculations and the relevant data sources.

## Metallic Ore Mining

The emissions factor for metallic ore mining includes emissions from overburden removal, drilling and blasting, and loading and unloading activities, and are taken from emissions factors for copper ore mining from EPA’s National Air Pollutant Emission Trends Procedures Document for 1900-1996. The emissions factors are applied to all three activities with PM10/TSP ratios of 0.35 for overburden removal, 0.81 for drilling and blasting, and 0.43 for loading and unloading operations.6

| EFPM10,m=EFo+B×EFb+EFl+EFd |  |
| --- | --- |

Where:

EFPM10,m	=	PM10-PRI metallic ore mining emissions factor, in lbs./ton

EFo	=	PM10-PRI open pit overburden removal emissions factor for copper ore, in lbs./ton

B	=	Fraction of total ore production that is obtained by blasting at metallic ore mines

EFb	=	PM10-PRI drilling/blasting emissions factor for copper ore, in lbs./ton

EFl	=	PM10-PRI loading emissions factor for copper ore, in lbs./ton

EFd	=	PM10-PRI truck dumping emissions factor for copper ore, in lbs./ton

Using values from the National Air Pollutant Emission Trends Procedures Document for 1900-1996, Table 3.1-3, the PM10-PRI emissions factor is calculated as:

| 0.0548 lbs/ton=0.0003 + (0.57625 × 0.0008) + 0.022 + 0.032 | (5a) |
| --- | --- |

The PM25-PRI emissions factor is assumed to be 12.5% of the PM10-PRI emissions factor.

| EFPM25,m=EFPM10,m×0.125 |  |
| --- | --- |
| 0.0069=0.0548×0.125 | (6a) |

Where:

EFPM25,m	=	PM25-PRI metallic ore mining emissions factor, in lbs./ton

EFPM10,m	=	PM10-PRI metallic ore mining emissions factor, in lbs./ton

## Non-Metallic Ore Mining

The emissions factor for non-metallic ore mining includes overburden removal, drilling and blasting, and loading and unloading activities. The emissions factor is based on western surface coal mining operations from AP-42 and a PM10/TSP ratio.

| EFPM10,nm=EFv+D×EFr+EFa+(0.5×EFe+EFt) |  |
| --- | --- |

Where:

EFPM10,nm	=	PM10-PRI non-metallic ore mining emissions factor, in lbs./ton

EFv	=	PM10-PRI open pit overburden removal emissions factor at western surface coal mining operations, in lbs./ton

D	=	fraction of total ore production that is obtained by blasting at non-metallic ore mines

EFr	=	PM10-PRI drilling/blasting emissions factor at western surface coal mining operations, in lbs./ton

EFa	=	PM10-PRI loading emissions factor at western surface coal mining operations, in lbs./ton

EFe 	=	PM10-PRI truck unloading: end dump-coal emissions factor at western surface coal mining operations, in lbs./ton

EFt	=	PM10-PRI truck unloading: bottom dump-coal emissions factor at western surface coal mining operations, in lbs./ton

Applying the TSP emissions factors developed for western surface coal mining operations from AP-427 and a PM10/TSP ratio of 0.4 yields the following non-metallic ore mining emissions factor:

| 0.293 lbs./ton =0.225 + (0.61542 × 0.00005) + 0.05 + 0.5 (0.0035 + 0.033) | (7a) |
| --- | --- |

The PM25-PRI emissions factor is assumed to be 12.5% of the PM10-PRI emissions factor.

| EFPM25,nm=EF10,nm×0.125 |  |
| --- | --- |
| 0.037 lbs/ton=0.293×0.125 | (8a) |

Where:

EFPM25,nm	=	PM25-PRI non-metallic ore mining emissions factor, in lbs./ton

EFPM10,nm	=	PM10-PRI non-metallic ore mining emissions factor, in lbs./ton

## Coal Mining

The emissions factor for coal mining includes overburden removal, drilling and blasting, loading and unloading and overburden replacement activities. The amount of overburden material handled is assumed to equal ten times the quantity of coal mined and coal unloading is assumed to split evenly between end-dump and bottom-dump operations. The emissions factor is based on the PM10 emissions factors developed for western surface coal mining operations from AP-42.7

| EFPM10,co=10×EFto+EFor+EFdt+EFv+EFr+EFa+(0.5×EFe+EFt) |  |
| --- | --- |

Where:

EFPM10,co	=	PM10-PRI coal mining emissions factor, in lbs./ton

EFto	=	PM10-PRI emissions factor for truck loading overburden at western surface coal mining operations, in lbs./ton of overburden

EFor	=	PM10-PRI emissions factor for overburden replacement at western surface coal mining operations, in lbs./ton of overburden

EFdt	=	PM10-PRI emissions factors for truck unloading: bottom dump-overburden at western surface coal mining operations, in lbs./ton of overburden

EFv	=	PM10-PRI open pit overburden removal emissions factor at western surface coal mining operations, in lbs./ton

EFr	=	PM10-PRI drilling/blasting emissions factor at western surface coal mining operations, in lbs./ton

EFa	=	PM10-PRI loading emissions factor at western surface coal mining operations, in lbs./ton

EFe	=	PM10-PRI truck unloading: end dump-coal emissions factor at western surface coal mining operations, in lbs./ton

EFt	=	PM10-PRI truck unloading: bottom dump-coal emissions factor at western surface coal mining operations, in lbs./ton

Applying the PM10-PRI emissions factors developed for western surface coal mining operations7 yields the following coal mining emissions factor:

| 0.513 lbs/ton=(10 × (0.015 + 0.001 + 0.006)) + 0.225 + 0.00005 + 0.05 + (0.5 × (0.0035 + 0.033)) | (9a) |
| --- | --- |

The PM25-PRI emissions factor is assumed to be 12.5% of the PM10-PRI emissions factor.

| EFPM25,co=EF10,co×0.125 |  |
| --- | --- |

Where:

EFPM25,co	=	PM25-PRI coal mining emissions factor, in lbs./ton

EFPM10,co	=	PM10-PRI coal mining emissions factor, in lbs./ton

## PM-FIL and PM2.5-PRI Emissions Factors

PM-FIL emissions factors are assumed to be the same as PM-PRI emissions factors. In reality, there is a small amount of PM-CON emissions included in the PM-PRI emissions but insufficient data exists to estimate the PM-CON portion. In 2006, the EPA adopted new PM2.5/PM10 ratios for several fugitive dust categories and concluded that the PM2.5/PM10 ratios for fugitive dust categories should be in the range of 0.1 to 0.15. Consequently, a ratio of 0.125 was applied to the PM10 emissions factors to estimate PM2.5 emissions factors for mining and quarrying. A summary of emissions factors is presented in Table 4.

Table 4. Emissions factors for Mining and Quarrying (2325000000)

| Mining Type | Pollutant | Emissions Factor | Emissions Factor Units | Emissions Factor Reference |
| --- | --- | --- | --- | --- |
| Metallic | PM10-PRI | 0.0548 | lbs./ton | 4 |
| Metallic | PM10-FIL | 0.0548 | lbs./ton | 4 |
| Metallic | PM25-PRI | 0.0069 | lbs./ton | PM10 × 0.125 |
| Metallic | PM25-FIL | 0.0069 | lbs./ton | PM10 × 0.125 |
| Non-Metallic | PM10-PRI | 0.293 | lbs./ton | 7, 8 |
| Non-Metallic | PM10-FIL | 0.293 | lbs./ton | 7, 8 |
| Non-Metallic | PM25-PRI | 0.037 | lbs./ton | PM10 × 0.125 |
| Non-Metallic | PM25-FIL | 0.037 | lbs./ton | PM10 × 0.125 |
| Coal | PM10-PRI | 0.513 | lbs./ton | 7 |
| Coal | PM10-FIL | 0.513 | lbs./ton | 7 |
| Coal | PM25-PRI | 0.064 | lbs./ton | PM10 × 0.125 |
| Coal | PM25-FIL | 0.064 | lbs./ton | PM10 × 0.125 |

# Controls

There are no controls assumed for this category.

# Emissions

Emissions from mining and quarrying are calculated by multiplying the amount of mining material produced (from equation 4 for metallic and non-metallic mining, and from the EIA2 for coal) by an emissions factor (from Table 4).

| Ep,t,c=EFt,p ×MPt,c |  |
| --- | --- |

Where:

Et,p,c	=	Annual emissions of pollutant p from mining material type t in county c, in lbs.

EFt,p	=	Emissions factor for pollutant p from mining material type t, in lbs./ton of material produced

MPt,c	=	Amount of mining material type t produced in county c, in tons

The final step of the process is to sum the mining emissions estimates for each pollutant in each county. Emissions estimates are then converted from pounds to tons.

| AEp,c=tEp,t,c×0.0005tonlb. |  |
| --- | --- |

Where:

AEp,c	=	Annual emissions of pollutant p in county c, in tons

Et,p,c	=	Annual emissions of pollutant p from mining material type t in county c, in lbs.

# Point Source Subtraction

There are no point source-specific SCCs for mining and quarrying; therefore point source subtraction is not performed for this category.

# Sample Calculations

The steps below provide sample calculations to determine the PM25-PRI emissions from mining and quarrying operations in Barbour County, Alabama. Constant emissions factor calculations that are used in all counties are not repeated here.

Table 5 provides a summary of these calculations. Note that equations 5-10 produce constant emissions factors that are used in all counties. Those calculations are not repeated here.

Table 5. Sample calculations for estimating PM25-PRI emissions from mining and quarrying in Barbour County, Alabama

| Eq. # | Equation | Values for Barbour County, AL | Result |
| --- | --- | --- | --- |
| 1 | Ws=OsOUS×WUS | N/A | Waste data is not withheld for Alabama. |
| 2 | MPt,s=Ws+Os×MPt,USMPUS×1.1023short tonmetric ton | 3,720+42,900×2,660,000÷5,060,000×1.1023tonmetric ton | 27,015 thousand tons metallic ore in Alabama |
|  |  | 3,720+42,900×2,400,000÷5,060,000×1.1023tonmetric ton | 24,375 thousand tons non-metallic ore in Alabama |
| 3 | EmpFract,c=Empt,cEmpt,s | 67 metallic mining employess in Barbour67 metallic mining employees in Alabama | Metallic employee fraction of 1 for Barbour County, AL |
|  |  | 8 nonmetallic mining employess in Barbor1,778 nonmetallic mining employess in AL | Nonmetallic employee fraction of 4.5 × 10-3 for Barbour County, AL |
| 4 | MPt,c=MPt,s×EmpFract,c | 27,015 tons×1 | 27,015 thousand tons metallic ore in Barbour County, AL |
|  |  | 24,375 tons×4.5×10-3 | 112 thousand tons non-metallic ore in Barbour County, AL |
| 11 | Ep,t,c=EFt,p ×MPt,c | 0.0068 lbs.ton×27,015,167 tons | 184,922.19 lbs. PM25-PRI emissions from metallic ore in Barbour County, AL |
|  |  | 0.037lbs.ton×112,039 tons | 4,107.38 lbs. PM25-PRI emissions from non-metallic ore in Barbour County, AL |
|  |  | 0.064lbston×0 tons | 0 lbs. PM25-PRI from coal mining in Barbour County, AL |
| 12 | AEp,c=tEp,c×0.0005short tonlb. | 184,922.19 lbs.+4,107.38 lbs.+0 lbs.×0.0005tonlb. | 95 tons PM25-PRI from mining and quarrying in Barbour County, AL |

# Changes from 2014 Methodology

There are no significant changes for this methodology from the methodology used for the 2014 NEI.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Since insufficient data exists to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emissions factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emissions factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

U.S. Geologic Survey. Minerals Yearbook 2012, http://minerals.usgs.gov/minerals/pubs/commodity/m&q/index.html#myb.

U.S. Department of Energy, Energy Information Administration. “Detailed data from the EIA-7A and the U.S. Mine Safety and Health Administration”, data pulled for year 2017, https://www.eia.gov/coal/data.php.

U.S. Census Bureau. 2016 County Business Patterns, available at https://www.census.gov/content/census/en/data/datasets/2016/econ/cbp/2016-cbp.html

U.S. Environmental Protection Agency. 1998. National Air Pollutant Emission Trends Procedure Document for 1900-1996, EPA-454/R-98-008.

U.S. Environmental Protection Agency, AP-42, Fifth Edition, Volume 1, Chapter 13: Miscellaneous Sources, Section 13.2.4: Aggregate Handling and Storage Piles https://www3.epa.gov/ttn/chief/ap42/ch13/final/c13s0204.pdf.

U.S. Environmental Protection Agency. 1986. Generalized Particle Size Distributions for Use in Preparing Size-Specific Particulate Emissions Inventories, EPA-450/4-86-013.

U.S. Environmental Protection Agency, AP-42, Fifth Edition, Volume 1, Chapter 11: Mineral Products Industry, Section 11.9: Western Surface Coal Mining, http://www.epa.gov/ttn/chief/ap42/ch11/final/c11s09.pdf.

United States Environmental Protection Agency, AIRS Facility Subsystem Source Classification Codes and Emission Factor Listing for Criteria Air Pollutants, EPA-450/4-90-003, March 1990.

Midwest Research Institute. 2006. Background Document for Revisions to Fine Fraction Ratios Used for AP-42 Fugitive Dust Emission Factors, MRI Project No. 110397 http://www.epa.gov/ttnchie1/ap42/ch13/bgdocs/b13s02.pdf.