# OPEN BURNING – LAND CLEARING DEBRIS

# Source Category Description

Open burning of land clearing debris is the purposeful burning of debris, such as trees, shrubs, and brush, from the clearing of land for the construction of new buildings and highways. Criteria air pollutant (CAP) and hazardous air pollutant (HAP) emission estimates from open burning of land clearing debris are a function of the amount of material or fuel subject to burning per year. In 2014, open burning from land clearing debris resulted in approximately 87,277 tons of PM25-PRI emissions.

For this source category, the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2610000500 | Waste Disposal, Treatment, and Recovery | Open Burning | All Categories | Land Clearing Debris (use 28-10-005-000 for Logging Debris Burning) |

# Overview of Calculations

The emissions from open burning from land clearing debris are estimated based on the number of acres disturbed from non-residential, residential, and road construction. The number of acres disturbed is multiplied by a fuel loading factor to determine the amount of land clearing debris burned in each county. This number is multiplied by emissions factors to determine emissions of CAPs and HAPs. Sources of activity data and calculations are discussed in section C. The process of allocating non-residential construction activity data to the county level, allocating building permit data to the county level, and allocating road construction activity data to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from open burning from land clearing debris is discussed in section G.

# Activity Data

The amount of material burned is estimated using the county-level total number of acres disturbed by residential, non-residential, and road construction.  County-level weighted loading factors are applied to the total number of construction acres to convert acres to tons of available fuel.

## Acres Disturbed from Non-Residential Construction

The activity data for this non-residential construction is the acreage disturbed from non-residential construction, which is estimated using data from the U.S. Census Bureau’s Annual Value of Construction Put in Place in the U.S. and a conversion factor from MRI’s Estimating Particulate Matter Emissions from Construction Operations, Final Report.  The national-level non-residential construction spending data are allocated to the county-level based on the proportion of non-residential construction employees in each county. Employment data are taken from the U.S. Census Bureau’s 2016 County Business Patterns (CBP), and gaps in employment data are filled using a process described in detail in section D.

| EmpFrc=EmpcEmpUS |  |
| --- | --- |
| CSc=EmpFrc×CSUS |  |

Where:

EmpFrc	= 	The fraction of non-residential construction employees in county c

Empc	=	The number of non-residential construction employees in county c

EmpUS	=	The number of non-residential construction employees in the US

CSc	=	Non-residential construction spending in county c

CSUS	=	Non-residential construction spending in the US

Non-residential construction spending is converted to acres disturbed using a conversion factor from MRI’s report. For the average acres disturbed per million dollars of non-residential construction, MRI reported a conversion factor of 2 acres/$1 million (in 1992 constant dollars). The 1992 conversion factor is adjusted to 2017 using the Price Deflator (Fisher) Index of New Single‐Family Houses under Construction. In 2017 the conversion factor was 1.009 acres per million dollars spent on non-residential construction activities.

| Apd2017= 2 acres $1 million×PD1992PD2017 |  |
| --- | --- |

Where:

Apd2017	=	Acres disturbed per million dollars in 2017

PD1992	=	Price Deflator (Fisher) Index value in 1992

PD2017	=	Price Deflator (Fisher) Index value in 2017

County-level non-residential construction spending (from equation 2) is then multiplied by this conversion factor to estimate county-level acreage disturbed from non-residential construction activities.

| ANRc=CSc×Apd2017 |  |
| --- | --- |

Where:

ANRc	=	Acres disturbed from non-residential construction in county c

CSc	=	Non-residential construction spending in county c, in million dollars

Apd2017	=	Acres disturbed per million dollars in 2017

## Acres Disturbed from Residential Construction

The US Census Bureau has 2017 data for Housing Starts - New Privately Owned Housing Units Started,, which provides regional level housing starts based on the groupings of 1 unit, 2-4 units, 5 or more units. Regional-level results are also provided for quarterly totals and 1 unit structures (Table 1). The 2-to-4 unit category is broken down using a ratio calculated from the 2000 US Census Bureau National Housing Starts data for 2 and 3-4 units, for each quarter in 2017. Note that 2000 is the last full year when Census housing starts data were available separately for 2-unit and 3-4 unit homes. Table 2 shows a breakdown of the 2 units and 3-4 unit structures based on the following calculation.

| SQ,n=UnUt×SQ,2-4 |  |
| --- | --- |

Where:

SQ,n	= 	Housing starts, by quarter, Q, and number of units, n (2 units or 3-4 units), in thousand units

Un	= 	Number of housing starts by number of units, n, from the 2000 National Housing Starts data, in thousand housing starts

Ut	= 	Total number of housing starts for both 2 units and 3-4 units from the 2000 National Housing Starts data, in thousand housing starts

SQ,2-4	=	Number of 2-4 units by quarter, Q, from the 2017 New Privately Owned Housing Units Started by Purpose and Design data, in thousand units

Table 1. Housing Start Data for 2017

| Quarter | Total | Structure |  |  | Region |  |  |  | Regional Starts of Structures with 1 unit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 unit | 2 to 4 units | 5 units or more | NE | MW | S | W | NE | MW | S | W |
| Q1-14 | 267.0 | 181.0 | 2.0 | 84.0 | 24.0 | 29.0 | 150.0 | 64.0 | 12.0 | 21.0 | 107.0 | 41.0 |
| Q2-14 | 327.0 | 238.0 | 3.0 | 86.0 | 31.0 | 56.0 | 152.0 | 89.0 | 16.0 | 41.0 | 122.0 | 58.0 |
| Q3-14 | 319.0 | 230.0 | 3.0 | 86.0 | 31.0 | 50.0 | 155.0 | 84.0 | 20.0 | 35.0 | 119.0 | 56.0 |
| Q4-14 | 290.0 | 200.0 | 3.0 | 87.0 | 28.0 | 45.0 | 141.0 | 76.0 | 15.0 | 33.0 | 104.0 | 48.0 |

Source: Reference 5

Table 2. Breakdown of 2 to 4 unit structures in 2017

| Quarter | Structure |  |  |
| --- | --- | --- | --- |
|  | 2 to 4 units | 2 units | 3-4 units |
| Q1-14 | 2.0 | 0.74 | 1.26 |
| Q2-14 | 3.0 | 1.11 | 1.89 |
| Q3-14 | 3.0 | 1.11 | 1.89 |
| Q4-14 | 3.0 | 1.11 | 1.89 |

Ratios of the number of 2, 3 and 4, and 5 unit structures are then used to estimate the number of structures of each type in each region. The ratios are calculated by dividing the housing starts by quarter for each unit type by the total housing starts for buildings with more than 2 units.

| rQ,n=SQ,nSQ.t |  |
| --- | --- |

Where:

rQ,n	= 	Ratio of structures with number of units, n, to total number of units by quarter, Q

SQ,n	= 	Housing starts, by quarter, Q, and number of units, n, from distributed calculation in Step 1 for the 2-unit or 3-4 unit categories or directly from the 2017 New Privately Owned Housing Units Started by Purpose and Design data for the 5 units or more category, in thousand housing starts

SQ,t	= 	Housing starts, by quarter, Q, for total number of units greater than 2 units, t (excludes 1-unit

category), in thousand housing starts

The ratio is then used to distribute the 2017 New Privately Owned Housing Units Started by Purpose and Design5 regional data for all unit types to the 2, 3-4, or 5 or more unit categories within each Census region – Northeast, Midwest, South, and West.

| AQ,n,rgn=rQ,n×(RSt,rgn-RS1,rgn) |  |
| --- | --- |

Where:

AQ,n,rgn	=	Number of housing units started in quarter Q, by number of units, n, and region of the country, rgn, in thousand units

rQ,n	= 	Ratio of structures with number of units, n, to total number of units by quarter, Q

RSt,rgn	=	Total regional starts from the 2017 New Privately Owned Housing Units Started by Purpose and

Design data, in thousand housing starts

RS1,rgn	=	Regional starts of structures with 1 unit from the 2017 New Privately Owned Housing Units Started by Purpose and Design data, in thousand housing starts

Data from the Census report New Privately Owned Housing Units Authorized Unadjusted Units is used to calculate a conversion factor to determine the ratio of structures to units in the 5 or more unit category. The conversion factor is calculated by dividing the total number of units in structures with 5 or more units by region6 by the total number of buildings with 5 or more units by region.7

| CF5= U5,rgnB5,rgn |  |
| --- | --- |

Where:

CF5,rgn	=	Ratio of 5 units or more to the number of buildings with 5 units or more by region, rgn

U5,rgn	=	Total number of 5 or more units by region, rgn

B5,rgn	=	Total number of buildings with 5 or more units by region, rgn

Structures started by category are then calculated at a regional level by summing the number of housing unit starts across all four quarters and dividing the by number of units in each building type. For the 3-4 unit type, the number of units per building is 3.5. The value is multiplied by 1,000 because the Census data are in units of thousand building starts.

For buildings with 1, 2, or 3-4 units:

| Bn,rgn=(Q1Q4AQ.n.rgn)×1,000n |  |
| --- | --- |

Where:

Bn,rgn	=	Number of building starts by the unit number category, n, and by region, rgn

AQ,n,rgn	=	Number of housing units started in quarter Q, by number of units, n, and region of the country, rgn, in thousand units

n	=	Number of units per building

For buildings with 5 or more units:

| Bn,rgn=(Q1Q4AQ.n.rgn)×1,000CF5 |  |
| --- | --- |

Where:

Bn,rgn	=	Number of building starts by the unit number category, n, and by region, rgn

AQ,n,rgn	=	Number of housing units started in quarter Q, by number of units, n, and region of the country, rgn, in thousand units

CF5	=	Ratio of 5 units or more to the number of buildings with 5 units or more

Annual county building permit data were purchased from the US Census Bureau for 2017. The 2017 County Level Residential Building Permit dataset has 2017 data to allocate regional housing starts to the county level. This results in county level housing starts by number of units.

The number of building permits for each unit number category by region is calculated by summing the county-level Census data to the region level.

| BPn,rgn=BPn,c |  |
| --- | --- |

Where:

BPn,rgn	=	Number of building permits by the unit number category, n, and by region, rgn

BPn,c	=	Number of building permits by the unit number category, n, and by county, c

The ratio of the number of building permits by county to the total number of building permits by region in which the county is located, for each unit number category, is then calculated.

| RBP,c=BPn,cBPn,rgn |  |
| --- | --- |

Where:

RBP,c	=	Ratio building permits, BP, to total regional building permits in county c

BPn,c	=	Number of building permits by the unit number category, n, and by county, c

BPn,rgn	=	Number of building permits by the unit number category, n, and by region, rgn

The final number of building starts for each unit type category is then calculated at the county-level by multiplying the number of structures started at the regional level and the building permit ratio.

| Bn,c=Bn,rgn×RBP,c |  |
| --- | --- |

Where:

Bn,c	=	Number of building starts by the unit number category, n, and by county, c

Bn,rgn	=	Number of building starts by the unit number category, n, and by region, rgn

RBP,c	=	Ratio building permits, BP, to total regional building permits in county, c

The number of acres of surface area disturbed by the construction of residential buildings is calculated for apartment buildings, buildings with 2 units, and buildings with 1 unit. Table 3 shows the assumptions used for the surface area disturbed for each unit type. Buildings with unit types of 3-4 and 5 or more are grouped together as apartments in this step.

Table 3. Surface Soil removed per unit type

| Structure | Acres disturbed |
| --- | --- |
| 1-Unit | 1/4 acre/structure |
| 2-Unit | 1/3 acre/structure |
| Apartment | 1/2 acre/structure |

The acres of soil disturbed by the construction of residential buildings are calculated for apartment buildings, buildings with 2 units, and buildings with 1 unit.

| ARn,c=Bn.c×an |  |
| --- | --- |

Where:

ARn,c	=	Surface soil disturbed by building construction by county, c, and unit type category, n, in acres

Bn,c	=	Number of building starts by the unit number category, n, and by county, c

an	=	Acres of surface soil disturbed by each unity type category, n. See Table 3.

## Acres Disturbed by Road Construction

The activity data for this source category is the acreage disturbed from new road construction, which is estimated using data from FHWA’s Highway Statistics, State Highway Agency Capital Outlay 2014, Table SF-12A  and FLDOT’s Generic Cost per Mile Models. From the FHWA table, the following columns are used: New Construction, Relocation, Added Capacity, Major Widening, and Minor Widening. These columns are also differentiated according to the following six classifications:

1.	Interstate, urban

2.	Interstate, rural

3.	Other arterial, urban

4.	Other arterial, rural

5.	Collectors, urban

6.	Collectors, rural

Construction spending for each road type is summed across all construction types to determine the total annual highway spending for each road type.

| HSs,r=ctSs,r |  |
| --- | --- |

Where:

HSs,r	=	Annual highway spending for road type r in state  s, in dollars

ct	=	Construction type

Ss,r	=	Annual spending per construction type for road type r in state s, in dollars

State expenditure data are converted to miles of new road and acres disturbed per mile of new road based on conversions obtained from FLDOT. The conversions are shown in

Table 4, and the acres disturbed per mile conversions are calculated by multiplying the total affected roadway width (including all lanes, shoulders, and areas affected beyond the road width) by one mile and converting the resulting land area to acres.

| RCm,s,r=HSs,rTDM |  |
| --- | --- |
| RCa,s,r=RCm,s,r×ADM |  |

Where:

RCm,s,r	=	Miles of FHWA road type r constructed in state s

RCa,s,r	=	Acres of land disturbed for construction of FHWA road type r in state s

HSs,r	=	Annual highway spending for road type r in state s

TDM	=	Conversion of dollars spent to road miles constructed, in thousand dollars per mile

ADM	=	Conversion of road miles constructed to acres disturbed, in acres per mile

Table 4. Spending per Mile and Acres Disturbed per Mile by Highway Type

| Road Type | Thousand Dollars per mile | Total Affected Roadway Width (ft)* | Acres Disturbed per mile |
| --- | --- | --- | --- |
| Urban Areas, Interstate | 6,895 | 94 | 11.4 |
| Rural Areas, Interstate | 3,810 | 89 | 10.8 |
| Urban Areas, Other Arterials | 4,112 | 63 | 7.6 |
| Rural Areas, Other Arterials | 2,076 | 55 | 6.6 |
| Urban Areas, Collectors | 4,112 | 63 | 7.6 |
| Rural Areas, Collectors | 2,076 | 55 | 6.6 |
| *Total Affected Roadway Width = (lane width (12 ft) * number of lanes) + (shoulder width * number of shoulders) + area affected beyond road width (25 ft) |  |  |  |

Source: Reference 10.

The acres of land disturbed by road type can then be summed across all road types in a state to calculate the total state-level acreage disturbed due to new road construction.

| ARCs=rRCa,s |  |
| --- | --- |

Where:

ARCs	=	Acres of land disturbed for all road construction in state s

RCa,s	=	Acres of land disturbed for construction of FHWA road type r in state s

Similar to residential construction, county-level building permits data from the U.S. Census Bureau are used to allocate the state-level acres disturbed by road construction to the county.  Specifically, the ratio of the county-to state-level number of building starts is calculated and multiplied by the state-level acreage disturbed (from equation 18) to estimate the county-level acreage disturbed by road construction.

| BFracc=BuildcBuilds |  |
| --- | --- |
| ARCc=ARCs×BFracc |  |

Where:

BFracc	=	The fraction of building starts in county c

Buildc	=	The number of building starts in county c

Builds	=	The number of building starts in state s

ARCc	=	Acres of land disturbed for road construction in county c

ARCs	=	Acres of land disturbed for all road construction in state s

## Converting Acres Disturbed to Tons of Land Clearing Debris Burned

The total acres disturbed by all construction types is calculated by summing the acres disturbed from residential, non-residential, and road construction.

| TADc=ANRc+(ARn.c)+ARCc |  |
| --- | --- |

Where:

TADc	=	Total acres disturbed in from nonresidential, residential, and road construction in county c

ANRc	=	Acres disturbed from non-residential construction in county c

ARn,c	=	Acres of surface soil disturbed from residential construction in county c and unit type category n (summed to one value for residential construction for the county)

ARCc	=	Acres of land disturbed for road construction in county c

Version 2 of the Biogenic Emissions Land cover Database (BELD2) within EPA’s Biogenic Emission Inventory System (BEIS) is used to identify the acres of hardwoods, softwoods, and grasses in each county.

Because BELD2 does not contain data on Alaska and Hawaii, the acres of hardwoods, softwoods, and grasses in each county is estimated by using the state-level land cover statistics from the USGS National Land Cover Database on the percent land cover under each vegetation type. These percentages are multiplied by the county area (acres), from the U.S. Census Bureau.

| AAK/HI,c,f=LAAK/HI,c×LCAK/HI,%,f |  |
| --- | --- |

Where:

AAK/HI,c,f		=	  Total acres of each fuel type, f, for each county, c, in Alaska or Hawaii

LAAK/HI,c,	=	County acres from the U.S. Census Bureau of each fuel type, f, for each county, c, in Alaska or Hawaii

LCAK/HI,%,f	=  Land cover percentages for each fuel type (hardwood, softwood, grass) in Alaska or Hawaii

Table 5 presents the average fuel loading factors by vegetation type.  The average loading factors for slash hardwood and slash softwood are adjusted by a factor of 1.5 to account for the mass of tree that is below the soil surface that would be subject to burning once the land is cleared. Weighted average county-level loading factors are calculated by multiplying the average loading factors by the percent contribution of each type of vegetation class to the total land area for each county.

| WFLFc,f=Ac,fAc,total×LFf |  |
| --- | --- |

Where:

WFLFc,f	=	Weighted average fuel loading factor by for fuel type f in county c

Ac,f	=	Acres of land cover in county c, by fuel type f (from BELD2 for continental U.S.; from equation 22 for Alaska and Hawaii)

Ac,total	= 	Total acres of land cover of all fuel types in county c

LFf	= 	Fuel loading factor by fuel type, f, in tons/acre, from Table 5

Table 5. Fuel Loading Factors by Vegetation Type

| Vegetation Type | Unadjusted Average Fuel Loading Factor (Ton/acre) | Adjusted Average Fuel Loading Factor (Ton/acre) |
| --- | --- | --- |
| Hardwood | 66 | 99 |
| Softwood | 38 | 57 |
| Grass | 4.5 | Not Applicable |

Source: Reference 14, averages of data on page III-19

The weighted average county-level loading factors for each fuel type are then summed across fuel types to calculate a single weighted average loading factor for each county.

| WFLFc=fWFLFc,f |  |
| --- | --- |

Where:

WFLFc	=	Weighted average fuel loading factors for county c

WFLFc,f	=	Weighted average fuel loading factor by for fuel type f in county c

The county-level total acres disturbed are then multiplied by the weighted average loading factor to derive tons of land clearing debris.

| LCDc=TADc×WFLFc |  |
| --- | --- |

Where:

LCDc	=	Land clearing debris in county c, in tons

TADc	=	Total acres disturbed in county c

WFLFc	=	Weighted average fuel loading factors for county c

The total land clearing debris burned per county is calculated by multiplying acres of land clearing debris by county by a control factor, based on the percent of urban land from the 2010 U.S. Census data.13  See Section F for more information on the control factor.

| BLCDc=LCDc×CFc |  |
| --- | --- |

Where:

BLCDc	=	Land clearing debris burned in county c, in tons

LCDc	=	Land clearing debris in county c, in tons

CFc	=	Control factor. The control factor is 1 for counties with less than 80% urban population and 0 for Colorado or in counties with an urban population of 0.8% or more based on the 2010 U.S. Census data13 as no burning occurs in these counties. See Section F for more information on the control factor.

# Allocation Procedure

## Acres disturbed by Non-residential Construction – County business patterns allocation

Employment data are obtained from the U.S. Census Bureau’s 2016 County Business Patterns (CBP). Due to concerns with releasing confidential business information, the CBP does not release exact numbers for a given NAICS code if the data can be traced to an individual business. Instead, a series of employment flags is used. To estimate employment in counties and states with withheld data, the following procedure is used for NAICS code 2362 (non-residential construction).

To gap-fill withheld state-level employment data:

- State-level data for states with known employment are summed to the national level.

- State-level known employment is subtracted from the national total reported in the national-level CBP.

- Each of the withheld states is assigned the midpoint of the employment flag. Table 6 lists the employment flags and midpoints.

- The midpoints for the states with withheld data are summed to the national-level.

- An adjustment factor is created by dividing the number of withheld employees (calculated in step 2 of this section) by the sum of the midpoints (step 4)

- For the states with withheld employment data, the midpoint of the range for that state (step 3) is multiplied by the adjustment factor (step 5) to calculate the adjusted state-level employment for non-residential construction.

These same steps are then followed to fill in withheld data in the county-level business patterns.

- County-level data for counties with known employment are summed by state.

- County-level known employment is subtracted from the state total reported in state-level CBP (or, if the state-level data are withheld, from the state total estimated using the procedure discussed above).

- Each of the withheld counties is assigned the midpoint of the employment flag (Table 6).

- The midpoints for the counties with withheld data are summed to the state level.

- An adjustment factor is created by dividing the number of withheld employees (step 2) by the sum of the midpoints (step 4).

- For counties with withheld employment data, the midpoints (step 3) are multiplied by the adjustment factor (step 5) to calculate the adjusted county-level employment for non-residential construction.

Note that step 5 adjusts all counties within each state with withheld employment data by the same state-based proportion.  It is unlikely that actual employment corresponds exactly with this smoothed adjustment method, but this method is the best option given the availability of the data.

Table 6. Ranges and Midpoints for Data Withheld from State And County Business Patterns

| Employment Flag | Employment Range | Midpoint |
| --- | --- | --- |
| A | 0-19 | 10 |
| B | 20-99 | 60 |
| C | 100-249 | 175 |
| E | 250-499 | 375 |
| F | 500-999 | 750 |
| G | 1,000-2,499 | 1,750 |
| H | 2,500-4,999 | 3,750 |
| I | 5,000-9,999 | 7,500 |
| J | 10,000-24,999 | 17,500 |
| K | 25,000-49,999 | 37,500 |
| L | 50,000-99,999 | 75,000 |
| M | 100,000+ |  |

For example, take the 2016 CBP data for NAICS 2362 (nonresidential construction) in Arizona provided in Table 7.

Table 7. 2016 CBP for NAICS 2361 in Arizona

| State FIPS | County FIPS | NAICS | Employment Flag | Employment |
| --- | --- | --- | --- | --- |
| 04 | 001 | 2362 | B | withheld |
| 04 | 003 | 2362 |  | 125 |
| 04 | 005 | 2362 |  | 166 |
| 04 | 007 | 2362 |  | 24 |
| 04 | 011 | 2362 | B | withheld |
| 04 | 012 | 2362 | A | withheld |
| 04 | 013 | 2362 |  | 8,580 |
| 04 | 015 | 2362 |  | 64 |
| 04 | 017 | 2362 |  | 53 |
| 04 | 019 | 2362 |  | 2,085 |
| 04 | 021 | 2362 |  | 115 |
| 04 | 023 | 2362 |  | 16 |
| 04 | 025 | 2362 |  | 260 |
| 04 | 027 | 2362 |  | 233 |

- The total of employees not including withheld counties is 11,831.

- The state-level CBP reports 11,721 employees for NAICS 2362.  The difference is 110.

- Withheld counties are given the midpoint of the employment range. County 001 is given a midpoint of 60 (since employment flag A is 20 – 99) and County 012 is given a midpoint of 10 (since employment flag H is 0 – 19).

- State total for these all withheld counties is 130.

- 110/130 = 0.846.

- The adjusted employment for county 001 is 60 × 0.846  = 51.36 employees. County 012 has an adjusted employment of 10 × 0.846 = 8.46 employees.

The county-level employment data are used to allocate the national-level non-residential construction spending data to the county-level (see equation 1).

## Acres disturbed by Residential Construction – Building permits allocation

Annual county building permit data were purchased from the U.S. Census Bureau for 20178 and used to allocate regional housing starts to the county level. This results in county level housing starts by number of units. See equations 11-13 in section C.

## Acres Disturbed by Road Construction – Building permits allocation

State-level estimates of acres disturbed by road construction is distributed to the counties based on county-level data on residential building starts from the U.S. Census Bureau.11 See equations 19 and 20 in section C.

# Emissions Factors

Emissions factors are reported in Table 8 below. Emissions factors for CAPs and HAPs are from the AP-42 and U.S. EPA Emissions Inventory Improvement Program. The PM25 to PM10 emissions factor ratio for brush burning (0.7709) is multiplied by the PM10 emissions factors for land clearing debris burning to develop PM25 emissions factors.  Emissions factors for HAPs are from an EPA Control Technology Center report.

Table 8. Emissions Factors for Open Burning of Land Clearing Debris (SCC 2610000500)

| Pollutant | Pollutant Code | Emissions Factor (lb/ton) | Emission Factor Reference |
| --- | --- | --- | --- |
| VOC | VOC | 11.3 | Reference 16, Table 2.5-5, a |
| NOX | NOX | 4.0 | Reference 16, Table 2.5-5, b |
| CO | CO | 164.8 | Reference 17, Table 16.4-2, c |
| PM10-FIL | PM10-FIL | 20.4 | Reference 17, Table 16.4-2, c |
| PM25-FIL | PM25-FIL | 18.6 | Reference 17, Table 16.4-2, c |
| PM10-PRI | PM10-PRI | 20.4 | Reference 17, Table 16.4-2, c |
| PM25-PRI | PM25-PRI | 18.6 | Reference 17, Table 16.4-2, c |
| Cumene | 98828 | 0.012 | Reference 16, Table 16.4-3, d |
| Ethyl Benzene | 100414 | 0.048 | Reference 16, Table 16.4-3, d |
| Phenol | 108952 | 0.115 | Reference 16, Table 16.4-3, d |
| Styrene | 100425 | 0.102 | Reference 16, Table 16.4-3, d |

- Average of factors for forest residues.

- Emissions factor is from footnote to Table 2.5-5

- Average of factors from Table 16.4-2 except for last two rows (test burn with blower)

- Average of factors from Table 16.4-3 except for last two columns (test burn with blower)

# Controls

Controls for land clearing debris burning are generally in the form of a ban on open burning of waste in a given municipality or county. Counties that are more than 80% urban by land area determined by the 2010 U.S. Census data,13 are assumed not to practice any open burning of land clearing debris. Therefore, CAP and HAP emissions from open burning of land clearing debris are zero in these counties.

Additionally, it is assumed that even in counties that are less than 80% urban by land area, open burning will only be practiced in areas that are rural. Therefore, the total land clearing debris burned per county (from equation 26) will be scaled based on the fraction of rural land area in each county from the 2010 Census.

| BLCDr,c=BLCDc×RLandcTLandc |  |
| --- | --- |

Where:

BLCDr,c	=	Land clearing debris burned in rural areas by county, c, in tons

BLCDc	=	Land clearing debris burned by county, c, in tons

RLandc	=	Amount of rural land by land area in county c

TLandc	=	Total amount of land in county c

Further controls on burning (i.e., burn bans in rural areas) are represented by multiplying the land clearing debris burned in rural counties by a burn ban’s effectiveness; effectiveness is a value between 0 and 1.

| BLCDr,c=BLCDr,c×BEc |  |
| --- | --- |

Where:

BLCDr,c	=	Land clearing debris burned in rural areas by county, c, in tons

BEc	=	Burn ban effectiveness in county c

In this methodology, burn ban effectiveness is represented by a single value between 0 and 1 that is multiplied by the amount of land clearing debris burned in the rural areas of each county. In practice, the burn ban effectiveness is a function of both a rule’s penetration and effectiveness. Rule penetration refers to the extent to which a regulation covers emissions for a specified controlled area, and effectiveness concerns the ability of the regulatory program to achieve emissions reductions compared to full compliance. By default the burn ban effectiveness for each county is 1 (i.e. the methodology assumes no burn bans in each county), although this may be updated by state, local, or tribal agencies.

# Emissions

County-level criteria pollutant and HAP emissions are calculated by multiplying the mass of land clearing debris burned in rural areas per year (from equation 28) by an emissions factor from Table 8.

| Ec,p=BLCDr,c×EFp×1 ton2000 lb |  |
| --- | --- |

Where:

Ec,p	=	Emissions by county, c, and pollutant, p, in tons

BLCDr,c	=	Land clearing debris burned in rural areas by county, c, in tons

EFp	=	Emissions factor by pollutant, p, in pounds/ton

# Point Source Subtraction

There are no point source-specific SCCs for open burning of land clearing debris; therefore point source subtraction is not performed for this category.

# Sample Calculations

Table 9 shows sample calculations for PM25-PRI emissions from open burning of land clearing debris in McLean County, Illinois. Equations 5 through 7 use the first quarter (Q1) of 2017 for 2-unit structures as an example. However these calculations would need to be repeated to calculate values for all 4 quarters for all 3 unit sizes. Note that structures with 5 or more units and structures with 1 unit with or without a basement have additional steps not shown in the sample calculations here. Equations 15 through 20 use urban roads as an example for acres of land disturbed from road construction. For full calculations of acres of land disturbed from road construction the calculations for rural roads would also need to be incorporated.

Table 9. Sample calculations for PM25-PRI emissions from open burning of land clearing debris in McLean County, IL

| Eq. # | Equation | Values for McLean County, IL | Result |
| --- | --- | --- | --- |
| 1 | EmpFrc=EmpcEmpUS | 140 nonres construction employees581,963 nonres construction employees | 0.000241 fraction on non-residential construction employees in McLean County, IL |
| 2 | CSc=EmpFrc×CSUS | 0.000241fraction of employees ×$ 347,666 million in nonres construction spending in the US | $83.79 million in non-residential construction spending in McLean County, IL |
| 3 | Apd2017= 2 acres $1 million×PD1992PD2017 | 2 acres disturbed $1 million×57 in 1992113 in 2017 | 1.009 acres disturbed per million dollars spent on non-residential construction spending, nationally |
| 4 | ANRc=CSc×Apdy | $83.79 million×1.009acres disturbedmillion $ | 84.4 acres disturbed from non-residential construction in McLean County, IL |
| 5 | SQ,n=UnUt×SQ,2-4 | 14 two unit housing starts in 200238 total housing starts in 2002 ×2 two to four unit housing starts in Q1 2017 | 0.74 thousand housing starts for 2-unit structures in Q1 2017, nationally |
| 6 | rQ,n=SQ,nSQ.t | 0.74 two unit housing starts72 two or more unit housing starts | 0.01 ratio of buildings with 2 units to all units greater than 2 for Q1 2017, nationally |
| 7 | AQ,n,rgn=rQ,n×(RSt-RS1) | 0.01×(21 total Q1 housing starts in Midwest-14 one unit housing starts in Midwest ) | 0.07 thousand housing starts for 2-unit structures for Q1 2017 in the Midwest |
| 8 | CF5= U5,rgnB5,r | N/A | Equation is for 5 or more unit buildings; example is for 2-unit buildings |
| 9 | Bn,rgn=(AQ.n.rgn)×1,000n | 0.775 two unit structures ×1,0002 units per building | 388 2-unit structures constructed in the Midwest |
| 10 | Bn,rgn=(AQ.n.rgn)×1,000CF5 | N/A | Equation is for 5 or more unit buildings; example is for 2-unit buildings |
| 11 | BPn,rgn=BPn,c | Midwest two unit building permits | 1,571 2-unit structure building permits in the Midwest |
| 12 | RBP,c=BPn,cBPn,rgn | 1 McLean County building permits1,571 Midwest building permits | 0.000637 ratio of county-level building permits to regional-level building permits in McLean County, IL |
| 13 | Bn,c=Bn,rgn×RBP,c | 388 two unit building starts in the Midwest×0.000637 | 0.25 total 2-unit structure building starts for McLean County, IL |
| 14 | ARn,c=Bn.c×an | 0.25 two unit structures ×0.33 acres per structure | 0.08 acres surface soil disturbed by 2-unit structures in McLean County, IL |
| 15 | HSs,r=ctSs,r | $20,399,000+$33,029,000+$93,892,000 | $147,320,000 spent on urban interstate construction in IL |
|  |  | $58,519,000+$2,626,000+$35,1367,000+$206,057,000+$17,193,000 | $319,532,000 spent on urban other arterial construction in IL |
|  |  | $16,093,000+$338,000+$355,000 | $16,786,000 spend on urban collector construction in IL |
| 16 | RCm,s,r=HSs,rTDM | $147,320,0006,895,000 $ per mile | 21.4 miles of urban interstate constructed in IL |
|  |  | $319,532,000 4,112,000 $ per mile | 77.7 miles of urban other arterial constructed in IL |
|  |  | $16,786,0004,112,000 $ per mile | 4.1 miles of urban collector constructed in IL |
| 17 | RCa,s,r=RCm,s,r×ADM | 21.4 miles×11.4 acres per mile | 242.9 acres disturbed from urban interstate construction in IL |
|  |  | 77.7 miles×7.6 acres per mile | 589.6 acres disturbed from urban other arterial construction in IL |
|  |  | 4.1 miles×7.6 acres per mile | 31 acres disturbed from urban collector construction in IL |
| 18 | ARCs=rRCa,s | 242.9 acres+589.6 acres+31 acres | 863.5 acres disturbed from urban road construction in IL |
| 19 | BFracc=BuildcBuilds | 246 building starts in McLean County20,578 building starts in IL | 0.012 fraction of building starts in McLean County, IL |
| 20 | ARCc=ARCs×BFracc | 863.5 acres×0.012 | 10.4 acres disturbed from urban road construction in McLean County, IL |
| 21 | TADc=ANRc+(Sn.c)+ARCc | 84.4 acres+62.02* acres+13.95** acres * note that the value for residential construction is for all unit types, not just 2-unit buildings as shown in example above ** note the value for road construction is for all road types, not just urban roads as shown in the example above | 160.4 total acres disturbed in McLean County, IL |
| 22 | AAK/HI,c,f=LAAK/HI,c×LCAK/HI,%,f | N/A | Equation is for Alaska or Hawaii |
| 23 | WFLFc,f=Ac,fAtotal×LFf | 17,516 acres758,793 acres×99 tons per acre | 2.3 tons/ acre weighted factor for hardwood fuel in McLean County, IL |
|  |  | 0 acres758,793 acres×57 tons per acre | 0.0 tons/ acre weighted factor for softwood fuel in McLean County, IL |
|  |  | 741,276 acres758,793 acres×4.5 tons per acre | 4.4 tons/ acre weighted factor for grass fuel in McLean County, IL |
| 24 | WFLFc=WFLFc,f | 2.3tonsacre+0.0tonsacre+4.4tonsacre | 6.7 tons/acre weighted factor for all fuels in McLean County, IL |
| 25 | LCDc=TADc×WFLFc | 160.4 acres×6.7tonsacre | 1,071 tons of land clearing debris in McLean County, IL |
| 26 | BLCDc=LCDc×CFc | 1,071 tons×1 control factor | 1,071 tons of land clearing debris burned in McLean County, IL |
| 27 | BLCDr,c=BLCDc×RLandcTLandc | 1,071 tons×2,923,414,473 m2 rural land3,064,933,852 m2 total land | 1,022 tons of land clearing debris burned in rural areas in McLean County, IL |
| 28 | Ec,p=BLCDr,c×EFp×1 ton2000 lb | 1,022 tons×13.1053lbton×1 ton2000 lb | 6.7 tons PM25-PRI emissions in McLean County, IL |

# Changes from 2014 Methodology

There main change to this methodology from the methodology used to calculate the 2014 v2 NEI is that the estimated amount of land clearing debris in each county is multiplied by the fraction of rural land area in each county. This step was not done in the methodology used for the 2014 v2 NEI.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Since insufficient data exist to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emissions factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emissions factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

U.S. Census Bureau. 2017. Value of Construction Put in Place, from http://www.census.gov/construction/c30/c30index.html.

Midwest Research Institute. 1999. Estimating Particulate Matter Emissions from Construction Operations, Final Report (prepared for the Emission Factor and Inventory Group, Office of Air Quality Planning and Standards, U.S. Environmental Protection Agency).

U.S. Census Bureau. Price Deflator (Fisher) Index of New Single‐Family Houses Under Construction. https://www.census.gov/construction/nrs/pdf/price_uc.pdf.

U.S. Census Bureau. 2018.  “New Privately Owned Housing Units Started, Annual Data,” https://www.census.gov/construction/nrc/pdf/startsan.pdf

U.S. Census Bureau. 2018. “New Privately Owned Housing Units by Purpose and Design,” https://www.census.gov/construction/nrc/pdf/quarterly_starts_completions.pdf

U.S. Census Bureau. 2001. Housing Starts. Table 1. https://www.census.gov/prod/2001pubs/c20-0103.pdf.

U.S. Census Bureau, New Privately Owned Housing Units Authorized - Unadjusted Units for Regions, Divisions, and States, Annual 2017, Table 2au. https://www.census.gov/construction/bps/txt/tb2u2017.txt..

U.S. Census Bureau, Annual Housing Units Authorized by Building Permits CO2017A, https://www2.census.gov/econ/bps/County/.

Federal Highway Administration. 2014 Highway Spending, http://www.fhwa.dot.gov/policyinformation/statistics/2014/sf12a.cfm.

Florida Department of Transportation. Cost Per Mile Models. https://www.fdot.gov/programmanagement/estimates/lre/costpermilemodels/cpmsummary.shtm.

2017 Building Permits data from US Census “BPS01”, http://www.census.gov/construction/bps/

U.S. Geological Survey (USGS). 2015. National Land Cover Database (NLCD). http://www.mrlc.gov/nlcd2011.php

U.S. Census Bureau, Decennial Censuses. 2010. Census: Summary File 1, http://www2.census.gov/census_2010/04-Summary_File_1/.

D.V. Sandberg, D.E. Ward, R.D. Ottmar, C.C. Hardy, T.E. Reinhardt, and J.N. Hall. 1989. Mitigation of Prescribed Fire Atmospheric Pollution through Increased Utilization of Hardwoods, Piled Residues, and Long-Needled Conifers. Final Report. USDA Forest Service, Pacific Northwest Research Station, Fire and Air Resource Management.

U.S Census Bureau, County Business Patterns. 2016. Complete County File [15.6mb zip], from https://www.census.gov/data/datasets/2016/econ/cbp/2016-cbp.html.

U.S. Environmental Protection Agency. 1992. AP-42, Fifth Edition, Volume 1, Chapter 2: Solid Waste Disposal. https://www3.epa.gov/ttn/chief/ap42/ch02/index.html

U.S. Environmental Protection Agency. 2001. Emission Inventory Improvement Program, Volume III, Chapter 16, Open Burning, https://www.epa.gov/sites/production/files/2015-08/documents/iii16_apr2001.pdf

U.S. Environmental Protection Agency. 1997. Evaluation of Emissions from the Open Burning of Household Waste in Barrels, EPA-600/R-97-134a.