# OTHER MERCURY

# Source Category Description

The other mercury emission categories include Switches and Relays, Fluorescent Lamp Breakage, Dental Amalgam, General Laboratory Activities, Thermostats, Thermometers, Fluorescent Lamp Recycling, and Batteries.

## Switches and Relays

Switches and relays make up the largest potential source of mercury from products that intentionally contain mercury. Mercury is an excellent electrical conductor and is liquid at room temperature, making it useful in a variety of products, including switches used to indicate motion or tilt, as the mercury will flow when the switch is in a certain position, completing the circuit.

While mercury switches in cars were phased out as of the 2002 model year, there are still millions of cars on the road that contain them. The switches and relays in these cars are potential emissions sources when the cars are recycled at the end of their useful lives, which involves crushing and shredding of the car. The shredded material is then sent to an arc furnace to recycle the steel. To avoid double counting point source emissions from arc furnaces, this source category only includes an estimate of nonpoint emissions from crushing/shredding operations. In 2014, switches and relays in the US, Puerto Rico, and US Virgin Islands resulted in 3,296 pounds of mercury emissions.

## Fluorescent Lamp Breakage/Recycling

Fluorescent lights are a potentially significant source of mercury emissions. Although each lamp contains only a small amount of mercury, which has been decreasing in recent years, the increased demand for fluorescent lamps could lead to increases in mercury emissions. Increased demand for fluorescent lamps, particularly compact fluorescents, is driven partly by the phase out of many types of incandescent bulbs from the Energy Independence and Security Act of 2007 (PL 110-140 § 321),  In 2014, fluorescent lamp breakage in the US, Puerto Rico, and US Virgin Islands resulted in 1,779.06 pounds of mercury emissions.

In addition to emissions of mercury from the breakage of fluorescent light bulbs (SCC 2861000000), there is a small amount of emissions from recycling fluorescent bulbs (SCC 2861000010). In 2014, fluorescent lamp recycling in the US, Puerto Rico, and US Virgin Islands resulted in 0.642 pounds of mercury emissions.

## Dental Amalgam

Dental amalgam is used to fill cavities in teeth, and it is composed of approximately 45% mercury; however, the use of dental amalgam is declining due to the increased popularity of composite fillings for teeth. Nevertheless, there is still a small amount of mercury emissions from dental amalgam in restored teeth. There are two potential sources of mercury emissions from dental amalgam: emissions from the preparation of amalgam in dental offices, and emissions directly from restored teeth. In 2014, dental amalgam in the US, Puerto Rico, and US Virgin Islands resulted in 922 pounds of mercury emissions.

## General Laboratory Activities

Documentation for previous versions of the NEI have cited personal communications with USGS staff for estimates of the amount of mercury used in general laboratory activities. In discussions with Robert Virta of the USGS (2013), EPA learned that the USGS stopped conducting its survey of the end uses of mercury in the economy in 2002. However, the Interstate Mercury Education and Reduction Clearinghouse (IMERC) tracks the use of mercury-added chemical products that are sold as a consistent mixture of chemicals. Since this trend indicates that the use of mercury-added chemical products has remained relatively consistent since 2002, the estimate of mercury emissions from general laboratory activities in the 2008 NEI is pulled forward for the 2017 NEI.

## Thermostats/Thermometers

Mercury has been used in thermostats to switch on or off a heater or air conditioner based on the temperature of a room. Most of the historic production of mercury thermostats came from three corporations: Honeywell, White-Rogers, and General Electric. In 1998 these corporations formed the Thermostat Recycling Corporation (TRC), a voluntary program that attempts to collect and recycle mercury thermostats as they come out of service. In 2014, thermostats in the US, Puerto Rico, and US Virgin Islands resulted in 231 pounds of mercury emissions.

Mercury thermometers have all but been phased out in the United States, with the USEPA and National Institute of Standards and Technology (NIST) working to phase out mercury thermometers in industrial and laboratory settings. NIST issued notice in 2011 that it would no longer calibrate mercury-in-glass thermometers for traceability purposes. EPA issued a rule in 2012 that provides flexibility to use alternatives to mercury thermometers when complying with certain regulations pertaining to petroleum refining, power generation, and PCB waste disposal. Furthermore, thirteen states have laws that limit the manufacture, sale, and/or distribution of mercury-containing fever thermometers. Nevertheless, given the historical prevalence of mercury thermometers, it is likely that a significant amount of mercury remains in thermometers in homes in the United States. In 2014, thermometers in the US, Puerto Rico, and US Virgin Islands resulted in 11 pounds of mercury emissions.

## Batteries

Mercury use in batteries has decreased from a peak of 1,000 tons per year in the early 1980s to less than one ton in the mid-1990s., The Mercury-Containing and Rechargeable Battery Management Act of 1996 (P.L. 104-142) phased out the use of most types of mercury containing batteries. The batteries that were manufactured before Congress enacted this statute are not expected to still be in use. In addition, the amount of mercury emissions from batteries prior to disposal is also minimal. Therefore no mercury emissions are expected to be emitted to the air from batteries in 2017.

For this source category, the following SCCs are assigned:

| Category | SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- | --- |
| Switches and Relays | 2650000002 | Waste Disposal, Treatment, and Recovery | Scrap and Waste Materials | Scrap and Waste Materials | Shredding |
| Fluorescent Lamp Breakage | 2861000000 | Miscellaneous Area Sources | Fluorescent Lamp Breakage | Non-recycling Related Emissions | Total |
| Fluorescent Lamp Recycling | 2861000010 | Miscellaneous Area Sources | Fluorescent Lamp Breakage | Recycling Related Emissions | Total |
| Dental Amalgam | 2850001000 | Miscellaneous Area Sources | Health Services | Dental Alloy Production | Overall Process |
| Laboratory Activities | 2851001000 | Miscellaneous Area Sources | Laboratories | Bench Scale Reagents | Total |
| Thermostats / Thermometers | 2650000000 | Waste Disposal, Treatment, and Recovery | Scrap and Waste Materials | Scrap and Waste Materials | Total: All Processes |

# Overview of Calculations

This section provides an overview of the calculations used to estimate mercury emissions from each source category. Data sources and calculations are discussed in section C. The process of allocating activity data to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from these mercury categories is discussed in section G.

## Switches and Relays

The calculations for estimating mercury emissions from switches and relays involve first estimating the number of switches unrecovered by the state by taking the difference between the total estimated number of switches available and the total switches recovered in each state. The number of unrecovered switches is then apportioned to each county based on the number of car recycling facilities from the US Census County Business Patterns data for NAICS 423930. The total amount of switches unrecovered by county is multiplied by the emissions factor for mercury to estimate mercury emissions from switches and relays.

## Fluorescent Lamp Breakage/Recycling

The calculations for estimating the emissions from fluorescent lamp breakage and recycling involve first estimating the average life, in hours, of various fluorescent lamp types. Data from a Freedonia Group Industry Study on the U.S. lamp market is used to estimate the total number of lamps that are discarded or recycled. The number of bulbs recycled is calculated using a recycling rate percentage. This number is then subtracted from all bulbs discarded or recycled to determine the number of bulbs discarded. The activity data are allocated to the county-level based on the share of the population present in each county. An emissions factor is calculated using the amount of mercury available in each fluorescent bulb type. The total amount of fluorescent bulbs recycled or discarded is multiplied by the emissions factor for mercury to estimate mercury emissions from fluorescent lamp breakage and recycling.

## Dental Amalgam

The calculations for estimating the emissions from dental amalgam include estimating emissions from both dental fillings and dental office preparation. The number of fillings by age group (for dental fillings) and the total mercury sold in dental amalgam (for dental office preparation) are allocated to the county-level based on the share of the population present in each county. The dental filling data by age group are multiplied by the percent of mercury present in dental fillings to determine the amount of mercury from dental fillings. The total amount of mercury from dental fillings and from dental office preparation are multiplied by emissions factors for mercury and summed together to estimate the total mercury emissions from dental amalgam.

## Thermostats/Thermometers

The calculations for estimating the emissions from thermostats and thermometers involve first estimating the total number of thermostats disposed and the amount of mercury in thermometers available for release. The number of thermostats disposed and the amount of mercury in thermometers available for release are allocated to the county-level based on the share of the population present in each county. The total number of thermostats disposed and the amount of mercury in thermometers available for release are multiplied by the emissions factor for mercury and summed together to estimate mercury emissions from thermostats and thermometers.

# Activity Data

## Switches and Relays

The End of Life Vehicle Solutions Corporation (ELVS) provides information on the estimated number of switches available for recovery in each state and the amount of switches actually recovered in 2017., There were 1.8 million mercury-containing automobile switches available nationwide in 2017 and 217,634 switches collected for recycling, for a collection rate of 11.7%. Therefore, there were approximately 1.6 million unrecycled automotive switches in 2017 in the United States. The state level number of switches unrecovered is calculated by taking the difference between the total estimated number of switches available and the total switches recovered in each state.

| UnSs=TotSs-RecSs |  |
| --- | --- |

Where:

UnSs	=	Total switches unrecovered by state s

TotSs	= 	Total switches available in state s

RecSs	= 	Total switches recovered by state s

## Fluorescent Lamp Breakage/Recycling

Data from a Freedonia Group Industry Study on the U.S. lamp market were used to estimate that approximately 1.485 billion mercury containing lamps, including compact fluorescents (CFLs), linear, and high impact discharge (HID) lamps, were discarded or recycled in 2017. Bulb sales for 2002, 2007, 2012 and projections for 2017 were obtained from Freedonia; sales for all other years were calculated by extrapolating data. Average rated life (hours) of lamp types is used to calculate lifetimes (years), assuming that CFLs are on for 4 hours per day and all other fluorescents and HIDs are on for 8 hours per day., Table 1 includes the lifetime data assumed for each bulb type. The lifetime data are used to estimate the year in which bulbs that are discarded or recycled in 2017 would have been purchased.

Table 1. Lifetime in hours and years for each bulb type12,13

| Bulb Type | Life (hrs) | Life (yr) | Purchase Year* | Number of bulbs (million) |
| --- | --- | --- | --- | --- |
| CFL | 9,000 | 6 | 2011 | 722 |
| Linear | 25,000 | 9 | 2008 | 583 |
| HID | 17,000 | 6 | 2011 | 180 |
| Total | -- | -- | -- | 1,485 |
| *If bulbs are discarded or recycled in 2017 |  |  |  |  |

| TotB=bPBb |  |
| --- | --- |

Where:

TotB	=	Total number of bulbs discarded and recycled, in million units (see Table 1)

PBb	=	Total number of bulb type b purchased

According to a 2010 study by Silveira and Chang, the recycling rate for mercury containing lamps in the U.S. is 23%. Taking into account recycling, this suggests that there were approximately 1,143 million mercury-containing lamps discarded at landfills and approximately 341 million mercury-containing lamps recycled in 2017.

For fluorescent bulbs recycled:

| RecB=TotB×RR |  |
| --- | --- |

Where:

RecB	=	Total number of bulbs recycled, in million units

TotB	=	Total number of bulbs discarded and recycled, in million units

RR	= 	Recycling rate for mercury containing lamps in the US

For fluorescent bulbs discarded:

| DiscB=TotB-RecB |  |
| --- | --- |

Where:

DiscB	=	Total number of bulbs discarded, in million units

TotB	=	Total number of bulbs discarded and recycled, in million units

RecB	=	Total number of bulbs recycled, in million units

## Dental Amalgam

According to a NEWMOA’s IMERC factsheet (2015), the amount of mercury in dental amalgam was estimated to be 15.97 tons (31,940 lbs.) in 2013.

The amount of mercury emissions from restored teeth is estimated using data from the National Institutes of Health’s National Institute of Dental and Craniofacial Research, which provides estimates of the average number of filled teeth per person, from the CDC National Health and Nutrition Examination Survey (NHANES), in nine different age brackets: 2-5 years, 6-11 years, 12-15 years, 16-19 years, 20-34 years, 35-49 years, 50-64 years, 65-74 years, and 75 and up. The filling data for the age groups 6-11 years, 12-15 years, and 16-19 years are averaged together as are the filling data for the age groups 65-74 years and 75 and up to match the U.S. Census age category, 5-19 and 65 and up. Table 2 lists the average number of filled teeth per person by age group.

Table 2. Average number of filled teeth per person and percentage of fillings containing mercury by age group.17

| Age Group | Average Number of Filled Teeth Per Person | Percentage of Fillings Containing Mercury |
| --- | --- | --- |
| 0–4 | 0.47 | 15.8% |
| 5–19 | 1.756 | 31.6% |
| 20–34 | 4.61 | 40.8% |
| 35–49 | 7.78 | 50% |
| 50–64 | 9.20 | 62.5% |
| 65+ | 8.69 | 75.0% |

According to the American Dental Association (ADA 1998) more than 75% of restorations before the 1970s used amalgam, which declined to 50% by 1991. Using these numbers, it is assumed that 40.8% of the filled teeth for 20-34 age group contain amalgam, 50% of filled teeth in the 35-49 age group, 62.5% of filled teeth in the 50-64 age group, and 75% of filled teeth for people over 65. The BAAQMD memorandum is used to estimate that 31.6% of filled teeth in the 1-19 age group contain amalgam. The Food and Drug Administration has discouraged the use of dental amalgam in children under 6. While EPA does not have data on the percent of fillings containing dental amalgam for the 0-4 age group, it is assumed that the percentage of fillings containing mercury in this age group is approximately half that of the overall under 20 age group.

## Thermostats/Thermometers

A 2002 EPA report estimated that 2-3 million thermostats came out of service in 1994.7 A 2013 report from a consortium of environmental groups, which assumed that the estimate from the 2002 EPA report remained viable, estimated that the TRC collects at most 8% of the retired thermostats each year. A literature search revealed no new data that could be used to estimate the number of thermostats coming out of service. Therefore, using this estimate, there are approximately 2.3 million thermostats that are not recycled each year.

| DispTs= RemTs×(1-0.08) |  |
| --- | --- |

Where:

DispTs	=	Total thermostats disposed

RemTs	=	Total thermostats removed from service

Data from a NEWMOA’s IMERC factsheet suggests that there were 546 lbs. of mercury used in thermometers in 2013. Using past NEWMOA IMERC thermometer data we forecasted the values for mercury in 2014-2017. See Table 3 for the amount of mercury used in thermometers each year from 2013-2017.

Table 3. Total mercury in thermometers sold and mercury available from thermometers, annually20

| Year | Total Mercury in Thermometers Sold (lbs.) | Mercury Available from Thermometers each year (lbs.) |
| --- | --- | --- |
| 2013 | 546 | 519 |
| 2014 | 532 | 1,024 |
| 2015 | 523 | 1,496 |
| 2016 | 514 | 1,936 |
| 2017 | 506 | 2,345 |

The US EPA assumes that the average lifespan of a glass thermometer is 5 years, and that 5% of glass thermometers are broken each year.7 Therefore, using the pounds of mercury available in thermometers each year (shown in Table 3 above) there would be an estimated 2,345 pounds of mercury remaining in thermometers in 2017 (accounting for the breakage rate each year). The following equation calculates the total amount of mercury remaining in thermometers for each year during the lifespan of the thermometer. To calculate the value at the 5 year lifespan mark, the following equation (equation T2) needs to be used to calculate the value for years 2 through 5, with each year building upon the previous year (i.e., the calculation needs to be conducted for all years to find the final year 5 data). See Table 3 for the final values of mercury available from thermometers in 2017, and Table 15 for detailed calculations on how to arrive at the final number.

| HgTmn= HgTmn-1×95% +HgTmSoldn |  |
| --- | --- |

Where:

HgTmn		=	Amount of mercury remaining in thermometers in year n, in pounds

HgTmn-1		=	Amount of mercury remaining in thermometers in the year prior to year n, in pounds

HgTmSoldn	=	Amount of mercury in thermometers in year 1, in pounds

n		=	Year

King et al. (2008) estimate that during the period 2000-2006 there were 350 lbs. of mercury from thermometers collected in recycling programs.

Subtracting the amount of mercury removed due to thermometers being collected in recycling programs from the total amount of mercury remaining in thermometers in 2017 estimates the total amount of mercury in thermometer available for release, in tons. Therefore, there were 1,995 lbs. (0.99 tons) of mercury available for release in 2017.

| HgTRl=HgTm5-HgTRm×1 ton2,000 lbs. |  |
| --- | --- |

Where:

HgTRl	=	Amount of mercury in thermometers available for release, in tons

HgTm5	=	Amount of mercury remaining in thermometers in year 5, the lifespan of a thermometer, in pounds

HgTRm	=	Amount of mercury removed in thermometer collections, in pounds

# Allocation Procedure

## Switches and Relays

The number of unrecovered switches is apportioned to each county based on the number of car recycling facilities. The number of car recycling facilities is estimated using establisment data for recyclable material merchant wholesalers (NAICS 423930) from the U.S. Census Bureau’s 2016 County Business Patterns (CBP).

The number of car recycling facilities by county from the US Census County Business Patterns data is first summed to the state level.

| Fs=cFc |  |
| --- | --- |

Where:

Fs	=	Total car recycling facilities in state s

Fc	= 	Total car recycling facilities in county c

The share of state car recycling facilities by county is calculated by taking the total number of car recycling facilities in a given county by the total number of car recycling facilities in the state.

| FracFc=FcFs |  |
| --- | --- |

Where:

FracFc	=	Total fraction of state car recycling facilities in county c

Fc	= 	Total car recycling facilities in county c

Fs	=	Total car recycling facilities in state s

The share of unrecovered switches by county is calculated using the state number of unrecovered switches and the total share of state car recycling facilities by county, calculated above.

| UnSc=UnSs×FracFc |  |
| --- | --- |

Where:

UnSc	=	Total switches unrecovered in county c

UnSs	=	Total switches unrecovered in state s

FracFc	=	Total share of state car recycling facilities in county c

## Fluorescent Lamp Breakage/Recycling

The national-level mercury emissions from fluorescent lamp breakage are allocated to each county based on population.

| FracPc=PcPUS |  |
| --- | --- |

Where:

FracPc,	=	Fraction of total US population in county c

Pc	=	Population in county c

PUS	=	Population in the US

The fraction of total US population in a county is multiplied by the national data for fluorescent bulbs recycled or discarded to calculate the number of fluorescent bulbs recycled or discarded at the county-level.

For fluorescent bulbs discarded:

| DiscBc=FracPc×DiscB |  |
| --- | --- |

Where:

DiscBc	=	Total number of bulbs discarded in county c, in million units

FracPc	=	Fraction of total US population in county c

DiscB	=	Total number of bulbs discarded in the US, in million units

For fluorescent bulbs recycled:

| RecBc=FracPc×RecB |  |
| --- | --- |

Where:

RecBc	=	Total number of bulbs recycled in county c, in million units

FracPc	=	Fraction of total US population in county c

RecB	=	Total number of bulbs recycled in the US, in million units

## Dental Amalgam

The amount of mercury from dental office preparations, based on the amount of mercury in dental amalgam from NEWMOA’s IMERC factsheet15, are allocated to the county level based on population.

| FracPc=PcPUS |  |
| --- | --- |

Where:

FracPc	=	Fraction of total US population in county c

Pc	=	Total population in county c

PUS	=	Total population for the United States

The county-level population fraction is multiplied by the amount of mercury sold for dental amalgam to calculate the total mercury from dental office preparations by county.

| HgOc=FracPc×HgDA |  |
| --- | --- |

Where:

HgOc	= 	Total mercury from dental office preparations in county c, in pounds

FracPc	=	Fraction of total US population in county c

HgDA	=	Total mercury sold for dental amalgam in the US, in pounds

The emissions from filled teeth are allocated to each county by multiplying the county population by the proportion of the national population in each age group, the average number of filled teeth per person, and the fraction of fillings containing mercury (Table 2; fraction = percentage/100). The age groups listed in Table 2, hereafter referred to as filling groups, are different than official US census bureau age groups; therefore national fractions of each US census bureau age group were calculated, summed, and multiplied by county level population to estimate the county level population for each filling group. Table 6 shows how the US Census age groups correspond to each filling group.

Table 6. US Census age groups and filling groups

| US Census Age Group | Corresponding Filling Age Group |
| --- | --- |
| Under 5 | 0–4 |
| 5–9 | 5–19 |
| 10–14 |  |
| 15–19 |  |
| 20–24 | 20–34 |
| 25–29 |  |
| 30–34 |  |
| 35–39 | 35–49 |
| 40–44 |  |
| 45–49 |  |
| 50–54 | 50–64 |
| 55–59 |  |
| 60–64 |  |
| 65–69 | 65+ |
| 70–74 |  |
| 75–79 |  |
| 80–84 |  |
| 85 and up |  |

First, the share of total population each US Census age group represents to the entire US population is calculated.

| FracPa=PaPUS |  |
| --- | --- |

Where:

FracPa	=	Fraction of the total US population in Census Bureau age group a

Pa	=	Total population in Census Bureau age group a

PUS	=	Total population for the United States

The fraction of the population for each US Census age group is then summed to match the filling groups.

| FracPfg=aFracPa |  |
| --- | --- |

Where:

FracPfg	=	Fraction of the total US population in filling group fg

FracPa,	=	Fraction of the total US population in census bureau age group a, where age group a falls within filling group fg

The fraction of population for each filling group is multiplied by the county-level population data to get the total population for each filling group.

| Pfg,c=FracPfg×Pc |  |
| --- | --- |

Where:

Pfg,c	=	Total population in filling group fg in county c

FracPfg	=	Fraction of the total US population in filling group fg

Pc	=	Total population in county c

The filling group county-level population is multiplied by the average number of fillings per person in each filling group to determine the total number of fillings in each filling group in each county.

| Ffg,c=Pfg,c×Ffg |  |
| --- | --- |

Where:

Ffg,c	=	Total fillings in filling group fg in county c

Pfg,c	=	Total population in filling group fg in county c

Ffg	=	Average number of fillings per person in filling group fg

The total fillings in each filling group is then multiplied by the fraction of fillings that contain mercury in each filling group to determine the total number of fillings by filling group in each county.

| HgFfg,c=Ffg,c×FracHgFfg |  |
| --- | --- |

Where:

HgFfg,c		=	Total fillings containing mercury in filling group fg in county c

Ffg,c		=	Total fillings in filling group fg in county c

FracHgFfg	=	Fraction of fillings containing mercury in filling group fg

## Thermostats/Thermometers

The national-level mercury emissions from thermostats and thermometers are allocated to the county level based on population.

| FracPc=PcPUS |  |
| --- | --- |

Where:

FracPc	=	Fraction of total US population in county c

Pc	=	Total population in county c

PUS	=	Total population for the United States

The fraction of the US population in the county is multiplied by the national data for thermostats and thermometers to calculate the number of thermostats disposed and the amount of mercury in thermometers available for release at the county-level.

For thermostats:

| DispTsc=FracPc×DispTs |  |
| --- | --- |

Where:

DispTsc	=	Total thermostats disposed of in county c

FracPc	=	Fraction of total US population in county c

DispTs	=	Total thermostats disposed of in the US

For thermometers:

| HgTmc=FracPc×HgTmRl |  |
| --- | --- |

Where:

HgTmc	=	Amount of mercury in thermometers available for release in county c, in pounds

FracPc	=	Fraction of total US population in county c

HgTmRl	=	Amount of mercury in thermometers available for release in the US, in tons

# Emission Factors

## Switches and Relays

The response to comments for the 2007 EPA Significant New Use Rule on Mercury Switches (72 Fed. Reg. 56903), suggests that the weighted average amount of mercury in switches is 1.2 grams (0.0026 lbs.). A report by Griffith et al. (2001) shows that 60% of mercury in switches is released at the shredding operation, while 40% is sent to arc furnaces for smelting. Therefore, the emissions factor for switches is 60% of the emissions factor reported in the 2007 EPA Significant New Use Rule on Mercury Switches response to comment document, 0.00156 lbs. per switch.

## Fluorescent Lamp Breakage/Recycling

The average amount of mercury in a CFL has been studied extensively, with the amount of mercury in each CFL commonly reported as 1.27–4.0 mg (2.63 mg average, Table 7). Linear fluorescent bulbs contain more mercury than CFLs, with a range of 8.3 to 12 mg per bulb (10.15 average, Table 8). Data from the USGS suggests that there is an average of 17 mg of mercury per HID bulb.

Table 7. Mercury used in CFLs (mg/bulb) as determined by three different studies

| Study | Average Amount of Mercury per CFL (mg) | Source |
| --- | --- | --- |
| Li and Jin (2011) | 1.27 |  |
| Arendt and Katers (2013) | 4.00* |  |
| Singhvi et al. (2011) | 2.63 |  |
| Average | 2.63 | -- |

*Adjusted from 4.5 mg to 4 mg due to increased market penetration of Energy Star CFLs with a lower Hg content.

Table 8. Mercury used in linear fluorescent bulbs (mg/bulb) as determined by two different studies

| Study | Average Amount of Mercury per Linear Fluorescent Bulb (mg) | Source |
| --- | --- | --- |
| Aucott et al. (2004) | 12.0 |  |
| NEMA (2005) | 8.3 |  |
| Average | 10.2 | -- |

Cain et. al (2007) provides the most comprehensive materials flow analysis of mercury intentionally used in products. Their analysis estimates that 10% of all mercury used in fluorescent light bulbs is eventually released to the atmosphere after production and before disposal, with the majority being released during transport to the disposal facility.

The emissions factor for CFL, linear, and HID bulbs are calculated by multiplying the average amount of mercury per bulb discussed above by 10%.

| EFb,p=Hgb×0.10 |  |
| --- | --- |

Where:

EFb,p	=	Emissions factor by bulb b for pollutant p, in mg/bulb

Hgb	=	Average mercury content per bulb b, in mg

The emissions factors for all three bulb types can be found in Table 9.

Table 9. Mercury emissions factors for CFLs, linear fluorescents and HIDs

| Bulb type | Pollutant | Pollutant Code | Emissions Factor | Emissions Factor Units |
| --- | --- | --- | --- | --- |
| CFL | Mercury | 7439976 | 0.263 | mg/bulb |
| Linear | Mercury | 7439976 | 1.015 | mg/bulb |
| HID | Mercury | 7439976 | 1.7 | mg/bulb |

A weighted average of all three emissions factors in Table 9 is calculated to estimate total emissions from all fluorescent lamp breakage. The first step estimates the fraction each bulb represents of the total amount of bulbs discarded and recycled.

| FracTotBb=PBbTotB |  |
| --- | --- |

Where:

FracTotBb	=	Fraction of bulb type b discarded and recycled

PBb	=	Total number of bulb type b discarded and recycled, in million bulbs

TotB	=	Total number of bulbs discarded and recycled in the US, in million bulbs

A weighted emissions factor for fluorescent lamp breakage is then calculated by multiplying the fraction the bulb type represents of the total number of bulbs by the bulb type-specific emissions factor.

| EFbr,p=bEFb,p×FracTotBb×2.2×10-6lbs.mg |  |
| --- | --- |

Where:

EFbr,p	=	Weighted emissions factor for pollutant p for fluorescent bulb breakage, br, in lbs./bulb

EFb,p	=	Emissions factor for bulb type b and pollutant p, in mg/bulb (see Table 9)

FracTotBb	=	Fraction of the number of bulb type b discarded and recycled

For mercury-containing bulb recycling, the US EPA has estimated an emissions factor of 0.00088 mg/bulb (1.9 × 10-9 lbs./bulb).

## Dental Amalgam

US EPA (1997) estimates that 2% of mercury used in dental offices is emitted to the air.31

Richardson et al. (2011) estimate emissions from filled teeth of approximately 0.3 µg/day of mercury per filled tooth, or 2.4 × 10-7 lbs. per year per filled tooth. The emissions factors used for estimating mercury emissions from dental amalgam are shown in Table 10.

Table 10. Mercury emissions factors for dental amalgam

| Activity | Pollutant | Pollutant Code | Emissions Factor | Emissions Factor Units | Source |
| --- | --- | --- | --- | --- | --- |
| Released from dental offices | Mercury | 7439976 | 0.02 | Lbs./Lb. | 31 |
| Filled teeth | Mercury | 7439976 | 2.4x10-7 | Lbs./tooth filled | 32 |

## Thermostats/Thermometers

The 2002 EPA report estimates that there are 3 grams of mercury per thermostat.7 Cain et al. (2007) estimate that 1.5% of mercury in “control devices,” including thermostats, is emitted to the air before it is disposed of at a landfill or incinerator. Therefore the amount of mercury emitted is 0.045 grams per thermostat, or 9.92× 10-5 lbs. per thermostat. 7,30

, Leopold (2002) estimates that 5% of thermometers are broken each year. EPA assumes that the remaining 95% of thermometers that are not broken are still in use and therefore do not contribute to emissions. Cain et al. (2007) estimate that 10% of mercury from thermometers is emitted to the air before disposal in a landfill Therefore the emissions factor is estimated to be 10 lbs. of mercury emissions per ton of mercury in thermometers.7,30

The emissions factors used for estimating mercury emissions from thermostats and thermometers are shown in Table 11.

Table 11. Mercury emissions factors for thermostats and thermometers

| Type | Pollutant | Pollutant Code | Emissions Factor | Emissions Factor Units | Source |
| --- | --- | --- | --- | --- | --- |
| Thermostats | Mercury | 7439976 | 9.92 x 10-5 | Lbs./Thermostat | 7,30 |
| Thermometers | Mercury | 7439976 | 10 | Lbs./Ton | 7,30 |

# Controls

There are no controls assumed for this category.

# Emissions

## Switches and Relays

The total county-level mercury emissions from switches and relays, in pounds, is estimated by multiplying the total switches unrecovered for each county by the emissions factor.

| Es,p,c=UnSc×EFs,p |  |
| --- | --- |

Where:

Es,p,c	=	Annual emissions of pollutant p in county c from switches and relays, s, in lbs.

UnSc	=	Total switches unrecovered by county c

EFs,p	=	Emissions factor for pollutant p for switches and relays, s, in lbs./switch

## Fluorescent Lamp Breakage/Recycling

The total county-level mercury emissions for fluorescent lamp breakage and recycling, in pounds, is estimated by multiplying the total fluorescent lamps broken or recycled for each county by the emissions factor.

For fluorescent lamp breakage:

| Ebr,p,c=DiscBc×1,000 units×EFbr,p |  |
| --- | --- |

Where:

Ebr,p,c	=	Annual emissions of pollutant p from fluorescent bulb breakage, br, by county c, in lbs.

DiscBc	=	Total number of bulbs discarded for county c, in million units

EFbr,p	=	Weighted emissions factor for pollutant p for fluorescent bulb breakage, br, in lbs./bulb

For fluorescent lamp recycling:

| Er,p,c=RecBc×1,000 units×EFr,p |  |
| --- | --- |

Where:

Er,p,c	=	Annual emissions of pollutant p from fluorescent lamp recycling, r, by county c, in lbs.

RecBc	=	Total number of bulbs recycled for county c, in million bulbs

EFr,p	=	Weighted emissions factor for pollutant p for fluorescent bulb recycling, r, in lbs./bulb

## Dental Amalgam

The total county-level mercury emissions for dental amalgam from fillings, in pounds, is estimated by multiplying the total number of fillings containing mercury for each county by the emissions factor.

| Ef,p,c=fgHgFfg,c ×EFf,p |  |
| --- | --- |

Where:

Ef,p,c	=	Annual emissions of pollutant p from dental fillings, f, by county c, in lbs.

HgFfg,c	=	Total fillings containing mercury in filling group fg in county c

Eff,p	=	Emissions factor for pollutant p from dental fillings, f, in lbs./tooth filled

The total county-level mercury emissions for dental office preparation, in pounds, is estimated by multiplying the total pounds mercury from dental office preparations for each county by the emissions factor.

| Eo,p,c=HgOc×EFo,p |  |
| --- | --- |

Where:

Eo,p,c	=	Annual emissions of pollutant p from dental office preparations, o, by county c, in lbs.

HgOc	= 	Total mercury from dental office preparations by county c, by pounds

EFo,p	=	Emissions factor for pollutant p for dental office preparations, o, by lbs./lb.

The emissions from dental fillings and dental office preparations are summed to get the total mercury emissions from dental amalgam.

| Eda,p,c=Ef,p,c+Eo,p,c |  |
| --- | --- |

Where:

Eda,p,c	=	Annual emissions of pollutant p from total dental amalgam, da, by county c, in lbs.

Ef,p,c	=	Annual emissions of pollutant p from dental fillings, f, by county c, in lbs.

EOP,p,c	=	Annual emissions of pollutant p from dental office preparations , o, by county c, in lbs.

## Thermostats/Thermometers

The total county-level mercury emissions for thermostats, in pounds, is estimated by multiplying the total number of thermostats disposed in each county by the emissions factor.

| Ets,p,c=DispTsc×EFts,p |  |
| --- | --- |

Where:

Ets,p,c	=	Annual emissions of pollutant p for thermostats in county c, in lbs.

DispTsc	=	Total thermostats disposed in county c

Efts,p	=	Emissions factor for pollutant p for thermostats, ts, in lbs./thermostat

The total county-level mercury emissions for thermometers, in pounds, is estimated by multiplying the total amount of mercury remaining in thermometers over their lifespan for each county by the emissions factor.

| Et,p,c=HgTmc×EFt,p |  |
| --- | --- |

Where:

Et,p,c	=	Annual emissions of pollutant p for thermometers in county c, in lbs.

HgTmc	=	Amount of mercury remaining in thermometers over their lifespan in county c, in pounds

EFt,p	=	Emissions factor for pollutant p for thermometers, in lbs./ton

The emissions from thermostats and thermometers are summed to get the total mercury emissions.

| Ett,p.c=Ets,p,c+Et,p,c |  |
| --- | --- |

Where:

Ett,p,c	=	Annual emissions of pollutant p for thermostats and thermometers in county c, in lbs.

Ets,p,c	=	Annual emissions of pollutant p for thermostats in county c, in lbs.

Etm,p,c	=	Annual emissions of pollutant p for thermometers in county c, in lbs.

# Point Source Subtraction

There are no point source-specific SCCs for the other mercury categories; therefore, point source subtraction is not performed for this category.

# Sample Calculations

## Switches and Relays

Table 12 lists sample calculations to estimate the mercury emissions from switches and relays in Hartford County, Connecticut.

Table 12. Sample calculations for mercury emissions from switches and relays for Hartford County, CT

| Eq. # | Equation | Values for Hartford County, Connecticut | Result |
| --- | --- | --- | --- |
| 1 | UnSs=TotSs-RecSs | 22,000 switches available- 618 switches recovered | 21,382 unrecovered switches in Connecticut |
| 2 | Fs=csFc | All facilities in Connecticut | 85 car recycling facilities in Connecticut |
| 3 | FracFc=FcFs | 18 facilities in Hartford County, CT85 facilities in CT | 0.2118 share of state car recycling facilities in Hartford County, CT |
| 4 | UnSc=UnSs×FracFc | 21,382 unrecovered switches ×0.2118 share of state facilities | 4,528 unrecovered switches in Hartford County, CT |
| 5 | Es,p,c=UnSc×EFs,p | 4,528 switches × 0.00156lbs.switch | 7.06 pounds of mercury from switches and relays in Hartford County, CT |

## Fluorescent Lamp Breakage/Recycling

Table 13 lists sample calculations to estimate the mercury emissions from fluorescent lamp breakage in Hartford County, Connecticut.

Table 13. Sample calculations for mercury emissions from fluorescent lamp breakage for Hartford County, Connecticut

| Eq. # | Equation | Values for Hartford County, Connecticut | Result |
| --- | --- | --- | --- |
| 1 | TotB=bPBb | all bulbs recycled or discarded | 1,485 million bulbs discarded and recycled in the US in 2014 |
| 2 | RecB=TotB×RR | 1,485 million recycled and discarded bulbs ×23% recycling rate | 341 million bulbs recycled in the US in 2014 |
| 3 | DiscB=TotB-RecB | 1,485 million recycled and discarded bulbs-341 million recycled bulbs | 1,143 million bulbs discarded in the US in 2014 |
| 4 | FracPc=PcPUS | 895,388 people in Hartford County, CT 318,857,056 people in the US | 0.272% of total US population is in Hartford County, CT |
| 5 | DiscBc=FracPc×DiscB | 0.00272×1,143 million bulbs | 3.109 million fluorescent bulbs discarded in Hartford County, CT |
| 6 | RecBc=FracPc×RecB | 0.00272 ×341 million bulbs | 0.928 million fluorescent bulbs recycled in Hartford County, CT |
| 7 | EFb,p=Hgb×0.10 | CFL:2.63 mg Hg ×10% Linear:10.2 mg Hg x 10% HID:17 mg Hg x 10% | 0.263 mg Hg/CFL bulb 1.02 mg Hg/linear bulb 1.7 mg Hg/HID bulb |
| 8 | FracTotBb=PBbTotB | CFL: 722 million CFL bulbs1,485 million bulbs total Linear: 583 million Linear bulbs1,485 million bulbs total HID: 180 million HID bulbs1,485 million bulbs total | 48.6% of total for CFL 39.2% of total for Linear 12.1% of total for HID |
| 9 | EFbr,p=bEFb,p×FracTotBb×2.2×10-6lbs.mg | 0.263 mgbulb×48.6%+1.02 mgbulb×39.2%+1.7 mgbulb×12.1%×2.2×10-6lbs.mg | 1.61 x 10-6 lbs. Hg/bulb weighted emissions factor for mercury for fluorescent lamp breakage |
| 10 | Ebr,p,c=DiscBc×EFbr,p | 3,109,617 bulbs ×1.61×10-6lbs. Hgbulb | 5.0 lbs. of mercury from fluorescent lamp breakage in Hartford County, CT |
| 11 | Er,p,c=RecBc×EFr,p | 928,846 bulbs×1.94×10-9lbs. Hgbulb | 1.8 x 10-4 lbs. of mercury from fluorescent lamp recycling in Hartford County, CT |

## Dental Amalgam

Table 14 lists sample calculations to determine the mercury emissions from dental amalgam in Hartford County, Connecticut. The example will show the process for the 5-19 age group, with the total sum of emissions in the final step.

Table 14. Sample calculations for mercury emissions from dental amalgam for Hartford County, Connecticut

| Eq. # | Equation | Values for Hartford County, Connecticut | Result |
| --- | --- | --- | --- |
| 1 | FracPc=PcPUS | 895,338 people in Hartford County, CT 329,164,967 people in the US | 0.272% of total US population is in Hartford County, CT |
| 2 | HgOc=FracPc×HgDA | 0.272%×31,940 lbs. | 86.88 lbs. total mercury from dental office preparations in Hartford County, CT |
| 3 | FracPa=PaPUS | 5 to 9: 20,304,238 people, 5 to 9 age group325,719,178 people in the US 10 to 14: 20,778,454 people, 10 to 14 age group325,719,178 people in the US 15 to 19: 21,131,660 people, 14 to 19 age group325,719,178 people in the US | 6.23% of total US population for 5-9 age group 6.38% of total US population for 10-14 age group 6.49% of total US population for 14-19 age group |
| 4 | FracPfg=aFracPa | 6.23%+6.38%+6.49% | 19.1006% of total US population for 5-19 age group |
| 5 | Pfg,c=FracPfg×Pc | 19.1006%×895,338 people in Hartford County, CT | 171,025 people in the 5-19 age group in Hartford County, CT |
| 6 | Ffg,c=Pfg,c×Ffg | 171,025 people 5-19 in Hartford County, CT×1.756 fillings, 5-19 age group | 300,433 fillings in the 5-19 age group in Hartford County, CT |
| 7 | HgFfg,c=Ffg,c×FracHgFfg | 300,433 fillings, 5-19 age group×31.6% | 94,936 total fillings containing mercury in the 5-19 age group in Hartford County, CT |
| 8 | Ef,p,c=fgHgFfg,c×EFf,p | 94,936fillings with mercury, 5-19 age group×2.4×10-7 lbs.tooth filled | 0.023 pounds of mercury emissions from fillings in the 5-19 age group (0.722 pounds of mercury in all age groups) in Hartford County, CT |
| 9 | Eo,p,c=HgOc×EFo,p | 86.88 lbs. ×0.02lbs.lb. | 1.74 pounds of mercury emissions from dental office preparations in Hartford County, CT |
| 10 | Eda,p,c=Ef,p,c+Eo,p,c | 0.722 pounds+1.74 pounds | 2.46 pounds of mercury from dental amalgam in Hartford County, CT |

## Thermostats/Thermometers

Table 15 lists sample calculations to determine the mercury emissions from thermostats and thermometers in Hartford County, Connecticut.

Table 15. Sample calculations for mercury emissions from thermostats and thermometers for Hartford County, Connecticut

| Eq. # | Equation | Values for Hartford County, Connecticut | Result |
| --- | --- | --- | --- |
| 1 | DispTs= RemTs×(1-8%) | 2,500,000 thermostats removed from service ××92% | 2,300,000 thermostats disposed of in the United States in 2017 |
| 2 | HgTmn= HgTmn-1×95% +HgTm 1 | y=1:546 lbs ×95% y=2: 518.7 lbs. ×95%+532 lbs. y=3: 1,024 lbs.×95%+523 lbs. y=4: 1,496 lbs.×95%+514 lbs. y=5: 1,935 lbs.×95%+506 lbs. | 2,345 pounds of mercury available for release in thermometers in year 2017 |
| 3 | HgTRl=HgTm5-HgTRm×1 ton2,000 lbs. | 2,345 lbs.-350 lbs. ×1 ton2,000 lbs. | 0.99 tons of total mercury in thermometers available for release |
| 4 | FracPc=PcPUS | 895,388people in Hartford County, CT329,164,967 people in the US | 0.272% of total US population is in Hartford County, CT |
| 5 | DispTsc=FracPc×DispTs | 0.272%×2,300,000 thermostats | 6,256 thermostats disposed in Hartford County, CT |
| 6 | HgTmc=FracPc×HgTmRl | 0.272%×0.99 tons | 0.0027 tons of mercury from thermometers available for release in Hartford County, CT |
| 7 | Ets,p,c=DispTsc×EFts,p | 6,256 thermostats ×9.92×10-5lbs.thermostat | 0.62 pounds of mercury emissions from thermostats in Hartford County, CT |
| 8 | Et,p,c=HgTmc×EFt,p | 0.0027 tons×10lbs.ton | 0.027 pounds of mercury emissions from thermometers in Hartford County, CT |
| 9 | Ett,p.c=Ets,p,c+Et,p,c | 0.62 lbs. +0.027 lbs. | 0.647 pounds of mercury emissions from thermostats and thermometers in Hartford County, CT |

# Changes from 2014 Methodology

There are no significant changes from the 2014 NEI emissions estimation methodology for these mercury sources.

# Puerto Rico and US Virgin Islands Emissions Calculations

Since insufficient data exists to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in pounds for these two Florida counties are divided by their respective populations creating a pound per capita emission factor. For each Puerto Rico and US Virgin Island county, the pound per capita emission factor is multiplied by the county population (from the same year as the inventory’s activity data) which serves as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

Rathore, M., Singh, A., & Pant, V. A. 2012. The Dental Amalgam Toxicity Fear: A Myth or Actuality. Toxicology International, 19(2), 81–88. https://www.ncbi.nlm.nih.gov/pmc/articles/PMC3388771/, last accessed August 2018.

Vandeven, J.A. and S.L. McGinnis. 2005. An Assessment of Mercury in the Form of Amalgam in Dental Wastewater in the United States. Water, Air, and Soil Pollution, 164:349-366. Available at https://pdfs.semanticscholar.org/7e3a/f7771fd5e2e968caa0621f7ce1d260ec3c42.pdf, last accessed May 2018.

Virta, R. 2013. US Geological Survey. Personal communication with David Cooley, Abt Associates, August 21, 2013.

IMERC, 2015. IMERC Fact Sheet – Formulated Mercury-Added Products, available at http://www.newmoa.org/prevention/mercury/imerc/factsheets/formulated_products_2015.pdf, last accessed August 2018.

Thermostat Recycling Corporation. 2018. About. Available at: https://www.thermostat-recycle.org/about/, last accessed May 2018.

US EPA. 2016. Phasing of Mercury Thermometers Used in Industrial and Laboratory Settings. https://www.epa.gov/mercury/mercury-thermometers, last accessed August 2018.

USGS. 2005. Mercury End-Use Statistics. Available at: http://minerals.usgs.gov/ds/2005/140/mercury-use.pdf, last accessed May 2018.

Leopold, B.R. 2002. Use and Release of Mercury in the United States. U.S. Environmental Protection Agency. Report EPA/600/R-02/104. https://cfpub.epa.gov/si/si_public_record_report.cfm?dirEntryId=88418, last accessed May 2018.

US EPA. 1996. The Mercury-Containing and Rechargeable Battery Management Act - Public Law 104-142. Available at: https://www.epa.gov/rcra/mercury-containing-and-rechargeable-battery-management-act-public-law-104-142, last accessed May 2018.

End of Life Vehicle Solution Solutions Corporation. 2018a. Collection Reporting. Available at: https://www.usecology.com/Services/Environmental-Services/Recycling/ELVS-Mercury-Switch-Program.aspx, last accessed May 2018.

End of Life Vehicle Solution Solutions Corporation. 2018b. Estimating Population of Mercury

Convenience Light Switches. Available at: http://elvsolutions.org/?page_id=1298, last accessed May 2018.

Freedonia Group, 2013. Industry Study 3054 Lamps.

Buildings.com, 2008. Fluorescent Lamps 101. Available at: http://www.buildings.com/article-details/articleid/6002/title/fluorescent-lamps-101.aspx, last accessed May 2018.

Bulbs.com. nd. What Does Average Rated Life Mean? Available at: https://www.bulbs.com/learning/arl.aspx, last accessed May 2018

Silveira, Geraldo TR, and Shoou-Yuh Chang, 2010. Fluorescent lamp recycling initiatives in the United States and a recycling proposal based on extended producer responsibility and product stewardship concepts. Waste Management & Research, 29(6):656-668. Available at: http://journals.sagepub.com/doi/pdf/10.1177/0734242X10383744, last accessed May 2018.

NEWMOA. 2015a. IMERC Fact Sheet Mercury Use in Dental Amalgam. Available at: http://www.newmoa.org/prevention/mercury/imerc/factsheets/dental_amalgam_2015.pdf, last accessed August 2018.

National Institute of Dental and Craniofacial Research. 2013. Dental Caries (Tooth Decay). Available at: https://www.nidcr.nih.gov/research/data-statistics/dental-caries, last accessed May 2018.

American Dental Association (ADA). 1998. Dental Amalgam: Update on Safety Concerns. Journal of the American Dental Association, 129:494:503. Available at: https://www.ada.org/~/media/ADA/Member%20Center/FIles/safety.pdf, last accessed May 2018.

Food and Drug Administration. 2017. About Dental Amalgam Fillings. https://www.fda.gov/MedicalDevices/ProductsandMedicalProcedures/DentalProducts/DentalAmalgam/ucm171094.htm, last accessed August 2018.

Natural Resources Defense Council, Product Stewardship Institute, Clean Water Fund, and Mercury Policy Project. 2013. Turning Up the Heat II: Exposing the continued failures of the manufacturers’ thermostat recycling program.  Available at: http://www.cleanwateraction.org/files/TurningUpTheHeatII.pdf, last accessed August 2018.

NEWMOA. 2015b. IMERC Fact Sheet Mercury Use in Measuring Devices. Available at: http://www.newmoa.org/prevention/mercury/imerc/factsheets/measuring_devices_2015.pdf, last accessed August 2018.

King, S. et al. May 2008. Reducing Mercury in the Northeast United States. EM Magazine. Air and Waste Management Association. Available at: http://www.nescaum.org/documents/reducing-mercury-in-the-northeast-united-states/ne-mercury-progress-em-200805.pdf, last accessed August 2018.

US Census Bureau, County Business Patterns 2016. Available at: https://www.census.gov/data/datasets/2016/econ/cbp/2016-cbp.html last accessed May 2018.

US EPA. 2007. Mercury Switches in Motor Vehicles; Significant New Use Rule. Available at:  https://www.federalregister.gov/documents/2007/10/05/E7-19705/mercury-switches-in-motor-vehicles-significant-new-use-rule, last accessed May 2018.

Griffith, C., et al. 2001. Toxics in Vehicles: Mercury. A Report by Ecology Center, Great Lakes United, and University of Tennessee Center for Clean Products and Clean Technologies. Available at: http://infohouse.p2ric.org/ref/19/18304.pdf, last accessed May 2018.

Goonan, T.G. 2006. Mercury Flow Through the Mercury-Containing Lamp Sector of the Economy of the United States. US Geological Survey Scientific Investigations Report 2006-5264. Available at: https://pubs.usgs.gov/sir/2006/5264/sir20065264.pdf, last accessed May 2018.

Li, Y. and L. Jin. 2011. Environmental Release of Mercury from Broken Compact Fluorescent Lamps. Environmental Engineering Science, 28:687-691. https://www.sustainlv.org/wp-content/uploads/Mercury-from-Broken-CFLs.pdf, last accessed May 2018.

Arendt, J. and J.F. Katers. 2013. Compact fluorescent lighting in Wisconsin: elevated atmospheric emission and landfill deposition post-EISA implementation. Waste Management and Research, 0:1-12. Available at https://www.researchgate.net/publication/236601998_Compact_fluorescent_lighting_in_Wisconsin_Elevated_atmospheric_emission_and_landfill_deposition_post-EISA_implementation, last accessed August 2018.

Singhvi, R, A. Taneja, V. Kansal, C.J. Gasser, and D.J. Kalnicky. 2011. Determination of Total Metallic Mercury in Compact Fluorescent Lamps (CFLs). Environmental Forensics, 12:143-148. Available at: https://www.researchgate.net/publication/233238138_Determination_of_Total_Metallic_Mercury_in_Compact_Fluorescent_Lamps_CFLs, last accessed May 2018.

Aucott, M., M. McLinden, and M. Winka. 2004. Release of Mercury from Broken Fluorescent Bulbs. New Jersey Department of Environmental Protection. Environmental Assessment and Risk Analysis Element, Research Project Summary. Available at:  http://www.state.nj.us/dep/dsr/research/mercury-bulbs.pdf, last accessed May 2018.

National Electrical Manufacturers Association (NEMA). 2005. Fluorescent and other Mercury-Containing Lamps and the Environment. Available at: http://www.nema.org/Policy/Environmental-Stewardship/Lamps/Documents/Lamp%20Brochure.pdf, last accessed May 2018.

Cain, A., S. Disch, C. Twaroski, J. Reindl, and C.R. Case. 2007. Substance Flow Analysis of Mercury Intentionally Used in Products in the United States. Journal of Industrial Ecology, 11: 61-75. Available at: http://citeseerx.ist.psu.edu/viewdoc/download?doi=10.1.1.564.4140&rep=rep1&type=pdf, last accessed May 2018.

US EPA. 1997. Mercury Study Report to Congress, Volume II: An Inventory of Anthropogenic Mercury Emissions in the United States. Available at: https://nepis.epa.gov/Exe/ZyPURL.cgi?Dockey=2000EI7C.txt, last accessed May 2018.

Richardson, G.M., R. Wilson, D. Allard, C. Purtill, S. Douma, and J. Graviere. 2011. Mercury exposure and risks from dental amalgam in the US population, post-2000. Science of the Total Environment, 409:4257-4268. Available at: https://www.researchgate.net/publication/51514541_Mercury_exposure_and_risks_from_dental_amalgam_in_the_US_population_post-2000, last accessed May 2018.