pUBLICLY oWNED tREATMENT wORKS

Source Category Description

Publicly Owned Treatment Works (POTWs) include treatment works that are owned by a state, municipality, city, town, special sewer district, or other publicly owned and financed entity as opposed to a privately (industrial) owned treatment facility. The definition includes intercepting sewers, outfall sewers, sewage collection systems, pumping, power, and other equipment. The wastewater treated by these POTWs is generated by industrial, commercial, and domestic sources.1 In 2017, POTWs in the US, Puerto Rico, and US Virgin Islands resulted in more than 4,300 tons of VOC emissions and 860 tons of NH3 emissions.

For this source category, the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2630020000 | Waste Disposal, Treatment, and Recovery | Wastewater Treatment | Public Owned | Total Processed |

Overview of Calculations

The calculations for estimating the emissions from POTWs involve multiplying the wastewater flow rate by emissions factors for VOCs, NH3, and 53 HAPs. Sources of data and calculations for the wastewater flow rate are discussed in section C. The process of allocating activity data to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from POTWs is discussed in section G. Correcting for point sources is discussed in section H.

Activity Data

The activity data for this source category is the wastewater flow rate. The EPA Clean Watersheds Needs Survey provides flow rate by facility and estimates the national POTW flow rate in 2012 for all facilities as 32,822 million gallons per day (MMGD).2 The nationwide flow rate includes Puerto Rico and the US Virgin Islands. To estimate flow rates in 2017, facility-level daily flow rates in 2012 are multiplied by the ratio of 2017 to 2012 population in the county where the facility resides.3 County-level annual 2017 wastewater flow rates are calculated by summing the daily flow rates for all POTWs within the county and multiplying by 365 days in a year.

| FRc,2017=f=1n FRf,2012×365×P c,2017Pc, 2012 |  |
| --- | --- |

Where:

FRc,2017	=	The annual wastewater flow rate of county c in 2017

FRf,2012	=	The daily wastewater flow rate at facility f in 2012

Pc,2017	=	Total population of county c in 2017

Pc,2012	=	Total population of county c in 2012

Allocation Procedure

For a given county, county-level wastewater flow rates are calculated by summing the flow rates for all POTWs within the county.

Emissions Factors

Emissions factors for POTWs are reported in Table 1. The ammonia emissions factor was obtained from an EPA report5 and the VOC emissions factor was based on a TriTAC study.6 Emissions factors for HAPs were derived using 1996 area source emissions estimates that were provided by Bob Lucas7 and the 1996 nationwide flow rate.8 These HAP emissions factors were then multiplied by the 2008 to 2002 VOC emissions factor ratio (0.85/9.9) to obtain the final HAP emissions factors applied in the 2017 inventory.

Table 1. Emissions Factors for Publicly Owned Treatment Works (SCC 2630020000)

| Pollutant | Pollutant Codes | Emissions Factor (lbs./MMGAL) |  | Emissions Factor Reference(s) |
| --- | --- | --- | --- | --- |
| 1,1,2,2-Tetrachloroethane | 79345 | 1.75E-06 |  | 7, 8 |
| 1,1,2-Trichloroethane | 79005 | 1.17E-06 |  | 7, 8 |
| 1,2,4-Trichlorobenzene | 120821 | 8.67E-05 |  | 7, 8 |
| 1,3-Butadiene | 106990 | 2.51E-05 |  | 7, 8 |
| 1,4-Dichlorobenzene | 106467 | 2.16E-04 |  | 7, 8 |
| 1-Chloro-2,3-Epoxypropane | 106898 | 4.52E-06 |  | 7, 8 |
| 2,4-Dinitrotoluene | 121142 | 4.81E-05 |  | 7, 8 |
| 2-Nitropropane | 79469 | 2.92E-07 |  | 7, 8 |
| Acetaldehyde | 75070 | 3.10E-04 |  | 7, 8 |
| Acetonitrile | 75058 | 3.45E-04 |  | 7, 8 |
| Acrolein | 107028 | 3.84E-04 |  | 7, 8 |
| Acrylonitrile | 107131 | 3.86E-04 |  | 7, 8 |
| Allyl Chloride | 107051 | 1.94E-05 |  | 7, 8 |
| Ammonia | NH3 | 1.69E-01 |  | 5 |
| Benzene | 71432 | 6.73E-03 |  | 7, 8 |
| Benzyl Chloride | 100447 | 8.17E-06 |  | 7, 8 |
| Biphenyl | 92524 | 7.52E-05 |  | 7, 8 |
| Carbon Disulfide | 75150 | 4.32E-03 |  | 7, 8 |
| Carbon Tetrachloride | 56235 | 1.12E-03 |  | 7, 8 |
| Chlorobenzene | 108907 | 4.83E-04 |  | 7, 8 |
| Chloroform | 67663 | 6.44E-03 |  | 7, 8 |
| Chloroprene | 126998 | 2.38E-05 |  | 7, 8 |
| Cresols/Cresylic Acid (Isomers and Mixture) | 1319773 | 1.61E-06 |  | 7, 8 |
| Dimethyl Sulfate | 77781 | 1.31E-06 |  | 7, 8 |
| Ethyl Acrylate | 140885 | 1.75E-06 |  | 7, 8 |
| Ethyl Benzene | 100414 | 7.66E-03 |  | 7, 8 |
| Ethylene Oxide | 75218 | 2.22E-04 |  | 7, 8 |
| Formaldehyde | 50000 | 1.97E-05 |  | 7, 8 |
| Glycol Ethers | 171 | 1.15E-02 |  | 7, 8 |
| Hexachlorobutadiene | 87683 | 7.29E-07 |  | 7, 8 |
| Hexachlorocyclopentadiene | 77474 | 5.83E-07 |  | 7, 8 |
| Methanol | 67561 | 1.14E-02 |  | 7, 8 |
| Methyl Chloroform | 71556 | 5.63E-04 |  | 7, 8 |
| Methyl Isobutyl Ketone | 108101 | 2.69E-03 |  | 7, 8 |
| Methyl Methacrylate | 80626 | 3.11E-04 |  | 7, 8 |
| Methyl Tert-Butyl Ether | 1634044 | 6.37E-05 |  | 7, 8 |
| Methylene Chloride | 75092 | 9.10E-03 |  | 7, 8 |
| N,N-Dimethylaniline | 121697 | 3.22E-04 |  | 7, 8 |
| Naphthalene | 91203 | 1.31E-03 |  | 7, 8 |
| Nitrobenzene | 98953 | 6.56E-06 |  | 7, 8 |
| O-Toluidine | 95534 | 1.75E-06 |  | 7, 8 |
| P-Dioxane | 123911 | 1.79E-05 |  | 7, 8 |
| Propionaldehyde | 123386 | 3.50E-06 |  | 7, 8 |
| Propylene Dichloride | 78875 | 1.15E-05 |  | 7, 8 |
| Propylene Oxide | 75569 | 7.32E-04 |  | 7, 8 |
| Styrene | 100425 | 2.73E-03 |  | 7, 8 |
| Tetrachloroethylene | 127184 | 4.27E-03 |  | 7, 8 |
| Toluene | 108883 | 1.23E-02 |  | 7, 8 |
| Trichloroethylene | 79016 | 3.06E-04 |  | 7, 8 |
| Vinyl Acetate | 108054 | 7.66E-05 |  | 7, 8 |
| Vinyl Chloride | 75014 | 6.71E-06 |  | 7, 8 |
| Vinylidene Chloride | 75354 | 4.23E-04 |  | 7, 8 |
| Volatile Organic Compounds | VOC | 8.50E-01 |  | 6 |
| Xylenes (Mixture of O, M, And P Isomers) | 1330207 | 5.98E-02 |  | 7, 8 |

Controls

There are no controls assumed for this category.

Emissions

Emissions are estimated by multiplying an emissions factor by the county flow rate. A conversion factor was used to convert pounds to tons.

| Ep,c,2017= FRc,2017×EFp×1 ton2000 lbs. |  |
| --- | --- |

Where:

Ep,c,2017 	=	Nonpoint emissions in 2017 of pollutant p in county c, in tons

FRc,2017	=	Flow rate in 2017 in county c, in MMGY

EFp 	= 	Emissions factor for pollutant p, in lbs. per MMGAL

Point Source Subtraction

The county-level flow rates include all facilities reported as POTWs in the EPA Clean Watersheds Needs Survey. In some cases, SLT agencies might include facilities under their point source inventory reporting. In these cases, SLT agencies have two options for submitting state-level point source data to EPA for point source subtraction:

- Option A: County-level flow rates associated with POTWs reported as point sources; or

- Option B: County-level emissions of VOC and NH3 for POTWs reported as point sources.

Sample Calculations

Table 2 lists sample calculations to determine the benzene emissions for nonpoint source POTWs for Autauga County, Alabama.

| Eq. # | Equation | Values for Autauga County, AL | Result |
| --- | --- | --- | --- |
| 1 | FRc,2017=f=1n FRf,2012×365×P c,2017Pc, 2012 | 2.866 MMGD×365 days×55,504 people54,927 people | 1,057.07 MMGY |
| 2 | Ep,c,2017= FRc,2017×EFp×1 ton2000 lbs. | 1,057.07 MMGY×0.00673 lb/MMG×1 ton2000 lbs. | 0.003557 tons benzene per year |

Table 2. Sample calculations for benzene emissions for nonpoint source POTWs for Autauga County, AL.

Changes from 2014 Methodology

County-level flow rates in 2017 were determined by summing facility-level data to the county-level rather than allocating the national flow rate to counties based on the ratio of county to US population.

Puerto Rico and U.S. Virgin Islands Emissions Calculations

Emissions from Puerto Rico are calculated using the same method described above. For the U.S. Virgin Islands, emissions are not multiplied by the ratio of 2017 to 2012 population since 2017 Census Data does not exist for the U.S. Virgin Islands.

Instructions for Submitting Point Activity or Emissions Data to Input Template

The POTWs Input Template includes a template for submitting point source activity or emissions data. The template includes two options for submitting point source data: either submitting point source activity (flow rates in millions of gallons associated with point-source POTWs) or emissions (VOC and NH3). SLT agencies should submit data only for one option. The point data, whether it is activity or emissions, should be submitted at the county level.

References

1 U.S. EPA,  64FR57572, National Emission Standards for Publicly Owned Treatment Works, Final Rule,  40 CFR Part 63,  26 October 1999. https://www.gpo.gov/fdsys/pkg/FR-1999-10-26/pdf/99-27799.pdf

2 U.S. Environmental Protection Agency, Clean Watersheds Needs Survey 2012 Data and Reports, Detail Report, at https://ofmpub.epa.gov/apex/cwns2012/f?p=CWNS2012:1::::::

3 U.S. Census Bureau. Total Population, American Community Survey, https://data.census.gov/cedsci/table?hidePreview=false&tid=ACSDT1Y2017.B01003&vintage=2018

4 U.S. Environmental Protection Agency, “Wastewater Flow Projections for POTWs and Privately and Federally Owned Treatment Works in 2000, 2005, and 2010,” Table A-8 in Biosolids Generation, Use, and Disposal in the United States, EPA530-R-99-009, September 1999. https://nepis.epa.gov/Exe/ZyNET.exe/10001529.TXT?ZyActionD=ZyDocument&Client=EPA&Index=1995+Thru+1999&Docs=&Query=&Time=&EndTime=&SearchMethod=1&TocRestrict=n&Toc=&TocEntry=&QField=&QFieldYear=&QFieldMonth=&QFieldDay=&IntQFieldOp=0&ExtQFieldOp=0&XmlQuery=&File=D%3A%5Czyfiles%5CIndex%20Data%5C95thru99%5CTxt%5C00000014%5C10001529.txt&User=ANONYMOUS&Password=anonymous&SortMethod=h%7C-&MaximumDocuments=1&FuzzyDegree=0&ImageQuality=r75g8/r75g8/x150y150g16/i425&Display=hpfr&DefSeekPage=x&SearchBack=ZyActionL&Back=ZyActionS&BackDesc=Results%20page&MaximumPages=1&ZyEntry=1&SeekPage=x&ZyPURL

5 Stephen M. Roe, Melissa D. Spivey, Holly C. Lindquist, Kirstin B. Thesing, and Randy P. Strait, E.H. Pechan & Associates, Inc., Estimating Ammonia Emissions from Anthropogenic Nonagricultural Sources – Draft Final Report, prepared for U.S. Environmental Protection Agency, Emission Inventory Improvement Program, April 2004. https://www.epa.gov/sites/production/files/2015-08/documents/eiip_areasourcesnh3.pdf

6 Prakasam Tata, Jay Witherspoon, Cecil Lue-Hing (eds.), VOC Emissions from Wastewater Treatment Plants: Characterization, Control, and Compliance, Lewis Publishers, 2003, p. 261.

7 Memorandum from Bob Lucas, U.S Environmental Protection Agency to Greg Nizich, U.S. Environmental Protection Agency,  “Review of Baseline Emissions Inventory,”  16 October 1998.

8 U.S. Environmental Protection Agency, “Facilities Database (Needs Survey) - Frequently Asked Questions,” at https://permanent.access.gpo.gov/websites/epagov/www.epa.gov/OWM/mtb/cwns/1996rtc/faqwfd.htm, accessed 30 April 2019.