# Construction Dust - RESIDENTIAL CONSTRUCTION

# Source Category Description

Emissions from residential construction activity are a function of the acreage disturbed and volume of soil excavated for residential construction. PM emissions are released through a number of residential construction related activities including movement of equipment on unpaved surfaces, excavation and earthmoving activities, material transfer operations, material alterations including drilling, cutting, blasting, and surface cleaning, tracking of dirt to nearby paved surfaces that are re-suspended by traffic, land clearing, and wind erosion of soil exposed from construction.8  Residential construction activity is developed from data obtained from the U.S. Department of Commerce (DOC)’s Bureau of the Census. In 2014, the emissions were 57,961 tons for PM10-PRI and PM10-FIL and 5,796 tons for PM25-PRI and PM25-FIL.

For this source category, the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2311010000 | Industrial Processes | Construction: SIC 15 - 17 | Residential | Total |

# Overview of Calculations

The calculations for estimating emissions from residential construction involve three main steps: 1) determining the number of housing starts in each county; 2) determining the amount of soil disturbed for 1-unit homes with basements; and 3) determining the amount of surface soil disturbed for all building types. The amount of soil disturbed in each county is multiplied by emissions factors to determine the emission of PM10 and PM25 in each county. Data on housing starts in each region is from the U.S. Census Bureau, and it is distributed to the counties based on county-level data on building permits. Sources of data and calculations for the activity data are discussed in more detail in section C. The process of allocating building permit data to the county level is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from residential construction is discussed in section G.

# Activity Data

There are two activity calculations performed for this SCC, acres of surface soil disturbed and volume of soil removed for basements.

Determine the Number of Housing Starts in Each County

The US Census Bureau has 2017 data for New Privately Owned Housing Units Started by Purpose and Design which provides data on housing starts based on the groupings of 1 unit, 2-4 units, and 5 or more units. Regional-level results are also provided for quarterly totals and 1 unit structures (Table 1). In order to breakdown the 2 to 4 unit category, data from a consultation with the Census Bureau in 2002 are used; approximately 1/3 of the housing starts are for 2 unit structures, and 2/3 are for 3 and 4 unit structures.

The 2017 US Census Bureau New Privately Owned Housing Units Started by Purpose and Design1 data for 2-4 units are distributed to two categories, 2 and 3-4 units, based on a ratio for 2 and 3-4 units calculated from the 2000 US Census Bureau National Housing Starts data, for each quarter in 2017. Note that 2000 is the last full year when Census housing starts data are available separately for 2-unit and 3-4 unit homes. Table 2 shows a breakdown of the 2 unit and 3-4 unit structures based on the following calculation.

| SQ,n=UnUt×SQ,2-4 |  |
| --- | --- |

Where:

SQ,n	= 	Housing starts, by quarter, Q, and number of units, n (2 units or 3-4 units), in thousand units

Un	= 	Number of housing starts by number of units, n, from the 2000 National Housing Starts data, in thousand housing starts

Ut	= 	Total number of housing starts for both 2 units and 3-4 units from the 2000 National Housing Starts data, in thousand housing starts

SQ,2-4	=	Number of 2-4 units by quarter, Q, from the 2017 New Privately Owned Housing Units Started by Purpose and Design data, in thousand units

Table 1. Housing Start Data for 2017

| Quarter | Total | Structure |  |  | Region |  |  |  | Regional Starts of Structures with 1 unit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 unit | 2 to 4 units | 5 units or more | NE | MW | S | W | NE | MW | S | W |
| Q1-14 | 206.0 | 134.0 | 2.0 | 70.0 | 23.0 | 21.0 | 113.0 | 49.0 | 9.0 | 14.0 | 79.0 | 32.0 |
| Q2-14 | 275.0 | 183.0 | 3.0 | 89.0 | 28.0 | 53.0 | 130.0 | 62.0 | 15.0 | 34.0 | 91.0 | 42.0 |
| Q3-14 | 282.0 | 178.0 | 4.0 | 100.0 | 32.0 | 49.0 | 134.0 | 65.0 | 14.0 | 32.0 | 92.0 | 39.0 |
| Q4-14 | 241.0 | 154.0 | 4.0 | 84.0 | 26.0 | 39.0 | 118.0 | 58.0 | 13.0 | 25.0 | 83.0 | 32.0 |

Source: Reference 1

Table 2. Breakdown of 2 to 4 unit structures

| Quarter | Structure |  |  |
| --- | --- | --- | --- |
|  | 2 to 4 units | 2 units | 3-4 units |
| Q1-14 | 2.0 | 0.74 | 1.26 |
| Q2-14 | 3.0 | 1.11 | 1.89 |
| Q3-14 | 4.0 | 1.47 | 2.53 |
| Q4-14 | 4.0 | 1.47 | 2.53 |

Ratios of the number of 2, 3-4, and 5 or more unit structures are then used to estimate the number of structures of each type in each region. The ratios are calculated by dividing the housing starts by quarter for each unit type by the total housing starts for buildings with 2 or more units.

| rQ,n=SQ,nSQ.t |  |
| --- | --- |

Where:

rQ,n	= 	Ratio of structures with number of units, n, to total number of units by quarter, Q

SQ,n	= 	Housing starts, by quarter, Q, and number of units, n, from distributed calculation in Step 1 for the 2-unit or 3-4 unit categories or directly from the 2017 New Privately Owned Housing Units Started by Purpose and Design data for the 5 units or more category, in thousand housing starts

SQ,t	= 	Housing starts, by quarter, Q, for total number of buildings with 2 or more units, t (excludes 1-unit category), in thousand housing starts

The ratio is then used to distribute the 2017 New Privately Owned Housing Units Started by Purpose and Design regional data for all unit types to the 2, 3-4, or 5 or more unit categories within each Census region – Northeast, Midwest, South, and West.

| AQ,n,rgn=rQ,n×(RSt,rgn-RS1,rgn) |  |
| --- | --- |

Where:

AQ,n,rgn	=	Number of housing units started in quarter Q, by number of units, n, and region of the country, rgn, in thousand units

rQ,n	= 	Ratio of structures with number of units, n, to total number of units by quarter, Q

RSt,rgn	=	Total regional starts from the 2017 New Privately Owned Housing Units Started by Purpose and

Design data, in thousand housing starts

RS1,rgn	=	Regional starts of structures with 1 unit from the 2017 New Privately Owned Housing Units Started by Purpose and Design data, in thousand housing starts

Data from the Census report New Privately Owned Housing Units Authorized Unadjusted Units is used to calculate a conversion factor to determine the ratio of structures to units in the 5 or more unit category. The conversion factor is calculated by dividing the total number of units in structures with 5 or more units by region2 by the total number of buildings with 5 or more units by region.3

| CF5,rgn= U5,rgnB5,rgn |  |
| --- | --- |

Where:

CF5,rgn	=	Ratio of 5 units or more to the number of buildings with 5 units or more by region, rgn

U5,rgn	=	Total number of 5 or more units by region, rgn

B5,rgn	=	Total number of buildings with 5 or more units by region, rgn

Structures started by category are then calculated at a regional level by summing the number of housing unit starts across all four quarters and dividing by the number of units in each building type. For the 3-4 unit type, the number of units per building is 3.5. The value is multiplied by 1,000 because the Census data are in units of thousand building starts.

For buildings with 1, 2, or 3-4 units:

| Bn,rgn=(Q1Q4AQ.n.rgn)×1,000n |  |
| --- | --- |

Where:

Bn,rgn	=	Number of building starts by the unit number category, n, and by region, rgn

AQ,n,rgn	=	Number of housing units started in quarter Q, by number of units, n, and region of the country, rgn, in thousand units

n	=	Number of units per building

For buildings with 5 or more units:

| Bn,rgn=(Q1Q4AQ.n.rgn)×1,000CF5 |  |
| --- | --- |

Where:

Bn,rgn	=	Number of building starts by the unit number category, n, and by region, rgn

AQ,n,rgn	=	Number of housing units started in quarter Q, by number of units, n, and region of the country, rgn, in thousand units

CF5	=	Ratio of 5 units or more to the number of buildings with 5 units or more

Annual county-level building permit data were from the US Census Bureau for 2017. The 2017 County Level Residential Building Permit dataset has 2017 data to allocate regional housing starts to the county level. This results in county-level housing starts by number of units.

The number of building permits for each unit number category by region is calculated by summing the county-level Census data to the Census region level.

| BPn,rgn=BPn,c |  |
| --- | --- |

Where:

BPn,rgn	=	Number of building permits by the unit number category, n, and by region, rgn

BPn,c	=	Number of building permits by the unit number category, n, and by county, c

The ratio of the number of building permits by county to the total number of building permits by region in which the county is located, for each unit number category, is then calculated.

| RBP,c=BPn,cBPn,rgn |  |
| --- | --- |

Where:

RBP,c	=	Ratio of building permits, BP, to total regional building permits in county c

BPn,c	=	Number of building permits by the unit number category, n, and by county, c

BPn,rgn	=	Number of building permits by the unit number category, n, and by region, rgn

The final number of building starts for each unit type category is then calculated at the county-level by multiplying the number of structures started at the regional level and the building permit ratio.

| Bn,c=Bn,rgn×RBP,c |  |
| --- | --- |

Where:

Bn,c	=	Number of building starts by the unit number category, n, and by county, c

Bn,rgn	=	Number of building starts by the unit number category, n, and by region, rgn

RBP,c	=	Ratio of building permits, BP, to total regional building permits in county c

Determine Amount of Soil Removed for Basements

To calculate basement soil removal, the 2017 Characteristics of New Single-Family Houses Completed, Foundation table is used to estimate the percentage of 1-unit structures that have a basement at the regional level. The data indicate whether the structure has a full/partial basement, slab or other type, or crawl space. However, only structures with full/partial basements are used in this calculation.

| BMrgn=BMfp,rgnBMt,rgn |  |
| --- | --- |

Where:

BMrgn	=	Fraction of basements for buildings in the region

BMfp,rgn	=	Number of full or partial basements, fp, by region, rgn

BMt,rgn	=	Total number of houses regardless of basement type (full/partial, slab/other, crawl space by region, rgn

To estimate the number of building starts with and without basements in each county, the county level estimate of the number of 1-unit starts (from equation 9) is multiplied by the percent of 1-unit houses in the region that have a basement.

| Bc,BM=Bn.c×BMrgn |  |
| --- | --- |
| Bc,nBM=Bn.c×(1-BMrgn) | (11a) |

Where:

Bc,BM	=	Number of building starts by county, c, with a basement, BM

Bc,nBM	=	Number of building starts by county, c, without a basement, BM

Bn,c	=	Number of building starts by the unit number category, n, and by county, c

BMrgn	=	Fraction of basements for buildings in the region

Basement volume is calculated by assuming a house with a 2000 square foot footprint has a basement dug to a depth of 8 feet (making 16,000 ft3 per basement). An additional 10% is added for peripheral dirt bringing the total to 17,600 ft3 (651.85 yd3) per basement.

Determine Amount of Soil Disturbed by Unit Type

The number of acres of soil disturbed by the construction of residential buildings is calculated for apartment buildings, buildings with 2 units, and buildings with 1 unit. Table 3 below shows the assumptions used for the surface area disturbed for each unit type. Buildings with unit types of 3-4 and 5 or more are grouped together as apartments in this step.

Table 3. Surface Soil removed per unit type

| Structure Type | Acres disturbed |
| --- | --- |
| 1-Unit | 1/4 acre per structure |
| 2-Unit | 1/3 acre per structure |
| Apartment | 1/2 acre per structure |

For apartment buildings (sum of 3-4 and 5 or more units) and buildings with 2 units:

| Sn,c=Bn.c×an |  |
| --- | --- |

Where:

Sn,c	=	Surface soil disturbed by building construction by county, c, and unit type category, n, in acres

Bn,c	=	Number of building starts by the unit type category, n, and by county, c

an	=	Acres of surface soil disturbed by each unit type category, n. See Table 3 for values for each type.

For buildings with 1 unit, with or without a basement:

| Sn,c=Bc,BM×an |  |
| --- | --- |

Where:

Sn,c	=	Surface soil disturbed by building construction by county, c, and unit type category, n, in acres

Bc,BM	=	Number of buildings by county, c, with or without a basement, BM

an	=	Acres of surface soil disturbed by each unit type category, n. See Table 3 for values for each type.

# Allocation Procedure

Annual county building permit data were purchased from the US Census Bureau for 2017.4 The 2014 County Level Residential Building Permit dataset is used to allocate regional housing starts to the county level. See Section C for more information.

# Emissions Factors

Initial PM10 emissions from construction of single family, 2-unit, and apartments structures are calculated using the emissions factors given in Table 4. These emissions factors describe average “unit operations,” such as “loading and unloading of earth and aggregate materials, land clearing and general vehicle traffic.” They therefore take into account the entire duration of construction, and not simply the duration of active excavation. The duration of construction activity for houses is assumed to be 6 months and the duration of construction for apartments is assumed to be 12 months.

Table 4. Emissions Factors for Residential Construction

| Type of Structure | Emissions Factor | Duration of Construction |
| --- | --- | --- |
| Apartments | 0.11 tons PM10/acre-month | 12 months |
| 2-Unit Structures | 0.032 tons PM10/acre-month | 6 months |
| 1-unit Structures with Basements | 0.011 tons PM10/acre-month | 6 months |
|  | 0.059 tons PM10/1000 cubic yards |  |
| 1-Unit Structures w/o Basements | 0.032 tons PM10/acre-month | 6 months |

Source: Reference 5

To account for the soil moisture level, the PM10 emissions are weighted using the 30-year average precipitation-evaporation (PE) values from Thornthwaite’s PE Index. Average precipitation evaporation values for each state are estimated based on PE values for specific climatic divisions within a state. The average PE value for the test sites from which the PM10 emissions factor was developed is 24.6 Equation 14 is used to adjust the county-level emissions factor based on this PE value.

To account for the silt content, the PM10-PRI emissions are weighted using average silt content for each county. EPA used the National Cooperative Soil Survey Microsoft Access Soil Characterization Database to develop county-level, average silt content values for surface soil. The U.S. Department of Agriculture and the National Cooperative Soil Survey define silt content of surface soil as the percentage of particles (mass basis) of diameter smaller than 50 micrometers (µm) found in the surface soil. This database contains the most commonly requested data from the National Cooperative Soil Survey Laboratories including data from the Kellogg Soil Survey Laboratory and cooperating universities. The average silt content for the test sites from which the PM10 emissions factor was developed is 9%.6 Equation 7 is used to adjust the county-level emissions factor based on this silt content value.

| AFPM10=24PE×s9% |  |
| --- | --- |

Where:

AFPM10	=	PM10-PRI adjustment factor

PE	=	precipitation-evaporation value for each State

s	=	% dry silt content, by county, in soil for area being inventoried

This adjustment factor is used to adjust the PM10-PRI emissions factor for each unit type category – apartment, 2-unit, 1-unit with basement, and 1-unit without basement.

| EFp,n,c= AFPM10×Dn×EForig |  |
| --- | --- |

Where:

EFp,n,c	=	Adjusted county-level, c, PM10-PRI emissions factor, p, for each unit type category, n, in tons/acre

AFPM10	=	PM10-PRI adjustment factor

Dn	= 	Duration of construction by unit type category, n, in months. See Table 4 for duration values.

EForig	=	Original unadjusted PM10 emissions factor, in tons/acre. See Table 4 for original emissions factors.

# Controls

There are no controls assumed for this category.

# Emissions

The PM10-PRI emissions are calculated by taking the sum of the surface soil disturbed by county and unit type category and multiplying it by the corresponding adjusted PM10-PRI emissions factor. Once PM10-PRI adjustments have been made, PM25-PRI emissions are estimated by applying a particle size multiplier of 0.10 to PM10-PRI emissions. Primary PM emissions are equal to filterable emissions since there are no condensible emissions from residential construction.

The PM10-PRI emissions are calculated at the county-level by multiplying the surface soil disturbed from construction for each unit type by the corresponding emissions factor for that unit type, and then summed across unit types.

| EPM10,c= n=1NSn,c×EFPM10,n,c |  |
| --- | --- |

Where:

EPM10,c	=	Total PM10-PRI emissions in county c, in tons

Sn,c	=	Surface soil disturbed by building construction by county, c, and unit type category, n

EFp,n,c	=	Adjusted county-level, c, PM10 emissions factor, p, for each unit type category, n, in tons/acre

The PM25-PRI emissions are calculated based on the assumption that they are 10% of the PM10-PRI emissions.

| EPM2.5,c= EPM10,c×0.1 |  |
| --- | --- |

Where:

EPM2.5,c	=	Total county-level, c, PM25-PRI emissions

EPM10,c	=	Total county-level, c, PM10-PRI emissions

0.1	=	Particle size multiplier

# Point Source Subtraction

There are no point source-specific SCCs for residential construction; therefore point source subtraction is not performed for this category.

# Sample Calculations

Table 5 shows sample calculations for PM10-PRI and PM25-PRI emissions from residential construction for a 2-unit structure in Suffolk County, Massachusetts. The first 3 equations use the first quarter (Q1) of 2014 for 2-unit structures as an example. However these calculations would need to be repeated to calculate values for all 4 quarters for all 3 unit sizes. Note that structures with 5 or more units and structures with 1 unit with or without a basement have additional steps not shown in the sample calculations here.

Table 5. Sample calculations for PM-10 PRI and PM25-PRI emissions from residential construction of 2-unit structures in Suffolk County, MA.

| Eq. # | Equation | Values for Suffolk County, MA | Result |
| --- | --- | --- | --- |
| 1 | SQ,n=UnUt×SQ,2-4 | 14 two unit housing starts in 200238 total housing starts in 2002 ×2 two to four unit housing starts in Q1 2017 | 0.74 thousand housing starts for 2-unit structures in Q1 2014, nationally |
| 2 | rQ,n=SQ,nSQ.t | 0.74 two unit housing starts72 two or more unit housing starts | 0.01 ratio of buildings with 2 units to all 2 or more unit housing starts for Q1 2014, nationally |
| 3 | AQ,n,rgn=rQ,n×(RSt-RS1) | 0.01×(23 total Q1 housing starts in Northeast-9 one unit housing starts in Northeast ) | 0.14 thousand housing starts for 2-unit structures for Q1 2014 in the Northeast |
| 4 | CF5= U5,rgnB5,r | N/A | Equation is for 5 or more unit buildings; example is for 2-unit buildings |
| 5 | Bn,rgn=(Q1Q4AQ.n.rgn)×1,000n | 0.772 two unit structures ×1,0002 units per building | 386 2-unit structures constructed in the Northeast |
| 6 | Bn,rgn=(Q1Q4AQ.n.rgn)×1,000CF5 | N/A | Equation is for 5 or more unit buildings; example is for 2-unit buildings |
| 7 | BPn,rgn=BPn,c | Northeast two unit building permits | 1,545 2-unit structure building permits in the Northeast |
| 8 | RBP,c=BPn,cBPn,rgn | 49 Suffolk county building permits1,545 Northeast building permits | 0.03172 ratio of county-level building permits to regional-level building permits in Suffolk County, MA |
| 9 | Bn,c=Bn,rgn×RBP,c | 386×0.03172 | 12.25 total 2-unit structure building starts for Suffolk County, MA |
| 10 | BMrgn=BMfp,rgnBMt,rgn | N/A | Equation is for 1-unit buildings; example is for 2-unit buildings |
| 11 | Bc,BM=Bn.c×BMrgn | N/A | Equation is for 1-unit buildings; example is for 2-unit buildings |
| 12 | Sn,c=Bn.c×an | 12.25 two unit structures ×0.33 acres per structure | 4.08 acres surface soil disturbed by 2-unit structures in Suffolk County, MA |
| 13 | Sn,c=Bc,BM×an | N/A | Equation is for 1-unit buildings; example is for 2-unit buildings |
| 14 | AFPM10=24PE×s9% | 24119.7 PE value for Massachusetts×27.07% silt content9% | 0.603 PM10-PRI adjustment factor for 2-unit structures in Suffolk County, MA |
| 15 | EFp,n,c= AFPM10×Dm,n×EForig | 0.603×6 months×0.032 tons per acre | 0.1158 tons/acre PM10-PRI emissions factor for 2-unit structures in Suffolk County, MA |
| 16 | EPM10,c= Sn,c×EFp,n,c | 4.08 acres×0.1158 tons per acre | 0.47 tons PM10-PRI emissions for 2-unit structures in Suffolk County, MA |
| 17 | EPM2.5,c= EPM10,c×0.1 | 0.47 tons ×0.1 | 0.047 tons PM25-PRI emissions for 2-unit structures in Suffolk County, MA |

# Changes from 2014 Methodology

There are no significant changes from the methodology used to calculate the 2014 v2 NEI.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Since insufficient data exist to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emissions factor. For each Puerto Rico and US Virgin Island county, the tons per capita emissions factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

U.S. Census Bureau, 2018. New Privately Owned Housing Units Started by Purpose and Design https://www.census.gov/construction/nrc/pdf/quarterly_starts_completions.pdf.

U.S. Census Bureau. 2001. Housing Starts. Table 1. https://www.census.gov/prod/2001pubs/c20-0103.pdf.

U.S. Census Bureau, New Privately Owned Housing Units Authorized - Unadjusted Units for Regions, Divisions, and States, Annual 2017, Table 2au. https://www.census.gov/construction/bps/txt/tb2u2017.txt.

U.S. Census Bureau, Annual Housing Units Authorized by Building Permits CO2017A, https://www2.census.gov/econ/bps/County/.

U.S. Census Bureau, Characteristics of New Single Family Houses Completed, Annual 2014, Foundation Table. https://www.census.gov/construction/chars/pdf/foundation.pdf

Midwest Research Institute. 1996. Improvement of Specific Emission Factors (BACM Project No. 1). Prepared for South Coast Air Quality Management District.

U.S. Department of Agriculture, National Cooperative Soil Survey, NCSS Microsoft Access Soil Characterization Database, from http://ncsslabdatamart.sc.egov.usda.gov/.

Cowherd, C. J. Donaldson, R. Hegarty, and D. Ono. 2006. Proposed Revisions to Fine Fraction Ratios Used for AP-42 Fugitive Dust Emission Factors. 15th International Emission Inventory Conference, New Orleans, LA.  http://www.epa.gov/ttn/chief/conference/ei15/session14/cowherd.pdf.

8 Midwest Research Institute. 1999. Estimating Particulate Matter Emissions from Construction Operations. Prepared for Emission Factor and Inventory Group, Office of Air Quality Planning and Standards US EPA.

Note that this definition is different than the U.S. Environmental Protection Agency’s definition that includes all particles (mass basis) of diameter smaller than 75 micrometers.