RESIDENTIAL HEATING – COAL, DISTILLATE OIL, KEROSENE, NATURAL GAS, AND LPG

# Source Category Description

Residential heating includes the combustion of fuel, including coal, distillate oil, kerosene, natural gas, and liquefied propane gas (LPG) to heat homes. Common uses of energy associated with this category include space heating, water heating, and cooking. This category does not include the combustion of wood, which is estimated separately.

The general approach to calculating emissions for these SCCs is to take state-level fuel consumption from the EIA State Energy Data System (SEDS) and allocate it to the county level based on data from the Census Bureau on the number of homes in each county that use each fuel type. County-level fuel consumption is multiplied by emissions factors to calculate emissions.

Note that SEDS no longer includes data on residential coal consumption, as it is assumed to be near zero, and therefore emissions will be nonexistent for residential coal consumption. However, the methodology for estimating emissions from coal has been retained if states have additional data on residential coal consumption that they would like to use.

For this source category, the following SCC is assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2104001000 | Stationary Source Fuel Combustion | Residential | Anthracite Coal | Total: All Combustor Types |
| 2104002000 | Stationary Source Fuel Combustion | Residential | Bituminous/ Subbituminous Coal | Total: All Combustor Types |
| 2104004000 | Stationary Source Fuel Combustion | Residential | Distillate Oil | Total: All Combustor Types |
| 2104011000 | Stationary Source Fuel Combustion | Residential | Kerosene | Total: All Heater Types |
| 2104007000 | Stationary Source Fuel Combustion | Residential | Liquefied Petroleum Gas (LPG) | Total: All Combustor Types |
| 2104006000 | Stationary Source Fuel Combustion | Residential | Natural Gas | Total: All Combustor Types |

# Overview of Calculations

The calculations for estimating emissions from residential heating involve distributing state-level energy consumption data from SEDS to each county based on the proportion of houses in that county that use each fuel type as a primary fuel source. Additional calculations are necessary to distribute coal consumption to anthracite or bituminous coal consumption and to distribute fuel oil consumption to distillate fuel oil and kerosene consumption. County-level consumption of each fuel is multiplied by an emissions factor to estimate emissions of criteria air pollutants (CAPs) and hazardous air pollutants (HAPs). Sources of data and calculations for fuel consumed by each fuel type and SEDS data on anthracite and bituminous coal ratios are discussed in section C. The process of allocating fuel consumption to the county level based on housing unit data is discussed in section D. Emissions factors are discussed in section E. The estimation of emissions from residential heating is discussed in section G.

# Activity Data

The amount of fuel consumed by residential sector in the United States from SEDS1 is used to estimate emissions for this source category. The relevant fuel codes from SEDS are shown in Table 1.

Table 1. EIA State Energy Data System Fuel Codes.

| Fuel | SEDS Fuel Code |
| --- | --- |
| Coal | CLRCP |
| Distillate fuel oil | DFRCP |
| Kerosene | KSRCP |
| Natural Gas | NGRCP |
| LPG | LGRCP |

The SEDS data do not distinguish between anthracite and bituminous/subbituminous coal consumption estimates. The EIA table “Domestic Distribution of U.S. Coal by Destination State, Consumer, Origin and Method of Transportation” provides state-level residential coal distribution data for 2006 that is used to estimate the fraction of coal consumption that is anthracite and bituminous/subbituminous. The amount of anthracite distributed to each state and the total coal delivered to each state is used to estimate the proportion of anthracite and bituminous coal consumption. Table 2 presents the anthracite and bituminous coal ratios for each state.

Table 2. Anthracite and Bituminous Coal Distribution for the Residential and Commercial Sectors

| State | Ratio of Bituminous | Ratio of Anthracite | State | Ratio of Bituminous | Ratio of Anthracite |
| --- | --- | --- | --- | --- | --- |
| Alabama | 1.000 | 0.000 | Montana | 1.000 | 0.000 |
| Alaska | 1.000 | 0.000 | Nebraska | 1.000 | 0.000 |
| Arizona | 0.814 | 0.186 | Nevada | 1.000 | 0.000 |
| Arkansas | 0.814 | 0.186 | New Hampshire | 0.000 | 1.000 |
| California | 1.000 | 0.000 | New Jersey | 0.000 | 1.000 |
| Colorado | 0.996 | 0.004 | New Mexico | 1.000 | 0.000 |
| Connecticut | 0.000 | 1.000 | New York | 0.600 | 0.400 |
| Delaware | 0.814 | 0.186 | North Carolina | 1.000 | 0.000 |
| Dist. Columbia | 1.000 | 0.000 | North Dakota | 1.000 | 0.000 |
| Florida | 0.814 | 0.186 | Ohio | 0.873 | 0.127 |
| Georgia | 1.000 | 0.000 | Oklahoma | 0.917 | 0.083 |
| Hawaii | 1.000 | 0.000 | Oregon | 1.000 | 0.000 |
| Idaho | 0.979 | 0.021 | Pennsylvania | 0.194 | 0.806 |
| Illinois | 0.998 | 0.002 | Rhode Island | 0.000 | 1.000 |
| Indiana | 0.947 | 0.053 | South Carolina | 0.997 | 0.003 |
| Iowa | 0.999 | 0.001 | South Dakota | 1.000 | 0.000 |
| Kansas | 1.000 | 0.000 | Tennessee | 0.994 | 0.006 |
| Kentucky | 0.998 | 0.002 | Texas | 0.814 | 0.186 |
| Louisiana | 1.000 | 0.000 | Utah | 1.000 | 0.000 |
| Maine | 0.000 | 1.000 | Vermont | 0.000 | 1.000 |
| Maryland | 0.929 | 0.071 | Virginia | 0.963 | 0.037 |
| Massachusetts | 0.500 | 0.500 | Washington | 1.000 | 0.000 |
| Michigan | 0.667 | 0.333 | West Virginia | 0.905 | 0.095 |
| Minnesota | 0.997 | 0.003 | Wisconsin | 0.991 | 0.009 |
| Mississippi | 1.000 | 0.000 | Wyoming | 1.000 | 0.000 |
| Missouri | 1.000 | 0.000 |  |  |  |

Source: Reference 3

The SEDS data on residential coal consumption are split into consumption of anthracite and bituminous/subbituminous coal based on the ratios in Table 2.

| FCant/bit,s=FCcoal,s×Rant/bit |  |
| --- | --- |

Where:

FCant/bit,s		= 	anthracite or bituminous coal consumption in state s, in tons

FCcoal,s		= 	total fuel consumption of coal in state s from SEDS, in tons

Rant/bit		= 	ratio of anthracite or bituminous coal to total coal, as found in Table 2

# Allocation Procedure

State-level fuel consumption is allocated to each county using the US Census Bureau’s 5-year estimate Census Detailed Housing Information,2 which includes the number of housing units using a specific type of fuel for their primary fuel source. State fuel consumption is allocated to each county using the ratio of the number of houses using each fuel in each county to the total number of houses using each fuel in the state.

For most fuels, the fuel type in SEDS matches well to the fuel type used in the Census data. However, the Census data report only for total fuel oil, which does not distinguish between distillate fuel oil and kerosene. Therefore, the ratio of distillate fuel oil versus kerosene in the heating fuel oil mix, which is used to determine the fraction of homes in each county that use distillate and those that use kerosene, is calculated.

| Rdfo/ker,s=FCdfo/ker, sFCdfo,s+FCker,s |  |
| --- | --- |

Where:

Rdfo/ker,s	= 	ratio of residential distillate fuel oil or kerosene to total distillate fuel oil and kerosene in state s

Adfo/ker,s	= 	fuel consumption of distillate fuel oil or kerosene in state s from SEDS, in thousand barrels

Then, the ratio of distillate fuel oil or kerosene to total fuel oil is used to determine how many housing units in each county use distillate fuel oil or kerosene.

| HUdfo/ker,c=HUfo,c×Rdfo/ker,s |  |
| --- | --- |

Where:

HUdfo/ker,c	= 	housing units in county c using distillate fuel oil or kerosene as the primary heating fuel

HUfo,c		= 	housing units in county c using any fuel oil as primary heating fuel

To distribute the state-level energy consumption data for all fuel types, the ratio of county-level housing units using each fuel type as primary heating fuel to state-level housing units using that fuel type is calculated. This ratio is used to distribute state-level fuel consumption to the county level. The county-level values for housing units using distillate oil and kerosene as primary fuel are calculated in equations 2 and 3 above.

| Rf,c=HUf,cHUf,s |  |
| --- | --- |

Where:

Rf,c	= 	ratio of homes in county c to homes in state s  that use fuel f as primary heating fuel

HUf,c	= 	housing units in county c using fuel type f as primary heating fuel

HU f,s	= 	housing units in state s using fuel type f as primary heating fuel

The state-level fuel consumption of each fuel type from SEDS is multiplied by the county-level ratio of homes using each fuel type. State-level fuel consumption of anthracite and bituminous/subbituminous coal is calculated in equations 1 and 2 in Section C.

| FCf,c=FCf,s× Rf,c |  |
| --- | --- |

Where:

FCf,c	= 	fuel consumption of fuel type f in county c, in tons, thousand barrels, or thousand cubic feet

FCf,s	= 	fuel consumption of fuel type f in state s, in tons, thousand barrels, or thousand cubic feet, from SEDS

Rf,c	= 	ratio of homes in county c to homes in state s  that use fuel f as primary heating fuel

Fuel consumption of distillate fuel oil is converted from barrels to gallons using a conversion factor of 42 gallons per barrel.

# Emissions Factors

All emissions factors for CAPs, except ammonia, are from AP-42. The ammonia emissions factor is from EPA’s Estimating Ammonia Emissions from Anthropogenic Sources, Draft Final Report. In some cases HAP emissions factors are from a memorandum to EPA called “Baseline Emission Inventory of HAP Emissions from MACT Sources – Interim Final Report”.

For many residential heating fuels, the emissions factors for SO2 and PM species are adjusted using sulfur or ash content data for the fuel at the county level. Note that for coal emissions, this step need only be done if a state supplies data on residential coal consumption, because SEDS currently assumes zero residential coal consumption.

| EFf,s,p=SACf,s×EFunadj,f |  |
| --- | --- |

Where:

EFx,p	=	emissions factor of pollutant p for fuel type f  in state s

SACx	=	sulfur or ash content for fuel type f in state s

EFunadj,f	=	unadjusted emissions factor for fuel type f, from EPA AP-42

Table 5 presents a summary of the emissions factors for all fuel types for residential heating: anthracite coal, bituminous/subbituminous coal, distillate fuel oil, kerosene, LPG, and natural gas.

For coal combustion, the SO2 emission factors are based on the sulfur content of the coal burned, and some of the PM emission factors for anthracite coal require information on the ash content of the coal. State-specific coal sulfur contents for bituminous coal are obtained from the EIA’s Coal Data Browser and applied at the county level. Bituminous sulfur content data can be found in the Coal Consumption and Quality Data Set, filtered to only account for commercial and institutional sources. For anthracite coal, an ash content value of 13.38% and a sulfur content of 0.89% are applied to all counties except those in New Mexico (ash content 16.61%, sulfur content 0.77%), Washington (ash content 12%, sulfur content 0.9%), and Virginia (ash content 13.38%, sulfur content 0.43%). Table 3 shows the coal SO2 and PM emissions factors. Table 4 presents the bituminous coal sulfur content values used for each state.

Table 3. SO2 and PM Emissions Factors for Residential Anthracite and Bituminous Coal Combustion

| Pollutant | Emissions Factor (lb/ton) | Data Source, AP-42 Table No. |
| --- | --- | --- |
| Anthracite Emissions Factors (SCC 2104001000) |  |  |
| PM-CON | 0.08 * % Ash | 1.2-3 (stoker) |
| PM10-FIL | 10 | 1.2-3 (hand-fired) |
| PM25-FIL | 4.6 | Fig. 1.2-1 (ratio of PM25/PM10=1.25/2.70=0.46) 0.46*10=4.6 |
| PM10-PRI | 10 + 0.08 * % Ash | 1.2-3 |
| PM25-PRI | 4.6 + 0.08 * % Ash | 1.2-3 and Fig 1.2-1 |
| Sulfur Dioxide | 39 * % Sulfur | 1.2-1 (residential space heater) |
| Bituminous Emissions Factors (SCC 2104002000) |  |  |
| PM-CON | 1.04+ | 1.1-5 (stoker) |
| PM10-FIL | 6.2 | 1.1-4 (hand-fed) |
| PM25-FIL | 3.8 | 1.1-11 (underfeed stoker) |
| PM10-PRI | 7.24 | Sum of FIL and CON |
| PM25-PRI | 4.84 | Sum of FIL and CON |
| Sulfur Dioxide | 31 * % Sulfur | 1.1-3 (hand-fed) |
| +Emissions factor provided in AP-42 is 0.04 lb/MMBtu. This is multiplied by the conversion factor of 26 MMBtu/ton provided in AP-42 for bituminous coal. |  |  |

Table 4. State-Specific Sulfur Content for Bituminous Coal (SCC 2104002000)

| State | Percent Sulfur Content | State | Percent Sulfur Content |
| --- | --- | --- | --- |
| Alabama | 0.00 | Montana | 0.46 |
| Alaska | 0.15 | Nebraska | 0.00 |
| Arizona | 0.00 | Nevada | 0.00 |
| Arkansas | 0.00 | New Hampshire | 0.00 |
| California | 0.00 | New Jersey | 0.00 |
| Colorado | 0.31 | New Mexico | 0.00 |
| Connecticut | 0.00 | New York | 0.00 |
| Delaware | 0.00 | North Carolina | 1.63 |
| District of Columbia | 0.51 | North Dakota | 0.64 |
| Florida | 0.00 | Ohio | 0.88 |
| Georgia | 0.00 | Oklahoma | 0.00 |
| Hawaii | 0.00 | Oregon | 0.00 |
| Idaho | 0.00 | Pennsylvania | 0.83 |
| Illinois | 3.21 | Rhode Island | 0.00 |
| Indiana | 2.95 | South Carolina | 0.00 |
| Iowa | 2.60 | South Dakota | 0.00 |
| Kansas | 0.00 | Tennessee | 0.00 |
| Kentucky | 0.71 | Texas | 0.00 |
| Louisiana | 0.00 | Utah | 0.00 |
| Maine | 0.00 | Vermont | 0.00 |
| Maryland | 0.00 | Virginia | 1.08 |
| Massachusetts | 0.00 | Washington | 0.00 |
| Michigan | 0.00 | West Virginia | 0.00 |
| Minnesota | 0.22 | Wisconsin | 0.78 |
| Mississippi | 0.00 | Wyoming | 0.44 |
| Missouri | 3.03 |  |  |

Source: Reference 7

The emissions factors for CO, VOC, and some HAPs for anthracite coal are the emissions factors provided in AP-42 for bituminous coal. See Table 5 for the reference for each emissions factor. Emission rates for these pollutants are dependent upon combustion efficiency, with the mass of emissions per unit of heat input generally increasing with decreasing unit size. No anthracite emission rates are provided for residential heaters for these pollutants. Therefore, it was felt that it the AP-42 emission rates from bituminous coal that are derived for smaller hand-fed units, are more appropriate to use than applying anthracite emissions factors derived for much larger boilers.

Note that while AP-42 provides emissions factors for emissions of some metals from coal combustion, these factors are based on tests at controlled and/or pulverized coal boilers. These test conditions are not expected to be a good representation of emission rates for metals from residential heaters, so these pollutants are not included.

For all counties in the United States, the distillate oil consumed by residential combustion is assumed to be No. 2 fuel oil with a heating value of 140,000 Btu per gallon. The SO2 emissions factor for distillate oil assumes a sulfur content of 500 parts per million (ppm) and is calculated at the county level.

Emissions factors for kerosene are based on the emissions factors for distillate oil, which are multiplied by a factor of 135/140 to convert them for this use. This factor is based on the ratio of the heat content of kerosene (135,000 Btu/gallon) to the heat content of distillate oil (140,000 Btu/gallon).4 Criteria pollutant and HAP emissions factors are from the same sources discussed above for distillate fuel oil. The distillate sulfur content (500 ppm) is used for kerosene as well.8

Pollutant emissions factors for residential LPG are based on the residential natural gas emissions factors. The natural gas emissions factors are converted to LPG emissions factors by multiplying by 96,750 Btu/gallon.

Table 5. National Criteria and HAP Emissions Factors for All Fuel Types

| Pollutant code | Pollutant Code Description | Bituminous Coal 2104002000 |  | Anthracite Coal 2104001000 |  | Distillate Fuel Oil 2104004000 |  | Kerosenea 2104011000 |  | Natural Gas 2104006000 |  | LPGb 2104007000 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Factor numeric value (lb/ton) | Data source (Table) | Factor numeric value (lb/ton) | Data source (Table) | Factor numeric value (lb/ E3GAL) | Data source (Table) | Factor numeric value (lb/E3BBL) | Data source (Table) | Factor numeric value (lb/E6FT3) | Data source (Table) | Factor numeric value (lb/ E3BBL) | Data source (Table) |
| CO | CARBON MONOXIDE | 275 | 4; 1.1-3 | 275c | 4; 1.1-3 | 5 | 4; 1.3-1 | 202.5 | N/A | 40 | 4; 1.4-1 | 159.60 | N/A |
| NOX | NITROGEN OXIDES | 9.1 | 4; 1.1-3 | 3 | 4; 1.2-1 | 18 | 4; 1.3-1 | 729.0 | N/A | 94 | 4; 1.4-1 | 562.80 | N/A |
| PM-CON | PRIMARY PM CONDENSIBLE | See Table 3 |  | See Table 3 |  | 1.3 | 4; 1.3-2 | 52.7 | N/A | 0.32 |  | 1.27 | N/A |
| PM10-PRI | PRIMARY PM10 | See Table 3 |  | See Table 3 |  | 2.38d |  | 96.4 | N/A | 0.52 | 8 | 2.07 | N/A |
| PM10-FIL | PRIMARY PM10, FILTERABLE | See Table 3 |  | See Table 3 |  | 1.08 | 4; 1.3-7 | 43.7 | N/A | 0. 2 | 8 | 0.80 | N/A |
| PM25-PRI | PRIMARY PM2.5 | See Table 3 |  | See Table 3 |  | 2.13d |  | 86.3 | N/A | 0.43 | 8 | 1.71 | N/A |
| PM25-FIL | PRIMARY PM2.5, FILTERABLE | See Table 3 |  | See Table 3 |  | 0.83 | 4; 1.3-7 | 33.6 | N/A | 0.11 | 8 | 0.44 | N/A |
| SO2 | SULFUR DIOXIDE | See Table 3 |  | See Table 3 |  | 7.1e | 4; 1.3-1 | 287.6e | N/A | 0.6 | 4; 1.4-2 | 2.390 | N/A |
| VOC | VOLATILE ORGANIC COMPOUNDS | 10 | 4; 1.1-19 | 10c | 4; 1.1-19 | 0.713 | 4; 1.3-3 | 28.4 | N/A | 5.5 | 4; 1.4-2 | 21.91 | N/A |
| 121142 | 2,4-DINITROTOLUENE | 2.80E-07 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 532274 | 2-CHLOROACETOPHENONE | 7.00E-06 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 3697243 | 5-METHLY CHRYSENE | 2.20E-08 | 4; 1.1-13 |  |  |  |  |  |  |  |  |  |  |
| 83329 | ACENAPHTHENE | 5.10E-07 | 4; 1.1-13 | 2.20E-05 | 4; 1.2-5 | 2.11E-05 | 4; 1.3-9 | 8.53E-04 | N/A |  |  |  |  |
| 208968 | ACENAPHTHYLENE | 2.50E-07 | 4; 1.1-13 | 8.60E-05 | 4; 1.2-5 | 2.53E-07 | 4; 1.3-9 | 1.02E-05 | N/A |  |  |  |  |
| 75070 | ACETALDEHYDE | 5.70E-04 | 4; 1.1-14 |  |  | 4.92E-03 | 6 | 0.199 | N/A | 1.37E-05 | 6 | 5.44E-05 | N/A |
| 98862 | ACETOPHENONE | 1.50E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 107028 | ACROLEIN | 2.90E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| NH3 | AMMONIA | 2 | 5; III-1 | 2 | 5; III-1 | 1 | 5; III-1 | 40.5 | N/A | 20 | 5; III-1 | 1.95 | N/A |
| 120127 | ANTHRACENE | 2.10E-07 | 4; 1.1-13 | 2.50E-05 | 4; 1.2-5 | 1.22E-06 | 4; 1.3-9 | 4.95E-05 | N/A |  |  |  |  |
| 7440382 | ARSENIC |  |  |  |  | 5.62E-04 | 6 | 2.28E-02 | N/A |  |  |  |  |
| 56553 | BENZ[A]ANTHRACENE | 8.00E-08 | 4; 1.1-13 | 7.10E-05 | 4; 1.2-5 | 4.01E-06 | 4; 1.3-9 | 1.65E-04 | N/A |  |  |  |  |
| 71432 | BENZENE | 0.0013 | 4; 1.1-14 |  |  | 2.14E-04 | 4; 1.3-9 | 8.53E-03 | N/A | 2.21E-03 | 6 | 8.78E-03 | N/A |
| 50328 | BENZO[A]PYRENE | 3.80E-08 | 4; 1.1-13 | 5.30E-06 | 4; 1.2-5 |  |  |  |  |  |  |  |  |
| 192972 | BENZO[E]PYRENE |  |  | 6.20E-06 | 4; 1.2-5 |  |  |  |  |  |  |  |  |
| 191242 | BENZO[G,H,I,]PERYLENE | 2.70E-08 | 4; 1.1-13 | 5.50E-06 | 4; 1.2-5 | 2.26E-06 | 4; 1.3-9 | 9.10E-05 | N/A |  |  |  |  |
| 207089 | BENZO[K]FLUORANTHRENE |  |  | 2.50E-05 | 4; 1.2-5 |  |  |  |  |  |  |  |  |
| 100447 | BENZYL CHLORIDE | 7.00E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 7440417 | BERYLLIUM |  |  |  |  | 4.214E-04 | 6 | 0.017 | N/A |  |  |  |  |
| 92524 | BIPHENYL | 1.70E-06 | 4; 1.1-13 |  |  |  |  |  |  |  |  |  |  |
| 117817 | BIS(2-ETHYLHEXYL) PHTHALATE | 7.30E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 75252 | BROMOFORM | 3.90E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 7440439 | CADMIUM |  |  |  |  | 4.214E-04 | 6 | 0.017 | N/A |  |  |  |  |
| 75150 | CARBON DISULFIDE | 1.30E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 108907 | CHLOROBENZENE | 2.20E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 67663 | CHLOROFORM | 5.90E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 18540299 | Chromium (VI) |  |  |  |  | 7.5854e-05 |  | 3.07E-03 | N/A |  |  |  |  |
| 16065831 | Chromium III |  |  |  |  | 3.46E-04 |  | 0.014 | N/A |  |  |  |  |
| 218019 | CHRYSENE | 1.00E-07 | 4; 1.1-13 | 8.30E-05 | 4; 1.2-5 | 2.38E-06 | 4; 1.3-9 | 9.67E-05 | N/A |  |  |  |  |
| 98828 | CUMENE | 5.30E-06 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 57125 | CYANIDE | 2.50E-03 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 53703 | DIBENZO[A,H]ANTHRACENE |  |  |  |  | 1.67E-06 | 4; 1.3-9 | 6.83E-05 | N/A |  |  |  |  |
| 77781 | DIMETHYL SULFATE | 4.80E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 100414 | ETHYL BENZENE | 9.40E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 75003 | ETHYL CHLORIDE | 4.20E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 106934 | ETHYLENE DIBROMIDE | 1.20E-06 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 107062 | ETHYLENE DICHLORIDE | 4.00E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 206440 | FLUORANTHENE | 7.10E-07 | 4; 1.1-13 | 1.70E-04 | 4; 1.2-5 | 4.84E-06 | 4; 1.3-9 | 1.99E-04 | N/A | 3E-06 | 4; 1.4-3 | 1.26E-05 | N/A |
| 86737 | FLUORENE | 9.10E-07 | 4; 1.1-13 | 2.50E-05 | 4; 1.2-5 | 4.47E-06 | 4; 1.3-9 | 1.82E-04 | N/A | 2.8E-06 | 4; 1.4-3 | 1.17E-05 | N/A |
| 50000 | FORMALDEHYDE | 2.40E-04 | 4; 1.1-14 |  |  | 3.30E-02 | 4; 1.3-9 | 1.365 | N/A | 7.5E-02 | 4; 1.4-3 | 0.31 | N/A |
| 110543 | HEXANE | 6.70E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 7647010 | HYDROGEN CHLORIDE | 1.2 | 4; 1.1-15 | 1.2c | 4; 1.1-15 |  |  |  |  |  |  |  |  |
| 7664393 | HYDROGEN FLUORIDE | 0.15 | 4; 1.1-15 | 0.15c | 4; 1.1-15 |  |  |  |  |  |  |  |  |
| 193395 | INDENO[1,2,3-C,D]PYRENE | 6.10E-08 | 4; 1.1-13 |  |  | 2.14E-06 | 4; 1.3-9 | 8.53E-05 | N/A |  |  |  |  |
| 78591 | ISOPHORONE | 5.80E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 7439921 | LEAD |  |  |  |  | 1.264E-03 | 6 | 0.051 | N/A |  |  |  |  |
| 7439965 | MANGANESE |  |  |  |  | 8.428E-04 | 6 | 0.034 | N/A |  |  |  |  |
| 7439976 | MERCURY | 8.30E-05 | 4; 1.1-18 | 1.30E-04 | 4; 1.2-7 | 4.214E-04 | 6 | 0.017 | N/A |  |  |  |  |
| 74839 | METHYL BROMIDE | 1.60E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 74873 | METHYL CHLORIDE | 5.30E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 80626 | METHYL METHACRYLATE | 2.00E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 1634044 | METHYL TERT BUTYL ETHER | 3.50E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 75092 | METHYLENE CHLORIDE | 2.90E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 91203 | NAPHTHALENE | 1.30E-05 | 4; 1.1-13 | 2.20E-04 | 4; 1.2-5 | 1.13E-03 | 4; 1.3-9 | 0.046 | N/A | 6.1E-04 | 4; 1.4-3 | 2.55E-03 | N/A |
| 7440020 | NICKEL |  |  |  |  | 4.214E-04 | 6 | 0.017 | N/A |  |  |  |  |
| 198550 | PERYLENE |  |  | 1.20E-06 | 4; 1.2-5 |  |  |  |  |  |  |  |  |
| 85018 | PHENANTHRENE | 2.70E-06 | 4; 1.1-13 | 2.40E-04 | 4; 1.2-5 | 1.05E-05 | 4; 1.3-9 | 4.27E-04 | N/A | 1.7E-05 | 4; 1.4-3 | 7.11E-05 | N/A |
| 108952 | PHENOL | 1.60E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 123386 | PROPIONALDEHYDE | 3.80E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 129000 | PYRENE | 3.30E-07 | 4; 1.1-13 | 1.20E-04 | 4; 1.2-5 | 4.25E-06 | 4; 1.3-9 | 1.71E-04 | N/A | 5E-06 | 4; 1.4-3 | 2.09E-05 | N/A |
| 7782492 | SELENIUM |  |  |  |  | 2.107E-03 | 6 | 0.085 | N/A |  |  |  |  |
| 100425 | STYRENE | 2.50E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 127184 | TETRACHLOROETHYLENE | 4.30E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 108883 | TOLUENE | 2.40E-04 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 108054 | VINYL ACETATE | 7.60E-06 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| 1330207 | XYLENES | 3.70E-05 | 4; 1.1-14 |  |  |  |  |  |  |  |  |  |  |
| - Kerosene emissions factors are based on distillate fuel oil emissions factors, with conversions. See text in section E for more information. - LPG emissions factors are based on natural gas emissions factors, with conversions. See text in section E for more information. - Emissions factors for CO, VOC, and some HAPs for anthracite coal are based on emissions factors for bituminous coal. See text in section E for more information. - Primary PM25-PRI and PM10-PRI emissions factors are the sum of the emissions factors for the filterable and condensable portions of PM emissions. - Assumes a sulfur content of 500 ppm. See text in section E for more information. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Controls

There are no controls assumed for this category.

# Emissions

The criteria pollutant and HAP emissions from residential heating are calculated by multiplying the distributed county-level residential fuel consumption by the corresponding emissions factor for each pollutant. The adjusted emissions factors for SO2 and PM for anthracite and bituminous/subbituminous coal are calculated above in equation 9 in Section E.

| Ef,c,p=FCf,c× EFf,p×1 ton2000 lb |  |
| --- | --- |

Where:

Ef,c,p	=	annual emissions of pollutant p from combustion of fuel type f in county c, in tons

FCf,c	= 	fuel consumption of fuel type f in county c, in tons, thousand barrels, or thousand cubic feet, from equation 5

EFf,p	=	emissions factor pollutant p and fuel type f, in pounds of emissions per unit (tons, thousand barrels, or thousand cubic feet) of fuel consumption, from Table 5

# Point Source Subtraction

There are no point source-specific SCCs for residential heating; therefore, point source subtraction is not performed for this category.

# Sample Calculations

Table 6 provides sample calculations for CO emissions from residential heating from distillate fuel oil in Allegheny County, PA.

Table 6. Sample calculations for CO emissions from residential heating from distillate fuel oil in Allegheny County, PA

| Eq. # | Equation | Values for Allegheny County, PA | Result |
| --- | --- | --- | --- |
| 1 | FCanth/bit,s=FCcoal,s×Ranth/bit | N/A | This example is for distillate. Equation 1 is for coal. |
| 2 | Rdfo/ker,s=FCdfo/ker, sFCdfo,s+FCker,s | 15,062 thousand barrels(15,062 thousand barrels+238 thousand barrels) | 0.9844 ratio of DFO to total fuel oil |
| 3 | HUdfo/ker,c=HUfo,c×Rdfo/ker,s | 8,081 houses× 0.9844 | 7,955.30 houses using DFO in Allegheny County, PA |
| 4 | Rf,c=HUf,cHUf,s | 7,955.30 houses916,301.2 houses | 0.0086 county housing allocation ratio for Allegheny County, PA |
| 5 | FCf,c=FCf,s× Rf,c×42 gal. per barrel | 15,062 thous. barrels×0.0086×42 gal. per barrel | 5,492.25 thousand gallons DFO consumed in Allegheny County, PA |
| 6 | EFanth/bit,s,p=SACf,s×EFunadj,f | N/A | This example is for distillate. Equation 6 is for coal. |
| 7 | Ef,c,p=FCf,c× EFf,p×1 ton2000 lb | 5,492.25 thous. gal. ×5 lbs. per thous. gal×1 ton2000 lb | 13.7 tons CO from DFO in Allegheny County, PA |

# Changes from 2014 Methodology

The 2017 methodology used a lower sulfur content value of 500 ppm for distillate fuel oil and kerosene compared to the value of 3% used in the 2014 methodology.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Since insufficient data exist to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emissions factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emissions factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

# References

U.S. Department of Energy. Energy Information Administration (EIA).  2019. State Energy Data System (SEDS): 1960-2017 Consumption  http://www.eia.gov/state/seds/seds-data-complete.cfm?sid=US#CompleteDataFile.

U.S. Census Bureau. 2018. American Community Survey. B25040 House Heating Fuel, 2017 ACS 5-Year Estimates. https://data.census.gov/cedsci/table?q=B25040&g=0100000US_0400000US29_0500000US29019&tid=ACSDT5Y2017.B25040&hidePreview=true&y=2017

U.S. Department of Energy, Energy Information Administration. 2018. “Domestic Distribution of U.S. Coal by Destination State, Consumer, Origin and Method of Transportation” http://www.eia.doe.gov/cneaf/coal/page/coaldistrib/coal_distributions.html.

U.S. Environmental Protection Agency.  1996. Compilation of Air Pollutant Emission Factors, 5th Edition, AP-42, Volume I:  Stationary Point and Area Sources.  Research Triangle Park, North Carolina. https://www.epa.gov/air-emissions-factors-and-quantification/ap-42-compilation-air-emission-factors#5thed

U.S. Environmental Protection Agency. 2004. Emission Inventory Improvement Program. Estimating Ammonia Emissions from Anthropogenic Sources, Draft Final Report.  Prepared by E.H. Pechan and Associates, Inc.  Research Triangle Park, NC. https://www.epa.gov/sites/production/files/2015-08/documents/eiip_areasourcesnh3.pdf

Porter, Fred, U.S. Environmental Protection Agency, Emission Standards Division. Note to Anne Pope, U.S. Environmental Protection Agency/Emissions Monitoring and Analysis Division. Comments on Industrial Boiler information in the “Baseline Emission Inventory of HAP Emissions from MACT Sources – Interim Final Report,” September 18, 1998. November 13, 1998.

U.S. Department of Energy, Energy Information Administration. 2017. Coal Data Browser. https://www.eia.gov/coal/data/browser/#/topic/26?agg=1,0&geo=vvvvvvvvvvvvo&sec=0g&linechart=COAL.SULFUR_CONTENT.US-8.A&columnchart=COAL.SULFUR_CONTENT.US-8.A&map=COAL.SULFUR_CONTENT.US-8.A&freq=A&start=2002&end=2017&ctype=map&ltype=pin&rtype=s&pin=&rse=0&maptype=0

U.S. Environmental Protection Agency. 2016. Technical Support Document (TSD) Preparation of Emissions Inventories for the Version 6.3, 2011 Emissions Modeling Platform. Prepared by U.S. Environmental Protection Agency Office of Air and Radiation Office of Air Quality Planning and Standards Air Quality Assessment Division. https://www.epa.gov/sites/production/files/2016-09/documents/2011v6_3_2017_emismod_tsd_aug2016_final.pdf

Huntley, Roy. 2012. Spreadsheet: “natgas_procgas_lpg_pm_efs_not_ap42_032012_revisions.xls”