# Solvent Utilization

# Source Category Description

Solvent utilization includes a variety of industrial, commercial and residential applications of solvents that are not captured in the point source inventory. Estimates of emissions of volatile organic compounds (VOC) and hazardous air pollutants (HAPs) from solvent utilization are based on national-level estimates of solvent usage from the Freedonia Group. In 2014, Solvent utilization in the US, Puerto Rico, and US Virgin Islands resulted in over 3 million tons of VOC emissions.

For this source category, the following SCCs are assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2401001000 | Solvent Utilization | Surface Coating | Architectural Coatings | Total: All Solvent Types |
| 2401005000 | Solvent Utilization | Surface Coating | Auto Refinishing | Total: All Solvent Types |
| 2401008000 | Solvent Utilization | Surface Coating | Traffic Markings | Total: All Solvent Types |
| 2401015000 | Solvent Utilization | Surface Coating | Factory Finished Wood | Total: All Solvent Types |
| 2401020000 | Solvent Utilization | Surface Coating | Wood Furniture | Total: All Solvent Types |
| 2401025000 | Solvent Utilization | Surface Coating | Metal Furniture | Total: All Solvent Types |
| 2401030000 | Solvent Utilization | Surface Coating | Paper | Total: All Solvent Types |
| 2401040000 | Solvent Utilization | Surface Coating | Metal Cans | Total: All Solvent Types |
| 2401055000 | Solvent Utilization | Surface Coating | Machinery and Equipment | Total: All Solvent Types |
| 2401060000 | Solvent Utilization | Surface Coating | Large Appliances | Total: All Solvent Types |
| 2401065000 | Solvent Utilization | Surface Coating | Electronics and Other Electrical | Total: All Solvent Types |
| 2401070000 | Solvent Utilization | Surface Coating | Motor Vehicles | Total: All Solvent Types |
| 2401075000 | Solvent Utilization | Surface Coating | Aircraft | Total: All Solvent Types |
| 2401085000 | Solvent Utilization | Surface Coating | Railroad | Total: All Solvent Types |
| 2401080000 | Solvent Utilization | Surface Coating | Marine | Total: All Solvent Types |
| 2401090000 | Solvent Utilization | Surface Coating | Misc. Manufacturing | Total: All Solvent Types |
| 2401100000 | Solvent Utilization | Surface Coating | Industrial Maintenance Coatings | Total: All Solvent Types |
| 2401200000 | Solvent Utilization | Surface Coating | Other Special Purpose Coatings | Total: All Solvent Types |
| 2415000000 | Solvent Utilization | Degreasing | All Processes/All Industries | Total: All Solvent Types |
| 2425000000 | Solvent Utilization | Graphic Arts | All Processes | Total: All Solvent Types |
| 2460100000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | All Personal Care Products | Total: All Solvent Types |
| 2460200000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | All Household Products | Total: All Solvent Types |
| 2460400000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | All Automotive Aftermarket Products | Total: All Solvent Types |
| 2460600000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | All Adhesives and Sealants | Total: All Solvent Types |
| 2460800000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | All FIFRA Related Products | Total: All Solvent Types |
| 2460500000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | All Coatings and Related Products | Total: All Solvent Types |
| 2460900000 | Solvent Utilization | Misc. Non-industrial: Consumer and Commercial | Misc. Products | Total: All Solvent Types |
| 2420000000 | Solvent Utilization | Dry Cleaning | All Processes | Total: All Solvent Types |

# Overview of Calculations

The emissions from solvent use are calculated based on national-level data on solvent use from the Freedonia Group. This data is used to develop emissions factors per capita, per employee, or per lane mile of highway, depending on the SCC. The emissions factors are used to estimate VOC emissions in each county. HAP emissions are estimated using the VOC emissions and HAP speciation factors. Because the data from Freedonia is for total solvent use, point source emissions must be subtracted to estimate the nonpoint source emissions. The activity data for solvent use is discussed in section C. The process for allocating activity data to the county level is discussed in section D. The method for developing the emissions factors is discussed n section E. The emissions calculation methods are discussed in section G. The method for subtracting point source emissions data is discussed in section H.

# Activity Data

The activity data for solvent utilization varies by SCC; it is based on population data from the U.S. Census Bureau, lane miles data from the Federal Highway Administration, or employment data from the U.S. Census Bureau.

Population

The activity data for the categories listed in Table 1 are based on county-level population data. Population data are from the U.S. Census Bureau’s population estimates for 2017.

Table 1. Source Categories That Use Population Activity Data

| SCC | Description |
| --- | --- |
| 2401001000 | Architectural Coatings |
| 2401100000 | Industrial Maintenance Coatings |
| 2401200000 | Other Special Purpose Coatings |
| 2460100000 | All Personal Care Products |
| 2460200000 | All Household Products |
| 2460400000 | All Automotive Aftermarket Products |
| 2460600000 | All Adhesives and Sealants |
| 2460800000 | All FIFRA Related Products |
| 2460500000 | All Coatings and Related Products |
| 2460900000 | Misc. Products |

Lane Miles

County-level lane mile data are used as activity data for one source category (Table 2). The Federal Highway Administration (FHWA) provides state-level lane mile data yearly as part of the Highway Statistics Report. State-level data is allocated to the county level using population data. The process used to distribute the state-level lane miles data to the counties is discussed in section D.

Table 2. Source Categories That Use Lane Mile Activity Data

| SCC | Description |
| --- | --- |
| 2401008000 | Traffic Markings |

Employment Data

The source categories listed in Table 3 use county-level employment data as activity data. Employment data are provided by the U.S. Census Bureau’s 2016 County Business Patterns (CBP).

Table 3. Source Categories That Use Employment Activity Data

| SCC | Description | NAICS |
| --- | --- | --- |
| 2401005000 | Auto Refinishing | 81112, 4411, 4412 |
| 2401015000 | Factory Finished Wood | 321 |
| 2401020000 | Wood Furniture | 337110, 337121, 337122, 337127*, 337211, 337212, 337215* |
| 2401025000 | Metal Furniture | 337124, 337127*, 337214, 337215* |
| 2401030000 | Paper | 322220 |
| 2401040000 | Metal Cans | 33243 |
| 2401055000 | Machinery and Equipment | 3331, 3332, 3333, 33341 |
| 2401060000 | Large Appliances | 3352 |
| 2401065000 | Electronics and Other Electrical | 331318, 331420, 331491, 335921, 335929, 335311 |
| 2401070000 | Motor Vehicles | 3361, 3362, 3363 |
| 2401075000 | Aircraft | 3364 |
| 2401085000 | Railroad | 3365 |
| 2401080000 | Marine | 3366, 488390 |
| 2401090000 | Misc. Manufacturing | 339, 3369 |
| 2415000000 | Degreasing: All Processes/All Industrial | 331, 332, 333, 334, 335, 336, 337, 339, 441, 483, 484, 485, 488, 8111, 8112 |
| 2425000000 | Graphic Arts | 32311, 322211, 322212, 322219, 322220, 322230, 322291, 322299 |
| 2420000000 | Dry Cleaning | 812320 |

*Employment data is split equally between Wood Furniture and Metal Furniture Source Categories

Employment data for select NAICS codes and counties must be allocated based on state-level data. The process used to distribute the state-level amount employment data to the counties is discussed in section D.

# Allocation Procedure

Lane Miles

Lane miles data is published yearly by FHWA at the state-level. Population data is used to allocate the state-level data to the county-level. In order to allocate the state-level data, a fraction of county to state-level population is created.

| PFracc=PcPst |  |
| --- | --- |

Where:

PFracc	=	Population fraction for county c

Pc	=	Population of county c

Pst	=	Population of state st where county c is located

This fraction is then applied to the state-level lane miles data to estimate county-level lane miles.

| LMc=PFracc×LMst |  |
| --- | --- |

Where:

LMc	=	Lane-miles in county c

PFracc	=	Population fraction for county c

LMst	=	Lane miles in state st where county c is located

Employment Data

Employment data are from the U.S. Census Bureau’s 2016 CBP. Due to concerns with releasing confidential business information, the CBP does not release exact numbers for a given North American Industrial Classification Standard (NAICS) code if the data can be traced to an individual business. Instead, a series of range codes is used. Many counties and some smaller states have only one business per NAICS code, leading to withheld data in the county and/or state business pattern data. To estimate employment in counties and states with withheld data, the following procedure is used for NAICS code 322220.

To gap-fill withheld state-level employment data:

- State-level data for states with known employment in NAICS 322220 are summed to the national level.

- The total sum of state-level known employment from step a is subtracted from the national total reported employment for NAICS 322220 in the national-level CBP to determine the employment total for the withheld states.

- Each of the withheld states is assigned the midpoint of the range code reported for that state. Table 4 lists the range codes and midpoints.

- The midpoints for the states with withheld data are summed to the national-level.

- An adjustment factor is created by dividing the number of withheld employees (calculated in step b of this section) by the sum of the midpoints (step d).

- For the states with withheld employment data, the midpoint of the range for that state (step c) is multiplied by the adjustment factor (step e) to calculate the adjusted state-level employment for landfills.

These same steps are then followed to fill in withheld data in the county-level business patterns.

- County-level data for counties with known employment are summed by state.

- County-level known employment is subtracted from the state total reported in state-level CBP (or, if the state-level data are withheld, from the state total estimated using the procedure discussed above).

- Each of the withheld counties is assigned the midpoint of the range code (Table 4).

- The midpoints for the counties with withheld data are summed to the state level.

- An adjustment factor is created by dividing the number of withheld employees (step h) by the sum of the midpoints (step j).

- For counties with withheld employment data, the midpoints (step i) are multiplied by the adjustment factor (step k) to calculate the adjusted county-level employment for landfills.

Table 4. Ranges and midpoints for data withheld from state and county business patterns

| Employment Code | Ranges | Midpoint |
| --- | --- | --- |
| A | 0-19 | 10 |
| B | 20-99 | 60 |
| C | 100-249 | 175 |
| E | 250-499 | 375 |
| F | 500-999 | 750 |
| G | 1,000-2,499 | 1,750 |
| H | 2,500-4,999 | 3,750 |
| I | 5,000-9,999 | 7,500 |
| J | 10,000-24,999 | 17,500 |
| K | 25,000-49,999 | 37,500 |
| L | 50,000-99,999 | 75,000 |
| M | 100,000+ |  |

For example, take the 2016 CBP data for NAICS 322220 (paper bag and coated and treated paper manufacturing) in Kentucky provided in Table 5.

Table 5. 2016 County Business Pattern for NAICS 322220 in Kentucky

| State FIPS | County FIPS | County Name | NAICS | Employment Code | Employment |
| --- | --- | --- | --- | --- | --- |
| 21 | 015 | Boone | 322220 | F | withheld |
| 21 | 041 | Carroll | 322220 | B | withheld |
| 21 | 097 | Harrison | 322220 | F | withheld |
| 21 | 111 | Jefferson | 322220 |  | 391 |
| 21 | 117 | Kenton | 322220 | A | withheld |
| 21 | 211 | Shelby | 322220 |  | 338 |
| 21 | 213 | Simpson | 322220 | F | withheld |
| 21 | 219 | Todd | 322220 | B | withheld |

Note: Counties in Kentucky that do not have employment in paper bag and coated and treated paper manufacturing are excluded from this table.

- The total number of known county-level employees in Kentucky is 729.

- The state-level CBP reports 2,517 employees for NAICS 322220 in Kentucky. This means there are 1,788 employees total for the 6 counties for which data are withheld.

- The counties with withheld data are assigned midpoints according to their employment code in Table 4. For example, Carroll County is given a midpoint of 60 employees (since range code B is 20-99) and Kenton County is given a midpoint of 10 employees.

- The state total of the midpoints for all withheld counties is 2,380 employees.

- The adjustment factor is 1,788/2,380 = 0.7513.

- The adjusted employment for Carroll County is 60 × 0.7513 = 45. Kenton County has an adjusted employment of 10 × 0.7513 = 8 employees.

# Emissions Factors

Emissions factors for most solvent utilization categories are based on national-level estimates of solvent usage from the Freedonia Group. The Freedonia data includes historical usage of solvents in 2015 and projected solvent usage for 2020. Assuming a linear change in solvent demand, EPA estimated solvent usage for 2017 (Table 6).

Table 6. Solvent Usage in the US

| Description | Solvent Usage (Million Pounds) |  |  |
| --- | --- | --- | --- |
|  | 2015 | 2017 | 2020 |
| Paints & Coatings Solvent Demand: Architectural | 735 | 777 | 840 |
| Paints & Coatings Solvent Demand: Other | 1,318 | 1,321 | 1,325 |
| Printing Ink Solvent Demand | 1,132 | 1,134 | 1,138 |
| Cleaning Products Solvent Demand: Household | 653 | 657 | 662 |
| Cleaning Products Solvent Demand: Industrial & Institutional | 385 | 390 | 398 |
| Cosmetics & Toiletries Solvent Demand | 628 | 645 | 670 |
| Adhesives & Sealants Solvent Demand | 572 | 600 | 643 |
| Transportation Solvent Demand: Motor Vehicles | 61 | 62 | 64 |
| Dry Cleaning | 20 | 18 | 16 |

Source: The Freedonia Group

Table 14, in the appendix, shows a crosswalk between the source categories and the data used to calculate their emissions factors. Some categories, such as personal care products, use only the Freedonia Group data. For these categories, the emissions factor is calculated by dividing the total amount of solvent used by the categories’ activity data.

| EFs=Fs×1,000,000As |  |
| --- | --- |

Where:

EFs	=	Emissions factor for source category s

Fs	=	The Freedonia Group data for source category s, in million pounds per year

As	=	National-level activity data for source category s, either population, lane miles, or employment

Freedonia data does not include usage estimates for all surface coating categories, therefore, additional data is used to allocate the non-architectural solvent data to the SCC level. A previous version of this methodology used data from the U.S. Census Bureau’s report on Paint and Allied Products to determine solvent use from surface coating, but this report was not produced after 2010. EPA grew the 2010 data from the most recent version of this report to estimate solvent use for surface coating in 2017. The estimated 2017 value is used to calculate the fraction of non-architectural coating use from each source category for surface coating. This fraction is then applied to total non-architectural solvent demand from the Freedonia Group to calculate 2017 solvent use for surface coating categories.

To grow the 2010 Paint and Allied Products data to 2017, EPA uses the U.S. Census Bureau’s Annual Survey of Manufactures data on the value of paint shipments in 2010 and 2016.  Using the relevant product codes (see Table 7), the value of paint shipments are summed for each category for 2010 and 2016. There are not corresponding product codes for all surface coating SCCs; in these cases, the general paint and coating manufacturing data are used.  The 2016 value of shipments for each category are converted to 2010 USD by multiplying by 0.9075, a conversation factor from the U.S. Bureau of Labor Statistics.

| TSs,y=NAICSVSy |  |
| --- | --- |
| TSs,2016c=TSs,2016×0.9075 |  |

Where:

TSs,y	=	Total value of shipments for source category s in year in year y, in thousand dollars

VSy	=	Value of shipments in year y, in thousand dollars

NAICS	=	NAICS codes corresponding to source category s

TSs,2016c	=	Total value of shipments for source category s in 2016, converted to 2010 USD

A ratio of the 2010 value of shipments, from the Survey of Manufactures, to 2010 volume of paint, from the Paint and Allied Products report, was then used with the converted 2016 value of shipments to estimate the 2016 volume of paint.

| VPs,2016=TSs,2016c×VPs,2010TSs,2010 |  |
| --- | --- |

Where:

VPs,2016	=	Volume of paint for source category s in 2016, in thousand gallons

TSs,2016c	=	Total value of shipments for source category s in 2016, converted to 2010 USD

VPs,2010	=	Volume of paint for source category s in 2010, in thousand gallons

TSs,2010	=	Total value of shipments for source category s in 2010, in thousand dollars

The estimated volume of paint in 2016 is then used to create a 2016 to 2010 paint ratio (Table 7). The paint ratio represents the fraction change in surface coating solvent use in each source category between 2010 and 2016. For example, a paint ratio greater than 1 means there was an increase in solvent use in that source category between 2010 and 2016.

| PRs=VPs,2016VPs,2010 |  |
| --- | --- |

Where:

PRs	=	2016-2010 Paint Ratio

VPs,2016	=	Volume of paint for source category s in 2016, in thousand gallons

VPs,2010	=	Volume of paint for source category s in 2010, in thousand gallons

Table 7. 2016-2010 Paint Ratio

| Product Codes | Description | Paint Ratio |
| --- | --- | --- |
| 325510 | Paint and coating manufacturing | 1.246 |
| 321 | Wood Products | 1.327 |
| 337 | Furniture | 1.196 |
| 32222/322220 | Paper bag and coated and treated paper manufacturing | 0.981 |
| 332431 & 332439 | Metal Can and Container Manufacturing | 0.835 |
| 3352 | Household Appliances | 1.121 |
| 3361 | Motor Vehicle Manufacturing | 1.513 |
| 3364 | Aircraft Manufacturing | 1.262 |
| 336510 | Railroad rolling stock manufacturing | 1.461 |
| 3366 | Boat Manufacturing | 1.088 |
| 339 | Misc. Manufacturing | 0.929 |
| 3331, 3332, 3333,33341 | Machinery Manufacturing | 0.901 |
| 335921, 335929, 335311 | Electronics Manufacturing | 0.944 |

The paint ratios are multiplied by the volume of paint sold in 2010 from the Paint and Allied Products report for each SCC to estimate the volume of paint sold in 2017.

| VPs,2017=PRs×VPs,2010 |  |
| --- | --- |

Where:

VPs,2017	=	Volume of paint for source category s in 2017, in thousand gallons

PRs	=	2016-2010 Paint Ratio for source category s, from Table 7

VPs,2010	=	Volume of paint for source category s in 2010, in thousand gallons

The total amount of non-architectural coatings from the Paint and Allied Products data is also calculated in order to estimate the fraction of non-architectural coatings for each SCC. The report includes data on the total amount of coatings sold in 2010, as well as the amount of architectural and powder coatings sold; these values are subtracted from the total to estimate the volume of non-architectural coatings (Table 8). These values are adjusted to 2017 using the paint and coating manufacturing paint ratio.

| NAC2017=(TC2010-AC2010-PC2010)×PRpaint and coatings |  |
| --- | --- |

Where:

NAC2017				=	Volume of non-architectural coatings sold in 2017, in gallons

TC2010				=	Total volume of coatings sold in 2010, in gallons

AC2010				=	Volume of architectural coatings sold in 2010, in gallons

PC2010				=	Volume of powder coatings sold in 2010, in gallons

PRpaint and coatings	=	Paint ratio for paint and coating manufacturing, from Table 7

Table 8. Coatings Sold in 2010

| Type of Coating | Amount Sold in 2010 (Gallons) |
| --- | --- |
| Volume of Total Coatings Sold | 1,301,333,355 |
| Volume of Architectural Coatings | 651,626,800 |
| Volume of Powder Coatings | 75,774,600 |
| Volume of Non-architectural Coatings | 573,931,955 |

Source: U.S. Census Bureau

The volume of paint sold in 2017 for each SCC (from equation 8) is then divided by the total volume of non-architectural coatings to estimate the fraction of non-architectural paint from each SCC. This fraction is then multiplied by the volume of solvent demand from “paints and coatings: other” for 2017 from Freedonia.

| NAFracs=VPs,2017×1000NAC2017 |  |
| --- | --- |
| SDs,2017=NAFracs×Oth2017 |  |

Where:

NAFracs	=	Fraction of non-architectural coatings from source category s

VPs,2017	=	Volume of paint for source category s in 2017, in thousand gallons

NAC2017	=	Volume of non-architectural coatings sold in 2017, in gallons

SDs,2017	=	Solvent demand for source category s in 2017, in million pounds

Oth2017	=	Other paint and coatings solvent demand in 2017 from Freedonia, in million pounds

After solvent use is estimated for each surface coating category, equation 3 is used to calculate the emissions factor for each SCC.

There are three exceptions to this method for surface coating solvents: aircraft coatings, railroad coatings, and other special purpose coatings. Data for solvent use for other special purpose coatings is not available in the 2010 version of the Paint and Allied Products Report. Therefore data for special purpose coatings from the 2006 version of the report was pulled forward and adjusted to 2017 using the same method as reported above.

The Paint and Allied Products report also aggregates aircraft and railroad coatings in the “other transportation equipment finishes” category. The 2010 volume of paint is grown to 2017 and used to determine solvent demand by the same method as described above. Solvent demand for the other transportation category was then divided in half and assigned equally to the aircraft and railroad SCCs.

Emissions factors for the three Consumer and Commercial categories—including FIFRA related products, coatings and related products, and misc. products—are not estimated by using Freedonia data, but rather come from EPA’s Air Emissions Inventory Improvement Program (EIIP).

The architectural coatings, industrial maintenance coatings, and consumer solvents source categories have controlled emissions factors that are used for states that have enacted regulations to control the VOC emissions from these types of solvents. These controlled emissions factors are discussed in section F.

VOC emissions factors for all SCCs in this category are listed in Table 9.

Table 9. VOC Emissions Factors for Solvent Utilization

| SCC | Description | Pollutant Code | Emissions Factor | Unit | Activity Data | Source |
| --- | --- | --- | --- | --- | --- | --- |
| 2401001000 | Architectural Coatings | VOC | 2.36 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2401001000 | Architectural Coatings (controlled) | VOC | 1.88 | LB/EACH | Pop. | ERTAC, U.S. Census Bureau |
| 2401005000 | Auto Refinishing | VOC | 75.58 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401008000 | Traffic Markings | VOC | 9.80 | LB/EACH | Lane Miles | Freedonia Group, U.S. Census Bureau |
| 2401015000 | Factory Finished Wood | VOC | 44.71 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401020000 | Wood Furniture | VOC | 282.87 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401025000 | Metal Furniture | VOC | 769.02 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401030000 | Paper | VOC | 398.22 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401040000 | Metal Cans | VOC | 2,239.43 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401055000 | Machinery and Equipment | VOC | 34.28 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401060000 | Large Appliances | VOC | 168.96 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401065000 | Electronic and Other Electrical | VOC | 15.58 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401070000 | Motor Vehicles | VOC | 160.31 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401075000 | Aircraft | VOC | 15.40 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401085000 | Railroad | VOC | 212.90 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401080000 | Marine | VOC | 176.75 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401090000 | Misc. Manufacturing | VOC | 69.99 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2401100000 | Industrial Maintenance Coatings | VOC | 0.36 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2401100000 | Industrial Maintenance Coatings (controlled) | VOC | 0.15 | LB/EACH | Pop. | ERTAC, U.S. Census Bureau |
| 2401200000 | Other Special Purpose Coatings | VOC | 0.01 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2415000000 | Degreasing: All Processes/All Industries | VOC | 32.36 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2425000000 | Graphic Arts | VOC | 1,583.65 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau |
| 2460100000 | All Personal Care Products | VOC | 1.96 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2460100000 | All Personal Care Products (controlled) | VOC | 1.15 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2460200000 | All Household Products | VOC | 1.99 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2460200000 | All Household Products (controlled) | VOC | 1.17 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2460400000 | All Automotive Aftermarket Products | VOC | 0.19 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2460400000 | All Automotive Aftermarket Products (controlled) | VOC | 0.11 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2460600000 | All Adhesives and Sealants | VOC | 1.82 | LB/EACH | Pop. | Freedonia Group, U.S. Census Bureau |
| 2460600000 | All Adhesives and Sealants (controlled) | VOC | 1.07 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2460800000 | All FIFRA Related Products | VOC | 1.78 | LB/EACH | Pop. | EIIP, III:5, Table 5.4-1 |
| 2460800000 | All FIFRA Related Products (controlled) | VOC | 1.05 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2460500000 | All Coatings and Related Products | VOC | 0.95 | LB/EACH | Pop. | EIIP, III:5, Table 5.4-1 |
| 2460500000 | All Coatings and Related Products (controlled) | VOC | 0.56 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2460900000 | Misc. Products | VOC | 0.07 | LB/EACH | Pop. | EIIP, III:5, Table 5.4-1 |
| 2460900000 | Misc. Products (controlled) | VOC | 0,04 | LB/EACH | Pop. | Ozone Transport Commission, U.S. Census Bureau |
| 2420000000 | Dry Cleaning | VOC | 20.40 | LB/EACH | Emp. | Freedonia Group, U.S. Census Bureau* |
| 2420000000 | Dry Cleaning | 127184 | 118.35 | LB/EACH | Emp | Freedonia Group, U.S. Census Bureau* |

* Dry cleaning emissions factor assumes that 85 percent of dry cleaning solvents are perchloroethylene, which are not considered VOCs. (https://greenamerica.org/green-living/green-dry-cleaning)

# Controls

Some states have regulations that limit the VOC content of solvent-containing products that are sold. In this methodology, these controls are taken into account where appropriate by using the controlled emissions factors shown in Table 9. In particular, the emissions factors for architectural coatings and industrial maintenance coatings are reduced for the states listed in Table 10, based on calculations done for the 2011 National Emissions Inventory by the Eastern Regional Technical Advisory Committee (ERTAC).

In addition, EPA developed controlled emissions factors for the consumer solvent categories, including personal care products, household products, automotive aftermarket products, adhesives and sealants, FIFRA regulated products, coatings, and miscellaneous consumer products. The controlled emissions factors were taken from the Ozone Transport Commission, based on emissions factors for states that had implemented model rules for consumer solvents. Note that the Ozone Transport Commission includes a single emissions factor for all consumer solvents (5.15 lbs./person), while EPA uses individual emissions factors for each of the seven consumer solvent categories. To estimate controlled emissions factors for the individual solvent categories, the uncontrolled emissions factors were scaled so that the sum of the factors equaled 5.15 lbs./person.

Table 10. States for which controlled emissions factors are used.

| State | Architectural Coatings | Industrial Maintenance Coatings | Consumer Solvents |
| --- | --- | --- | --- |
| AZ | ü | ü |  |
| CA | ü | ü | ü |
| CT | ü | ü | ü |
| DE | ü | ü | ü |
| DC | ü | ü | ü |
| ME | ü | ü | ü |
| MD | ü | ü | ü |
| MA | ü | ü | ü |
| NH | ü | ü | ü |
| NJ | ü | ü | ü |
| NY | ü | ü | ü |
| PA | ü | ü | ü |
| RI | ü | ü | ü |
| TX | ü | ü |  |
| VT | ü | ü |  |
| VA | ü | ü | ü |

The solvent tool also allows users to adjust emissions factors to account for controls and to implement a county-level control factor.

# Emissions

Total VOC emissions from solvent utilization are calculated by multiplying the activity data for the source category by the calculated emissions factor for that category.

| EVOCc,s=Ac,s×EFVOC,s |  |
| --- | --- |

Where:

EVOC,c,s	=	Annual VOC emissions in county c for source category s, in tons per year

Ac,s	=	Activity data for county c associated with source category s

EFVOC,s	=	Calculated VOC emissions factor for source category s

HAP emissions are estimated using the VOC emissions and HAP speciation factors shown in Table 11. This step is completed after the point source subtraction step discussed in section H.

| Ep,c,s=EVOC,c,s×SFp,s |  |
| --- | --- |

Where:

Ep,c,s	=	Annual emissions of HAP p county c for source category s, in tons per year

EVOC,c,s	=	Annual VOC emissions in county c for source category s, in tons per year

SFp,s	=	Speciation factor for HAP p for source category s

Table 11. HAP speciation factors for solvent use.

| SCC | Pollutant Code | Pollutant Description | Speciation Factor |
| --- | --- | --- | --- |
| 2401001000 | 123911 | 1,4-Dioxane (1,4-Diethyleneoxide) | 0.00002 |
| 2401001000 | 584849 | 2,4-Toluene diisocyanate | 0.00002 |
| 2401001000 | 101688 | 4,4'-Methylenediphenyl diisocyanate (MDI) | 0.00014 |
| 2401001000 | 75070 | Acetaldehyde | 0.0001 |
| 2401001000 | 117817 | Bis(2-ethylhexyl)phthalate (DEHP) | 0.00003 |
| 2401001000 | 98828 | Cumene | 0.00038 |
| 2401001000 | 84742 | Dibutyl phthalate | 0.00002 |
| 2401001000 | 131113 | Dimethyl phthalate | 0.00001 |
| 2401001000 | 100414 | Ethylbenzene | 0.00248 |
| 2401001000 | 107211 | Ethylene glycol | 0.05049 |
| 2401001000 | 50000 | Formaldehyde | 0.00002 |
| 2401001000 | 171 | Glycol Ethers | 0.02065 |
| 2401001000 | 110543 | Hexane | 0.00015 |
| 2401001000 | 67561 | Methanol | 0.012184699 |
| 2401001000 | 80626 | Methyl methacrylate | 0.00012 |
| 2401001000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.000980163 |
| 2401001000 | 91203 | Naphthalene | 0.00046 |
| 2401001000 | 100425 | Styrene | 0.00102 |
| 2401001000 | 108883 | Toluene | 0.0397 |
| 2401001000 | 121448 | Triethylamine | 0.00006 |
| 2401001000 | 108054 | Vinyl acetate | 0.00012 |
| 2401001000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401005000 | 107211 | Ethylene glycol | 0.0016 |
| 2401005000 | 171 | Glycol Ethers | 0.00953 |
| 2401005000 | 108101 | Methyl isobutyl ketone (Hexone) | 0.0103 |
| 2401005000 | 108883 | Toluene | 0.018 |
| 2401005000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401015000 | 171 | Glycol Ethers | 0.01382 |
| 2401015000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401015000 | 108883 | Toluene | 0.0397 |
| 2401015000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401100000 | 171 | Glycol Ethers | 0.01382 |
| 2401100000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401100000 | 108883 | Toluene | 0.0397 |
| 2401100000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401200000 | 171 | Glycol Ethers | 0.01382 |
| 2401200000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401200000 | 108883 | Toluene | 0.0397 |
| 2401200000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401090000 | 171 | Glycol Ethers | 0.01382 |
| 2401090000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401090000 | 108883 | Toluene | 0.0397 |
| 2401090000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401080000 | 171 | Glycol Ethers | 0.01382 |
| 2401080000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401080000 | 108883 | Toluene | 0.0397 |
| 2401080000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401085000 | 171 | Glycol Ethers | 0.01382 |
| 2401085000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401085000 | 108883 | Toluene | 0.0397 |
| 2401085000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401075000 | 171 | Glycol Ethers | 0.01382 |
| 2401075000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0397 |
| 2401075000 | 108883 | Toluene | 0.0397 |
| 2401075000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401070000 | 171 | Glycol Ethers | 0.01382 |
| 2401070000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401070000 | 108883 | Toluene | 0.0397 |
| 2401070000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401065000 | 171 | Glycol Ethers | 0.01382 |
| 2401065000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401065000 | 108883 | Toluene | 0.0397 |
| 2401065000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401060000 | 171 | Glycol Ethers | 0.01382 |
| 2401060000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401060000 | 108883 | Toluene | 0.0397 |
| 2401060000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401055000 | 171 | Glycol Ethers | 0.01382 |
| 2401055000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401055000 | 108883 | Toluene | 0.0397 |
| 2401055000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401040000 | 171 | Glycol Ethers | 0.01382 |
| 2401040000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401040000 | 108883 | Toluene | 0.0397 |
| 2401040000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401030000 | 171 | Glycol Ethers | 0.01382 |
| 2401030000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401030000 | 108883 | Toluene | 0.0397 |
| 2401030000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401025000 | 171 | Glycol Ethers | 0.01382 |
| 2401025000 | 108101 | Methyl isobutyl ketone(Hexone) | 0.0103 |
| 2401025000 | 108883 | Toluene | 0.0397 |
| 2401025000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2401008000 | 108883 | Toluene | 0.0397 |
| 2401008000 | 1330207 | Xylenes (mixed isomers) | 0.0034 |
| 2415000000 | 108883 | Toluene | 0.078204196 |
| 2460900000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460900000 | 108883 | Toluene | 0.00268 |
| 2460800000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460800000 | 108883 | Toluene | 0.003221139 |
| 2460600000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460600000 | 108883 | Toluene | 0.003221139 |
| 2460500000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460500000 | 108883 | Toluene | 0.00268 |
| 2460400000 | 107211 | Ethylene Glycol | 0.1595 |
| 2460400000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460400000 | 108883 | Toluene | 0.00268 |
| 2460200000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460200000 | 108883 | Toluene | 0.003221139 |
| 2460100000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460100000 | 108883 | Toluene | 0.003529334 |
| 2460000000 | 67561 | Methyl Alcohol | 0.0933 |
| 2460000000 | 108883 | Toluene | 0.00268 |
| 2425000000 | 67561 | Methyl Alcohol | 0.02634987 |
| 2425000000 | 108101 | Methyl Isobutyl Ketone | 0.0004259 |
| 2425000000 | 108883 | Toluene | 0.0397 |
| 2425000000 | 1330207 | Xylene | 0.0034 |
| 2401015000 | 107211 | Ethylene glycol | 0.0045 |
| 2401100000 | 107211 | Ethylene glycol | 0.0045 |
| 2401200000 | 107211 | Ethylene glycol | 0.0045 |
| 2401090000 | 107211 | Ethylene glycol | 0.0045 |
| 2401080000 | 107211 | Ethylene glycol | 0.0045 |
| 2401085000 | 107211 | Ethylene glycol | 0.0045 |
| 2401075000 | 107211 | Ethylene glycol | 0.0045 |
| 2401070000 | 107211 | Ethylene glycol | 0.0045 |
| 2401065000 | 107211 | Ethylene glycol | 0.0045 |
| 2401060000 | 107211 | Ethylene glycol | 0.0045 |
| 2401055000 | 107211 | Ethylene glycol | 0.0045 |
| 2401040000 | 107211 | Ethylene glycol | 0.0045 |
| 2401030000 | 107211 | Ethylene glycol | 0.0045 |
| 2401025000 | 107211 | Ethylene glycol | 0.0045 |
| 2415000000 | 110543 | N-hexane | 0.000057282 |
| 2415000000 | 111773 | Methyl carbitol (2-(2-methoxyethoxy)ethanol) (degme) | 0.019346982 |
| 2415000000 | 112345 | 2-(2-butoxyethoxy)ethanol (butyl carbitol) | 0.03330946 |
| 2415000000 | 127184 | Perchloroethylene (Tetrachloroethylene) | 0.010597163 |
| 2415000000 | 1330207 | Xylenes (Mixed Isomers) | 0.087841886 |
| 2415000000 | 67561 | Methyl alcohol (methanol) | 0.050236279 |
| 2415000000 | 71432 | Benzene | 0.001432049 |
| 2415000000 | 71556 | 1,1,1-trichloroethane | 0.053014454 |
| 2415000000 | 75092 | Dichloromethane (methylene chloride) | 0.00614349 |
| 2415000000 | 79016 | Trichloroethylene | 0.030201913 |
| 2415000000 | 86748 | Carbazole | 0.001074037 |
| 2415000000 | 91203 | Naphthalene | 4.29615E-05 |
| 2415000000 | 98828 | Isopropylbenzene (or cumene; 2-Phenylpropane) | 4.29615E-05 |

# Point Source Subtraction

Point source subtraction is necessary for this category to ensure that solvent emissions are not double-counted with the point source inventory. In order to accomplish this, nonpoint source solvent SCCs must be linked to corresponding point SCCs, using point source emissions data supplied by state, local, or tribal (SLT) agencies and a point-nonpoint source crosswalk, shown in Table 14 in the appendix.

Point source subtraction should be completed at the county level using uncontrolled point source emissions.

| NPs,c=TEs,c×PSs,c |  |
| --- | --- |

Where:

NPs,c	=	Nonpoint source solvent emissions in county c for source category s, in tons per year

TEs,c	=	Total solvent emissions s in county c for source category s, in tons per year

PSs,c	=	Point source solvent emissions in county c for source category s, in tons per year

If county-level data is not available, state-level emissions can be allocated to the county level using population or employment data.

Note that if point source subtraction results in a negative number because the point source emissions from solvents are larger than the estimated total emissions from solvents, the Solvent Tool will zero out emissions for that source category in that county.

After point source subtraction, the HAP emissions are speciated from the estimated nonpoint source VOC emissions, as discussed in section G.

# Sample Calculations

Table 12 lists sample calculations to determine the VOC emissions from traffic coating solvent utilization in Apache County, Arizona.

Table 12. Sample calculations for VOC emissions from solvent utilization in Apache County, AZ.

| Eq. # | Equation | Values for Apache County, AZ | Result |
| --- | --- | --- | --- |
| 1 | PFracc=PcPst | 71,606 people in Apache County7,016,270 people in Arizona | 0.0102 share of the population of Arizona in Apache County |
| 2 | LMc=PFracc×LMst | 0.0102×144,959 lane miles in Arizona | 1,479 lane miles in Apache County |
| 3 | EFs=Fs×1,000,000As | N/A | Equation 3 is not used at this point in the method for traffic coatings |
| 4 | TSs,y=NAICSVSy | Product code 325510 is used for traffic coatings | 2010 value of shipments is 19,994,229 thousand USD. 2016 value of shipments is 27,445,132 thousand USD |
| 5 | TSs,2016c=TSs,2016×0.9075 | 27,445,132 thous. USD×0.9075 | Value of 2016 paint shipments in 2010 USD is 24,906,457 thousand USD |
| 6 | VPs,2016=TSs,2016c÷TSs,2010VPs,2010 | 24,906,457 thous. USD÷19,994,229 thous. USD1,301,333 thous. gal. paint | 1,621,048 thousand gallons of paint sold in 2016 |
| 7 | PRs=VPs,2016VPs,2010 | 1,621,048 thous. gal. ofpaint in 20161,301,333 thous. gal. of paint in 2010 | 1.246 ratio of 2016 to 2010 paint |
| 8 | VPs,2017=PRs×VPs,2010 | 1.246×37,335 thous. gal. traffic coatings sold in 2010 | 46,508 thousand gallons of traffic coatings sold in 2017 |
| 9 | NAC2017=(TC2010-AC2010-PC2010)×PRpaint and coatings | (1,301,333,355 gal.-651,626,800 gal. -75,774,600 gal.)×1.246 | 714,936,882 gallons of non-architectural coatings sold in 2017 |
| 10 | NAFracs=VPs,2017×1000NAC2017 | 46,508 thous. gal. traffic coatings×1000714,936,882 gal. non-arch. coatings | 6.5% of non-architectural coatings sold in 2017 are traffic coatings |
| 11 | SDs,2017=NAFracs×Oth2017 | 6.5%×1,320.80 mil. lbs. other solvent demand in 2017 | 85.92 million pounds of traffic coating solvent demand in 2017 |
| 3 | EFs=Fs×1,000,000As | 85.92mil. lbs. traffic coatings sold in 2017×1,000,0008,765,578 lane miles in 2017 | 9.80 pounds of VOC emitted per lane mile |
| 12 | EVOCc,s=Ac,s×EFVOC,s | 1,479 lane miles×9.80 lbs. VOC per lane mile | 14,498 lbs. of VOC emitted from traffic coatings in Apache County, AZ |

# Changes from 2014 Methodology

There are no significant changes from the methodology used to calculate the 2014 v2 NEI emissions.

# Puerto Rico and U.S. Virgin Islands Emissions Calculations

Emissions from Puerto Rico are calculated using the same method described above. For the U.S. Virgin Islands, emissions are calculated using 2010 population data, because 2016 Census Data does not exist for the U.S. Virgin Islands.

# Instructions for Submitting Point Emissions Data to Input Template

The Solvent Input Template includes a template for submitting point source emissions data. The template includes all point SCCs that map to nonpoint solvent SCCs. These point source emissions should be submitted at the county level. The template does not include all combinations of county FIPS codes and point SCCs in order to limit the file size. Rather, the template contains a single row for each point SCC that maps to a nonpoint solvent SCC. To enter emissions for more than one county, copy the necessary rows to the bottom of the table. There is no need to copy rows for which you will not be submitting point source data. The 5-digit FIPS code must be entered for all counties for which point source emissions data is being submitted.

Point source subtraction should be completed at the county level using uncontrolled point source emissions.  Note that if point source subtraction results in a negative number because the point source emissions from solvents are larger than the estimated total emissions from solvents, the Solvent Tool will zero out emissions for that source category in that county.

After point source subtraction, the HAP emissions are speciated from the estimated nonpoint source VOC emissions, as discussed in section G.

# References

# APPENDIX

Table 13. Source Category and Data Source Crosswalk for Solvent Utilization

| SCC | SCC Level 2 | SCC Level 3 | Freedonia | Paint and Allied Products | Annual Survey of Manufactures |
| --- | --- | --- | --- | --- | --- |
| 2401001000 | Surface Coating | Architectural Coatings | Paints & Coatings Solvent Demand: Architectural |  |  |
| 2401005000 | Surface Coating | Auto Refinishing | Paints & Coatings Solvent Demand: Other | Automotive, other transportation and machinery refinish paints and enamels, including primers | Paint and coating manufacturing |
| 2401008000 | Surface Coating | Traffic Markings | Paints & Coatings Solvent Demand: Other | Traffic marking paints (all types; shelf goods and highway department) | Paint and coating manufacturing |
| 2401015000 | Surface Coating | Factory Finished Wood | Paints & Coatings Solvent Demand: Other | Wood and composition board flat stock finishes | Wood Products |
| 2401020000 | Surface Coating | Wood Furniture | Paints & Coatings Solvent Demand: Other | Wood furniture, cabinet, and fixture finishes | Furniture |
| 2401025000 | Surface Coating | Metal Furniture | Paints & Coatings Solvent Demand: Other | Nonwood furniture and fixture finishes, including business equipment finishes | Furniture |
| 2401030000 | Surface Coating | Paper | Paints & Coatings Solvent Demand: Other | Paper, paper board, film, and foil finishes, excluding pigment binders | Paper bag and coated and treated paper manufacturing |
| 2401040000 | Surface Coating | Metal Cans | Paints & Coatings Solvent Demand: Other | Container and closure finishes | Metal Can and Container Manufacturing |
| 2401055000 | Surface Coating | Machinery and Equipment | Paints & Coatings Solvent Demand: Other | Machinery and equipment finishes, including road building equipment and farm implement | Machinery Manufacturing |
| 2401060000 | Surface Coating | Large Appliances | Paints & Coatings Solvent Demand: Other | Appliance, heating equipment, and air-conditioner finishes | Household Appliances |
| 2401065000 | Surface Coating | Electronics and Other Electrical | Paints & Coatings Solvent Demand: Other | Electrical insulating coatings | Electronics Manufacturing |
| 2401070000 | Surface Coating | Motor Vehicles | Paints & Coatings Solvent Demand: Other | Automobile, light truck, van, and sport utility vehicle finishes & Automobile parts finishes & Heavy duty truck, bus, and recreational vehicle finishes | Motor Vehicle Manufacturing |
| 2401075000 | Surface Coating | Aircraft | Paints & Coatings Solvent Demand: Other | Other transportation equipment finishes, including aircraft and railroad |  |
| 2401085000 | Surface Coating | Railroad | Paints & Coatings Solvent Demand: Other | Other transportation equipment finishes, including aircraft and railroad |  |
| 2401080000 | Surface Coating | Marine | Paints & Coatings Solvent Demand: Other | Marine paints, ship and off-shore facilities and shelf goods for both new construction and marine refinish and maintenance. Excludes spar varnish & Marine paints for yacht and pleasure craft, new construction, refinish and maintenance | Boat Manufacturing |
| 2401090000 | Surface Coating | Misc. Manufacturing | Paints & Coatings Solvent Demand: Other | Other industrial product finishes | Misc. Manufacturing |
| 2401100000 | Surface Coating | Industrial Maintenance Coatings | Paints & Coatings Solvent Demand: Other | Industrial new construction and maintenance paints, interior & Industrial new construction and maintenance paints, exterior | Paint and coating manufacturing |
| 2401200000 | Surface Coating | Other Special Purpose Coatings | Paints & Coatings Solvent Demand: Other |  |  |
| 2415000000 | Degreasing | All Processes/All Industries | Cleaning Products Solvent Demand: Industrial & Institutional |  |  |
| 2425000000 | Graphic Arts | All Processes | Printing Ink Solvent Demand |  |  |
| 2460100000 | Misc. Non-industrial: Consumer and Commercial | All Personal Care Products | Cosmetics & Toiletries Solvent Demand |  |  |
| 2460200000 | Misc. Non-industrial: Consumer and Commercial | All Household Products | Cleaning Products Solvent Demand: Household |  |  |
| 2460400000 | Misc. Non-industrial: Consumer and Commercial | All Automotive Aftermarket Products | Transportation Solvent Demand: Motor Vehicles |  |  |
| 2460600000 | Misc. Non-industrial: Consumer and Commercial | All Adhesives and Sealants | Adhesives & Sealants Solvent Demand |  |  |
| 2460800000 | Misc. Non-industrial: Consumer and Commercial | All FIFRA Related Products |  |  |  |
| 2460500000 | Misc. Non-industrial: Consumer and Commercial | All Coatings and Related Products |  |  |  |
| 2460900000 | Misc. Non-industrial: Consumer and Commercial | Misc. Products |  |  |  |
| 2420000000 | Dry Cleaning | All Processes |  |  |  |

Table 14. Point-Nonpoint Crosswalk for Solvent Utilization

| Point SCC | SCC Level One | SCC Level Two | SCC Level Three | SCC Level Four | Nonpoint SCC | Nonpoint Description |
| --- | --- | --- | --- | --- | --- | --- |
| 30101470 | Industrial Processes | Chemical Manufacturing | Paint Manufacture | Equipment Cleaning | 2415000000 | Solvent - Degreasing |
| 30101471 | Industrial Processes | Chemical Manufacturing | Paint Manufacture | Equipment Cleaning: Hand Wipe | 2415000000 | Solvent - Degreasing |
| 30101472 | Industrial Processes | Chemical Manufacturing | Paint Manufacture | Equipment Cleaning: Tanks, Vessels, etc. | 2415000000 | Solvent - Degreasing |
| 30102070 | Industrial Processes | Chemical Manufacturing | Printing Ink Manufacture | Equipment Cleaning | 2415000000 | Solvent - Degreasing |
| 30102071 | Industrial Processes | Chemical Manufacturing | Printing Ink Manufacture | Equipment Cleaning: Hand Wipe | 2415000000 | Solvent - Degreasing |
| 30102072 | Industrial Processes | Chemical Manufacturing | Printing Ink Manufacture | Equipment Cleaning: Tanks, Vessels, etc. | 2415000000 | Solvent - Degreasing |
| 30102426 | Industrial Processes | Chemical Manufacturing | Synthetic Organic Fiber | Equipment Cleanup | 2415000000 | Solvent - Degreasing |
| 30400340 | Industrial Processes | Secondary Metal Production | Grey Iron Foundries | Grinding/Cleaning | 2415000000 | Solvent - Degreasing |
| 30400342 | Industrial Processes | Secondary Metal Production | Grey Iron Foundries | Casting Cleaning/Chippers | 2415000000 | Solvent - Degreasing |
| 30400711 | Industrial Processes | Secondary Metal Production | Steel Foundries | Cleaning | 2415000000 | Solvent - Degreasing |
| 30400725 | Industrial Processes | Secondary Metal Production | Steel Foundries | Casting Cleaning/Tumblers | 2415000000 | Solvent - Degreasing |
| 30503701 | Industrial Processes | Mineral Products | Coated Abrasives Manufacturing | Printing of Backing | 2425000000 | Solvent - Graphic Arts |
| 30510005 | Industrial Processes | Mineral Products | Bulk Materials Elevators | Cleaning | 2415000000 | Solvent - Degreasing |
| 30701199 | Industrial Processes | Pulp and Paper and Wood Products | Paper Coating and Glazing | Extrusion Coating Line with Solvent Free Resin/Wax | 2401030000 | Surface Coating: Paper, Film and Foil |
| 30900301 | Industrial Processes | Fabricated Metal Products | Abrasive Cleaning of Metal Parts | Brush Cleaning | 2415000000 | Solvent - Degreasing |
| 30900302 | Industrial Processes | Fabricated Metal Products | Abrasive Cleaning of Metal Parts | Tumble Cleaning | 2415000000 | Solvent - Degreasing |
| 30901101 | Industrial Processes | Fabricated Metal Products | Conversion Coating of Metal Products | Alkaline Cleaning Bath | 2415000000 | Solvent - Degreasing |
| 30901102 | Industrial Processes | Fabricated Metal Products | Conversion Coating of Metal Products | Acid Cleaning Bath (Pickling) | 2415000000 | Solvent - Degreasing |
| 31303502 | Industrial Processes | Electrical Equipment | Manufacturing - General Processes | Cleaning | 2415000000 | Solvent - Degreasing |
| 31306501 | Industrial Processes | Electrical Equipment | Semiconductor Manufacturing | Cleaning Processes: Wet Chemical: Specify Aqueous Solution | 2415000000 | Solvent - Degreasing |
| 31306502 | Industrial Processes | Electrical Equipment | Semiconductor Manufacturing | Cleaning Process: Plasma Process: Specify Gas Used | 2415000000 | Solvent - Degreasing |
| 31401560 | Industrial Processes | Transportation Equipment | Boat Manufacturing | Cleanup | 2415000000 | Solvent - Degreasing |
| 31401561 | Industrial Processes | Transportation Equipment | Boat Manufacturing | Cleanup: Tools (Other than Spray Guns) | 2415000000 | Solvent - Degreasing |
| 31401562 | Industrial Processes | Transportation Equipment | Boat Manufacturing | Cleanup: Spray Guns | 2415000000 | Solvent - Degreasing |
| 31401563 | Industrial Processes | Transportation Equipment | Boat Manufacturing | Cleanup: Molds | 2415000000 | Solvent - Degreasing |
| 31401570 | Industrial Processes | Transportation Equipment | Boat Manufacturing | Waste Disposal: Used Cleanup Solvents | 2415000000 | Degreasing |
| 31612001 | Industrial Processes | Photographic Film Manufacturing | Support Activities - Cleaning Operations | Tank Cleaning Operations | 2415000000 | Solvent - Degreasing |
| 31612002 | Industrial Processes | Photographic Film Manufacturing | Support Activities - Cleaning Operations | General Cleaning Operations | 2415000000 | Solvent - Degreasing |
| 31612003 | Industrial Processes | Photographic Film Manufacturing | Support Activities - Cleaning Operations | Parts Cleaning Operations | 2415000000 | Solvent - Degreasing |
| 33000102 | Industrial Processes | Textile Products | Miscellaneous | Printing | 2425000000 | Solvent - Graphic Arts |
| 40100101 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Perchloroethylene | 2420000000 | Solvent - Dry Cleaning |
| 40100102 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Stoddard (Petroleum Solvent) (Use 4-10-001-01 or 4-10-002-01) | 2420000000 | Dry Cleaning |
| 40100104 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Stoddard (Petroleum Solvent) (Use 4-10-001-02 or 4-10-002-02) | 2420000000 | Dry Cleaning |
| 40100105 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Trichlorotrifluoroethane (Freon) | 2420000000 | Solvent - Dry Cleaning |
| 40100107 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Ethylene Oxide: General | 2420000000 | Solvent - Dry Cleaning |
| 40100146 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Stoddard:Filtr Disp/Cooked Muck(Drained) (Use 4-10-001-61 or 002-61) | 2420000000 | Solvent - Dry Cleaning |
| 40100147 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Stoddard:Filtr Disp/Cooked Muck (Centrif) (Use 4-10-001-62 or 002-62) | 2420000000 | Solvent - Dry Cleaning |
| 40100160 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Trichlorofluoroethane: Washer/Dryer/Still | 2420000000 | Solvent - Dry Cleaning |
| 40100161 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Trichlorofluoroethane: Cartrige Filter Disposal | 2420000000 | Solvent - Dry Cleaning |
| 40100162 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Trichlorofluoroethane: Still Residue Disposal | 2420000000 | Solvent - Dry Cleaning |
| 40100163 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Trichlorofluoroethane: Miscellaneous Fugitive | 2420000000 | Solvent - Dry Cleaning |
| 40100198 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Other Not Classified | 2420000000 | Solvent - Dry Cleaning |
| 40100198 | Chemical Evaporation | Organic Solvent Evaporation | Dry Cleaning | Other Not Classified | 2420000000 | Dry Cleaning |
| 40100201 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Stoddard (Petroleum Solvent): Open-top Vapor Degreasing | 2415000000 | Degreasing |
| 40100202 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | 1,1,1-Trichloroethane (Methyl Chloroform): Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100203 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Perchloroethylene: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100204 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Methylene Chloride: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100205 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Trichloroethylene: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100206 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Toluene: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100207 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Trichlorotrifluoroethane (Freon): Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100208 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Chlorosolve: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100209 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Butyl Acetate: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100215 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Entire Unit: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100221 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Stoddard (Petroleum Solvent): Conveyorized Vapor Degreasing | 2415000000 | Degreasing |
| 40100222 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | 1,1,1-Trichloroethane (Methyl Chloroform):Conveyorized Vapor Degreaser | 2415000000 | Solvent - Degreasing |
| 40100223 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Perchloroethylene: Conveyorized Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100224 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Methylene Chloride: Conveyorized Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100225 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Trichloroethylene: Conveyorized Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100235 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Entire Unit: with Vaporized Solvent: Conveyorized Vapor Degreasing | 2415000000 | Degreasing |
| 40100236 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Entire Unit: with Non-boiling Solvent: Conveyorized Vapor Degreasing | 2415000000 | Degreasing |
| 40100251 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Stoddard (Petroleum Solvent): General Degreasing Units | 2415000000 | Degreasing |
| 40100252 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | 1,1,1-Trichloroethane (Methyl Chloroform): General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100253 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Perchloroethylene: General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100254 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Methylene Chloride: General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100255 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Trichloroethylene: General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100256 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Toluene: General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100257 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Trichlorotrifluoroethane (Freon): General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100258 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Trichlorofluoromethane: General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100296 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Other Not Classified: General Degreasing Units | 2415000000 | Solvent - Degreasing |
| 40100298 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Other Not Classified: Conveyorized Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100299 | Chemical Evaporation | Organic Solvent Evaporation | Degreasing | Other Not Classified: Open-top Vapor Degreasing | 2415000000 | Solvent - Degreasing |
| 40100301 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Methanol | 2415000000 | Degreasing |
| 40100302 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Methylene Chloride | 2415000000 | Degreasing |
| 40100303 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Stoddard (Petroleum Solvent) | 2415000000 | Degreasing |
| 40100304 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Perchloroethylene | 2415000000 | Degreasing |
| 40100305 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | 1,1,1-Trichloroethane (Methyl Chloroform) | 2415000000 | Degreasing |
| 40100306 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Trichloroethylene | 2415000000 | Degreasing |
| 40100307 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Isopropyl Alcohol | 2415000000 | Degreasing |
| 40100308 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Methyl Ethyl Ketone | 2415000000 | Degreasing |
| 40100309 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Freon | 2415000000 | Degreasing |
| 40100310 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Acetone | 2415000000 | Degreasing |
| 40100311 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Glycol Ethers | 2415000000 | Degreasing |
| 40100335 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Entire Unit | 2415000000 | Degreasing |
| 40100336 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Degreaser: Entire Unit | 2415000000 | Degreasing |
| 40100399 | Chemical Evaporation | Organic Solvent Evaporation | Cold Solvent Cleaning/Stripping | Other Not Classified | 2415000000 | Degreasing |
| 40100401 | Chemical Evaporation | Organic Solvent Evaporation | Knit Fabric Scouring with Chlorinated Solvent | Perchloroethylene | 2415000000 | Degreasing |
| 40100499 | Chemical Evaporation | Organic Solvent Evaporation | Knit Fabric Scouring with Chlorinated Solvent | Other Not Classified | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40188898 | Chemical Evaporation | Organic Solvent Evaporation | Fugitive Emissions | General | 2415000000 | Degreasing |
| 40200101 | Chemical Evaporation | Surface Coating Operations | Surface Coating Application - General | Paint: Solvent-base | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200201 | Chemical Evaporation | Surface Coating Operations | Surface Coating Application - General | Paint: Water-base | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200706 | Chemical Evaporation | Surface Coating Operations | Surface Coating Application - General | Adhesive: Solvent Mixing | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200710 | Chemical Evaporation | Surface Coating Operations | Surface Coating Application - General | Adhesive: General | 2460600000 | Adhesives and sealants |
| 40200801 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | General | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200801 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | General | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200802 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Dried < 175F | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200803 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Baked : 175F | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200820 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Prime/Base Coat Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200830 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Topcoat Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200840 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Two Piece Can Curing Ovens: General (Includes Codes 41, 42, and 43) | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200841 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Two Piece Can Base Coat Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200842 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Two Piece Can Over Varnish Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200843 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Two Piece Can Interior Body Coat Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200845 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Three Piece Can Curing Ovens (Includes Codes 46, 47, 48, and 49) | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200846 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Three Piece Can Sheet Base Coat (Interior) Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200847 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Three Piece Can Sheet Base Coat (Exterior) Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200848 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Three Piece Can Sheet Lithographic Coating Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200849 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Three Piece Can Interior Body Coat Oven | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40200855 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Filler Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200856 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Sealer Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200861 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Single Coat Application: Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200870 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Color Coat Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200871 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | Topcoat/Texture Coat Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200872 | Chemical Evaporation | Surface Coating Operations | Coating Oven - General | EMI/RFI Shielding Coat Oven | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40200901 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | General | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200902 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Acetone | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200903 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Butyl Acetate | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200904 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Butyl Alcohol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200905 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Carbitol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200906 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Cellosolve | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200907 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Cellosolve Acetate | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200908 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Dimethyl Formamide | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200909 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Ethyl Acetate | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200910 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Ethyl Alcohol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200911 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Gasoline | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200912 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Isopropyl Alcohol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200913 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Isopropyl Acetate | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200914 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Kerosene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200915 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Lactol Spirits | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200916 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Methyl Acetate | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200917 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Methyl Alcohol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200918 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Methyl Ethyl Ketone | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200919 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Methyl Isobutyl Ketone | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200920 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Mineral Spirits | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200921 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Naphtha | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200922 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Toluene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200923 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Varsol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200924 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Xylene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200925 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Benzene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200926 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Turpentine | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200927 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Hexylene Glycol | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200928 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Ethylene Oxide | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200929 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | 1,1,1-Trichloroethane (Methyl Chloroform) | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200930 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Methylene Chloride | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40200931 | Chemical Evaporation | Surface Coating Operations | Thinning Solvents - General | Perchloroethylene | 2415000000 | Degreasing |
| 40201101 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Coating Operation (Also See Specific Coating Method Codes 4-02-04X) | 2425000000 | Solvent - Graphic Arts |
| 40201103 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Coating Mixing (Also See Specific Coating Method Codes 4-02-04X) | 2425000000 | Solvent - Graphic Arts |
| 40201104 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Coating Storage (Also See Specific Coating Method Codes 4-02-04X) | 2425000000 | Solvent - Graphic Arts |
| 40201105 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Equipment Cleanup:Fabric Coating(Also Spec Coat Method Codes 4-02-04X) | 2415000000 | Solvent - Degreasing |
| 40201112 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Fabric Printing: Roller (Also See New Codes Under 4-02-040-XX) | 2425000000 | Solvent - Graphic Arts |
| 40201114 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Fabric Printing: Rotary Screen (Also See New Codes Under 4-02-040-XX) | 2425000000 | Solvent - Graphic Arts |
| 40201116 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Fabric Printing: Flat Screen (Also See New Codes Under 4-02-040-XX) | 2425000000 | Solvent - Graphic Arts |
| 40201121 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Fabric Print:Dryer: Steam Coil (Also See New Codes Under 4-02-040-XX) | 2425000000 | Solvent - Graphic Arts |
| 40201122 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Fabric Print:Dryer: Fuel-fired (Also See New Codes Under 4-02-040- XX) | 2425000000 | Solvent - Graphic Arts |
| 40201197 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Misc. Fugitives (Also New Codes 4-02-040-XX) | 2425000000 | Solvent - Graphic Arts |
| 40201199 | Chemical Evaporation | Surface Coating Operations | Fabric Coating/Printing | Other Not Classified (Also See New Codes Under 4-02-040-XX) | 2425000000 | Solvent - Graphic Arts |
| 40201301 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Coating Operation | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201303 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Coating Mixing | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201304 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Coating Storage | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201305 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Equipment Cleanup | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201310 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Coating Application: Knife Coater | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201320 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Coating Application: Reverse Roll Coater | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201330 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Coating Application: Rotogravure Printer | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201399 | Chemical Evaporation | Surface Coating Operations | Paper Coating | Other Not Classified | 2401030000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201401 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Coating Operation | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201402 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Cleaning/Pretreatment | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201403 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Coating Mixing | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201404 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Coating Storage | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201405 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Equipment Cleanup | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201406 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Topcoat Spray | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201410 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Coat Flashoff | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201411 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Topcoat Flashoff | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201431 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Coating Line: General | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201432 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Air Spray | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201433 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Electrostatic Spray | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201434 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Flow Coat | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201435 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Dip Coat | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201436 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Prime Electro-deposition | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201437 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Top Air Spray | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201438 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Top Electrostatic Spray | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201499 | Chemical Evaporation | Surface Coating Operations | Large Appliances | Other Not Classified | 2401060000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201501 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Coating/Application/Curing | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201502 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Cleaning/Pretreatment | 2415000000 | Solvent - Degreasing |
| 40201503 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Coating Mixing | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201504 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Coating Storage | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201505 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Equipment Cleanup | 2415000000 | Solvent - Degreasing |
| 40201531 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Coating Line: General | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201599 | Chemical Evaporation | Surface Coating Operations | Magnet Wire Surface Coating | Other Not Classified | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201601 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Prime Application/Electo-deposition/Dip/Spray | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201602 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Cleaning/Pretreatment | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201603 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Coating Mixing | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201604 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Coating Storage | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201605 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Equipment Cleanup | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201606 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Topcoat Operation | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201607 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Sealers | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201608 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Deadeners | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201609 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Anti-corrosion Priming | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201619 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Prime Surfacing Operation | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201620 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Repair Topcoat Application Area | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201621 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Prime Coating: Solvent-borne - Automobiles | 2401070000 | Surface Coating: Motor Vehicles |
| 40201622 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Prime Coating: Electro-deposition - Automobiles | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201623 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Guide Coating: Solvent-borne - Automobiles | 2401070000 | Surface Coating: Motor Vehicles |
| 40201624 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Guide Coating: Water-borne - Automobiles | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201625 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Topcoat: Solvent-borne - Automobiles | 2401070000 | Surface Coating: Motor Vehicles |
| 40201626 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Topcoat: Water-borne - Automobiles | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201627 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Prime Coating: Solvent-borne - Light Trucks | 2401070000 | Surface Coating: Motor Vehicles |
| 40201628 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Prime Coating: Electro-deposition - Light Trucks | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201629 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Guide Coating: Solvent-borne - Light Trucks | 2401070000 | Surface Coating: Motor Vehicles |
| 40201630 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Guide Coating: Water-borne - Light Trucks | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201631 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Topcoat: Solvent-borne - Light Trucks | 2401070000 | Surface Coating: Motor Vehicles |
| 40201632 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Topcoat: Water-borne - Light Trucks | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201699 | Chemical Evaporation | Surface Coating Operations | Automobiles and Light Trucks | Other Not Classified | 2401070000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201702 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Cleaning/Pretreatment | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201703 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Coating Mixing | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201704 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Coating Storage | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201705 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Equipment Cleanup | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201706 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Solvent Storage | 2401040000 | Surface Coating: Metal Cans |
| 40201721 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Two Piece Exterior Base Coating | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201722 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Interior Spray Coating | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201723 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Sheet Base Coating (Interior) | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201724 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Sheet Base Coating (Exterior) | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201725 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Side Seam Spray Coating | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201726 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | End Sealing Compound (Also See 4-02-017-36 & -37) | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201727 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Lithography | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201728 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Over Varnish | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201729 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Exterior End Coating | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201731 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Three-piece Can Sheet Base Coating | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201732 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Three-piece Can Sheet Lithographic Coating Line | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201733 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Three-piece Can-side Seam Spray Coating | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201734 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Three-piece Can Interior Body Spray Coat | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201735 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Two-piece Can Coating Line | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201736 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Two-piece Can End Sealing Compound | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201737 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Three Piece Can End Sealing Compound | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201738 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Two Piece Can Lithographic Coating Line | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201739 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Three Piece Can Coating Line (All Coating Solvent Emission Points) | 2401040000 | Surface Coating: Metal Cans |
| 40201799 | Chemical Evaporation | Surface Coating Operations | Metal Can Coating | Other Not Classified | 2401040000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201801 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Prime Coating Application | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40201802 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Cleaning/Pretreatment | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201803 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Solvent Mixing | 2401055000 | Surface Coating - Machinery and Equipment: SIC 37 |
| 40201804 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Solvent Storage (Use 4-07-004-01 thru 4-07-999-98 if possible) | 2401055000 | Surface Coating - Machinery and Equipment: SIC 38 |
| 40201805 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Equipment Cleanup | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201806 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Finish Coating | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201807 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Coating Storage | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201899 | Chemical Evaporation | Surface Coating Operations | Metal Coil Coating | Other Not Classified | 2401055000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201901 | Chemical Evaporation | Surface Coating Operations | Wood Furniture Surface Coating | Coating Operation | 2401020000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201903 | Chemical Evaporation | Surface Coating Operations | Wood Furniture Surface Coating | Coating Mixing | 2401020000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201904 | Chemical Evaporation | Surface Coating Operations | Wood Furniture Surface Coating | Coating Storage | 2401020000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40201999 | Chemical Evaporation | Surface Coating Operations | Wood Furniture Surface Coating | Other Not Classified | 2401020000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202001 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Coating Operation | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202002 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Cleaning/Pretreatment | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202003 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Coating Mixing | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202004 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Coating Storage | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202005 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Equipment Cleanup | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202010 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Prime Coat Application | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202011 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Prime Coat Application: Spray, High Solids | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202012 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Prime Coat Application: Spray, Water-borne | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202013 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Prime Coat Application: Dip | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202014 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Prime Coat Application: Flow Coat | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202015 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Prime Coat Application: Flashoff | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202020 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Topcoat Application | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202021 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Topcoat Application: Spray, High Solids | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202022 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Topcoat Application: Spray, Water-borne | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202023 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Topcoat Application: Dip | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202024 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Topcoat Application: Flow Coat | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202025 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Topcoat Application: Flashoff | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202031 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Single Spray Line: General | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202032 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Spray Dip Line: General (Use 4-02-020-37) | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202033 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Spray High Solids Coating (Use 4-02-020-35) | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202034 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Spray Water-borne Coating (Use 4-02-020-36) | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202035 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Single Coat Application: Spray, High Solids | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202036 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Single Coat Application: Spray, Water-borne | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202037 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Single Coat Application: Dip | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202038 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Single Coat Application: Flow Coat | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202039 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Single Coat Application: Flashoff | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202099 | Chemical Evaporation | Surface Coating Operations | Metal Furniture Operations | Other Not Classified | 2401025000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202101 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Base Coat | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202103 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Coating Mixing | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202104 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Coating Storage | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202105 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Equipment Cleanup | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202106 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Topcoat | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202107 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Filler | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202108 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Sealer | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202109 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Inks | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202110 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Grove Coat Application | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202111 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Stain Application | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202117 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Filler Sander | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202118 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Sealer Sander | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202131 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Water-borne Coating | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202132 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Solvent-borne Coating | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202133 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Ultraviolet Coating | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202140 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Surface Preparation (Includes Tempering, Sanding, Brushing, Grove Cut) | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202199 | Chemical Evaporation | Surface Coating Operations | Flatwood: Wood Building Products | Other Not Elsewhere Classified | 2401015000 | Surface Coating: Wood Products Manfacturing |
| 40202201 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Coating Operation | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202202 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Cleaning/Pretreatment | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202203 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Coating Mixing | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202204 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Coating Storage | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202205 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Equipment Cleanup | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202206 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Baseline Coating Mix | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202207 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Low Solids Solvent-borne Coating | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40202208 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Medium Solids Solvent-borne Coating | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40202209 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: High Solids Coating (25% Efficiency) | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202210 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: High Solids Solvent-borne Coating (40% Efficiency) | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40202211 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Water-borne Coating | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202212 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Low Solids Solvent-borne EMI/RFI Shielding Coating | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40202213 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Higher Solids Solvent-borne EMI/RFI Shielding Coating | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40202214 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Water-borne EMI/RFI Shielding Coating | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202215 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Business: Zinc Arc Spray | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202220 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Prime Coat Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202229 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Prime Coat Flashoff | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202230 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Color Coat Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202239 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Color Coat Flashoff | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202240 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Topcoat/Texture Coat Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202249 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Topcoat/Texture Coat Flashoff | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202250 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | EMI/RFI Shielding Coat Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202259 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | EMI/RFI Shielding Coat Flashoff | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202270 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Sanding/Grit Blasting Prior to EMI/RFI Shielding Coat Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202280 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Maskant Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202299 | Chemical Evaporation | Surface Coating Operations | Plastic Parts | Other Not Classified | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202301 | Chemical Evaporation | Surface Coating Operations | Large Ships | Prime Coating Operation | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202302 | Chemical Evaporation | Surface Coating Operations | Large Ships | Cleaning/Pretreatment | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202303 | Chemical Evaporation | Surface Coating Operations | Large Ships | Coating Mixing | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202304 | Chemical Evaporation | Surface Coating Operations | Large Ships | Coating Storage | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202305 | Chemical Evaporation | Surface Coating Operations | Large Ships | Equipment Cleanup | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202306 | Chemical Evaporation | Surface Coating Operations | Large Ships | Topcoat Operation | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202399 | Chemical Evaporation | Surface Coating Operations | Large Ships | Other Not Classified | 2401080000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202401 | Chemical Evaporation | Surface Coating Operations | Aerospace | Primer Application | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202402 | Chemical Evaporation | Surface Coating Operations | Aerospace | Cleaning/Pretreatment | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202403 | Chemical Evaporation | Surface Coating Operations | Aerospace | Coating Mixing | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202404 | Chemical Evaporation | Surface Coating Operations | Aerospace | Coating Storage | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202405 | Chemical Evaporation | Surface Coating Operations | Aerospace | Equipment Cleanup | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202406 | Chemical Evaporation | Surface Coating Operations | Aerospace | Topcoat Application | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202499 | Chemical Evaporation | Surface Coating Operations | Large Aircraft | Other Not Classified | 2401075000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202501 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Coating Operation | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202502 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Cleaning/Pretreatment | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202503 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Coating Mixing | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202504 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Coating Storage | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202505 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Equipment Cleanup | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202510 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Prime Coat Application | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202511 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Prime Coat Application: Spray, High Solids | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202512 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Prime Coat Application: Spray, Water-borne | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202515 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Prime Coat Application: Flashoff | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202520 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Topcoat Application | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202521 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Topcoat Application: Spray, High Solids | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202522 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Topcoat Application: Spray, Water-borne | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202523 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Topcoat Application: Dip | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202524 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Topcoat Application: Flow Coat | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202525 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Topcoat Application: Flashoff | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202531 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Conveyor Single Flow | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202532 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Conveyor Single Dip | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202533 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Conveyor Single Spray | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202534 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Conveyor Two Coat, Flow and Spray | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202535 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Conveyor Two Coat, Dip and Spray | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202536 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Conveyor Two Coat, Spray | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202537 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Manual Two Coat, Spray and Air Dry | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202542 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Single Coat Application: Spray, High Solids | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202543 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Single Coat Application: Spray, Water-borne | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202544 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Single Coat Application: Dip | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202545 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Single Coat Application: Flow Coat | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202546 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Single Coat Application: Flashoff | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202599 | Chemical Evaporation | Surface Coating Operations | Miscellaneous Metal Parts | Other Not Classified | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40202602 | Chemical Evaporation | Surface Coating Operations | Steel Drums | Cleaning/Pretreatment | 2415000000 | Solvent - Degreasing |
| 40202605 | Chemical Evaporation | Surface Coating Operations | Steel Drums | Equipment Cleanup | 2415000000 | Solvent - Degreasing |
| 40202701 | Chemical Evaporation | Surface Coating Operations | Glass Mirrors | Mirror Backing: Coating Operation | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202801 | Chemical Evaporation | Surface Coating Operations | Glass Optical Fibers | Chemical Vapor Deposition of Preforms | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202802 | Chemical Evaporation | Surface Coating Operations | Glass Optical Fibers | Plasma Overcladding | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40202899 | Chemical Evaporation | Surface Coating Operations | Glass Optical Fibers | Miscellaneous | 2401065000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40203001 | Chemical Evaporation | Surface Coating Operations | Semiconductors | Solvent | 2401065000 | Surface Coating: Electronic and Other Electrical Coatings |
| 40204001 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Roller: Print Paste | 2425000000 | Solvent - Graphic Arts |
| 40204002 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Roller: Application | 2425000000 | Solvent - Graphic Arts |
| 40204003 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Roller: Transfer | 2425000000 | Solvent - Graphic Arts |
| 40204004 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Roller: Steam Cans/Drying | 2425000000 | Solvent - Graphic Arts |
| 40204010 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Rotary Screen: Print Paste | 2425000000 | Solvent - Graphic Arts |
| 40204011 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Rotary Screen: Application | 2425000000 | Solvent - Graphic Arts |
| 40204012 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Rotary Screen: Transfer | 2425000000 | Solvent - Graphic Arts |
| 40204013 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Rotary Screen: Drying/Curing | 2425000000 | Solvent - Graphic Arts |
| 40204020 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Flat Screen: Print Paste | 2425000000 | Solvent - Graphic Arts |
| 40204021 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Flat Screen: Application | 2425000000 | Solvent - Graphic Arts |
| 40204022 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Flat Screen: Transfer | 2425000000 | Solvent - Graphic Arts |
| 40204023 | Chemical Evaporation | Surface Coating Operations | Fabric Printing | Flat Screen: Drying/Curing | 2425000000 | Solvent - Graphic Arts |
| 40204121 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204130 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204140 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Drying/Curing | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204150 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204151 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204152 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204160 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204161 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204162 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Knife Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204221 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204230 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204240 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Drying/Curing | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204250 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204251 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204252 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204260 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204261 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204262 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Roller Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204321 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204330 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204340 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Drying/Curing | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204350 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204351 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204352 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204360 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204361 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204362 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Dip Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204421 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204430 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204431 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Coating Application: First Roll Applicator | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204432 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Coating Application: Second Roll Applicator | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204435 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Lamination: Laminating Device | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204440 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Drying/Curing | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204441 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Drying/Curing: First Predrier | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204442 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Drying/Curing: Second Predrier | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204443 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Drying/Curing: Main Drying Tunnel | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204450 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Cooler | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204455 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Winding | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204460 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204461 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204462 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204470 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204471 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204472 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Transfer Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204521 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204530 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204531 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Coating Application: Extruder | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204532 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Coating Application: Coating Die | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204550 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Cooling Cylinder | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204555 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Winding | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204560 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204561 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204562 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204570 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204571 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204572 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Extrusion Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204621 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204630 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204631 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Coating Application: Calendar Rolls | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204632 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Coating Application: Pick Up Roll | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204650 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Cooling Rolls | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204655 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Winding | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204660 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204661 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204662 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204670 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204671 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204672 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Melt Roll Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204721 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Mixing Tanks | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204730 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Coating Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204735 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Coagulation Baths and Solvent Separation | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40204740 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Solvent Recovery | 2401090000 | Surface Coating: Misc. Manufacturing |
| 40204750 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Drying | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204755 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Winding | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204760 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Cleanup | 2415000000 | Solvent - Degreasing |
| 40204761 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Cleanup: Coating Application Equipment | 2415000000 | Solvent - Degreasing |
| 40204762 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Cleanup: Empty Coating Drums | 2415000000 | Solvent - Degreasing |
| 40204770 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40204771 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Waste: Cleaning Rags | 2415000000 | Solvent - Degreasing |
| 40204772 | Chemical Evaporation | Surface Coating Operations | Fabric Coating, Coagulation Coating | Waste: Waste Ink Disposal | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206010 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Preparation | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206030 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Application | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206031 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Application: Beam | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206032 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Application: Beck | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206033 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Application: Jig | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206034 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Application: Jet | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206035 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Dye Application: Continuous | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40206050 | Chemical Evaporation | Surface Coating Operations | Fabric Dyeing | Waste | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40282001 | Chemical Evaporation | Surface Coating Operations | Wastewater, Aggregate | Process Area Drains | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40282002 | Chemical Evaporation | Surface Coating Operations | Wastewater, Aggregate | Process Equipment Drains | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40282501 | Chemical Evaporation | Surface Coating Operations | Wastewater, Points of Generation | Printing Blanket, Rotary Screen | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40282599 | Chemical Evaporation | Surface Coating Operations | Wastewater, Points of Generation | Other Not Classified | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40288801 | Chemical Evaporation | Surface Coating Operations | Fugitive Emissions | General | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40288821 | Chemical Evaporation | Surface Coating Operations | Fugitive Emissions | Basecoat | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40288822 | Chemical Evaporation | Surface Coating Operations | Fugitive Emissions | Coating | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40288823 | Chemical Evaporation | Surface Coating Operations | Fugitive Emissions | Cleartop Coat | 2401055000 | Surface Coating - Machinery and Equipment: SIC 35 |
| 40288824 | Chemical Evaporation | Surface Coating Operations | Fugitive Emissions | Clean-up | 2415000000 | Solvent - Degreasing |
| 40299998 | Chemical Evaporation | Surface Coating Operations | Miscellaneous | Miscellaneous | 2401090000 | Solvent - Industrial Surface Coating & Solvent Use |
| 40500201 | Chemical Evaporation | Printing/Publishing | Letterpress | Printing | 2425000000 | Solvent - Graphic Arts |
| 40500204 | Chemical Evaporation | Printing/Publishing | Letterpress | Dryer | 2425000000 | Solvent - Graphic Arts |
| 40500205 | Chemical Evaporation | Printing/Publishing | Letterpress | Other non-dryer printing | 2425000000 | Solvent - Graphic Arts |
| 40500301 | Chemical Evaporation | Printing/Publishing | Flexographic | Printing | 2425000000 | Solvent - Graphic Arts |
| 40500308 | Chemical Evaporation | Printing/Publishing | Flexographic | Dryer | 2425000000 | Solvent - Graphic Arts |
| 40500309 | Chemical Evaporation | Printing/Publishing | Flexographic | Other non-dryer printing | 2425000000 | Solvent - Graphic Arts |
| 40500402 | Chemical Evaporation | Printing/Publishing | Lithographic | Dryer | 2425000000 | Solvent - Graphic Arts |
| 40500403 | Chemical Evaporation | Printing/Publishing | Lithographic | Other non-dryer printing | 2425000000 | Solvent - Graphic Arts |
| 40500515 | Chemical Evaporation | Printing/Publishing | Rotogravure | Dryer | 2425000000 | Solvent - Graphic Arts |
| 40500516 | Chemical Evaporation | Printing/Publishing | Rotogravure | Other non-dryer printing | 2425000000 | Solvent - Graphic Arts |
| 40500803 | Chemical Evaporation | Printing/Publishing | Screen Printing | Dryer | 2425000000 | Solvent - Graphic Arts |
| 40500804 | Chemical Evaporation | Printing/Publishing | Screen Printing | Other non-dryer printing | 2425000000 | Solvent - Graphic Arts |
| 40500805 | Chemical Evaporation | Printing/Publishing | Digital Printing | Dryer | 2425000000 | Solvent - Graphic Arts |
| 40500806 | Chemical Evaporation | Printing/Publishing | Digital Printing | Other non-dryer printing | 2425000000 | Solvent - Graphic Arts |
| 40600259 | Chemical Evaporation | Transportation and Marketing of Petroleum Products | Marine Vessels | Tanker/Barge Cleaning | 2415000000 | Solvent - Degreasing |
| 41000101 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Stoddard | 2420000000 | Dry Cleaning |
| 41000115 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Washer/Extractor | 2420000000 | Dry Cleaning |
| 41000125 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Solvent Settling Tank: Batch Flow | 2420000000 | Dry Cleaning |
| 41000126 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Solvent Settling Tank: Continuous Flow | 2420000000 | Dry Cleaning |
| 41000130 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Dryer | 2420000000 | Dry Cleaning |
| 41000131 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Dryer: Loading/Unloading | 2420000000 | Dry Cleaning |
| 41000132 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Dryer: Drying Cycle | 2420000000 | Dry Cleaning |
| 41000133 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Dryer: Cool Down Cycle | 2420000000 | Dry Cleaning |
| 41000140 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration | 2420000000 | Dry Cleaning |
| 41000141 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Diatomite: Single Charge | 2420000000 | Dry Cleaning |
| 41000142 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Diatomite: Multiple Charge | 2420000000 | Dry Cleaning |
| 41000143 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Diatomite: Regenerative | 2420000000 | Dry Cleaning |
| 41000144 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Cartridge, Carbon Core, Batch Operation | 2420000000 | Dry Cleaning |
| 41000145 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Cartridge, All Carbon, Batch Operation | 2420000000 | Dry Cleaning |
| 41000146 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Cartridge, Carbon Core, Continuous Operation | 2420000000 | Dry Cleaning |
| 41000147 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Filtration, Cartridge, All Carbon, Continuous Operation | 2420000000 | Dry Cleaning |
| 41000160 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal | 2420000000 | Dry Cleaning |
| 41000161 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal: Filter Waste, Drained | 2420000000 | Dry Cleaning |
| 41000162 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal: Filter Waste, Centrifuged | 2420000000 | Dry Cleaning |
| 41000163 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal: Settling Tank Sludge | 2420000000 | Dry Cleaning |
| 41000164 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal: Still Waste | 2420000000 | Dry Cleaning |
| 41000165 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal: Cartridge, All Carbon | 2420000000 | Dry Cleaning |
| 41000166 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Industrial | Waste Disposal: Cartridge, Carbon Core Only | 2420000000 | Dry Cleaning |
| 41000202 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Stoddard | 2420000000 | Dry Cleaning |
| 41000215 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Washer/Extractor | 2420000000 | Dry Cleaning |
| 41000225 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Solvent Settling Tank: Batch Flow | 2420000000 | Dry Cleaning |
| 41000226 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Solvent Settling Tank: Continuous Flow | 2420000000 | Dry Cleaning |
| 41000230 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Dryer | 2420000000 | Dry Cleaning |
| 41000231 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Dryer: Loading/Unloading | 2420000000 | Dry Cleaning |
| 41000232 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Dryer: Drying Cycle | 2420000000 | Dry Cleaning |
| 41000233 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Dryer: Cool Down Cycle | 2420000000 | Dry Cleaning |
| 41000240 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration | 2420000000 | Dry Cleaning |
| 41000241 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Diatomite: Single Charge | 2420000000 | Dry Cleaning |
| 41000242 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Diatomite: Multiple Charge | 2420000000 | Dry Cleaning |
| 41000243 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Diatomite: Regenerative | 2420000000 | Dry Cleaning |
| 41000244 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Cartridge, Carbon Core, Batch Operation | 2420000000 | Dry Cleaning |
| 41000245 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Cartridge, All Carbon, Batch Operation | 2420000000 | Dry Cleaning |
| 41000246 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Cartridge, Carbon Core, Continuous Operation | 2420000000 | Dry Cleaning |
| 41000247 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Filtration, Cartridge, All Carbon, Continuous Operation | 2420000000 | Dry Cleaning |
| 41000260 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal | 2420000000 | Dry Cleaning |
| 41000261 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal: Filter Waste, Drained | 2420000000 | Dry Cleaning |
| 41000262 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal: Filter Waste, Centrifuged | 2420000000 | Dry Cleaning |
| 41000263 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal: Settling Tank Sludge | 2420000000 | Dry Cleaning |
| 41000264 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal: Still Waste | 2420000000 | Dry Cleaning |
| 41000265 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal: Cartridge, All Carbon | 2420000000 | Dry Cleaning |
| 41000266 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Commercial | Waste Disposal: Cartridge, Carbon Core Only | 2420000000 | Dry Cleaning |
| 41080001 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Equipment Leaks | Equipment Leaks | 2420000000 | Dry Cleaning |
| 41082001 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Wastewater, Aggregate | Process Area Drains | 2420000000 | Dry Cleaning |
| 41082002 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Wastewater, Aggregate | Process Equipment Drains | 2420000000 | Dry Cleaning |
| 41082599 | Chemical Evaporation | Dry Cleaning | Petroleum Solvent - Wastewater, Points of Generation | Other Not Classified | 2420000000 | Dry Cleaning |
| 49000101 | Chemical Evaporation | Organic Solvent Evaporation | Solvent Extraction Process | Petroleum Naphtha (Stoddard) | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000102 | Chemical Evaporation | Organic Solvent Evaporation | Solvent Extraction Process | Methyl Ethyl Ketone | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000103 | Chemical Evaporation | Organic Solvent Evaporation | Solvent Extraction Process | Methyl Isobutyl Ketone | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000104 | Chemical Evaporation | Organic Solvent Evaporation | Solvent Extraction Process | Furfural | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000105 | Chemical Evaporation | Organic Solvent Evaporation | Solvent Extraction Process | Trichloroethylene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000199 | Chemical Evaporation | Organic Solvent Evaporation | Solvent Extraction Process | Other Not Classified | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000202 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Condenser Vent | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000203 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Incinerator Stack | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000206 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Fugitive Leaks | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000207 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Distillation Vent | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000208 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Decanting | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000209 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Salting | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000299 | Chemical Evaporation | Organic Solvent Evaporation | Waste Solvent Recovery Operations | Other Not Classified | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000301 | Chemical Evaporation | Organic Solvent Evaporation | Rail Car Cleaning | Ethylene Glycol | 2415000000 | Degreasing |
| 49000302 | Chemical Evaporation | Organic Solvent Evaporation | Rail Car Cleaning | Chlorobenzene | 2415000000 | Degreasing |
| 49000303 | Chemical Evaporation | Organic Solvent Evaporation | Rail Car Cleaning | o-Dichlorobenzene | 2415000000 | Degreasing |
| 49000304 | Chemical Evaporation | Organic Solvent Evaporation | Rail Car Cleaning | Creosote | 2415000000 | Degreasing |
| 49000399 | Chemical Evaporation | Organic Solvent Evaporation | Rail Car Cleaning | Other Not Classified | 2415000000 | Degreasing |
| 49000401 | Chemical Evaporation | Organic Solvent Evaporation | Tank Truck Cleaning | Acetone | 2415000000 | Degreasing |
| 49000402 | Chemical Evaporation | Organic Solvent Evaporation | Tank Truck Cleaning | Perchloroethylene | 2415000000 | Degreasing |
| 49000403 | Chemical Evaporation | Organic Solvent Evaporation | Tank Truck Cleaning | Methyl Methacrylate | 2415000000 | Degreasing |
| 49000404 | Chemical Evaporation | Organic Solvent Evaporation | Tank Truck Cleaning | Phenol | 2415000000 | Degreasing |
| 49000405 | Chemical Evaporation | Organic Solvent Evaporation | Tank Truck Cleaning | Propylene Glycol | 2415000000 | Degreasing |
| 49000499 | Chemical Evaporation | Organic Solvent Evaporation | Tank Truck Cleaning | Other Not Classified | 2415000000 | Degreasing |
| 49000501 | Chemical Evaporation | Organic Solvent Evaporation | Air Stripping Tower | Trichloroethylene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000502 | Chemical Evaporation | Organic Solvent Evaporation | Air Stripping Tower | Perchloroethylene | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000503 | Chemical Evaporation | Organic Solvent Evaporation | Air Stripping Tower | 1,1,1-Trichloroethane | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000504 | Chemical Evaporation | Organic Solvent Evaporation | Air Stripping Tower | Chloroform | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000599 | Chemical Evaporation | Organic Solvent Evaporation | Air Stripping Tower | Solvent | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49000601 | Chemical Evaporation | Organic Solvent Evaporation | Freon Recovery/Recycling Operations | CFC-12 Recovery - Auto Air Conditioning | 2401090000 | Surface Coating: Misc. Manufacturing |
| 49099998 | Chemical Evaporation | Organic Solvent Evaporation | Miscellaneous Volatile Organic Compound Evaporation | Miscellaneous | 2401090000 | Surface Coating: Misc. Manufacturing |

U.S. Census Bureau. 2017 Total Population, American Community Survey, https://data.census.gov/cedsci/table?hidePreview=false&tid=ACSDT1Y2017.B01003&vintage=2018

Federal Highway Administration. Highway Statistics 2017. https://www.fhwa.dot.gov/policyinformation/statistics/2017/

U.S. Census Bureau. 2016 County Business Patterns, https://www.census.gov/data/datasets/2016/econ/cbp/2016-cbp.html

The Freedonia Group. 2016. Industry Study #3429, Solvents.

U.S. Census Bureau. 2010. MA325F: Paints and Allied Products. https://www.census.gov/data/tables/time-series/econ/cir/ma325f.html

U.S. Census Bureau. 2016. Annual Survey of Manufactures. https://www.census.gov/programs-surveys/asm/data/tables.html

U.S. Bureau of Labor Statistics. CPI Inflation Calculator. https://www.bls.gov/data/inflation_calculator.htm

U.S. Environmental Protection Agency. 1996. Air Emissions Inventory Improvement Program (EIIP), Volume III: Chapter 5 Consumer and Commercial Solvent Use. https://www.epa.gov/sites/production/files/2015-08/documents/iii05.pdf

Ozone Transport Commission. 2016. Technical Support Document for the 2011 Ozone Transport Commission/Mid-Atlantic Northeastern Visibility Union Modeling Platform. Appendix A. https://otcair.org/document.asp?Fview=Report

U.S. Census Bureau, Decennial Censuses, 2010 Census: Summary File 1, http://www2.census.gov/census_2010/04-Summary_File_1/.

At the time of the publication of this methodology, 2016 was the most recent data year published.

There is one point source category for Adhesives and Sealants (40200710) that maps to the nonpoint Adhesives and Sealants category (2460600000). The Solvents methodology assumes that emissions from the nonpoint Adhesives and Sealants category are controlled in some states, as discussed in section F. However these controls are specific to consumer solvents, rather than the types of solvents likely used by point sources. Therefore, EPA still recommends subtracting uncontrolled point source emissions for this source category.

There is one point source category for Adhesives and Sealants (40200710) that maps to the nonpoint Adhesives and Sealants category (2460600000). The Solvents methodology assumes that emissions from the nonpoint Adhesives and Sealants category are controlled in some states, as discussed in section F. However these controls are specific to consumer solvents, rather than the types of solvents likely used by point sources. Therefore, EPA still recommends subtracting uncontrolled point source emissions for this source category.