1.	GASOLINE DISTRIBUTION – PORTABLE FUEL CONTAINERS

## a.	Source Category Description

Portable fuel containers (PFCs, or gas cans) are consumer products used to refuel a wide variety of gasoline-powered equipment.

The general approach to calculating emissions for this SCC is to take the inventories already prepared by the EPA for 2002 and 2010 and apply a linear fit between to get 2008 emissions.

For this source category, the following SCCs were assigned:

| SCC | Descriptor 1 | Descriptor 3 | Descriptor 6 | Descriptor 8 |
| --- | --- | --- | --- | --- |
| 2501011011 | Storage and Transport | Petroleum and Petroleum Product Storage | Residential Portable Gas Cans | Permeation |
| 2501011012 | Storage and Transport | Petroleum and Petroleum Product Storage | Residential Portable Gas Cans | Evaporation (includes Diurnal losses) |
| 2501011013 | Storage and Transport | Petroleum and Petroleum Product Storage | Residential Portable Gas Cans | Spillage During Transport |
| 2501011014 | Storage and Transport | Petroleum and Petroleum Product Storage | Residential Portable Gas Cans | Refilling at the Pump - Vapor Displacement |
| 2501011015 | Storage and Transport | Petroleum and Petroleum Product Storage | Residential Portable Gas Cans | Refilling at the Pump - Spillage |
| 2501012011 | Storage and Transport | Petroleum and Petroleum Product Storage | Commercial Portable Gas Cans | Permeation |
| 2501012012 | Storage and Transport | Petroleum and Petroleum Product Storage | Commercial Portable Gas Cans | Evaporation (includes Diurnal losses) |
| 2501012013 | Storage and Transport | Petroleum and Petroleum Product Storage | Commercial Portable Gas Cans | Spillage During Transport |
| 2501012014 | Storage and Transport | Petroleum and Petroleum Product Storage | Commercial Portable Gas Cans | Refilling at the Pump - Vapor Displacement |
| 2501012015 | Storage and Transport | Petroleum and Petroleum Product Storage | Commercial Portable Gas Cans | Refilling at the Pump - Spillage |

## b.	Activity Data

Activity data for the 2002 and 2010 inventories were developed using EPA’s Nonroad Model which uses a variety of variables like equipment size, equipment population, equipment age, RVP, and air temperature to estimate activity.

## c.	Control Factors

There are at least 15 states which have enacted or are considering rules for PFCs. Due to the complexities in calculating emissions, these rules have already been accounted for in the emissions, but will not be listed in the control table.

## d.	Emission Factors

Section 2.2 of the EPA PFC report details emission factors used for each of the activities associated with PFCs.

## e.	Sample Calculations

Emissions are calculated in the formula

EmisCounty, Pollutant = m x yeardiff + Emis2002

where:

m = (Emis2010 – Emis2002) / (2010 – 2002)

Example:

Using SCC 2501011011 in Allegheny County PA as an example:

Emis2010 = 60.7 tons VOC

Emis2002 = 181.7 tons VOC

m = (60.7 – 181.7) / (2010 – 2002) = -15.125

EmisAlegheny, VOC = -15.125 x (2008-2002) + 181.7

= 90.95 tons VOC

## f.	References

U.S. Environmental Protection Agency. Estimating Emissions Associated with Portable Fuel Containers (PFCs). Ann Arbor Michigan. February 2007