COMMERCIAL COOKING

## a.	Source Category Description

Commercial cooking emissions are for five source categories based on equipment type.  Emissions estimates are for all types of meat cooked in a particular piece of equipment. Deep fat frying of french fries was also included.

For this source category, the following SCCs were assigned:

| Source Classification Code | SCC Level One | SCC Level Two | SCC Level Three | SCC Level Four |
| --- | --- | --- | --- | --- |
| 2302002100 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking - Charbroiling | Conveyorized Charbroiling |
| 2302002200 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking - Charbroiling | Under-fired Charbroiling |
| 2302003000 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking - Frying | Deep Fat Frying |
| 2302003100 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking - Frying | Flat Griddle Frying |
| 2302003200 | Industrial Processes | Food and Kindred Products: SIC 20 | Commercial Cooking - Frying | Clamshell Griddle Frying |

## b.	Activity Data

Activity data was collected from the US Census Bureau’s county level population estimates for July 1, 2008.1

## c.	Control Factors

No controls were assumed for this category.

## d.	Emission Factors

Emission factors for PM10-PRI, PM10-FIL, PM25-FIL, and HAPs were developed and reviewed by an ERTAC advisory panel composed of state and EPA personnel (Contact: Roy Huntley, huntley.roy@epa.gov).  They were created by taking 2002 emissions in the NEI and dividing by the 2002 population to develop per capita emission factors.  The PM-CON emission factors were derived by subtracting PM10-FIL from PM10-PRI. The PM25-PRI emission factors were derived by adding PM25-FIL and PM-CON. These emission factors are listed in Table 1.

## e.	Sample Calculations

Emissions are calculated for each county using emission factors and activity as:

Ex,p = Ax × EFx,p

where:

Ex,p = annual emissions for category x and pollutant p;

Ax = population data associated with category x;

EFx,p = emission factor for category x and pollutant p.

Example:

Using conveyorized charbroiling in Allegheny County, PA as an example:

According to the US Census Bureau, population on July 1, 2008 is 1,215,103

The emission factor for VOC is 0.01205 lb/person

EVOC= 1,215,103 people ×  0.01205 lb VOC/ person

= 14,650 lb VOC or 7.3 ton VOC

## f.	References

1.	DOC, 2008:  U.S. Department of Commerce, Bureau of the Census, 2008 Population Estimates Program GCT-T1: Population Estimates, Washington, DC.

Table 1: Commercial Cooking Emission Factors Developed by ERTAC

| SCC | SCC description | Pollutant Code | Factor Numeric Value | Factor Unit Numerator | Factor Unit Denominator |
| --- | --- | --- | --- | --- | --- |
| 2302002100 | Conveyorized Charbroiling | 100027 | 1.326E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 100027 | 6.385E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 100414 | 7.668E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 100414 | 5.312E-04 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 100425 | 3.730E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 100425 | 2.747E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 106445 | 7.003E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 106445 | 4.214E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 107062 | 2.682E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 107062 | 2.183E-04 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 108883 | 3.597E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 108883 | 2.496E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 108952 | 4.421E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 108952 | 3.063E-04 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 120127 | 5.943E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 120127 | 1.914E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 120127 | 8.487E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 123386 | 1.466E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 123386 | 1.113E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 129000 | 8.852E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 129000 | 3.587E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 129000 | 3.139E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 130498292 | 2.709E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 130498292 | 7.911E-04 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 130498292 | 2.589E-04 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 1330207 | 7.799E-07 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 1330207 | 1.560E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 191242 | 1.309E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 191242 | 2.542E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 193395 | 1.278E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 193395 | 1.723E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 206440 | 6.484E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 206440 | 2.638E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 206440 | 2.371E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 208968 | 2.476E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 208968 | 6.372E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 208968 | 4.886E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 50000 | 7.796E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 50000 | 5.876E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 50328 | 1.450E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 50328 | 2.332E-06 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 50328 | 1.154E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 56553 | 1.772E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 56553 | 5.432E-06 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 56553 | 2.918E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 71432 | 1.006E-03 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 71432 | 7.351E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 75070 | 5.562E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 75070 | 4.242E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 83329 | 1.613E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 83329 | 2.776E-06 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 83329 | 1.357E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 84742 | 4.140E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 84742 | 2.413E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 85018 | 2.771E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 85018 | 8.647E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 85018 | 6.021E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 86737 | 6.408E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 86737 | 2.003E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 86737 | 6.484E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 91203 | 1.135E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 91203 | 2.649E-04 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 91203 | 1.308E-04 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 92524 | 1.271E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 92524 | 2.613E-05 | LB | EACH |
| 2302003100 | Flat Griddle Frying | 92524 | 2.788E-06 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 95476 | 6.167E-05 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 95476 | 4.387E-04 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 95487 | 3.482E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 95487 | 2.111E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | 98862 | 4.991E-06 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | 98862 | 3.250E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | CO | 4.245E-02 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | CO | 1.350E-01 | LB | EACH |
| 2302003000 | Deep Fat Flying | CO | 0.000E+00 | LB | EACH |
| 2302003100 | Flat Griddle Frying | CO | 1.269E-02 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | CO | 0.000E+00 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | NOX | 0.000E+00 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | PM10-FIL | 1.648E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | PM10-FIL | 1.048E-03 | LB | EACH |
| 2302003100 | Flat Griddle Frying | PM10-FIL | 2.727E-04 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | PM10-FIL | 1.981E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | PM10-PRI | 4.980E-02 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | PM10-PRI | 3.528E-01 | LB | EACH |
| 2302003000 | Deep Fat Flying | PM10-PRI | 0.000E+00 | LB | EACH |
| 2302003100 | Flat Griddle Frying | PM10-PRI | 1.031E-01 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | PM10-PRI | 6.994E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | PM25-FIL | 1.597E-04 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | PM25-FIL | 1.013E-03 | LB | EACH |
| 2302003100 | Flat Griddle Frying | PM25-FIL | 2.074E-04 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | PM25-FIL | 1.685E-05 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | PM25-PRI | 4.979E-02 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | PM25-PRI | 3.527E-01 | LB | EACH |
| 2302003000 | Deep Fat Flying | PM25-PRI | 0.000E+00 | LB | EACH |
| 2302003100 | Flat Griddle Frying | PM25-PRI | 1.030E-01 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | PM25-PRI | 6.991E-03 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | PM-CON | 4.963E-02 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | PM-CON | 3.517E-01 | LB | EACH |
| 2302003000 | Deep Fat Flying | PM-CON | 0.000E+00 | LB | EACH |
| 2302003100 | Flat Griddle Frying | PM-CON | 1.028E-01 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | PM-CON | 6.974E-03 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | SO2 | 0.000E+00 | LB | EACH |
| 2302002100 | Conveyorized Charbroiling | VOC | 1.206E-02 | LB | EACH |
| 2302002200 | Under-fired Charbroiling | VOC | 4.148E-02 | LB | EACH |
| 2302003000 | Deep Fat Flying | VOC | 1.261E-02 | LB | EACH |
| 2302003100 | Flat Griddle Frying | VOC | 5.943E-03 | LB | EACH |
| 2302003200 | Clamshell Griddle Frying | VOC | 2.316E-04 | LB | EACH |