AVIATION GASOLINE DISTRIBUTION – STAGE I

a. Source Category Description

Aviation gasoline (also called “AvGas”) is the only aviation fuel that contains tetraethyl lead (TEL) as a knock-out component for small reciprocating, piston-engine crafts in civil aviation.1  Commercial and military aviation rarely use this fuel.  AvGas is shipped to airports and is filled into bulk terminals, and then into tanker trucks.  These processes fall under the definition of stage I, displacement vapors during the transfer of gasoline from tank trucks to storage tanks, and vice versa.  These processes are subject to EPA’s maximum available control technology (MACT) standards for gasoline distribution.2

For this source category, the following SCC was assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2501080050 | Storage and Transport | Petroleum and Petroleum Product Storage | Airports : Aviation Gasoline | Stage 1: Total |

b. Activity Data

The amount of AvGas consumed was obtained from the Petroleum Supply Annual for designated Petroleum Administration Districts, or PADs.3  A nationwide total of 5,603,000 barrels of AvGas were consumed in 2008 (Table 1).3  This information was used to calculate national-level emissions estimates for one criteria pollutant and ten hazardous air pollutants (HAPs).  Assumptions for bulk plant processes are summarized in Table 2.

c. Emission Factors

Emission factors were provided by ESD and EIG publications (Tables 3 and 4).1,4,5,6

d. Emissions

In general, national-level emissions were calculated by multiplying AvGas consumption by the appropriate emission factors and then summing emissions. The national-level emission estimates were first allocated based on consumption reported for each PAD, and then allocated to the counties within the PADs based on 2008 Landing-Take Off (LTO) data for general aviation flights.7  General aviation flights were used in this allocation because they are the primary consumers of AvGas.

There are five PADs across the United States3

PAD 1 comprises seventeen states plus the District of Columbia along the Atlantic Coast;

PAD 2 comprises fifteen states in the Midwest;

PAD 3 comprises six states in South Central U.S.;

PAD 4 comprises five states in the Rocky Mountains; and

PAD 5 comprises seven states along the West Coast.

Table 1 - Summary of AvGas Consumed and LTOs by PAD in 2008

| PAD | AvGas Consumed (barrels) | LTOs |
| --- | --- | --- |
| 1 | 1,039,000 | 17,588,837 |
| 2 | 1,652,000 | 16,520,073 |
| 3 | 2,021,000 | 9,883,668 |
| 4 | 158,000 | 3,311,438 |
| 5 | 733,000 | 12,641,441 |
|  | 5,603,000 | 59,945,457 |

e. Sample Calculations

National-Level Calculations

Amount of AvGas consumed in 2008 (barrels) = 5,603,000

Conversion: 	1 barrel = 42 gallons

1 gallon = 3.78 liters

1 kg = 2.205 lb

1 kg = 1,000,000 mg

1 ton = 2000 lb

Step 1 - Convert AvGas consumption into gallons using conversion factors.

Amount of AvGas consumed in 2008 (gallons) = 5,603,000 barrels * 42 gallons/barrel

Amount of AvGas consumed in 2008 (gallons) = 235,326,000

Step 2 - Use the gallons of AvGas consumed and apply the non-fugitive VOC emission factors in Table 3 to calculate non-fugitive VOC estimates.

Unloading/Tank Filling:  tank fill VOC emissions = 0.009021383 LB/GAL * 235,326,000 GAL / 2,000 LB/TON

Unloading/Tank Filling:  tank fill VOC emissions = 1,061.48 tpy

Unloading/Tank Filling:  Storage tank VOC emissions = 0.003605215 LB/GAL * 235,326,000 GAL / 2,000 LB/TON

Unloading/Tank Filling:  Storage tank VOC emissions = 424.20 tpy

Tank Truck Filling - Composite VOC Emissions = 0.010306575 LB/GAL * 235,326,000 GAL * / 2,000 LB/TON

Tank Truck Filling - Composite VOC Emissions = 1,212.70 tpy

Storage Tank - Breathing losses VOC Emissions = 0.001694117 LB/GAL * 235,326,000 GAL * / 2,000 LB/TON

Storage Tank - Breathing losses VOC Emissions = 199.33 tpy

Total non-fugitive VOC emissions = 1,061.48 tpy + 424.20 tpy + 1,212.70 tpy + 199.33 tpy  =  2,897.72 tpy

Step 3 - Use the assumptions in Table 2 and the fugitive VOC emission factors in Table 3 to generate fugitive VOC emissions.

AvGas - Fugitive from valves VOC Emissions = (# Bulk Plant Equivalents)*(#valves/plant)*EF*days

AvGas - Fugitive from valves VOC Emissions = (2442 plants) * (50 valves/plant) * (0.573201882 LB/valve/day)* 300 days / 2,000 LB/TON

AvGas - Fugitive from valves VOC Emissions = 10,498.19 tpy

AvGas - Fugitive from pumps VOC Emissions =  (# Bulk Plant Equivalents)*(#pumps/plant)* (#seals/pump) * EF * days

AvGas - Fugitive from pumps VOC Emissions = (2442 plants) * (2 pumps/plant) * (4 seals/pump) * (5.952481079 LB/seal/day)* 300 days / 2,000 LB/TON

AvGas - Fugitive from pumps VOC Emissions = 17,443.15 tpy

Total fugitive VOC emissions = 10,498.19 tpy + 17,443.15 tpy

Total fugitive VOC emissions = 27,941.34 tpy

Step 4 - Sum the fugitive and non-fugitive VOC emissions together for total VOC emissions.

Total VOC emissions = 2,897.72 tpy + 27,941.34 tpy = 30,839.06 tpy

Step 5 - Apply the speciation emission factors in Table 4 for tetraethyl lead, 2,2,4-trimethylpentane, benzene, cumene, ethylbenzene, hexane, naphthalene, toluene, and xylene to calculate HAP emissions.

Tetraethyl Lead emissions = 30,839.06 tpy VOC * 9.78 E-6 = 0.30 tpy

2,2,4-Trimethylpentane emissions = 30,839.06 tpy VOC * 0.008 = 246.71 tpy

Benzene emissions = 30,839.06 tpy VOC * 0.009 = 277.55 tpy

Cumene emissions = 30,839.06 tpy VOC * 0.0001 = 3.08 tpy

Ethylbenzene emissions = 30,839.06 tpy VOC * 0.0010 = 30.84 tpy

Hexane emissions = 30,839.06 tpy VOC * 0.0160 = 493.43 tpy

Naphthalene emissions = 30,839.06 tpy VOC * 0.0005 = 15.42 tpy

Toluene emissions = 30,839.06 tpy VOC * 0.0130 = 400.91 tpy

Xylene emissions = 30,839.06 tpy VOC * 0.005 = 154.20 tpy

Step 6 - Use the ethylene dichloride emission factor in Table 4 to calculate ethylene dichloride emissions.

Ethylene dichloride emissions = 235,326,000 GAL * 2.167E-6 LB/GAL * TON/2000 LB = 0.25 tpy

Table 2 - Assumptions Used For Bulk Terminals Using AvGas

| Parameter | Data | Reference |
| --- | --- | --- |
| Number of Bulk Plant Equivalents (U.S.) | 2,442 plants | 1 |
| Number of valves per bulk plant | 50 valves/plant |  |
| Number of pumps per bulk plant | 2 pumps/plant |  |
| Number of seals per bulk plant | 4 seals/pump |  |
| Number of days per year used | 300 days |  |

Table 3 - VOC Emission Factors

| Pollutant | Emission Source | Emission Factor | Emission Factor Units | Factor Reference |
| --- | --- | --- | --- | --- |
| VOC | Aviation Gas Unloading/ Tank Filling - tank fill | 0.009021383 | LB/GAL AvGas | 1 |
|  | Aviation Gas Unloading/ Tank Filling - Storage tank working | 0.003605215 |  |  |
|  | Aviation Gas Tank Truck Filling - Composite | 0.010306575 |  |  |
|  | Aviation Gas Storage Tank - Breathing losses | 0.001694117 |  |  |
|  | Aviation Gas - Fugitive from valves | 0.573201882 | LB/valve/day |  |
|  | Aviation Gas - Fugitive from pumps | 5.952481079 | LB/seal/day |  |

Table 4 - HAP Emission Factors

| Pollutant | Emission Source | Emission Factor | Emission Factor Units | Factor Reference |
| --- | --- | --- | --- | --- |
| Ethylene Dichloride | All processes | 2.167 E-6 | LB/GAL AvGas | 4 |
| Tetraethyl Lead (TEL) | All processes | 9.78 E-6 | LB/LB VOC | 1 |
| 2,2,4-Trimethylpentane | All processes | 0.0080 | LB/ LB VOC | 5 |
| Benzene | All processes | 0.0090 |  |  |
| Cumene | All processes | 0.0001 |  | 6 |
| Ethylbenzene | All processes | 0.0010 |  | 5 |
| Hexane | All processes | 0.0160 |  |  |
| Naphthalene | All processes | 0.0005 |  |  |
| Toluene | All processes | 0.0130 |  |  |
| Xylene | All processes | 0.0050 |  |  |

Example Calculations for Wake County, NC

Wake County VOC emissions = (National VOC emissions) * (PAD 1 consumption/Total consumption) * (Wake County LTOs/PAD 1 LTOs)

Wake County VOC emissions = (30,839.06 tpy) * (1,039,000 bbl/5,603,000 bbl) * (95,234 LTOs/17,588,837 LTOs)

Wake County VOC emissions = 30.96 tpy

Wake County Benzene Emissions = (Wake County VOC emissions)*(Benzene Emission Factor)

Wake County Benzene Emissions = (30.96 tpy VOC) * (0.0090 LB benzene/ LB VOC)* (2000 LB VOC/2000 LB benzene)

Wake County Benzene Emissions = 0.28 tpy

f. References

1.	TRC Environmental Corporation.  Estimation of Alkylated Lead Emissions, Final Report.  Prepared for U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  RTP, NC 1993.

2.	U.S. Environmental Protection Agency.  National Emission Standards for Source Categories:  Gasoline Distribution (Stage I).  40 CFR Part 63.  Office of Air Quality Planning and Standards.  RTP, NC.  February 28, 1997.  Pages 9087-9093.

3.	Energy Information Administration. Petroleum Annual Supply, 2008.  Tables 3, 5, 7, 9, and 11.  U.S. Department of Energy.  Washington, D.C.  June 2009.  (Internet address: http://www.eia.doe.gov/oil_gas/petroleum/data_publications/petroleum_supply_annual/psa_volume1/psa_volume1.html)

4.	U.S. Environmental Protection Agency.  Locating and Estimating Air Emissions from Sources of Ethylene Dichloride.  EPA-450/4-84-007d.  RTP, NC.  March 1984.

5.	Memorandum from Greg LaFlam and Tracy Johnson (PES) to Stephen Shedd (EPA/OAQPS).  Speciated Hazardous Air Pollutants - Baseline Emissions and Emissions Reductions Under the Gasoline Distribution NESHAP.  August 9, 1996.

6.	Personal Communication via e-mail from Stephen Shedd (EPA/OAQPS) to Laurel Driver (EPA/OAQPS).  E-mail dated May 29, 2002.

7.	[LTObyCtyandSCC.mdb], electronic file from Laurel Driver, U.S. Environmental Protection Agency, OAQPS, to U.S. Environmental Protection Agency, OAQPS, November 12, 2009. Aircraft operations data compiled from FAA’s Terminal Area Forecasts (TAF) and 5010 Forms.