# Residential Construction

a. Source Category Description

Emissions from residential construction activity are a function of the acreage disturbed and volume of soil excavated for residential construction.  Residential construction activity is developed from data obtained from the U.S. Department of Commerce (DOC)’s Bureau of the Census.

For this source category, the following SCC was assigned:

| Source Classification Code | SCC Level One | SCC Level Two | SCC Level Three | SCC Level Four |
| --- | --- | --- | --- | --- |
| 2311010000 | Industrial Processes | Construction: SIC 15 - 17 | General Building Construction | Total |

## b. Activity Data

There are two activity calculations performed for this SCC, acres of surface soil disturbed and volume of soil removed for basements.

### b.1. Surface soil disturbed

The US Census Bureau has 2008 data for Housing Starts - New Privately Owned Housing Units Started1 which provides regional level housing starts based on the groupings of 1 unit, 2-4 units, 5 or more units. A consultation with the Census Bureau in 2002 gave a breakdown of approximately 1/3 of the housing starts being for 2 unit structures, and 2/3 being for 3 and 4 unit structures. The 2-4 unit category was then divided into 2-units, and 3-4 units based on this ratio. To get the number of structures for each grouping, the 1unit category was divided by 1, 2 unit category was divided by 2, and the 3-4 unit category was divided by 3.5. The 5 or more unit category listed may be made up of more than one structure. New Privately Owned Housing Units Authorized Unadjusted Units2 gives a conversion factor to determine the ratio of structures to units in the 5 or more unit category. For example if a county has one 40unit apartment building, the ratio would be 40/1. If there are 5 different 8 unit buildings in the same project, the ratio would be 40/5. Structures started by category are then calculated at a regional level. The table Annual Housing Units Authorized by Building Permit3 has 2007 data at the county level to allocate regional housing starts to the county level. This results in county level housing starts by number of units.  The following surface areas were assumed disturbed for each unit type:

### Table 1: Surface Soil removed per unit type

| 1-Unit | 1/4 acre/structure |
| --- | --- |
| 2-Unit | 1/3 acre/structure |
| Apartment | 1/2 acre/structure |

The 3-4 unit category was considered to be an apartment. Multiplication of housing starts to soil removed results in number of acres disturbed for each unit category.

### b.2. Basement soil removal

To calculate basement soil removal, 2007 Characteristics of New Houses4 is used to estimate the percentage of 1 unit structures that have a basement (on the regional level). The county level estimate of number of 1 unit starts is multiplied by the percent of 1 unit houses in the region that have a basement to get the number of basements in a county.  Basement volume is calculated by assuming a 2000 square foot house has a basement dug to a depth of 8 feet (making 16,000 ft3 per basement). An additional 10% is added for peripheral dirt bringing the total to 17,600 ft3 per basement.

## c. Emission Factors

Initial PM10 emissions from construction of single family, two family, and apartments structures are calculated using the emission factors given in Table 2.  The duration of construction activity for houses is assumed to be 6 months and the duration of construction for apartments is assumed to be 12 months.

Table 2.  Emission Factors for Residential Construction

| Type of Structure | Emission Factor | Duration of Construction |
| --- | --- | --- |
| Apartments | 0.11 tons PM10/acre-month | 12 months |
| 2-Unit Structures | 0.032 tons PM10/acre-month | 6 months |
| 1-Unit Structures w/o Basements | 0.032 tons PM10/acre-month | 6 months |
| 1-unit Structures with Basements | 0.011 tons PM10/acre-month | 6 months |
|  | 0.059 tons PM10/1000 cubic yards |  |

Regional variances in construction emissions are corrected using soil moisture level and silt content.  These correction parameters are applied to initial PM10 emissions from residential construction to develop the final emissions inventory.

To account for the soil moisture level, the PM10 emissions are weighted using the 30-year average precipitation-evaporation (PE) values from Thornthwaite’s PE Index.  Average precipitation evaporation values for each State were estimated based on PE values for specific climatic divisions within a State.  These values range from 7 to 41.

To account for the silt content, thePM10 emissions are weighted using average silt content for each county.  A data base containing county-level dry silt values was complied.  These values were derived by applying a correction factor developed by the California Air Resources Board to convert wet silt values to dry silt values.5

The equation for PM10 emissions corrected for soil moisture and silt content is:

where:	Corrected EPM10	=  PM10 emissions corrected for soil moisture and silt content,

PE  	=  precipitation-evaporation value for each State,

S  	=  % dry silt content in soil for area being inventoried.

Once PM10 adjustments have been made, PM2.5 emissions are set to 10% of PM10. Primary PM emissions are equal to filterable emissions since there are no condensible emissions from residential construction.

## d. Example Calculation

PM10 Emissions = ∑( Aunit x Tconstruction x EFunit ) x AdjPM

Where Aunit = HSUnit x SMUnit

HSUnit = Regional Housing Starts x (county building permits/Regional building permits)

SMUnit = Area or volume of soil moved for the given unit type

TConstruction = Construction time (in months) for given unit type

EFUnit = Unadjusted emission factor for PM10 for the given unit type

AdjPM = PM Adjustment factor

## e. References

- New Privately Owned Housing Units Started for 2008  ( Not seasonally adjusted) http://www.census.gov/const/startsua.pdf

- Table 2au. New Privately Owned Housing Units Authorized Unadjusted Units for Regions, Divisions, and States http://www.census.gov/const/C40/Table2/tb2u2007.txt

- Annual Housing Units Authorized by Building Permits CO2007A, purchased from US Department of Census

- Type of Foundation in New One-Family Houses Completed, http://www.census.gov/const/C25Ann/sftotalfoundation.pdf