1.	FOSSIL FUEL COMBUSTION – RESIDENTIAL – COAL

a.	Source Category Description

Residential Coal Combustion is coal that is burned to heat residential housing.

The general approach to calculating emissions for these two SCCs is to take State Coal Consumption from the EIA and allocate it to the county level using methods described below.  County-level coal consumption is multiplied by the emission factors to calculate emissions.

For this source category, the following SCCs were assigned:

| SCC | Descriptor 1 | Descriptor 3 | Descriptor 6 | Descriptor 8 |
| --- | --- | --- | --- | --- |
| 2104001000 | Stationary Source Fuel Combustion | Residential | Anthracite Coal | All Boiler Types |
| 2104002000 | Stationary Source Fuel Combustion | Residential | Bituminous/ Subbituminous Coal | All Boiler Types |

b.	Activity Data

The mass of coal consumed by residential combustion in the U.S. was used to estimate emissions.  Coal consumption by energy use sector is presented in State Energy Data 2006 Consumption tables published by the Energy Information Administration (EIA).1 Year 2006 consumption data were used as a surrogate for 2008 emissions because year 2006 data were the latest data available when this inventory was prepared.

EIA data do not distinguish between anthracite and bituminous coal consumption estimates.  The EIA table “Domestic Distribution of U.S. Coal by Destination State, Consumer, Origin and Method of Transportation,” provides state-level residential coal distribution data for 2006 that was used to estimate anthracite and bituminous coal consumption.  The amount of anthracite distributed to each state and the total coal delivered to each state were used to estimate the proportion of anthracite and bituminous coal consumption.2  The 2006 ratio of anthracite (and bituminous) coal consumption to total coal consumption was used to distribute the EIA’s total residential sector coal consumption data by coal type.  Table 1 presents the 2006 anthracite and bituminous coal ratios for each state.

State-level coal consumption was allocated to each county using the US Census Bureau’s 2000 Census Detailed Housing Information.3 These data include the number of housing units using a specific type of fuel for residential heating. State coal consumption was allocated to each county using the ratio of the number of houses burning coal in each county to the total number of houses burning coal in the State.

c.	Control Factors

No controls were assumed for this category

d.	Emission Factors

In some cases (see Table 2), SO2 and PM emission factors required information on the sulfur and/or ash content of the coal burned.  State-specific sulfur and ash contents of anthracite and bituminous coal were obtained from data compiled in preparing the 1999 residential coal combustion emissions estimates.4 This study mostly relied on data obtained from USGS COALQUAL database.  States not included in the database but that reported coal usage were assigned values based on their proximity to coal seams or using an average value for Pennsylvania (see report for details of the analysis).  Table 3 presents the bituminous coal sulfur content values used for each state.  For anthracite coal, an ash content value of 13.38% and a sulfur content of 0.89% were applied to all states except New Mexico (ash content 16.61%, sulfur content 0.77%), Washington (ash content 12%, sulfur content 0.9%), and Virginia (ash content 13.38%, sulfur content 0.43%).

The remaining criteria pollutant and HAP emissions were calculated by multiplying the total coal consumed in each county per year by an emission factor.  All emission factors except ammonia are from AP-42.5 The ammonia emission factor is from EPA’s Estimating Ammonia Emissions from Anthropogenic Sources, Draft Final Report.6 Table 4 presents a summary of the emission factors for residential anthracite coal combustion (SCC 2104001000).  Table 5 presents a summary of the emission factors for residential bituminous coal combustion (SCC 2104002000).

e.	Sample Calculations

Annual emissions are calculated for each county using emission factors and activity as:

Ex,p = FCx × (1 - CEx,p) × EFx,p

where:

Ex,p = annual emissions for fuel type x and pollutant p (ton/year),

FCx = annual fuel consumption for fuel type x,

CEx,p = control efficiency for fuel type x and pollutant p, and

EFx,p = emission factor for fuel type x and pollutant p.

County level fuel consumption is calculated using:

FCx = AState x RatioAnth, Bit x RatioCounty houses

where:

AState = total tons of coal reported by the EIA,

RatioAnth, Bit = ratio reported in Table 1, and

RatioCounty houses = county allocation ratio based on number of houses burning coal.

Example:

Using Allegheny County, PA as an example:

The State of Pennsylvania had a reported use of 49,935 tons of coal in the residential sector in 2006.

Statewide Anthracite coal use is calculated using the ratio of Anthracite to Bituminous in Table 1 for PA:  80.64%. Allegheny County, PA had 183 houses out of the state total of 67,986 that use coal as the primary heating fuel. This equates to a share of 0.27% of the coal used for residential heating in the state.

The emission factor for PM2.5-PRI (See Table 4) is 0.6 × state-specific % ash content + 0.08 lbs/ton× state-specific % ash content.  The ash content is 13.38%, (See Table 2) so the emission factor is 9.09 lbs/ton.

FCAllegheny, anth = 49,935 × 0.8064 × 0.0027 = 109 tons anthracite coal

EmisAllegheny, anth, PM2.5-PRI = 109 tons Coal × 9.09 lbs PM2.5-PRI per ton coal

= 990 lbs PM2.5-PRI

f.	References

1.	U.S. Department of Energy, Energy Information Administration (EIA).  State Energy Data 2006 Consumption.  Washington, DC  2008.  Internet Address:  http://www.eia.doe.gov/emeu/states/sep_use/total/csv/use_all_phy.csv accessed November 2008.

2.	EIA, 2008.  U.S. Department of Energy, Energy Information Administration, “Domestic Distribution of U.S. Coal by Destination State, Consumer, Origin and Method of Transportation”, 2006.  Available from:  http://www.eia.doe.gov/cneaf/coal/page/coaldistrib/coal_distributions.html, accessed December 2008.

3.	U.S. Census Bureau. "Table H40. House Heating Fuel Type", Census 2000: Summary File 3. Internet address: http://factfinder.census.gov/servlet/DatasetMainPageServlet?_program=DEC&_submenuId=&_lang=en&_ts=, accessed July 2009.

4.	U.S. Environmental Protection Agency.  Emission Factor and Inventory Group.  Final Summary of the Development and Results of a Methodology for Calculating Area Source Emissions from Residential Fuel Combustion.  Prepared by Pacific Environmental Services, Inc.  Research Triangle Park, NC.  September 2002.  Internet address:  http://www.epa.gov/ttn/chief/eiip/techreport/volume03/draft1999_ residfuel_inven_apr2003.zip accessed November 2004.

5.	U.S. Environmental Protection Agency.  Compilation of Air Pollutant Emission Factors, 5th Edition, AP-42, Volume I:  Stationary Point and Area Sources.  Research Triangle Park, North Carolina.  1996.

6.	U.S. Environmental Protection Agency.  Emission Inventory Improvement Program.  Estimating Ammonia Emissions from Anthropogenic Sources, Draft Final Report.  Prepared by E.H. Pechan and Associates, Inc.  Research Triangle Park, NC.  April 2004.

Table 1.  2006 Anthracite and Bituminous Coal Distribution for the Residential and Commercial Sectors

| State | Ratio of Bituminous | Ratio of Anthracite | State | Ratio of Bituminous | Ratio of Anthracite |
| --- | --- | --- | --- | --- | --- |
| Alabama | 1.0000 | 0.0000 | Montana | 1.0000 | 0.0000 |
| Alaska | 1.0000 | 0.0000 | Nebraska | 1.0000 | 0.0000 |
| Arizona | 0.8140 | 0.1860 | Nevada | 1.0000 | 0.0000 |
| Arkansas | 0.8140 | 0.1860 | New Hampshire | 0.0000 | 1.0000 |
| California | 1.0000 | 0.0000 | New Jersey | 0.0000 | 1.0000 |
| Colorado | 0.9962 | 0.0038 | New Mexico | 1.0000 | 0.0000 |
| Connecticut | 0.0000 | 1.0000 | New York | 0.6000 | 0.4000 |
| Delaware | 0.8140 | 0.1860 | North Carolina | 1.0000 | 0.0000 |
| Dist. Columbia | 1.0000 | 0.0000 | North Dakota | 1.0000 | 0.0000 |
| Florida | 0.8140 | 0.1860 | Ohio | 0.8727 | 0.1273 |
| Georgia | 1.0000 | 0.0000 | Oklahoma | 0.9167 | 0.0833 |
| Hawaii | 1.0000 | 0.0000 | Oregon | 1.0000 | 0.0000 |
| Idaho | 0.9792 | 0.0208 | Pennsylvania | 0.1936 | 0.8064 |
| Illinois | 0.9981 | 0.0019 | Rhode Island | 0.0000 | 1.0000 |
| Indiana | 0.9474 | 0.0526 | South Carolina | 0.9972 | 0.0028 |
| Iowa | 0.9992 | 0.0008 | South Dakota | 1.0000 | 0.0000 |
| Kansas | 1.0000 | 0.0000 | Tennessee | 0.9939 | 0.0061 |
| Kentucky | 0.9981 | 0.0019 | Texas | 0.8140 | 0.1860 |
| Louisiana | 1.0000 | 0.0000 | Utah | 1.0000 | 0.0000 |
| Maine | 0.0000 | 1.0000 | Vermont | 0.0000 | 1.0000 |
| Maryland | 0.9286 | 0.0714 | Virginia | 0.9630 | 0.0370 |
| Massachusetts | 0.5000 | 0.5000 | Washington | 1.0000 | 0.0000 |
| Michigan | 0.6667 | 0.3333 | West Virginia | 0.9048 | 0.0952 |
| Minnesota | 0.9973 | 0.0027 | Wisconsin | 0.9914 | 0.0086 |
| Mississippi | 1.0000 | 0.0000 | Wyoming | 1.0000 | 0.0000 |
| Missouri | 1.0000 | 0.0000 |  |  |  |

Table 2.  SO2 and PM Emission Factors for Industrial Anthracite and Bituminous Coal Combustion

| Pollutant | Emission Factor (lb/ton) | AP-42 Table |
| --- | --- | --- |
| Anthracite Emission Factors (SCC 2104001000) |  |  |
| PM-CON | 0.08* % Ash | 1.2-3 |
| PM10-FIL | 10 | 1.2-4 |
| PM25-FIL | 0.6 * % Ash | 1.2-4 |
| PM10-PRI | 10 + 0.08* % Ash | 1.2-3 and 1.2-4 |
| PM25-PRI | 0.6 * % Ash + 0.08* % Ash | 1.2-3 and 1.2-4 |
| Sulfur Dioxide | 39 * % Sulfur | 1.2-1 |
| Bituminous Emission Factors (SCC 2104002000) |  |  |
| PM-CON | .04 | 1.1-5 |
| PM10-FIL | 6.2 | 1.1-9 |
| PM25-FIL | 3.8 | 1.1-9 |
| PM10-PRI | 6.24 | 1.1-5 and 1.1-9 |
| PM25-PRI | 3.84 | 1.1-5 and 1.1-9 |
| Sulfur Dioxide | 31 * % Sulfur | 1.1-3 |
| NOTE: PM10, PM2.5, and condensable PM emission factors for bituminous coal do not require ash content, nor does the condensable PM emission factor for anthracite coal. |  |  |

Table 3.  State-Specific Sulfur Content for Bituminous Coal (SCC 2104002000)

| State | Percent Sulfur Content | State | Percent Sulfur Content |
| --- | --- | --- | --- |
| Alabama | 2.08 | Montana | 0.60 |
| Alaska | 0.31 | Nebraska | 2.43 |
| Arizona | 0.47 | Nevada | 2.30 |
| Arkansas | 1.20 | New Hampshire | 2.42 |
| California | 0.47 | New Jersey | 2.42 |
| Colorado | 0.61 | New Mexico | 0.75 |
| Connecticut | 2.42 | New York | 2.42 |
| Delaware | 1.67 | North Carolina | 1.62 |
| District of Columbia | 1.67 | North Dakota | 0.97 |
| Florida | 1.28 | Ohio | 3.45 |
| Georgia | 1.28 | Oklahoma | 3.08 |
| Hawaii | 1.00 | Oregon | 0.50 |
| Idaho | 0.31 | Pennsylvania | 2.42 |
| Illinois | 3.48 | Rhode Island | 2.42 |
| Indiana | 2.49 | South Carolina | 1.28 |
| Iowa | 4.64 | South Dakota | 0.97 |
| Kansas | 5.83 | Tennessee | 1.62 |
| Kentucky | 1.93 | Texas | 1.14 |
| Louisiana | 0.86 | Utah | 0.80 |
| Maine | 2.42 | Vermont | 2.42 |
| Maryland | 1.67 | Virginia | 1.19 |
| Massachusetts | 2.42 | Washington | 0.50 |
| Michigan | 1.20 | West Virginia | 1.25 |
| Minnesota | 0.97 | Wisconsin | 1.00 |
| Mississippi | 1.24 | Wyoming | 0.87 |
| Missouri | 3.39 |  |  |

Table 4.  National Criteria and HAP Emission Factors for Residential Anthracite Coal Combustion (SCC 2104001000):  Not Adjusted for Point Source Fuel Consumption

| SCC | Pollutant Code | Pollutant Code Description | Factor Numeric Value | Factor Unit Numerator | Factor Unit Denominator |
| --- | --- | --- | --- | --- | --- |
| 2104001000 | 100414 | ETHYL BENZENE | 0.000094 | LB | TON |
| 2104001000 | 100425 | STYRENE | 0.000025 | LB | TON |
| 2104001000 | 107028 | ACROLEIN | 0.00029 | LB | TON |
| 2104001000 | 107062 | ETHYLENE DICHLORIDE | 0.00004 | LB | TON |
| 2104001000 | 108883 | TOLUENE | 0.00024 | LB | TON |
| 2104001000 | 108907 | CHLOROBENZENE | 0.000022 | LB | TON |
| 2104001000 | 108952 | PHENOL | 0.000016 | LB | TON |
| 2104001000 | 110543 | HEXANE | 0.000067 | LB | TON |
| 2104001000 | 117817 | BIS(2-ETHYLHEXYL)PHTHALATE | 0.000073 | LB | TON |
| 2104001000 | 120127 | ANTHRACENE | 0.00000021 | LB | TON |
| 2104001000 | 123386 | PROPIONALDEHYDE | 0.00038 | LB | TON |
| 2104001000 | 127184 | TETRACHLOROETHYLENE | 0.000043 | LB | TON |
| 2104001000 | 129000 | PYRENE | 0.00000033 | LB | TON |
| 2104001000 | 191242 | BENZO[G,H,I,]PERYLENE | 0.000000027 | LB | TON |
| 2104001000 | 193395 | INDENO[1,2,3-C,D]PYRENE | 0.000000061 | LB | TON |
| 2104001000 | 206440 | FLUORANTHENE | 0.00000071 | LB | TON |
| 2104001000 | 208968 | ACENAPHTHYLENE | 0.00000025 | LB | TON |
| 2104001000 | 218019 | CHRYSENE | 0.0000001 | LB | TON |
| 2104001000 | 50000 | FORMALDEHYDE | 0.00024 | LB | TON |
| 2104001000 | 50328 | BENZO[A]PYRENE | 0.000000038 | LB | TON |
| 2104001000 | 56553 | BENZ[A]ANTHRACENE | 0.00000008 | LB | TON |
| 2104001000 | 56832736 | Benzofluoranthenes | 0.00000011 | LB | TON |
| 2104001000 | 71432 | BENZENE | 0.0013 | LB | TON |
| 2104001000 | 7439921 | LEAD | 0.00042 | LB | TON |
| 2104001000 | 7439965 | MANGANESE | 0.00049 | LB | TON |
| 2104001000 | 7439976 | MERCURY | 0.000083 | LB | TON |
| 2104001000 | 7440020 | NICKEL | 0.00028 | LB | TON |
| 2104001000 | 7440360 | ANTIMONY | 0.000018 | LB | TON |
| 2104001000 | 7440382 | ARSENIC | 0.00041 | LB | TON |
| 2104001000 | 7440417 | BERYLLIUM | 0.000021 | LB | TON |
| 2104001000 | 7440439 | CADMIUM | 0.000051 | LB | TON |
| 2104001000 | 7440473 | CHROMIUM | 0.000264 | LB | TON |
| 2104001000 | 7440484 | COBALT | 0.0001 | LB | TON |
| 2104001000 | 74839 | METHYL BROMIDE | 0.00016 | LB | TON |
| 2104001000 | 74873 | METHYL CHLORIDE | 0.00053 | LB | TON |
| 2104001000 | 75070 | ACETALDEHYDE | 0.00057 | LB | TON |
| 2104001000 | 75092 | METHYLENE CHLORIDE | 0.00029 | LB | TON |
| 2104001000 | 75150 | CARBON DISULFIDE | 0.00013 | LB | TON |
| 2104001000 | 7647010 | HYDROCHLORIC ACID | 1.2 | LB | TON |
| 2104001000 | 7664393 | HYDROGEN FLUORIDE | 0.15 | LB | TON |
| 2104001000 | 7782492 | SELENIUM | 0.0013 | LB | TON |
| 2104001000 | 78591 | ISOPHORONE | 0.00058 | LB | TON |
| 2104001000 | 78933 | METHYL ETHYL KETONE | 0.00039 | LB | TON |
| 2104001000 | 83329 | ACENAPHTHENE | 0.00000051 | LB | TON |
| 2104001000 | 85018 | PHENANTHRENE | 0.0000027 | LB | TON |
| 2104001000 | 86737 | FLUORENE | 0.00000091 | LB | TON |
| 2104001000 | 91203 | NAPHTHALENE | 0.000013 | LB | TON |
| 2104001000 | 98862 | ACETOPHENONE | 0.000015 | LB | TON |
| 2104001000 | CO | CARBON MONOXIDE | 275 | LB | TON |
| 2104001000 | NH3 | AMMONIA | 2 | LB | TON |
| 2104001000 | NOX | NITROGEN OXIDES | 3 | LB | TON |
| 2104001000 | VOC | VOLATILE ORGANIC COMPOUNDS | 10 | LB | TON |

Table 5.  National Criteria and HAP Emission Factors for Residential Bituminous Coal Combustion (SCC 2104002000):  Not Adjusted for Point Source Fuel Consumption

| SCC | Pollutant Code | Pollutant Code Description | Factor Numeric Value | Factor Unit Numerator | Factor Unit Denominator |
| --- | --- | --- | --- | --- | --- |
| 2104002000 | 100414 | ETHYL BENZENE | 0.000094 | LB | TON |
| 2104002000 | 100425 | STYRENE | 0.000025 | LB | TON |
| 2104002000 | 107028 | ACROLEIN | 0.00029 | LB | TON |
| 2104002000 | 107062 | ETHYLENE DICHLORIDE | 0.00004 | LB | TON |
| 2104002000 | 108883 | TOLUENE | 0.00024 | LB | TON |
| 2104002000 | 108907 | CHLOROBENZENE | 0.000022 | LB | TON |
| 2104002000 | 108952 | PHENOL | 0.000016 | LB | TON |
| 2104002000 | 110543 | HEXANE | 0.000067 | LB | TON |
| 2104002000 | 117817 | BIS(2-ETHYLHEXYL)PHTHALATE | 0.000073 | LB | TON |
| 2104002000 | 123386 | PROPIONALDEHYDE | 0.00038 | LB | TON |
| 2104002000 | 127184 | TETRACHLOROETHYLENE | 0.000043 | LB | TON |
| 2104002000 | 129000 | PYRENE | 0.00000033 | LB | TON |
| 2104002000 | 191242 | BENZO[G,H,I,]PERYLENE | 0.000000027 | LB | TON |
| 2104002000 | 193395 | INDENO[1,2,3-C,D]PYRENE | 0.000000061 | LB | TON |
| 2104002000 | 205992 | BENZO[B]FLUORANTHENE | 0.00000011 | LB | TON |
| 2104002000 | 218019 | CHRYSENE | 0.0000001 | LB | TON |
| 2104002000 | 50000 | FORMALDEHYDE | 0.00024 | LB | TON |
| 2104002000 | 56553 | BENZ[A]ANTHRACENE | 0.00000008 | LB | TON |
| 2104002000 | 71432 | BENZENE | 0.0013 | LB | TON |
| 2104002000 | 7439921 | LEAD | 0.00042 | LB | TON |
| 2104002000 | 7439965 | MANGANESE | 0.00049 | LB | TON |
| 2104002000 | 7439976 | MERCURY | 0.000083 | LB | TON |
| 2104002000 | 7440020 | NICKEL | 0.00028 | LB | TON |
| 2104002000 | 7440360 | ANTIMONY | 0.000018 | LB | TON |
| 2104002000 | 7440382 | ARSENIC | 0.00041 | LB | TON |
| 2104002000 | 7440417 | BERYLLIUM | 0.000021 | LB | TON |
| 2104002000 | 7440439 | CADMIUM | 0.000051 | LB | TON |
| 2104002000 | 7440473 | CHROMIUM | 0.00026 | LB | TON |
| 2104002000 | 7440484 | COBALT | 0.0001 | LB | TON |
| 2104002000 | 74839 | METHYL BROMIDE | 0.00016 | LB | TON |
| 2104002000 | 74873 | METHYL CHLORIDE | 0.00053 | LB | TON |
| 2104002000 | 75070 | ACETALDEHYDE | 0.00057 | LB | TON |
| 2104002000 | 75092 | METHYLENE CHLORIDE | 0.00029 | LB | TON |
| 2104002000 | 75150 | CARBON DISULFIDE | 0.00013 | LB | TON |
| 2104002000 | 7782492 | SELENIUM | 0.0013 | LB | TON |
| 2104002000 | 78591 | ISOPHORONE | 0.00058 | LB | TON |
| 2104002000 | 78933 | METHYL ETHYL KETONE | 0.00039 | LB | TON |
| 2104002000 | 98862 | ACETOPHENONE | 0.000015 | LB | TON |
| 2104002000 | CO | CARBON MONOXIDE | 275 | LB | TON |
| 2104002000 | NH3 | AMMONIA | 2.0 | LB | TON |
| 2104002000 | NOX | NITROGEN OXIDES | 9.1 | LB | TON |
| 2104002000 | VOC | VOLATILE ORGANIC COMPOUNDS | 10 | LB | TON |