## 1.	SOLVENT UTILIZATION

### a.	Source Category Description

Solvent Utilization describes a wide range of activities where the primary pollutants released are volatile organic compounds (VOCs).  Some examples would be surface coating operations (where a paint or finish is applied to a manufactured product), or consumer and commercial cleaning product application.

### b.	Activity Data

The three types of activity data collected are population, road miles (used for traffic markings), and employment data.  Table 1 shows the type of activity data used for each category and provides the SCC.  Employment data is listed by the North American Industrial Classification Standard (NAICS) code(s) that were used to determine county-level employment for the category.

Population data was collected from the U.S. Census Bureau’s population estimates for July 1, 2008.

For traffic paints, the Federal Highway Administration provides county-level road miles as a part of their HPMS data.  The most recent data set available at the time of compilation was 2006.

Employment data was allocated to each county using County Business Patterns employment data for 2006 (the most recent data available at the time of compilation).  One of the limitations of County Business Patterns is the fact that data can be withheld if certain conditions are present so that confidential business data is not revealed.  Divita 2009 describes how this limitation is overcome by applying a procedure to estimate the number of employees in a withheld county. For SCCs with multiple NAICS associated with them, this allocation procedure is performed for each individual NAICS.

### c.	Control Factors

For several categories, air pollution regulations exist that regulate the solvent content of products which can be sold.  These solvent content limits have been taken into account where appropriate by modifying emission factors rather than developing control efficiency information.

### d.	Emission Factors

Emission factors were developed and reviewed by an ERTAC advisory panel composed of state and EPA personnel (see attached Excel file named "Emission Factors for Solvents August 8 2009 (version 4).xls" or contact Roy Huntley at huntley.roy@epa.gov).  Table 2 lists the SCC, pollutant, and emission factors that were developed for categories with the same factor throughout the country. As mentioned in the control factors section above, there were several categories where the emission factor was adjusted to account for rules that limit solvent content.  Table 3 lists the categories with their uncontrolled and controlled emission factors while Table 4 lists the states for which controlled emission factors were applied.

### e.	Sample Calculations

Emissions are calculated for each county using emission factors and activity as:

Ex,p	= Ax × EFx,p

where:

Ex,p	= annual emissions for category x and pollutant p

Ax	= Population or employment data associated with category x

EFx,p	= emission factor for category x and pollutant p

Example:

Using architectural coatings in Allegheny County, PA as an example:

According to the U.S. Census Bureau, population on July 1, 2008 is 1,215,103

The emission factor for VOC is 2.41 lb/person

EVOC	= 1,215,103 people ×  2.41 lb VOC/ person

= 2,928,398 lb VOC or 1,464.2 ton VOC

### f.	References

Divita, 2009:  Divita, Frank, E.H. Pechan & Associates, Inc., memorandum to Roy Huntley, U.S. Environmental Protection Agency, “County Business Patterns Allocations,” June 30, 2009.

DOC, 2008:  U.S. Department of Commerce, Bureau of the Census, 2006 County Business Patterns, Washington, DC.  Address: http://www.census.gov/epcd/cbp/download/06_data/index.html accessed November 2008.

Table 1.  Activity Data Sources Used for Each SCC

| Category Name | SCC | NAICS | Comments |
| --- | --- | --- | --- |
| Architectural Coatings | 2401001000 | N/A | Population based Category |
| Automobile Refinishing | 2401005000 | 81112 |  |
|  |  | 4411 |  |
|  |  | 4412 |  |
| Traffic Paints | 2401008000 | N/A | Road Miles coming from 2006 HPMS Data |
| Wood and Composition Flat Stock | 2401015000 | 321 |  |
| Wood Furniture and Fixtures | 2401020000 | 337110 |  |
|  |  | 337121 |  |
|  |  | 337122 |  |
|  |  | 337127 | 50% to this and 50% to metal furniture |
|  |  | 337129 |  |
|  |  | 337211 |  |
|  |  | 337212 |  |
|  |  | 337215 | 50% to this and 50% to metal furniture |
|  |  | 339111 | 50% to this and 50% to metal furniture |
| Metal Furniture | 2401025000 | 337124 |  |
|  |  | 337127 | 50% to this and 50% to wood furniture |
|  |  | 337214 |  |
|  |  | 337215 | 50% to this and 50% to wood furniture |
|  |  | 339111 | 50% to this and 50% to wood furniture |
| Paper, Film and Foil | 2401030000 | 322221 |  |
|  |  | 322222 |  |
|  |  | 322223 |  |
|  |  | 322225 |  |
|  |  | 322226 |  |
| Metal Cans | 2401040000 | 33243 |  |
| Metal Sheet, Strip and Coils | 2401045000 | 332812 |  |
|  |  | 339911 |  |
|  |  | 339912 |  |
|  |  | 339914 |  |
| Misc. Finished Metals | 2401050000 |  | Consolidated with Miscellaneous Manufacturing |
| Machinery and Equipment | 2401055000 | 3331 |  |
|  |  | 3332 |  |
|  |  | 3333 |  |
|  |  | 33341 |  |
| Appliances | 2401060000 | 3352 |  |
| Electronic and Other Electrical Coatings | 2401065000 | 331319 |  |
|  |  | 331422 |  |
|  |  | 331491 |  |
|  |  | 335921 |  |
|  |  | 335929 |  |
|  |  | 335311 |  |
| Motor Vehicles | 2401070000 | 3361 |  |
|  |  | 3362 |  |
|  |  | 3363 |  |
| Aircraft | 2401075000 | 3364 |  |
| Railroads | 2401085000 | 3365 |  |
| Marine coatings | 2401080000 | 3366 |  |
|  |  | 488390 |  |
| Misc. Manufacturing | 2401090000 | 339 |  |
|  |  | 3369 |  |
| Industrial Maintenance Coatings | 2401100000 | N/A | Population based Category |
| Other Special Purpose Coatings | 2401200000 | N/A | Population based Category |
| Cleaning Products: Industrial and Institutional | 2415000000 | 331 |  |
|  |  | 332 |  |
|  |  | 333 |  |
|  |  | 334 |  |
|  |  | 335 |  |
|  |  | 336 |  |
|  |  | 337 |  |
|  |  | 339 |  |
|  |  | 441 |  |
|  |  | 483 |  |
|  |  | 484 |  |
|  |  | 485 |  |
|  |  | 488 |  |
|  |  | 8111 |  |
|  |  | 8112 |  |
| Dry Cleaning | 2420000000 | 81231 |  |
|  |  | 81232 |  |
|  |  | 81233 |  |
| Graphic Arts | 2425000000 | 32311 | Category Calculated using either population or employment |
|  |  | 322211 |  |
|  |  | 322212 |  |
|  |  | 322213 |  |
|  |  | 322214 |  |
|  |  | 322215 |  |
|  |  | 322221 |  |
|  |  | 322222 |  |
|  |  | 322223 |  |
|  |  | 322224 |  |
|  |  | 322225 |  |
|  |  | 322226 |  |
|  |  | 322231 |  |
|  |  | 322232 |  |
|  |  | 322233 |  |
|  |  | 322291 |  |
|  |  | 322299 |  |
| Personal Care Products (Cosmetics and Toiletries) | 2460100000 | N/A | Population based Category |
| Cleaning Products: Household | 2460200000 | N/A | Population based Category |
| Automotive Aftermarket (Transportation: Motor Vehicles) | 2460400000 | N/A | Population based Category |
| Coatings and Related Products | 2460500000 | N/A | Population based Category |
| Adhesives and Sealants | 2460600000 | N/A | Population based Category |
| FIFRA Regulated Products | 2460800000 | N/A | Population based Category |
| Misc. Products | 2460900000 | N/A | Population based Category |

Table 2.  National Emission Factors

| SCC | Description | Pollutant Code | Factor Numeric Value | Factor Unit Numerator | Factor Unit Denominator | Calculation Material Code | Material Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2401005000 | SC: Automobile Refinishing | 107211 | 0.1424 | LB | EACH | 992 | Employee |
| 2401005000 | SC: Automobile Refinishing | 171 | 0.84817 | LB | EACH | 992 | Employee |
| 2401005000 | SC: Automobile Refinishing | 78933 | 10.591 | LB | EACH | 992 | Employee |
| 2401005000 | SC: Automobile Refinishing | 108101 | 2.848 | LB | EACH | 992 | Employee |
| 2401005000 | SC: Automobile Refinishing | 108883 | 1.602 | LB | EACH | 992 | Employee |
| 2401005000 | SC: Automobile Refinishing | 1330207 | 1.78 | LB | EACH | 992 | Employee |
| 2401005000 | SC: Automobile Refinishing | VOC | 89 | LB | EACH | 992 | Employee |
| 2401008000 | SC: Traffic Paint | 71556 | 2.2155 | LB | ROAD MILE | 225 | Paint |
| 2401008000 | SC: Traffic Paint | 78933 | 1.3504 | LB | ROAD MILE | 225 | Paint |
| 2401008000 | SC: Traffic Paint | 108101 | 1.5403 | LB | ROAD MILE | 225 | Paint |
| 2401008000 | SC: Traffic Paint | 108883 | 0.18568 | LB | ROAD MILE | 225 | Paint |
| 2401008000 | SC: Traffic Paint | VOC | 22.1 | LB | ROAD MILE | 225 | Paint |
| 2401015000 | SC: Wood and Composition Flat Stock | 107211 | 0.1935 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | 171 | 0.59426 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | 78933 | 1.548 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | 108101 | 3.569 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | 71556 | 0.344 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | 108883 | 1.118 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | 1330207 | 0.946 | LB | EACH | 992 | Employee |
| 2401015000 | SC: Wood and Composition Flat Stock | VOC | 43 | LB | EACH | 992 | Employee |
| 2401020000 | SC: Wood Furniture and Fixtures | VOC | 244 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 107211 | 3.474 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 171 | 10.66904 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 78933 | 27.792 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 108101 | 64.076 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 71556 | 6.176 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 108883 | 20.072 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | 1330207 | 16.984 | LB | EACH | 992 | Employee |
| 2401025000 | SC: Metal Furniture | VOC | 772 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 107211 | 3.3075 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 171 | 10.1577 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 78933 | 26.46 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 108101 | 61.005 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 71556 | 5.88 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 108883 | 19.11 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | 1330207 | 16.17 | LB | EACH | 992 | Employee |
| 2401030000 | SC: Paper, Film and Foil | VOC | 735 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 107211 | 10.467 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 171 | 32.14532 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 78933 | 83.736 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 108101 | 193.058 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 71556 | 18.608 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 108883 | 60.476 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | 1330207 | 51.172 | LB | EACH | 992 | Employee |
| 2401040000 | SC: Metal Cans | VOC | 2,326 | LB | EACH | 992 | Employee |
| 2401045000 | SC: Metal Sheet, Strip and Coils | VOC | 2,877 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 107211 | 0.4905 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 171 | 1.50638 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 78933 | 3.924 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 108101 | 9.047 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 71556 | 0.872 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 108883 | 2.834 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | 1330207 | 2.398 | LB | EACH | 992 | Employee |
| 2401055000 | SC: Machinery and Equipment | VOC | 109 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 107211 | 1.1205 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 171 | 3.44118 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 78933 | 8.964 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 108101 | 20.667 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 71556 | 1.992 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 108883 | 6.474 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | 1330207 | 5.478 | LB | EACH | 992 | Employee |
| 2401060000 | SC: Appliances | VOC | 249 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 107211 | 0.11115 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 171 | 0.341354 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 78933 | 0.8892 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 108101 | 2.0501 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 71556 | 0.1976 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 108883 | 0.6422 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | 1330207 | 0.5434 | LB | EACH | 992 | Employee |
| 2401065000 | SC: Electronic and Other Electrical Coatings | VOC | 24.7 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 107211 | 0.738 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 171 | 2.26648 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 78933 | 5.904 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 108101 | 13.612 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 71556 | 1.312 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 108883 | 4.264 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | 1330207 | 3.608 | LB | EACH | 992 | Employee |
| 2401070000 | SC: Motor Vehicles (including motor vehicle parts) | VOC | 164 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 107211 | 0.0675 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 171 | 0.2073 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 78933 | 0.54 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 108101 | 1.245 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 71556 | 0.12 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 108883 | 0.39 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | 1330207 | 0.33 | LB | EACH | 992 | Employee |
| 2401075000 | SC: Aircraft | VOC | 15 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 107211 | 0.891 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 171 | 2.73636 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 78933 | 7.128 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 108101 | 16.434 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 71556 | 1.584 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 108883 | 5.148 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | 1330207 | 4.356 | LB | EACH | 992 | Employee |
| 2401080000 | SC: Marine | VOC | 198 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 107211 | 0.999 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 171 | 3.06804 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 78933 | 7.992 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 108101 | 18.426 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 71556 | 1.776 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 108883 | 5.772 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | 1330207 | 4.884 | LB | EACH | 992 | Employee |
| 2401085000 | SC: Railroads | VOC | 222 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 107211 | 0.612 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 171 | 1.87952 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 78933 | 4.896 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 108101 | 11.288 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 71556 | 1.088 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 108883 | 3.536 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | 1330207 | 2.992 | LB | EACH | 992 | Employee |
| 2401090000 | SC: Misc. Manufacturing | VOC | 136 | LB | EACH | 992 | Employee |
| 2401200000 | SC: Other Special Purpose Coatings | 107211 | 0.0000315 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | 171 | 0.0000967 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | 78933 | 0.000252 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | 108101 | 0.000581 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | 71556 | 0.000056 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | 108883 | 0.000182 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | 1330207 | 0.000154 | LB | EACH | 762 | Person |
| 2401200000 | SC: Other Special Purpose Coatings | VOC | 0.007 | LB | EACH | 762 | Person |
| 2415000000 | Cleaning Products: Industrial and Institutional | 79016 | 0.2092418 | LB | EACH | 992 | Employee |
| 2415000000 | Cleaning Products: Industrial and Institutional | VOC | 30.5 | LB | EACH | 992 | Employee |
| 2420000000 | Dry Cleaning | VOC | 467 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | 107211 | 0.0163326 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | 67561 | 0.1633260 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | 78933 | 0.0391982 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | 108101 | 0.0097996 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | 108883 | 0.1665924 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | 1330207 | 0.0424647 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | VOC | 4.4 | LB | EACH | 762 | Person |
| 2425000000 | Graphic Arts | 107211 | 5.5011136 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | 67561 | 55.011136 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | 78933 | 13.202673 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | 108101 | 3.3006682 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | 108883 | 56.111359 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | 1330207 | 14.302895 | LB | EACH | 992 | Employee |
| 2425000000 | Graphic Arts | VOC | 1,482 | LB | EACH | 992 | Employee |
| 2460100000 | Personal Care Products (Cosmetics and Toiletries) | 67561 | 0.255436 | LB | EACH | 762 | Person |
| 2460100000 | Personal Care Products (Cosmetics and Toiletries) | 108883 | 0.005092 | LB | EACH | 762 | Person |
| 2460100000 | Personal Care Products (Cosmetics and Toiletries) | VOC | 1.9 | LB | EACH | 762 | Person |
| 2460200000 | Cleaning Products: Household | 67561 | 0.241992 | LB | EACH | 762 | Person |
| 2460200000 | Cleaning Products: Household | 108883 | 0.004824 | LB | EACH | 762 | Person |
| 2460200000 | Cleaning Products: Household | VOC | 1.8 | LB | EACH | 762 | Person |
| 2460400000 | Automotive Aftermarket (Transportation: Motor Vehicles) | 67561 | 0.1828384 | LB | EACH | 762 | Person |
| 2460400000 | Automotive Aftermarket (Transportation: Motor Vehicles) | 108883 | 0.0036448 | LB | EACH | 762 | Person |
| 2460400000 | Automotive Aftermarket (Transportation: Motor Vehicles) | VOC | 1.36 | LB | EACH | 762 | Person |
| 2460500000 | Coatings and Related Products | 67561 | 0.127718 | LB | EACH | 762 | Person |
| 2460500000 | Coatings and Related Products | 108883 | 0.002546 | LB | EACH | 762 | Person |
| 2460500000 | Coatings and Related Products | VOC | 0.95 | LB | EACH | 762 | Person |
| 2460600000 | Adhesives and Sealants | 67561 | 0.0766308 | LB | EACH | 762 | Person |
| 2460600000 | Adhesives and Sealants | 108883 | 0.0015276 | LB | EACH | 762 | Person |
| 2460600000 | Adhesives and Sealants | VOC | 0.57 | LB | EACH | 762 | Person |
| 2460800000 | FIFRA Regulated Products | 67561 | 0.2393032 | LB | EACH | 762 | Person |
| 2460800000 | FIFRA Regulated Products | 108883 | 0.0047704 | LB | EACH | 762 | Person |
| 2460800000 | FIFRA Regulated Products | VOC | 1.78 | LB | EACH | 762 | Person |
| 2460900000 | Miscellaneous Products | 67561 | 0.0094108 | LB | EACH | 762 | Person |
| 2460900000 | Miscellaneous Products | 108883 | 0.0001876 | LB | EACH | 762 | Person |
| 2460900000 | Miscellaneous Products | VOC | 0.07 | LB | EACH | 762 | Person |

Table 3.  Emission Factors Which Vary Based on Presence of Controls

| SCC | Pollutant Code | Factor Numeric Value Uncontrolled | Factor Numeric Value Controlled | Factor Unit Numerator | Factor Unit Denominator | Calculation Material Code | Material Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2401001000 | 123911 | 6.18E-05 | 4.82E-05 | LB | EACH | 762 | Person |
| 2401001000 | 584849 | 6.18E-05 | 4.82E-05 | LB | EACH | 762 | Person |
| 2401001000 | 101688 | 0.000433 | 0.000337 | LB | EACH | 762 | Person |
| 2401001000 | 75070 | 0.000309 | 0.000241 | LB | EACH | 762 | Person |
| 2401001000 | 117817 | 9.27E-05 | 7.23E-05 | LB | EACH | 762 | Person |
| 2401001000 | 98828 | 0.001174 | 0.000916 | LB | EACH | 762 | Person |
| 2401001000 | 84742 | 6.18E-05 | 4.82E-05 | LB | EACH | 762 | Person |
| 2401001000 | 131113 | 3.09E-05 | 2.41E-05 | LB | EACH | 762 | Person |
| 2401001000 | 100414 | 0.007663 | 0.005977 | LB | EACH | 762 | Person |
| 2401001000 | 107211 | 0.156014 | 0.121681 | LB | EACH | 762 | Person |
| 2401001000 | 50000 | 6.18E-05 | 4.82E-05 | LB | EACH | 762 | Person |
| 2401001000 | 171 | 0.063809 | 0.049767 | LB | EACH | 762 | Person |
| 2401001000 | 110543 | 0.000464 | 0.000362 | LB | EACH | 762 | Person |
| 2401001000 | 67561 | 0.031147 | 0.024293 | LB | EACH | 762 | Person |
| 2401001000 | 80626 | 0.000371 | 0.000289 | LB | EACH | 762 | Person |
| 2401001000 | 71556 | 0.02883 | 0.022485 | LB | EACH | 762 | Person |
| 2401001000 | 108101 | 0.009301 | 0.007254 | LB | EACH | 762 | Person |
| 2401001000 | 78933 | 0.041684 | 0.032511 | LB | EACH | 762 | Person |
| 2401001000 | 75092 | 0.018447 | 0.014388 | LB | EACH | 762 | Person |
| 2401001000 | 91203 | 0.001421 | 0.001109 | LB | EACH | 762 | Person |
| 2401001000 | 100425 | 0.003152 | 0.002458 | LB | EACH | 762 | Person |
| 2401001000 | 127184 | 0.000185 | 0.000145 | LB | EACH | 762 | Person |
| 2401001000 | 108883 | 0.072059 | 0.056201 | LB | EACH | 762 | Person |
| 2401001000 | 121448 | 0.000185 | 0.000145 | LB | EACH | 762 | Person |
| 2401001000 | 108054 | 0.000371 | 0.000289 | LB | EACH | 762 | Person |
| 2401001000 | 1330207 | 0.088096 | 0.068709 | LB | EACH | 762 | Person |
| 2401001000 | VOC | 3.09 | 2.41 | LB | EACH | 762 | Person |
| 2401100000 | 107211 | 0.00495 | 0.000675 | LB | EACH | 762 | Person |
| 2401100000 | 171 | 0.015202 | 0.002073 | LB | EACH | 762 | Person |
| 2401100000 | 71556 | 0.0088 | 0.0012 | LB | EACH | 762 | Person |
| 2401100000 | 108101 | 0.0913 | 0.01245 | LB | EACH | 762 | Person |
| 2401100000 | 78933 | 0.0396 | 0.0054 | LB | EACH | 762 | Person |
| 2401100000 | 108883 | 0.0286 | 0.0039 | LB | EACH | 762 | Person |
| 2401100000 | 1330207 | 0.0242 | 0.0033 | LB | EACH | 762 | Person |
| 2401100000 | VOC | 1.1 | 0.15 | LB | EACH | 762 | Person |

Table 4.  States to Which Controlled Emission Factors are Applied

| State FIPS | Abbreviation |
| --- | --- |
| 04 | AZ |
| 06 | CA |
| 09 | CT |
| 10 | DE |
| 11 | DC |
| 23 | ME |
| 24 | MD |
| 25 | MA |
| 33 | NH |
| 34 | NJ |
| 36 | NY |
| 42 | PA |
| 44 | RI |
| 48 | TX |
| 50 | VT |
| 51 | VA |