FUGITIVE DUST FROM UNPAVED ROADS

a. Source Category Description

Fugitive dust emissions from unpaved road traffic were estimated for PM10-PRI, PM10-FIL, PM25-PRI, and PM25-FIL.  Since there are no PM-CON emissions for this category, PM10-PRI emissions are equal to PM10-FIL emissions and PM25-PRI emissions are equal to PM25-FIL.

For this source category, the following SCC was assigned:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2296000000 | Mobile Sources | Unpaved Roads | All Unpaved Roads | Total: Fugitives |

Table 1 provides a national summary of the 2008 annual VMT activity and emissions by pollutant.

Table 1. 2008 National Criteria Air Pollutant Emissions Summary for Fugitive Dust from Unpaved Roads

| Pollutant Description and NIF 3.0 Pollutant Code | Emission Factor Reference | National Activity (Million Miles) | National Emissions (Tons) |
| --- | --- | --- | --- |
| PM10-FIL | Reference 1 | 41,700 | 8,448,784 |
| PM10-PRI | Reference 1 | 41,700 | 8,448,784 |
| PM25-FIL | Reference 1 | 41,700 | 841,571 |
| PM25-PRI | Reference 1 | 41,700 | 841,571 |

Uncontrolled unpaved road emissions were calculated at the State level by roadway class and month.  This was done by multiplying the State/roadway class unpaved roadway VMT by the appropriate monthly temporal allocation factor and by the monthly unpaved road emission factor.  After the unpaved road dust emissions were calculated at the State/roadway class/monthly level of detail, the uncontrolled emissions were then allocated to the county level using 1990 rural population data as a surrogate.  Next, control factors were applied to the unpaved road emissions in PM10 nonattainment area counties.  Emissions and VMT by roadway class were then totaled to the county level for reporting in the NEI.  The following provides further details on the emission factor equation, temporal and spatial allocation procedures, and controls.

b. Emission Factor Equation

Reentrained road dust emissions for unpaved roads were estimated using unpaved road VMT and the emission factor equation from AP-42:1

E =[ k * (s/12)1 * (SPD/30)0.5] / (M/0.5)0.2 - C where k, and C are empirical constants given in Table 2, with

k = particle size multiplier (lb/VMT)

E = size specific emission factor (lb/VMT)

s = surface material silt content (%)

SPD = mean vehicle speed (mph)

M = surface material moisture content (%)

C = emission factor for 1980's vehicle fleet exhaust, brake wear, and tire wear (lb/VMT)

The uncontrolled emission factors are provided in the attachment “Unpaved Roads PM Emission Factors.xls” by State, roadway class, and month.

Average State-level unpaved road silt content values, developed as part of the 1985 NAPAP Inventory, were obtained from the Illinois State Water Survey.2  Silt contents of over 200 unpaved roads from over 30 States were obtained.  Average silt contents of unpaved roads were calculated for each sate that had three or more samples for that State.  For States that did not have three or more samples, the average for all samples from all States was used as a default value.  The silt content values by State, and identifies if the values were based on a sample average or default value.

Table 2. Constants for Unpaved Roads Reentrained Dust Emission Factor Equation1

| Constant | PM25-PRI/ PM25-FIL | PM10-PRI/ PM10-FIL |
| --- | --- | --- |
| k (lb/VMT) | 0.18 | 1.8 |
| C | 0.00036 | 0.00047 |

Table 3 lists the speeds modeled on the unpaved roads by roadway class.  These speeds were determined based on the average speeds modeled for onroad emission calculations and weighted to determine a single average speed for each of the roadway classes.  The value of 0.5 percent for M was chosen as the national default as sufficient resources were not available at the time the emissions were calculated to determine more locally-specific values for this variable.

Table 3. Speeds Modeled by Roadway Type on Unpaved Roads

Unpaved Roadway Type	Speed (mph)

Rural Minor Arterial	39

Rural Major Collector	34

Rural Minor Collector	30

Rural Local	30

Urban Other Principal Arterial	20

Urban Minor Arterial	20

Urban Collector	20

Urban Local	20

Correction factors were applied to the emission factors to account for the number of days with a sufficient amount of precipitation to prevent road dust resuspension.  Monthly corrected emission factors by State and roadway classification were calculated using the following equation:

Ecorr = E * [(D-p)/D]

where:	Ecorr = unpaved road dust emission factor corrected for precipitation effects

E = uncorrected emission factor

D = number of days in the month

p = number of days in the month with at least 0.01 inches of precipitation

The number of days with at least 0.01 inches of precipitation in each month was obtained from the National Climatic Data Center.3  Data were collected from a meteorological station selected to be representative of rural areas within the State.  The monthly precipitation data used by State for 2008 are included in Appendix C.

c. Activity

Unpaved roadway mileage estimates were obtained from the FHWA’s annual Highway Statistics report.4  Unpaved mileage data for 2007 were used, as data for 2008 were not yet available.  Separate calculations of VMT were performed for county- and noncounty- (State or federally) maintained roadways.  State-level, county-maintained roadway mileage was organized by surface type (rural and urban) and the average daily traffic volume (ADTV) groups shown in Table 4.  From these data, State-level unpaved roadway mileage estimates were made.  The following equation was then used to calculate State-level unpaved road VMT estimates:

VMTUP = ADTV * FSRM * 366 days/year

where:	VMTUP = VMT on unpaved roads (miles/year)

ADTV   = average daily traffic volume (vehicles/day/mile) FSRM   = functional system roadway mileage (miles)

State and federally maintained roadway mileage was categorized by arterial classification, not roadway traffic volume; therefore, the VMT was calculated differently than for county-maintained roadways. The ADTV was assumed to not vary by roadway maintenance responsibility, so the ADTV calculated from county-maintained VMT and mileage (ADTV = VMT / Mileage) was used with noncounty-maintained roadway mileage to calculate VMT in the above equation.

Table 4. Assumed Values for Average Daily Traffic Volume (ADTV) by Volume Group

|  | Rural Roads |  |  |  |
| --- | --- | --- | --- | --- |
| Volume Category (vehicles per day per mile) | < 50 | 50-199 | 200-499 | > 500 |
| Assumed ADTV | 5* | 125** | 350** | 550*** |
|  | Urban Roads |  |  |  |
| Volume Category (vehicles per day per mile) | < 200 | 200-499 | 500-1999 | > 2000 |
| Assumed ADTV | 20* | 350** | 1250** | 2200*** |

Notes:  *10% of volume group’s maximum range endpoint. 
** Average of volume group’s range endpoints. 
*** 110% of volume group’s minimum range endpoint.

d. Allocation

The unpaved road VMT estimates by State/roadway class were first temporally allocated by season using the NAPAP Inventory seasonal temporal allocations factors for VMT.5  These factors are provided in the attachment “Paved and Unpaved Road VMT temp factors.xls”.  The seasonal VMT values were then multiplied by the ratio of the number of days in a month to the number of days in a season to adjust to monthly VMT.  The emission factors were then applied to estimate emissions by month.

The State/roadway class unpaved road emissions were then spatially allocated to each county using estimates of the ratio of 1990 county rural population to the State rural population from the U.S. Census Bureau6 as shown by the following equation:

EMISx,y =(CLx / SL) * EMIS,y

where:	EMISx,y = unpaved road emissions (tons) for county x and roadway class y

CLx = rural population in county x SL = rural population in the State EMIS,y = unpaved road emissions in entire State for roadway class y

The county-level allocation factors are provided in the attachment “Unpaved Road county allocation fractions.xls.”

e. Controls

The controls assumed for unpaved roads varied by PM10 nonattainment area classification and by urban and rural areas.  On urban unpaved roads in moderate PM10 nonattainment areas, paving of the unpaved road was assumed, and a control efficiency of 96 percent and a rule penetration of 50 percent were applied.  Chemical stabilization, with a control efficiency or 75 percent and a rule penetration of 50 percent, was assumed for rural areas in serious PM10 nonattainment areas.  A combination of paving and chemical stabilization, with a control efficiency of 90 percent and a rule penetration of 75 percent, was assumed for urban unpaved roads in serious PM10 nonattainment areas.7

Note that the controls were applied at the county/roadway class level, and the controls differ by roadway class.  In the NIF 3.0 emissions table, the emissions for all roadway classes were summed to the county level.  Therefore, the emissions at the county level can represent several different control, rule effectiveness, and rule penetration levels.  As a result, the control efficiency, rule effectiveness, and rule penetration values were reported in the control equipment table as a composite, overall control level for each county; the rule effectiveness and rule penetration values were not reported separately in the emissions table.

f. References

- United States Environmental Protection Agency, Office of Air Quality Planning and Standards. “Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, Volume I: Stationary Point and Area Sources, Section 13.2.2, Unpaved Roads.” Research Triangle Park, NC. 2003.

- W. Barnard, G. Stensland, and D. Gatz, Illinois State Water Survey, “Evaluation of Potential Improvements in the Estimation of Unpaved Road Fugitive Emission Inventories,” paper 87-58.1, presented at the 80th Annual Meeting of the APCA . New York, New York. June 21-26, 1987.

- U.S. Department of Commerce, National Oceanic and Atmospheric Administration. Summary of the Day Element TD-3200, 2008 data provided via FTP. National Climatic Data Center, 2009.

- U.S. Department of Transportation, Federal Highway Administration. Highway Statistics 2007. Office of Highway Policy Information. Washington, DC. 2009. Available at http://www.fhwa.dot.gov/policyinformation/statistics/2007/.

- U.S. Environmental Protection Agency. “The 1985 NAPAP Emissions Inventory: Development of Temporal Allocation Factors,” EPA-600/7-89-010d. Air & Energy Engineering Research Laboratory. Research Triangle Park, NC. April 1990.

- U.S. Department of Commerce. “1990 Census of Population , Volume I Characteristics of Population,” Bureau of the Census. Washington, DC. July 1992.

- E.H. Pechan & Associates, Inc. “Phase II Regional Particulate Strategies; Task 4: Particulate Control Technology Characterization,” draft report prepared for U.S. Environmental Protection Agency, Office of Policy, Planning and Evaluation. Washington, DC. June 1995.

- EPA, 2006. Western Governors' Association. 2006. Background Document for Revisions to Fine Fraction Ratios Used for AP-42 Fugitive Dust Emission Factors. Western Regional Air Partnership (WRAP), 1515 Cleveland Place, Suite 200, Denver, Colorado 80202 .