GASOLINE DISTRIBUTION:  STAGE I

a. Source Category Description

Stage I gasoline distribution includes the following gasoline emission points:  1) bulk terminals; 2) pipeline facilities; 3) bulk plants; 4) tank trucks; and 5) service stations.  Emissions from Stage I gasoline distribution occur as gasoline vapors are released into the atmosphere.  These Stage I processes are subject to EPA’s maximum available control technology (MACT) standards for gasoline distribution.1

Emissions from gasoline distribution at bulk terminals and bulk plants take place when gasoline is loaded into a storage tank or tank truck, from working losses (for fixed roof tanks), and from working losses and roof seals (for floating roof tanks).  Working losses consist of both breathing and emptying losses.  Breathing losses are the expulsion of vapor from a tank vapor space that has expanded or contracted because of daily changes in temperature and barometric pressure; these emissions occur in the absence of any liquid level change in the tank.  Emptying losses occur when the air that is drawn into the tank during liquid removal saturates with hydrocarbon vapor and expands, thus exceeding the fixed capacity of the vapor space and overflowing through the pressure vacuum valve.2

Emissions from tank trucks in transit occur when gasoline vapor evaporates from (1) loaded tank trucks during transportation of gasoline from bulk terminals/plants to service stations, and (2) empty tank trucks returning from service stations to bulk terminals/plants.3 Pipeline emissions result from the valves and pumps found at pipeline pumping stations and from the valves, pumps, and storage tanks at pipeline breakout stations.  Stage I gasoline distribution emissions also occur when gasoline vapors are displaced from storage tanks during unloading of gasoline from tank trucks at service stations (Gasoline Service Station Unloading) and from gasoline vapors evaporating from service station storage tanks and from the lines going to the pumps (Underground Storage Tank Breathing and Emptying).

The following SCCs are included in Stage I Gasoline Distribution:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2501050120 | Storage and Transport | Petroleum and Petroleum Product Storage | Bulk Terminals: All Evaporative Losses | Gasoline |
| 2501055120 | Storage and Transport | Petroleum and Petroleum Product Storage | Bulk Plants: All Evaporative Losses | Gasoline |
| 2501060051 | Storage and Transport | Petroleum and Petroleum Product Storage | Gasoline Service Stations | Stage 1: Submerged Filling |
| 2501060052 | Storage and Transport | Petroleum and Petroleum Product Storage | Gasoline Service Stations | Stage 1: Splash Filling |
| 2501060053 | Storage and Transport | Petroleum and Petroleum Product Storage | Gasoline Service Stations | Stage 1: Balanced Submerged Filling |
| 2501060201 | Storage and Transport | Petroleum and Petroleum Product Storage | Gasoline Service Stations | Underground Tank: Breathing and Emptying |
| 2505030120 | Storage and Transport | Petroleum and Petroleum Product Transport | Truck | Gasoline |
| 2505040120 | Storage and Transport | Petroleum and Petroleum Product Transport | Pipeline | Gasoline |

b. Bulk Terminals and Pipelines

There are no generally accepted activity-based VOC emission factors for the pipelines and bulk terminals sectors because they are generally treated as point sources whose emissions are estimated using site-specific information.  For example, emission estimates for bulk terminal storage tanks are typically derived from tank specific parameters that are input into the TANKS program.4 Therefore, for bulk terminals and pipelines, EPA estimated 2008 national VOC emissions by multiplying 1998 national estimates developed in support of the Gasoline Distribution MACT standard5 by the 2008 to 1998 ratio of the national volume of wholesale gasoline supplied (see Table 1).  The gasoline supply information was obtained from Table 2 in Volume I of Petroleum Supply Annual 2008.6

Table 1. Estimation of National 2008 VOC Emissions for Pipelines and Bulk Terminals

| Category | 1998 Post-MACT Control Emissions (Mg) | Mg to Ton Conversion Factor | 1998 Emissions (tons) | Ratio of 2008 to 1998 Gasoline Supplied | 2008 Emissions (tons) |
| --- | --- | --- | --- | --- | --- |
| Pipelines | 79,830 | 1.1023 | 87,997 | (8,989 thousand barrels per day / 8,253 thousand barrels per day) = 1.089 | 95,844 |
| Bulk Terminals | 137,555 | 1.1023 | 151,627 |  | 165,149 |

To estimate HAP emissions, EPA applied national average speciation profiles to the VOC emission estimates.7  Table 2 presents these speciation profiles and the national bulk terminal and pipeline HAP emission estimates (note that unless otherwise noted, all emission values reported in this section exclude estimates for Puerto Rico and the U.S. Virgin Islands). EPA used total VOC emission estimates, so emissions represent total emissions. Where necessary, States should perform point source subtractions to obtain nonpoint emissions. The following describes how total national VOC estimates were allocated to counties.

Table 2. HAP Speciation Profiles and 2008 Bulk Terminal and Pipeline Emissions

| HAP | Pollutant Code | Percentage of VOC Emissions | Reference | 2008 National Emissions (tons) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Bulk Terminals | Pipelines |
| Benzene | 71432 | 0.27 | 7 | 4.46E+02 | 2.59E+02 |
| 2,2,4-Trimethylpentane | 540841 | 0.75 | 7 | 1.24E+03 | 7.19E+02 |
| Cumene | 98828 | 0.012 | 7 | 1.98E+01 | 1.15E+01 |
| Ethyl Benzene | 100414 | 0.053 | 7 | 8.75E+01 | 5.08E+01 |
| n-Hexane | 110543 | 1.8 | 7 | 2.97E+03 | 1.73E+03 |
| Naphthalene | 91203 | 0.00027 | 7 | 4.46E-01 | 2.59E-01 |
| Toluene | 108883 | 1.4 | 7 | 2.31E+03 | 1.34E+03 |
| Xylenes | 1330207 | 0.56 | 7 | 9.25E+02 | 5.37E+02 |

For both categories, EPA allocated national VOC and HAP emissions for these categories in a two-step manner.  First, EPA allocated emissions based on 2008 gasoline supply data reported by the U.S. Department of Energy (DOE).  Next, EPA allocated emissions based on employment data reported in the 2007 County Business Patterns.8

For pipelines, EPA allocated emissions to Petroleum Administration for Defense (PAD) Districts based on the total amount of finished motor gasoline moved by pipeline in each PAD in year 2008.  There are five PAD Districts across the United States:   PAD District 1 comprises seventeen states plus the District of Columbia along the Atlantic Coast; PAD District 2 comprises fifteen states in the Midwest; PAD District 3 comprises six states in South Central U.S.; PAD District 4 comprises five states in the Rocky Mountains; and PAD District 5 comprises seven states along the West Coast. These data, which are displayed below in Table 3, are reported in Table 35 of Volume 1 of Petroleum Supply Annual 2008.9  Next, EPA allocated pipeline emissions in each PAD District to counties based on County Business Patterns employment data.  Because employment data for NAICS code 48691 (Pipeline Transportation of Refined Petroleum Products) are often withheld due to confidentiality reasons, EPA used the number of employees in NAICS code 42471 (Petroleum Bulk Stations and Terminals) for this allocation.  To better account for the location of refined petroleum pipelines, however, EPA did not allocate any activity to States which had employees in this NAICS code, but did not have employees in NAICS code 48691 (i.e., District of Columbia, Idaho, Maine, New Hampshire, Vermont, and West Virginia).

Table 3. Movement of Finished Motor Gasoline by Pipeline Between PAD Districts, 2008

|  | From I | From II | From III | From IV | From V |
| --- | --- | --- | --- | --- | --- |
| To I | n/a | 393 | 333,462 | 0 | 0 |
| To II | 70,895 | n/a | 99,167 | 7,442 | 0 |
| To III | 0 | 9,193 | n/a | 0 | 0 |
| To IV | 0 | 8,680 | 5,778 | n/a | 0 |
| To V | 0 | 0 | 25,453 | 9,287 | n/a |

For bulk terminals, EPA first allocated national emissions to States based on the 2008 refinery, bulk terminal, and natural gas plant stocks of motor gasoline reported for each State in Table 33 of Volume 1 of DOE’s Petroleum Supply Annual 2008 (see Table 4).9  Next, EPA allocated emissions in each State to counties based on the number of NAICS code 42471 (Petroleum Bulk Stations and Terminals) employees reported in the 2007 County Business Patterns.8

Table 4. Refinery, Bulk Terminal, and Natural Gas Plant Stocks of Motor Gasoline, 2008

| State | Motor Gasoline (Thousand Barrels) | State | Motor Gasoline (Thousand Barrels) |
| --- | --- | --- | --- |
| Alabama | 1,090 | Montana | 872 |
| Alaska | 616 | Nebraska | 658 |
| Arizona | 470 | Nevada | 102 |
| Arkansas | 819 | New Hampshire | 0 |
| California | 460 | New Jersey | 2,956 |
| Colorado | 748 | New Mexico | 350 |
| Connecticut | 0 | New York | 1,469 |
| Delaware | 105 | North Carolina | 1,724 |
| District of Columbia | 0 | North Dakota | 291 |
| Florida | 1,877 | Ohio | 2,724 |
| Georgia | 1,724 | Oklahoma | 1,245 |
| Hawaii | 12 | Oregon | 525 |
| Idaho | 181 | Pennsylvania | 3,595 |
| Illinois | 1,940 | Rhode Island | 0 |
| Indiana | 2,464 | South Carolina | 720 |
| Iowa | 1,090 | South Dakota | 283 |
| Kansas | 2,347 | Tennessee | 923 |
| Kentucky | 1,045 | Texas | 9,530 |
| Louisiana | 5,209 | Utah | 793 |
| Maine | 374 | Vermont | 31 |
| Maryland | 31 | Virginia | 1,285 |
| Massachusetts | 0 | Washington | 1,902 |
| Michigan | 1,772 | West Virginia | 183 |
| Minnesota | 1,305 | Wisconsin | 704 |
| Mississippi | 1,580 | Wyoming | 910 |
| Missouri | 491 |  |  |

It is important to reiterate that the above discussion addresses the calculation of total VOC emissions. The 2008 point source NEI reports VOC emissions related to bulk terminal and pipeline processes. To obtain nonpoint emissions, States should subtract the 2008 point source VOC emission estimates from the total VOC emission estimates reported here. The relevant point source SCCs are listed in Tables 5 and 6 below.

Table 5. Pipeline Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40600501 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Pipeline Petroleum Transport - General - All Products | Pipeline Leaks |
| 40600502 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Pipeline Petroleum Transport - General - All Products | Pipeline Venting |
| 40600503 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Pipeline Petroleum Transport - General - All Products | Pump Station |
| 40600504 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Pipeline Petroleum Transport - General - All Products | Pump Station Leaks |

Table 6. Bulk Terminal Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40400101 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Breathing Loss (67000 Bbl Capacity) - Fixed Roof Tank |
| 40400102 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Breathing Loss (67000 Bbl Capacity) - Fixed Roof Tank |
| 40400103 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Breathing Loss (67000 Bbl. Capacity) - Fixed Roof Tank |
| 40400104 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Breathing Loss (250000 Bbl Capacity)-Fixed Roof Tank |
| 40400105 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Breathing Loss (250000 Bbl Capacity)-Fixed Roof Tank |
| 40400106 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Breathing Loss (250000 Bbl Capacity) - Fixed Roof Tank |
| 40400107 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Working Loss (Diam. Independent) - Fixed Roof Tank |
| 40400108 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Working Loss (Diameter Independent) - Fixed Roof Tank |
| 40400109 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Working Loss (Diameter Independent) - Fixed Roof Tank |
| 40400110 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Standing Loss (67000 Bbl Capacity)-Floating Roof Tank |
| 40400111 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Standing Loss (67000 Bbl Capacity)-Floating Roof Tank |
| 40400112 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Standing Loss (67000 Bbl Capacity)- Floating Roof Tank |
| 40400113 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Standing Loss (250000 Bbl Cap.) - Floating Roof Tank |
| 40400114 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Standing Loss (250000 Bbl Cap.) - Floating Roof Tank |
| 40400115 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Standing Loss (250000 Bbl Cap.) - Floating Roof Tank |
| 40400116 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13/10/7: Withdrawal Loss (67000 Bbl Cap.) - Float Rf Tnk |
| 40400117 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13/10/7: Withdrawal Loss (250000 Bbl Cap.) - Float Rf Tnk |
| 40400118 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Filling Loss (10500 Bbl Cap.) - Variable Vapor Space |
| 40400119 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Filling Loss (10500 Bbl Cap.) - Variable Vapor Space |
| 40400120 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Filling Loss (10500 Bbl Cap.) - Variable Vapor Space |
| 40400131 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Standing Loss - Ext. Floating Roof w/ Primary Seal |
| 40400132 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Standing Loss - Ext. Floating Roof w/ Primary Seal |
| 40400133 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Standing Loss - External Floating Roof w/ Primary Seal |
| 40400141 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Standing Loss - Ext. Floating Roof w/ Secondary Seal |
| 40400142 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Standing Loss - Ext. Floating Roof w/ Secondary Seal |
| 40400143 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Standing Loss - Ext. Floating Roof w/ Secondary Seal |
| 40400148 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13/10/7: Withdrawal Loss - Ext. Float Roof (Pri/Sec Seal) |
| 40400150 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Miscellaneous Losses/Leaks: Loading Racks |
| 40400151 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Valves, Flanges, and Pumps |
| 40400152 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Vapor Collection Losses |
| 40400153 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Vapor Control Unit Losses |
| 40400161 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Standing Loss - Int. Floating Roof w/ Primary Seal |
| 40400162 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Standing Loss - Int. Floating Roof w/ Primary Seal |
| 40400163 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Standing Loss - Internal Floating Roof w/ Primary Seal |
| 40400171 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13: Standing Loss - Int. Floating Roof w/ Secondary Seal |
| 40400172 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 10: Standing Loss - Int. Floating Roof w/ Secondary Seal |
| 40400173 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 7: Standing Loss - Int. Floating Roof w/ Secondary Seal |
| 40400178 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Gasoline RVP 13/10/7: Withdrawal Loss - Int. Float Roof (Pri/Sec Seal) |

c. Bulk Plants

EPA calculated VOC emissions from bulk plants by developing an average emission factor from the bulk plant motor gasoline VOC emissions and throughput data developed in support of the Gasoline Distribution MACT standards.2,5  To estimate 2008 national VOC emissions, the VOC emission factor (8.62 pounds of VOC per 1,000 gallons) was applied to the estimated national volume of gasoline passing through bulk plants in 2008.  The volume of bulk plant gasoline throughput was assumed to be 9 percent of total gasoline consumption.10 Total gasoline consumption for 2008 was assumed to be the same as the volume of finished motor gasoline supplied as reported on the U.S. Energy Information Administration’s Petroleum Navigator website.11 The resulting national VOC emission estimate was then allocated to counties based on employment data for NAICS code 42471 (Petroleum Bulk Stations and Terminals).  To estimate benzene emissions from bulk plants, EPA multiplied VOC emission estimates by county-level speciation profiles calculated from the annual onroad refueling (Stage 2) emissions from the 2008 NEI NMIM results.12 All other HAPs were estimated by multiplying VOC emissions by the national average speciation profiles displayed in Table 7.

Table 7.  Bulk Plant HAP Speciation Profiles and Total Emission Estimates

| Pollutant | Pollutant Code | Emission Factor | Reference | National Emissions (tpy) |
| --- | --- | --- | --- | --- |
| VOC | VOC | 8.62 lb./1,000 gallons | 2 and 5 | 5.35E+04 |
| 2,2,4-Trimethylpentane | 540841 | 0.75% of VOC | 7 | 4.01E+02 |
| Cumene | 98828 | 0.012% of VOC | 7 | 6.41E+00 |
| Ethyl Benzene | 100414 | 0.053% of VOC | 7 | 2.83E+01 |
| n-Hexane | 110543 | 1.8% of VOC | 7 | 9.62E+02 |
| Naphthalene | 91203 | 0.00027% of VOC | 7 | 1.44E-01 |
| Toluene | 108883 | 1.4% of VOC | 7 | 7.48E+02 |
| Xylenes | 1330207 | 0.56% of VOC | 7 | 2.99E+02 |
| Benzene | 71432 | county-specific % of VOC | 12 | 3.94E+02 |

It is important to reiterate that the above discussion addresses the calculation of total VOC emissions. The 2008 point source NEI reports VOC emissions related to bulk plants. To obtain nonpoint emissions, States should subtract the 2008 point source VOC emission estimates from the total VOC emission estimates reported here. The relevant point source SCCs are listed in Table 8 below.

Table 8. Bulk Plant Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40400201 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Breathing Loss (67000 Bbl Capacity) - Fixed Roof Tank |
| 40400202 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Breathing Loss (67000 Bbl Capacity) - Fixed Roof Tank |
| 40400203 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Breathing Loss (67000 Bbl. Capacity) - Fixed Roof Tank |
| 40400204 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Working Loss (67000 Bbl. Capacity) - Fixed Roof Tank |
| 40400205 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Working Loss (67000 Bbl. Capacity) - Fixed Roof Tank |
| 40400206 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Working Loss (67000 Bbl. Capacity) - Fixed Roof Tank |
| 40400207 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Standing Loss (67000 Bbl Cap.) - Floating Roof Tank |
| 40400208 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Standing Loss (67000 Bbl Cap.) - Floating Roof Tank |
| 40400209 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Standing Loss (67000 Bbl Cap.) - Floating Roof Tank |
| 40400210 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13/10/7: Withdrawal Loss (67000 Bbl Cap.) - Float Rf Tnk |
| 40400211 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Filling Loss (10500 Bbl Cap.) - Variable Vapor Space |
| 40400212 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Filling Loss (10500 Bbl Cap.) - Variable Vapor Space |
| 40400213 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Filling Loss (10500 Bbl Cap.) - Variable Vapor Space |
| 40400231 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Standing Loss - Ext. Floating Roof w/ Primary Seal |
| 40400232 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Standing Loss - Ext. Floating Roof w/ Primary Seal |
| 40400233 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Standing Loss - External Floating Roof w/ Primary Seal |
| 40400241 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Standing Loss - Ext. Floating Roof w/ Secondary Seal |
| 40400242 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Standing Loss - Ext. Floating Roof w/ Secondary Seal |
| 40400243 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Standing Loss - Ext. Floating Roof w/ Secondary Seal |
| 40400248 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10/13/7: Withdrawal Loss - Ext. Float Roof (Pri/Sec Seal) |
| 40400250 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Loading Racks |
| 40400251 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Valves, Flanges, and Pumps |
| 40400252 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Miscellaneous Losses/Leaks: Vapor Collection Losses |
| 40400253 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Miscellaneous Losses/Leaks: Vapor Control Unit Losses |
| 40400261 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Standing Loss - Int. Floating Roof w/ Primary Seal |
| 40400262 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Standing Loss - Int. Floating Roof w/ Primary Seal |
| 40400263 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Standing Loss - Internal Floating Roof w/ Primary Seal |
| 40400271 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 13: Standing Loss - Int. Floating Roof w/ Secondary Seal |
| 40400272 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10: Standing Loss - Int. Floating Roof w/ Secondary Seal |
| 40400273 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 7: Standing Loss - Int. Floating Roof w/ Secondary Seal |
| 40400278 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Gasoline RVP 10/13/7: Withdrawal Loss - Int. Float Roof (Pri/Sec Seal) |
| 40400401 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Petroleum Products - Underground Tanks | Gasoline RVP 13: Breathing Loss |
| 40400402 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Petroleum Products - Underground Tanks | Gasoline RVP 13: Working Loss |
| 40400403 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Petroleum Products - Underground Tanks | Gasoline RVP 10: Breathing Loss |
| 40400404 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Petroleum Products - Underground Tanks | Gasoline RVP 10: Working Loss |
| 40400405 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Petroleum Products - Underground Tanks | Gasoline RVP 7: Breathing Loss |
| 40400406 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Petroleum Products - Underground Tanks | Gasoline RVP 7: Working Loss |
| 40600101 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Splash Loading ** |
| 40600126 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Submerged Loading ** |
| 40600131 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Submerged Loading (Normal Service) |
| 40600136 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Splash Loading (Normal Service) |
| 40600141 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Submerged Loading (Balanced Service) |
| 40600144 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Splash Loading (Balanced Service) |
| 40600147 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Submerged Loading (Clean Tanks) |

d. Tank Trucks in Transit

The EPA calculated VOC emissions from Tank Trucks in Transit by multiplying county-level tank truck gasoline throughput by a 0.06 lb of VOC per 1,000 gallon emission factor.  As noted in Table 9, this emission factor is the sum of the individual emission factors reported in the Gasoline Distribution EIIP guidance document for gasoline-filled trucks (traveling to service station/bulk plant for delivery) and vapor-filled trucks (traveling to bulk terminal/plant for reloading).3 County-level gasoline consumption was estimated by summing county-level onroad and nonroad estimates. County-level onroad consumption was estimated by subtracting the NMIM-derived national nonroad consumption from the EIA’s estimate of finished motor gasoline supplied and then allocating to counties using NMIM-derived onroad county-level CO2 emissions.11,13 County-level nonroad consumption was estimated by allocating NMIM-derived state/SCC-level nonroad gasoline consumption to the county-level based on nonroad county/SCC-level CO2 emissions.13 Gasoline throughput for tank trucks was computed by multiplying the county-level gasoline consumption estimates by a factor of 1.09 to account for gasoline that is transported more than once in a given area (i.e., transported from bulk terminal to bulk plant and then from bulk plant to service station).10  Benzene emission estimates were calculated by multiplying county-level NMIM speciation profiles by the VOC emission estimates.12  Emissions for the remaining HAPs were calculated by multiplying VOC emissions by the national speciation profiles presented in Table 10.

Table 9. Tank Trucks in Transit VOC Emission Factors

|  | VOC Emission Factor |
| --- | --- |
| Vapor-Filled Trucks | 0.055 lb/1,000 gallons |
| Gasoline Filled Trucks | 0.005 lb/1,000 gallons |
| Total | 0.06 lb/1,000 gallons |

Table 10.  Tank Trucks in Transit HAP Speciation Profiles and Total Emission Estimates

| Pollutant | Pollutant Code | Emission Factor | Reference | National Emissions (tpy) |
| --- | --- | --- | --- | --- |
| VOC | VOC | 0.06 lb./1,000 gallons | 3 | 4.51E+03 |
| 2,2,4-Trimethylpentane | 540841 | 0.75% of VOC | 7 | 3.38E+01 |
| Cumene | 98828 | 0.012% of VOC | 7 | 5.41E-01 |
| Ethyl Benzene | 100414 | 0.053% of VOC | 7 | 2.39E+00 |
| n-Hexane | 110543 | 1.8% of VOC | 7 | 8.11E+01 |
| Naphthalene | 91203 | 0.00027% of VOC | 7 | 1.22E-02 |
| Toluene | 108883 | 1.4% of VOC | 7 | 6.31E+01 |
| Xylenes | 1330207 | 0.56% of VOC | 7 | 2.52E+01 |
| Benzene | 71432 | county-specific % of VOC | 12 | 3.13E+01 |

It is important to reiterate that the above discussion addresses the calculation of total VOC emissions. The 2008 point source NEI reports VOC emissions related to tank trucks in transit. To obtain nonpoint emissions, States should subtract the 2008 point source VOC emission estimates from the total VOC emission estimates reported here. The relevant point source SCCs are listed in Table 11 below.

Table 11. Tank Trucks in Transit Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40400154 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Terminals | Tank Truck Vapor Leaks |
| 40400254 | Petroleum and Solvent Evaporation | Petroleum Liquids Storage (non-Refinery) | Bulk Plants | Tank Truck Vapor Losses |
| 40600162 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Loaded with Fuel (Transit Losses) |
| 40600163 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Tank Cars and Trucks | Gasoline: Return with Vapor (Transit Losses) |

e. Underground Storage Tank (UST) Breathing and Emptying

The EPA calculated VOC emissions from UST breathing and emptying by multiplying county-level total gasoline consumption, calculated as described above in the Tank Trucks in Transit section, by the 1 lb/1,000 gallons emission factor recommended by the Gasoline Distribution EIIP guidance document.3  With the exception of benzene, HAP emissions were estimated by multiplying VOC emissions by the national HAP speciation profiles listed in Table 12.  To estimate benzene emissions, EPA multiplied VOC emissions by county-level speciation profiles from NMIM.12

Table 12.  Underground Storage Tank (UST) Breathing and Emptying Emissions

| Pollutant | Pollutant Code | Emission Factor | Reference | National Emissions (tpy) |
| --- | --- | --- | --- | --- |
| VOC | VOC | 1 lb./1,000 gallons | 3 | 6.89E+04 |
| 2,2,4-Trimethylpentane | 540841 | 0.75% of VOC | 7 | 5.17E+02 |
| Cumene | 98828 | 0.012% of VOC | 7 | 8.27E+00 |
| Ethyl Benzene | 100414 | 0.053% of VOC | 7 | 3.65E+01 |
| n-Hexane | 110543 | 1.8% of VOC | 7 | 1.24E+03 |
| Naphthalene | 91203 | 0.00027% of VOC | 7 | 1.86E-01 |
| Toluene | 108883 | 1.4% of VOC | 7 | 9.65E+02 |
| Xylenes | 1330207 | 0.56% of VOC | 7 | 3.86E+02 |
| Benzene | 71432 | county-specific % of VOC | 12 | 4.78E+02 |

It is important to reiterate that the above discussion addresses the calculation of total VOC emissions. The 2008 point source NEI reports VOC emissions related to UST breathing and emptying. To obtain nonpoint emissions, States should subtract the 2008 point source VOC emission estimates from the total VOC emission estimates reported here. The relevant point source SCCs are listed in Table 13 below.

Table 13. UST Breathing and Emptying Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40600307 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Gasoline Retail Operations - Stage I | Underground Tank Breathing and Emptying |
| 40600707 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage I | Underground Tank Breathing and Emptying |

f. Gasoline Service Station Unloading

The EPA estimated uncontrolled VOC emissions from unloading of gasoline into service station tanks from county-level total gasoline consumption estimates, calculated as described above in the Tank Trucks in Transit section, and the following AP-42 equation:

L = (12.46 x S x P x M)/T

where:

L	=	uncontrolled loading loss of liquid loaded (in lb/1,000 gallons)

S	=	saturation factor;

P	=	true vapor pressure of liquid loaded (pounds per square inch absolute);

M	=	molecular weight of vapors (lbs per lb/mole); and

T	=	temperature of liquid loaded (Rankine).14

This equation requires geographic-specific information.  This information includes the saturation factor, which differs by method of loading (e.g., submerged filling), Reid vapor pressure (RVP), temperature, and true vapor pressure of gasoline.

Gasoline RVP values were obtained from the NMIM 2008 database. Because NMIM is a county-level database that reports RVP values by month, EPA developed county-level monthly gasoline consumption estimates by multiplying annual county gasoline consumption by monthly allocation factors.  State-level monthly allocation factors were developed from monthly gasoline sales data reported in the Federal Highway Administration's Highway Statistics 2008.15  Geographic-specific information on the temperature of gasoline and the method of loading were obtained from a Stage I and II gasoline emission inventory study prepared for the EIIP.16

The true vapor pressure of gasoline was estimated for each county/month using the following equation:

P=exp0.7553−413.0T+459.6S0.5log10(RVP)−1.854−1,042T+459.6S0.5

where:

P		=	Stock true vapor pressure, in pounds per square inch absolute.

T		=	Stock temperature, in degrees Fahrenheit.

RVP	=	Reid vapor pressure, in pounds per square inch.

S		=	Slope of the ASTM distillation curve at 10 percent evaporated, in degrees Fahrenheit per percent (assumed that S = 3.0 for gasoline per Figure 7.1-14a of AP-42).17

This equation was used to calculate monthly county-level true vapor pressure estimates.  In cases where more than one filling method was assumed to apply in a county (e.g., due to vapor balancing requirement applying to a portion of a county’s total gasoline throughput due to a throughput exemption), EPA developed two sets of calculations for each month, one for each filling method.

The EIIP study regional stock temperature information was used to estimate the temperature of gasoline in each county in each month (see Table 14).16

Table 14. Temperature Data Used in Estimating True Vapor Pressure (ºF)

| Region | Jan | Feb | Mar | Apr | May | June | July | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (Northeast) | 46 | 44 | 44 | 48 | 57 | 64 | 70 | 73 | 70 | 64 | 60 | 51 |
| 2 (Southeast) | 66 | 67 | 69 | 74 | 78 | 81 | 80 | 81 | 80 | 77 | 69 | 60 |
| 3 (Southwest) | 60 | 61 | 62 | 66 | 73 | 78 | 81 | 84 | 82 | 78 | 71 | 62 |
| 4 (Midwest) | 33 | 35 | 40 | 47 | 55 | 62 | 71 | 73 | 68 | 65 | 64 | 63 |
| 5 (West) | 50 | 52 | 62 | 66 | 73 | 76 | 80 | 83 | 86 | 84 | 73 | 60 |
| 6 (Northwest) | 49 | 50 | 50 | 52 | 57 | 62 | 67 | 72 | 68 | 60 | 49 | 42 |

Region 1:	Alaska, Connecticut, Delaware, DC, Illinois, Indiana, Kentucky, Maine, Maryland, Massachusetts, Michigan, New Hampshire, New Jersey, New York, Ohio, Pennsylvania, Rhode Island, Vermont, Virginia, West Virginia, Wisconsin

Region 2:	Alabama, Arkansas, Florida, Georgia, Hawaii, Louisiana, Mississippi, N. Carolina, S. Carolina, Tennessee

Region 3:	Arizona, New Mexico, Oklahoma, Texas

Region 4:	Colorado, Iowa, Kansas, Minnesota, Missouri, Montana, Nebraska, N. Dakota, S. Dakota, Wyoming

Region 5:	California, Nevada, Utah

Region 6:	Idaho, Oregon, Washington

The EPA incorporated the effect of Stage I Gasoline Service Station vapor balancing controls based on the county-level control efficiency values (either 90 or 95 percent) that were compiled for the EIIP study.16 Table 15 presents the HAP speciation profiles and total VOC and HAP emission estimates calculated using these procedures.

Emissions are reported by SCC based on the filling methods used in each county as determined from the EIIP study: SCC 2501060051 (Submerged Filling); SCC 2501060052 (Splash Filling); and SCC 2501060053 (Balanced Submerged Filling).

Table 15.  Stage I Service Station Unloading HAP Speciation Profiles and Total Emission Estimates

| Pollutant | Pollutant Code | Emission Factor | Reference | National Emissions (tpy) |
| --- | --- | --- | --- | --- |
| VOC | VOC | Equation 1 | 14 | 3.82E+05 |
| 2,2,4-Trimethylpentane | 540841 | 0.75% of VOC | 7 | 2.86E+03 |
| Cumene | 98828 | 0.012% of VOC | 7 | 4.58E+01 |
| Ethyl Benzene | 100414 | 0.053% of VOC | 7 | 2.02E+02 |
| n-Hexane | 110543 | 1.8% of VOC | 7 | 6.87E+03 |
| Naphthalene | 91203 | 0.00027% of VOC | 7 | 1.03E+00 |
| Toluene | 108883 | 1.4% of VOC | 7 | 5.35E+03 |
| Xylenes | 1330207 | 0.56% of VOC | 7 | 2.14E+03 |
| Benzene | 71432 | county-specific % of VOC | 12 | 2.97E+03 |

It is important to reiterate that the above discussion addresses the calculation of total VOC emissions. The 2008 point source NEI reports VOC emissions related to service station unloading. To obtain nonpoint emissions, States should subtract the 2008 point source VOC emission estimates from the total VOC emission estimates reported here. The relevant point source SCCs are listed in Tables 16, 17, and 18 below.

Table 16. Service Station Unloading: Submerged Fill Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40600302 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Gasoline Retail Operations - Stage I | Submerged Filling w/o Controls |
| 40600702 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage I | Submerged Filling w/o Controls |

Table 17. Service Station Unloading: Splash Fill Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40600301 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Gasoline Retail Operations - Stage I | Splash Filling |
| 40600701 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage I | Splash Filling |

Table 18. Service Station Unloading: Balanced Submerged Fill Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40600305 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Gasoline Retail Operations - Stage I | Unloading ** |
| 40600306 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Gasoline Retail Operations - Stage I | Balanced Submerged Filling |
| 40600706 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage I | Balanced Submerged Filling |

Unloading emissions might also be reported in the point source inventory under SCC 40600399 (Gasoline Retail Operations – Stage I, Not Classified).

g. Example Emission Calculations

Bulk Terminals

2008 national benzene emissions =	VOC emissions x HAP speciation factor

1.65E+05 tons x 0.0027

4.46E+02 tons

Pipelines

2008 national cumene emissions =	VOC emissions x HAP speciation factor

9.58E+04 tons x 0.00012

1.15E+01 tons

Bulk Plants

2008 national VOC emissions

=	national gasoline consumption x proportion passing through bulk plants x VOC emission factor

=	137,801,370 thousand gallons x 0.09 x 8.62 lbs. VOC/thousand gallons

=	1.07E+08 lbs. / 2000 lbs.

=	5.35E+04 tons

Tank Trucks in Transit

2008 Alamance County, North Carolina VOC emissions

=	total county gasoline consumption x (1+proportion of gasoline transported twice) x VOC emission factor

=	61,446 thousand gallons x (1+0.09) x 0.06 lbs. VOC/thousand gallons

=	4.02E+03 lbs. / 2000 lbs.

=	2.01E+00 tons

UST Breathing and Emptying

2008 Alamance County, North Carolina VOC emissions

=	total county gasoline consumption x VOC emission factor

=	61,466 thousand gallons x 1 lb. VOC/thousand gallons

=	6.15E+04 lbs. / 2000 lbs.

=	30.73E+00 tons

Stage I Gasoline Service Station Unloading - uncontrolled VOC emissions in July for balanced submerged fill unloading in Alamance County, NC

=	annual county consumption x proportion of annual gasoline sold in July x VOC emission factor

=	61,466 thousand gallons x 0.1087 x VOC emission factor

=	6,681 thousand gallons x ((12.46 x saturation factor x true vapor pressure x vapor molecular weight) / temperature))

=	6,681 thousand gallons x ((12.46 x 1.0 x 6.309 x 67.811) / 540)

=	65,950 lbs

Incorporate effect of control (vapor balancing requirement)

=	Uncontrolled emissions x ((100-CE)/100)

=	65,950 lbs x ((100-90)/100)

=	6,595 lbs / 2,000 lbs

=	3.30E+00 tons

h. References

1.	U.S. Environmental Protection Agency, “National Emission Standards for Source Categories:  Gasoline Distribution (Stage I), 40 CFR Part 63.”  Office of Air Quality Planning and Standards, February 28, 1997.  Pages 9087-9093.

2.	U.S. Environmental Protection Agency, "Gasoline Distribution Industry (Stage I)-Background Information for Proposed Standards," EPA-453/R94-002a, Office of Air Quality Planning and Standards, January 1994.

3.	Eastern Research Group, Inc., "Volume III: Chapter 11, Gasoline Marketing (Stage I and Stage II), Revised Final," prepared for the Emission Inventory Improvement Program, January 2001.

4.	U.S. Environmental Protection Agency, “TANKS Emission Estimation Software,” Office of Air Quality Planning and Standards, Emission Inventory Group, available from http://www.epa.gov/ttn/chief/software/tanks/index.html, last updated October 29, 2004.

5.	U.S. Environmental Protection Agency, "Gasoline Distribution Industry (Stage I)-Background Information for Promulgated Standards," EPA-453/R94-002b, Office of Air Quality Planning and Standards, November 1994.

6.	U.S. Department of Energy, Energy Information Administration, “U.S. Daily Average Supply and Distribution of Crude Oil and Petroleum Products,” Table 2 in Petroleum Supply Annual 2008, Volume 1, retrieved from http://www.eia.doe.gov/oil_gas/petroleum/data_publications/petroleum_supply_annual/psa_volume1/psa_volume1.html, released June 2009.

7.	Hester, Charles, MACTEC, Inc. Memorandum from Charles Hester, MACTEC, Inc., to Stephen Shedd, U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards, Emission Standards Division, "Review of Data on HAP Content in Gasoline," May 18, 2006.

8.	U.S. Department of Commerce, Bureau of the Census, County Business Patterns 2007, retrieved from http://www.census.gov/econ/cbp/index.html, released July 2009.

9.	U.S. Department of Energy, Energy Information Administration, "Refinery, Bulk Terminal, and Natural Gas Plant Stocks of Selected Petroleum Products by PAD District and State, 2008” and "Movements of Crude Oil and Petroleum Products by Pipeline Between PAD Districts, 2008," Tables 33 and 35 in Petroleum Supply Annual 2008, Volume 1, retrieved from http://www.eia.doe.gov/oil_gas/petroleum/data_publications/petroleum_supply_annual/psa_volume1/psa_volume1.html, released June 2009.

10.	Cavalier, Julia, MACTEC, Inc., personal communication, "RE: Percentage of Gasoline Transported Twice By Truck," with Stephen Shedd, U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards, Emission Standards Division, July 6, 2004.

11.	U.S. Department of Energy, Energy Information Administration, Petroleum Navigator – Product Supplied, available from http://tonto.eia.doe.gov/dnav/pet/pet_cons_psup_dc_nus_mbblpd_a.htm, accessed January 2010.

12.	Benzene speciation profiles calculated by Jonathan Dorn, E.H. Pechan and Associates, Inc. from county-level VOC and benzene emissions developed from a 2008 NMIM run. The NMIM run was performed by John Van Bruggen, E.H. Pechan and Associates, Inc., January 2010.

13.	2008 NMIM runs performed by John Van Bruggen and Melissa Spivey, E.H. Pechan and Associates, Inc., January 2010.

14.	U.S. Environmental Protection Agency, "Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, Volume I:  Stationary Point and Area Sources, Section 5.2 Transportation and Marketing of Petroleum Liquids," Office of Air Quality Planning and Standards, January 1995.

15.	Federal Highway Administration, “Monthly Gasoline/Gasohol Reported by States,” Table MF-33GA in Highway Statistics 2008, Office of Highway Policy Information, available from http://www.fhwa.dot.gov/policyinformation/statistics/2008/, accessed January 2010.

16.	Pacific Environmental Services, Inc., "Draft Summary of the Analysis of the Emissions Reported in the 1999 NEI for Stage I and Stage II Operations at Gasoline Service Stations," prepared for the U.S. Environmental Protection Agency and the Emission Inventory Improvement Program, September 2002.

17.	U.S. Environmental Protection Agency, “Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, Volume I:  Stationary Point and Area Sources, Chapter 7: Liquid Storage Tanks,” Office of Air Quality Planning and Standards, Emission Inventory Group, September 1997.