## 1.	FUEL COMBUSTION – INDUSTRIAL

### a.	Source Category Description

This subsector covers emissions from fuel combustion at stationary nonpoint sources within the industrial sector (i.e., sources that are not reported in the point source inventory because their emissions are too small).  Industrial fuel combustion emissions will be computed for the following fuel types:  coal, distillate oil, residual oil, natural gas, liquefied petroleum gas (LPG), kerosene, and wood.

There are additional types of energy that are consumed in the industrial sector:  asphalt and road oil; feedstocks, naphtha (less than 401°F); feedstocks, other oils (greater than 401°F); lubricants; motor gasoline; miscellaneous petroleum products; pentanes plus; special naphthas; and waxes.  With the exception of motor gasoline, all of these additional fossil fuels are not actually combusted (oxidized) but are used as chemical feedstocks, construction materials, lubricants, solvents, or reducing agents.  Therefore, there are no industrial sector combustion emissions from these fuel types.  As described in more detail later, most of the fuel types that are included in the industrial combustion sector, also have a non-fuel use component.  Therefore, it is necessary to exclude this component in calculating nonpoint source industrial fuel combustion activity/emissions.  Motor gasoline is not inventoried as a nonpoint source because it is expected that gasoline combustion in this sector is included in the nonroad inventory.

The approach to be used in calculating emissions for industrial fuel combustion is to first develop state-level fuel consumption estimates, then to allocate these to the county-level, and then to multiply the resulting county-level consumption estimates by appropriate emission factors.

Source Classification.  Table 1 displays the nonpoint SCCs associated with industrial fuel combustion.

### b.	Activity Data

Table 2 summarizes this sector’s nonpoint source activity data, units, and data source.  Total state-level industrial sector energy consumption data are available from the Energy Information Administration (EIA)’s State Energy Data System (SEDS), and were used for most source categories (EIA, 2008a).  However, as documented below, a number of adjustments were made to the SEDS data, and a separate EIA data source was used for distillate oil.  Year 2006 SEDS data were used to estimate 2008 emissions because these were the latest year consumption data available at the time this work was performed in 2009.

Table 1.  Nonpoint Source Industrial Fuel Combustion SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2102001000 | Stationary Source Fuel Combustion | Industrial | Anthracite Coal | Total: All Boiler Types |
| 2102002000 | Stationary Source Fuel Combustion | Industrial | Bituminous/Subbituminous Coal | Total: All Boiler Types |
| 2102004000 | Stationary Source Fuel Combustion | Industrial | Distillate Oil | Total: Boilers and IC Engines |
| 2102005000 | Stationary Source Fuel Combustion | Industrial | Residual Oil | Total: All Boiler Types |
| 2102006000 | Stationary Source Fuel Combustion | Industrial | Natural Gas | Total: Boilers and IC Engines |
| 2102007000 | Stationary Source Fuel Combustion | Industrial | Liquefied Petroleum Gas (LPG) | Total: All Boiler Types |
| 2102008000 | Stationary Source Fuel Combustion | Industrial | Wood | Total: All Boiler Types |
| 2102011000 | Stationary Source Fuel Combustion | Industrial | Kerosene | Total: All Boiler Types |

Table 2.  Nonpoint Source Industrial Fuel Combustion Activity Data

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 | Activity | Activity Data Source |
| --- | --- | --- | --- | --- | --- | --- |
| 2102001000 | Stationary Source Fuel Combustion | Industrial | Anthracite Coal | Total: All Boiler Types | Thousand short tons of coal consumed | EIA’s SEDSa |
| 2102002000 | Stationary Source Fuel Combustion | Industrial | Bituminous/ Subbituminous Coal | Total: All Boiler Types | Thousand short tons of coal consumed | EIA’s SEDSa |
| 2102004000 | Stationary Source Fuel Combustion | Industrial | Distillate Oil | Total: Boilers and IC Engines | Thousand barrels of distillate oil consumed | EIA’s Fuel Oil and Kerosene Salesa |
| 2102005000 | Stationary Source Fuel Combustion | Industrial | Residual Oil | Total: All Boiler Types | Thousand barrels of residual oil consumed | EIA’s SEDSa |
| 2102006000 | Stationary Source Fuel Combustion | Industrial | Natural Gas | Total: Boilers and IC Engines | Million cubic feet of natural gas consumed | EIA’s SEDSa |
| 2102007000 | Stationary Source Fuel Combustion | Industrial | Liquefied Petroleum Gas (LPG) | Total: All Boiler Types | Thousand barrels of LPG consumed | EIA’s SEDSa |
| 2102008000 | Stationary Source Fuel Combustion | Industrial | Wood | Total: All Boiler Types | Billion Btu of wood consumed | EIA’s SEDSa |
| 2102011000 | Stationary Source Fuel Combustion | Industrial | Kerosene | Total: All Boiler Types | Thousand barrels of kerosene consumed | EIA’s SEDSa |

a With adjustments that are described in the text of this section.

Fuel-Specific Activity Data  Adjustments

Coal - for industrial coal combustion, it was necessary to compile data representing a sub-set of total sector coal consumption.  Data representing non-coke plant consumption were compiled from EIA because coal consumed by coke plants is accounted for in the point source inventory.  The SEDS data do not provide coal consumption estimates by type of coal (i.e., anthracite versus bituminous/subbituminous).  Therefore, EPA used state-level industrial coal distribution data for 2006 from the EIA table “Domestic and Foreign Distribution of U.S. Coal by State of Origin, 2006,” to allocate coal consumption between the two types of coal.  The 2006 ratio of anthracite (and bituminous) coal consumption to total coal consumption was used for this allocation procedure.  Table 3 presents the 2006 anthracite and bituminous coal ratios for each state.

Distillate Oil and LPG – the SEDS industrial sector distillate oil and LPG consumption data include consumption estimates for equipment that EPA includes in the nonroad sector inventory.  In particular, SEDS considers the following nonroad source category activities to be part of the industrial sector:  farming, logging, mining, and construction.  To avoid double-counting of LPG consumption, EPA ran the National Mobile Inventory Model (NMIM) for 2006 and calculated the national volume of nonroad LPG consumption from agriculture, logging, mining, and construction source categories.  This estimate was then divided into the SEDS total industrial sector LPG consumption estimate to yield the proportion of total industrial sector LPG consumption attributable to the nonroad sector in that year (approximately 9 percent).  To estimate the volume of industrial sector LPG consumption that should not be included in the nonpoint source industrial fuel combustion category, EPA subtracted 9 percent from each state’s industrial sector LPG consumption estimate reported in SEDS.

To avoid double-counting of distillate oil consumption between the nonpoint and nonroad sector emission inventories, EPA relied on a source other than SEDS to estimate consumption.  The approach uses more detailed distillate oil consumption estimates reported in EIA’s Fuel Oil and Kerosene Sales, and assumptions used in the regulatory impact analysis (RIA) for EPA’s nonroad diesel emissions rulemaking (EIA, 2008b; EPA, 2003).  Table 4 displays the assumptions that were applied to the state-level distillate oil consumption estimates reported in Fuel Oil and Kerosene Sales to estimate total stationary source industrial sector consumption.

Non-Fuel Use Energy Adjustment

Some industrial sector energy is consumed for non-fuel purposes.  For example, natural gas is used as a feedstock in chemical manufacturing plants and to make nitrogenous fertilizer, and LPG is used to create intermediate products that are made into plastics.  To estimate the volume of fuel that is associated with industrial combustion, it is necessary to subtract the volume of fuel consumption for non-energy uses from the volume of total fuel consumption.  The EPA’s State Greenhouse Gas Inventory Tool (SIT) provides national defaults representing the percentage of total industrial fuel consumption from non-energy uses.  These default values have an additional limitation beyond their lack of geographic detail - they represent the EIA’s definition of the

Table 3.  Proportion of 2006 Industrial Sector Coal Consumption by Type of Coal

| FIPS | State | % Anthracite | % Bituminous/ Subbituminous |
| --- | --- | --- | --- |
| 01 | Alabama | 0.0% | 100.0% |
| 02 | Alaska | 0.0% | 100.0% |
| 04 | Arizona | 0.0% | 100.0% |
| 05 | Arkansas | 0.0% | 100.0% |
| 06 | California | 0.0% | 100.0% |
| 08 | Colorado | 0.0% | 100.0% |
| 09 | Connecticut | 0.0% | 100.0% |
| 10 | Delaware | 0.3% | 99.7% |
| 11 | District of Columbia | 0.0% | 100.0% |
| 12 | Florida | 0.0% | 100.0% |
| 13 | Georgia | 0.0% | 100.0% |
| 15 | Hawaii | 0.0% | 100.0% |
| 16 | Idaho | 0.0% | 100.0% |
| 17 | Illinois | 0.0% | 100.0% |
| 18 | Indiana | 0.0% | 100.0% |
| 19 | Iowa | 0.0% | 100.0% |
| 20 | Kansas | 0.0% | 100.0% |
| 21 | Kentucky | 0.0% | 100.0% |
| 22 | Louisiana | 0.0% | 100.0% |
| 23 | Maine | 0.0% | 100.0% |
| 24 | Maryland | 0.0% | 100.0% |
| 25 | Massachusetts | 0.0% | 100.0% |
| 26 | Michigan | 0.0% | 100.0% |
| 27 | Minnesota | 0.1% | 99.9% |
| 28 | Mississippi | 0.0% | 100.0% |
| 29 | Missouri | 0.0% | 100.0% |
| 30 | Montana | 0.0% | 100.0% |
| 31 | Nebraska | 0.0% | 100.0% |
| 32 | Nevada | 0.0% | 100.0% |
| 33 | New Hampshire | 0.0% | 100.0% |
| 34 | New Jersey | 100.0% | 0.0% |
| 35 | New Mexico | 0.0% | 100.0% |
| 36 | New York | 0.0% | 100.0% |
| 37 | North Carolina | 0.0% | 100.0% |
| 38 | North Dakota | 0.0% | 100.0% |
| 39 | Ohio | 0.0% | 100.0% |
| 40 | Oklahoma | 0.0% | 100.0% |
| 41 | Oregon | 0.0% | 100.0% |
| 42 | Pennsylvania | 14.6% | 85.4% |
| 44 | Rhode Island | 0.0% | 100.0% |
| 45 | South Carolina | 0.0% | 100.0% |
| 46 | South Dakota | 0.0% | 100.0% |
| 47 | Tennessee | 0.0% | 100.0% |
| 48 | Texas | 0.0% | 100.0% |
| 49 | Utah | 0.0% | 100.0% |
| 50 | Vermont | 0.0% | 100.0% |
| 51 | Virginia | 0.0% | 100.0% |
| 53 | Washington | 0.0% | 100.0% |
| 54 | West Virginia | 0.0% | 100.0% |
| 55 | Wisconsin | 0.0% | 100.0% |
| 56 | Wyoming | 0.1% | 99.9% |

Table 4.  Assumptions Used to Estimate Industrial Sector Stationary Source Distillate Fuel Consumption

| Sector | Distillate Fuel Type | % of Total Consumption from Stationary Sources |
| --- | --- | --- |
| Industrial | No. 1 Distillate Fuel Oil | 60 |
|  | No. 2 Distillate Fuel Oil | 100 |
|  | No. 2 Distillate/Low and High Sulfur Diesel | 15a |
|  | No. 4 Distillate Fuel Oil | 100 |
| Farm | Diesel | 0 |
|  | Other Distillate Fuel Oil | 100 |
| Off-Highway (Construction and Other) | Distillate Fuel Oil | 5 |

a This value differs from the 0% assumption adopted in EPA’s nonroad diesel emissions rulemaking because it is known that some diesel fuel is used by stationary sources (a 15 percent value was selected for use as an approximate mid-point of a potential range of 8 to 24% stationary source use computed from a review of data from the EIA’s Manufacturing Energy Consumption Survey and Fuel Oil and Kerosene Sales).

industrial sector, which includes fuel consumption from industrial, farm, mining, and construction sectors that is accounted for in the nonroad inventory.  Furthermore, a review of the SIT default data indicates that all residual oil is consumed as a fuel source.  However, the EIA’s 2002 Manufacturing Energy Consumption Survey (MECS) indicates significant non-fuel use of residual oil in the manufacturing sector (EIA, 2007a).  In discussions about this issue, EIA noted that the non-fuel use data in the SIT appeared to be incorrect, and that the MECS data were representative of 2002 non-fuel use (Lorenz, 2009).  Given this concern, a review was conducted of the non-fuel use assumptions incorporated into the EIA’s GHG emissions inventory for 2005 (EIA, 2007b).  For each type of industrial sector energy consumed in 2006, Table 5 summarizes EIA’s non-fuel use assumptions.

Table 5.  Non-Fuel Use Information from EIA’s 2005 GHG Inventory

| Fuel | Non-Fuel Use Assumption |
| --- | --- |
| Asphalt and Road Oil | 100% |
| Coal (non-coke) | Yes (<100%) |
| Distillate Oil | Yes (<100%) |
| Feedstocks, Naphtha <401°F | 100% |
| Feedstocks, Other oils >401°F | 100% |
| Kerosene | None |
| LPG | Yes (<100%) |
| Lubricants | 100% |
| Misc. Petroleum Products | 100% |
| Motor Gasoline | None |
| Natural Gas | Yes (<100%) |
| Pentanes Plus | 100% |
| Residual Oil | Yes (<100%) |
| Special Naphthas | 100% |
| Waxes | 100% |

In calculating the emission activity for industrial fuel combustion, EPA excluded all SEDS fuel types for which EIA assumes 100 percent of consumption is non-fuel use.  For fuel types for which non-fuel use occurs, but is less than 100 percent, Pechan reviewed two information sources to identify the non-fuel use percentage to apply in the NEI.  The first source was that listed by EIA in its 2005 national GHG inventory; the second was the EIA’s 2002 MECS.  The MECS data has the advantage of providing region-specific estimates as well as values that exclude fuel consumption that is already accounted for in the nonroad inventory. For distillate and residual fuel and natural gas, the EIA GHG inventory source was MECS.  For LPG, it was not possible to use the EIA GHG inventory source for non-fuel consumption as it is a proprietary source published by the American Petroleum Institute.  For non-coke coal, EIA extrapolated data from 1994.  The MECS non-fuel consumption data treat coal that is used to produce coke as a feedstock, and therefore, MECS can not be used to represent non-coke coal consumption.  Because of the limitations of the MECS data, EPA obtained from EIA a rough estimate of the percentage of non-coking coal that is used for non-fuel purposes (while also treating “synthetic coal” as a fuel use).  Based on the EIA’s estimate that 5-10 percent of non-coke coal is used for non-fuel purposes, EPA assumed that 7 percent of non-coke plant industrial sector coal consumption is for non-fuel purposes (Lorenz, 2009).  Table 6 presents the non-fuel use percentages that were applied to estimate industrial fuel combustion activity.

Table 6.  Industrial Sector Non-Fuel Use Estimates

| Fuel | % Non-Fuel Use by Region |  |  |  | Source |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Northeast | South | Midwest | West |  |  |
| Distillate Oil | 5a | 9b | 9b | 5a | 2002 MECS |  |
| LPG | 41a | 98 | 88 | 41a | 2002 MECS |  |
| Natural Gas | 1 | 15 | 6 | 41 | 2002 MECS |  |
| Non-Coke Coal | 7c | 7c | 7c | 7c | Lorenz, 2009 |  |
| Residual Oil | 0 | 25b | 25b | 8 | 2002 MECS |  |
| a Due to withheld data, value represents combination of Northeast and West region data. b Due to withheld data, value represents combination of Midwest and South region data. c Region-specific values not available. |  |  |  |  |  |  |

Point Source Energy Adjustment

Because the point source inventory also includes industrial fuel combustion categories, it is necessary to subtract point source inventory fuel use from the SEDS fuel consumption estimates.  Table 7 displays a crosswalk between industrial fuel combustion nonpoint SCCs and associated point SCCs that EPA plans to use in the subtraction procedure.  The subtractions will be performed after summing the point source throughput estimates to the state-level.

### c.	County Allocation of State Activity Data

Year 2005 county-level activity estimates will be developed by allocating the state-level activity resulting from the adjustments to the EIA data described above.  The EPA compiled 2006 estimates of manufacturing sector employment from the Bureau of Census’ County Business Patterns 2006 for use in this procedure (Census, 2009).  State-level industrial fuel combustion by fuel type will be allocated to each county using the ratio of the number of manufacturing sector (NAICS codes 31-33) employees in each county to the total number of manufacturing sector employees in the state.  A separate document describes how withheld County Business Patterns employment data were estimated (Divita, 2008).

Table 7.  Point Source Subtraction SCCs Relevant to Nonpoint Source Industrial Fuel Combustion

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 (Fuel-if not specified at level 3) |
| --- | --- | --- | --- |
| 2102002000 – Industrial; Bituminous/Subbituminous Coal; Total: All Boiler Types |  |  |  |
| 102002xx | External Combustion Boilers | Industrial | Bituminous/Subbituminous Coal |
| 10500102 | External Combustion Boilers | Space Heaters | Industrial (Coal) |
| 390002xx | Industrial Processes | In-Process Fuel Use | Bituminous Coal |
| 50390002 | Waste Disposal | Solid Waste Disposal – Industrial | Auxiliary Fuel/No Emissions (Coal) |
| 2102004000 – Industrial; Distillate Oil; Total: All Boilers and IC Engines |  |  |  |
| 102005xx | External Combustion Boilers | Industrial | Distillate Oil |
| 10201403 | External Combustion Boilers | Industrial | CO Boiler (Distillate Oil) |
| 10500105 | External Combustion Boilers | Space Heaters | Industrial (Distillate Oil) |
| 202001xx | Internal Combustion Engines | Industrial | Distillate Oil (Diesel) |
| 20200401 | Internal Combustion Engines | Industrial | Large Bore Engine (Diesel) |
| 20200405 | Internal Combustion Engines | Industrial | Crankcase Blowby (Diesel) |
| 20200406 | Internal Combustion Engines | Industrial | Evaporative Losses (Fuel Storage and Delivery System) (Diesel) |
| 20200407 | Internal Combustion Engines | Industrial | Exhaust (Diesel) |
| 27000320 | Internal Combustion Engines | Off-Highway Diesel Engines | Industrial Equipment |
| 30190001 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Distillate Oil) |
| 30190011 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Distillate Oil) |
| 30190021 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Distillate Oil) |
| 30290001 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment (Distillate Oil) |
| 30390001 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30390011 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30390021 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30400406 | Industrial Processes | Secondary Metal Production | Lead (Distillate Oil) |
| 30490001 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30490011 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30490021 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30490031 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Distillate Oil) |
| 30500208 | Industrial Processes | Mineral Products | Asphalt Concrete (Distillate Oil) |
| 30505022 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) (Distillate Oil) |
| 30590001 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Distillate Oil) |
| 30590011 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Distillate Oil) |
| 30590021 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Distillate Oil) |
| 30600901 | Industrial Processes | Petroleum Industry | Flares (Distillate Oil) |
| 30609901 | Industrial Processes | Petroleum Industry | Incinerators (Distillate Oil) |
| 30790001 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Distillate Oil) |
| 30790011 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Distillate Oil) |
| 30790021 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Distillate Oil) |
| 30890001 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Distillate Oil) |
| 30890011 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Distillate Oil) |
| 30890021 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Distillate Oil) |
| 30990001 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Distillate Oil) |
| 30990011 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Distillate Oil) |
| 31000401 | Industrial Processes | Oil and Gas Production | Process Heaters (Distillate Oil) |
| 31000411 | Industrial Processes | Oil and Gas Production | Process Heaters (Distillate Oil) |
| 31390001 | Industrial Processes | Electrical Equipment | Process Heaters (Distillate Oil) |
| 390005xx | Industrial Processes | In-Process Fuel Use | Distillate Oil |
| 39900501 | Industrial Processes | Miscellaneous Manufacturing Industries | Process Heater/Furnace (Distillate Oil) |
| 39990001 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Distillate Oil) |
| 39990011 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Distillate Oil) |
| 39990021 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Distillate Oil) |
| 40201002 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater (Distillate Oil) |
| 40290011 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment (Distillate Oil) |
| 49090011 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment (Distillate Oil) |
| 49090021 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment (Distillate Oil) |
| 50390005 | Waste Disposal | Solid Waste Disposal – Industrial | Auxiliary Fuel/No Emissions (Distillate Oil) |
| 2102005000 – Industrial; Residual Oil; Total: All Boiler Types |  |  |  |
| 102004xx | External Combustion Boilers | Industrial | Residual Oil |
| 10201404 | External Combustion Boilers | Industrial | CO Boiler (Residual Oil) |
| 202005xx | Internal Combustion Boilers | Industrial | Residual/Crude Oil |
| 30190002 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Residual Oil) |
| 30190012 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Residual Oil) |
| 30190022 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Residual Oil) |
| 30290002 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment (Residual Oil) |
| 30390002 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30390012 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30390022 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30490002 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30490012 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30490022 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30490032 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Residual Oil) |
| 30500207 | Industrial Processes | Mineral Products | Asphalt Concrete (Residual Oil) |
| 30505021 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) (Residual Oil) |
| 30590002 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Residual Oil) |
| 30590012 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Residual Oil) |
| 30600111 | Industrial Processes | Petroleum Industry | Process Heaters (Residual Oil) |
| 30600902 | Industrial Processes | Petroleum Industry | Flares (Residual Oil) |
| 30609902 | Industrial Processes | Petroleum Industry | Incinerators (Residual Oil) |
| 30790002 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Residual Oil) |
| 30790012 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Residual Oil) |
| 30790022 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Residual Oil) |
| 30890002 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Residual Oil) |
| 30890012 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Residual Oil) |
| 30890022 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Residual Oil) |
| 30990002 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Residual Oil) |
| 30990012 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Residual Oil) |
| 31000402 | Industrial Processes | Oil and Gas Production | Process Heaters (Residual Oil) |
| 31000412 | Industrial Processes | Oil and Gas Production | Process Heaters (Residual Oil) |
| 31390002 | Industrial Processes | Electrical Equipment | Process Heaters (Residual Oil) |
| 390004xx | Industrial Processes | In-Process Fuel Use | Residual Oil |
| 39990002 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Residual Oil) |
| 39990012 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Residual Oil) |
| 39990022 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Residual Oil) |
| 40201003 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater (Residual Oil) |
| 40290012 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment (Residual Oil) |
| 49090012 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment (Residual Oil) |
| 49090022 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment (Residual Oil) |
| 2102006000 – Industrial; Natural Gas; Total: Boilers and IC Engines |  |  |  |
| 102006xx | External Combustion Boilers | Industrial | Natural Gas |
| 10201401 | External Combustion Boilers | Industrial | CO Boiler (Natural Gas) |
| 10500106 | External Combustion Boilers | Space Heaters | Commercial/Institutional (Natural Gas) |
| 202002xxa | Internal Combustion Engines | Industrial | Natural Gas |
| 30190003 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Natural Gas) |
| 30190013 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Natural Gas) |
| 30190023 | Industrial Processes | Chemical Manufacturing | Fuel Fired Equipment (Natural Gas) |
| 30290003 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment (Natural Gas) |
| 30291001 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment (Natural Gas) |
| 30390003 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30390013 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30390023 | Industrial Processes | Primary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30400407 | Industrial Processes | Secondary Metal Production | Lead (Natural Gas) |
| 30490003 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30490013 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30490023 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30490033 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Natural Gas) |
| 30500206 | Industrial Processes | Mineral Products | Asphalt Concrete (Natural Gas) |
| 30505020 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) (Natural Gas) |
| 30590003 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Natural Gas) |
| 30590013 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Natural Gas) |
| 30590023 | Industrial Processes | Mineral Products | Fuel Fired Equipment (Natural Gas) |
| 30600105 | Industrial Processes | Petroleum Industry | Process Heaters (Natural Gas) |
| 30600903 | Industrial Processes | Petroleum Industry | Flares (Natural Gas) |
| 30602401 | Industrial Processes | Petroleum Industry | Reciprocating Engine Compressors (Natural Gas) |
| 30609903 | Industrial Processes | Petroleum Industry | Incinerators (Natural Gas) |
| 30622003 | Industrial Processes | Petroleum Industry | Underground Storage and Other Remediation: Soil: Natural Gas |
| 30622403 | Industrial Processes | Petroleum Industry | Underground Storage and Other Remediation: Air Stripping: Natural Gas |
| 30790003 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Natural Gas) |
| 30790013 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Natural Gas) |
| 30790023 | Industrial Processes | Pulp and Paper and Wood Products | Fuel Fired Equipment (Natural Gas) |
| 30890003 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Natural Gas) |
| 30890013 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Natural Gas) |
| 30890023 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (Natural Gas) |
| 30990003 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Natural Gas) |
| 30990013 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Natural Gas) |
| 30990023 | Industrial Processes | Fabricated Metal Products | Fuel Fired Equipment (Natural Gas) |
| 31000404 | Industrial Processes | Oil and Gas Production | Process Heaters (Natural Gas) |
| 31000414 | Industrial Processes | Oil and Gas Production | Process Heaters (Natural Gas) |
| 31390003 | Industrial Processes | Electrical Equipment | Process Heaters (Natural Gas) |
| 390006xx | Industrial Processes | In-process Fuel Use | Natural Gas |
| 39900601 | Industrial Processes | Miscellaneous Manufacturing Industries | Process Heater/Furnace (Natural Gas) |
| 39990003 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Natural Gas) |
| 39990013 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Natural Gas) |
| 39990023 | Industrial Processes | Miscellaneous Manufacturing Industries | Miscellaneous Manufacturing Industries (Natural Gas) |
| 40201001 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater (Natural Gas) |
| 40290013 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment (Natural Gas) |
| 40290023 | Petroleum and Solvent Evaporation | Surface Coating Operations | Fuel Fired Equipment (Natural Gas) |
| 49090013 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment (Natural Gas) |
| 49090023 | Petroleum and Solvent Evaporation | Organic Solvent Evaporation | Fuel Fired Equipment (Natural Gas) |
| 50390006 | Waste Disposal | Solid Waste Disposal – Industrial | Auxiliary Fuel/No Emissions (Natural Gas) |
| 2102007000 – Industrial; LPG; Total: All Boiler Types |  |  |  |
| 102010xx | External Combustion Boilers | Industrial | Liquefied Petroleum Gas |
| 10500110 | External Combustion Boilers | Space Heaters | Industrial (LPG) |
| 202010xx | Internal Combustion Engines | Industrial | Liquefied Petroleum Gas |
| 27300320 | Internal Combustion Engines | Off-highway LPG-fueled Engines | Industrial Equipment |
| 30290005 | Industrial Processes | Food and Agriculture | Fuel Fired Equipment (LPG) |
| 30490035 | Industrial Processes | Secondary Metal Production | Fuel Fired Equipment (Propane) |
| 30500209 | Industrial Processes | Mineral Products | Asphalt Concrete (LPG) |
| 30505023 | Industrial Processes | Mineral Products | Asphalt Processing (Blowing) (LPG) |
| 30590005 | Industrial Processes | Mineral Products | Fuel Fired Equipment (LPG) |
| 30600107 | Industrial Processes | Petroleum Industry | Process Heaters (LPG) |
| 30600905 | Industrial Processes | Petroleum Industry | Flares (LPG) |
| 30609905 | Industrial Processes | Petroleum Industry | Incinerators (LPG) |
| 30890004 | Industrial Processes | Rubber and Miscellaneous Plastics Products | Fuel Fired Equipment (LPG) |
| 31000406 | Industrial Processes | Oil and Gas Production | Process Heaters (Propane/Butane) |
| 390010xx | Industrial Processes | In-process Fuel Use | Liquefied Petroleum Gas |
| 39901001 | Industrial Processes | Miscellaneous Manufacturing Industries | Process Heater/Furnace (LPG) |
| 40201004 | Petroleum and Solvent Evaporation | Surface Coating Operations | Coating Oven Heater (LPG) |
| 50390010 | Waste Disposal | Solid Waste Disposal - Industrial | Auxiliary Fuel/No Emissions (LPG) |
| 2102008000 – Industrial; Wood; Total: All Boiler Types |  |  |  |
| 102009xx | External Combustion Boilers | Industrial | Wood/Bark Waste |
| 390009xx | Industrial Processes | In-process Fuel Use | Wood |
| 2102011000 – Industrial; Kerosene; Total: All Boiler Types |  |  |  |
| 202009xx | Internal Combustion Engines | Industrial | Kerosene/Naphtha (Jet Fuel) |
| a Note that EIA’s SEDS includes natural gas used as pipeline fuel (e.g., for use in compressor engines) in the Transportation sector rather than the Industrial sector Therefore, such pipeline use should not be included in the point source subtraction procedure. |  |  |  |

### d.	Emission Factors

The EPA has compiled criteria and hazardous air pollutant emission factors for nonpoint source industrial fuel combustion categories (Huntley, 2009).  These emission factors, which are too numerous to list here, are included in a spreadsheet within the ICI fuel combustion workbook.  In most cases, these are the same emission factors that were used in preparing the 2002 nonpoint source NEI (Pechan, 2006).  Industrial LPG and wood combustion emission factors were obtained from an ICI fuel combustion study being performed for the Central Regional Air Planning Association (CENRAP) (Pechan, 2009a).

### e.	Potential Improvements

The following are potential improvements that states may wish to incorporate in developing nonpoint source industrial fuel combustion emission estimates:

- Investigate the availability of more geographic- and year-specific estimates of non-fuel use consumption of industrial energy (e.g., MECS is planning to release 2006 year data in the coming months);

- Consider implementing a county allocation procedure that Pechan developed for the Central Regional Air Planning Association (CENRAP) that reflects the energy-intensity of each industrial sector--current methods only reflect number of employees in each industry sector (Pechan, 2009a); and

- Update SO2 emission factors to reflect coal sulfur content estimates for coal used in the industrial sector in 2006--a recent review of industrial coal sulfur content values for CENRAP concluded that the 2002 NEI values are based on unrepresentative data (Pechan, 2009b).

### f.	References

Census, 2009:  Bureau of the Census, U.S. Department of Commerce, County Business Patterns 2006, Washington, DC, available from: http://www.census.gov/epcd/cbp/download/06_data/index.html accessed March 2009.

Divita, 2008:  Divita, Frank, E.H. Pechan & Associates, Inc., memorandum to Roy Huntley, U.S. Environmental Protection Agency, “County Business Patterns Calculations,” December 4, 2008.

EIA, 2008a:  Energy Information Administration, U.S. Department of Energy, State Energy Data System – Consumption, Physical Units, 1960-2006, available from http://www.eia.doe.gov/emeu/states/_seds.html, released November 28, 2008.

EIA, 2008b:  Energy Information Administration, U.S. Department of Energy, Fuel Oil and Kerosene Sales, data available from http://tonto.eia.doe.gov/dnav/pet/pet_cons_821use_dcu_nus_a.htm, release date December 23, 2008.

EIA, 2007a:  Energy Information Administration, U.S. Department of Energy, 2002 Manufacturing Energy Consumption Survey, U.S. Department of Energy, Energy Information Administration, accessed from http://www.eia.doe.gov/emeu/mecs/mecs2002/data02/shelltables.html, issued January 2007.

EIA, 2007b:  Energy Information Administration, US Department of Energy, Documentation for Emissions of Greenhouse Gases in the United States 2005, DOE/EIA-0638 (2005), October 2007.

EPA, 2003: U.S. Environmental Protection Agency, “Draft Regulatory Impact Analysis:  Control of Emissions from Nonroad Diesel Engines,” EPA420-R-03-008, Office of Transportation and Air Quality, April 2003.

Huntley, 2009:  Huntley, Roy, U.S. Environmental Protection Agency, “SCCs & emission factors to be used in 2008 NEI to Bollman May 1 2009.mdb [electronic file],” May 1, 2009.

Lorenz, 2009:  Lorenz, Thomas, Energy Information Administration, personal communication with Andy Bollman, E.H. Pechan & Associates, Inc., March 19, 2009.

Pechan, 2009a:  E.H. Pechan & Associates, Inc., “Area Combustion Source Emissions Inventory Improvement Methodology, Technical Memorandum,” E.H. Pechan & Associates, Inc., prepared for Central Regional Air Planning Association, March 20, 2009.

Pechan, 2009b:  E.H. Pechan & Associates, Inc., “Evaluation of ICI Combustion Coal Sulfur Content, Technical Memorandum,” prepared for Central Regional Air Planning Association, March 28, 2009.

Pechan, 2006:  E.H. Pechan & Associates, Inc. “Documentation for the Final 2002 Nonpoint Sector (Feb 06 Version) National Emission Inventory for Criteria and Hazardous Air Pollutants,” prepared for U.S. Environmental Protection Agency, July 2006.

2.	FUEL COMBUSTION – COMMERCIAL/INSTITUTIONAL

### a.	Source Description and Classification

This subsector covers emissions from fuel combustion at stationary nonpoint sources within the commercial/institutional sector.  The following are the fuel types that will be included in the inventory for the commercial/institutional nonpoint source subsector:  anthracite coal, bituminous/subbituminous coal, distillate oil, residual oil, natural gas, LPG, wood, and kerosene.  Motor gasoline is not included as a nonpoint source because it is expected that all nonpoint gasoline combustion in this sector is included in the nonroad inventory.

The approach to be used in calculating emissions for commercial/institutional fuel combustion is to first develop state-level fuel consumption estimates, then to allocate these to the county-level, and then to multiply the resulting county-level consumption estimates by appropriate emission factors.

Source Classification.  The nonpoint SCCs associated with commercial/institutional fuel combustion are displayed in Table 1.

### b.	Activity Data

Table 2 summarizes this sector’s nonpoint source activity data, units, and data source.  Total state-level commercial sector energy consumption data are available from the Energy Information Administration (EIA)’s State Energy Data System (SEDS), and were used for most source categories (EIA, 2008a).  However, as documented below, a number of adjustments were made to the SEDS data, and a separate EIA data source was used for distillate oil.  Year 2006 SEDS data were used to estimate 2008 emissions because these were the latest year consumption data available at the time this work was performed in 2009.

Fuel-Specific Activity Data  Adjustments

Coal - for commercial/institutional coal combustion, SEDS data do not provide coal consumption estimates by type of coal (i.e., anthracite versus bituminous/subbituminous).  Therefore, EPA used state-level commercial/residential coal distribution data for 2006 from the EIA table “Domestic and Foreign Distribution of U.S. Coal by State of Origin, 2006,” to allocate coal consumption between the two types of coal.  The 2006 ratio of anthracite (and bituminous) coal consumption to total coal consumption was used for this allocation procedure.  Table 3 presents the 2006 anthracite and bituminous coal ratios for each state.

Distillate Oil and LPG – the SEDS commercial/institutional sector distillate oil and LPG consumption data include consumption estimates for equipment that EPA includes in the nonroad sector inventory.  To avoid double-counting of LPG consumption, EPA ran the National Mobile Inventory Model (NMIM) for 2006 and calculated the national volume of nonroad LPG consumption from commercial sector source categories.  This estimate was then divided into the SEDS total industrial sector LPG consumption estimate to yield the proportion of total commercial/institutional sector LPG consumption attributable to the nonroad sector in that year

Table 1.  Nonpoint Source Commercial/Institutional Fuel Combustion SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2103001000 | Stationary Source Fuel Combustion | Commercial/Institutional | Anthracite Coal | Total: All Boiler Types |
| 2103002000 | Stationary Source Fuel Combustion | Commercial/Institutional | Bituminous/Subbituminous Coal | Total: All Boiler Types |
| 2103004000 | Stationary Source Fuel Combustion | Commercial/Institutional | Distillate Oil | Total: Boilers and IC Engines |
| 2103005000 | Stationary Source Fuel Combustion | Commercial/Institutional | Residual Oil | Total: All Boiler Types |
| 2103006000 | Stationary Source Fuel Combustion | Commercial/Institutional | Natural Gas | Total: Boilers and IC Engines |
| 2103007000 | Stationary Source Fuel Combustion | Commercial/Institutional | Liquefied Petroleum Gas (LPG) | Total: All Combustor Types |
| 2103008000 | Stationary Source Fuel Combustion | Commercial/Institutional | Wood | Total: All Boiler Types |
| 2103011000 | Stationary Source Fuel Combustion | Commercial/Institutional | Kerosene | Total: All Combustor Types |

Table 2.  Nonpoint Source Commercial/Institutional Fuel Combustion Activity Data

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 | Activity | Activity Data Source |
| --- | --- | --- | --- | --- | --- | --- |
| 2103001000 | Stationary Source Fuel Combustion | Commercial/Institutional | Anthracite Coal | Total: All Boiler Types | Thousand short tons of coal consumed | EIA’s SEDSa |
| 2103002000 | Stationary Source Fuel Combustion | Commercial/Institutional | Bituminous/ Subbituminous Coal | Total: All Boiler Types | Thousand short tons of coal consumed | EIA’s SEDSa |
| 2103004000 | Stationary Source Fuel Combustion | Commercial/Institutional | Distillate Oil | Total: All Boiler Types | Thousand barrels of distillate oil consumed | EIA’s Fuel Oil and Kerosene Salesa |
| 2103005000 | Stationary Source Fuel Combustion | Commercial/Institutional | Residual Oil | Total: All Boiler Types | Thousand barrels of residual oil consumed | EIA’s SEDSa |
| 2103006000 | Stationary Source Fuel Combustion | Commercial/Institutional | Natural Gas | Total: All Boiler Types | Million cubic feet of natural gas consumed | EIA’s SEDSa |
| 2103007000 | Stationary Source Fuel Combustion | Commercial/Institutional | Liquefied Petroleum Gas (LPG) | Total: All Combustor Types | Thousand barrels of LPG consumed | EIA’s SEDSa |
| 2103008000 | Stationary Source Fuel Combustion | Commercial/Institutional | Wood | Total: All Boiler Types | Billion Btu of wood consumed | EIA’s SEDSa |
| 2103011000 | Stationary Source Fuel Combustion | Commercial/Institutional | Kerosene | Total: All Combustor Types | Thousand barrels of kerosene consumed | EIA’s SEDSa |

a With adjustments that are described in the text of this section.

Table 3.  Proportion of 2006 Commercial/Institutional Sector Coal Consumption by Type of Coal

| FIPS | State | % Anthracite | % Bituminous/ Subbituminous |
| --- | --- | --- | --- |
| 01 | Alabama | 0.0% | 100.0% |
| 02 | Alaska | 0.0% | 100.0% |
| 04 | Arizona | 18.6% | 81.4% |
| 05 | Arkansas | 18.6% | 81.4% |
| 06 | California | 0.0% | 100.0% |
| 08 | Colorado | 0.4% | 99.6% |
| 09 | Connecticut | 100.0% | 0.0% |
| 10 | Delaware | 18.6% | 81.4% |
| 11 | District of Columbia | 0.0% | 100.0% |
| 12 | Florida | 18.6% | 81.4% |
| 13 | Georgia | 0.0% | 100.0% |
| 15 | Hawaii | 0.0% | 100.0% |
| 16 | Idaho | 2.1% | 97.9% |
| 17 | Illinois | 0.2% | 99.8% |
| 18 | Indiana | 5.3% | 94.7% |
| 19 | Iowa | 0.1% | 99.9% |
| 20 | Kansas | 0.0% | 100.0% |
| 21 | Kentucky | 0.2% | 99.8% |
| 22 | Louisiana | 0.0% | 100.0% |
| 23 | Maine | 100.0% | 0.0% |
| 24 | Maryland | 7.1% | 92.9% |
| 25 | Massachusetts | 50.0% | 50.0% |
| 26 | Michigan | 33.3% | 66.7% |
| 27 | Minnesota | 0.3% | 99.7% |
| 28 | Mississippi | 0.0% | 100.0% |
| 29 | Missouri | 0.0% | 100.0% |
| 30 | Montana | 0.0% | 100.0% |
| 31 | Nebraska | 0.0% | 100.0% |
| 32 | Nevada | 0.0% | 100.0% |
| 33 | New Hampshire | 100.0% | 0.0% |
| 34 | New Jersey | 100.0% | 0.0% |
| 35 | New Mexico | 0.0% | 100.0% |
| 36 | New York | 40.0% | 60.0% |
| 37 | North Carolina | 0.0% | 100.0% |
| 38 | North Dakota | 0.0% | 100.0% |
| 39 | Ohio | 12.7% | 87.3% |
| 40 | Oklahoma | 8.3% | 91.7% |
| 41 | Oregon | 0.0% | 100.0% |
| 42 | Pennsylvania | 80.6% | 19.4% |
| 44 | Rhode Island | 100.0% | 0.0% |
| 45 | South Carolina | 0.3% | 99.7% |
| 46 | South Dakota | 0.0% | 100.0% |
| 47 | Tennessee | 0.6% | 99.4% |
| 48 | Texas | 18.6% | 81.4% |
| 49 | Utah | 0.0% | 100.0% |
| 50 | Vermont | 100.0% | 0.0% |
| 51 | Virginia | 3.7% | 96.3% |
| 53 | Washington | 0.0% | 100.0% |
| 54 | West Virginia | 9.5% | 90.5% |
| 55 | Wisconsin | 0.9% | 99.1% |
| 56 | Wyoming | 0.0% | 100.0% |

(approximately 18 percent).  To estimate the volume of commercial/institutional sector LPG consumption that should not be included in the nonpoint source inventory, EPA subtracted 18 percent from each state’s commercial sector LPG consumption estimate reported in SEDS.

To avoid double-counting of distillate oil consumption between the nonpoint and nonroad sector emission inventories, EPA relied on a source other than SEDS to estimate consumption.  The approach uses more detailed distillate oil consumption estimates reported in EIA’s Fuel Oil and Kerosene Sales, and assumptions from the regulatory impact analysis (RIA) for EPA’s nonroad diesel emissions rulemaking (EIA, 2008b; EPA, 2003).  Table 4 displays the assumptions that were applied to the state-level distillate oil consumption estimates reported in Fuel Oil and Kerosene Sales to estimate total stationary source commercial/institutional sector consumption.

Table 4. Assumptions Used to Estimate Commercial/Institutional Sector Stationary Source Distillate Fuel Consumption

| Sector | Distillate Fuel Type | % of Total Consumption from Stationary Sources |
| --- | --- | --- |
| Commercial | No. 1 Distillate Fuel Oil | 80 |
|  | No. 2 Distillate Fuel Oil | 100 |
|  | No. 2 Distillate/Ultra-Low, Low, and High Sulfur Diesel | 0a |
|  | No. 4 Distillate Fuel Oil | 100 |

a A very small portion of total commercial/institutional diesel is consumed by point sources (SCC 203001xx).

Point Source Energy Adjustment

Because the point source inventory also includes commercial/institutional fuel combustion source categories, it is necessary to subtract point source inventory fuel use from the SEDS fuel consumption estimates.  Table 5 displays the crosswalk between commercial/institutional fuel combustion nonpoint SCCs and associated point SCCs that EPA plans to use in the subtraction procedure.  The subtractions will be performed after summing the point source throughput estimates to the state-level.

Table 5.  Point Source Subtraction SCCs Relevant to Nonpoint Source Commercial/Institutional Fuel Combustion

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 (Fuel-if not specified at level 3) |
| --- | --- | --- | --- |
| 2103001000 & 2103002000 -- Commercial/Institutional; Anthracite Coal; Total: All Boiler Types and Bituminous/Subbituminous Coal; Total: All Boiler Types |  |  |  |
| 103002xx | External Combustion Boilers | Commercial/Institutional | Bituminous/Subbituminous Coal |
| 10500202 | External Combustion Boilers | Space Heaters | Commercial/Institutional (Coal) |
| 50190002 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions (Coal) |
| 50290002 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions (Coal) |
| 2103004000 -- Commercial/Institutional; Distillate Oil; Total: Boilers and IC Engines |  |  |  |
| 103005xx | External Combustion Boilers | Commercial/Institutional | Distillate Oil |
| 10500205 | External Combustion Boilers | Space Heaters | Commercial/Institutional (Distillate Oil) |
| 50190005 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions (Distillate Oil) |
| 50290005 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions (Distillate Oil) |
| 2103005000 -- Commercial/Institutional; Residual Oil; Total: All Boiler Types |  |  |  |
| 103004xx | External Combustion Boilers | Commercial/Institutional | Residual Oil |
| 2103006000 -- Commercial/Institutional; Natural Gas; Total: Boilers and IC Engines |  |  |  |
| 103006xx | External Combustion Boilers | Commercial/Institutional | Natural Gas |
| 10500206 | External Combustion Boilers | Space Heaters | Commercial/Institutional (Natural Gas) |
| 203002xx | Internal Combustion Engines | Commercial/Institutional | Natural Gas |
| 50190006 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions (Natural Gas) |
| 50290006 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions (Natural Gas) |
| 2103007000 -- Commercial/Institutional; Liquid Petroleum Gas; Total: All Combustor Types |  |  |  |
| 103010xx | External Combustion Boilers | Commercial/Institutional | Liquefied Petroleum Gas (LPG) |
| 10500210 | External Combustion Boilers | Space Heaters | Commercial/Institutional (LPG) |
| 203010xx | Internal Combustion Engines | Commercial/Institutional | Liquefied Petroleum Gas (LPG) |
| 50190010 | Waste Disposal | Solid Waste Disposal - Government | Auxiliary Fuel/No Emissions (LPG) |
| 50290010 | Waste Disposal | Solid Waste Disposal - Commercial/Institutional | Auxiliary Fuel/No Emissions (LPG) |
| 2103008000 -- Commercial/Institutional; Wood; Total: All Boiler Types |  |  |  |
| 103009xx | External Combustion Boilers | Commercial/Institutional | Wood/Bark Waste |
| 10500209 | External Combustion Boilers | Space Heaters | Commercial/Institutional (Wood) |
| 2103011000 -- Commercial/Institutional; Kerosene; Total: All Combustor Types |  |  |  |
| 203009xx | Internal Combustion Engines | Commercial/Institutional | Kerosene/Naphtha (Jet Fuel) |
| 50100603 | Waste Disposal | Solid Waste Disposal - Government | Fire Fighting (Kerosene) |

### c.	County Allocation of State Activity Data

Year 2005 county-level activity estimates will be developed by allocating the state-level activity resulting from the adjustments to the SEDS data described above.  The EPA compiled 2006 estimates of commercial sector (NAICS codes 42 through 81) employment from the Bureau of Census’ County Business Patterns 2006 for use in this procedure (Census, 2009a).  A separate document describes how withheld County Business Patterns employment data were estimated (Divita, 2008).  The EPA also developed 2006 county-level estimates of institutional sector (NAICS code 92) employment from 2007 local government employment data in the 2007 Census of Governments and adjustments reflecting each state’s 2006/2007 local government

employment ratio (Census, 2009b).  State-level commercial/institutional fuel combustion by fuel type was allocated to each county using the ratio of the number of commercial/institutional sector employees in each county to the total number of commercial/institutional sector employees in the state.

### d.	Emission Factors

The EPA has compiled criteria and hazardous air pollutant emission factors for nonpoint source commercial/institutional fuel combustion categories (Huntley, 2009).  These emission factors, which are too numerous to list here, are included in a spreadsheet within the ICI fuel combustion workbook.  In most cases, these are the same emission factors that were used in preparing the 2002 nonpoint source NEI (Pechan, 2006).  Commercial/institutional wood combustion emission factors were obtained from an ICI fuel combustion study being performed for the Central Regional Air Planning Association (CENRAP) (Pechan, 2009a).

### e.	Potential Improvements

States may wish to incorporate the following improvements in developing nonpoint source commercial/institutional fuel combustion emission estimates:

### Update SO2 emission factors to reflect coal sulfur content estimates for coal used in the industrial sector in 2006--a recent review of industrial and commercial coal sulfur content values for the Central Regional Air Planning Association concluded that the 2002 NEI values are based on unrepresentative data (Pechan, 2009b).

### f.	References

Census, 2009a:  Bureau of the Census, U.S. Department of Commerce, County Business Patterns 2006, available from:   http://www.census.gov/epcd/cbp/download/06_data/index.html, accessed March 2009.

Census, 2009b:  Bureau of the Census, U.S. Department of Commerce, “Local Government Employment and Payroll, March 2006,” 2007 Census of Governments, available from: http://www.census.gov/govs/www/apesloc06.html, accessed March 2009.

Divita, 2008:  Divita, Frank, E.H. Pechan & Associates, Inc., memorandum to Roy Huntley, U.S. Environmental Protection Agency, “County Business Patterns Calculations,” December 4, 2008.

EIA, 2008a:  Energy Information Administration, U.S. Department of Energy, State Energy Data System – Consumption, Physical Units, 1960-2006, available from:  http://www.eia.doe.gov/emeu/states/_seds.html, released November 28, 2008.

EIA, 2008b:  Energy Information Administration, U.S. Department of Energy, Fuel Oil and Kerosene Sales, data available from: http://tonto.eia.doe.gov/dnav/pet/pet_cons_821use_dcu_nus_a.htm, release date December 23, 2008.

EPA, 2003: U.S. Environmental Protection Agency, “Draft Regulatory Impact Analysis:  Control of Emissions from Nonroad Diesel Engines,” EPA420-R-03-008, Office of Transportation and Air Quality, April 2003.

Huntley, 2009:  Huntley, Roy, U.S. Environmental Protection Agency, “SCCs & emission factors to be used in 2008 NEI to Bollman May 1 2009.mdb [electronic file],” May 1, 2009.

Pechan, 2009a:  E.H. Pechan & Associates, Inc., “Area Combustion Source Emissions Inventory Improvement Methodology, Technical Memorandum,” E.H. Pechan & Associates, Inc., prepared for Central Regional Air Planning Association, March 20, 2009.

Pechan, 2009b:  E.H. Pechan & Associates, Inc., “Evaluation of ICI Combustion Coal Sulfur Content, Technical Memorandum,” prepared for Central Regional Air Planning Association, March 28, 2009.

Pechan, 2006:  E.H. Pechan & Associates, Inc. “Documentation for the Final 2002 Nonpoint Sector (Feb 06 Version) National Emission Inventory for Criteria and Hazardous Air Pollutants,” prepared for U.S. Environmental Protection Agency, July 2006.

In addition, the 2002 MECS data treat “synthetic coal” that is ultimately combusted as regular coal, as a non-fuel (feedstock) use.

As noted in a footnote to Table 7, it is important to exclude any natural gas pipeline fuel use from the point source subtraction procedure because SEDS includes such use in the Transportation sector rather than the Industrial sector.

County-level federal and state government employment data are not available from the Bureau of the Census.

Actual data characterizing sulfur content of coal consumed in the commercial sector is limited, but appears to be in-line with industrial coal consumption values.  Therefore, industrial sector sulfur content can reasonably be assumed to be representative of commercial sector sulfur content.