GASOLINE DISTRIBUTION:  STAGE II

a. Source Category Description

Stage II gasoline distribution emissions are the refueling emissions that occur during the transfer of gasoline from storage tanks at service stations to vehicle fuel tanks.

Stage II emissions are reported under the following nonpoint SCC:

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 2501060100 | Storage and Transport | Petroleum and Petroleum Product Storage | Gasoline Service Stations | Stage 2: Total |

This SCC is only used in the NEI to report emissions for onroad gasoline vehicle refueling.  (Estimates for Stage II nonroad equipment gasoline refueling are included in the nonroad source inventory.)

b. Emissions Calculations

The VOC emission estimates were generated via the 2008 NMIM model using county-specific fuel parameters, temperature, humidity, Reid vapor pressure, and other relevant inputs. The EPA calculated HAP emission estimates by applying the HAP speciation profiles reported in Table 1.

Table 1.  Stage II Service Station Refueling HAP Speciation Profiles and Total Emission Estimates

| Pollutant | Pollutant Code | Emission Factor | Reference | National Emissions (tpy) |
| --- | --- | --- | --- | --- |
| VOC | VOC | n/a | 1 | 1.60E+05 |
| 2,2,4-Trimethylpentane | 540841 | 0.75% of VOC | 2 | 1.21E+03 |
| Cumene | 98828 | 0.012% of VOC | 2 | 1.94E+01 |
| Ethyl Benzene | 100414 | 0.053% of VOC | 2 | 8.58E+01 |
| n-Hexane | 110543 | 1.8% of VOC | 2 | 2.91E+03 |
| Naphthalene | 91203 | 0.00027% of VOC | 2 | 4.37E-01 |
| Toluene | 108883 | 1.4% of VOC | 2 | 2.27E+03 |
| Xylenes | 1330207 | 0.56% of VOC | 2 | 9.07E+02 |
| Benzene | 71432 | county-specific % of VOC | 3 | 1.23E+03 |

It is important to note that the above discussion addresses the calculation of total onroad VOC emissions from refueling. The 2008 point source NEI reports VOC emissions related to service station refueling. To obtain nonpoint emissions, States should subtract the 2008 point source VOC emission estimates from the total VOC emission estimates reported here. The relevant point source SCCs are listed in Table 2 below.

Table 2. Gasoline Vehicle Refueling Point Source SCCs

| SCC | SCC Level 1 | SCC Level 2 | SCC Level 3 | SCC Level 4 |
| --- | --- | --- | --- | --- |
| 40600401 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Filling Vehicle Gas Tanks - Stage II | Vapor Loss w/o Controls |
| 40600402 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Filling Vehicle Gas Tanks - Stage II | Liquid Spill Loss w/o Controls |
| 40600499 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Filling Vehicle Gas Tanks - Stage II | Not Classified ** |
| 40600601 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage II | Vapor Loss w/o Controls |
| 40600602 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage II | Liquid Spill Loss w/o Controls |
| 40600603 | Petroleum and Solvent Evaporation | Transportation and Marketing of Petroleum Products | Consumer (Corporate) Fleet Refueling - Stage II | Vapor Loss w/controls |

c. References

1.	2008 NMIM runs performed by John Van Bruggen and Melissa Spivey, E.H. Pechan and Associates, Inc., January 2010.

2.	Hester, Charles, MACTEC, Inc. Memorandum from Charles Hester, MACTEC, Inc., to Stephen Shedd, U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards, Emission Standards Division, "Review of Data on HAP Content in Gasoline," May 18, 2006.

3.	Benzene speciation profiles calculated by Jonathan Dorn, E.H. Pechan and Associates, Inc. from county-level VOC and benzene emissions developed from a 2008 NMIM run. The NMIM run was performed by John Van Bruggen, E.H. Pechan and Associates, Inc., January 2010.