FOSSIL FUEL COMBUSTION – RESIDENTIAL – DISTILLATE OIL

## a.	Source Category Description

Residential distillate oil combustion is oil that is burned in residential housing.

The general approach to calculating emissions for this SCC is to take State-level distillate oil consumption from the EIA and allocate it out to the county level using the methods described below.  County-level oil consumption is multiplied by the emission factors to calculate emissions.

For this source category, the following SCC was assigned:

| Source Classification Code | SCC Level One | SCC Level Two | SCC Level Three | SCC Level Four |
| --- | --- | --- | --- | --- |
| 2104004000 | Stationary Source Fuel Combustion | Residential | Distillate Oil | Total Boilers and IC Engines |

## b.	Activity Data

The state-level volume of distillate oil consumed by residential combustion in the U.S. was used to estimate emissions.  Distillate oil consumption by energy use sector is presented in State Energy Data System (SEDS) 2014 Consumption tables published by the Energy Information Administration (EIA).1 Distillate consumption is represented in the SEDS table by the Data Series Name (MSN) DFRCP.

State-level distillate oil consumption was allocated to each county using the US Census Bureau’s 2014 5-year estimate Census Detailed Housing Information.2 These data include the number of housing units using a specific type of fuel for residential heating. State distillate oil consumption was allocated to each county using the ratio of the number of houses burning distillate oil in each county to the total number of houses burning distillate oil in the State.

## c.	Control Factors

No control measures are assumed for this category.

## d.	Emission Factors

Criteria pollutant emission factors for distillate oil are from AP-42 (see Table 1). 3  For all counties in the United States, the distillate oil consumed by residential combustion is assumed to be No. 2 fuel oil with a heating value of 140,000 Btu per gallon and a sulfur content of 0.30%.4  Dioxin/furan and HAP emission factors are from “Documentation of Emissions Estimation methods for Year 2000 and 2001 Mobile Source and Nonpoint Source Dioxin Inventories”5 and “Documentation for the 1999 Base Year Nonpoint Area Source National Emission Inventory for Hazardous Air Pollutants,”6 respectively.  Sulfur content was 0.30% and was obtained from data compiled in preparing the 1999 residential coal combustion emissions estimates.4  The ammonia emission factor is from EPA’s Estimating Ammonia Emissions from Anthropogenic Sources, Draft Report.7

## e.	Sample Calculations

Emissions are calculated for each county using emission factors and activity as:

Ex,p = FCx × EFx,p

where:

Ex,p = annual emissions for fuel type x and pollutant p

FCx = annual fuel consumption for fuel type x

EFx,p = emission factor for fuel type x and pollutant p

And FCx = AState x (Hcounty / HState)

where:

AState = State activity data from EIA

HCounty = number of houses in the county using distillate oil as the primary heating fuel

HState = number of houses in the state using distillate oil as the primary heating fuel

And HCounty = (DCState/CState) x CCounty

where:

DCState = State distillate consumption from EIA

CState = State distillate and kerosene consumption from EIA

CCounty = number of houses in the county using distillate oil or kerosene as the primary heating fuel

Example:

Using Allegheny County, PA as an example:

The State of Pennsylvania had a reported use of 15,798 thousand barrels of distillate oil and 358 barrels of kerosene in the residential sector in 2014.  Allegheny County, PA had 8,081 houses that use distillate fuel oil or kerosene as the primary heating fuel.  Using the state ratio of distillate to kerosene, Allegheny County can be assumed to have 7,902 houses using distillate as the primary heating fuel, out of 910,155 houses in the state.  This equates to a share of 1% of the distillate oil used for residential heating in the state.  From Table 5, the emission factor for CO is 5 lb/thousand gallons.  Because the emission factor is in lbs/thousand gallons, a conversion factor of 42 gallons per barrel is applied.

AAlegheny 	= 15,798 thousand barrels × (7,902 houses / 910,155 houses)

× 42 gal / barrel

= 5,760.62 thousand gallons

EmisAlegheny, CO 	= 5,760.62 thousand gallons × 5 lb CO/ thousand gallons

= 28,803 lbs CO or 14.4 tons CO

## f. Changes to Methodology

The primary difference between 2011 and 2014 was the allocation of distillate oil consumption.  The US Census Bureau Detailed Housing Information category for homes using distillate oil also includes kerosene as a fuel source. To tease apart the number of houses using each of these fuels, the number was multiplied by the ratio of state distillate consumption to the total state consumption of distillate oil and kerosene.  These steps were not taken in 2011.

g. Puerto Rico and US Virgin Islands Emissions Calculations

Since insufficient data exists to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emission factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emission factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

## h. References

1.	U.S. Department of Energy, Energy Information Administration (EIA).  State Energy Data System (SEDS): 1960-2014 Consumption.  Washington, DC 2015.  Available from:  http://www.eia.gov/state/seds/seds-data-complete.cfm?sid=US#CompleteDataFile, accessed July 2016.

2.	U.S. Census Bureau. B25040 House Heating Fuel, 2009-2014 ACS 5-Year Estimates. Available from: http://factfinder.census.gov/faces/tableservices/jsf/pages/productview.xhtml?pid=ACS_14_5YR_B25040&prodType=table, accessed July 2014.

3.	U.S. Environmental Protection Agency.  Compilation of Air Pollutant Emission Factors, 5th Edition, AP-42, Volume I:  Stationary Point and Area Sources.  Research Triangle Park, North Carolina.  1996.

4.	U.S. Environmental Protection Agency.  Emission Factor and Inventory Group.  Final Summary of the Development and Results of a Methodology for Calculating Area Source Emissions from Residential Fuel Combustion.  Prepared by Pacific Environmental Services, Inc.  Research Triangle Park, NC.  September 2002.  Available from:  http://www3.epa.gov/ttn/chief/eiip/techreport/volume03/index.html, accessed September 2015.

5.	U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  “Documentation of Emissions Estimation methods for Year 2000 and 2001 Mobile Source and Nonpoint Source Dioxin Inventories.”  Prepared by E.H. Pechan & Associates, Inc., Durham, NC.  May 2003.

6.	U.S. Environmental Protection Agency, Emission Factors and Inventory Group.  “Documentation for the 1999 Base Year Nonpoint Area Source National Emission Inventory for Hazardous Air Pollutants.”  Prepared by Eastern Research Group, Inc.  Morrisville, NC.  September 2002.

7.	U.S. Environmental Protection Agency.  Emission Factor and Inventory Group.  Estimating Ammonia Emissions from Anthropogenic Sources, Draft Report.  Prepared by E.H. Pechan and Associates, Inc.  Research Triangle Park, NC.  September 2003.

8.	U.S. Environmental Protection Agency.  Emission Factor and Inventory Group.  Estimating Ammonia Emissions from Anthropogenic Sources, Draft Report.  Prepared by E.H. Pechan and Associates, Inc.  Research Triangle Park, NC.  September 2003.

Table 1.  National Emission Factors for Residential Distillate Oil Combustion

| Pollutant Code | Code Description | Emissions Factor (LB/E3GAL) | Reference |
| --- | --- | --- | --- |
| 120127 | ANTHRACENE | 1.222E-06 | 6 |
| 129000 | PYRENE | 4.214E-06 | 6 |
| 1746016 | 2,3,7,8-TETRACHLORODIBENZO-P-DIOXIN | 4.659E-10 | 5 |
| 191242 | BENZO[G,H,I,]PERYLENE | 2.248E-06 | 6 |
| 193395 | INDENO[1,2,3-C,D]PYRENE | 2.107E-06 | 6 |
| 206440 | FLUORANTHENE | 4.916E-06 | 6 |
| 208968 | ACENAPHTHYLENE | 2.528E-07 | 6 |
| 218019 | CHRYSENE | 2.388E-06 | 6 |
| 3268879 | OCTACHLORODIBENZO-P-DIOXIN | 5.491E-10 | 5 |
| 39001020 | OCTACHLORODIBENZOFURAN | 2.496E-10 | 5 |
| 50000 | FORMALDEHYDE | 3.371E-02 | 6 |
| 51207319 | 2,3,7,8-TETRACHLORODIBENZOFURAN | 4.410E-10 | 5 |
| 53703 | DIBENZO[A,H]ANTHRACENE | 1.686E-06 | 6 |
| 56553 | BENZ[A]ANTHRACENE | 4.074E-06 | 6 |
| 71432 | BENZENE | 2.107E-04 | 6 |
| 7439921 | LEAD | 1.264E-03 | 6 |
| 7439965 | MANGANESE | 8.428E-04 | 6 |
| 7439976 | MERCURY | 4.214E-04 | 6 |
| 7440020 | NICKEL | 4.214E-04 | 6 |
| 7440382 | ARSENIC | 5.619E-04 | 6 |
| 7440417 | BERYLLIUM | 4.214E-04 | 6 |
| 7440439 | CADMIUM | 4.214E-04 | 6 |
| 16065831 | Chromium III | 0.000345556 |  |
| 18540299 | Chromium (VI) | 0.0000758538 |  |
| 75070 | ACETALDEHYDE | 4.916E-03 | 6 |
| 7782492 | SELENIUM | 2.107E-03 | 6 |
| 83329 | ACENAPHTHENE | 2.107E-05 | 6 |
| 85018 | PHENANTHRENE | 1.054E-05 | 6 |
| 86737 | FLUORENE | 4.495E-06 | 6 |
| 91203 | NAPHTHALENE | 1.138E-03 | 6 |
| CO | CARBON MONOXIDE | 5.000E+00 | 3 |
| NH3 | AMMONIA | 1.000E+00 | 8 |
| NOX | NITROGEN OXIDES | 1.800E+01 | 3 |
| PM10-FIL | PRIMARY PM10, FILTERABLE PORTION ONLY | 1.080E+00 | 3 |
| PM10-PRI | PRIMARY PM10 (INCLUDES FILTERABLES + CONDENSIBLES) | 2.380E+00 | 3 |
| PM25-FIL | PRIMARY PM2.5, FILTERABLE PORTION ONLY | 8.300E-01 | 3 |
| PM25-PRI | PRIMARY PM2.5 (INCLUDES FILTERABLES + CONDENSIBLES) | 2.130E+00 | 3 |
| PM-CON | PRIMARY PM CONDENSIBLE PORTION ONLY (< 1 MICRON) | 1.300E+00 | 3 |
| SO2 | SULFUR DIOXIDE | 4.260E+01 | 3 |
| VOC | VOLATILE ORGANIC COMPOUNDS | 7.000E-01 | 3 |