FOSSIL FUEL COMBUSTION – RESIDENTIAL – KEROSENE

## a.	Source Category Description

Residential kerosene combustion is kerosene that is burned in residential housing.  Common uses of energy associated with this sector include space heating, water heating, cooking, and running a wide variety of other equipment.

The general approach to calculating emissions for this SCC is to take State kerosene consumption from the EIA and allocate it to the county level using methods described below.  County-level kerosene consumption is multiplied by the emission factors to calculate emissions.

For this source category, the following SCC was assigned:

| Source Classification Code | SCC Level One | SCC Level Two | SCC Level Three | SCC Level Four |
| --- | --- | --- | --- | --- |
| 2104011000 | Stationary Source Fuel Combustion | Residential | Kerosene | Total: All Combustor Types |

## b.	Activity Data

The volume of kerosene consumed by residential combustion in the U.S. was used to estimate emissions.  Kerosene consumption by energy use sector is presented in State Energy Data System (SEDS) 2014 Consumption tables published by the Energy Information Administration (EIA).1 Kerosene consumption is represented in the SEDS table by the Date Series Name (MSN) KSRCP.

State-level kerosene consumption was allocated to each county using the US Census Bureau’s 2014 5-year estimate Census Detailed Housing Information.2 These data include the number of housing units using a specific type of fuel for residential heating. State kerosene consumption was allocated to each county using the ratio of the number of houses burning kerosene in each county to the total number of houses burning kerosene in the State.

## c.	Control Factors

No control measures are assumed for this category.

## d.	Emission Factors

Emission factors for distillate oil were used for kerosene, but the distillate oil emission factors were multiplied by a factor of 135/140 to convert them for this use.  This factor is based on the ratio of the heat content of kerosene (135,000 Btu/gallon) to the heat content of distillate oil (140,000 Btu/gallon).3 Criteria pollutant emission factors are from AP-42 (see Table 1). 3  Dioxin/furan and HAP emission factors are from “Documentation of Emissions Estimation methods for Year 2000 and 2001 Mobile Source and Nonpoint Source Dioxin Inventories”5 and “Documentation for the 1999 Base Year Nonpoint Area Source National Emission Inventory for Hazardous Air Pollutants,”6 respectively.  Distillate sulfur content (0.30%) was used for kerosene as well.4

## e.	Sample Calculations

Emissions are calculated for each county using emission factors and activity as:

Ex,p = FCx × EFx,p

where:

Ex,p = annual emissions for fuel type x and pollutant p,

FCx = annual fuel consumption for fuel type x,

EFx,p = emission factor for fuel type x and pollutant p,

And FCx = AState x (Hcounty / HState)

where:

AState = State activity data from EIA

HCounty = number of houses in the county using kerosene as the primary heating fuel

HState = number of houses in the state using kerosene as the primary heating fuel

And HCounty = (DCState/CState) x CCounty

where:

DCState = State kerosene consumption from EIA

CState = State distillate and kerosene consumption from EIA

CCounty = number of houses in the county using distillate oil or kerosene as the primary heating fuel

Example:

Using Allegheny County, PA as an example:

The State of Pennsylvania had a reported use of 15,798 thousand barrels of distillate oil and 358 thousand barrels of kerosene in the residential sector in 2014.  Allegheny County, PA had 8,081 houses that use distillate fuel oil or kerosene as the primary heating fuel.  Using the state ratio of distillate to kerosene, Allegheny County can be assumed to have 179.07 houses using kerosene as the primary heating fuel, out of 20,625 houses in the state. This equates to a share of 0.87% of the kerosene used for residential heating in the state.  From Table 1, CO Emission factor is 202.5 lb/thousand barrels. Because the emission factor is in lbs/thousand gallons, a conversion factor of 42 gallons per barrel is applied.

AAlegheny 	= 358 thousand barrels × (179.07 houses / 20,625 houses)

= 3.1 thousand gallons

EmisAlegheny, CO 	= 3.1 thousand gallons × 202.5 lb CO/ thousand gallons

= 629.4 lbs CO or 0.31 tons CO

## f.	Changes to Methodology

The primary difference between 2011 and 2014 was the allocation of distillate oil consumption.  The US Census Bureau Detailed Housing Information category for homes using distillate oil also includes kerosene as a fuel source. To tease apart the number of houses using each of these fuels, the number was multiplied by the ratio of state kerosene consumption to the total state consumption of distillate oil and kerosene.  These steps were not taken in 2011.

g. Puerto Rico and US Virgin Islands Emissions Calculations

Since insufficient data exists to calculate emissions for the counties in Puerto Rico and the US Virgin Islands, emissions are based on two proxy counties in Florida: 12011, Broward County for Puerto Rico and 12087, Monroe County for the US Virgin Islands. The total emissions in tons for these two Florida counties are divided by their respective populations creating a tons per capita emission factor.  For each Puerto Rico and US Virgin Island county, the tons per capita emission factor is multiplied by the county population (from the same year as the inventory’s activity data) which served as the activity data. In these cases, the throughput (activity data) unit and the emissions denominator unit are “EACH”.

## h.      References

1.	U.S. Department of Energy, Energy Information Administration (EIA).  State Energy Data System (SEDS): 1960-2014 Consumption.  Washington, DC 2016.

2.	U.S. Census Bureau. B25040 House Heating Fuel, 2009-2014 ACS 5-Year Estimates. Available from, accessed May 2019.

3.	U.S. Environmental Protection Agency.  Compilation of Air Pollutant Emission Factors, 5th Edition, AP-42, Volume I:  Stationary Point and Area Sources.  Research Triangle Park, North Carolina.  1996.

4.	U.S. Environmental Protection Agency.  Emission Factor and Inventory Group.  Final Summary of the Development and Results of a Methodology for Calculating Area Source Emissions from Residential Fuel Combustion.  Prepared by Pacific Environmental Services, Inc.  Research Triangle Park, NC.  September 2002 accessed May 2019.

5.	U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  “Documentation of Emissions Estimation methods for Year 2000 and 2001 Mobile Source and Nonpoint Source Dioxin Inventories.”  Prepared by E.H. Pechan & Associates, Inc., Durham, NC.  May 2003.

6.	U.S. Environmental Protection Agency, Emission Factors and Inventory Group.  “Documentation for the 1999 Base Year Nonpoint Area Source National Emission Inventory for Hazardous Air Pollutants.”  Prepared by Eastern Research Group, Inc.  Morrisville, NC.  September 2002.

Table 1.  National Emission Factors for Residential Kerosene Combustion

| Pollutant Code | Pollutant Code Description | Factor Numeric Value | Factor Unit Numerator | Factor Unit Denominator |
| --- | --- | --- | --- | --- |
| 120127 | ANTHRACENE | 4.9495E-05 | LB | E3BBL |
| 129000 | PYRENE | 0.00017067 | LB | E3BBL |
| 1746016 | 2,3,7,8-TETRACHLORODIBENZO-P-DIOXIN | 1.887E-08 | LB | E3BBL |
| 191242 | BENZO[G,H,I,]PERYLENE | 9.1025E-05 | LB | E3BBL |
| 193395 | INDENO[1,2,3-C,D]PYRENE | 8.5336E-05 | LB | E3BBL |
| 206440 | FLUORANTHENE | 0.00019912 | LB | E3BBL |
| 208968 | ACENAPHTHYLENE | 1.024E-05 | LB | E3BBL |
| 218019 | CHRYSENE | 9.6714E-05 | LB | E3BBL |
| 3268879 | OCTACHLORODIBENZO-P-DIOXIN | 2.2239E-08 | LB | E3BBL |
| 39001020 | OCTACHLORODIBENZOFURAN | 1.0109E-08 | LB | E3BBL |
| 50000 | FORMALDEHYDE | 1.3653684 | LB | E3BBL |
| 51207319 | 2,3,7,8-TETRACHLORODIBENZOFURAN | 1.7859E-08 | LB | E3BBL |
| 53703 | DIBENZO[A,H]ANTHRACENE | 6.8268E-05 | LB | E3BBL |
| 56553 | BENZ[A]ANTHRACENE | 0.00016498 | LB | E3BBL |
| 71432 | BENZENE | 0.00853355 | LB | E3BBL |
| 7439921 | LEAD | 0.05120132 | LB | E3BBL |
| 7439965 | MANGANESE | 0.03413421 | LB | E3BBL |
| 7439976 | MERCURY | 0.01706711 | LB | E3BBL |
| 7440020 | NICKEL | 0.01706711 | LB | E3BBL |
| 7440382 | ARSENIC | 0.02275614 | LB | E3BBL |
| 7440417 | BERYLLIUM | 0.01706711 | LB | E3BBL |
| 7440439 | CADMIUM | 0.01706711 | LB | E3BBL |
| 16065831 | Chromium III | 0.013995026 | LB | E3BBL |
| 18540299 | Chromium (VI) | 0.003072079 | LB | E3BBL |
| 75070 | ACETALDEHYDE | 0.19911623 | LB | E3BBL |
| 7782492 | SELENIUM | 0.08533553 | LB | E3BBL |
| 83329 | ACENAPHTHENE | 0.00085336 | LB | E3BBL |
| 85018 | PHENANTHRENE | 0.00042668 | LB | E3BBL |
| 86737 | FLUORENE | 0.00018205 | LB | E3BBL |
| 91203 | NAPHTHALENE | 0.04608118 | LB | E3BBL |
| NH3 | AMMONIA | 40.5 | LB | E3BBL |
| CO | CARBON MONOXIDE | 202.5 | LB | E3BBL |
| NOX | NITROGEN OXIDES | 729 | LB | E3BBL |
| PM10-PRI | PRIMARY PM10 (INCLUDES FILTERABLES + CONDENSIBLES) | 96.39 | LB | E3BBL |
| PM25-PRI | PRIMARY PM2.5 (INCLUDES FILTERABLES + CONDENSIBLES) | 86.265 | LB | E3BBL |
| PM10-FIL | PRIMARY PM10, FILTERABLE PORTION ONLY | 43.74 | LB | E3BBL |
| PM25-FIL | PRIMARY PM2.5, FILTERABLE PORTION ONLY | 33.615 | LB | E3BBL |
| PM-CON | PRIMARY PM CONDENSIBLE PORTION ONLY (ALL LESS THAN 1 MICRON) | 52.65 | LB | E3BBL |
| SO2 | SULFUR DIOXIDE | 1,725.3 | LB | E3BBL |
| VOC | VOLATILE ORGANIC COMPOUNDS | 28.35 | LB | E3BBL |