www.erg.com

DRAFT MEMORANDUM

TO: 		Jeffrey Vukovich, U.S. EPA

Alison Eyth, U.S. EPA

Janice Godfrey, U.S. EPA

FROM:	Mike Pring, Eastern Research Group, Inc. (ERG)

William Barr, ERG

Regi Oommen, ERG

DATE: 	June 20, 2025

SUBJECT:	2023 Abandoned Well Emission Estimates for VOC and HAP

# INTRODUCTION

The purpose of this memorandum is to present a draft 2023 emissions inventory with estimates of VOC and hazardous air pollutants (HAP) emissions from abandoned wells across the U.S. This work builds upon the initial methodology development and emissions inventory completed for abandoned wells for 2020 and 2022. Results from both the 2022 and 2023 inventories are presented here for comparison.

The term "abandoned wells" encompasses various types of wells:

- Wells with no recent production, and not plugged. Common terms (such as those used in state databases) might include: inactive, temporarily abandoned, shut-in, dormant, and idle.

- Wells with no recent production and no responsible operator. Common terms might include: orphaned, deserted, long-term idle, and abandoned.

- Wells that have been plugged to prevent migration of gas or fluids.

There are approximately 4.1 million abandoned wells in the U.S., with approximately 2.4 million abandoned oil wells, 0.7 million abandoned gas wells, and 1.0 million abandoned dry wells (may be oil or gas wells). Abandoned wells may emit CH4, CO2, VOC, and various HAP.

Estimates of greenhouse gas (GHG) emissions (CH4 and CO2) from abandoned wells have been estimated as part of the Inventory of U.S. Greenhouse Gas Emissions and Sinks since 2018. The GHG inventory (GHGI) methodology, estimates of emissions, and emission factors for abandoned wells served as the starting point for development of the VOC and HAP emissions inventory for abandoned wells presented here.

# DATA RESOURCES

The GHGI abandoned well inventory and emissions estimation methodology form the basis of the VOC and HAP inventory described in this memorandum. In addition to the GHGI, several other data sources were used to resolve the inventory to the county level and to allow for estimation of VOC and HAP emissions. These include the following:

- Enverus database;

- Historical Oil and Gas Commission Websites;

- Abandoned Well CH4 Emission Factors from the GHGI; and

- EPA’s Nonpoint Oil and Gas Emission Estimation Tool.

These data sources are discussed in more detail below.

## Enverus Database

The primary activity data source used to develop the abandoned well GHGI is the Enverus database. The Enverus database was also used as the primary data source for activity data to develop the VOC and HAP abandoned well emissions inventory. One distinction between the GHGI abandoned well counts and the abandoned well counts used for the VOC and HAP abandoned well emissions inventory is that for purposes of the VOC and HAP inventory, well counts were developed at the county level. Otherwise, the methodology and approach used in the GHGI were retained to develop the abandoned well counts for the VOC and HAP inventory. The Enverus data allows for distinction between plugged and unplugged wells, which is needed to apply the abandoned well emission factors (discussed in Section 2.3). Additional details on the methodology used to extract abandoned well activity data from Enverus are available in the memorandum “Inventory of U.S. Greenhouse Gas Emissions and Sinks 1990-2020: Updates for Abandoned Oil and Gas Wells” (April 2022).

Table 1 presents the Enverus abandoned well counts for 2023 and 2022 at the state level.

Table 1. Enverus Abandoned Well Counts

| State | 2023 Abandoned Well Counts |  |  | 2022 Abandoned Well Counts |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Oil Wells | Gas Wells | Dry Wells | Oil Wells | Gas Wells | Dry Wells |
| AK | 3,524 | 534 | 680 | 2,853 | 406 | 620 |
| AL | 2,910 | 6,256 | 2,476 | 2,849 | 6,162 | 2,469 |
| AR | 6,832 | 5,695 | 6,898 | 6,688 | 5,497 | 6,890 |
| AZ | 71 | 26 | 1,240 | 59 | 25 | 1,240 |
| CA | 131,682 | 8,667 | 14,809 | 128,613 | 8,581 | 14,811 |
| CO | 53,638 | 10,777 | 25,966 | 46,366 | 9,335 | 24,353 |
| FL | 367 | - | 714 | 361 | - | 714 |
| GA | - | - | 3 | - | - | 3 |
| IA | - | - | 90 | - | - | 90 |
| ID | - | 2 | 85 | - | 2 | 85 |
| IL | 104 | 4 | 193,689 | 1 | - | 3 |
| IN | 2 | - | 2 | 2 | - | 2 |
| KS | 126,848 | 36,650 | 110,094 | 125,736 | 34,933 | 110,464 |
| KY | 37,178 | 24,700 | 46,648 | 33,565 | 23,078 | 46,670 |
| LA | 95,283 | 34,253 | 73,050 | 92,327 | 32,711 | 72,758 |
| MD | - | 11 | 33 | - | 11 | 33 |
| MI | 14,810 | 17,261 | 23,837 | 14,140 | 14,052 | 22,947 |
| MN | - | - | 1 | - | - | 1 |
| MO | 1,889 | 1,357 | 3,016 | 1,889 | 1,357 | 3,016 |
| MS | 10,555 | 2,838 | 7,103 | 10,253 | 2,745 | 7,097 |
| MT | 13,000 | 7,692 | 16,758 | 11,906 | 7,542 | 16,714 |
| NC | - | - | 123 | - | - | 123 |
| ND | 11,050 | 133 | 8,137 | 10,478 | 128 | 8,097 |
| NE | 6,000 | 439 | 13,369 | 5,910 | 375 | 13,329 |
| NM | 44,455 | 16,602 | 551 | 42,399 | 15,676 | 670 |
| NV | 774 | 1 | 145 | 325 | 2 | 585 |
| NY | 4,503 | 3,690 | 3,662 | 4,273 | 3,355 | 3,523 |
| OH | 30,698 | 13,782 | 78,682 | 27,785 | 12,746 | 78,729 |
| OK | 195,681 | 48,780 | 173,981 | 182,584 | 45,060 | 184,265 |
| OR | 1 | 98 | 247 | 1 | 99 | 245 |
| PA | 28,551 | 23,827 | 4,034 | 28,764 | 24,467 | 4,025 |
| SD | 601 | 289 | 964 | 850 | 361 | 613 |
| TN | 3,699 | 2,004 | 8,327 | 3,609 | 1,970 | 8,177 |
| TX | 642,198 | 142,347 | 160,766 | 618,634 | 137,256 | 159,760 |
| UT | 7,453 | 4,563 | 5,261 | 8,580 | 4,982 | 5,205 |
| VA | 353 | 4,992 | 170 | 351 | 4,855 | 169 |
| WA | - | - | 525 | - | - | 519 |
| WV | 14,181 | 30,194 | 15,430 | 13,610 | 28,133 | 15,010 |
| WY | 25,678 | 36,583 | 34,630 | 24,190 | 35,831 | 34,634 |
| TOTAL | 1,514,569 | 485,047 | 1,036,196 | 1,449,951 | 461,733 | 848,658 |

## Historical Oil and Gas Commission Websites

Wells abandoned after 1975 are assumed to be fully characterized in the Enverus dataset. While Enverus also contains data on abandoned wells prior to 1975, this data is not considered to be complete. Therefore, historical datasets were evaluated under the GHGI program to account for old wells with installation and abandonment pre-dating Enverus data coverage (e.g., wells that were drilled in the 1800’s). Based on an analysis of oil and gas well counts from individual U.S. State databases and the U.S. Geological Survey (USGS), the historical number of abandoned wells not captured by Enverus (for the 2023 inventory) were estimated to equal approximately 1.1 million wells as of 1975. These wells are assumed to be unplugged. Additional details on the methodology used to develop these historical abandoned well counts are available in the memorandum “Inventory of U.S. Greenhouse Gas Emissions and Sinks 1990-2020: Updates for Abandoned Oil and Gas Wells” (April 2022).

The state level historical 1975 abandoned well estimates used in the 2023 and 2022 emission inventories are presented in Table 2. As shown in Table 2, there are fewer historical/1975 abandoned wells used in the 2023 inventory than in the 2022 inventory. This indicates more historical/pre-1975 wells were found in the Enverus database during preparation of the 2023 inventory than were found preparing the 2022 inventory. This is also reflected in Table 1 above, which shows more abandoned wells in Enverus in 2023 than were identified in 2023. In particular, the 2023 Enverus data included nearly 200,000 abandoned wells in Illinois that were not present in the 2022 data.

Table 2. 1975 Historical Abandoned Well Counts

| State | 2023 Historical Abandoned Well Counts |  | 2022 Historical Abandoned Well Counts |  |
| --- | --- | --- | --- | --- |
|  | Oil Wells | Gas Wells | Oil Wells | Gas Wells |
| AR | 10,375 | 1,122 | 10,376 | 1,122 |
| CA | 32,623 | 1,005 | 32,621 | 1,005 |
| CO | 0 | 0 | 686 | 254 |
| FL | 2 | 0 | 2 | 0 |
| IL | 0 | 0 | 82,914 | 1,565 |
| IN | 37,391 | 13,558 | 37,391 | 13,558 |
| KS | 14,212 | 2,594 | 14,484 | 2,644 |
| KY | 26,047 | 5,107 | 26,035 | 5,105 |
| LA | 22,018 | 5,112 | 22,198 | 5,154 |
| MS | 7,620 | 570 | 7,620 | 570 |
| MT | 486 | 107 | 453 | 100 |
| NM | 2,121 | 873 | 1,853 | 763 |
| OH | 53,662 | 12,968 | 53,665 | 12,968 |
| OK | 7,591 | 876 | 7,211 | 832 |
| PA | 263,765 | 44,777 | 263,733 | 44,771 |
| TX | 409,471 | 72,937 | 410,590 | 73,137 |
| WV | 3,233 | 3,586 | 3,219 | 3,572 |
| Total | 890,617 | 165,192 | 975,051 | 167,120 |

To develop a complete estimate of abandoned wells, the 1975 historical abandoned well counts were added to the counts of abandoned wells from Enverus. However, only state totals are available for this historical data. Therefore, the state level abandoned well counts presented in Table 2 were disaggregated to the county level. For all states but IN, the historical 1975 state level abandoned oil and gas well counts were disaggregated to the county level using Enverus ratios of abandoned oil and gas wells in each county. Enverus only identified four abandoned wells in IN, which was insufficient to employ the same disaggregation methodology used for the rest of the U.S. Therefore, the historical 1975 state level abandoned oil and gas well counts for IN were disaggregated to the county level using ratios of producing oil and gas wells in each county from EPA’s Nonpoint Oil and Gas Emission Estimation Tool.

## Abandoned Well CH4 Emission Factors from the GHGI

Based on a review of existing literature and studies, region-specific emission factors (EFs) for plugged and unplugged abandoned wells were developed under the GHGI. Wells that are plugged have much lower average emissions than wells that are unplugged, with around 40 percent of abandoned wells in the U.S. being plugged. The fraction of abandoned wells that are plugged has increased over time as more wells fall under regulations and programs requiring or promoting plugging of abandoned wells. As a result of this literature review, two sets of CH4 emission factors were developed – one set for the Appalachia region and one set for the rest of the U.S. Each set of emission factors contains one factor for plugged wells, and one factor for unplugged wells.

Table 3 provides the two sets of CH4 emission factors used in the GHGI.

Table 3. Abandoned Well Emission Factors

| Well Category | EF (g CH4/h/well) |
| --- | --- |
| Plugged wells (non-Appalachia) | 0.002 |
| Unplugged wells (non-Appalachia) | 10.02 |
| Plugged wells (Appalachia) | 0.357 |
| Unplugged wells (Appalachia) | 30.57 |

## EPA’s Nonpoint Oil and Gas Emission Estimation Tool Data

As described in Section 2.2, producing well counts in EPA’s Nonpoint Oil and Gas Emission Estimation Tool were used to disaggregate the historical 1975 well counts for IN to the county level.

CH4 emission factors for abandoned wells used in the GHGI are used in this analysis to form the basis of the VOC and HAP emission estimates. In order to develop VOC and HAP estimates, ratios of CH4 to the pollutant of interest (VOC or HAP) are needed. This information is available in EPA’s Nonpoint Oil and Gas Emission Estimation Tool for several source categories for VOC and the HAPs Benzene, Toluene, Ethylbenzene, and Xylenes (BTEX). No speciation data is available specific to abandoned well emissions, therefore, speciation data for oil and gas well completions were used as a surrogate to estimate VOC and BTEX emissions.

# EMISSIONS INVENTORY

VOC and HAP emissions from abandoned wells are calculated as follows:

- Step 1 - develop county-level populations of plugged and unplugged abandoned oil, gas, and dry wells using the Enverus and historical 1975 abandoned well datasets described in Sections 2.1 and 2.2;

- Step 2 - multiply the county-level abandoned well counts by the CH4 emission factors described in Section 2.3; and

- Step 3 - apply the CH4 to VOC and CH4 to BTEX pollutant ratios from EPA’s Nonpoint Oil and Gas Emission Estimation Tool described in Section 2.4. Gas well speciation is currently used to estimate emissions from “dry wells” as categorized in the Enverus dataset.

Appendix A (Abandoned_Wells_Calculations_2023.xlsx) contains the county-level VOC and HAP emission calculations for 2023, including the detailed analysis of well counts and speciation data using the data sources discussed above.

Table 4 shows the estimated VOC and BTEX emissions from abandoned wells at the state level for 2023, while Table 5 shows the estimated VOC and BTEX emissions from abandoned wells at the state level for 2022.

Table 4. 2023 Abandoned Wells Inventory

| State | VOC (tpy) | Benzene (tpy) | Ethylbenzene (tpy) | Toluene (tpy) | Xylenes(tpy) |
| --- | --- | --- | --- | --- | --- |
| AK | 82 | 3.16E-01 | 3.79E-02 | 2.73E-01 | 8.21E-02 |
| AL | 169 | 3.48E-01 | 9.18E-02 | 1.58E-01 | 1.02E-01 |
| AR | 795 | 1.29E+01 | 4.05E+00 | 5.82E+00 | 3.24E+00 |
| AZ | 19 | 3.11E-02 | 1.12E-03 | 2.47E-02 | 1.04E-02 |
| CA | 5,009 | 5.26E+01 | 6.34E+01 | 6.85E+01 | 8.22E+01 |
| CO | 455 | 1.16E+00 | 6.87E-02 | 1.42E+00 | 4.64E-01 |
| FL | 6 | 1.14E-02 | 3.23E-03 | 3.98E-03 | 4.71E-03 |
| GA | 0 | 7.74E-05 | 2.33E-06 | 6.73E-05 | 2.04E-05 |
| IA | 0 | 2.62E-05 | 8.60E-07 | 2.27E-05 | 6.91E-06 |
| ID | 0 | 5.17E-04 | 1.55E-05 | 4.49E-04 | 1.36E-04 |
| IL | 2,023 | 3.69E+00 | 1.11E-01 | 3.21E+00 | 1.16E-01 |
| IN | 3,215 | 2.68E+00 | 2.39E-01 | 4.03E-01 | 2.45E+00 |
| KS | 5,244 | 6.49E+00 | 2.15E+00 | 4.51E+00 | 2.85E+00 |
| KY | 13,490 | 1.20E+01 | 9.87E-01 | 2.82E+00 | 1.03E+01 |
| LA | 3,210 | 2.52E+01 | 7.55E+00 | 1.01E+01 | 6.17E+00 |
| MD | 1 | 1.08E-03 | 3.26E-05 | 9.42E-04 | 2.86E-04 |
| MI | 570 | 6.57E-01 | 3.88E-02 | 3.39E-01 | 3.92E-01 |
| MN | 0 | 2.58E-05 | 7.76E-07 | 2.24E-05 | 6.80E-06 |
| MO | 118 | 1.55E-01 | 7.99E-02 | 2.91E-02 | 4.27E-02 |
| MS | 743 | 1.93E+00 | 6.51E-01 | 7.93E-01 | 4.96E-01 |
| MT | 734 | 6.79E-01 | 5.27E-02 | 1.05E-01 | 5.11E-01 |
| NC | 0 | 6.34E-07 | 1.90E-08 | 5.51E-07 | 1.67E-07 |
| ND | 326 | 1.70E-01 | 1.37E-02 | 4.24E-02 | 1.44E-01 |
| NE | 138 | 7.23E-01 | 4.32E-02 | 9.02E-01 | 2.90E-01 |
| NM | 356 | 4.80E-03 | 2.32E-04 | 3.11E-03 | 2.06E-03 |
| NV | 64 | 5.08E-02 | 4.80E-03 | 4.27E-03 | 5.09E-02 |
| NY | 630 | 6.21E-01 | 4.49E-02 | 2.20E-01 | 4.64E-01 |
| OH | 23,051 | 2.06E+01 | 1.68E+00 | 4.91E+00 | 1.76E+01 |
| OK | 8,567 | 1.03E+01 | 4.04E+00 | 3.50E+00 | 3.06E+00 |
| OR | 3 | 6.38E-03 | 1.92E-04 | 5.54E-03 | 1.68E-03 |
| PA | 68,458 | 5.60E+01 | 5.25E+00 | 5.24E+00 | 5.56E+01 |
| SD | 33 | 2.35E-02 | 1.75E-03 | 3.96E-03 | 1.72E-02 |
| TN | 1,359 | 1.52E+00 | 9.78E-02 | 7.28E-01 | 9.47E-01 |
| TX | 31,087 | 5.59E+01 | 1.95E+01 | 2.17E+01 | 1.47E+01 |
| UT | 132 | 3.86E-01 | 2.44E-02 | 4.37E-01 | 1.90E-01 |
| VA | 58 | 9.64E-02 | 3.37E-03 | 7.81E-02 | 3.08E-02 |
| WA | 3 | 5.32E-03 | 1.60E-04 | 4.62E-03 | 1.40E-03 |
| WV | 3,160 | 3.28E+00 | 2.22E-01 | 1.35E+00 | 2.28E+00 |
| WY | 526 | 2.51E+00 | 1.73E-01 | 7.08E-01 | 1.02E+00 |
| Total: | 173,836 | 273 | 111 | 138 | 206 |

Table 5. 2022 Abandoned Wells Inventory

| State | VOC (tpy) | Benzene (tpy) | Ethylbenzene (tpy) | Toluene (tpy) | Xylenes(tpy) |
| --- | --- | --- | --- | --- | --- |
| AK | 64 | 2.46E-01 | 2.95E-02 | 2.13E-01 | 6.37E-02 |
| AL | 198 | 4.07E-01 | 1.03E-01 | 1.96E-01 | 1.19E-01 |
| AR | 794 | 1.29E+01 | 4.05E+00 | 5.81E+00 | 3.24E+00 |
| AZ | 10 | 1.49E-02 | 5.76E-04 | 1.14E-02 | 5.39E-03 |
| CA | 5,357 | 5.57E+01 | 6.72E+01 | 7.25E+01 | 8.71E+01 |
| CO | 451 | 1.16E+00 | 6.66E-02 | 1.39E+00 | 4.52E-01 |
| FL | 32 | 4.44E-02 | 8.48E-03 | 1.77E-02 | 2.21E-02 |
| GA | 4.23E-02 | 7.74E-05 | 2.33E-06 | 6.73E-05 | 2.04E-05 |
| IA | 1.44E-02 | 2.62E-05 | 8.60E-07 | 2.27E-05 | 6.91E-06 |
| ID | 2.82E-01 | 5.17E-04 | 1.55E-05 | 4.49E-04 | 1.36E-04 |
| IL | 6,739 | 5.52E+00 | 7.98E-01 | 5.66E-01 | 5.67E+00 |
| IN | 3,326 | 3.15E+00 | 6.34E-01 | 7.95E-01 | 2.92E+00 |
| KS | 6,663 | 6.17E+00 | 2.46E+00 | 2.89E+00 | 2.02E+00 |
| KY | 12,817 | 1.14E+01 | 9.37E-01 | 2.71E+00 | 9.74E+00 |
| LA | 3,195 | 2.43E+01 | 7.29E+00 | 9.74E+00 | 5.95E+00 |
| MD | 1 | 1.08E-03 | 3.26E-05 | 9.42E-04 | 2.86E-04 |
| MI | 487 | 5.48E-01 | 3.34E-02 | 2.70E-01 | 3.38E-01 |
| MN | 1.41E-02 | 2.58E-05 | 7.76E-07 | 2.24E-05 | 6.80E-06 |
| MO | 118 | 1.55E-01 | 7.99E-02 | 2.91E-02 | 4.27E-02 |
| MS | 749 | 1.94E+00 | 6.56E-01 | 7.97E-01 | 5.00E-01 |
| MT | 740 | 9.00E-01 | 6.15E-02 | 1.34E-01 | 5.60E-01 |
| NC | 3.46E-04 | 6.34E-07 | 1.90E-08 | 5.51E-07 | 1.67E-07 |
| ND | 401 | 2.04E-01 | 1.69E-02 | 4.54E-02 | 1.78E-01 |
| NE | 141 | 7.20E-01 | 4.31E-02 | 9.00E-01 | 2.89E-01 |
| NM | 348 | 6.17E-03 | 2.73E-04 | 4.29E-03 | 2.35E-03 |
| NV | 34 | 3.42E-02 | 2.40E-03 | 1.30E-02 | 2.47E-02 |
| NY | 596 | 5.81E-01 | 4.26E-02 | 1.99E-01 | 4.41E-01 |
| OH | 22,286 | 2.00E+01 | 1.63E+00 | 4.82E+00 | 1.70E+01 |
| OK | 8,944 | 1.08E+01 | 4.25E+00 | 3.49E+00 | 3.23E+00 |
| OR | 3 | 6.35E-03 | 1.91E-04 | 5.52E-03 | 1.67E-03 |
| PA | 69,730 | 5.72E+01 | 5.35E+00 | 5.42E+00 | 5.67E+01 |
| SD | 31 | 2.27E-02 | 1.67E-03 | 4.01E-03 | 1.63E-02 |
| TN | 1,329 | 1.49E+00 | 9.57E-02 | 7.15E-01 | 9.26E-01 |
| TX | 31,588 | 5.72E+01 | 2.00E+01 | 2.23E+01 | 1.51E+01 |
| UT | 178 | 4.91E-01 | 3.15E-02 | 5.59E-01 | 2.48E-01 |
| VA | 69 | 1.17E-01 | 3.97E-03 | 9.55E-02 | 3.61E-02 |
| WA | 3 | 5.22E-03 | 1.57E-04 | 4.53E-03 | 1.37E-03 |
| WV | 2,723 | 2.80E+00 | 1.92E-01 | 1.12E+00 | 1.97E+00 |
| WY | 552 | 2.81E+00 | 1.87E-01 | 7.06E-01 | 1.08E+00 |
| Total: | 180,695 | 279 | 116 | 138 | 216 |

# FUTURE IMPROVEMENTS

The VOC and HAP abandoned well emissions inventory presented in this memorandum reflects the results of an analysis of existing and readily available information currently used by EPA to develop the GHGI for abandoned wells. Additional information, as described above, was then used to develop VOC and HAP emission estimates based on the GHGI inventory and methodology. Additional information and analysis that would allow for additional refinement and improvement of the VOC and HAP abandoned well inventory include:

- Distinction of coal-bed methane (CBM) abandoned wells from the gas well category;

- Resolution of the Enverus “dry well” abandoned well counts into oil, gas, and CBM wells; and

- Improved speciation profiles for abandoned well emissions by well type (oil, gas, CBM), including CH4, VOC, and HAP composition data.

Appendix A – Abandoned Well VOC and HAP Emissions Calculations

(See Abandoned_Wells_Calculations_2023.xlsx)

Memorandum “Abandoned Well Emission Estimates for VOC and HAP” from Mike Pring, Gopi Manne, and Regi Oommen (ERG) to Jeffrey Vukovich and Carolina Farkas (EPA), April 19, 2023.

Memorandum “2022 Abandoned Well Emission Estimates for VOC and HAP” from Mike Pring and Regi Oommen (ERG) to Jeffrey Vukovich, Alison Eyth, and Janice Godfrey (EPA), March 21, 2024.

https://www.enverus.com/

2023 Nonpoint Oil and Gas Emission Estimation Tool Version 1.1, April, 2025.

“Inventory of U.S. Greenhouse Gas Emissions and Sinks 1990-2020: Updates for Abandoned Oil and Gas Wells”, April 2022, available online at: https://www.epa.gov/system/files/documents/2022-04/2022_ghgi_update_-_wells.pdf

“Inventory of U.S. Greenhouse Gas Emissions and Sinks 1990-2020: Updates for Abandoned Oil and Gas Wells”, April 2022, available online at: https://www.epa.gov/system/files/documents/2022-04/2022_ghgi_update_-_wells.pdf

“Inventory of U.S. Greenhouse Gas Emissions and Sinks 1990-2016: Abandoned Oil and Gas Wells”, April 2018, Available online at: https://www.epa.gov/sites/default/files/2018-04/documents/ghgemissions_abandoned_wells.pdf