Promoting Global Consistency in Estimating Greenhouse Gas Emissions from Oil and Gas Industry Operations

Karin Ritter

American Petroleum Institute, 1220 L St NW, Washington, DC 20005

ritterk@api.org

Susann Nordrum

ChevronTexaco, 100 Chevron Way, Richmond, CA 94802-0627

SBNordrum@chevrontexaco.com

Terri Shires

URS Corporation, 9400 Amberglen Blvd., Austin, Texas 78729

terri_shires@urscorp.com

# ABSTRACT

Estimating and reporting greenhouse gas emissions is extremely complex for the operationally and geographically diverse oil and gas industry.  Companies are further challenged by the fact that greenhouse gas emission estimates can vary significantly depending on the approach used to calculate the emissions and the methodologies governing the accounting and reporting of emission inventories.

Since first releasing the Compendium of Greenhouse Gas Emissions Estimation Methodologies for the Oil and Gas Industry, the American Petroleum Industry (API) has engaged in a number of activities to promote harmonization of estimation methodologies and meaningful comparisons of reported emissions.  A revised version of the Compendium is planned for early 2004, incorporating modifications and recommendations from a two-year “pilot test” of the original document.  Timing of the revised Compendium also corresponds to the first release of a petroleum industry guidance document that addresses the implementation aspects of a corporate greenhouse gas inventory.  In addition, ChevronTexaco has made available, free of charge, their proprietary software, the SANGEA™ Energy and Emissions Estimating System, to enable more companies to develop inventories that are consistent with the industry-wide Guidelines and consistently apply the methodologies from the API Compendium.

This paper will address the global collaboration among API and other industry organizations in revising the Compendium and in developing Petroleum Industry Guidelines for Reporting Greenhouse Gas Emissions.  The paper will also present a case study applying the SANGEA™ System to create an inventory that is consistent with the Guidelines and employs the API Compendium methodologies.

# INTRODUCTION to a SUITE OF Inventory Tools

Understanding the magnitude and sources of greenhouse gas emissions is a critical first step to managing emissions.  In addition, with increased attention focussed on the potential value – and risk – associated with greenhouse gas emissions, consistent, standardized methodologies for estimating greenhouse gas emissions is crucial to making informed decisions and to tracking progress toward reduction targets.

The worldwide petroleum industry has voluntarily undertaken a significant effort to improve consistency in estimating emissions.  The American Petroleum Institute (API) developed the Compendium of Greenhouse Gas Emissions Estimation Methodologies for the Oil and Gas Industry to provide a current and complete compilation of emission estimation techniques.  Soon after distributing the Pilot Test version of the Compendium1, a second project was initiated to develop consistent guidelines for reporting greenhouse gas emissions from petroleum industry operations.  The International Petroleum Industry Environmental Conservation Association (IPIECA), International Association of Oil and Gas Producers (OGP) and API collaborated to develop the Petroleum Industry Guidelines for Reporting Greenhouse Gas Emissions.2  Finally, an estimating and reporting tool, the SANGEA Energy and Emissions Estimating System, is now available to enable companies to develop an inventory that is consistent with the Guidelines and utilizes methodologies from the Compendium.3  Each of these initiatives were conducted in an effort to better harmonize greenhouse gas emissions estimation and enable improved global comparability of emission estimates.

Each of these inventory tools: the Compendium, the Guidelines, and the SANGEA System, are available at http://ghg.api.org.  The Guidelines are also available at www.ipieca.org/reporting/ghg.html and www.ogp.org.uk.

## Guidelines for Reporting

IPIECA, OGP, and API jointly initiated the development of industry guidelines to promote credible, consistent, and reliable greenhouse gas accounting and reporting practices from oil and gas operations.  The Petroleum Industry Guidelines for Reporting Greenhouse Gas Emissions (the Guidelines) were issued in December 2003.  These guidelines build upon other existing protocols for reporting greenhouse gas emissions by providing information to address the unique operational arrangements of the petroleum industry.  For example, the Guidelines provide:

- Industry-specific definitions and examples for 'operated' and 'equity share';

- Specific guidance on which methodologies from the API Compendium are most appropriate for various groups of emission sources and desired levels of accuracy;

- Guidance on data aggregation within the petroleum industry subsectors (e.g., production and refining);

- Guidance on inclusion of indirect emission sources; and

- Recommendations for allocating emissions from combined heat and power plants.

The Guidelines were developed with broad participation by petroleum industry representatives. The focus is on the accounting of emissions at the facility level and the aggregation and reporting of results at the corporate level.  The objective is to provide guidance rather than prescribe standards and to complement the Compendium, which details the emission estimation approaches.

## Compendium of Methodologies

API recently released an updated version of the Compendium of Greenhouse Gas Emissions Estimation Methodologies for the Oil and Gas Industry (referred to as the API Compendium).  The revised API Compendium is the result of collaboration with other industry-related protocol development organizations and represents industry best practices for estimating greenhouse gas emissions for the petroleum industry.

The Compendium is neither a standard nor a recommended practice for the development of GHG inventories.  Rather, it represents a compilation of recognized methodologies for estimating GHG emissions specific to oil and gas industry operations.  The API Compendium presents and illustrates the use of preferred and alternative calculation approaches for CO2, CH4, and N2O emissions for all common sources, including combustion, vented, fugitive, and indirect sources.  Decision trees are provided to guide the user in selecting an estimation technique based on considerations of materiality, data availability, and accuracy.

## SANGEA System

With interest in global climate change rising, many companies have recognized the need to track greenhouse gas emissions.  For an integrated petroleum company, or any company with complex operating arrangements, the task of developing an inventory can be overwhelming.  ChevronTexaco’s goal of having a consistent and verifiable system for tracking emissions led to the development of the SANGEA™ Energy and Emissions Estimating System.

The SANGEA System is an Excel-based program with a Visual Basic add-in containing the calculations, emission factors and macros to help users set up their files.  The software tool uses methodologies from the API Compendium and enables users to develop an inventory that is consistent with the Guidelines.  ChevronTexaco has donated the SANGEA software to API, and it is available free of charge to industry.

# Developing An Inventory

Development of a greenhouse gas inventory could be viewed as an initial step in formulating a corporation’s climate change strategy.  While the objectives may vary among companies, inventories are expected to share some common attributes, as presented in the Industry Guidelines:

- Relevance – boundaries are defined in a manner that appropriately reflects the greenhouse gas emissions of the organization and the decision-making needs of the users.

- Completeness – all greenhouse gas sources and activities are accounted for within the chosen organizational and operations boundaries.  Exclusions are identified and explained.

- Consistency – methodologies and measurements are applied consistently to allow meaningful comparison of emissions over time.  Changes to data, methods, or factors are documented.

- Transparency – All relevant issues are addressed in a factual and coherent manner, based on a clear audit trail.  Assumptions are disclosed and appropriate references for calculation methodologies and data sources are provided.

- Accuracy – greenhouse gas estimates are “bias free”, as far as can be judged, and uncertainties are quantified and reduced as far as practicable.  The accuracy achieved should be sufficient for users to make decisions with reasonable assurance as to the integrity of the reported information.

The application of these principles is greatly simplified for the petroleum industry now that specific guidance is available through the Industry Guidelines and the API Compendium.  The following sections describe the application of this guidance for developing a greenhouse gas inventory.  SANGEA data fields and results are also illustrated for an example facility.

## Application of the Guidelines

In planning the consolidation of greenhouse gas information into an inventory, it is important to understand the distinction between the “accounting for” and “reporting of” emissions.  Greenhouse gas accounting addresses the recognition and consolidation of greenhouse gas emissions from operations in which a parent Company holds an interest, and linking the data to specific operations, sites, geographic locations, business processes and owners.  Greenhouse gas reporting deals with the presentation of greenhouse gas data in formats tailored to the needs of various reporting uses.  The Guidelines address the key accounting and reporting considerations for the petroleum industry sector.

Petroleum industry operations are commonly conducted by two of more parties working together in joint ventures.  The accounting of greenhouse gas emissions from joint ventures may be performed on the basis of equity share or on the basis of operational control.  Because companies may need to provide information for more than one purpose, the Guidelines encourage reporters to develop an inventory that can be used to report both operated facilities and the company's equity share of emissions.  Building this flexibility into the design of the accounting and reporting system will enable companies to more easily report to existing and emerging programs.

According to the Guidelines, a company is deemed to have operational control of a facility when: The company has authority to introduce and implement its operational and environmental, health, and safety (EHS) policies at the joint venture.  Under the operational control approach, a company reports 100 percent of the emissions from joint ventures over which it has operational control and none of the emissions from joint ventures it does not control.

The Guidelines also provide specific examples for calculating equity share.  In general, greenhouse gas emissions are apportioned according to the economic interest or benefit derived from a joint venture.  This general rule is applied unless there are specific contractual arrangements that alter the normal practice of allocating benefits in proportion to the contribution of costs.  One type of arrangement that alters the allocation of benefits is a Production Sharing Agreement (PSA).  A PSA is an agreement between one or more participating oil companies to provide financing and bear the risk of exploration and production activities in exchange for a share of the production remaining after royalties are paid to the government.  All of the parties receiving a share of the net production, whether they are a state oil company or private companies, receive a proportionate share of emissions.  No emissions are allocated to the royalties.

The next key consideration in developing an inventory is how to characterize direct and indirect emissions from a facility.  Direct emissions are defined as those from sources that are owned or controlled by the reporting company, such as emissions from exhaust stacks or process vents.  Indirect emissions are generally considered to be emissions that are a consequence of the activities of the reporting company, but occur from sources owned or controlled by another party.  For example, indirect emissions could include emissions from purchased energy (electricity, heat and steam); contract manufacturing; contracted drilling operations; and product transport by third parties.  The Guidelines provide a rationale for accounting and reporting indirect emissions from energy consumption.  An example for the petroleum industry is the accounting for power plants located within a refinery that export electricity and/or steam to other companies.  Table 1 illustrates how this accounting should be accomplished for three different ownership and operational scenarios.

Another unique situation for the petroleum industry is contract operations.  For example, in exploration and production operations a petroleum company may supply fuel to a contract operator responsible for well drilling and maintenance activities.  The Guidelines suggest that significant contract operations should be accounted for as indirect emissions and that these emissions should be reported separately from both direct emissions and indirect emissions from the consumption of purchased energy.  This is consistent with Scope 3 reporting under the World Resources Institute/World Business Council for Sustainable Development (WRI/WBCSD) greenhouse gas protocol.5

Once decisions regarding inventory boundaries and reporting needs have been made, the next step is to understand the specific emission sources associated with each facility, and to understand what methodology is appropriate for estimating the emissions.  The Guidelines provide an overview of the types of methods that are appropriate for various levels of accuracy using a tiered approach.  Tier A represents the highest level of accuracy, with an uncertainty of 10 to 30%.  Tier B includes somewhat less rigorous methods, and is therefore likely to be more straightforward and less costly to implement.  However, the uncertainty associated with these methods is about 20 to 40%.  Finally, for an assessment of emissions where uncertainties of 30-60% are acceptable, Tier C methods may be employed.  As an example, if combustion emissions from an upstream operation are to be estimated to the highest degree of accuracy (Tier A), the quantities of fuels consumed should be based on measurements/metering and/or fuel purchase records.  Fuel composition should be measured at appropriate intervals to meet the needed accuracy.  The second-tier methodology for fuel combustion recommends that the quantity of fuel consumed may be based on equipment ratings.  Less frequent measurements of gas composition are required.

Details on the types of approaches available for estimating emissions and example calculations are found in the API Compendium.  For a given facility, and a given range of uncertainty, the specific method used for each source will depend on the information available about that source.  For example, the API Compendium recommends that the preferred method for estimating emissions from combustion sources is to determine the carbon fraction of the fuel and the quantity of fuel used.  However, the Guidelines and the API Compendium recognize that this type of information is not available for all devices in all facilities.  In cases where the preferred information is not available, the API Compendium provides alternative methodologies that utilize existing information to develop a reasonable estimate of emissions.  Tables 2 and 3 cross reference the calculation tiers presented in the Guidelines to the detailed methodologies provided in the Compendium for upstream and downstream petroleum operations respectively.

## Emission Estimation Methodologies

As described above, the Guidelines provide information to establish the boundaries of an inventory.  A companion publication, the Compendium of Greenhouse Gas Emissions Estimation Methodologies for the Oil and Gas Industry documents a number of currently recognized calculation techniques and emission factors available for developing greenhouse gas emissions inventories for oil and gas industry operations.  The Compendium was developed to accomplish the following:

- Assemble an expanse of relevant emission factors for estimating greenhouse gas emissions from oil and gas industry activities, based on currently available public documents;

- Outline detailed procedures for conversions between different measurement unit systems, with particular emphasis on implementation of oil and gas industry best practices;

- Provide descriptions of the multitude of oil and gas industry operations—from exploration and production through refining to the marketing of products, as well as the transportation of crude oil, natural gas and petroleum products—and the associated emissions sources that should be considered;

- Develop emission inventory examples—based on selected facilities from the various industry segments—to demonstrate the broad applicability of the methodologies; and

With the interest in addressing climate change issues gaining momentum, API members have continued to refine and promote a common global methodology for estimating GHG emissions within the industry.  A revised version of the API Compendium was published in February 2004, incorporating the lessons learned from several API special studies, as well as other protocol development organizations and petroleum industry groups worldwide.  Enhancements to the API Compendium include:

- Reorganization to enhance “user friendliness” and to be consistent with the recently released Guidelines discussed above.

- Emission factors reported in terms of International System of Units (SI), in addition to the existing unit convention, in order to increase the global applicability of the API Compendium.

- Factors for CO2 emissions from fuels presented both in terms of lower heating value (LHV), in addition to the existing higher heating value (HHV). This will increase awareness of the different conventions in various parts of the world and will discourage the use of inappropriate approaches.

- Updated electricity emission factors to address additional information sources and to include international electricity “grid” emission factors.

- Detailed source-specific emission factors for vented and fugitive emissions to accommodate emission reduction calculations.

- Expanded emission estimation approaches for other petroleum industry sources such as: geothermal steam and electricity production, produced water, drilling fluid, casing gas, pipeline pigging, and natural gas distribution systems.

- Results of special studies on refinery fugitive CH4 emissions and N2O emissions from combustion sources are incorporated. These studies aim to provide data for determining circumstances in which these emission sources may, or may not be, negligible.

## SANGEA(TM) Inventory Tool

In order to effectively manage greenhouse gas emission data within the boundaries established by the Guidelines and consistently applying the methodologies from the Compendium, an electronic data management tool is highly valuable.

The SANGEA system is a comprehensive energy and emissions management system that can be used to:

- Account for emissions on both an Operated and Equity Share basis;

- Account for and report direct and indirect emissions separately;

- Assess energy utilization and greenhouse gas emissions to identify and prioritize major sources;

- Guide energy and greenhouse gas emissions management activities by enabling comparisons of emissions per barrel for similar activities or similar fields;

- Forecast emissions, both for business as usual and for new energy efficiency projects;

- Set goals for improving energy efficiency and decreasing greenhouse gas emissions;

- Track progress toward interim and final goals;

- Document progress against a baseline for potential future crediting; and

- Provide a basis for discussions with regulators and other stakeholders about the various parameters that affect energy utilization and greenhouse gas emissions from a mature oilfield.

The SANGEA system is modular so that it can be used to estimate energy utilization and greenhouse gas emissions from all types of petroleum industry sources, including combustion devices, flaring, venting, fugitives, and indirect emissions associated with energy purchases.  Users configure the system for their site by first entering information about the operational status and equity ownership for each location in the reporting entity.  The next step is to select the applicable modules, such as the combustion module and then to specify the sources within each module (e.g., turbines, engines and boilers).

The SANGEA™ software allows users to have flexibility in setting up the inventory.  For example, the level of aggregation can be specified by the user – each engine or boiler can be listed separately, or if there is only a single meter for several combustion devices, the devices can be grouped together.  The SANGEA™ system allows the user to choose from a variety of SI and English units of measure for input and output data.  The SANGEA™ software includes default fuel composition information and default values for emission factors for all modules, based on the API Compendium.  If data are available, users can customize these input sources.

Once the SANGEA spreadsheet has been configured, data entry is straightforward.  The SANGEA spreadsheet has tables for monthly data entry for each source, and prompts the user to enter audit trail information for each data source.  The input tables can be linked to other data management or accounting systems, such as Excel, J.D. Edwards or SAP.  Reports are generated on a quarterly basis and pre-formatted charts are available for plotting emission results.

One of the goals in developing the SANGEA system was to ensure the information is verifiable.  The SANGEA system allows user to document the data source associated with each input value.  An automatic review of the input data and documentation fields is conducted prior to generating a facility’s quarterly report.  If necessary, the system will display an error message to guide the user to incomplete cells, so that the report can be completed.  The quarterly report includes a statement of assurance for both the inventory preparer and the facility manager, in order to provide certification that the inventory is complete and accurate, and that it was prepared by a competent, knowledgeable professional.  A “flat file”, suitable for loading into an Oracle or Access database is automatically generated along with the quarterly report.

As shown in Figures 1 and 2, the summary tables and charts generated by the SANGEA™ software allow the user to analyze the data in many different ways -- by location, by module or by species.  Because the SANGEA system uses Microsoft Excel as a basis, the tables can be copied and pasted to other Excel spreadsheets for customized data presentation formats.

# Next Steps

Estimating greenhouse gas emissions is a relatively new and evolving process.  As such, API has established a process for ongoing review and updates—at appropriate intervals—to incorporate regulatory changes, industry experience, scientific and technical developments, and stakeholder comments.  API’s goal is also to maintain consistency, where relevant to the petroleum sector, between the documents and nationally or internationally recognized greenhouse gas inventory standards and guidelines.  For example, API is active on the US Technical Advisory Group to the International Standards Organization (ISO) that is currently developing an international standard for GHG emissions estimation, reporting and verification (ISO 14064 Parts 1, 2, and 3), and it aims to maintain a consistent approach between the Industry Guidelines and the emerging ISO standards. Therefore, future revisions will be undertaken to address essential changes, while at the same time recognizing the need for continuity and the effort required in implementing such changes.

API welcomes a continuing exchange of information and broad discussion of greenhouse gas emission estimation methodologies from users of these tools and other interested parties.  Please direct comments to Karin Ritter, Regulatory and Scientific Affairs Department, at API.

# CONclusions

A standardized, systematic approach, as embodied in the Petroleum Industry Guidelines and API Compendium, is essential for cost-effectively managing greenhouse gas emissions.  Consistent methodologies lend credibility to the estimates and enable aggregation and comparison.  In addition, this industry guidance for calculating, reporting, and tracking emissions have spurred new initiatives to progress toward harmonization of methodologies and improved global compatibility of greenhouse gas emission estimates.

The vast amount of material in both the API Compendium and the Industry Guidelines should make it clear that:

- Companies face a range of options on how they account, estimate and report emissions from the facility to the corporate level.

- The specific approach selected by each company will depend, to a large extent, on the intended use of the information companies are reporting.

- The rules for reporting these emissions will ultimately be determined by the specific reporting program that may, or may not, correspond either with general industry practice for reporting corporate emissions, or the way the company that owns the facility conducts its corporate accounting and reporting.

# AcknowledgementS

The authors would like to acknowledge the support and contributions of the API, IPIECA and OGP member companies in the development of the API Compendium and the Industry Guidelines. Company experts on multiple working groups, drafting task forces and steering committees have contributed valuable time and resources throughout the development and outreach activities.  We are particularly indebted to the companies that shared internal company information and facilitated progress in the program activities.

# Keywords

Oil and gas industry

Petroleum industry

Greenhouse gas emissions

Protocol

Emission reductions

Emission inventory

Emissions reporting

# References

1. American Petroleum Institute (API). Compendium of Greenhouse Gas Emissions Estimation Methodologies for the Oil and Gas Industry; Pilot Test Version; API, Washington, DC, 2001.

2. IPIECA/OGP/API. Petroleum Industry Guidelines for Reporting Greenhouse Gas Emissions, IPIECA, London, United Kingdom, December 2003.

3. ChevronTexaco. “ChevronTexaco Greenhouse Gas Management System Now Available to Industry”, Press Release, San Ramon, California, February 10, 2003.

4. American Petroleum Institute (API). Compendium of Greenhouse Gas Emissions Estimation Methodologies for the Oil and Gas Industry; API, Washington, DC, 2004.

5. World Resources Institute and World Business Council for Sustainable Development. The Greenhouse Gas Protocol a corporate accounting and reporting standard, Washington, DC, 2001.

Table 1.  Accounting examples for power generation.

- A power plant is located at a refinery owned and operated by Company B, which also uses 100 percent of the power plant’s output:

Scenario 1 - The power plant is owned and operated by Company A: Company B (the refinery) has outsourced power generation and reports no direct GHG emissions for either the control or equity approach, but reports 100% of the plant’s emissions as indirect emissions. Company A reports 100% of the plant’s emissions as direct emissions.

Scenario 2 – The power plant is owned by Company A but operated by Company B (the refinery): Under the operational control approach, Company B reports 100% of emissions as direct emissions and none as indirect emissions. Under the equity share approach it reports none of the power plant’s emissions as direct emissions, but 100% of the emissions as indirect emissions.

Scenario 3 – The power plant is operated by Company A but it is owned by Company B (the refinery). Under the operational control approach Company B reports none of the emissions as direct emissions but 100% of the emissions as indirect emissions. Under the equity share approach, Company B reports 100% of the emissions as direct emissions (since it owns the power plant) but none of the emissions as indirect emissions.

- Table 2.  Upstream (exploration and production) petroleum industry tiers for estimation options.

| Source Category |  |  | Estimation Tiers |  |
| --- | --- | --- | --- | --- |
|  |  | Tier C | Tier B | Tier A |
|  | GHG |  | Estimation Approach |  |
|  |  |  | (increasing accuracy) |  |
| Combustion Sources | CO2 | Fuel consumption based on ratings, hours of operation and assumed loads for engines/turbines (energy balance for boilers/heaters); [Demonstrated in Compendium Exhibit 3-3] | Fuel consumption based on ratings, hours of operation and loads for engines/turbines (energy balance for boilers/heaters); [Demonstrated in Compendium Exhibit 3-3] | Fuel consumption based on single point metering and integrating mass flow for fuel gas, purchase records or tank measurements for commodity fuels (e.g., natural gas, diesel); [Compendium Section 4.2. |
|  |  | Or default emission factors; [Compendium Section 4.2 Demonstrated in Exhibits 4.3 and 4.5] | Or fuel emission factors (mass/mass or mass/heating value) based on default factors. [Compendium Section 4.2. Demonstrated in Exhibits 4.3 and 4.5 | Or fuel emission factors (mass/mass or mass/heating value) based on default factors. [Compendium Section 4.2. Demonstrated in Exhibits 4.3 and 4.5 ] |
|  |  |  |  | Or factors based on actual measurements of fuel composition if available [Compendium Section 4.1. Demonstrated in Exhibit 4.1] |
|  | CH4 | Not considered [CH4 emission factors are provided in Compendium Section 4.3. Calculations are demonstrated in Exhibit 4.7. Small contribution of CH4 from combustion sources is demonstrated in Compendium Section 7 |  |  |
| Flaring | CO2 | Quantity of gas flared based on available GOR measurements and quantity of oil produced. Local default CO2 emission factors applied [Compendium Section 4.4, Table 4-7. Demonstrated in Exhibit 4.8] | Quantity of gas flared based on periodic GOR measurement and quantity of oil produced. Gas composition measured at similar intervals, [Compendium Section 4.4. Demonstrated in Exhibit 4.8] | Quantity of gas flared based on measurements/metering and /or frequent GOR measurement and quantity of oil produced. Gas composition measured at similar intervals [Compendium Section 4.4. Demonstrated in Exhibit 4.8] |
| Flaring (continued) |  | Or factor based on gas composition if available Available [Compendium Section 4.4. Demonstrated in Compendium Exhibit 4.8] | Or local default CO2 emission factors applied. [Compendium Section 4.4, Table 4-7. Demonstrated in Exhibit 4.8] | Or local default CO2 emission factors applied. [Compendium Section 4.4, Table 4-7. Demonstrated in Exhibit 4.8] |
|  | CH4 | Calculated from information above, assumed or known CH4 fraction in flare gas [Compendium Section 4.4, Table 4-6] and default residual methane (flare efficiency) [Demonstrated in Compendium Exhibit 4.8] | Calculated from gas quantity information above, measured or assumed gas composition [Compendium Section 4.4, Table 4-6] and default residual CH4 (flare efficiency) [Demonstrated in Compendium Exhibit 4.8] | Calculated from gas quantity information above, measured or assumed gas composition [Compendium Section 4.4, Table 4-6] and default residual CH4 (flare efficiency) [Demonstrated in Compendium Exhibit 4.8] |
| Associated Gas Venting | CO2 | Include only for CO2-rich streams. Quantity of gas vented based on available GOR measurements and quantity of oil produced; assumed duration of flare outages for inadvertent venting. Local default [Compendium Table 4-6] or actual gas composition used as available. [Cold vent approach from Compendium Section 5.3. Demonstrated in Exhibit 5.12.] | Quantity of gas vented estimated (e.g., by periodic GOR measurement and quantity of oil produced; duration of flare outages for inadvertent flaring). Gas composition measured at similar intervals. [Cold vent approach from Compendium Section 5.3. Demonstrated in Exhibit 5.12.] | Quantity of gas vented estimated (e.g., by frequent GOR measurement and quantity of oil produced; duration of flare outages for inadvertent flaring). Gas composition measured at similar intervals. [Cold vent approach from Compendium Section 5.3. Demonstrated in Exhibit 5.12.] |
|  | CH4 | Quantity of gas vented based on available GOR measurements and quantity of oil produced; assumed duration of flare outages for inadvertent venting. Local default (composition) [Compendium Table 4-6] or actual gas composition used as available. [Cold vent approach from Compendium Section 5.3. Demonstrated in Exhibit 5.12.] | Quantity of gas vented estimated (e.g., by periodic GOR measurement and quantity of oil produced; duration of flare outages for inadvertent flaring). Gas composition measured at similar intervals. [Cold vent approach from Compendium Section 5.3. Demonstrated in Exhibit 5.12.] | Quantity of gas vented estimated (e.g., by frequent GOR measurement and quantity of oil produced; duration of flare outages for inadvertent flaring). Gas composition measured at similar intervals. [Cold vent approach from Compendium Section 5.3. Demonstrated in Exhibit 5.12.] |
| Acid Gas Removal | CO2 | Emissions based on quantity of gas produced and assumed residual CO2 content. Local or actual inlet gas composition used as available. [Compendium Section 5.1.4. Demonstrated in Exhibit 5.4.] | Results of process simulation, such as AmineCalc. [Compendium Section 5.1.4.] | Mass balance across amine unit (e.g., based on difference between inlet gas flow and CO2 fraction and outlet gas flow and CO2 fraction—measured parameters.) [Compendium Section 5.1.4. Demonstrated in Exhibit 5.4.] |
|  | CH4 | Not considered [Compendium Section 5.1.3. CH4 emissions estimated in Exhibit 5.3] | Application of generic emission factors or results from process simulation such as AmineCalc [Compendium Section 5.1.3, Table 5-4. Demonstrated in Exhibit 5.3] | Results of process simulation, such as AmineCalc [Compendium Section 5.1.3] |
| Glycol Dehydration | CO2 | Not considered [Compendium Section 5.1.1. CO2 emissions from glycol dehydrators calculated in Exhibits 5.1 and 5.2] |  |  |
|  | CH4 | Not considered [Compendium Section 5.1.1. CH4 emissions from glycol dehydrators calculated in Exhibits 5.1 and 5.2 | Application of generic emission factors [Compendium Section 5.1.1, Tables 5-1 to 5-3. Demonstrated in Exhibits 5.1 and 5.2.] | Application of generic emission factors [Compendium Section 5.1.1, Tables 5-1 to 5-3. Demonstrated in Exhibits 5.1 and 5.2.] |
| Tank Flashing | CO2 | Not considered [Tank flashing addressed in Compendium Section 5.4.1. CO2 emissions from tank flashing are estimated in Compendium Section 7.1. |  |  |
|  | CH4 | Not considered [Compendium Section 5.4.1. Exhibit 5.13 provides a comparison of results from different estimation techniques.] | Application of generic emission factors [Compendium Table 5-7] -or- emission estimation equations [Compendium Section 5.4.1. Exhibit 5.13 provides a comparison of results from different estimation techniques.] | Measurement of vent gas -or- application of process simulation such as E&P Tank [Compendium Section 5.4.1] |
| Other Process Sources | CO2 | Not considered [CO2 emissions from other process vents are addressed in Compendium Sections 5.3 (Cold Vents) and 5.6 (Other Venting Sources). Case studies provided in Compendium Section 7.1 show negligible CO2 emissions from other process sources for upstream facilities] | Process mass balance as in Compendium using activity data based on best engineering estimates [Compendium Sections 5.3 (Cold Vents) and 5.6 (Other Venting Sources). Demonstrated in Exhibit 5.12.] | Process mass balance as in Compendium using activity data based on best engineering estimates [Compendium Sections 5.3 (Cold Vents) and 5.6 (Other Venting Sources). Demonstrated in Exhibit 5.12.] |
|  | CH4 | Not considered [CH4 emissions from other process vents are addressed in Compendium Sections 5.3 (Cold Vents) and 5.6 (Other Venting Sources) Demonstrated in Exhibit 5.12, and exhibits for particular emission sources. |  |  |
| Non Routine Sources | CO2 | Not considered [CO2 emissions from non-routine sources are addressed in Compendium Section 5.7. Case studies provided in Compendium Section 7.1 show negligible CO2 emissions from non-routine sources for upstream facilities] | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibit 5.27.] | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibit 5.27.] |
|  | CH4 | Not considered [CH4 emissions from non-routine sources are addressed in Compendium Section 5.7. Case studies provided in Compendium Section 7.1 show negligible contribution to total CO2 Eq. emissions from non-routine sources for upstream facilities] | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibits 5.26 and 5.27.] | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibits 5.26 and 5.27.] |
| Process Fugitives | CO2 | Not considered [Addressed in Compendium Section 6.1. Case studies provided in Compendium Section 7.1 show negligible CO2 emissions from fugitive sources for upstream facilities] | Include only for streams that are >30% CO2 based on component level average emission factors and typical component counts [Compendium Section 6.1. Demonstrated in Exhibit 6.1.] | Include only for streams that are >30% CO2 based on component level average emission factors and actual component counts [Compendium Section 6.1. Demonstrated in Exhibit 6.1.] |
|  | CH4 | Not considered [Addressed in Compendium Section 6. Case studies provided in Compendium Section 7.1 show 0.1 to 6% contribution to total CH4 emissions from fugitive sources for upstream facilities] | Based on component level average emission factors and typical component counts [Compendium Section 6.1.3. Demonstrated in Exhibit 6.3.] | Based on component level average emission factors and actual component counts [Compendium Section 6.1.3. Demonstrated in Exhibit 6.3.] |
| Non-Operated Facilities | CO2 | If the operator is unwilling or unable to provide GHG emissions data or the activity data, E&P emissions may be estimated by prorating to the nearest equivalent company operated production facility. [Not addressed in the Compendium] |  |  |
|  | CH4 |  |  |  |

Table 3.  Downstream (refining) petroleum industry tiers for estimation options.

| Source Category |  |  |  |  |  |  | Estimation Tiers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Tier C |  |  | Tier B | Tier A |
|  | GHG |  |  |  |  |  | Estimation Approach |  |
|  |  |  |  |  |  |  | (increasing accuracy) |  |
| Combustion Sources | CO2 |  |  | Thermal input (fuel burnt) estimated based on design rating of plant and hours operated, default fuel factors [Compendium Section 4.2. Demonstrated in Exhibits 3.3, 4.3 and 4.5.] |  |  | Thermal input (fuel input) based on metering or energy balances on heaters/boilers, fuel composition obtained from occasional spot sampling . [Compendium Section 4.1. Demonstrated in Exhibit 4.1.] | Thermal input (fuel input) based on metering or energy balances on heaters/boilers, fuel composition obtained from frequent spot sampling . [Compendium Section 4.1. Demonstrated in Exhibit 4.1.] |
|  | CH4 |  |  | Not considered [CH4 emission factors provided in Compendium Section 4.3. Negligible contribution of CH4 emissions from combustion sources is demonstrated in Compendium Section 7.3, and Exhibit 4.6 and 4.7. Less than 1% of total CO2 Eq emissions for downstream sector facility.] |  |  |  |  |
| FCC Coke Burn | CO2 |  |  | Thermal input (fuel burnt) estimated based on design rating of plant and hours operated, default coke factor |  |  | Coke burn rate calculated based on process mass/energy balance and average coke composition based on spot samples . [Compendium Section 5.2.1. Demonstrated in Exhibit 5.5.] | Coke burn rate calculated based on process mass/energy balance and average coke composition based on spot samples [Compendium Section 5.2.1. Demonstrated in Exhibit 5.5.] |
|  |  |  |  |  |  |  | OR – estimated directly from measured CO and CO2 concentrations in exhaust (spot samples) and air/oxygen flow rate to regenerator [Compendium Section 5.2.1. Demonstrated in Exhibit 5.5.] | OR – estimated directly from measured CO and CO2 concentrations in exhaust (spot samples) and air/oxygen flow rate to regenerator [Compendium Section 5.2.1. Demonstrated in Exhibit 5.5.] |
|  | CH4 |  |  | Not considered [Compendium does not address CH4 emissions from FCC Coke Burn.] |  |  |  |  |
| Flaring | CO2 |  |  | Engineering estimates of gas flared i.e., using API flame length correlation and default factor for refinery gas |  |  | Process engineering estimates of flared volume based on known purge rates, process unit flows to flare and estimates of non-routine flaring based on plant logs. Weighted average flare gas composition based on estimated composition [Compendium Section 4.4. Flare emission calculations demonstrated in Exhibit 4.8.] | Flared volume estimated from flare gas meters where available, known purge rates and best process engineering estimates, average flare gas composition based on spot samples throughout the year adjusted if significant non-routine flaring [Compendium Section 4.4. Demonstrated in Exhibit 4.8.] |
|  | CH4 |  |  | Not considered [CH4 emission approach for refinery flares is provided in Compendium Section 4.4. Demonstrated in Exhibit 4.8. Negligible contribution of CH4 from refinery flares is demonstrated in Compendium Section 7.3] |  |  |  |  |
| Hydrogen Plant (process) | CO2 |  |  | Process mass balance based on estimated hydrogen production [Compendium Section 5.2.2. Demonstrated in Exhibit 5.6.] |  |  | Compendium “simple” method based on estimated hydrogen make [Compendium Section 5.2.2; Demonstrated in Exhibit 5.6.] | Compendium “complex” method i.e. process mass balance based on known reformer feed rate and composition [Compendium Section 5.2.2; Demonstrated in Exhibit 5.7.] |
|  | CH4 |  |  | Not considered [Compendium Equation 5-5 assumes complete conversion of feed gas] |  |  | Not considered [Compendium Equation 5-5 assumes complete conversion of feed gas] | Not considered (spot check on methane content of CO2 vent stream) [Compendium Equation 5-5 assumes complete conversion of feed gas] |
| Other Process Sources | CO2 |  |  | Not considered [Negligible contribution of CO2 emissions from other process sources is demonstrated in Compendium Section 7 case study for a downstream facility] |  |  | Process mass balance as in Compendium using activity data based on best engineering estimates [Compendium Sections 5.2 and 5.3. Demonstrated in exhibits for particular emission sources.] | Process mass balance as in Compendium using activity data based on best engineering estimates [Compendium Sections 5.2 and 5.3. Demonstrated in exhibits for particular emission sources.] |
|  | CH4 |  |  | Not considered [Cold vent approach provided in Compendium Section 5.3. Compendium generally assumes CH4 emissions from other refinery process vents are negligible.] |  |  |  |  |
| Non Routine Sources | CO2 |  |  | Not considered [Compendium Section 5.7.6 assumes non-routine emissions from refineries are routed to flare.] |  |  | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibit 5.26.] | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibit 5.26.] |
|  | CH4 |  |  | Not considered [Compendium Section 5.7.6 assumes non-routine emissions from refineries are routed to flare.] |  |  | Engineering estimates [Compendium Section 5.7.1 Demonstrated in Exhibit 5.26.] | Engineering estimates [Compendium Section 5.7.1. Demonstrated in Exhibit 5.26.] |
| Process Fugitives | CO2 |  |  | Not considered [CO2 emissions from fugitive sources are addressed in Compendium Section 6. Compendium states that CO2 emission from equipment leaks are negligible for most refineries.] |  |  |  |  |
|  | CH4 |  |  | Not considered [CH4 emissions from fugitive sources are addressed in Compendium Section 6, though CH4 contribution In refinery VOC emissions is assumed negligible.] |  |  |  | Not considered (possibly significant for natural gas supply pipe work) [Fugitive CH4 emissions from refinery fuel gas system are addressed in Compendium Section 6.1 |
| Other Area Sources | CO2 |  |  | Not considered [Addressed in Compendium Section 6.2. Compendium states that emissions from these sources are generally insignificant.] |  |  |  |  |
|  | CH4 |  |  | Not considered [Addressed in Compendium Section 6.2. Compendium states that emissions from these sources are generally insignificant. CH4 emission from water treatment are demonstrated in Exhibit 6.5.] |  |  |  |  |
| Non-Operated Refineries and Petrochemical Plants |  | CO2 |  |  | For non-operated refineries where the operator is unwilling or unable to provide GHG emissions data or the activity data, refinery emissions may be estimated by prorating to the nearest equivalent company operated refinery with suitable adjustments for processing severity and crude properties. A similar approach may be applied for petrochemical facilities if the feedstocks, processes, and products are sufficiently similar. |  |  |  |
|  |  |  | CH4 |  |  |  |  |  |

Figure 1.  SANGEA system summary report for an example refinery.

Figure 2.  SANGEA system summary chart of emissions by module