# Dry Deposition of NH3 in the Vicinity of a Swine Production Facility

# John T. Walker

# U.S. EPA National Risk Management Research Laboratory

Air Pollution Prevention and Control Division

Atmospheric Protection Branch

Research Triangle Park, NC 27711

Abstract -  Globally, domestic animals are the largest source (22 Tg N yr-1, 1 Tg = 1012 g) of atmospheric NH3, comprising approximately 40% of natural and anthropogenic emissions combined, while synthetic fertilizers and agricultural crops together contribute an additional 12.6 Tg NH3-N y-1 (23% of total emissions).Within and downwind of mixed (animal and crop production) agricultural regions, NH3 therefore plays a significant role in atmospheric chemistry, particularly the formation of inorganic PM2.5, and deposition of nitrogen to terrestrial and aquatic systems. While animal production facilities have been identified as important sources of NH3, there are no estimates of local NH3 dry deposition for U.S. systems. This project investigates the dry deposition of NH3 near a 5000 head swine production facility located in eastern North Carolina. Passive samplers are used to measure weekly-integrated NH3 concentrations along horizontal gradients from the lagoon/housing complex out to a distance of 500 m. Dry deposition is estimated using a resistance model that accounts for vegetation and soil compensation points as well as cuticular and stomatal uptake.  Preliminary results indicate dry deposition rates ranging from 200 kg NH3-N ha-1 yr-1 within 25 m of the lagoon/housing complex to 5 kg NH3-N ha-1 yr-1 at a distance of 500 m.  During a 1-week period of concurrent emission and deposition measurements during October 2003, cumulative deposition (0.062 g NH3 s-1) over the 500 m radius accounted for approximately 8% of barn emissions (0.755 g NH3 s-1).  Deposition measurements, which began during April 2003, will continue through April 2005 in order to capture the seasonal variability in emission rates, meteorology, and surface characteristics.

walker.johnt@epa.gov

Poster Presentation:

U.S. Environmental Protection Agency,

Office of Air Quality Planning and Standards

13th Annual Emission Inventory Conference

Clearwater, Florida, June 7 - 10, 2004