# Case study # 4:	NONPOINT Source burning

### Part 1:  Goal

The objective of this case study is to learn the principles of estimating emissions from relevant nonpoint sources.  A nonpoint source does not release pollutants into the atmosphere from a single stack where emissions could be easily measured.  Nonpoint sources tend to emit pollutants over broad geographic areas such as from multiple homes in a specified community or wild fires that occur across many hundreds or thousands of hectares.  Two different nonpoint source combustion categories are considered in this case study, residential heating and cooking, and prescribed burning.  The specific skills intended to be developed in this case study include:

- Identification of nonpoint emission sources;

- Compilation of appropriate and representative activity data;

- Identification and use of appropriate emission factors; and

- Implementation of emission estimation procedures.

### Part 2:  Problem Description

Note to the student: All of the activity data provided in this case study are fictitious and made up for the sole purpose of demonstrating the emissions inventory methodology. However, the emission factors and their references are based on the actual references as provided.

As winter approaches, the members of a community have noted an increase in the number of respiratory illness cases report to the local clinic.  Respiratory illnesses are closely associated with exposure to particulate matter (PM), especially fine PM.  Members of the community have identified two possible PM sources that could be associated with this increase in respiratory illness, residential heating and cooking and prescribed burning practices.  This case study quantifies emission from both of these nonpoint sources, but what other PM sources should be considered in this study?

Instructor – PM emissions can also be generated by mobile sources such as buses, trucks, cars, tractors, portable generators, and agricultural pumps.  Additional nonpoint sources should also be considered such as wind blown dust from plowing dry agricultural lands and vehicle traffic on dirt roads.  Small businesses such as small breweries, brick making, corn grinding operations, and charcoal production are also nonpoint sources of PM.

The village is located in the Mountains and is surrounded by an extensive pine forest.  The village has a population of 367 people living in 49 homes.  Figure 1 is a map of the village, noting the location of each home or shop and the associated property boundaries.  The number inside each property boundary is the area of the property in hectares.

Figure 1.  Village - Agricultural Property and Residential locations

### Residential Heating and Cooking

Community members burn a variety of softwoods found in the adjacent forests for heating and cooking purposes.  A small survey was implemented by community members that provided an average daily residential fuel usage of 10 kg of wood per day per home.

To estimate emissions from residential wood burning the amount of fuel combusted annually by the community needs to be estimated.  This can be done by developing typical home fuel usage rates that can be extrapolated to all homes in the community.  To estimate the amount of a given pollutant emitted by burning firewood by all homes in the community, the total fuel usage estimate should be applied to appropriate emission factors noted in Table 1.  For the purpose of this case study we are only interested in the PM emissions.  The same procedures used to calculate PM emissions can be used to calculate emissions for other pollutants if needed.

Table 1. Emission Factors for Residential Wood Combustion

| Pollutant | Emission Factor (g/kg of wood) |
| --- | --- |
| PM-10 | 17.3 |
| Carbon Monoxide | 126.3 |
| Sulfur Oxides | 0.2 |
| Nitrogen Oxides | 1.3 |
| Carbon Dioxide | 1,700 |
| Total VOCs | 114.5 |
| POM | 0.0008 |
| Aldehydes | 1.2 |

### Prescribed Burning

The village includes land for homes and shops, small vegetable gardens, and areas for cows and goats to graze upon.  95% of the land is used for grazing purposes.  All of the grazing land is burned every year in winter to encourage new growth for the animals to feed upon.  The grazing lands are mostly grass lands with 10% of the grazing area covered with a few trees and bushes.  The grasses are sparse clumps that have been over grazed by local cattle for several years.

To estimate emissions from prescribed burning the mass of material burned needs to be quantified and applied to appropriate emission factors.  The mass of material burned is referred to as fuel loading.  Some example fuel loading values are provided in Table 2.  To use this table, the appropriate fuel default value must be applied to the total number of hectares burned to approximate the total amount of material burned.

Table 2.  Sample Default Fuel Loading Data

| Fuel Type | Loading Defaults (kg/hectares) |  |
| --- | --- | --- |
|  | Sparse | Abundant |
| Interior ponderosa pine Litter | 567 |  |
| Downed woody debris |  |  |
| 0-1 inch diameter | 283 |  |
| 1-3 inches | 324 |  |
| ≥ 3 inches | 202 |  |
| Duff | 5 |  |
| Herbaceous ground cover | 40 | 121 |
| Shrubs | 0.0 | 202 |
| Tree regeneration | 0.0 | 121 |

Table 3 presents emission factors for various pollutants.  The emission factors are averages and can vary by as much as 50 percent with fuel and fire conditions.  To use these factors, multiply the mass of fuel consumed (i.e., land area x loading factor) by the emission factor for the appropriate fuel type.

Table 3.  Emission Factors for Prescribed Burning

| Fire Fuel Configuration | Pollutants (g/kg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Particulate |  |  | Carbon Monoxide | Volatile Organics |  |
|  | PM-2.5 | PM-10 | Total |  | Methane | Nonmethane |
| Hardwood | 11 | 12 | 18 | 112 | 6.1 | 6.4 |
| Conifer - short needle | 12 | 13 | 17 | 175 | 5.6 | 3.5 |
| Conifer - Long needle | 13 | 13 | 20 | 126 | 5.7 | 4.2 |
| Logging slash debris | 4 | 4 | 6 | 37 | 1.8 | ND |
| Juniper Slash | 9 | 10 | 14 | 82 | 6.0 | 5.2 |

Table 3.  Emission Factors for Prescribed Burning (Continued)

| Fire Fuel Configuration | Pollutants (g/kg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Particulate |  |  | Carbon Monoxide | Volatile Organics |  |
|  | PM-2.5 | PM-10 | Total |  | Methane | Nonmethane |
| Sagebrush | 13 | 15 | 23 | 103 | 6.2 | 6.9 |
| Chaparral shrub Communities | 10 | 11 | 20 | 101 | 4.5 | 12.5 |
| Palmetto gallberry | ND | 8-22 | ND | ND | ND | ND |
| Grasslands | ND | 10 | 10 | 75 | ND | 0 |

ND - No data

### Part 3:  Planning

The contents of the Inventory Preparation Plan/Quality Assurance Plan are outlined as follows:

- Background and uses of the inventory;

- Inventory scope (i.e., geographic area and population, pollutants of concern, base year, temporal resolution);

- Data quality objectives;

- Background and uses of the inventory;

- Emissions estimation methodologies; and

- QA/QC Procedures

- Internal QA/QC procedures; and

- External QA/QC procedures (to be conducted in Step 6 by exchanging solutions with another group, and completing the QA/QC Checklist).

### Part 4:  Solution

The solution to this case study has three parts: the IPP/QAP, and the emissions calculations and documentation.

### Solution – Inventory Preparation Plan/Quality Assurance Plan

The contents of the Inventory Preparation Plan/Quality Assurance Plan should be based upon the outline provided in Part 3, above.  Time limitations will dictate the level of detail that can be included in the Inventory Preparation Plan/Quality Assurance Plan.  An example of the minimum level of detail that should be included in the Inventory Preparation Plan/Quality Assurance Plan for this case study is as follows:

- Background and uses of the inventory – There have been no previous efforts carried out to develop emission estimates for the residential heating and cooking and prescribed burning in the village.  The basis for this inventory is to evaluate which source may have the greatest impact on local air quality.

- Inventory Scope:

- Inventory area – Village of Malkyne, 480.8 hectares inventory area consisting of 367 individuals who reside in 49 households;

- Pollutants of concern – PM

- Source categories – Residential heating and cooking and prescribed burning; and

- Temporal resolution – Annual emissions for the current year.

- Data quality objectives:

- The inventory should include emissions for the entire village; and

- Emissions estimates (calculations) should be 100% correct.

- Inventory Resources:

- Team

- Overall project manager

- Team manager

- Data evaluator

- Emissions Estimation Methodologies – Emission factors, and activity data from local surveys or land use maps.  Emissions can be calculated using the following general equation.

E=A×EF

Where:

E = emissions (kg/year);

A = activity level (kg of material combusted/year); and

EF = emission factor (g of pollutant /kg of material combusted).

- QA/QC Procedures – Internal QC steps should be briefly outlined in the IPP, and could include:

- 1.	Checking emission calculations for errors;

- 2.	Checking emission factors to ensure appropriateness of the factors used;

- 3.	Documenting all the assumptions made during emission calculations; and

- 4.	Comparing the magnitude of calculated emissions with other source categories.

### Solution - Calculations

### Residential Heating and Cooking

Instructor – question to class “What sort of data will be needed to estimate emissions from residential heating and cooking and how would you try and compile the necessary data?”

The first step would be to consider what units the emission factors are in.  In both the residential heating and prescribed burning, the emission factors are in terms of mass of material combusted.  For residential heating, the amount of material combusted can be determined from a variety of ways:

- Implementation of local survey;

- Use of national wood fuel consumption divided by national population to get a per capita annual fuel usage rate.  It is best if the fuel consumption usage and population data are by region in order to develop per capita fuel usage data that reflects local differences; and

- If you were considering other types of fuel such as LPG or coal, you might be able to obtain local fuel usage data from facilities that distribute or sell the fuel.

In this study, the first step would be to use results from the survey to estimate the amount of fuel a typical home burns in a year:

10 kg of wood / (home/day) × 365 days/yr = 3650 kg of wood / (home/yr)

Next, this value should be extrapolated to all homes in the community to estimate the total village residential fuel consumption:

3,650 kg of wood / (home/yr) × 49 homes = 178,850 kg of wood /yr

To estimate the amount of PM emitted by combusting this amount of wood, the fuel usage estimate should be applied to the PM emission factor from Table 1.

178,850 kg of wood / yr × 17.3 g of PM/kg of wood = 3,094,105 g of PM = 3,094 kg of PM

### Prescribed Burning

Prescribed burning is a land treatment, used under controlled conditions, to accomplish natural resource management objectives.  It is one of several land treatments, used individually or in combination, including chemical and mechanical methods.  Prescribed fire is a cost-effective and ecologically sound tool for forest, range, and wetland management.  Its use reduces the potential for destructive wildfires and thus maintains long-term air quality.  Also, the practice removes logging residues, controls insects and disease, improves wildlife habitat and forage production, increases water yield, maintains natural succession of plant communities, and reduces the need for pesticides and herbicides.

Instructor – question to class “What sort of data will be needed to estimate emissions from prescribed burning and how would you try and compile the necessary data?”

As noted above, the first step would be to consider what units the emission factors are in.  For prescribed burning the emission factors are in terms of mass of material combusted.  The amount of material combusted can be determined from a variety of ways:

- Studying land use maps of the area to define the amount of land burned and the types of vegetation on the burned land;

- By contacting local government agricultural staff, detailed information can be provided that help quantify prescribed burning practices; and

- In some cases national or regional land use and land coverage data are available from satellite data.  These photos can be used to identify agricultural lands and sometimes even the types of vegetation on the land.

To estimate prescribe burning emissions, the land which is to be included in the inventory needs to be identified and the total surface area estimated.  For the purpose of this case study, we will assume that all property that is shown on Figure 1 is to be considered in this study.  The total amount of hectares that are considered to be agricultural land is 480.8 hectares.  Ninety-five percent of the land is used for grazing purposes and it is this section of the land that is subject to prescribed burning, therefore:

480.8 hectares × 0.95 = 456.76 hectares are subject to prescribed burning

Ninety percent of the area where prescribed burning occurs is covered with grass land while 10 percent includes bushes and small trees:

456.76 hectares × 0.90 = 411.08 hectares of grass

456.76 hectares × 0.10 = 45.68 hectares of bushes

We can assume that the sparse clumps of grass that dominate the grazing area is considered similar to herbaceous ground cover and bushes and trees are similar to shrubs.  The total biomass burnt can be estimated by using the area calculated above and the fuel loading data note in Table 2.

411.08 hectares × 40 kg/hectare = 16,443.20 kg of grass burned

45.68 hectares × 202 kg/hectare = 9,227.36 kg of bush material burned

The mass of grass and bushes burned can now be applied to emission factor in Table 3 to estimate the amount of PM10 emitted:

16,443.2 kg of grass burnt × 10g/kg = 164,4320g = 164.43 kg of PM10

9,227.36 kg of bush burnt × 11g/kg = 101,500.96g = 101.50 kg of PM10

Total PM 10 emissions from prescribed burning:

164.43 kg +101.50 kg = 265.93 kg

### Part 5:  Documentation

Due to time limitations, it is not possible to develop a complete emissions inventory report.  However, an outline or annotated outline should be developed which contains the following elements:

- Description of the source category including any sub-categories that were excluded (e.g., residential heating and open burning) and why;

- Explanation of the methods used for data collection and sources of data collected (e.g., surveys and surface area maps);

- Explanation of the assumptions made in data collection and in the data analysis phase;

- Emission estimation methods;

- Emission factors and their sources (i.e., EPA, 1995);

- Emission calculations and assumptions;

- Internal QC checks implemented and results of external QA including findings and corrections made; and

- Results (e.g., tables, pie charts) and analysis (e.g., comparisons/controls) among sub-categories).

### Part 6:  Quality Assurance

Have students exchange all documentation and conduct external QA audit using the QA checklist.

### Part 7:  Discussion of Results

- Review the following with the students:

- The external QA checklist;

- The content of the inventory report; and

- The emission calculations and results.

- Part 8:   Additional References

Residential Heating and Cooking

1.	Jean M. Dash, "Particulate And Gaseous Emissions From Wood-burning Fireplaces," Environmental Science And Technology, 16(10):643-67, October 1982.

2.	Lipari, F., et al., "Aldehyde Emissions From Wood-Burning Fireplaces," Publication GMR-4377R, General Motors Research Laboratories, Warren, MI, March 1984.

3.	Kosel, P., et al., Emissions From Residential Fireplaces, CARB Report C-80-027, California Air Resources Board, Sacramento, CA, April 1980.

4.	DeAngelis, D.G., et al., Source Assessment: Residential Combustion of Wood, EPA-600/2-80-042b, U.S. Environmental Protection Agency, Cincinnati, OH, March 1980.

5.	DeAngelis, D.G., et al., "Preliminary Characterization Of Emissions From Wood Fired Residential Combustion Equipment," EPA-600/7-80-040, U.S. Environmental Protection Agency, Cincinnati, OH, March 1980.

6.	Burnett, P.G., "The Northeast Cooperative Woodstove Study," Volume 1, EPA- 600/7-87-026a, U.S. Environmental Protection Agency, Research Triangle Park, NC, November 1987.

Prescribed Burning

1.	D. E. Ward and C. C. Hardy, Advances In The Characterization And Control Of Emissions From Prescribed Broadcast Fires Of Coniferous Species Logging Slash On Clearcut Units, EPA DW12930110-01-3/DOE DE-A179-83BP12869, U. S. Forest Service, Seattle, WA, January 1986.

2.	L. F. Radke, et al., Airborne Monitoring And Smoke Characterization Of Prescribed Fires On Forest Lands In Western Washington And Oregon, EPA-600/X-83-047, U. S. Environmental Protection Agency, Cincinnati, OH, July 1983.

3.	H. E. Mobley, et al., A Guide For Prescribed Fire In Southern Forests, U. S. Forest Service, Atlanta, GA, 1973.

4.	Southern Forestry Smoke Management Guidebook, SE-10, U. S. Forest Service, Asheville, NC, 1976.

5.	D. V. Sandberg, et al., Effects Of Fire On Air — A State Of Knowledge Review, WO-9, U. S. Forest Service, Washington, DC, 1978.

6.	Prescribed Fire Smoke Management Guide, 420-1, BIFC-BLM Warehouse, Boise, ID, February 1985.

7.	Colin C. Hardy, Emission Factors For Air Pollutants From Range Improvement Prescribed Burning of Western Juniper And Basin Big Sagebrush, PNW 88-575, Office Of Air Quality Planning And Standards, U. S. Environmental Protection Agency, Research Triangle Park, NC, March 1990.

8.	Colin C. Hardy And D. R. Teesdale, Source Characterization and Control Of Smoke Emissions From Prescribed Burning Of California Chaparral, CDF Contract No. 89CA96071, California Department Of Forestry And Fire Protection, Sacramento, CA 1991.

9.	W. M. Hao and D. Ward, "Methane Production From Global Biomass Burning", Journal Of Geophysical Research, 98(D11):20,657-20,661, pp. 20, 656, November 1993.