# Case study #5: Nonpoint source – Livestock Management

### Part 1:  Goal

The objective of this case study is to learn how to characterize, estimate, and report emissions for a nonpoint source category.  The specific goals to be achieved by this case study are:

- To estimate methane emissions from the livestock enteric fermentation nonpoint source category within a localized boundary;

- To use local cattle population and characterizations to prepare the inventory estimates;

- To document the inventory process so that the results can be duplicated.

### Part 2:  Problem Description

In this agricultural region of the country methane emissions from livestock enteric fermentation are a key source contributor to greenhouse gas emissions.  Methane is produced as part of normal digestive processes in animals.  During digestion, microbes resident in an animal’s digestive system ferment food consumed by the animal.  This microbial fermentation process, referred to as enteric fermentation, produces CH4 as a by-product, which can be exhaled or eructated by the animal.  The amount of CH4 produced and excreted by an individual animal depends primarily upon the animal's digestive system, and the amount and type of feed it consumes.

Cattle are the main ruminant animal of concern for this source category.  In the inventory region, large populations of cattle are grazed in open pasture, often on relatively poor quality grasses and ground cover.  However, certain portions of the livestock population have access to higher quality feeds as more feedlots are used for animal fattening, along with some high producing dairy farms, which can affect the methane produced from the animals.  The objective is to prepare an estimate of methane that better reflects the region’s population of cattle.

Note to the student: All of the activity data and their references provided in this case study are fictitious and made up for the sole purpose of demonstrating the emissions inventory methodology. However, the emission factors and their references are based on the actual references as provided.

Livestock Population

- In order to estimate CH4 enteric fermentation emissions from cattle in the study region, the population is divided into age, sub-type (e.g., calves, heifer replacements, cows, etc.), and production (i.e., lactating, fattening, etc.) groupings to more fully capture differences in CH4 emissions from these animal types.

- For the local inventory area being evaluated, the breakdown of cattle population is as follows:

- Cattle Type 							 	Population

- High-producing dairy cows (4000+ kg/head/yr milk)		4000

- Feedlot-fed beef cattle							4500

- Mature grazing males (bulls)						2000

- Mature grazing females (beef cows)					10000

- Young cattle (replacements, growing heifers and steers)		7500

- Calves (on milk)							4000

- Population values are in terms of average annual standing herd size.  The population data was developed from local farm surveys and cross-verified against national level agricultural statistics to verify reasonableness.

- The dairy cows and feedlot cattle are stall-fed in commercialized operations striving for maximum production of milk and meat products.  The high-producing dairy cows and feedlot-fed cattle are fed relatively high-quality feed, with high digestibility (70 to 80%).  The producing dairy cows are rather large (450-500kg).  The rest of the cattle population is primarily in a pasture grazing management system, with relatively low quality grasses (digestibility of 55% of less).

- Emission factors are available from the Intergovernmental Panel on Climate Change (IPCC) Revised 1996 Guidelines for National Greenhouse Gas Inventories.  Factors are provided for different regions of the world based on animal and feed characteristics.  The following factors best represent the conditions of the localized animal population as described above:

| Cattle Type | Emission Factor (kg CH4/head/yr) |
| --- | --- |
| Dairy Cows | 70 |
| Feedlot Cattle | 40 |
| Grazing Mature Males (Bulls) | 50 |
| Grazing Mature Females (Beef Cows) | 50 |
| Young Cattle | 40 |
| Calves (on milk) | 0 |

- Emission Factor Source:  Compiled from the Revised 1996 IPCC Guidelines for

- National Greenhouse Gas Inventories, Tables A-1 and A-2, Data for Estimating Enteric

- Fermentation Emission Factors for Dairy and Non-dairy Cattle.

### Part 3:  Planning

The contents of the Inventory Preparation Plan/Quality Assurance Plan are outlined as follows:

- Background and uses of the inventory;

- Inventory area status (i.e., attainment or nonattainment);

- Inventory scope (i.e., geographic area and population, pollutants of concern, base year, temporal resolution);

- Data quality objectives;

- Inventory resources;

- Emissions estimation methodologies; and

- QA/QC Procedures

- Internal QA/QC procedures; and

- External QA/QC procedures (to be conducted in Step 6 by exchanging solutions with another group, and completing the QA/QC Checklist).

### Part 4:  Solution

The solution to this case study has three parts: the Inventory Preparation Plan/Quality Assurance Plan, and the emissions calculations and documentation.

### Solution – Inventory Preparation Plan/Quality Assurance Plan

The contents of the Inventory Preparation Plan/Quality Assurance Plan should be based upon the outline provided in Part 3, above.  Time limitations will dictate the level of detail that can be included in the Inventory Preparation Plan/Quality Assurance Plan.  An example of the minimum level of detail that should be included in the Inventory Preparation Plan/Quality Assurance Plan for this case study is as follows:

- Background and uses of the inventory – National level estimates have been prepared already; this inventory is to provide regional estimates using local livestock characterizations.  These regional estimates will aid in considering potential mitigation strategies at the smaller scale.

- Inventory area status – attainment area; methane is important from as a greenhouse gas;

- Inventory Scope:

- Inventory area – 300 km2 inventory area, primarily agricultural and scattered residential;

- Pollutants of concern – Methane;

- Source categories – Cattle enteric fermentation

- Temporal resolution – Annual emissions for the year 2002 (base year).

- Data quality objectives:

- The inventory should include emissions for the entire region (based annual average standing herd populations for the region); and

- Emissions estimates (calculations) should be 100% correct.

- Inventory Resources:

- Team

- Overall project manager

- Team manager

- Data evaluator

- Emissions Estimation Methodologies – Emission factors and activity data from agricultural statistics and local surveys.  Emissions can be calculated using the following general equation.

E=A×EF

Where:

E = emissions (kg/year);

A = activity level (average annual standing animal population); and

EF = emission factor (kg/head of cattle).

- QA/QC Procedures – Internal QC steps should be briefly outlined in the IPP, and could include:

- 1.	Checking emission calculations for errors;

- 2.	Checking emission factors to ensure appropriateness of the factors used;

- 3.	Documenting all the assumptions made during emission calculations; and

- 4.	Comparing the magnitude of calculated emissions with other greenhouse gas source categories and national level enteric fermentation emissions.

### Solution - Calculations

See attached spreadsheet.  Note that calves that have not yet been weaned are not considered to contribute to methane emissions from enteric fermentation since their milk diet results in negligible methane production.

### Part 5:  Documentation

Due to time limitations, it is not possible to develop a complete emissions inventory report.  However, an outline or annotated outline should be developed which contains the following elements:

- Description of the source category including any sub-categories that were excluded (e.g., calves on milk since they have 0%  CH4 conversion) and why;

- Explanation of the methods used for data collection and sources of data collected (e.g., farm surveys in conjunction with national agriculture statistics);

- Explanation of the assumptions made in data collection and in the data analysis phase;

- Emission estimation methods;

- Emission factors and their sources (i.e., modified IPCC) ;

- Emission calculations and assumptions;

- Internal QC checks implemented and results of external QA including findings and corrections made and;

- Results (e.g., tables, pie charts) and analysis (e.g., comparisons/controls) among sub-categories).

### Part 6:  Quality Assurance

Have students exchange all documentation and conduct external QA audit using the QA checklist.

### Part 7:  Discussion of Results

- Review the following with the students:

- The external QA checklist;

- The content of the inventory report; and

- The emission calculations and results.

### Attachment

- Solution FOR

- Nonpoint source – Livestock Management case study

(spreadsheet)