# Case study #2: point source - FOUNDRY

### Part 4:  Solution

The solution to this case study has three parts: the Inventory Preparation Plan/Quality Assurance Plan, and the emissions calculations and documentation.

## Solution – Inventory Preparation Plan/Quality Assurance Plan

The contents of the Inventory Preparation Plan/Quality Assurance Plan should be based upon the outline provided in Part 3, above.  Time limitations will dictate the level of detail that can be included in the Inventory Preparation Plan/Quality Assurance Plan.  An example of the minimum level of detail that should be included in the Inventory Preparation Plan/Quality Assurance Plan for this case study is as follows:

- Background and purpose of the inventory – There have been no previous efforts to develop emission estimates for this facility. The basis for this point source inventory is to aid in policy making by the local air quality regulating agency.

- Inventory area status – attainment vs. nonattainment status;

- Inventory scope:

- Inventory area/facility: Gray iron foundry;

- Pollutants of concern: NOx, SOx, CO, VOC, PM10, and PM2.5;

- Sources: Materials handling (2 conveyors), metal melting (4 furnaces), iron refining (fugitive emissions), mold and core production (2 sand handling units), casting and finish (fugitive emissions); and

- Temporal resolution: Annual emissions for the year 2004.

- Data quality objectives:

- The inventory should include all the sources listed; and

- Emission estimates should be 100% correct.

- Inventory resources:

- Team

- Overall project manager

- Team manager

- Data evaluator

- Emissions estimation methodologies – Emission factors and activity data; stack monitoring data.

- Table1.  Emission Estimation Methodology:  Uncontrolled Emissions

| - Process | - Pollutant | - Methodology | - Equation | - Data needed |
| --- | --- | --- | --- | --- |
| - Materials Handling: - 2 Conveyor belts | - PM2.5 - PM10 | - Emission Factor and Activity Data | - EF = k x 0.0016 x [(U/2.2)1.3/(M/2)1.4)] E = EF x A x 12 month/year EF = Emission factor (kg/Mg of material) k = Particle size multiplier (k = 0.35 for PM10 and 0.11 for PM2.5) U = Mean wind speed, meters per second (m/s) M = Material moisture content (%) - E = Emissions (Mg/Year) - A = Throughput (Mg/month) | - U - M - A |
| - Metal Melting: - Degreasing – 2 booths | - VOC | - Material Balance | - E = S x VOC - E = Emissions (g/year) - S = Solvent used (liters/year) - VOC = VOC content of solvent (g/liter) | - S - VOC |
| - Metal Melting: - 4 Electric Arc Furnaces | - PM | - Source test | - E = (PM test/V) x FR x 60 x (1kg/1000g) - E = Emissions (kg/hr) - PM test = PM collected on filter (g) - V = volume of gas sampled (dscm) - FR = flow rate (dscmm) | - PM test - V - FR |
| - Metal Melting: - 4 Electric Arc Furnaces | - SO2 - NOx - CO | - CEM | - E = (P conc x MW x FR x 60)/(24.07 m3/kg-mole x 106 ) - E = Emissions (kg/hr) - P conc = pollutant concentration (ppmv) - MW = molecular weight of pollutant (kg/kg-mole) - FR = stack flow rate (dscmm) | - P conc - MW - FR |
| - Metal Melting: - 4 Electric Arc Furnaces | - VOC | - Emission Factor and Activity Data | - E = EF x A - E = Emissions (kg/yr) - EF = emission factor =0.09 kg/Mg Iron produced - A = Mg iron produced /yr | - A |
| - Process | - Pollutant | - Methodology | - Equation | - Data needed |
| - Iron Refining - Iron Refining - Magnesium treatment | - PM | - Emission Factor and Activity Data | - E = EF x A x 0.49 for PM10 - E = EF x A x 0.24 for PM2.5 - E = Emissions (kg/yr) - EF = emission factor - EF = 2 kg/Mg for iron refining - EF = 0.2 kg/Mg for magnesium treatment - A = Mg material/yr | - A |
| - Mold and Core Production: - Shakeout - Sand handling – 2 units - Baking | - PM | - Emission Factor and Activity Data | - E = EF x A x 0.70 for PM10 - E = EF x A x 0.42 for PM2.5 - E = Emissions (kg/yr) - EF = emission factor - EF = 1.6 kg/Mg for shakeout - EF = 1.8 kg/Mg for sand handling - EF = 0.6 kg/Mg for baking - A = Mg material /yr | - A |
| - Casting and Finishing: Pouring - Finishing | - PM | - Emission Factor and Activity Data | - E = EF x A x 0.49 for PM10 - E = EF x A x 0.24 for PM2.5 - E = Emissions (kg/yr) - EF = emission factor - EF = 2.1 kg/Mg for pouring - EF = 0.05 kg/Mg for finishing - A = Mg material /yr | - A |

- What units have hourly emissions that need to be converted?  What is equation to convert hourly emissions to annual?

- For Electric Arc Furnaces emissions of PM, SO2, CO and NOx, convert hourly emissions to annual emissions by using operational data for facility.

- Emissions (kg/yr) = emissions (kg/hr) x 12 (hrs/day) x 5 (days/week) x 52 (weeks/yr) = 3120 (hr/yr) x emissions (kg/hr)

- What is equation to calculate controlled emissions?

- For controlled emissions, use capture and control efficiency data.

- Controlled emissions = Uncontrolled emissions (1 – (CapEff x ConEff))

- Cap Eff – Capture efficiency expressed as a fraction

- Con Eff – Control efficiency expressed as a fraction

- Complete Table 2 to calculate controlled emissions.

- Table 2.  Control Emissions Data

| - Process | - Unit | - Pollutant | - Control Equipment | - Capture Efficiency (%) | - Control Efficiency (%) | - Equation |
| --- | --- | --- | --- | --- | --- | --- |
| - Materials Handling | - Coke | - PM | - Baghouse with fabric filter | - 85 | - 95 | - Controlled emissions = Uncontrolled emissions x (1- (0.95 x 0.85)) |
| - Materials Handling | - Pellet | - PM | - Baghouse with fabric filter | - 100 | - 95 | - Controlled emissions = Uncontrolled emissions x (1- (1 x 0.95)) |
| - Metal Melting |  | - PM | - Baghouse | - 100 | - 95 | - Controlled emissions = Uncontrolled emissions x (1- (1 x 0.95)) |

- QA/QC procedures – Internal QC steps should be briefly outlined in the Inventory Preparation Plan, and could include:

- Checking emission calculations for errors;

- Checking emission factors to ensure the appropriateness of the factors used; and

- Documenting all the assumptions made during emission calculations.

### Solution - Calculations

See attached spreadsheet.

### Part 5:  Documentation

Due to time limitations, it is not possible to develop a complete emissions inventory report.  However, an outline or annotated outline can be developed which contains the following elements:

- Description of the source category (i.e., gray iron foundries);

- Explanation of the methods used for data collection, and sources of data collected (e.g., source tests for some sources);

- Explanation of the assumptions made in data collection and in the data analysis phase (e.g., assume equal production for each furnace);

- Emission estimation methods;

- Emission factors and their sources;

- Emission calculations and assumptions;

- Internal QC checks implemented and results of external QA including findings and corrections made; and

- Results (e.g., tables, pie charts) and analysis (e.g., comparisons/controls) among sub-categories.

### Attachment

- Solution FOR

- point source - foundry case study

(spreadsheet)