# Case study # 4:	Nonpoint Source burning

### Part 4:  Solution

The solution to this case study has three parts: the IPP/QAP, and the emissions calculations and documentation.

### Solution – Inventory Preparation Plan/Quality Assurance Plan

The contents of the Inventory Preparation Plan/Quality Assurance Plan should be based upon the outline provided in Part 3, above.  Time limitations will dictate the level of detail that can be included in the Inventory Preparation Plan/Quality Assurance Plan.  An example of the minimum level of detail that should be included in the Inventory Preparation Plan/Quality Assurance Plan for this case study is as follows:

- Background and uses of the inventory – There have been no previous efforts carried out to develop emissions estimates for residential heating and cooking and prescribed burning in the village.  The basis for this inventory is to evaluate which source may have the greatest impact on local air quality.

- Inventory Scope:

- Inventory area – Village, 480.8 hectares inventory area consisting of 367 individuals who reside in 49 households;

- Pollutants of concern – PM

- Source categories – Residential heating and cooking and prescribed burning; and

- Temporal resolution – Annual emissions for the current year.

- Data quality objectives:

- The inventory should include emissions for the entire village; and

- Emissions estimates (calculations) should be 100% correct.

- Inventory Resources:

- Team

- Overall project manager

- Team manager

- Data evaluator

- Emissions Estimation Methodologies – Emission factors, and activity data from local surveys or land use maps.  Emissions can be calculated using the following general equation.

E=A×EF

Where:

E = emissions (kg/year);

A = activity level (kg of material combusted/year); and

EF = emission factor (g of pollutant /kg of material combusted).

- QA/QC Procedures – Internal QC steps should be briefly outlined in the IPP, and could include:

- Checking emission calculations for errors;

- Checking emission factors to ensure appropriateness of the factors used;

- Documenting all the assumptions made during emission calculations; and

- Comparing the magnitude of calculated emissions with other source categories.

### Solution - Calculations

### Residential Heating and Cooking

In this study, the first step would be to use results from the survey to estimate the amount of fuel a typical home burns in a year:

10 kg of wood / (home/day) × 365 days/yr = 3650 kg of wood / (home/yr)

Next, this value should be extrapolated to all homes in the community to estimate the total village residential fuel consumption:

3,650 kg of wood / (home/yr) × 49 homes = 178,850 kg of wood /yr

To estimate the amount of PM emitted by combusting this amount of wood, the fuel usage estimate should be applied to the PM emission factor from Table 1.

178,850 kg of wood / yr × 17.3 g of PM/kg of wood = 3,094,105 g of PM = 3,094 kg of PM

### Prescribed Burning

Prescribed burning is a land treatment, used under controlled conditions, to accomplish natural resource management objectives.  It is one of several land treatments, used individually or in combination, including chemical and mechanical methods.  Prescribed fire is a cost-effective and ecologically sound tool for forest, range, and wetland management.  Its use reduces the potential for destructive wildfires and thus maintains long-term air quality.  Also, the practice removes logging residues, controls insects and disease, improves wildlife habitat and forage production, increases water yield, maintains natural succession of plant communities, and reduces the need for pesticides and herbicides.

To estimate prescribe burning emissions, the land which is to be included in the inventory needs to be identified and the total surface area estimated.  For the purpose of this case study, we will assume that all property that is shown on Figure 1 is to be considered in this study.  The total amount of hectares that are considered to be agricultural land is 480.8 hectares.  Ninety-five percent of the land is used for grazing purposes and it is this section of the land that is subject to prescribed burning, therefore:

480.8 hectares × 0.95 = 456.76 hectares are subject to prescribed burning

Ninety percent of the area where prescribed burning occurs is covered with grass land while 10 percent includes bushes and small trees:

456.76 hectares × 0.90 = 411.08 hectares of grass

456.76 hectares × 0.10 = 45.68 hectares of bushes

We can assume that the sparse clumps of grass that dominate the grazing area are considered similar to herbaceous ground cover and bushes and trees are similar to shrubs.  The total biomass burnt can be estimated by using the area calculated above and the fuel loading data note in Table 2.

411.08 hectares × 40 kg/hectare = 16,443.20 kg of grass burned

45.68 hectares × 202 kg/hectare = 9,227.36 kg of bush material burned

The mass of grass and bushes burned can now be applied to emission factor in Table 3 to estimate the amount of PM10 emitted:

16,443.2 kg of grass burnt × 10g/kg = 164,4320g = 164.43 kg of PM10

9,227.36 kg of bush burnt × 11g/kg = 101,500.96g = 101.50 kg of PM10

Total PM 10 emissions from prescribed burning:

164.43 kg +101.50 kg = 265.93 kg

### Part 5:  Documentation

Due to time limitations, it is not possible to develop a complete emissions inventory report.  However, an outline or annotated outline should be developed which contains the following elements:

- Description of the source category including any sub-categories that were excluded (e.g., residential heating and open burning) and why;

- Explanation of the methods used for data collection and sources of data collected (e.g., surveys and surface area maps);

- Explanation of the assumptions made in data collection and in the data analysis phase;

- Emission estimation methods;

- Emission factors and their sources (i.e., EPA, 1995);

- Emission calculations and assumptions;

- Internal QC checks implemented and results of external QA including findings and corrections made; and

- Results (e.g., tables, pie charts) and analysis (e.g., comparisons/controls) among sub-categories).

### Part 6:  Quality Assurance

Have students exchange all documentation and conduct external QA audit using the QA checklist.

### Part 7:  Discussion of Results

- Review the following with the students:

- The external QA checklist;

- The content of the inventory report; and

- The emission calculations and results.

- Part 8:   Additional References

Residential Heating and Cooking

1.	Jean M. Dash, "Particulate And Gaseous Emissions From Wood-burning Fireplaces," Environmental Science And Technology, 16(10):643-67, October 1982.

2.	Lipari, F., et al., "Aldehyde Emissions From Wood-Burning Fireplaces," Publication GMR-4377R, General Motors Research Laboratories, Warren, MI, March 1984.

3.	Kosel, P., et al., Emissions From Residential Fireplaces, CARB Report C-80-027, California Air Resources Board, Sacramento, CA, April 1980.

4.	DeAngelis, D.G., et al., Source Assessment: Residential Combustion of Wood, EPA-600/2-80-042b, U.S. Environmental Protection Agency, Cincinnati, OH, March 1980.

5.	DeAngelis, D.G., et al., "Preliminary Characterization Of Emissions From Wood Fired Residential Combustion Equipment," EPA-600/7-80-040, U.S. Environmental Protection Agency, Cincinnati, OH, March 1980.

6.	Burnett, P.G., "The Northeast Cooperative Woodstove Study," Volume 1, EPA- 600/7-87-026a, U.S. Environmental Protection Agency, Research Triangle Park, NC, November 1987.

Prescribed Burning

1.	D. E. Ward and C. C. Hardy, Advances In The Characterization And Control Of Emissions From Prescribed Broadcast Fires Of Coniferous Species Logging Slash On Clearcut Units, EPA DW12930110-01-3/DOE DE-A179-83BP12869, U. S. Forest Service, Seattle, WA, January 1986.

2.	L. F. Radke, et al., Airborne Monitoring And Smoke Characterization Of Prescribed Fires On Forest Lands In Western Washington And Oregon, EPA-600/X-83-047, U. S. Environmental Protection Agency, Cincinnati, OH, July 1983.

3.	H. E. Mobley, et al., A Guide For Prescribed Fire In Southern Forests, U. S. Forest Service, Atlanta, GA, 1973.

4.	Southern Forestry Smoke Management Guidebook, SE-10, U. S. Forest Service, Asheville, NC, 1976.

5.	D. V. Sandberg, et al., Effects Of Fire On Air — A State Of Knowledge Review, WO-9, U. S. Forest Service, Washington, DC, 1978.

6.	Prescribed Fire Smoke Management Guide, 420-1, BIFC-BLM Warehouse, Boise, ID, February 1985.

7.	Colin C. Hardy, Emission Factors For Air Pollutants From Range Improvement Prescribed Burning of Western Juniper And Basin Big Sagebrush, PNW 88-575, Office Of Air Quality Planning And Standards, U. S. Environmental Protection Agency, Research Triangle Park, NC, March 1990.

8.	Colin C. Hardy And D. R. Teesdale, Source Characterization and Control Of Smoke Emissions From Prescribed Burning Of California Chaparral, CDF Contract No. 89CA96071, California Department Of Forestry And Fire Protection, Sacramento, CA 1991.

9.	W. M. Hao and D. Ward, "Methane Production From Global Biomass Burning", Journal Of Geophysical Research, 98(D11):20,657-20,661, pp. 20, 656, November 1993.