# Case study #5: Nonpoint source – Livestock Management

### Part 4:  Solution

The solution to this case study has three parts: the Inventory Preparation Plan/Quality Assurance Plan, and the emissions calculations and documentation.

### Solution – Inventory Preparation Plan/Quality Assurance Plan

The contents of the Inventory Preparation Plan/Quality Assurance Plan should be based upon the outline provided in Part 3, above.  Time limitations will dictate the level of detail that can be included in the Inventory Preparation Plan/Quality Assurance Plan.  An example of the minimum level of detail that should be included in the Inventory Preparation Plan/Quality Assurance Plan for this case study is as follows:

- Background and uses of the inventory – National level estimates have been prepared already; this inventory is to provide regional estimates using local livestock characterizations.  These regional estimates will aid in considering potential mitigation strategies at the smaller scale.

- Inventory area status – attainment area; methane is important from as a greenhouse gas;

- Inventory Scope:

- Inventory area – 300 km2 inventory area, primarily agricultural and scattered residential;

- Pollutants of concern – Methane;

- Source categories – Cattle enteric fermentation

- Temporal resolution – Annual emissions for the year 2002 (base year).

- Data quality objectives:

- The inventory should include emissions for the entire region (based annual average standing herd populations for the region); and

- Emissions estimates (calculations) should be 100% correct.

- Inventory Resources:

- Team

- Overall project manager

- Team manager

- Data evaluator

- Emissions Estimation Methodologies – Emission factors and activity data from agricultural statistics and local surveys.  Emissions can be calculated using the following general equation.

E=A×EF

Where:

E = emissions (kg/year);

A = activity level (average annual standing animal population); and

EF = emission factor (kg/head of cattle).

- QA/QC Procedures – Internal QC steps should be briefly outlined in the Inventory Preparation Plan, and could include:

- 1.	Checking emission calculations for errors;

- 2.	Checking emission factors to ensure appropriateness of the factors used;

- 3.	Documenting all the assumptions made during emission calculations; and

- 4.	Comparing the magnitude of calculated emissions with other greenhouse gas source categories and national level enteric fermentation emissions.

### Solution - Calculations

See attached spreadsheet.  Note that calves that have not yet been weaned are not considered to contribute to methane emissions from enteric fermentation since their milk diet results in negligible methane production.

### Part 5:  Documentation

Due to time limitations, it is not possible to develop a complete emissions inventory report.  However, an outline or annotated outline should be developed which contains the following elements:

- Description of the source category including any sub-categories that were excluded (e.g., calves on milk since they have 0%  CH4 conversion) and why;

- Explanation of the methods used for data collection and sources of data collected (e.g., farm surveys in conjunction with national agriculture statistics);

- Explanation of the assumptions made in data collection and in the data analysis phase;

- Emission estimation methods;

- Emission factors and their sources (i.e., modified IPCC) ;

- Emission calculations and assumptions;

- Internal QC checks implemented and results of external QA including findings and corrections made and;

- Results (e.g., tables, pie charts) and analysis (e.g., comparisons/controls) among sub-categories).

### Part 6:  Quality Assurance

Have students exchange all documentation and conduct external QA audit using the QA checklist.

### Part 7:  Discussion of Results

- Review the following with the students:

- The external QA checklist;

- The content of the inventory report; and

- The emission calculations and results.

### Attachment

- Solution FOR

- Nonpoint source – Livestock Management case study

(spreadsheet)