File: 20170910 Intracellular Study - CZX.docx

Location: (A.Swank) M:\Work Documents\Stryner-NERL\DeGroot - 201607 Metabolism\Experiments\ 2017910 Intracellular Study

Project: DeGroot - 201607 Metabolism, (CSS project- task HTT 10.3.2.)

QA Track ID:   A-IO-0030767

QAPP Title: Retrofit Strategies for Incorporating Xenobiotic Metabolism into High-Throughput Screening Assays

Prepared By: Adam Swank

Instrument data collected to Dell Optiplex 745 in D278, D:\Analyst data\Project\Degroot-Metabolism\Data\

Project files stored (A.Swank)M:\Work Documents\Stryner-NERL\DeGroot - 201607 Metabolism

Method QC Acceptance Criteria

Calibration Curve: use Blank (matrix processed without analyte and internal standard), Zero Blank/Method Blank (matrix processed without analyte but with internal standard), 7 non-zero samples (matrix, analyte, and IS) covering range from LLOQ to ULOQ. LLOQ is > to concentration with 5-10x response compared to blank.

Acceptance criteria:

- accuracy w/in 20% (30% LLOQ) of nominal concentration and precision should not exceed 15% (20% LLOQ) for CV

- 75% of non-zero standards must meet the accuracy and precision criteria

- Correlation coefficient of > 0.99

Accuracy and Precision Assessment (QC Samples):

- accuracy w/in 20% (30% LLOQ) of nominal concentration and precision should not exceed 15% (20% LLOQ) for CV

Selectivity Assessment:

- Blank Matrix Response < LOD

- Matrix Blank Response < LOD

Overview of Sample Prep:

- Add 300 µL of the ISTD 1% formic acid in acetonitrile solution to 100 µL of sample, vortex for ~15 seconds.

- Transfer entire volume to Phree plate and filter by centrifuge or positive pressure

- Transfer filtrate to LC vial for analysis (analysis for both the parent and metabolite compound).

Note - Refer to the worksheets “Supplies and Equipment”, “Reagents”, and “Stock Materials” of file 20170910 Intracellular Study - DCF .xlsx for descriptions of these items.

20170910 Intracellular Study

- Calibration Standards

Refer to worksheet Calibration Standards with IS in Excel file 20170910 Intracellular Study - CZX.xlsx

  - Prepare Solutions

    - Intermediate Stock CZX. Prepare CZX Intermediate Stock at ~ 5 mM with acetonitrile.

    - Mixed Intermediate Standard – 1. Prepare Mixed Intermediate Standard – 1 (MIS-1) at ~ 100 µM for each analyte by diluting stocks with acetonitrile.

    - Mixed Intermediate Standard – 2. Prepare MIS-2 at ~ 5 µM for each analyte by diluting 100 µL MIS-1 to 1000 µL with acetonitrile.

    - IS Mixed Intermediate Standard-1. Prepare IS Mixed Intermediate Standard-1 (ISMIS-1) by dilution with acetonitrile.

    - IS Mixed Intermediate Standard-2. Prepare IS Mixed Intermediate Standard-2 (ISMIS-2) labeled stocks and 120 µL ISMIS-1 by dilution with 1 % FA in acetonitrile.

Note – use the formic acid to prepare the 10 mL of 1% FA in ACN used for the blank in the selectivity Assessment.

    - Cal. Std. Spiking Solutions. Prepare 8 Calibration Standard Spiking Solutions by diluting the MIS-1 and MIS-2 with acetonitrile in 500 µL tubes. The spiking solutions are used to deliver a constant amount of acetonitrile during the preparation of the matrix matched standards.

Note – the 250 nM, 750 nM, and 2 µM Cal Std Spiking solutions are used to prepare QC standards as well.

  - Prepare Matrix Matched Standards

    - Aliquot 75 µL of the matrix to 500 µL centrifuge tubes

    - Add 300 µL of ISMIS-2

    - Prepare Matrix Matched Calibration Standards for analysis by adding 25 µL of the individual Calibration Standard Spiking Solutions. Prepare 4 replicates at the 250 nM, 750 nM, and 2 µM levels and 1 replicate at all others.

    - Vortex for ~15 seconds then centrifuge at 14000 RPM and 4°C for 10 minutes

    - Transfer to Phree plate

- Selectivity Assessment

Refer to worksheet Selectivity Assessment in Excel file 20170910 Intracellular Study - CZX.xls

  - Prepare 1% FA in ACN by adding 50 µL FA to 5 mL acetonitrile in a tube.

  - Prepare the Method Blank for analysis

    - Add 100 µL of matrix to 500 µL tube

    - Add 300 µL of IS Mixed Intermediate Standard-2

    - Vortex for ~15 seconds then centrifuge at 14000 RPM and 4°C for 10 minutes

    - Transfer to Phree plate

  - Prepare the Blank Matrix for analysis

    - Add 100 µL of matrix to 500 µL tube

    - Add 300 µL of 1% FA in acetonitrile and Vortex for ~15 seconds then centrifuge at 14000 RPM and 4°C for 10 minutes

    - Transfer to Phree plate

  - Prepare the Reagent Blank for analysis

    - Add 100 µL of water to 500 µL tube

    - Add 300 µL of 1% FA in acetonitrile and Vortex for ~15 seconds then centrifuge at 14000 RPM and 4°C for 10 minutes

    - Transfer to Phree plate

- Metabolite Sample Preparation

  - Transfer 100 µL of sample to 500 µL centrifuge tube (if required)

  - Add 300 µL of IS Mixed Intermediate Standard-2

  - Vortex for ~15 seconds then centrifuge at 14000 RPM and 4°C for 10 minutes

  - Transfer to Phree plate

- Phree Filtration

  - Filter by positive pressure

    - Place the Phree 96-Well Plate on top of a collection plate

    - Apply 2-5 psi using a positive pressure manifold.

  - Filter by centrifuge (alternativel to + pressure)

    - Place the PhreeTM 96-Well Plate on top of a collection plate and centrifuge at

    - 500 g for 5 minutes or until filtrate is collected

  - Transfer ~250 µL aliquot to LC vial for analysis

- Substrate Sample Preparation

Refer to worksheet Sample Dilutions in Excel file 20170910 Intracellular Study - CZX.xlsx

  - Dilute 7.5 mL of ISMS-2 with 2.5 mL of water for ISMIS-3

  - Dilute the sample extracts in LC vials with ISMIS-3

  - Vortex to mix