# RSEI Custom Data: Mexican Facilities with Impacts in California

## Method

Abt received a file on December 18, 2018 from Andrew Slocombe, “RETC2014_2016_emissions49km_finalForABT.xlsx,” containing emissions data from a set of Mexican facilities for years 2014-2016. The emissions were expressed as kg, except for one dioxin release expressed as grams.  All emissions were converted to pounds for modeling purposes.

Abt prepared an input table to run the RSEI air elements. Stack parameters for stack releases were taken from US industry-level medians, based on existing RSEI data and NAICS codes for each facility provided by OEHHA. Power plants were assigned special median parameters provided by the Electric Power Research Institute (EPRI) and assigned the source code “EPRI med”.  Industries without specific parameter data were assigned overall median values (source code “overall”). The proportion of stack versus fugitive releases was also taken from NAICS medians. Assigned facility parameters are listed in the input table.

The results for Mexican facilities were modeled using RSEI Version 2.3.7 code in 2019.  In February 2021, these results were combined with the most recent results for facilities located in California, which were modeled using RSEI version 2.3.9. Mexican data from 2014 were combined with California data for 2017; Mexican data for 2015 were combined with California data for 2018; and Mexican data for 2016 were combined with California data for 2019. Minor changes to the RSEI methodology have been made since the Mexican data were run in version 2.3.7. The changes that might produce slightly different results if the Mexican data were reprocessed with Version 2.3.9 include:

- Minor changes to toxicity weights (carbonyl sulfide, antimony, and antimony compounds).

- Update to NEI data (from the 2014 NEI plus a new approach) that might produce slightly different median stack parameters and stack/fugitive split.

Abt is providing a series of files to OEHHA, as described below. Additional data can be provided upon request.

## Files provided:

- Tract files- These files are the same as the aggregated Microdata files, but instead of being presented at the grid cell level, the values are averaged over Census tracts. Tract geographies are from the 2010 Census.  There are three sets of files, XXXX is the emissions  year:

  - CensusMicroTracts2019_YYYY_ca.zip - these are the baseline aggregated California results without the Mexican facilities, as reflected in the publicly released RY2019 (V2.3.9) RSEI Microdata.

  - CensusMicroTracts2017_ YYYY _camx06_aggregated.zip. These files contain the impacts from the Mexican facilities only. These are the same files that were included in the delivery in 2019.

  - CensusMicroTracts2019_YYYY_ca_plusmx.zip. These files contain the impacts from the Mexican facilities combined with the California baseline results.

# Appendix 1. Field Descriptions for Microdata Tables

These tables, along with the tables for the RSEI facility-level data, can also be found in the RSEI Data Dictionary.

| Tract Microdata |  |  |
| --- | --- | --- |
| Field Number | Name | Description |
| 1 | GEOID10 | US Census Tract ID |
| 2 | NUMFACS | Number of facilities impacting the tract. |
| 3 | P_NUMFACS | Percentile associated with field NUMFACS. |
| 4 | NUMRELEASES | Number of releases impacting the tract. |
| 5 | P_NUMRELEASES | Percentile associated with field NUMRELEASES. |
| 6 | NUMCHEMS | Number of chemicals impacting the tract. |
| 7 | P_ NUMCHEMS | Percentile associated with field NUMCHEMS. |
| 8 | TOXCONC | Average toxicity-weighted concentration of the cells in the tract. |
| 9 | PTOXCONC | Percentile associated with field TOXCONC. [NULL] |
| 10 | SCORE | Sum of the score (surrogate dose * toxicity weight * population) of the cells in the tract. |
| 11 | PSCORE | Percentile associated with field SCORE. [NULL] |
| 12 | POP | Average population of the cells in the tract, averaged over three years. |
| 13 | P_POP | Percentile associated with field POP. [NULL] |
| 14 | CSCORE | Sum of the scores (surrogate dose * toxicity weight * population) of the cells in the tract. CScore is calculated using only cancer toxicity weights. |
| 15 | P_CSCORE | Percentile associated with field CSCORE. [NULL] |
| 16 | NCSCORE | Sum of the scores (surrogate dose * toxicity weight * population) of the cells in the tract. NCScore is calculated using only noncancer toxicity weights. |
| 17 | P_NCSCORE | Percentile associated with field NCSCORE. [NULL] |
| 18 | NCSCORE | Sum of the scores (surrogate dose * toxicity weight * population) of the cells in the tract. NCScore is calculated using only noncancer toxicity weights. |
| 19 | P_NCSCORE | Percentile associated with field NCSCORE. [NULL] |

[updated 2/9/2021]