WQX 3.0 elements to be added to the WQX schema

Types of Impacts:

- Elements changed from Numeric to Text by naming convention and data type (low)

- Capture User Supplied names for WQX domain value list and element identifiers (low)

- Alignment with WQP Outbound elements (low)

- Optional DescriptionText and CommentText elements for existing schema definitions (low)

- Elements adopted from Biological Workgroup discussions (medium, conditional)

- Deprecated Elements and Business Rule modification (medium, element no longer valid)

| WQX Version | Elements |
| --- | --- |
| Schema v2.2 and below | - SourceMapScaleNumeric, IndexScoreNumeric, MetricScoreNumeric, SubstanceDilutionFactorNumeric, - ResultLaboratoryCommentCode**, SamplingComponentPlaceInSeriesNumeric**, ResultDetectionConditionText***, |
| Schema v3.0 | - SourceMapScale, IndexScore, MetricScore, SubstanceDilutionFactor, - AquiferTypeName*, NationalAquiferCode, AquiferInformation/LocalAquiferCode /LocalAquiferCodeContext /LocalAquiferName /LocalAquiferDescriptionText, ConstructionDate, ActivityIdentifierUserSupplied, ActivityLocationDescriptionText, CollectionDescriptionText, SampleContainerLabelName, HydrologicCondition*, HydrologicEvent*, CharacteristicNameUserSupplied, TargetCount, ProportionSampleProcessedNumeric, StatisticalNValueNumeric, ResultSamplingPointType*, ResultSamplingPointPlaceInSeries, ResultSamplingPointCommentText, RecordIdentifierUserSupplied, SubjectTaxonomicNameUserSupplied, SubjectTaxonomicNameUserSuppliedReferenceText, GroupSummaryCount, HabitatSelectionMethod*, ComparableAnalyticalMethod / MethodIdentifier*/ MethodContext / MethodModificationText, LaboratoryCommentText, DetectionQuantitationLimitCommentText, LaboratorySampleSplitRatio, - DrainageAreaMeasure/Value/Unit, ContributingDrainageAreaMeasure/Value/Unit, VerticalAccuracyMeasure/Value/Unit, WellDepthMeasure/Value/Unit, CollectionEffort/Value/GearProcedureUnitCode, CollectionArea/Value/Unit, GroupSummaryWeight/Value/Unit, - WQP Outbound ONLY: ResultConvertedMeasure/Value/Unit |

Note:   * Denotes From domain values list

** Deprecated, no longer valid in schema v3.0

*** Business rule has been changed significantly or removed

Fields to be added to the WQX schema:

| Schema entity | Element / Field | Definition | Required? | Datatype | Notes |
| --- | --- | --- | --- | --- | --- |
| MonitoringLocation MonitoringLocationIdentity | SourceMapScale | The relationship or ratio between distance on a map and corresponding distance on the ground. Example 1:100,000 where 1 cm = 1 km | Conditional (see business rule) | String max 60 non Negative Integer | Renamed elements designated as “numeric” SourceMapScale SourceMapScaleNumeric |
|  | AquiferTypeName | The type of aquifer, such as confined or unconfined. | Optional | From domain values String max 120 | WQP Outbound schema alignment |
|  | ConstructionDate | The date of construction, when well was completed. | Optional | Date | WQP Outbound schema alignment |
|  | DrainageAreaMeasure | Header; The surface drainage area of a lake, stream, wetland, or estuary site. | Optional; Up to one allowed per site |  | WQP Outbound schema alignment |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Optional | String max 60 |  |
|  | MeasureUnitCode | The code that represents the unit for measuring the item. | Optional | From domain values String max 12 |  |
|  | ContributingDrainageAreaMeasure | Header; The contributing subset of a drainage area for the lake, stream, wetland, or estuary site. | Optional; Up to one allowed per site |  | WQP Outbound schema alignment |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Optional | String max 60 |  |
|  | MeasureUnitCode | The code that represents the unit for measuring the item. | Optional | From domain values String max 12 |  |
|  | VerticalAccuracyMeasure | Header; The vertical measure of the relative accuracy of the elevation (i.e., the altitude). | Optional; Up to one allowed per site |  | WQP Outbound schema alignment |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Optional | String max 60 |  |
|  | MeasureUnitCode | The code that represents the unit for measuring the item. | Optional | From domain values String max 12 |  |
|  | NationalAquiferCode | A code that designates the principal aquifer associated with groundwater. | Optional | String max 120 | National aquifers, or principal aquifers of the United States, are defined as regionally extensive aquifers |
|  | AquiferInformation | Header; Designates the local or organizational aquifer associated with groundwater. | Optional; Up to one allowed per instance of WellInformation |  | WQP Outbound schema alignment |
|  | LocalAquiferCode | A code that designates the aquifer associated with groundwater samples. | Optional | String max 120 | A local aquifer code begins with a 2-character state abbreviation (such as TX for Texas) followed by a colon followed by the 7-character aquifer code. |
|  | LocalAquiferCodeContext | The code that Identifies the source or data system that created or defined the identifier | Optional | String max 30 | LOCAL, ORGANIZATION |
|  | LocalAquiferName | A name that designates the aquifer associated with groundwater samples. | Optional | String max 255 | USGS National Ground-Water Monitoring Network (NGWMN), local name |
|  | LocalAquiferDescriptionText | A description that designates the aquifer associated with groundwater samples. | Optional | String max 512 | Aquifer description |
|  | WellDepthMeasure | Header; Depth below land surface datum (LSD) to the bottom of the well inner casing. | Optional; Up to one allowed per instance of Well Information WellDepthMeasure VERSUS WellHoleDepthMeasure: Depth below land surface datum (LSD) to the bottom of the hole on completion of drilling. Domestic wells are only slightly larger than the well casing. |  | WQP Outbound schema alignment |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Optional | String max 60 |  |
|  | MeasureUnitCode | The code that represents the unit for measuring the item. | Optional | From domain values String max 12 |  |
| Biological Habitat Index | IndexScore | The score for the index. | Conditional | String max 60 String max 10 | Renamed elements designated as “numeric” IndexScoreNumeric IndexScore |
| Activity ActivityDescription | ActivityIdentifierUserSupplied | User Supplied ID that uniquely identifies an activity within an organization. WQX Business Rules may require additional naming conventions for sample identifiers | Optional | String max 55 | Key difference between ActID and ActIDUser Supplied is this free text matches the exact inhouse sample identifier |
| Activity ActivityLocation | SourceMapScale | The relationship or ratio between distance on a map and corresponding distance on the ground. Example 1:100,000 where 1 cm = 1 km | Conditional (see business rule) | String max 60 non Negative Integer | Renamed elements designated as “numeric” SourceMapScaleNumeric SourceMapScale |
|  | ActivityLocationDescriptionText | Text description of the activity location. | Optional | String max 4000 |  |
| BIOLOGICAL Habitat Collection Information 1st BIOLOGICAL Workgroup | CollectionArea | Header; The area of a collection procedure or protocol was performed (e.g. total area coverage for electrofishing, or total area kick net used). | Optional; Up to one allowed per instance of BiologicalHabitatColectionInformation |  | These fields capture effort type when it is constrained by area along with units. |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Optional | String max 60 |  |
|  | MeasureUnitCode | The code that represents the unit for measuring the item. | Optional | From domain values String max 35 |  |
| BIOLOGICAL Habitat Collection Information | CollectionEffort | Header; The number of individuals or equipment deployed to collect the sample. | Optional with strong guidance |  |  |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Optional | String max 60 |  |
|  | GearProcedureUnitCode | The procedural code or equipment that represents the unit for measuring the effort. | Optional | From domain values String max 35 | Example: nets |
| BIOLOGICAL Habitat Collection Information | CollectionDescriptionText | Remark / Text description of the reach. | Optional | String max 4000 |  |
| SAMPLE Prep | SampleContainerLabelName | The identification number or code assigned by the LAB or data collector. Sample Identification Codes and Labeling | Optional | String max 60 35 | Example: Chain of custody tracking |
| SAMPLE Description | HydrologicCondition | Hydrologic condition is the hydrologic condition that is represented by the sample collected (i.e.? falling stage, rising stage, peak stage) | Optional | From domain values String 60 | WQP Outbound schema alignment |
| SAMPLE Description | HydrologicEvent | A hydrologic event that is represented by the sample collected (i.e. - storm, drought, Flood, snowmelt, Hurricane). | Optional | From domain values String 60 | WQP Outbound schema alignment |
| ACTIVITY Metric | MetricScore | The calculated score for the activity metric. | Conditional (see business rule) | String max 60 String max 10 | Renamed elements designated as “numeric” MetricScoreNumeric MetricScore |
| Result ResultDescription | CharacteristicNameUserSupplied | The user supplied object, property, or substance which is evaluated or enumerated by either a direct field measurement, a direct field observation, or by laboratory analysis of material collected in the field. | Optional | String max 120 255 | New, key difference |
| 2nd BIOLOGICAL Workgroup | TargetCount | A code used to identify the intended count that the sorter was aiming for. | Conditionally required for benthic assemblages (see business rule) | From domain values String max 12 Allowable value list (100, 200, 300, 500, full) | The intended count that the sorter was aiming for. Actuals can be calculated by summing the counts. Full == counted the entire sample |
| Naming convention required by exchange network review 3rd BIOLOGICAL Workgroup | ProportionSampleProcessedNumeric | This field captures the proportion of the sample processed. Proportion is stored as a number between 0 and 1. Example Large/rare count would be documented as 1 (100%) | Conditionally required for benthic assemblages (see business rule) - Required when assemblage is "Benthic Macroinvertebrates". - Must be a positive number. - If value is equal to 0.000000001 then the value will be set to NULL. | Non-Negative Decimal | For example, 0.5 equals 50% Numeric-only, number between 0 and 1 |
| Existing Element Business Rule UPDATE/modified | ResultDetectionConditionText | The textual descriptor of a result. | Conditional: (see business rule) Optional Allow both ResultMeasureValue and “condition” for a result | From domain values String max 35 | Example: Present Above Quantitation, 0.5 mg/l |
| Deprecated element: ResultLaboratoryCommentCode Guidance use instead, existing element | MeasureQualifierCode | A code used to identify any qualifying issues that affect the results. | Optional (May have 0 to 6 for each record). | From domain values string max 35 | Duplicate elements in previous schema v2.x Converted a single string to a list of comma delimited codes. Example: “UJCNT” <MeasureQualifierCode>U</MeasureQualifierCode> <MeasureQualifierCode>J</MeasureQualifierCode> <MeasureQualifierCode>CNT</MeasureQualifierCode> |
| Naming convention required by exchange network review | StatisticalNValueNumeric | The number of repeated measurements taken to calculate the result value as an average. | Optional | Non-Negative Integer |  |
|  | ResultSamplingPointType | Identifies the type of sampling point within a sampling frame or position that is associated with the reported result. | Optional | From domain values String max 60 | Example domain values: Line Transect. Point Transect. Strip Transect. Belt Transect. |
| Deprecated element: SamplingComponentPlaceInSeriesNumeric | ResultSamplingPointPlaceInSeries | The order in which a single point within a sampling frame was visited in relation to other components. | Optional | String max 60 | New, Moved from activity to result level and deprecating the existing activity element |
| Result Description | ResultSamplingPointCommentText | Text description of a single point within a sampling frame for the result. | Optional | String max 4000 |  |
|  | RecordIdentifierUserSupplied | The user supplied record identifier associated with data entered. | Optional | String max 60 | QAQC checks |
| BiologicalResultDescription | SubjectTaxonomicNameUserSupplied | The user supplied name of the organism sampled as part of a biological sample. | Optional | String max 255 |  |
|  | SubjectTaxonomicNameUserSuppliedReferenceText | Identifies the source or data system that created or defined the identifier. | Optional | String max 255 |  |
| Deprecated element: GroupSummaryCountWeight | GroupSummaryCount | Captures the total count for a Group Summary | Conditional (see business rules) | String max 60 | Split one into two: add new element. |
| Deprecated element: GroupSummaryCountWeight | GroupSummaryWeight | Header: Captures the total sample weight for a Group Summary | Conditional (see business rules) |  | Renamed GroupSummaryCountWeight to GroupSummaryWeight |
| Consisting of 2 elements: | MeasureValue | The recorded dimension, capacity, quality, or amount of something ascertained by measuring or observing. | Conditional (see business rules) | String max 60 |  |
|  | MeasureUnitCode | The code that represents the unit for measuring the item. | Conditional (see business rules) | From domain values String max 12 |  |
| 4th BIOLOGICAL Workgroup | HabitatSelectionMethod | This captures how and which habitat to sample was chosen. | Conditionally required for benthic assemblages (see business rule) (Guidance will say it is conditionally required for benthic assemblages and river/stream water type) | From domain values String max 35 Directed, Random, Systematic | This field captures how and which habitat to sample was chosen. Directed - I want to sample this because it is a desirable habitat Systematic - I want to sample this because it is a meter from the bank Random - I want to sample here because it was randomly chosen |
| Result Analytical Method Provides metadata for data analysis. When referencing Organizational methods | ComparableAnalyticalMethod | Header; Allows for the optional association of a nationally comparable analytical method to an employed organization analytical method with any result. | Optional; may have 0 can have 1 per Result |  |  |
| Result Analytical Method Consisting of 3 elements: | MethodIdentifier | The identification number or code assigned by the “national” method publisher. | Optional | From domain values String max 35 (ONLY National methods maybe assigned) | self -documenting So, when two methods are compared to the same National methods. Both may have a comparison to each other for analysis. |
| Result Analytical Method Consisting of 3 elements: | MethodContext | Identifies the source or data system that created or defined the identifier. | Conditional (see business rules) | From domain values String max 120 | self -documenting |
| Result Analytical Method Consisting of 3 elements: | MethodModificationText | A brief summary that provides general information about the method. | Optional | String max 4000 | Provides metadata for data analysis. |
|  | LaboratoryCommentText | Remarks which further describe the laboratory procedures that produced the result. | Optional | String max 4000 |  |
|  | DetectionQuantitationLimitCommentText | Text providing further description and comment on the detection and/or quantitation limits. | Optional | String max 4000 |  |
|  | LaboratorySampleSplitRatio | The proportion of all material collected that was sent to lab for analysis. | Optional | String max 60 |  |
| LAB Sample Prep | SubstanceDilutionFactor | The overall dilution of the substance subjected to this analysis. | Optional | String max 60 String max 10 | Renamed elements designated as “numeric” SubstanceDilutionFactorNumeric SubstanceDilutionFactor |