- INTRODUCTION

This document describes the development of the three modules used in the Hybrid Roadway Intersection Model (HYROAD) – Traffic, Emission, and Dispersion.  HYROAD is designed to predict the concentrations of carbon monoxide (CO) that occur near intersections.  Each module describes one of the key aspects that govern the magnitude of these concentrations.  The modules are also linked to provide a detailed treatment of the traffic behavior, vehicle emissions, and the effects of meteorology on concentration patterns within 500 meters of the intersection.

This document has been developed to assist the user of HYROAD to understand the model.  Sections 2-4 describe the formulation of each of the three modules used in HYROAD.  In Section 5, a model verification study was performed for a number of roadway intersections.  In Section 6, a sensitivity analysis was conducted for typical changes to model inputs to determine model response behavior.

# 1.1	BACKGROUND

In response to concerns about high air pollution levels near major intersections, state and federal regulators began conducting roadway air quality assessments in the 1970s.  Of primary interest was determining the CO concentrations caused by vehicle activities (idling, accelerations/decelerations) that occur at and near roadway intersection “hot-spots.”  Also of interest was the impact of intersection traffic control devices (e.g., signal timing) on traffic flow and vehicle emissions.  While intersection monitoring and modeling continues today with more advanced analytical tools, the primary objectives of this work remain the same.

Understanding intersection air quality requires knowledge of the inter-relationship between several key parameters: the traffic environment, vehicle emissions, and the effects of atmospheric dispersion processes on pollution concentrations.  For each of these key parameters, detailed information must be known:

- Traffic environment: roadway geometry, traffic control devices, pedestrian activity, turning movement, and traffic volumes entering the intersection.

- Vehicle emissions: vehicle fleet composition, vehicle age distribution, vehicle activity patterns (idle, acceleration/deceleration), vehicle speeds, temperature, and fuel type (oxygenated, reformulated).

- Meteorology/pollution dispersion: wind speeds, wind direction, surface roughness, and atmospheric stability

To perform an intersection air quality assessment, detailed information is needed on the inter-relationship of these parameters at many discrete points in time.  Computer simulation models have been developed to simulate various aspects of the roadway traffic environment and air quality processes.  Most models, however, have focused on only one of the three elements required for intersection air quality modeling (traffic, vehicle emissions, or pollution dispersion), but not all three.  As a result, most of the modeling systems developed to date are inadequate for conducting detailed intersection air quality modeling.

In order to conduct roadway intersection pollution studies, many researchers have had to work with individual traffic and dispersion models that weren’t designed to function as an integrated whole.  The problems encountered working with separate and independent models include: difficulties in linking input/outputs from one module to another; inconsistencies in model behavior from changes in input values; inadequacy in simulating all of the relevant and important conditions.  As a result, the accuracy and sensitivity of modeling exercises may be difficult to establish.  For example, intersection models have typically relied on user-supplied estimates of average speed and idle time to provide emission estimates.  Measurements conducted as part of this project indicate that the use of average speed for emission estimates obscures the considerable variation of travel speeds and their associated emission distribution on air concentrations (Carr and Ireson, 1997).  Similarly, at low ambient wind speeds the movement of vehicles is of significant influence in changing the dispersion and transport of air.

The integrated air quality dispersion model developed in this project addresses these issues.  The modeling system described herein integrates each of the separate modules to a greater extent so that they can more easily be run as a single system, and the parameters and variables carried in each of the modules have been selected with the recognition of the formulation and needs of the other modules.  The integration of the modules has allowed for ease in creating the detailed input files required.  Important input parameters are created and formatted in previous modules, thereby eliminating the need for post-processing of output data.  For example, the traffic module of the integrated model is used to calculate detailed information on vehicle dynamics, such as vehicle speed and acceleration distributions, thereby eliminating the need to rely on average speeds.  This detailed information is then used by the emission and dispersion modules to more accurately estimate emissions and their spatial and temporal allocation.

The modular structure of the integrated air quality model is shown in Figure 1.  The design of the individual modules in the system was based on analyses of data collected in intensive measurement programs at three intersections (Carr and Ireson, 1997).  Rather than attempt to develop an entirely new model, the data analysis was designed to provide an independent basis for evaluating the required capabilities of each module, and the performance of existing models or model formulations.  Thus, the three modules are built from existing algorithms and software, but are more directly linked, and utilize several new approaches to more accurately describe critical processes.  This approach offered several advantages.  Computer simulation models are complex descriptions of real world environments, and as such, they are time consuming and expensive to develop.  Furthermore, new models must be validated by comparing simulation results with data collected in the field.  Full validation activities for new models are both time consuming and costly.

Most of the modules selected for the integrated air quality intersection model have been extensively applied and validated.  TRAF-NETSIM was chosen as the core element of the traffic module for generating the traffic information needed by the emission and dispersion modules.  Internal code modifications track vehicle speed and acceleration distributions by signal phase and 10-meter roadway segment for use in both emissions calculations and calculation of induced flows and turbulence.  Emission factors from current mandated models (MOBILE5 or MOBILE6) are used as inputs, but speed distributions from the traffic module are used in a regression analysis for each time period to calculate composite emission factors whose underlying speed distribution best fits current conditions.  Spatial and temporal distribution of emissions is based on the vehicle operation predictions rather than simply being uniformly distributed.  A Lagrangian (trajectory-based) puff formulation was developed for use with a gridded non-uniform wind and stability field derived from traffic module outputs, to describe near-roadway dispersion.

# 1.2	HOW TO USE THIS MANUAL

This document provides the formulation information for the individual modules that compose the HYROAD model.  The manual pulls pertinent material from the user documentation for the TRAF User Reference Guide (FHWA, 1995).  Section 2 of this model formulation manual includes material from the TRAF-NETSIM documentation and information on any changes that have been made to the model.  Where necessary, the reader is referred to the original model documentation for more detailed discussion.

After having developed a basic understanding of how to use the modeling system from the User’s Guide, the user should review Sections 2-4 of the model formulation document.  A model verification and sensitivity analysis are presented in Sections 5 and 6 of this manual to assist the user in understanding the capabilities, performance and limitations of the modeling system.

The model includes the option to use MOBILE6 emission factors, although the model was tested using MOBILE5.