# Appendix C:

Basic Physicochemical Properties of Individual Gaseous

Hazardous Air Pollutants and Mixtures

Table C.1.  Basic physicochemical properties of individual hazardous air pollutants at 298.2 K.

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Acetaldehyde | 75-07-0 | C2H4O | 44.05 | -123.0 | 20.1 | 47.2 | 51.8 | 0.1281 | 1.450 |
| 2 | Acetamide | 60-35-5 | C2H5NO | 59.07 | 81.0 | 222.0 | 54.0 | 55.5 | 0.1153 | 1.402 |
| 3 | Acetonitrile | 75-05-8 | C2H3N | 41.05 | -43.8 | 81.6 | 43.3 | 56.3 | 0.1342 | 1.393 |
| 4 | Acetophenone | 98-86-2 | C8H8O | 120.15 | 20.0 | 202.0 | 133.5 | 140.4 | 0.07241 | 0.8054 |
| 5 | 2-Acetylamino- fluorene | 53-96-3 | C15H13NO | 223.28 | 193.0 | NAa | 224.3 | 248.0 | 0.05473 | 0.4807 |
| 6 | Acrolein | 107-02-8 | C3H4O | 56.06 | -87.7 | 52.6 | 63.1 | 66.6 | 0.1094 | 1.279 |
| 7 | Acrylamide | 79-06-1 | C3H5NO | 71.08 | 84.5 | 192.6 | 69.9 | 80.8 | 0.1012 | 1.151 |
| 8 | Acrylic acid | 79-10-7 | C3H4O2 | 72.06 | 12.3 | 141.0 | 69.2 | 83.2 | 0.1014 | 1.132 |
| 9 | Acrylonitrile | 107-13-1 | C3H3N | 53.06 | -83.5 | 77.3 | 59.2 | 71.1 | 0.1133 | 1.235 |
| 10 | Allyl chloride | 107-05-1 | C3H5Cl | 76.53 | -134.5 | 45.1 | 80.3 | 87.5 | 0.09463 | 1.099 |
| 11 | 4-Aminobiphenyl | 92-67-1 | C12H11N | 169.23 | 53.5 | 302.0 | 184.2 | 198.8 | 0.06097 | 0.6032 |
| 12 | Aniline | 62-53-3 | C6H7N | 93.13 | -6.0 | 184.1 | 97.8 | 110.2 | 0.08504 | 0.9533 |
| 13 | o-Anisidine | 90-04-0 | C7H9NO | 123.16 | 6.2 | 224.0 | 124.4 | 143.4 | 0.07449 | 0.7928 |
| 14 | Benzene | 71-43-2 | C6H6 | 78.11 | 5.5 | 80.0 | 91.0 | 96.0 | 0.08962 | 1.040 |
| 15 | Benzidine | 92-87-5 | C12H12N2 | 184.24 | 120 | 401.0 | 191.0 | 213.0 | 0.05962 | 0.5644 |
| 16 | Benzotrichloride | 98-07-7 | C7H5Cl3 | 195.48 | -5.0 | 221.0 | 167.6 | 180.9 | 0.06299 | 0.6568 |
| 17 | Benzyl chloride | 100-44-7 | C7H7Cl | 126.59 | -45.0 | 179.0 | 130.2 | 124.3 | 0.07284 | 0.8791 |
| 18 | Biphenyl | 92-52-4 | C12H10 | 154.21 | 69.0 | 256.1 | 177.3 | 199.6 | 0.06247 | 0.6009 |
| 19 | Bis(chloromethyl) ether | 542-88-1 | C2H4Cl2O | 114.96 | -41.5 | 106.0 | 89.2 | 102.7 | 0.08636 | 0.9974 |
| 20 | Bis(2-ethylhexyl) phthalate | 117-81-7 | C24H38O4 | 390.57 | -55.0 | 384.0 | 475.5 | 524.8 | 0.03724 | 0.1002 |
| 21 | Bromoform | 75-25-2 | CHBr3 | 252.75 | 8.0 | 149.1 | 83.9 | 99.5 | 0.08357 | 1.017 |
| 22 | 1,3-Butadiene | 106-99-0 | C4H6 | 54.09 | -108.9 | -4.4 | 77.5 | 81.4 | 0.1013 | 1.146 |
| 23 | Caprolactan | 105-60-2 | C6H11NO | 113.16 | NA | NA | 113.2 | 133.9 | 0.07823 | 0.8336 |
| 24 | Captan | 133-06-2 | C9H8Cl3NO2S | 300.59 | 172.5 | NA | 227.6 | 262.5 | 0.05355 | 0.4501 |
| 25 | Carbaryl | 63-25-2 | C12H11NO2 | 201.22 | 145.0 | NA | 196.4 | 220.8 | 0.05853 | 0.5445 |

Table C.1.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | Carbon disulfide | 75-15-0 | CS2 | 76.14 | -111.5 | 46.0 | 61.7 | 66.0 | 0.1053 | 1.285 |
| 27 | Carbon tetrachloride | 56-23-5 | CCl4 | 153.82 | -23.0 | 76.8 | 99.9 | 113.2 | 0.08024 | 0.9366 |
| 28 | Carbonyl sulfide | 463-58-1 | COS | 60.07 | -138.8 | -50.0 | 44.9 | 48.7 | 0.1234 | 1.493 |
| 29 | Catechol | 120-80-9 | C6H6O2 | 110.11 | 105.0 | 245.0 | 103.2 | 110.8 | 0.08159 | 0.9499 |
| 30 | Chloramben | 133-90-4 | C7H5Cl2NO2 | 206.03 | 200.0 | NA | 163.3 | 180.3 | 0.06351 | 0.6587 |
| 31 | cis-Chlordane | 5103-71-9 | C10H6Cl8 | 409.78 | 108 | NA | 286.0 | 332.5 | 0.04756 | 0.3256 |
| 32 | trans-Chlordane | 5103-74-2 | C10H6Cl8 | 409.78 | 104 | NA | 286.0 | 332.5 | 0.04756 | 0.3256 |
| 33 | Chloroacetic acid | 79-11-8 | C2H3O2Cl | 94.50 | 63.0 | 189.3 | 72.0 | 89.3 | 0.09631 | 1.086 |
| 34 | 2-Chloroacetophenone | 532-27-4 | C8H7OCl | 154.60 | 56.5 | 247.0 | 152.2 | 161.3 | 0.06686 | 0.7233 |
| 35 | Chlorobenzene | 108-90-7 | C6H5Cl | 112.56 | -45.2 | 131.7 | 109.7 | 116.9 | 0.07933 | 0.9167 |
| 36 | Chlorobenzilate | 510-15-6 | C16H14Cl2O3 | 325.19 | 37.0 | 157.0 (0.07 mm Hg) | 310.5 | 337.2 | 0.04613 | 0.3184 |
| 37 | Chloroform | 67-66-3 | CHCl3 | 119.38 | -63.6 | 61.1 | 81.2 | 92.3 | 0.08944 | 1.065 |
| 38 | Chloromethyl methyl ether | 107-30-2 | C2H5OCl | 80.51 | -103.5 | 59.5 | 70.5 | 81.8 | 0.09910 | 1.143 |
| 39 | Chloroprene | 126-99-8 | C4H5Cl | 88.54 | -130.0 | 59.4 | 96.2 | 102.3 | 0.08615 | 0.9996 |
| 40 | o-Cresol | 95-48-7 | C7H8O | 108.14 | 29.8 | 191.0 | 117.6 | 125.6 | 0.07732 | 0.8727 |
| 41 | m-Cresol | 108-39-4 | C7H8O | 108.14 | 11.8 | 202.2 | 117.6 | 125.6 | 0.07732 | 0.8727 |
| 42 | p-Cresol | 106-44-5 | C7H8O | 108.14 | 35.5 | 201.9 | 117.6 | 125.6 | 0.07732 | 0.8727 |
| 43 | o-Cresyl acetate | 533-18-6 | C9H10O2 | 150.18 | NA | 208.0 | 160.1 | 177.2 | 0.06552 | 0.6685 |
| 44 | Cumene | 98-82-8 | C9H12 | 120.20 | -96.0 | 152.4 | 152.5 | 162.6 | 0.06830 | 0.7186 |
| 45 | Diazomethane | 334-88-3 | CH2N2 | 42.04 | -145.0 | -23.0 | 29.6 | 53.4 | 0.1535 | 1.429 |
| 46 | Dibenzofuran | 132-64-9 | C12H8O | 168.2 | 86.5 | 287.0 | 160.5 | 176.7 | 0.06489 | 0.6702 |
| 47 | 1,2-Dibromo-3-chloropropane | 96-12-8 | C3H5ClBr2 | 236.34 | NA | 196.0 | 124.1 | 141.5 | 0.07110 | 0.8005 |
| 48 | 2,4-Dichloro-phenoxyacetic acid | 94-75-7 | C8H6O3Cl2 | 221.04 | 140.5 | 160.0 (0.4 mm Hg) | 183.1 | 209.8 | 0.06008 | 0.5727 |
| 49 | 1,1-Dichloro-2,2-bis(p-chlorophenyl)ethylene | 72-55-9 | C14H8Cl4 | 318.03 | 89.0 | NA | 288.5 | 305.2 | 0.04782 | 0.3701 |
| 50 | Dibutyl phthalate | 84-74-2 | C16H22O4 | 278.35 | -35.0 | 340.0 | 311.4 | 347.2 | 0.04639 | 0.3034 |

Table C.1.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 1,4-Dichlorobenzene | 106-46-7 | C6H4Cl2 | 147.00 | 52.7 | 174.0 | 128.3 | 137.8 | 0.07236 | 0.8164 |
| 52 | 3,3'-Dichlorobenzidine | 84-68-4 | C12H10N2Cl2 | 253.13 | 165.0 | NA | 228.4 | 265.0 | 0.05390 | 0.4448 |
| 53 | Dichloroethyl ether | 111-44-4 | C4H8Cl2O | 143.01 | -51.9 | 178.5 | 130.2 | 147.9 | 0.07206 | 0.7742 |
| 54 | trans-1,3-Dichloropropene | 10061-02-6 | C3H4Cl2 | 110.97 | NA | 112.0 | 98.9 | 108.4 | 0.08301 | 0.9633 |
| 55 | Dichlorvos | 62-73-7 | C4H7Cl2O4P | 220.98 | NA | 140.0 (20 mm Hg) | 169.1 | 193.1 | 0.06227 | 0.6194 |
| 56 | Diethanolamine | 111-42-2 | C4H11NO2 | 105.14 | 28.0 | 268.8 | 105.8 | 126.7 | 0.08114 | 0.8672 |
| 57 | Diethyl sulfate | 64-67-5 | C4H10SO4 | 154.19 | -24.0 | 208.0 | 134.0 | 155.0 | 0.07073 | 0.7465 |
| 58 | Diethylene glycol monobutyl ether | 112-34-5 | C8H18O3 | 162.23 | -68.0 | 231.0 | 187.1 | 213.3 | 0.06073 | 0.5635 |
| 59 | 3,3'-Dimethoxybenzidine | 119-90-4 | C14H16N2O2 | 244.30 | 137.0 | NA | 244.3 | 279.4 | 0.05234 | 0.4166 |
| 60 | 4-Dimethylamino- azobenzene | 60-11-7 | C14H15N3 | 225.30 | 117.0 | NA | 234.3 | 275.9 | 0.05325 | 0.4233 |
| 61 | N,N-Dimethylaniline | 121-69-7 | C8H11N | 121.18 | 2.4 | 194.1 | 138.9 | 156.1 | 0.07111 | 0.7423 |
| 62 | 3,3'-Dimethylbenzidine | 119-93-7 | C14H16N2 | 212.30 | 131.5 | NA | 232.0 | 257.4 | 0.05404 | 0.4605 |
| 63 | Dimethylcarbamyl chloride | 79-44-7 | C3H6NOCl | 107.54 | -33.0 | 167.0 | 93.2 | 110.6 | 0.08538 | 0.9508 |
| 64 | N,N-Dimethylformamide | 68-12-2 | C3H7NO | 73.10 | -60.4 | 153.0 | 74.5 | 89.7 | 0.09820 | 1.083 |
| 65 | 1,1-Dimethylhydrazine | 57-14-7 | C2H8N2 | 60.10 | -58.0 | 63.9 | 59.4 | 90.4 | 0.1108 | 1.078 |
| 66 | Dimethyl phthalate | 131-11-3 | C10H10O4 | 194.19 | 5.5 | 283.7 | 188.2 | 206.4 | 0.05981 | 0.5819 |
| 67 | Dimethyl sulfate | 77-78-1 | C2H6SO4 | 126.13 | -27.0 | 76.0 (15 mm Hg) | 93.0 | 110.6 | 0.08411 | 0.9508 |
| 68 | 4,6-Dinitro-o-cresol | 534-52-1 | C7H6N2O5 | 198.14 | 86.5 | NA | 146.5 | 172.4 | 0.06681 | 0.6844 |
| 69 | 2,4-Dinitrophenol | 51-28-5 | C6H4N2O5 | 184.11 | 115.5 | sublimes | 126.0 | 150.2 | 0.07171 | 0.7650 |
| 70 | 2,4-Dinitrotoluene | 121-14-2 | C7H6N2O4 | 182.14 | 71.0 | 300.0 decomposes | 140.4 | 165.0 | 0.06845 | 0.7098 |
| 71 | 1,4-Dioxane | 123-91-1 | C4H8O2 | 88.11 | 11.8 | 101.5 | 76.0 | 93.6 | 0.09502 | 1.056 |
| 72 | 1,2-Diphenylhydrazine | 122-66-7 | C12H12N2 | 184.24 | 131.0 | NA | 191.0 | 216.0 | 0.05962 | 0.5565 |
| 73 | Epichlorohydrin | 13403-37-7 | C3H5ClO | 92.53 | -26.0 | 117.0 | 68.1 | 88.9 | 0.09874 | 1.089 |
| 74 | 1,2-Epoxybutane | 106-88-7 | C4H8O | 72.11 | -150.0 | 63.3 | 69.9 | 90.2 | 0.1010 | 1.080 |
| 75 | Ethyl acrylate | 140-88-5 | C5H8O2 | 100.12 | -71.2 | 99.4 | 110.2 | 123.4 | 0.08017 | 0.8833 |

Table C.1.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 76 | Ethylbenzene | 100-41-4 | C8H10 | 106.17 | -94.9 | 136.1 | 132.0 | 140.4 | 0.07369 | 0.8052 |
| 77 | Ethyl carbamate | 51-79-6 | C3H7NO2 | 89.09 | 49.0 | 185.0 | 80.6 | 100.6 | 0.09264 | 1.010 |
| 78 | Ethyl chloride | 75-00-3 | C2H5Cl | 64.52 | -138.7 | 12.3 | 64.4 | 72.7 | 0.1061 | 1.220 |
| 79 | Ethylene dibromide | 106-93-4 | C2H4Br2 | 187.87 | 9.9 | 131.6 | 84.8 | 98.4 | 0.08467 | 1.024 |
| 80 | Ethylene dichloride | 107-06-2 | C2H4Cl2 | 98.96 | -35.5 | 83.5 | 83.0 | 93.6 | 0.09040 | 1.056 |
| 81 | Ethylene glycol | 107-21-1 | C2H6O2 | 62.07 | -13.0 | 197.3 | 57.9 | 66.6 | 0.1113 | 1.279 |
| 82 | Ethylene glycol monobutyl ether | 111-76-2 | C6H14O2 | 118.18 | -74.8 | 168.4 | 140.0 | 159.0 | 0.07104 | 0.7315 |
| 83 | Ethyleneimine | 151-56-4 | C2H5N | 43.07 | -77.9 | 56.0 | 47.9 | 63.7 | 0.1279 | 1.309 |
| 84 | Ethylene oxide | 75-21-8 | C2H4O | 44.05 | -111.7 | 10.6 | 28.9 | 45.8 | 0.1534 | 1.535 |
| 85 | Ethylene thiourea | 96-45-7 | C3H6N2S | 102.16 | 131.0 | NA | 93.5 | 118.3 | 0.08574 | 0.9092 |
| 86 | Ethylidene dichloride | 75-34-3 | C2H4Cl2 | 98.96 | -96.9 | 57.4 | 83.0 | 93.6 | 0.09040 | 1.056 |
| 87 | Formaldehyde | 50-00-0 | CH2O | 30.03 | -92.0 | -19.1 | 26.6 | 29.6 | 0.1720 | 1.845 |
| 88 | Heptachlor | 76-44-8 | C8H5Cl7 | 373.30 | 95.5 | NA | 230.9 | 274.6 | 0.05274 | 0.4258 |
| 89 | Hexachlorobenzene | 118-74-1 | C6Cl6 | 284.78 | 231.8 | 325.0 | 203.1 | 221.4 | 0.05657 | 0.5428 |
| 90 | Hexachlorobutadiene | 87-68-3 | C4Cl6 | 260.76 | -21.0 | 215.0 | 189.6 | 206.8 | 0.05861 | 0.5808 |
| 91 | (α)1,2,3,4,5,6-Hexachlorocyclohexane | 60291-32-9 | C6H6Cl6 | 290.83 | 159.5 | 288.0 | 217.0 | 243.6 | 0.05482 | 0.4903 |
| 92 | (γ)1,2,3,4,5,6-Hexachlorocyclohexane | 58-89-9 | C6H6Cl6 | 290.83 | 112.5 | 323.4 | 217.0 | 243.6 | 0.05482 | 0.4903 |
| 93 | Hexachloro- cyclopentadiene | 77-47-4 | C5Cl6 | 272.77 | -9 | 239.0 | 187.2 | 210.1 | 0.05882 | 0.5719 |
| 94 | Hexachloroethane | 67-72-1 | C2Cl6 | 236.74 | 187.0 (tripl. pt.) | NA | 157.8 | 177.2 | 0.06397 | 0.6685 |
| 95 | Hexamethylene diisocyanate | 822-06-0 | C8H12N2O2 | 168.20 | NA | 255.0 | 176.2 | 208.8 | 0.06223 | 0.5754 |
| 96 | Hexamethyl- phosphoramide | 680-31-9 | C6H18N3OP | 179.20 | 7.0 | 232.5 | 179.6 | 225.3 | 0.06142 | 0.5332 |
| 97 | Hexane | 110-54-3 | C6H14 | 86.18 | -95.3 | 68.7 | 127.7 | 140.6 | 0.07660 | 0.8043 |
| 98 | Hydrazine | 302-01-2 | N2H4 | 32.05 | 1.0 | 113.6 | 18.3 | 35.8 | 0.1922 | 1.709 |
| 99 | Hydroquinone | 123-31-9 | C6H6O2 | 110.11 | 172.3 | 287.0 | 103.2 | 110.8 | 0.08159 | 0.9497 |
| 100 | Isophorone | 78-59-1 | C9H14O | 138.21 | -8.1 | 215.2 | 163.3 | 177.4 | 0.06541 | 0.6679 |

Table C.1.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | Maleic anhydride | 108-31-6 | C4H2O3 | 98.06 | 52.8 | 202 | 68.3 | 79.8 | 0.09796 | 1.159 |
| 102 | Methanol | 67-56-1 | CH4O | 32.04 | -97.6 | 64.6 | 31.3 | 37.0 | 0.1598 | 1.686 |
| 103 | Methoxychlor | 72-43-5 | C16H15O2Cl3 | 345.66 | 87.0 |  | 327.7 | 358.1 | 0.04485 | 0.2877 |
| 104 | Methyl bromide | 74-83-9 | CH3Br | 94.94 | -93.7 | 3.5 | 44.7 | 52.9 | 0.1153 | 1.435 |
| 105 | Methyl chloride | 74-87-3 | CH3Cl | 50.49 | -97.7 | -24.0 | 43.8 | 50.5 | 0.1284 | 1.467 |
| 106 | Methyl chloroform | 71-55-6 | C2H3Cl3 | 133.41 | -30.4 | 74.0 | 101.7 | 114.5 | 0.08059 | 0.9293 |
| 107 | Methyl ethyl ketone | 78-93-3 | C4H8O | 72.11 | -86.7 | 79.6 | 88.2 | 96.2 | 0.09180 | 1.039 |
| 108 | Methylhydrazine | 60-34-4 | CH6N2 | 46.07 | -52.4 | 87.5 | 38.8 | 59.5 | 0.1367 | 1.355 |
| 109 | Methyl iodide | 74-88-4 | CH3I | 141.94 | -66.4 | 42.5 | 52.6 | 62.9 | 0.1047 | 1.317 |
| 110 | Methyl isobutyl ketone | 108-10-1 | C6H12O | 100.16 | -84.0 | 116.5 | 129.2 | 140.6 | 0.07486 | 0.8043 |
| 111 | Methyl isocyanate | 624-83-9 | C2H3NO | 57.05 | -45.0 | 39.5 | 49.4 | 63.7 | 0.1200 | 1.309 |
| 112 | Methyl methacrylate | 80-62-6 | C5H8O2 | 100.12 | -48.0 | 100.5 | 110.2 | 121.8 | 0.08017 | 0.8913 |
| 113 | Methyl tert-butyl ether | 1634-04-4 | C5H12O | 88.15 | -108.6 | 55.2 | 113.3 | 129.4 | 0.08043 | 0.8544 |
| 114 | 4,4'-Methylenebis (2-chloroaniline) | 101-14-4 | C13H12N2Cl2 | 267.16 | NA | NA | 248.9 | 277.0 | 0.05166 | 0.4212 |
| 115 | Methylene chloride | 75-09-2 | CH2Cl2 | 84.93 | -95.1 | 40.0 | 62.5 | 71.4 | 0.1033 | 1.232 |
| 116 | 4,4'-Methylenediphenyl diisocyanate | 101-68-8 | C15H10N2O2 | 250.26 | NA | NA | 246.3 | 275.0 | 0.05208 | 0.4250 |
| 117 | 4,4'-Methylenedianiline | 101-77-9 | C13H14N2 | 198.27 | 92.5 | 398 | 211.5 | 235.2 | 0.05664 | 0.5095 |
| 118 | Naphthalene | 91-20-3 | C10H8 | 128.18 | 80.2 | 217.9 | 140.9 | 147.6 | 0.07029 | 0.7754 |
| 119 | Nitrobenzene | 98-95-3 | C6H5NO2 | 123.11 | 5.7 | 210.8 | 105.4 | 124.5 | 0.07997 | 0.8778 |
| 120 | 4-Nitrobiphenyl | 92-93-3 | C12H9NO2 | 199.21 | 114 | 340.0 | 191.8 | 208.0 | 0.05920 | 0.5776 |
| 121 | 4-Nitrophenol | 100-02-7 | C6H5NO3 | 139.11 | 113.8 | 279.0 | 111.5 | 126.8 | 0.07722 | 0.8697 |
| 122 | 2-Nitropropane | 79-46-9 | C3H7NO2 | 89.09 | -91.3 | 120.2 | 80.6 | 97.4 | 0.09264 | 1.031 |
| 123 | N-Nitroso-N-methylurea | 684-93-5 | C2H5N3O2 | 103.08 | 123.5 decomposes | NA | 69.2 | 101.9 | 0.09691 | 1.002 |
| 124 | N-Nitrosodimethylamine | 62-75-9 | C2H6N2O | 74.08 | NA | 154.0 | 60.9 | 87.7 | 0.1063 | 1.098 |
| 125 | N-Nitrosomorpholine | 59-89-2 | C4H8N2O2 | 116.12 | 29.0 | 225.0 | 85.1 | 135.7 | 0.08797 | 0.8256 |

Table C.1.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 126 | Parathion | 56-38-2 | C10H14NO5PS | 291.26 | 6.1 | 375.0 | 253.9 | 288.0 | 0.05097 | 0.4005 |
| 127 | Pentachloro- nitrobenzene | 82-68-8 | C6Cl5NO2 | 295.34 | 144.0 | 328.0 decomposes | 198.9 | 223.9 | 0.05702 | 0.5366 |
| 128 | Pentachlorophenol | 87-86-5 | C6Cl5OH | 266.34 | 174.0 | 310.0 decomposes | 190.5 | 204.2 | 0.05843 | 0.5879 |
| 129 | Phenol | 108-95-2 | C6H5OH | 94.11 | 40.9 | 181.8 | 97.1 | 99.7 | 0.07844 | 1.016 |
| 130 | p-Phenylenediamine | 106-50-3 | C6H8N2 | 108.14 | 146.0 | 267.0 | 104.7 | 124.4 | 0.07598 | 0.8783 |
| 131 | Phosgene | 75-44-5 | COCl2 | 98.92 | -127.9 | 8.0 | 64.0 | 71.4 | 0.1004 | 1.232 |
| 132 | Phosphine | 7803-51-2 | H3P | 34.00 | -133.0 | -87.8 | 29.8 | 36.7 | 0.1604 | 1.716 |
| 133 | Phthalic anhydride | 85-44-9 | C8H4O3 | 148.12 | 130.8 | 295.0 | 118.2 | 132.5 | 0.07492 | 0.8400 |
| 134 | 1,3-Propane sultone | 1120-71-4 | C3H6SO3 | 122.14 | 32.0 | 180.0 (30 mm Hg) | 84.5 | 105.6 | 0.08779 | 0.9797 |
| 135 | β -Propiolactone | 57-57-8 | C3H4O2 | 72.06 | -33.4 | 162.0 | 50.9 | 69.1 | 0.1144 | 1.254 |
| 136 | Propionaldehyde | 123-38-6 | C3H6O | 58.08 | -80.0 | 48.0 | 67.7 | 74.0 | 0.1057 | 1.209 |
| 137 | Propoxur | 114-26-1 | C11H15NO3 | 209.25 | 87.0 | decomposes | 214.1 | 248.3 | 0.05613 | 0.4800 |
| 138 | 1,2-Dichloropropane | 26198-63-0 | C3H6Cl2 | 112.99 | -100.4 | 96.4 | 103.6 | 115.8 | 0.08124 | 0.9223 |
| 139 | Propylene oxide | 75-56-9 | C3H6O | 58.08 | -111.9 | 35.0 | 49.4 | 70.5 | 0.1197 | 1.241 |
| 140 | 1,2-Propylenimine | 75-55-8 | C3H7N | 57.10 | NA | 67.0 | 50.1 | 76.3 | 0.1194 | 1.188 |
| 141 | Quinoline | 91-22-5 | C9H7N | 129.16 | -14.8 | 237.1 | 127.2 | 144.7 | 0.07345 | 0.7872 |
| 142 | Quinone | 106-51-4 | C6H4O2 | 108.10 | 115.7 | sublimes | 98.6 | 103.4 | 0.08334 | 0.9929 |
| 143 | Styrene | 100-42-5 | C8H8 | 104.15 | -31.0 | 145.0 | 127.4 | 133.0 | 0.07500 | 0.8377 |
| 144 | Styrene oxide | 96-09-3 | C8H8O | 120.15 | -35.6 | 194.1 | 115.2 | 138.0 | 0.07718 | 0.8155 |
| 145 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin | 1746-01-6 | C12H4O2Cl4 | 321.98 | 305.0 | 446.5 | 241.4 | 267.8 | 0.05196 | 0.4392 |
| 146 | 1,1,2,2-Tetrachloroethane | 79-34-5 | C2H2Cl4 | 167.85 | -43.8 | 146.5 | 120.4 | 135.4 | 0.07361 | 0.8270 |
| 147 | Tetrachloroethylene | 127-18-4 | C2Cl4 | 165.83 | -22.3 | 121.3 | 115.8 | 128.0 | 0.07492 | 0.8610 |
| 148 | Toluene | 108-88-3 | C7H8 | 92.14 | -94.9 | 110.6 | 111.5 | 118.0 | 0.08054 | 0.9097 |
| 149 | Toluene-2,4-diamine | 95-80-7 | C7H10N2 | 122.17 | 99.0 | 292.0 | 125.2 | 146.6 | 0.07663 | 0.7780 |
| 150 | 2,4-Toluene diisocyanate | 584-84-9 | C9H6N2O2 | 174.16 | 20.5 | 251 | 160.0 | 186.4 | 0.06481 | 0.6395 |

Table C.1.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 151 | o-Toluidine | 95-53-4 | C7H9N | 107.16 | -16.3 | 200.3 | 118.3 | 132.4 | 0.07720 | 0.8405 |
| 152 | Toxaphene | 8001-35-2 | C10H10Cl8 | 413.80 | 65-90 | NA | 313.5 | 358.8 | 0.04552 | 0.2867 |
| 153 | 1,2,4-Trichlorobenzene | 120-82-1 | C6H3Cl3 | 181.45 | 17.0 | 213.5 | 147.0 | 158.7 | 0.06710 | 0.7326 |
| 154 | 1,1,2-Trichloroethane | 79-00-5 | C2H3Cl3 | 133.41 | -36.6 | 113.8 | 101.7 | 114.5 | 0.08059 | 0.9293 |
| 155 | Trichloroethylene | 79-01-6 | C2HCl3 | 131.39 | -84.7 | 87.2 | 97.1 | 107.1 | 0.08229 | 0.9709 |
| 156 | 2,4,5-Trichlorophenol | 95-95-4 | C6H3Cl3O | 197.45 | 69.0 | 247.0 | 153.1 | 166.1 | 0.06554 | 0.7060 |
| 157 | 2,4,6-Trichlorophenol | 88-06-2 | C6H3Cl3O | 197.45 | 69.0 | 246.0 | 153.1 | 166.1 | 0.06554 | 0.7060 |
| 158 | Triethylamine | 121-44-8 | C6H15N | 101.19 | -114.7 | 89.0 | 134.6 | 156.3 | 0.07346 | 0.7415 |
| 159 | Trifluralin | 1582-09-8 | C13H16F3N3O4 | 335.28 | 49.0 | 140.0 (4.2 mm Hg) | 307.5 | 339.1 | 0.04629 | 0.3155 |
| 160 | 2,2,4-Trimethylpentane | 540-84-1 | C8H18 | 114.23 | -107.3 | 99.2 | 168.8 | 185.0 | 0.06561 | 0.6438 |
| 161 | Vinyl acetate | 108-05-4 | C4H6O2 | 86.09 | -93.2 | 72.5 | 89.7 | 101.2 | 0.08902 | 1.006 |
| 162 | Vinyl bromide | 593-60-2 | C2H3Br | 106.96 | -137.8 | 15.8 | 60.6 | 67.7 | 0.1018 | 1.268 |
| 163 | Vinyl chloride | 75-01-4 | C2H3Cl | 62.50 | -153.7 | -13.3 | 59.7 | 65.3 | 0.1099 | 1.292 |
| 164 | Vinylidene chloride | 75-35-4 | C2H2Cl2 | 96.94 | -122.5 | 31.6 | 78.4 | 86.2 | 0.09276 | 1.109 |
| 165 | o-Xylene | 95-47-6 | C8H10 | 106.17 | -25.2 | 144.5 | 132.0 | 140.4 | 0.07369 | 0.8052 |
| 166 | m-Xylene | 108-38-3 | C8H10 | 106.17 | -47.8 | 139.1 | 132.0 | 140.4 | 0.07369 | 0.8052 |
| 167 | p-Xylene | 106-42-3 | C8H10 | 106.17 | 13.2 | 138.3 | 132.0 | 140.4 | 0.07369 | 0.8052 |

aNA, not available

Table C.2.  Basic physicochemical properties of polychlorinated byphenyl (PCB) congeners at 298.2 K.

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2-Monochloro | 2051-60-7 | C12H9Cl | 188.66 | 34.0 | 274 | 196.0 | 205.5 | 0.05883 | 0.5843 |
| 2 | 3-Monochloro | 2051-61-8 | C12H9Cl | 188.66 | 25.1 | 284 | 196.0 | 205.5 | 0.05883 | 0.5843 |
| 3 | 4-Monochloro | 2051-62-9 | C12H9Cl | 188.66 | 77.9 | 291 | 196.0 | 205.5 | 0.05883 | 0.5843 |
| 4 | 2,2'-Dichloro | 13029-08-8 | C12H8Cl2 | 223.10 | 61.0 | NAa | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 5 | 2,3-Dichloro | 16605-91-7 | C12H8Cl2 | 223.10 | NA | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 6 | 2,3'-Dichloro | 25569-80-6 | C12H8Cl2 | 223.10 | NA | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 7 | 2,4-Dichloro | 33284-50-3 | C12H8Cl2 | 223.10 | 24.4 | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 8 | 2,4'-Dichloro | 34883-43-7 | C12H8Cl2 | 223.10 | 43.0 | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 9 | 2,5-Dichloro | 34883-39-1 | C12H8Cl2 | 223.10 | 25.1 | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 10 | 2,6-Dichloro | 33146-45-1 | C12H8Cl2 | 223.10 | 34.9 | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 11 | 3,3'-Dichloro | 2050-67-1 | C12H8Cl2 | 223.10 | 29.0 | 322 | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 12 | 3,4-Dichloro | 2974-92-7 | C12H8Cl2 | 223.10 | 49.0 | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 13 | 3,4'-Dichloro | 2974-90-5 | C12H8Cl2 | 223.10 | NA | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 14 | 3,5-Dichloro | 34883-41-5 | C12H8Cl22 | 223.10 | 31.0 | NA | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 15 | 4,4'-Dichloro | 2050-68-2 | C12H8Cl2 | 223.10 | 149.0 | 315 | 214.7 | 226.4 | 0.05585 | 0.5305 |
| 16 | 2,2',3-Trichloro | 38444-78-9 | C12H7Cl3 | 257.55 | 28.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 17 | 2,2',4-Trichloro | 37680-66-3 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 18 | 2,2',5-Trichloro | 37680-65-2 | C12H7Cl3 | 257.55 | 44.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 19 | 2,2',6-Trichloro | 38444-73-4 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 20 | 2,3,3'-Trichloro | 38444-84-7 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 21 | 2,3,4-Trichloro | 55702-46-0 | C12H7Cl3 | 257.55 | 102.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 22 | 2,3,4'-Trichloro | 38444-85-8 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 23 | 2,3,5-Trichloro | 55720-44-0 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 24 | 2,3,6-Trichloro | 58702-45-9 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 25 | 2,3',4-Trichloro | 55712-37-3 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 2,3',5-Trichloro | 38444-81-4 | C12H7Cl3 | 257.55 | 40.5 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 27 | 2,3',6-Trichloro | 38444-76-7 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 28 | 2,4,4'-Trichloro | 7012-37-5 | C12H7Cl3 | 257.55 | 57.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 29 | 2,4,5-Trichloro | 15862-07-4 | C12H7Cl3 | 257.55 | 78.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 30 | 2,4,6-Trichloro | 35693-92-6 | C12H7Cl3 | 257.55 | 62.5 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 31 | 2,4',5-Trichloro | 16606-02-3 | C12H7Cl3 | 257.55 | 67.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 32 | 2,4',6-Trichloro | 38444-77-8 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 33 | 2',3,4-Trichloro | 38444-86-9 | C12H7Cl3 | 257.55 | 60.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 34 | 2',3,5-Trichloro | 37680-68-5 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 35 | 3,3',4-Trichloro | 37680-69-6 | C12H7Cl3 | 257.55 | 87.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 36 | 3,3',5-Trichloro | 38444-87-0 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 37 | 3,4,4'-Trichloro | 38444-90-5 | C12H7Cl3 | 257.55 | 87.0 | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 38 | 3,4,5-Trichloro | 53555-66-1 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 39 | 3,4',5-Trichloro | 38444-88-1 | C12H7Cl3 | 257.55 | NA | NA | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 40 | 2,2',3,3'-Tetrachloro | 38444-93-8 | C12H7Cl4 | 291.99 | 121.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 41 | 2,2'3,4-Tetrachloro | 52663-59-9 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 42 | 2,2',3,4'-Tetrachloro | 36559-22-5 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 43 | 2,2',3,5-Tetrachloro | 70362-46-8 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 44 | 2,2',3,5'-Tetrachloro | 41464-39-5 | C12H7Cl4 | 291.99 | 47.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 45 | 2,2'3,6-Tetrachloro | 70362-45-7 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 46 | 2,2',3,6'-Tetrachloro | 41464-47-5 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 47 | 2,2',4,4'-Tetrachloro | 2437-79-8 | C12H7Cl4 | 291.99 | 83.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 48 | 2,2',4,5-Tetrachloro | 70362-47-9 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 49 | 2,2',4,5'-Tetrachloro | 41464-40-8 | C12H7Cl4 | 291.99 | 64.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 50 | 2,2',4,6-Tetrachloro | 62796-65-8 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 2,2',4,6'-Tetrachloro | 65194-04-7 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 52 | 2,2',5,5'-Tetrachloro | 35693-99-3 | C12H7Cl4 | 291.99 | 87.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 53 | 2,2',5,6'-Tetrachloro | 41464-41-9 | C12H7Cl4 | 291.99 | 104.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 54 | 2,2',6,6'-Tetrachloro | 15968-05-5 | C12H7Cl4 | 291.99 | 198.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 55 | 2,3,3',4-Tetrachloro | 74338-24-2 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 56 | 2,3,3',4'-Tetrachloro | 41464-43-1 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 57 | 2,3,3',5-Tetrachloro | 70424-67-8 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 58 | 2,3,3',5'-Tetrachloro | 41464-49-7 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 59 | 2,3,3',6-Tetrachloro | 74472-33-6 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 60 | 2,3,4,4'-Tetrachloro | 33025-41-1 | C12H7Cl4 | 291.99 | 142.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 61 | 2,3,4,5-Tetrachloro | 33284-53-6 | C12H7Cl4 | 291.99 | 92.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 62 | 2,3,4,6-Tetrachloro | 54230-23-7 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 63 | 2,3,4',5-Tetrachloro | 74472-35-8 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 64 | 2,3,4',6-Tetrachloro | 52663-58-8 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 65 | 2,3,5,6-Tetrachloro | 33284-54-7 | C12H7Cl4 | 291.99 | 79.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 66 | 2,3',4,4'-Tetrachloro | 32598-10-0 | C12H7Cl4 | 291.99 | 124.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 67 | 2,3',4,5-Tetrachloro | 73575-53-8 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 68 | 2,3',4,5'-Tetrachloro | 73575-52-7 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 69 | 2,3',4,6-Tetrachloro | 60233-24-1 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 70 | 2,3',4',5-Tetrachloro | 32598-11-1 | C12H7Cl4 | 291.99 | 104.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 71 | 2,3',4',6-Tetrachloro | 41464-46-4 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 72 | 2,4',5,5'-Tetrachloro | 41464-42-0 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 73 | 2,3',5',6-Tetrachloro | 74338-23-1 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 74 | 2,4,4',5-Tetrachloro | 32690-93-0 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 75 | 2,4,4',6-Tetrachloro | 32598-12-2 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 76 | 2',3,4,5-Tetrachloro | 70362-48-0 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 77 | 3,3',4,4'-Tetrachloro | 32598-13-3 | C12H7Cl4 | 291.99 | 180.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 78 | 3,3'4,5-Tetrachloro | 70362-49-1 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 79 | 3,3',4,5'-Tetrachloro | 41464-48-6 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 80 | 3,3',5,5'-Tetrachloro | 33284-52-5 | C12H7Cl4 | 291.99 | 164.0 | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 81 | 3,4,4',5-Tetrachloro | 70362-50-4 | C12H7Cl4 | 291.99 | NA | NA | 252.1 | 268.2 | 0.05113 | 0.4384 |
| 82 | 2,2',3,3',4-Pentachloro | 52663-62-4 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 83 | 2,2',3,3',5-Pentachloro | 60145-20-2 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 84 | 2,2',3,3',6-Pentachloro | 52663-60-2 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 85 | 2,2',3,4,4'-Pentachloro | 65510-45-4 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 86 | 2,2',3,4,5-Pentachloro | 55312-69-1 | C12H5Cl5 | 326.44 | 100.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 87 | 2,2',3,4,5'-Pentachloro | 38380-02-8 | C12H5Cl5 | 326.44 | 114.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 88 | 2,2',3,4,6-Pentachloro | 55215-17-3 | C12H5Cl5 | 326.44 | 100.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 89 | 2,2',3,4,6'-Pentachloro | 73575-57-2 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 90 | 2,2',3,4',5-Pentachloro | 68194-07-0 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 91 | 2,2',3,4',6-Pentachloro | 58194-05-8 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 92 | 2,2',3,5,5'-Pentachloro | 52663-61-3 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 93 | 2,2',3,5,6-Pentachloro | 73575-56-1 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 94 | 2,2',3,5,6'-Pentachloro | 73575-55-0 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 95 | 2,2',3,5',6-Pentachloro | 38379-99-6 | C12H5Cl5 | 326.44 | 100.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 96 | 2,2',3,6,6'-Pentachloro | 73575-54-9 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 97 | 2,2',3',4,5-Pentachloro | 41464-51-1 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 98 | 2,2',3',4,6-Pentachloro | 60233-25-2 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 99 | 2,2',4,4',5-Pentachloro | 38380-01-7 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 100 | 2,2',4,4',6-Pentachloro | 39485-83-1 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | 2,2'4,5,5'-Pentachloro | 37680-73-2 | C12H5Cl5 | 326.44 | 76.5 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 102 | 2,2',4,5,6'-Pentachloro | 68194-06-9 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 103 | 2,2',4,5',6-Pentachloro | 60145-21-3 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 104 | 2,2',4,6,6'-Pentachloro | 56558-16-8 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 105 | 2,3,3',4,4'-Pentachloro | 32598-14-4 | C12H5Cl5 | 326.44 | 105.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 106 | 2,3,3',4,5-Pentachloro | 70424-69-0 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 107 | 2,3,3',4',5-Pentachloro | 70424-68-9 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 108 | 2,3,3',4,5'-Pentachloro | 70362-41-3 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 109 | 2,3,3',4,6-Pentachloro | 74472-35-8 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 110 | 2,3,3',4',6-Pentachloro | 38380-03-9 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 111 | 2,3,3',5,5'-Pentachloro | 39635-32-0 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 112 | 2,3,3',5,6-Pentachloro | 74472-36-9 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 113 | 2,3,3',5',6-Pentachloro | 68194-10-5 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 114 | 2,3,4,4',5-Pentachloro | 74472-37-0 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 115 | 2,3,4,4',6-Pentachloro | 74472-38-1 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 116 | 2,3,4,5,6-Pentachloro | 18259-05-7 | C12H5Cl5 | 326.44 | 124.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 117 | 2,3,4',5,6-Pentachloro | 68194-11-6 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 118 | 2,3'4,4',5-Pentachloro | 31508-00-6 | C12H5Cl5 | 326.44 | 107.0 | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 119 | 2,3',4,4',6-Pentachloro | 56558-17-9 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 120 | 2,3',4,5,5'-Pentachloro | 68194-12-7 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 121 | 2,3',4,5',6-Pentachloro | 56558-18-0 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 122 | 2',3,3',4,5-Pentachloro | 76842-07-4 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 123 | 2',3,4,4',5-Pentachloro | 65510-44-3 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 124 | 2',3,4,5,5'-Pentachloro | 70424-70-3 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 125 | 2',3,4,5,6'-Pentachloro | 74472-39-2 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 126 | 3,3'4,4',5-Pentachloro | 57465-28-8 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 127 | 3,3',4,5,5'-Pentachloro | 39635-33-1 | C12H5Cl5 | 326.44 | NA | NA | 270.8 | 289.1 | 0.04921 | 0.3985 |
| 128 | 2,2'3,3'4,4'-Hexachloro | 38380-07-3 | C12H4Cl6 | 360.88 | 150.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 129 | 2,2',3,3',4,5-Hexachloro | 55215-18-4 | C12H4Cl6 | 360.88 | 85.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 130 | 2,2'3,3',4,5'-Hexachloro | 52663-66-8 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 131 | 2,2',3,3',4,6-Hexachloro | 61798-70-7 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 132 | 2,2',3,3',4,6'-Hexachloro | 38380-05-1 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 133 | 2,2'3,3'5,5'-Hexachloro | 35694-04-3 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 134 | 2,2',3,3',5,6-Hexachloro | 52704-70-8 | C12H4Cl6 | 360.88 | 100.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 135 | 2,2',3,3',5,6'-Hexachloro | 52744-13-5 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 136 | 2,2',3,3',6,6'-Hexachloro | 38411-22-2 | C12H4Cl6 | 360.88 | 112.2 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 137 | 2,2',3,4,4',5-Hexachloro | 35694-06-5 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 138 | 2,2',3,4,4',5'-Hexachloro | 35065-28-2 | C12H4Cl6 | 360.88 | 80.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 139 | 2,2',3,4,4',6-Hexachloro | 56030-56-9 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 140 | 2,2',3,4,4',6'-Hexachloro | 59291-64-4 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 141 | 2,2',3,4,5,5'-Hexachloro | 52712-04-6 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 142 | 2,2',3,4,5,6-Hexachloro | 41411-61-4 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 143 | 2,2',3,4,5,6'-Hexachloro | 68194-15-0 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 144 | 2,2',3,4,5',6-Hexachloro | 68194-14-9 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 145 | 2,2',3,4,6,6'-Hexachloro | 74472-40-5 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 146 | 2,2',3,4',5,5'-Hexachloro | 51908-16-8 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 147 | 2,2',3,4',5,6-Hexachloro | 68194-13-8 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 148 | 2,2',3,4',5,6'-Hexachloro | 74472-42-7 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 149 | 2,2',3,4',5',6-Hexachloro | 38380-04-0 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 150 | 2,2',3,4',6,6'-Hexachloro | 68194-08-1 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 151 | 2,2',3,5,5',6-Hexachloro | 52663-63-5 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 152 | 2,2',3,5,6,6'-Hexachloro | 68194-09-2 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 153 | 2,2',4,4',5,5'-Hexachloro | 35065-27-1 | C12H4Cl6 | 360.88 | 103.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 154 | 2,2',4,4',5,6'-Hexachloro | 60145-22-4 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 155 | 2,2',4,4',6,6'-Hexachloro | 33979-03-2 | C12H4Cl6 | 360.88 | 114.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 156 | 2,3,3',4,4',5-Hexachloro | 38380-08-4 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 157 | 2,3,3',4,4',5'-Hexachloro | 69782-90-7 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 158 | 2,3,3',4,4',6-Hexachloro | 74472-42-7 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 159 | 2,3,3',4,5,5'-Hexachloro | 39635-35-3 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 160 | 2,3,3',4,5,6-Hexachloro | 41411-62-5 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 161 | 2,3,3',4,5',6-Hexachloro | 74472-43-8 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 162 | 2,3,3',4',5,5'-Hexachloro | 39635-34-2 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 163 | 2,3,3',4',5,6-Hexachloro | 74472-44-9 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 164 | 2,3,3',4',5',6-Hexachloro | 74472-45-0 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 165 | 2,3,3',5,5',6-Hexachloro | 74472-46-1 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 166 | 2,3,4,4',5,6-Hexachloro | 41411-63-6 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 167 | 2,3',4,4',5,5'-Hexachloro | 52663-72-6 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 168 | 2,3',4,4',5',6-Hexachloro | 59291-65-5 | C12H4Cl6 | 360.88 | NA | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 169 | 3,3',4,4',5,5'-Hexachloro | 32774-16-6 | C12H4Cl6 | 360.88 | 202.0 | NA | 289.4 | 310.0 | 0.04751 | 0.3619 |
| 170 | 2,2',3,3',4,4',5-Heptachloro | 35065-30-6 | C12H3Cl7 | 395.33 | 135.5 | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 171 | 2,2',3,3',4,4',6-Heptachloro | 52663-71-5 | C12H3Cl7 | 395.33 | 122.4 | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 172 | 2,2',3,3',4,5,5'-Heptachloro | 52663-74-8 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 173 | 2,2',3,3',4,5,6-Heptachloro | 68194-16-1 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 174 | 2,2',3,3',4,5,6'-Heptachloro | 38411-25-5 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 175 | 2,2',3,3',4,5',6-Heptachloro | 40186-70-7 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 176 | 2,2',3,3',4,6,6'-Heptachloro | 52663-65-7 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 177 | 2,2',3,3',4',5,6-Heptachloro | 52663-70-4 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 178 | 2,2',3,3',5,5',6-Heptachloro | 52663-67-9 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 179 | 2,2',3,3',5,6,6'-Heptachloro | 52663-64-6 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 180 | 2,2',3,4,4',5,5'-Heptachloro | 35065-29-3 | C12H3Cl7 | 395.33 | 110.0 | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 181 | 2,2',3,4,4',5,6-Heptachloro | 74472-47-2 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 182 | 2,2',3,4,4',5,6'-Heptachloro | 60145-23-5 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 183 | 2,2',3,4,4',5',6-Heptachloro | 52663-69-1 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 184 | 2,2',3,4,4',6,6'-Heptachloro | 74472-48-3 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 185 | 2,2',3,4,5,5',6- Heptachloro | 52712-05-7 | C12H3Cl7 | 395.33 | 149.0 | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 186 | 2,2',3,4,5,6,6'-Heptachloro | 74472-49-4 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 187 | 2,2',3,4',5,5',6-Heptachloro | 52663-68-0 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 188 | 2,2',3,4',5,6,6'-Heptachloro | 74487-85-7 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 189 | 2,3,3',4,4',5,5'-Heptachloro | 39635-31-9 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 190 | 2,3,3',4,4',5,6-Heptachloro | 41411-64-7 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 191 | 2,3,3',4,4',5',6-Heptachloro | 74472-50-7 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 192 | 2,3,3',4,5,5',6-Heptachloro | 74472-51-8 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 193 | 2,3,3',4',5,5',6-Heptachloro | 69782-91-8 | C12H3Cl7 | 395.33 | NA | NA | 308.1 | 330.9 | 0.04597 | 0.3281 |
| 194 | 2,2',3,3',4,4',5,5'-Octachloro | 35694-08-7 | C12H2Cl8 | 429.77 | 159.0 | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 195 | 2,2',3,3',4,4',5,6-Octachloro | 52663-78-2 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 196 | 2,2',3,3',4,4',5,6'-Octachloro | 42740-50-1 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 197 | 2,2',3,3',4,4',6,6'-Octachloro | 33091-17-7 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 198 | 2,2',3,3',4,5,5',6-Octachloro | 68194-17-2 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 199 | 2,2',3,3',4,5,5',6'-Octachloro | 52663-75-9 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 200 | 2,2',3,3',4,5,6,6'-Octachloro | 52663-73-7 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |

Table C.2.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | MolecularWeight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 201 | 2,2',3,3',4,5',6,6'-Octachloro | 40186-71-8 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 202 | 2,2',3,3',5,5',6,6'-Octachloro | 2136-99-4 | C12H2Cl8 | 429.77 | 162.0 | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 203 | 2,2',3,4,4',5,5',6-Octachloro | 52663-76-0 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 204 | 2,2',3,4,4',5,6,6'-Octachloro | 74472-52-9 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 205 | 2,3,3',4,4',5,5',6-Octachloro | 74472-53-0 | C12H2Cl8 | 429.77 | NA | NA | 326.8 | 351.8 | 0.04457 | 0.2967 |
| 206 | 2,2',3,3',4,4',5,5',6-Nonachloro | 40186-72-9 | C12HCl9 | 464.22 | 206.0 | NA | 345.5 | 372.7 | 0.04329 | 0.2674 |
| 207 | 2,2',3,3',4,4',5,6,6'-Nonachloro | 52663-79-3 | C12HCl9 | 464.22 | NA | NA | 345.5 | 372.7 | 0.04329 | 0.2674 |
| 208 | 2,2',3,3',4,5,5',6,6'-Nonachloro | 52663-77-1 | C12HCl9 | 464.22 | 182.8 | NA | 345.5 | 372.7 | 0.04329 | 0.2674 |
| 209 | 2,2',3,3',4,4',5,5',6,6'-Decachloro | 2051-24-3 | C12Cl10 | 498.66 | 305.9 | NA | 364.2 | 393.6 | 0.04212 | 0.2401 |

aNA, not available

Table C.3.  Basic hysicochemical properties of polychlorinated byphenyls (PCBs) at 298.2 K – aroclor mixtures.

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Aroclor 1016 | 12674-11-2 | C12H7Cl3 | 257.55 | 233.4 | 247.3 | 0.05332 | 0.4821 |
| 2 | Aroclor 1221 | 111-042-82 | C12H8.85Cl1.15 | 193.83 | 198.8 | 208.6 | 0.05835 | 0.5759 |
| 3 | Aroclor 1232 | 111-411-65 | C12H7.96Cl2.04 | 224.48 | 215.4 | 227.2 | 0.05574 | 0.5285 |
| 4 | Aroclor 1242 | 534-692-19 | C12H6.9Cl3.1 | 260.99 | 235.2 | 249.4 | 0.05309 | 0.4476 |
| 5 | Aroclor 1248 | 126-722-96 | C12H6.1Cl3.9 | 288.55 | 250.2 | 266.1 | 0.05134 | 0.4426 |
| 6 | Aroclor 1254 | 110-976-91 | C12H5.04Cl4.96 | 325.06 | 270.0 | 288.3 | 0.04929 | 0.4000 |
| 7 | Aroclor 1260 | 110-968-25 | C12H3.7Cl6.3 | 371.22 | 295.0 | 316.3 | 0.04703 | 0.3514 |

Table C.4.  Basic physicochemical properties of dioxins at 298.2 K.

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dibenzo-p-dioxin | 262-23-4 | C12H8O2 | 184.20 | 123.0 | 283.5 | 166.6 | 184.2 | 0.06340 | 0.6463 |
| 2 | 1-Chlorodibenzo-p-dioxin | 39227-53-7 | C12H7O2Cl | 218.65 | 105.5 | 315.5 | 185.3 | 205.1 | 0.05979 | 0.5854 |
| 3 | 2-Chlorodibenzo-p-dioxin | 39227-54-8 | C12H7O2Cl | 218.65 | 89.0 | 316.0 | 185.3 | 205.1 | 0.05979 | 0.5854 |
| 4 | 2,3-Dichlorodibenzo-p-dioxin | 29446-15-9 | C12H6O2Cl2 | 253.09 | 164.0 | 358.0 | 204.0 | 226.0 | 0.05678 | 0.5315 |
| 5 | 2,7-Dichlorodibenzo-p-dioxin | 33857-26-0 | C12H6O2Cl2 | 253.09 | 210.0 | 373.5 | 204.0 | 226.0 | 0.05678 | 0.5315 |
| 6 | 2,8-Dichlorodibenzo-p-dioxin | 38964-22-6 | C12H6O2Cl2 | 253.09 | 151.0 | NAa | 204.0 | 226.0 | 0.05678 | 0.5315 |
| 7 | 1,2,4-Trichlorodibenzo-p-dioxin | 39227-58-2 | C12H5O2Cl3 | 287.54 | 129.0 | 375.0 | 222.7 | 246.9 | 0.05420 | 0.4830 |
| 8 | 1,2,3,4-Tetrachlorodibenzo-p-dioxin | 30756-58-8 | C12H4O2Cl4 | 321.98 | 190.0 | 419.0 | 241.4 | 267.8 | 0.05196 | 0.4392 |
| 9 | 1,2,3,7-Tetrachlorodibenzo-p-dioxin | 67028-18-6 | C12H4O2Cl4 | 321.98 | 172.0 | 438.3 | 241.4 | 267.8 | 0.05196 | 0.4392 |
| 10 | 1,3,6,8-Tetrachlorodibenzo-p-dioxin | 30746-58-8 | C12H4O2Cl4 | 321.98 | 219.0 | 438.3 | 241.4 | 267.8 | 0.05196 | 0.4392 |
| 11 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin | 1746-01-6 | C12H4O2Cl4 | 321.98 | 305.0 | 446.5 | 241.4 | 267.8 | 0.05196 | 0.4392 |
| 12 | 1,2,3,4,7-Pentachlorodibenzo-p-dioxin | 39227-61-7 | C12H3O2Cl5 | 356.43 | 195.0 | 464.7 | 260.1 | 288.7 | 0.04998 | 0.3993 |
| 13 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin | 39227-26-8 | C12H2O2Cl6 | 390.87 | 273.0 | 487.7 | 278.7 | 309.6 | 0.04822 | 0.3626 |
| 14 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin | 35822-46-9 | C12HO2Cl7 | 425.32 | 265.0 | 507.2 | 297.4 | 330.5 | 0.04663 | 0.3287 |
| 15 | Octachlorodibenzo-p-dioxin | 3268-87-9 | C12O2Cl8 | 459.76 | 322.0 | 510.0 | 316.1 | 351.4 | 0.04519 | 0.2973 |

aNA, not available

Table C.5.  Basic physicochemical properties of dibenzofurans at 298.2 K.

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dibenzofuran | 132-64-9 | C12H8O | 168.20 | 86.5 | 287.0 | 160.5 | 176.7 | 0.06488 | 0.6702 |
| 2 | 2,8-Dichlorodibenzofuran | 5409-83-6 | C12H7OCl | 202.64 | 184.0 | 375.0 | 179.2 | 197.6 | 0.06099 | 0.6064 |
| 3 | 2,3,7,8-Tetrachlorodibenzofuran | 51207-31-9 | C12H4OCl4 | 305.98 | 227.0 | 438.3 | 235.3 | 260.3 | 0.05269 | 0.4544 |
| 4 | 2,3,4,7,8-Pentachlorodibenzofuran | 51207-31-4 | C12H3OCl5 | 340.42 | 196.0 | 464.7 | 253.9 | 281.2 | 0.05063 | 0.4132 |
| 5 | 1,2,3,4,7,8-Hexachlorodibenzofuran | 70658-26-9 | C12H2OCl6 | 374.87 | 225.5 | 487.7 | 272.6 | 302.1 | 0.04880 | 0.3754 |
| 6 | 1,2,3,6,7,8-Hexachlorodibenzofuran | 57117-44-9 | C12H2OCl6 | 374.87 | 232.0 | 487.7 | 272.6 | 302.1 | 0.04880 | 0.3754 |
| 7 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran | 67462-39-4 | C12HOCl7 | 409.31 | 236.0 | 507.2 | 291.3 | 323.0 | 0.04715 | 0.3406 |
| 8 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran | 55673-89-7 | C12HOCl7 | 409.31 | 221.0 | 507.2 | 291.3 | 323.0 | 0.04715 | 0.3406 |
| 9 | Octachlorodibenzofuran | 39001-02-0 | C12OCl8 | 443.76 | 258.0 | 537.0 | 310.0 | 343.9 | 0.04566 | 0.3083 |

Table C.6.  Basic physicochemical properties of polycyclic organic matter (POM) at 298.2 K.

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Indan | 496-11-7 | C9H10 | 118.18 | -51.0 | 178.0 | 129.6 | 143.7 | 0.07347 | 0.7913 |
| 2 | Naphthalene | 91-20-3 | C10H8 | 128.18 | 80.2 | 217.9 | 140.9 | 147.6 | 0.07029 | 0.7754 |
| 3 | 1-Methylnaphthalene | 90-12-0 | C11H10 | 142.20 | -22.0 | 244.6 | 161.4 | 169.8 | 0.06559 | 0.6932 |
| 4 | 2-Methylnaphthalene | 91-57-6 | C11H10 | 142.20 | 34.6 | 241.9 | 161.4 | 169.8 | 0.06559 | 0.6932 |
| 5 | 1,2-Dimethylnaphthalene | 573-98-8 | C12H12 | 156.23 | -1.0 | 267.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 6 | 1,3-Dimethylnaphthalene | 575-41-7 | C12H12 | 156.23 | NAa | 265.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 7 | 1,4-Dimethylnaphthalene | 571-58-4 | C12H12 | 156.23 | 7.7 | 262.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 8 | 1,5-Dimethylnaphthalene | 571-61-9 | C12H12 | 156.23 | 81.0 | 249.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 9 | 2,3-Dimethylnaphthalene | 581-40-8 | C12H12 | 156.23 | 105.0 | 269.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 10 | 2,6-Dimethylnaphthalene | 581-40-2 | C12H12 | 156.23 | 108.0 | 262.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 11 | 1-Ethylnaphthalene | 1127-76-0 | C12H12 | 156.23 | -13.8 | 258.7 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 12 | 2-Ethylnaphthalene | 939-27-5 | C12H12 | 156.23 | -70.0 | 252.0 | 181.9 | 192.0 | 0.06169 | 0.6227 |
| 13 | 1,4,5-Trimethylnaphthalene | 2131-41-1 | C13H14 | 170.26 | 64.0 | 185.0 | 202.4 | 214.2 | 0.05839 | 0.5611 |
| 14 | Biphenyl | 92-52-4 | C12H10 | 154.21 | 71.0 | 256.0 | 177.3 | 184.6 | 0.06247 | 0.6451 |
| 15 | 4-Methylbiphenyl | 644-08-6 | C13H12 | 168.24 | 45.5 | 268.0 | 197.8 | 206.8 | 0.05906 | 0.5808 |
| 16 | 4,4'-Dimethylbiphenyl | 613-33-2 | C13H14 | 170.26 | 119.0 | 295.0 | 202.4 | 214.2 | 0.05839 | 0.5611 |
| 17 | Diphenylmethane | 101-81-5 | C13H12 | 168.24 | 25.0 | 264.3 | 197.8 | 206.8 | 0.05906 | 0.5808 |
| 18 | Bibenzyl | 103-29-7 | C14H14 | 182.27 | 52.2 | 285.0 | 218.3 | 229.0 | 0.05613 | 0.5242 |
| 19 | trans-Stilbene | 103-30-0 | C14H12 | 180.25 | 124.0 | NA | 213.7 | 221.6 | 0.05672 | 0.5423 |
| 20 | Acenaphthene | 83-32-9 | C12H10 | 154.21 | 96.2 | 277.5 | 159.0 | 173.1 | 0.06559 | 0.6821 |
| 21 | Acenaphthylene | 208-96-8 | C12H8 | 152.20 | 92.0 | 270.0 | 154.4 | 165.7 | 0.06652 | 0.7074 |
| 22 | Fluorene | 86-73-7 | C13H10 | 166.22 | 116.0 | 295.0 | 174.9 | 187.9 | 0.06249 | 0.6350 |
| 23 | 1-Methylfluorene | 1730-37-6 | C14H12 | 180.25 | 85.0 | NA | 195.4 | 210.1 | 0.05909 | 0.5719 |
| 24 | Phenanthrene | 85-01-8 | C14H10 | 178.24 | 101.0 | 339.0 | 190.8 | 196.7 | 0.05978 | 0.6090 |
| 25 | 1-Methylphenanthrene | 832-69-6 | C15H12 | 192.26 | 123.0 | 359.0 | 211.3 | 218.9 | 0.05677 | 0.5491 |
| 26 | Anthracene | 120-12-7 | C14H10 | 178.24 | 216.2 | 340.0 | 190.8 | 196.7 | 0.05978 | 0.6090 |
| 27 | 9-Methylanthracene | 779-02-2 | C15H12 | 192.26 | 81.5 | 355.0 | 211.3 | 218.9 | 0.05677 | 0.5491 |

Table C.6.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 2-Methylanthracene | 613-12-7 | C15H12 | 192.26 | 209.0 | 359.0 | 211.3 | 218.9 | 0.05677 | 0.5491 |
| 29 | 9,10-Dimethylanthracene | 781-43-1 | C16H14 | 206.29 | 182.0 | NA | 231.8 | 241.1 | 0.05416 | 0.4960 |
| 30 | Pyrene | 129-00-0 | C16H10 | 202.26 | 151.2 | 404.0 | 204.3 | 213.8 | 0.05747 | 0.5622 |
| 31 | Fluoranthene | 206-44-0 | C16H10 | 202.26 | 107.8 | 384.0 | 204.3 | 217.3 | 0.05747 | 0.5532 |
| 32 | Benzo[a]fluorene | 238-84-6 | C17H12 | 216.29 | 189.5 | 405.0 | 224.8 | 239.5 | 0.05477 | 0.4996 |
| 33 | Benzo[b]fluorene | 243-17-4 | C17H12 | 216.29 | 212.0 | 402.0 | 224.8 | 239.5 | 0.05477 | 0.4996 |
| 34 | Chrysene | 218-01-9 | C18H12 | 228.30 | 252.2 | 448.0 | 240.7 | 250.8 | 0.05290 | 0.4745 |
| 35 | Triphenylene | 217-59-4 | C18H12 | 228.30 | 199.0 | 425.0 | 240.7 | 250.8 | 0.05290 | 0.4745 |
| 36 | p-Terphenyl | 92-94-4 | C18H14 | 230.31 | 213.0 | 389.0 | 263.6 | 273.2 | 0.05068 | 0.4286 |
| 37 | Naphthacene | 92-24-0 | C18H12 | 228.30 | 357.0 | sublimes | 240.7 | 250.8 | 0.05290 | 0.4745 |
| 38 | Benz[a]anthracene | 56-55-3 | C18H12 | 228.30 | 158.0 | 437.6 | 240.7 | 250.8 | 0.05290 | 0.4745 |
| 39 | Benzo[b]fluoranthene | 205-99-2 | C20H12 | 252.32 | 164.0 | NA | 254.2 | 268.9 | 0.05130 | 0.4370 |
| 40 | Benzo[j]fluoranthene | 205-82-2 | C20H12 | 252.32 | 166.0 | 480.0 | 254.2 | 268.9 | 0.05130 | 0.4370 |
| 41 | Benzo[k]fluoranthene | 207-08-9 | C20H12 | 252.32 | 217.0 | 480.0 | 254.2 | 268.9 | 0.05130 | 0.4370 |
| 42 | Benzo[a]pyrene | 50-32-8 | C20H12 | 252.32 | 176.5 | 495.0 | 254.2 | 265.4 | 0.05130 | 0.4440 |
| 43 | Benzo[e]pyrene | 192-97-2 | C20H12 | 252.32 | 177.5 | 311.0 | 254.2 | 265.4 | 0.05130 | 0.4440 |
| 44 | Perylene | 198-55-0 | C20H12 | 252.32 | 274.0 | 495.0 | 254.2 | 265.4 | 0.05130 | 0.4440 |
| 45 | 7,12-Dimethylbenz[a] anthracene | 57-97-6 | C20H16 | 256.35 | 122.5 | NA | 281.8 | 295.2 | 0.04883 | 0.3875 |
| 46 | 9,10-Dimethylbenz[a] anthracene | 56-56-4 | C20H16 | 256.35 | 122.0 | NA | 281.8 | 295.2 | 0.04883 | 0.3875 |
| 47 | 3-Methylcholanthrene | 56-49-5 | C21H16 | 268.36 | 179.0 | 280.0 (80 mm Hg) | 279.4 | 298.5 | 0.04892 | 0.3817 |
| 48 | Benzo[ghi]perylene | 191-24-2 | C21H16 | 268.36 | 278.0 | NA | 261.1 | 280.0 | 0.05050 | 0.4155 |
| 49 | Pentacene | 135-48-8 | C22H14 | 278.36 | 257.0 | NA | 290.6 | 302.4 | 0.04793 | 0.3749 |
| 50 | Dibenz[a,c]anthracene | 215-58-7 | C22H14 | 278.36 | 206.0 | 518.0 | 290.6 | 302.4 | 0.04793 | 0.3749 |
| 51 | Dibenz[a,h]anthracene | 53-70-3 | C22H14 | 278.36 | 269.5 | 524.0 | 290.6 | 302.4 | 0.04793 | 0.3749 |
| 52 | Dibenz[a,j]anthracene | 58-70-3 | C22H14 | 278.36 | 196.0 | NA | 290.6 | 302.4 | 0.04793 | 0.3749 |
| 53 | Coronene | 191-07-1 | C24H12 | 300.36 | 437.3 | 525.0 | 281.2 | 294.6 | 0.04852 | 0.3886 |

aNA, not available

Table C.7.  Basic physicochemical properties of coke oven emissions at 298.2 K – monoaromatic and phenolic compounds.

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ethylbenzene | 100-41-4 | C8H10 | 106.17 | -95.0 | 136.2 | 132.0 | 140.4 | 0.07369 | 0.8052 |
| 2 | 1,2,3-Trimethylbenzene | 526-73-8 | C9H12 | 120.20 | -25.4 | 176.1 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 3 | 1,2,4-Trimethylbenzene | 95-63-6 | C9H12 | 120.20 | -43.8 | 169.4 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 4 | 1,3,5-Trimethylbenzene | 108-67-8 | C9H12 | 120.20 | -44.7 | 164.7 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 5 | n-Propylbenzene | 103-65-1 | C9H12 | 120.20 | -101.6 | 159.2 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 6 | Isopropylbenzene | 98-82-8 | C9H12 | 120.20 | -96.6 | 154.2 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 7 | 1-Ethyl-2-methylbenzene | 611-14-3 | C9H12 | 120.20 | -80.8 | 165.2 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 8 | 1-Ethyl-3-methylbenzene | 620-14-4 | C9H12 | 120.20 | -95.5 | 161.5 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 9 | 1-Ethyl-4-methylbenzene | 622-96-8 | C9H12 | 120.20 | -62.3 | 162.0 | 152.5 | 162.6 | 0.06829 | 0.7184 |
| 10 | 1-Isopropyl-4-methylbenzene | 99-87-6 | C10H14 | 134.22 | -67.9 | 177.1 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 11 | n-Butylbenzene | 104-51-8 | C10H14 | 134.22 | -88.0 | 183.0 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 12 | Isobutylbenzene | 538-93-2 | C10H14 | 134.22 | -51.0 | 170.0 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 13 | sec-Butylbenzene | 135-98-8 | C10H14 | 134.22 | -75.5 | 173.0 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 14 | tert-Butylbenzene | 98-06-6 | C10H14 | 134.22 | -57.8 | 169.0 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 15 | 1,2,3,4-Tetramethybenzene | 48-23-3 | C10H14 | 134.22 | -6.25 | 205.0 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 16 | 1,2,3,5-Tetramethylbenzene | 527-53-7 | C10H14 | 134.22 | -23.68 | 198.0 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 17 | 1,2,4,5-Tetramethylbenzene | 95-93-2 | C10H14 | 134.22 | -79.2 | 196.8 | 173.0 | 184.8 | 0.06391 | 0.6444 |
| 18 | n-Pentylbenzene | 538-68-1 | C11H16 | 148.25 | -75.0 | 205.4 | 193.6 | 207.0 | 0.06022 | 0.5803 |
| 19 | Pentamethylbenzene | 538-61-1 | C11H16 | 148.25 | 54.5 | 231.0 | 193.6 | 207.0 | 0.06022 | 0.5803 |
| 20 | n-Hexylbenzene | 1077-16-3 | C12H18 | 162.28 | -61.0 | 226.0 | 214.1 | 229.2 | 0.05711 | 0.5237 |
| 21 | Hexamethylbenzene | 87-85-4 | C12H18 | 162.28 | 166.7 | 265.0 | 214.1 | 229.2 | 0.05711 | 0.5237 |
| 22 | 2,3-Dimethylphenol | 526-75-0 | C8H10O | 122.17 | 75.0 | 218.0 | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 23 | 2,4-Dimethylphenol | 105-67-9 | C8H10O | 122.17 | 26.0 | 211.5 | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 24 | 2,5-Dimethylphenol | 105-67-9 | C8H10O | 122.17 | 75.0 | 212.0 | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 25 | 2,6-Dimethylphenol | 576-26-1 | C8H10O | 122.17 | 49.0 | 203.0 | 138.1 | 147.8 | 0.07124 | 0.7746 |

Table C.7.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 3,4-Dimethylphenol | 95-65-8 | C8H10O | 122.17 | 67.0 | 225.0 | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 27 | 3,5-Dimethylphenol | 108-68-9 | C8H10O | 122.17 | 68.0 | 219.5 | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 28 | 2,3,5-Trimethylphenol | 697-82-5 | C9H12O | 136.20 | 93.0 | NAa | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 29 | 2,3,6-Trimethylphenol | 2416-94-6 | C9H12O | 136.20 | 63.0 | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 30 | 2,4,5-Trimethylphenol | 496-78-6 | C9H12O | 136.20 | 72.0 | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 31 | 2,4,6-Trimethylphenol | 527-60-6 | C9H12O | 136.20 | 72.0 | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 32 | 3,4,5-Trimethylphenol | 527-54-8 | C9H12O | 136.20 | 108.0 | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 33 | 3-Methyl-5-ethylphenol | NA | C9H12O | 136.20 | NA | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 34 | 2-Ethylphenol | 90-00-6 | C8H10O | 122.17 | NA | NA | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 35 | 3-Ethylphenol | 620-17-7 | C8H10O | 122.17 | NA | NA | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 36 | 4-Ethylphenol | 123-07-9 | C8H10O | 122.17 | 44.0 | NA | 138.1 | 147.8 | 0.07124 | 0.7746 |
| 37 | 2-Propylphenol | 644-35-0 | C9H12O | 136.20 | NA | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 38 | 4-n-Propylphenol | 645-56-7 | C9H12O | 136.20 | NA | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 39 | 2-Isopropylphenol | 88-69-7 | C9H12O | 136.20 | NA | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 40 | 4-Isopropylphenol | 99-89-8 | C9H12O | 136.20 | 60.0 | NA | 158.6 | 170.0 | 0.06635 | 0.6925 |
| 41 | 4-n-Butylphenol | 1638-22-8 | C10H14O | 150.22 | NA | NA | 179.2 | 192.2 | 0.06230 | 0.6221 |
| 42 | 2-sec-Butylphenol | 89-72-5 | C10H14O | 150.22 | NA | NA | 179.2 | 192.2 | 0.06230 | 0.6221 |
| 43 | 2-tert-Butylphenol | 88-18-6 | C10H14O | 150.22 | NA | NA | 179.2 | 192.2 | 0.06230 | 0.6221 |
| 44 | 3-tert-Butylphenol | 585-34-2 | C10H14O | 150.22 | 41.0 | NA | 179.2 | 192.2 | 0.06230 | 0.6221 |
| 45 | 4-sec-Butylphenol | 99-71-8 | C10H14O | 150.22 | 61.0 | 241.0 | 179.2 | 192.2 | 0.06230 | 0.6221 |
| 46 | 4-tert-Butylphenol | 98-54-4 | C10H14O | 150.22 | 99.0 | 239.5 | 179.2 | 192.2 | 0.06230 | 0.6221 |
| 47 | 2-Methyl-5-tert-Butylphenol | NA | C11H16O | 164.25 | NA | NA | 199.7 | 214.4 | 0.05891 | 0.5606 |
| 48 | 2,6-Di-sec-Butylphenol | 128-39-2 | C14H22O | 206.33 | NA | NA | 261.2 | 281.0 | 0.05122 | 0.4136 |
| 49 | 3,5-Di-tert-Butylphenol | 1138-52-9 | C14H22O | 206.33 | NA | NA | 261.2 | 281.0 | 0.05122 | 0.4136 |
| 50 | 2,6-Di-t-butyl-4-Methylphenol | 128-37-0 | C15H24O | 220.36 | 71.0 | NA | 281.8 | 303.2 | 0.04923 | 0.3735 |

Table C.7.  (Cont.).

| No. | Chemical | CAS No. | Molecular Formula | Molecular Weight (g mol-1) | Melting Point (ºC) | Boiling Point (ºC) | Molecular Volume | Le Bas Molar Volume (cm3 mol-1) | Diffusivity in Air Da (cm2 s-1) | Diffusivity in Water Dw x 105 (cm2 s-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 3-Pentylphenol | NA | C11H16O | 164.25 | NA | NA | 199.7 | 214.4 | 0.05891 | 0.5606 |
| 52 | 4-Hexylphenol | 2446-69-7 | C12H18O | 178.28 | 32.0 | NA | 220.2 | 236.6 | 0.05599 | 0.5063 |
| 53 | 2-Heptylphenol | NA | C13H20O | 192.30 | NA | NA | 240.7 | 258.8 | 0.05346 | 0.4576 |
| 54 | 4-Heptylphenol | 1987-50-4 | C13H20O | 192.30 | NA | NA | 240.7 | 258.8 | 0.05346 | 0.4576 |
| 55 | 4-Octylphenol | 1806-26-4 | C14H22O | 206.33 | 41.0 | NA | 261.2 | 281.0 | 0.05122 | 0.4136 |
| 56 | 4-tert-Octylphenol | 140-66-9 | C14H22O | 206.33 | 81.0 | NA | 261.2 | 281.0 | 0.05122 | 0.4136 |
| 57 | 4-Nonylphenol | 104-40-5 | C15H24O | 220.36 | 96.0 | NA | 281.8 | 303.2 | 0.04923 | 0.3735 |
| 58 | 1-Naphthol | 90-15-3 | C10H8O | 144.17 | 96.0 | 282.5 | 147.0 | 155.0 | 0.06828 | 0.7464 |
| 59 | 2-Naphthol | 135-19-3 | C10H8O | 144.17 | 122.0 | NA | 147.0 | 155.0 | 0.06828 | 0.7464 |
| 60 | 2-Phenylphenol | 90-43-7 | C12H10O | 170.21 | 56.0 | NA | 183.4 | 192.0 | 0.06107 | 0.6227 |
| 61 | 3-Phenylphenol | 588-51-8 | C12H10O | 170.21 | 77.0 | NA | 183.4 | 192.0 | 0.06107 | 0.6227 |
| 62 | 4-Phenylphenol | 92-69-3 | C12H10O | 170.21 | 164.0 | NA | 183.4 | 192.0 | 0.06107 | 0.6227 |
| 63 | 2-Allylphenol | 1745-81-9 | C9H10O | 134.18 | -6.0 | 220.0 | 154.0 | 162.6 | 0.06731 | 0.7184 |

aNA, not available