Appendix D:

Additional Physicochemical Properties of Individual Gaseous

Hazardous Air Pollutants and Mixtures

Table D.1.  Additional physicochemical properties of individual hazardous air pollutants at 298.2 K.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Acetaldehyde | 75-07-0 | 1.23 x 105 | miscible | 7.60 | 0.45 | 0.48 | 3.26 x 102 | 1.50 x 1011 | -9.65 |
| 2 | Acetamide | 60-35-5 | 2.44 | 6.91 x 103 | 3.53 x 10-4 | -1.26 | -1.18 | 7.02 x 106 | 7.96 x 107 | -7.02 |
| 3 | Acetonitrile | 75-05-8 | 1.18 x 104 | miscible | 2.75 | -0.34 | -0.41 | 9.01 x 102 | 2.96 x 108 | -10.0 |
| 4 | Acetophenone | 98-86-2 | 4.5 x 101 | 4.58 x 101 | 9.83 x 10-1 | 1.63 | 1.63 | 2.52 x 103 | 3.30 x 103 | -7.58 |
| 5 | 2-Acetylamino- fluorene | 53-96-3 | NAa | NA | NA | NA | NA | NA | NA | NA |
| 6 | Acrolein | 107-02-8 | 3.65 x 104 | 3.71 x 103 | 9.84 | -0.01 | 0.04 | 2.52 x 102 | 5.80 x 105 | -11.20 |
| 7 | Acrylamide | 79-06-1 | 4.15 x 10-1 | 2.88 x 103 | 1.44 x 10-4 | -0.90 | -0.83 | 1.72 x 107 | 7.86 x 104 | -6.27 |
| 8 | Acrylic acid | 79-10-7 | 5.50 x 102 | miscible | NA | 0.43 | 0.46 | NA | NA | NA |
| 9 | Acrylonitrile | 107-13-1 | 1.10 x 104 | 1.42 x 103 | 7.75 | 0.25 | 0.29 | 3.19 x 102 | 9.10 x 106 | -9.85 |
| 10 | Allyl chloride | 107-05-1 | NA | NA | NA | NA | NA | NA | NA | NA |
| 11 | 4-Aminobiphenyl | 92-67-1 | NA | NA | NA | 2.83 | NA | NA | NA | NA |
| 12 | Aniline | 62-53-3 | 6.52 x 101 | 3.87 x 102 | 1.68 x 10-1 | 0.90 | 0.92 | 1.47 x 104 | 3.36 x 103 | -7.54 |
| 13 | o-Anisidine | 90-04-0 | NA | NA | NA | NA | NA | NA | NA | NA |
| 14 | Benzene | 71-43-2 | 1.27 x 104 | 2.28 x 101 | 5.57 x 102 | 2.13 | 2.12 | 4.44 | 2.51 x 104 | -9.83 |
| 15 | Benzidine | 92-87-5 | 1.0 x 10-6 | 2.17 | 4.61 x 10-7 | 1.81 | 1.82 | 5.38 x 109 | 8.86 x 10-3 | -1.07 |
| 16 | Benzotrichloride | 98-07-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 17 | Benzyl chloride | 100-44-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 18 | Biphenyl | 92-52-4 | 1.3 | 4.54 x 10-2 | 2.86 x 101 | 3.90 | 3.84 | 8.70 x 101 | 2.76 x 103 | -7.04 |
| 19 | Bis(chloromethyl) ether | 542-88-1 | 4.00 x 103 | 1.91 x 102 | 2.09 x 101 | -0.38 | 0.89 | 1.19 x 102 | 3.54 x 105 | -10.90 |
| 20 | Bis(2-ethylhexyl) phthalate | 117-81-7 | 2 x 10-5 | 7.3 x 10-4 | 2.7 x 10-2 | 5.11 | 5.02 | 9.1 x 104 | 2.79 x 102 | -2.54 |
| 21 | Bromoform | 75-25-2 | 7.27 x 102 | 1.23 x 101 | 5.91 x 101 | 2.38 | 2.00 | 4.20 x 101 | 1.88 x 103 | -8.61 |
| 22 | 1,3-Butadiene | 106-99-0 | 2.81 x 105 | 1.36 x 101 | 7.45 x 103b | 1.99 | 1.98 | 1.20 x 10-1 | 1.14 x 104 | -11.50 |
| 23 | Caprolactan | 105-60-2 | NA | NA | NA | NA | NA | NA | NA | NA |
| 24 | Captan | 133-06-2 | 1.10 x 10-5 | 1.70 x 10-2 | 6.47 x 10-4 | 2.30 | NA | 3.83 x 106 | NA | -3.73 |
| 25 | Carbaryl | 63-25-2 | 2.67 x 10-5 | 5.96 x 10-1 | 4.48 x 10-5 | 2.36 | NA | 5.52 x 107 | NA | -2.51 |

Table D.1.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | Carbon disulfide | 75-15-0 | 4.82 x 104 | 2.76 x 101 | 1.75 x 103 | 1.86 | 1.85 | 1.42 | 7.45 x 102 | -10.60 |
| 27 | Carbon tetrachloride | 56-23-5 | 1.53 x 104 | 5.20 | 2.94 x 103 | 2.64 | 2.61 | 8.40 x 10-1 | 2.88 x 104 | -10.00 |
| 28 | Carbonyl sulfide | 463-58-1 | NA | NA | NA | NA | NA | NA | NA | NA |
| 29 | Catechol | 120-80-9 | 1.34c | 2.47 x 103 | 5.43 x 10-4 | 0.77 | 0.79 | 4.57 x 106 | 2.08 x 101 | -5.18 |
| 30 | Chloramben | 133-90-4 | 9.3 x 10-1 | 3.40 | 2.74 x 10-1 | 1.11 | NA | 9.01 x 103 | NA | -7.54 |
| 31 | cis-Chlordane | 5103-71-9 | 4.0 x 10-4 | 1.37 x 10-4 | 2.92 | 6.0 | 5.88 | 8.47 x 102 | 4.12 x 101 | -2.75 |
| 32 | trans-Chlordane | 5103-74-2 | 5.2 x 10-4 | 1.37 x 10-4 | 3.80 | 6.0 | 5.88 | 6.34 x 102 | 3.16 x 101 | -2.63 |
| 33 | Chloroacetic acid | 79-11-8 | 18.7c | NA | NA | 0.22 | 0.26 | NA | NA | NA |
| 34 | 2-Chloroacetophenone | 532-27-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 35 | Chlorobenzene | 108-90-7 | 1.58 x 103 | 4.30 | 3.67 x 102 | 2.80 | 2.49 | 6.76 | 6.02 x 103 | -8.98 |
| 36 | Chlorobenzilate | 510-15-6 | NA | NA | NA | NA | NA | NA | NA | NA |
| 37 | Chloroform | 67-66-3 | 2.62 x 104 | 6.87 x 101 | 3.81 x 102 | 1.97 | 1.96 | 6.49 | 1.62 x 105 | -9.83 |
| 38 | Chloromethyl methyl ether | 107-30-2 | 2.49 x 104 | decomposes | NA | NA | NA | NA | NA | NA |
| 39 | Chloroprene | 126-99-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 40 | o-Cresol | 95-48-7 | 41c | 2.37 x 102 | 1.72 x 10-1 | 1.98 | 1.97 | 1.44 x 104 | 5.58 x 101 | -6.43 |
| 41 | m-Cresol | 108-39-4 | 1.6 x 101 | 1.75 x 102 | 9.14 x 10-2 | 1.98 | 1.97 | 2.71 x 104 | 3.27 x 101 | -6.20 |
| 42 | p-Cresol | 106-44-5 | 1.3 x 101c | 1.99 x 102 | 6.53 x 10-2 | 1.96 | 1.95 | 3.80 x 104 | 2.54 x 101 | -6.07 |
| 43 | oCresyl acetate | 533-18-6 | NA | NA | NA | NA | NA | NA | NA | NA |
| 44 | Cumene | 98-82-8 | 6.10 x 102 | 4.16 x 10-1 | 1.47 x 103 | 3.63 | 3.46 | 1.69 | 2.44 x 104 | -8.75 |
| 45 | Diazomethane | 334-88-3 | NA | NA | NA | NA | NA | NA | NA | NA |
| 46 | Dibenzofuran | 132-64-9 | 3.0 x 10-1 | 2.82 x 10-2 | 1.06 x 101 | 4.31 | 4.24 | 2.34 x 102 | 4.04 x 101 | -5.93 |
| 47 | 1,2-Dibromo-3-chloropropane | 96-12-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 48 | 2,4-Dichloro-phenoxyacetic acid | 94-75-7 | 8.0 x 10-5 | 1.81 | 4.42 x 10-5 | 2.81 | 3.09 | 5.62 x 107 | 2.08 | -2.05 |
| 49 | 1,1-dichloro-2,2-bis(p-chlorophenyl)ethylene | 72-55-9 | 8.66 x 10-4 | 1.26 x 10-4 | 6.87 | 5.70 | NA | 3.61 x 102 | NA | -4.42 |
| 50 | Dibutyl phthalate | 84-74-2 | 2 x 10-3 | 4.0 x 10-2 | 5.0 x 10-2 | 4.72 | 4.64 | 4.95 x 104 | 6.46 x 101 | -3.19 |

Table D.1.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 1,4-Dichlorobenzene | 106-46-7 | 9.02 x 101 | 5.65 x 10-1 | 1.60 x 102 | 3.40 | 3.35 | 1.55 x 101 | 5.04 x 102 | -8.02 |
| 52 | 3,3'-Dichlorobenzidine | 84-68-4 | 5.6 x 10-5 | 1.22 x 10-2 | 4.60 x 10-3 | 3.51 | 3.46 | 5.38 x 105 | 7.62 | -3.40 |
| 53 | Dichloroethyl ether | 111-44-4 | 2.06 x 102 | 7.13 x 101 | 2.89 | 1.12 | 1.18 | 8.55 x 102 | 7.20 x 104 | -8.56 |
| 54 | trans-1,3-Dichloropropene | 10061-02-6 | NA | NA | NA | NA | NA | NA | NA | NA |
| 55 | Dichlorvos | 62-73-7 | 7.02 | 3.62 x 101 | 1.94 x 10-1 | 1.45 |  | 1.28 x 104 | NA | -7.05 |
| 56 | Diethanolamine | 111-42-2 | 3.73 x 10-2 | miscible | NA | -1.43 | -1.35 | NA | NA | NA |
| 57 | Diethyl sulfate | 64-67-5 | 4.91 x 101 | NA | NA | NA | NA | NA | NA | NA |
| 58 | Diethylene glycol monobutyl ether | 112-34-5 | NA | NA | NA | NA | NA | NA | NA | NA |
| 59 | 3,3'-Dimethoxybenzidine | 119-90-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 60 | 4-Dimethylamino- azobenzene | 60-11-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 61 | N,N-Dimethylaniline | 121-69-7 | 1.07 x 102 | 9.12 | 1.17 x 101 | 2.31 | 2.29 | 2.12 x 102 | 7.30 x 103 | -7.98 |
| 62 | 3,3'-Dimethylbenzidine | 119-93-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 63 | Dimethylcarbamyl chloride | 79-44-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 64 | N,N-Dimethylformamide | 68-12-2 | NA | NA | NA | NA | NA | NA | NA | NA |
| 65 | 1,1-Dimethylhydrazine | 57-14-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 66 | Dimethyl phthalate | 131-11-3 | 2.2 x 10-1 | 2.06 x 101 | 1.07 x 10-2 | 2.12 | 2.11 | 2.31 x 105 | 7.96 x 101 | -5.14 |
| 67 | Dimethyl sulfate | 77-78-1 | 1.28 x 102 | NA | NA | NA | NA | NA | NA | NA |
| 68 | 4,6-Dinitro-o-Cresol | 534-52-1 | NA | 1.77 | NA | 2.12 | 2.11 | NA | NA | NA |
| 69 | 2,4-Dinitrophenol | 51-28-5 | NA | 1.82 | NA | 1.67 | 1.67 | NA | NA | NA |
| 70 | 2,4-Dinitrotoluene | 121-14-2 | 1.33 x 10-1 | 1.48 | 8.99 x 10-2 | 2.01 | 2.00 | 2.75 x 104 | 2.28 x 102 | -6.16 |
| 71 | 1,4-Dioxane | 123-91-1 | 5.00 x 103 | miscible | NA | -0.27 | -0.22 | NA | NA | NA |
| 72 | 1,2-Diphenylhydrazine | 122-66-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 73 | Epichlorohydrin | 13403-37-7 | 2.40 x 103 | 7.11 x 102 | 3.38 | 0.30 | 0.29 | 7.35 x 102 | 1.07 x 106 | -9.44 |
| 74 | 1,2-Epoxybutane | 106-88-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 75 | Ethyl acrylate | 140-88-5 | 5.10 x 103 | 1.50 x 102 | 3.40 x 101 | 1.33 | 1.01 | 7.30 x 101 | 7.63 x 105 | -9.42 |

Table D.1.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 76 | Ethylbenzene | 100-41-4 | 1.27 x 103 | 1.43 | 8.88 x 102 | 3.13 | 2.96 | 2.79 | 1.65 x 104 | -9.03 |
| 77 | Ethyl carbamate | 51-79-6 | NA | NA | NA | NA | NA | NA | NA | NA |
| 78 | Ethyl chloride | 75-00-3 | 1.60 x 104 | 8.83 x 101 | 1.81 x 102 | 1.43 | 1.29 | 1.37 x 101 | 2.11 x 104 | -10.00 |
| 79 | Ethylene dibromide | 106-93-4 | 1.04 x 104 | 2.21 x 101 | 4.71 x 102 | NA | 1.87 | 5.26 | 2.75 x 104 |  |
| 80 | Ethylene dichloride | 107-06-2 | 1.05 x 104 | 8.70 x 101 | 1.21 x 102 | 1.48 | 1.49 | 2.05 x 101 | 1.31 x 105 | -9.82 |
| 81 | Ethylene glycol | 107-21-1 | 1.2 x 101 | miscible | 6 x 10-3 | -1.36 | -1.28 | 4.13 x 105 | 4.32 x 108 | -8.35 |
| 82 | Ethylene glycol monobutyl ether | 111-76-2 | NA | NA | NA | NA | NA | NA | NA | NA |
| 83 | Ethyleneimine | 151-56-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 84 | Ethylene oxide | 75-21-8 | 1.46 x 105 | 8.70 x 103 | 1.16 x 101b | -0.30 | -0.25 | 2.14 x 102 | 1.97 x 1011 | -10.60 |
| 85 | Ethylene thiourea | 96-45-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 86 | Ethylidene dichloride | 75-34-3 | 3.03 x 104 | 4.82 x 101 | 6.29 x 102 | 1.79 | 1.58 | 3.94 | 1.16 x 105 | -10.20 |
| 87 | Formaldehyde | 50-00-0 | NA | miscible | 3.20 x 10-2 | 0.35 | 0.39 | 7.75 x 104 | 4.95 x 101 | -7.37 |
| 88 | Heptachlor | 76-44-8 | 5.3 x 10-2 | 1.50 x 10-4 | 3.53 x 102 | 5.27 | NA | 7.04 | NA | -6.49 |
| 89 | Hexachlorobenzene | 118-74-1 | 2.3 x 10-3 | 1.76 x 10-5 | 1.31 x 102 | 5.50 | 5.40 | 1.89 x 101 | 4.03 x 102 | -5.83 |
| 90 | Hexachlorobutadiene | 87-68-3 | 2.00 x 101 | 1.3 x 10-2 | 1.54 x 103 | 4.70 | 4.62 | 1.61 | 1.45 x 104 | -7.70 |
| 91 | (α)1,2,3,4,5,6-Hexachlorocyclohexane | 60291-32-9 | 3 x 10-3 | 3.44 x 10-3 | 8.72 x 10-1 | 3.81 | 3.75 | 2.84 x 103 | 3.69 x 102 | -5.35 |
| 92 | (γ)1,2,3,4,5,6-Hexachlorocyclohexane | 58-89-9 | 3.74 x 10-3 | 2.51 x 10-2 | 1.49 x 10-1 | 3.70 | 3.65 | 1.66 x 104 | 7.89 x 101 | -4.69 |
| 93 | Hexachloro- cyclopentadiene | 77-47-4 | 1.09 x 101 | 6.6 x 10-3 | 1.65 x 103 | 5.04 | 4.95 | 1.50 | 8.07 x 103 | -7.40 |
| 94 | Hexachloroethane | 67-72-1 | 5.0 x 101 | 2.11 x 10-1 | 2.37 x 102 | 3.93 | 3.87 | 1.05 x 101 | 2.94 x 103 | -7.66 |
| 95 | Hexamethylene diisocyanate | 822-06-0 | NA | NA | NA | NA | NA | NA | NA | NA |
| 96 | Hexamethyl- phosphoramide | 680-31-9 | NA | NA | NA | NA | NA | NA | NA | NA |
| 97 | Hexane | 110-54-3 | 2.02 x 104 | 1.10 x 10-1 | 1.84 x 105 | 4.11 | 4.20 | 1.35 x 10-2 | 2.73 x 104 | -10.40 |
| 98 | Hydrazine | 302-01-2 | NA | NA | NA | NA | NA | NA | NA | NA |
| 99 | Hydroquinone | 123-31-9 | 2.55 x 10-3 | 6.36 x 102 | 4.01 x 10-6 | 0.59 | 0.62 | 6.17 x 108 | 3.66 x 10-1 | -3.23 |
| 100 | Isophorone | 78-59-1 | 5.0 x 101 | 8.68 x 101 | 5.76 x 10-1 | 1.70 | NA | 4.31 x 103 | NA | -7.28 |

Table D.1.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | Maleic anhydride | 108-31-6 | NA | NA | NA | NA | NA | NA | NA | NA |
| 102 | Methanol | 67-56-1 | 1.62 x 104 | miscible | 4.5 x 10-1 | -0.77 | -1.32 | 5.49 x 103 | 2.68 x 1014 | -9.64 |
| 103 | Methoxychlor | 72-43-5 | 1.3 x 10-4 | 1.30 x 10-4 | 1.00 | 5.08 |  | 2.48 x 103 |  | -4.14 |
| 104 | Methyl bromide | 74-83-9 | 2.18 x 105 | 1.60 x 102 | 6.33 x 102b | 1.19 | 0.82 | 3.92 | 2.49 x 105 | -10.80 |
| 105 | Methyl chloride | 74-87-3 | 5.70 x 105 | 1.04 x 102 | 9.74 x 102b | 0.91 | 0.64 | 2.54 | 1.89 x 106 | -11.30 |
| 106 | Methyl chloroform | 71-55-6 | 1.65 x 104 | 1.12 x 101 | 1.47 x 103 | 2.49 | 2.19 | 1.69 | 8.15 x 104 | -9.89 |
| 107 | Methyl ethyl ketone | 78-93-3 | 1.21 x 104 | 3.33 x 103 | 3.63 | 0.29 | -0.30 | 6.85 x 102 | 3.88 x 107 | -9.49 |
| 108 | Methylhydrazine | 60-34-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 109 | Methyl iodide | 74-88-4 | 5.4 x 104 | 9.79 x 101 | 5.52 x 102 | 1.50 | 1.30 | 4.48 | 1.26 x 104 | -10.50 |
| 110 | Methyl isobutyl ketone | 108-10-1 | 2.60 x 103 | 1.70 x 102 | 1.53 x 101 | 0.90 | 0.92 | 1.62 x 102 | 8.24 x 105 | -9.50 |
| 111 | Methyl isocyanate | 624-83-9 | NA | NA | NA | NA | NA | NA | NA | NA |
| 112 | Methyl methacrylate | 80-62-6 | 5.00 x 103 | 1.56 x 102 | 3.21 x 101 | 1.38 | 0.99 | 7.75 x 101 | 7.36 x 105 | -9.34 |
| 113 | Methyl tert-butyl ether | 1634-04-4 | 3.35 x 104 | 4.76 x 102 | 7.04 x 101 | 0.94 | 0.59 | 3.52 x 101 | 1.11 x 107 | -10.10 |
| 114 | 4,4'-Methylenebis (2-chloroaniline) | 101-14-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 115 | Methylene chloride | 75-09-2 | 2.62 x 104 | 1.55 x 102 | 1.69 x 102 | 1.25 | 1.26 | 1.47 x 101 | 9.07 x 104 | -10.20 |
| 116 | 4,4'-Methylenediphenyl diisocyanate | 101-68-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 117 | 4,4'-Methylenedianiline | 101-77-9 | NA | NA | NA | NA | NA | NA | NA | NA |
| 118 | Naphthalene | 91-20-3 | 1.04 x 101 | 2.42 x 10-1 | 4.30 x 101 | 3.37 | 3.32 | 5.78 x 101 | 3.65 x 102 | -7.48 |
| 119 | Nitrobenzene | 98-95-3 | 2.0 x 101 | 1.54 x 101 | 1.30 | 1.85 | 1.85 | 1.91 x 103 | 7.51 x 102 | -7.48 |
| 120 | 4-Nitrobiphenyl | 92-93-3 | NA | 6.18 x 10-3 | NA | 3.78 | 3.72 | NA | NA | NA |
| 121 | 4-Nitrophenol | 100-02-7 | NA | 1.07 x 102 | 5.07 x 10-5 | 1.91 | 1.90 | 4.88 x 107 | NA | NA |
| 122 | 2-Nitropropane | 79-46-9 | NA | NA | NA | NA | NA | NA | NA | NA |
| 123 | N-Nitroso-N-methylurea | 684-93-5 | NA | NA | NA | NA | NA | NA | NA | NA |
| 124 | N-Nitrosodimethylamine | 62-75-9 | NA | miscible | 3.34 | -0.57 | -0.51 | 7.41 x 102 | 1.58 x 108 | -10.30 |
| 125 | N-Nitrosomorpholine | 59-89-2 | NA | NA | NA | NA | NA | NA | NA | NA |

Table D.1.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 126 | Parathion | 56-38-2 | 6.0 x 10-4 | 4.26 x 10-2 | 1.41 x 10-2 | 3.80 | NA | 1.76 x 105 | NA | -3.56 |
| 127 | Pentachloro- nitrobenzene | 82-68-8 | 6.6 x 10-3 | 1.5 x 10-3 | 4.40 | 4.64 | NA | 5.65 x 102 | NA | -5.22 |
| 128 | Pentachlorophenol | 87-86-5 | 4.15 x 10-3 | 5.3 x 10-2 | 7.83 x 10-2 | 5.05 | 4.96 | 3.16 x 104 | 2.56 x 10-1 | -3.06 |
| 129 | Phenol | 108-95-2 | 4.7 x 101 | 9.39 x 102 | 5.01 x 10-2 | 1.46 | 1.47 | 4.95 x 104 | 3.33 x 101 | -6.49 |
| 130 | p-Phenylenediamine | 106-50-3 | NA | NA | NA | NA | NA | NA | NA | NA |
| 131 | Phosgene | 75-44-5 | NA | NA | NA | NA | NA | NA | NA | NA |
| 132 | Phosphine | 7803-51-2 | NA | NA | NA | NA | NA | NA | NA | NA |
| 133 | Phthalic anhydride | 85-44-9 | NA | NA | NA | NA | NA | NA | NA | NA |
| 134 | 1,3-Propane sultone | 1120-71-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 135 | β-Propiolactone | 57-57-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 136 | Propionaldehyde | 123-38-6 | 4.24 x 104 | 5.34 x 102 | 7.94 x 101 | 0.59 | 0.55 | 3.13 x 101 | 2.57 x 106 | -10.50 |
| 137 | Propoxur | 114-26-1 | 1.70 x 10-5 | 8.60 | 1.98 x 10-6 | 1.50 | NA | 1.25 x 109 | NA | -2.01 |
| 138 | 1,2-Dichloropropane | 26198-63-0 | 6.62 x 103 | 2.48 x 101 | 2.67 x 102 | 2.00 | NA | 9.26 | NA | -9.64 |
| 139 | Propylene oxide | 75-56-9 | 7.10 x 104 | 8.20 x 103 | 8.66 | 0.03 | -0.61 | 2.87 x 102 | 1.91 x 109 | -10.10 |
| 140 | 1,2-Propylenimine | 75-55-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 141 | Quinoline | 91-22-5 | 1.21 | 4.73 x 101 | 2.56 x 10-2 | 2.06 | 2.05 | 9.71 x 104 | 2.16 x 101 | -5.57 |
| 142 | Quinone | 106-51-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 143 | Styrene | 100-42-5 | 8.80 x 102 | 2.88 | 3.06 x 102 | 3.05 | 2.64 | 8.13 | 1.13 x 104 | -8.65 |
| 144 | Styrene oxide | 96-09-3 | 4.0 x 101 | 2.33 x 101 | 1.72 | 1.61 | 1.61 | 1.44 x 103 | 5.57 x 103 | -7.84 |
| 145 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin | 1746-01-6 | 2.00 x 10-7 | 6 x 10-5 | 3.34 | 6.80 | NA | 7.43 x 102 | NA | -2.94 |
| 146 | 1,1,2,2-Tetrachloroethane | 79-34-5 | 7.93 x 102 | 1.77 x 101 | 4.48 x 101 | 2.39 | 1.85 | 5.52 x 101 | 4.95 x 104 | -8.48 |
| 147 | Tetrachloroethylene | 127-18-4 | 2.42 x 103 | 9.0 x 10-1 | 2.69 x 103 | 2.88 | 2.85 | 9.17 x 10-1 | 6.04 x 103 | -9.76 |
| 148 | Toluene | 108-88-3 | 3.80 x 103 | 5.59 | 6.80 x 102 | 2.69 | 2.42 | 3.65 | 1.74 x 104 | -9.36 |
| 149 | Toluene-2,4-diamine | 95-80-7 | NA | NA | NA | NA | NA | NA | NA | NA |
| 150 | 2,4-Toluene diisocyanate | 584-84-9 | NA | NA | NA | NA | NA | NA | NA | NA |

Table D.1.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 151 | o-Toluidine | 95-53-4 | 1.33 x 101 | 1.40 x 102 | 9.50 x 10-2 | NA | NA | 2.61 x 104 | NA | NA |
| 152 | Toxaphene | 8001-35-2 | 9 x 10-4 | 1.21 x 10-3 | 7.44 x 10-1 | 5.50 | 5.40 | 3.33 x 103 | 3.80 x 102 | -3.59 |
| 153 | 1,2,4-Trichlorobenzene | 120-82-1 | 6.1 x 101 | 2.20 x 10-1 | 2.77 x 102 | 4.10 | 4.03 | 8.93 | 4.82 x 102 | -7.56 |
| 154 | 1,1,2-Trichloroethane | 79-00-5 | 3.22 x 103 | 3.29 x 101 | 9.79 x 101 | 2.38 | 1.64 | 2.53 x 101 | 7.33 x 104 | -8.83 |
| 155 | Trichloroethylene | 79-01-6 | 9.90 x 103 | 8.37 | 1.18 x 103 | 2.53 | 2.51 | 2.10 | 1.88 x 104 | -9.76 |
| 156 | 2,4,5-Trichlorophenol | 95-95-4 | 2.5 | 4.80 | 5.21 x 10-1 | 3.72 | 3.66 | 4.76 x 103 | 5.70 | -5.21 |
| 157 | 2,4,6-Trichlorophenol | 88-06-2 | 1.25 | 2.20 | 5.68 x 10-1 | 3.69 | 3.64 | 4.37 x 103 | 6.84 | -5.28 |
| 158 | Triethylamine | 121-44-8 | 7.61 x 103 | 5.40 x 102 | 1.41 x 101 | 1.64 | 0.47 | 1.76 x 102 | 5.30 x 106 | -8.72 |
| 159 | Trifluralin | 1582-09-8 | NA | 1.49 x 10-3 | 1.01 x 101 | 5.34 | NA | 2.46 x 102 | NA | -4.88 |
| 160 | 2,2,4-Trimethylpentane | 540-84-1 | 6.56 x 103 | 2.14 x 10-2 | 3.07 x 105 | 4.95 | 4.86 | 8.06 x 10-3 | 3.30 x 105 | -9.72 |
| 161 | Vinyl acetate | 108-05-4 | 1.41 x 104 | 2.32 x 102 | 6.08 x 101 | 0.73 | 0.86 | 4.08 x 101 | 1.10 x 106 | -10.30 |
| 162 | Vinyl bromide | 593-60-2 | 1.40 x 105 | NA | NA | NA | NA | NA | NA | NA |
| 163 | Vinyl chloride | 75-01-4 | 3.55 x 105 | 4.42 x 101 | 2.29 x 103b | 1.38 | 1.39 | 3.09 x 10-1 | 7.35 x 103 | -11.70 |
| 164 | Vinylidene chloride | 75-35-4 | 8.05 x 104 | 3.45 x 101 | 2.33 x 103 | 2.13 | 1.77 | 1.06 | 5.78 x 104 | -10.50 |
| 165 | o-Xylene | 95-47-6 | 1.17 x 103 | 2.07 | 5.65 x 102 | 3.15 | 2.80 | 4.39 | 2.00 x 104 | -8.82 |
| 166 | m-Xylene | 108-38-3 | 1.10 x 103 | 1.51 | 7.28 x 102 | 3.20 | 2.93 | 3.40 | 1.53 x 104 | -8.88 |
| 167 | p-Xylene | 106-42-3 | 1.17 x 103 | 2.02 | 5.79 x 102 | 3.18 | 2.81 | 4.27 | 1.97 x 104 | -8.80 |

aNA, not available

bVapor pressure exceeds atmospheric pressure.  H calculated as 101325 Pa divided by the aqueous solubility (mol m-3).

cVapor pressure of subcooled liquid.

Table D.2.  Additional physicochemical properties of polychlorinated byphenyls (PCB) congeners at 298.2 K.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2-Monochloro | 2051-60-7 | 2.04 | 29.15 | 30.2 | 4.51 | 4.43 | 8.21 x 101 | 4.36 x 102 | -6.19 |
| 2 | 3-Monochloro | 2051-61-8 | 1 | 13.25 | 29.0 | 4.70 | 4.62 | 8.55 x 101 | 2.52 x 102 | -5.98 |
| 3 | 4-Monochloro | 2051-62-9 | 0.271 | 6.36 | 27.8 | 4.71 | 4.63 | 8.92 x 101 | 2.35 x 102 | -5.95 |
| 4 | 2,2'-Dichloro | 13029-08-8 | 0.265 | 4.48 | 33.3 | 4.84 | 4.75 | 7.44 x 101 | 6.59 x 102 | -5.90 |
| 5 | 2,3-Dichloro | 16605-91-7 | NAc | NA | 24.2 | 4.95 | 4.86 | 1.02 x 102 | 3.69 x 102 | -5.65 |
| 6 | 2,3'-Dichloro | 25569-80-6 | NA | NA | 33.1 | 5.02 | 4.93 | 7.49 x 101 | 4.24 x 102 | -5.72 |
| 7 | 2,4-Dichloro | 33284-50-3 | 0.254 | 5.6 | 38.2 | 5.03 | 4.94 | 6.49x 101 | 4.77 x 102 | -5.77 |
| 8 | 2,4'-Dichloro | 34883-43-7 | NA | 4.48 | 30.7 | 5.03 | 4.94 | 8.07 x 101 | 3.84 x 102 | -5.67 |
| 9 | 2,5-Dichloro | 34883-39-1 | 0.18 | 8.960 | 32.8 | 5.02 | 4.93 | 7.56 x 101 | 4.20 x 102 | -5.71 |
| 10 | 2,6-Dichloro | 33146-45-1 | NA | 6.280 | 42.9 | 4.83 | 4.74 | 5.78 x 101 | 8.69 x 102 | -6.02 |
| 11 | 3,3'-Dichloro | 2050-67-1 | 0.027 | 1.587 | 29.4 | 5.21 | 5.11 | 8.43 x 101 | 2.44 x 102 | -5.47 |
| 12 | 3,4-Dichloro | 2974-92-7 | NA | 0.04 | 23.7 | 5.16 | 5.07 | 1.05 x 102 | 2.15 x 102 | -5.43 |
| 13 | 3,4'-Dichloro | 2974-90-5 | NA | NA | 25.7 | 5.22 | 5.12 | 9.65 x 101 | 2.08 x 102 | -5.41 |
| 14 | 3,5-Dichloro | 34883-41-5 | 0.105 | NA | 42.6 | 5.21 | 5.11 | 5.82 x 101 | 3.53 x 102 | -5.64 |
| 15 | 4,4'-Dichloro | 2050-68-2 | 0.0048 | 0.269 | 22.7 | 5.22 | 5.12 | 1.09 x 102 | 1.84 x 102 | -5.35 |
| 16 | 2,2',3-Trichloro | 38444-78-9 | NA | NA | 25.5 | 5.27 | 5.17 | 9.72 x 101 | 5.25 x 102 | -5.35 |
| 17 | 2,2',4-Trichloro | 37680-66-3 | NA | NA | 37.8 | 5.35 | 5.25 | 6.56 x 101 | 6.51 x 102 | -5.44 |
| 18 | 2,2',5-Trichloro | 37680-65-2 | 0.143 | 1.550 | 32.4 | 5.35 | 5.25 | 7.65 x 101 | 5.59 x 102 | -5.38 |
| 19 | 2,2',6-Trichloro | 38444-73-4 | NA | NA | 44.7 | 5.16 | 5.07 | 5.55 x 101 | 1.15 x 103 | -5.71 |
| 20 | 2,3,3'-Trichloro | 38444-84-7 | NA | NA | 22.0 | 5.46 | 5.36 | 1.13 x 102 | 2.97 x 102 | -5.10 |
| 21 | 2,3,4-Trichloro | 55702-46-0 | NA | NA | 23.0 | 5.41 | 5.31 | 1.08 x 102 | 3.47 x 102 | -5.17 |
| 22 | 2,3,4'-Trichloro | 38444-85-8 | NA | NA | 19.4 | 5.47 | 5.37 | 1.28 x 102 | 2.56x 102 | -5.03 |
| 23 | 2,3,5-Trichloro | 55720-44-0 | NA | NA | 32.3 | 5.46 | 5.36 | 7.67 x 101 | 4.38 x 102 | -5.27 |
| 24 | 2,3,6-Trichloro | 58702-45-9 | NA | NA | 31.5 | 5.27 | 5.17 | 7.87 x 101 | 6.48 x 102 | -5.44 |
| 25 | 2,3',4-Trichloro | 55712-37-3 | NA | NA | 32.0 | 5.54 | 5.44 | 7.75 x 101 | 3.62 x 102 | -5.18 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 2,3',5-Trichloro | 38444-81-4 | NA | 0.975 | 30.2 | 5.54 | 5.44 | 8.21 x 101 | 3.42 x 102 | -5.16 |
| 27 | 2,3',6-Trichloro | 38444-76-7 | NA | NA | 41.0 | 5.35 | 5.25 | 6.05 x 101 | 7.06 x 102 | -5.48 |
| 28 | 2,4,4'-Trichloro | 7012-37-5 | NA | 0.621 | 29.0 | 5.55 | 5.45 | 8.55 x 101 | 3.21 x 102 | -5.13 |
| 29 | 2,4,5-Trichloro | 15862-07-4 | 0.0132 | 0.544 | 30.0 | 5.49 | 5.39 | 8.26 x 101 | 3.80 x 102 | -5.20 |
| 30 | 2,4,6-Trichloro | 35693-92-6 | 0.0384 | 0.777 | 58.0 | 5.35 | 5.25 | 4.27 x 101 | 1.00 x 103 | -5.63 |
| 31 | 2,4',5-Trichloro | 16606-02-3 | NA | NA | 27.8 | 5.54 | 5.44 | 8.92 x 101 | 3.15 x 102 | -5.12 |
| 32 | 2,4',6-Trichloro | 38444-77-8 | NA | NA | 39.7 | 5.35 | 5.25 | 6.24 x 101 | 6.85 x 102 | -5.46 |
| 33 | 2',3,4-Trichloro | 38444-86-9 | 0.0136 | 0.311 | 24.3 | 5.49 | 5.39 | 1.02 x 102 | 3.07x 102 | -5.11 |
| 34 | 2',3,5-Trichloro | 37680-68-5 | NA | NA | 42.7 | 5.54 | 5.44 | 5.81 x 101 | 4.83 x 102 | -5.31 |
| 35 | 3,3',4-Trichloro | 37680-69-6 | NA | NA | 18.2 | 5.68 | 5.57 | 1.36 x 102 | 1.55 x 102 | -4.80 |
| 36 | 3,3',5-Trichloro | 38444-87-0 | NA | NA | 34.1 | 5.73 | 5.62 | 7.27 x 101 | 2.60 x 102 | -5.02 |
| 37 | 3,4,4'-Trichloro | 38444-90-5 | NA | 0.0582 | 15.4 | 5.68 | 5.57 | 1.61 x 102 | 1.31 x 102 | -4.72 |
| 38 | 3,4,5-Trichloro | 53555-66-1 | NA | NA | 23.5 | 5.62 | 5.51 | 1.05 x 102 | 2.29 x 102 | -4.97 |
| 39 | 3,4',5-Trichloro | 38444-88-1 | NA | NA | 30.3 | 5.73 | 5.62 | 8.18 x 101 | 2.31 x 102 | -4.97 |
| 40 | 2,2',3,3'-Tetrachloro | 38444-93-8 | 0.00225 | 0.103 | 18.5 | 5.71 | 5.60 | 1.34 x 102 | 3.87 x 102 | -4.77 |
| 41 | 2,2'3,4-Tetrachloro | 52663-59-9 | NA | NA | 24.8 | 5.74 | 5.63 | 1.00 x 102 | 4.88 x 102 | -4.87 |
| 42 | 2,2',3,4'-Tetrachloro | 36559-22-5 | NA | NA | 25.9 | 5.79 | 5.68 | 9.57 x 101 | 4.60 x 102 | -4.84 |
| 43 | 2,2',3,5-Tetrachloro | 70362-46-8 | NA | NA | 33.9 | 5.79 | 5.68 | 7.31 x 101 | 6.03x 102 | -4.96 |
| 44 | 2,2',3,5'-Tetrachloro | 41464-39-5 | NA | 0.342 | 23.3 | 5.79 | 5.68 | 1.06x 102 | 4.16 x 102 | -4.79 |
| 45 | 2,2'3,6-Tetrachloro | 70362-45-7 | NA | NA | 36.0 | 5.60 | 5.49 | 6.89 x 101 | 9.42 x 102 | -5.17 |
| 46 | 2,2',3,6'-Tetrachloro | 41464-47-5 | NA | NA | 34.3 | 5.51 | 5.41 | 7.23 x 101 | 1.06 x 103 | -5.24 |
| 47 | 2,2',4,4'-Tetrachloro | 2437-79-8 | 0.0054 | 0.308 | 37.3 | 5.87 | 5.76 | 6.65 x 101 | 5.63 x 102 | -4.92 |
| 48 | 2,2',4,5-Tetrachloro | 70362-47-9 | NA | NA | 30.7 | 5.81 | 5.70 | 8.07 x 101 | 5.23 x 102 | -4.89 |
| 49 | 2,2',4,5'-Tetrachloro | 41464-40-8 | NA | 0.0548 | 35.8 | 5.89 | 5.78 | 6.92 x 101 | 5.18 x 102 | -4.88 |
| 50 | 2,2',4,6-Tetrachloro | 62796-65-8 | NA | NA | 61.8 | 5.68 | 5.57 | 4.01 x 101 | 1.38 x 103 | -5.33 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 2,2',4,6'-Tetrachloro | 65194-04-7 | NA | NA | 51.7 | 5.68 | 5.57 | 4.78 x 101 | 1.15 x 103 | -5.25 |
| 52 | 2,2',5,5'-Tetrachloro | 35693-99-3 | 0.0049 | 0.103 | 32.3 | 5.86 | 5.75 | 7.67 x 101 | 4.98 x 102 | -4.87 |
| 53 | 2,2',5,6'-Tetrachloro | 41464-41-9 | NA | NA | 43.6 | 5.67 | 5.56 | 5.69 x 101 | 9.87 x 102 | -5.18 |
| 54 | 2,2',6,6'-Tetrachloro | 15968-05-5 | NA | NA | 58.0 | 5.32 | 5.22 | 4.27 x 101 | 2.64 x 103 | -5.66 |
| 55 | 2,3,3',4-Tetrachloro | 74338-24-2 | NA | NA | 18.5 | 5.92 | 5.81 | 1.34 x 102 | 2.52 x 102 | -4.56 |
| 56 | 2,3,3',4'-Tetrachloro | 41464-43-1 | NA | NA | 15.3 | 5.92 | 5.81 | 1.62 x 102 | 2.09 x 102 | -4.48 |
| 57 | 2,3,3',5-Tetrachloro | 70424-67-8 | NA | NA | 27.4 | 5.97 | 5.85 | 9.05 x 101 | 3.44 x 102 | -4.68 |
| 58 | 2,3,3',5'-Tetrachloro | 41464-49-7 | NA | NA | 25.3 | 5.97 | 5.85 | 9.80 x 101 | 3.18 x 102 | -4.65 |
| 59 | 2,3,3',6-Tetrachloro | 74472-33-6 | NA | NA | 30.8 | 5.78 | 5.67 | 8.05 x 101 | 5.57 x 102 | -4.92 |
| 60 | 2,3,4,4'-Tetrachloro | 33025-41-1 | NA | NA | 15.5 | 5.93 | 5.81 | 1.60 x 102 | 2.11 x 102 | -4.48 |
| 61 | 2,3,4,5-Tetrachloro | 33284-53-6 | NA | 0.0685 | 24.1 | 5.87 | 5.76 | 1.03 x 102 | 3.64 x 102 | -4.73 |
| 62 | 2,3,4,6-Tetrachloro | 54230-23-7 | NA | NA | 37.5 | 5.73 | 5.62 | 6.61 x 101 | 7.53 x 102 | -5.06 |
| 63 | 2,3,4',5-Tetrachloro | 74472-35-8 | NA | NA | 24.6 | 5.98 | 5.86 | 1.01 x 102 | 3.02 x 102 | -4.63 |
| 64 | 2,3,4',6-Tetrachloro | 52663-58-8 | NA | NA | 27.6 | 5.79 | 5.68 | 8.98 x 101 | 4.91 x 102 | -4.87 |
| 65 | 2,3,5,6-Tetrachloro | 33284-54-7 | NA | NA | 34.1 | 5.71 | 5.60 | 7.27 x 101 | 7.13 x 102 | -5.04 |
| 66 | 2,3',4,4'-Tetrachloro | 32598-10-0 | NA | 0.0147 | 20.5 | 6.01 | 5.89 | 1.21 x 102 | 2.37 x 102 | -4.52 |
| 67 | 2,3',4,5-Tetrachloro | 73575-53-8 | NA | NA | 23.7 | 6.00 | 5.88 | 1.05 x 102 | 2.78 x 102 | -4.59 |
| 68 | 2,3',4,5'-Tetrachloro | 73575-52-7 | NA | NA | 38.2 | 6.06 | 5.94 | 6.49 x 101 | 3.99 x 102 | -4.74 |
| 69 | 2,3',4,6-Tetrachloro | 60233-24-1 | NA | NA | 51.3 | 5.87 | 5.76 | 4.83 x 101 | 7.75 x 102 | -5.06 |
| 70 | 2,3',4',5-Tetrachloro | 32598-11-1 | NA | NA | 20.5 | 6.00 | 5.88 | 1.21 x 102 | 2.42 x 102 | -4.53 |
| 71 | 2,3',4',6-Tetrachloro | 41464-46-4 | NA | NA | 31.8 | 5.81 | 5.70 | 7.80 x 101 | 5.41 x 102 | -4.91 |
| 72 | 2,4',5,5'-Tetrachloro | 41464-42-0 | NA | NA | 36.7 | 6.05 | 5.93 | 6.75 x 101 | 3.91 x 102 | -4.73 |
| 73 | 2,3',5',6-Tetrachloro | 74338-23-1 | NA | NA | 52.7 | 5.86 | 5.75 | 4.70 x 101 | 8.12 x 102 | -5.08 |
| 74 | 2,4,4',5-Tetrachloro | 32690-93-0 | NA | NA | 21.8 | 6.01 | 5.89 | 1.14 x 102 | 2.51 x 102 | -4.54 |
| 75 | 2,4,4',6-Tetrachloro | 32598-12-2 | NA | 0.32 | 47.1 | 5.87 | 5.76 | 5.26 x 101 | 7.12 x 102 | -5.02 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 76 | 2',3,4,5-Tetrachloro | 70362-48-0 | NA | NA | 24.2 | 5.94 | 5.82 | 1.02 x 102 | 3.25 x 102 | -4.66 |
| 77 | 3,3',4,4'-Tetrachloro | 32598-13-3 | 5.88 x 10-5 | 0.0342 | 10.4 | 6.14 | 6.02 | 2.38 x 102 | 9.23 x 101 | -4.09 |
| 78 | 3,3'4,5-Tetrachloro | 70362-49-1 | NA | NA | 16.5 | 6.14 | 6.02 | 1.50 x 102 | 1.47 x 102 | -4.29 |
| 79 | 3,3',4,5'-Tetrachloro | 41464-48-6 | NA | NA | 20.0 | 6.19 | 6.07 | 1.24 x 102 | 1.60 x 102 | -4.33 |
| 80 | 3,3',5,5'-Tetrachloro | 33284-52-5 | NA | 0.0041 | 38.0 | 6.24 | 6.12 | 6.52 x 101 | 2.75 x 102 | -4.56 |
| 81 | 3,4,4',5-Tetrachloro | 70362-50-4 | NA | NA | 14.5 | 6.14 | 6.02 | 1.71 x 102 | 1.29 x 102 | -4.24 |
| 82 | 2,2',3,3',4-Pentachloro | 52663-62-4 | NA | NA | 14.8 | 6.17 | 6.05 | 1.68 x 102 | 3.01 x 102 | -4.21 |
| 83 | 2,2',3,3',5-Pentachloro | 60145-20-2 | NA | NA | 21.5 | 6.23 | 6.11 | 1.15 x 102 | 3.92 x 102 | -4.32 |
| 84 | 2,2',3,3',6-Pentachloro | 52663-60-2 | NA | NA | 25.5 | 6.03 | 5.91 | 9.72 x 101 | 6.77 x 102 | -4.59 |
| 85 | 2,2',3,4,4'-Pentachloro | 65510-45-4 | NA | NA | 19.5 | 6.25 | 6.13 | 1.27 x 102 | 3.42 x 102 | -4.26 |
| 86 | 2,2',3,4,5-Pentachloro | 55312-69-1 | 0.00927 | 0.0613 | 24.1 | 6.19 | 6.07 | 1.03 x 102 | 4.71 x 102 | -4.41 |
| 87 | 2,2',3,4,5'-Pentachloro | 38380-02-8 | 3.04 x 10-4 | 0.0123 | 18.6 | 6.25 | 6.13 | 1.33 x 102 | 3.27 x 102 | -4.24 |
| 88 | 2,2',3,4,6-Pentachloro | 55215-17-3 | NA | 0.0368 | 39.0 | 6.06 | 5.94 | 6.36 x 101 | 9.77 x 102 | -4.75 |
| 89 | 2,2',3,4,6'-Pentachloro | 73575-57-2 | NA | NA | 30.2 | 6.06 | 5.94 | 8.21 x 101 | 7.57 x 102 | -4.64 |
| 90 | 2,2',3,4',5-Pentachloro | 68194-07-0 | NA | NA | 29.8 | 6.30 | 6.17 | 8.32 x 101 | 4.85 x 102 | -4.39 |
| 91 | 2,2',3,4',6-Pentachloro | 58194-05-8 | NA | NA | 35.1 | 6.11 | 5.99 | 7.06 x 101 | 8.00 x 102 | -4.65 |
| 92 | 2,2',3,5,5'-Pentachloro | 52663-61-3 | NA | NA | 26.3 | 6.30 | 6.17 | 9.43 x 101 | 4.28 x 102 | -4.34 |
| 93 | 2,2',3,5,6-Pentachloro | 73575-56-1 | NA | NA | 34.5 | 6.03 | 5.91 | 7.19 x 101 | 9.15 x 102 | -4.72 |
| 94 | 2,2',3,5,6'-Pentachloro | 73575-55-0 | NA | NA | 39.7 | 6.11 | 5.99 | 6.24 x 101 | 9.05 x 102 | -4.70 |
| 95 | 2,2',3,5',6-Pentachloro | 38379-99-6 | NA | NA | 30.4 | 6.11 | 5.99 | 8.15 x 101 | 6.93 x 102 | -4.59 |
| 96 | 2,2',3,6,6'-Pentachloro | 73575-54-9 | NA | NA | 41.6 | 5.75 | 5.64 | 5.96 x 101 | 1.84 x 103 | -5.08 |
| 97 | 2,2',3',4,5-Pentachloro | 41464-51-1 | NA | NA | 18.2 | 6.25 | 6.13 | 1.36 x 102 | 3.20 x 102 | -4.23 |
| 98 | 2,2',3',4,6-Pentachloro | 60233-25-2 | NA | NA | 39.7 | 6.11 | 5.99 | 6.24 x 101 | 9.05 x 102 | -4.70 |
| 99 | 2,2',4,4',5-Pentachloro | 38380-01-7 | NA | NA | 25.3 | 6.33 | 6.20 | 9.80 x 101 | 3.88 x 102 | -4.29 |
| 100 | 2,2',4,4',6-Pentachloro | 39485-83-1 | NA | NA | 57.0 | 6.20 | 6.08 | 4.35 x 101 | 1.09 x 103 | -4.77 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | 2,2'4,5,5'-Pentachloro | 37680-73-2 | 0.00109 | 0.0306 | 24.9 | 6.33 | 6.20 | 9.96 x 101 | 3.82 x 102 | -4.28 |
| 102 | 2,2',4,5,6'-Pentachloro | 68194-06-9 | NA | NA | 37.6 | 6.40 | 6.27 | 6.59 x 101 | 5.06 x 102 | -4.39 |
| 103 | 2,2',4,5',6-Pentachloro | 60145-21-3 | NA | NA | 51.0 | 6.19 | 6.07 | 4.86 x 101 | 9.99 x 102 | -4.73 |
| 104 | 2,2',4,6,6'-Pentachloro | 56558-16-8 | NA | 0.0306 | 75.1 | 5.83 | 5.72 | 3.30 x 101 | 2.86 x 103 | -5.26 |
| 105 | 2,3,3',4,4'-Pentachloro | 32598-14-4 | NA | NA | 10.1 | 6.39 | 6.26 | 2.45 x 102 | 1.38 x 102 | -3.83 |
| 106 | 2,3,3',4,5-Pentachloro | 70424-69-0 | NA | NA | 16.7 | 6.38 | 6.25 | 1.48 x 102 | 2.34 x 102 | -4.06 |
| 107 | 2,3,3',4',5-Pentachloro | 70424-68-9 | NA | NA | 16.1 | 6.44 | 6.31 | 1.54 x 102 | 2.00 x 102 | -3.98 |
| 108 | 2,3,3',4,5'-Pentachloro | 70362-41-3 | NA | NA | 17.8 | 6.44 | 6.31 | 1.39 x 102 | 2.22 x 102 | -4.03 |
| 109 | 2,3,3',4,6-Pentachloro | 74472-35-8 | NA | NA | 28.6 | 6.25 | 6.13 | 8.67 x 101 | 5.01 x 102 | -4.42 |
| 110 | 2,3,3',4',6-Pentachloro | 38380-03-9 | NA | 0.01 | 19.9 | 6.25 | 6.13 | 1.25 x 102 | 3.48 x 102 | -4.26 |
| 111 | 2,3,3',5,5'-Pentachloro | 39635-32-0 | NA | NA | 27.0 | 6.49 | 6.36 | 9.18 x 101 | 3.06 x 102 | -4.16 |
| 112 | 2,3,3',5,6-Pentachloro | 74472-36-9 | NA | NA | 27.0 | 6.22 | 6.10 | 9.18 x 101 | 5.00 x 102 | -4.43 |
| 113 | 2,3,3',5',6-Pentachloro | 68194-10-5 | NA | NA | 33.0 | 6.30 | 6.17 | 7.51 x 101 | 5.37 x 102 | -4.43 |
| 114 | 2,3,4,4',5-Pentachloro | 74472-37-0 | NA | NA | 14.5 | 6.39 | 6.26 | 1.71 x 102 | 1.98 x 102 | -3.99 |
| 115 | 2,3,4,4',6-Pentachloro | 74472-38-1 | NA | NA | 24.9 | 6.25 | 6.13 | 9.96 x 101 | 4.37 x 102 | -4.36 |
| 116 | 2,3,4,5,6-Pentachloro | 18259-05-7 | NA | 0.0145 | 30.0 | 6.11 | 5.99 | 8.26 x 101 | 6.84 x 102 | -4.58 |
| 117 | 2,3,4',5,6-Pentachloro | 68194-11-6 | NA | NA | 24.4 | 6.23 | 6.11 | 1.02 x 102 | 4.42 x 102 | -4.37 |
| 118 | 2,3'4,4',5-Pentachloro | 31508-00-6 | NA | NA | 12.7 | 6.46 | 6.33 | 1.95 x 102 | 1.52 x 102 | -3.86 |
| 119 | 2,3',4,4',6-Pentachloro | 56558-17-9 | NA | NA | 31.5 | 6.33 | 6.20 | 7.87 x 101 | 4.83 x 102 | -4.38 |
| 120 | 2,3',4,5,5'-Pentachloro | 68194-12-7 | NA | NA | 24.9 | 6.51 | 6.38 | 9.96 x 101 | 2.71 x 102 | -4.10 |
| 121 | 2,3',4,5',6-Pentachloro | 56558-18-0 | NA | NA | 56.6 | 6.38 | 6.25 | 4.38 x 101 | 7.90 x 102 | -4.59 |
| 122 | 2',3,3',4,5-Pentachloro | 76842-07-4 | NA | NA | 12.7 | 6.38 | 6.25 | 1.95 x 102 | 1.77 x 102 | -3.94 |
| 123 | 2',3,4,4',5-Pentachloro | 65510-44-3 | NA | NA | 17.6 | 6.46 | 6.33 | 1.41 x 102 | 2.10 x 102 | -4.00 |
| 124 | 2',3,4,5,5'-Pentachloro | 70424-70-3 | NA | NA | 17.3 | 6.46 | 6.33 | 1.43 x 102 | 2.08 x 102 | -3.99 |
| 125 | 2',3,4,5,6'-Pentachloro | 74472-39-2 | NA | NA | 29.2 | 6.27 | 6.15 | 8.49 x 101 | 4.93 x 102 | -4.41 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 126 | 3,3'4,4',5-Pentachloro | 57465-28-8 | NA | NA | 8.29 | 6.60 | 6.47 | 2.99 x 102 | 7.62 x 101 | -3.53 |
| 127 | 3,3',4,5,5'-Pentachloro | 39635-33-1 | NA | NA | 15.8 | 6.65 | 6.52 | 1.57 x 102 | 1.32 x 102 | -3.76 |
| 128 | 2,2'3,3'4,4'-Hexachloro | 38380-07-3 | 1.98 x 10-5 | 0.00166 | 10.5 | 6.63 | 6.50 | 2.36 x 102 | 2.10 x 102 | -3.61 |
| 129 | 2,2',3,3',4,5-Hexachloro | 55215-18-4 | NA | 0.00166 | 14.2 | 6.63 | 6.50 | 1.75 x 102 | 2.83 x 102 | -3.74 |
| 130 | 2,2'3,3',4,5'-Hexachloro | 52663-66-8 | NA | NA | 15.4 | 6.69 | 6.55 | 1.61 x 102 | 2.81 x 102 | -3.71 |
| 131 | 2,2',3,3',4,6-Hexachloro | 61798-70-7 | NA | NA | 24.5 | 6.49 | 6.36 | 1.01 x 102 | 6.27 x 102 | -4.12 |
| 132 | 2,2',3,3',4,6'-Hexachloro | 38380-05-1 | NA | NA | 20.5 | 6.49 | 6.36 | 1.21 x 102 | 5.23 x 102 | -4.04 |
| 133 | 2,2'3,3'5,5'-Hexachloro | 35694-04-3 | NA | NA | 20.6 | 6.74 | 6.60 | 1.20 x 102 | 3.44 x 102 | -3.79 |
| 134 | 2,2',3,3',5,6-Hexachloro | 52704-70-8 | NA | 0.00111 | 23.3 | 6.47 | 6.34 | 1.06 x 102 | 6.17 x 102 | -4.11 |
| 135 | 2,2',3,3',5,6'-Hexachloro | 52744-13-5 | NA | NA | 27.2 | 6.55 | 6.42 | 9.11 x 101 | 6.24 x 102 | -4.10 |
| 136 | 2,2',3,3',6,6'-Hexachloro | 38411-22-2 | NA | 0.00222 | 32.6 | 6.19 | 6.07 | 7.60 x 101 | 1.39 x 103 | -4.54 |
| 137 | 2,2',3,4,4',5-Hexachloro | 35694-06-5 | NA | NA | 18.8 | 6.71 | 6.57 | 1.32 x 102 | 3.31 x 102 | -3.78 |
| 138 | 2,2',3,4,4',5'-Hexachloro | 35065-28-2 | NA | NA | 13.2 | 6.71 | 6.57 | 1.88 x 102 | 2.32 x 102 | -3.63 |
| 139 | 2,2',3,4,4',6-Hexachloro | 56030-56-9 | NA | NA | 33.3 | 6.58 | 6.45 | 7.44 x 101 | 7.26 x 102 | -4.16 |
| 140 | 2,2',3,4,4',6'-Hexachloro | 59291-64-4 | NA | NA | 31.2 | 6.58 | 6.45 | 7.95 x 101 | 6.80 x 102 | -4.13 |
| 141 | 2,2',3,4,5,5'-Hexachloro | 52712-04-6 | NA | NA | 17.6 | 6.71 | 6.57 | 1.41 x 102 | 3.10 x 102 | -3.75 |
| 142 | 2,2',3,4,5,6-Hexachloro | 41411-61-4 | NA | NA | 31.9 | 6.44 | 6.31 | 7.77 x 101 | 8.88 x 102 | -4.28 |
| 143 | 2,2',3,4,5,6'-Hexachloro | 68194-15-0 | NA | NA | 29.8 | 6.52 | 6.39 | 8.32 x 101 | 7.20 x 102 | -4.17 |
| 144 | 2,2',3,4,5',6-Hexachloro | 68194-14-9 | NA | NA | 30.0 | 6.57 | 6.44 | 8.26 x 101 | 6.65 x 102 | -4.12 |
| 145 | 2,2',3,4,6,6'-Hexachloro | 74472-40-5 | NA | NA | 47.6 | 6.22 | 6.10 | 5.21 x 101 | 1.92 x 103 | -4.67 |
| 146 | 2,2',3,4',5,5'-Hexachloro | 51908-16-8 | NA | NA | 19.0 | 6.76 | 6.62 | 1.31 x 102 | 3.05 x 102 | -3.73 |
| 147 | 2,2',3,4',5,6-Hexachloro | 68194-13-8 | NA | NA | 32.0 | 6.55 | 6.42 | 7.75 x 101 | 7.33 x 102 | -4.17 |
| 148 | 2,2',3,4',5,6'-Hexachloro | 74472-42-7 | NA | NA | 43.5 | 6.63 | 6.50 | 5.70 x 101 | 8.69 x 102 | -4.22 |
| 149 | 2,2',3,4',5',6-Hexachloro | 38380-04-0 | NA | NA | 24.0 | 6.57 | 6.44 | 1.03 x 102 | 5.33 x 102 | -4.03 |
| 150 | 2,2',3,4',6,6'-Hexachloro | 68194-08-1 | NA | NA | 51.3 | 6.27 | 6.15 | 4.83 x 101 | 1.90 x 103 | -4.66 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 151 | 2,2',3,5,5',6-Hexachloro | 52663-63-5 | NA | NA | 28.7 | 6.55 | 6.42 | 8.64 x 101 | 6.58 x 102 | -4.12 |
| 152 | 2,2',3,5,6,6'-Hexachloro | 68194-09-2 | NA | NA | 43.3 | 6.19 | 6.07 | 5.73 x 101 | 1.84 x 103 | -4.66 |
| 153 | 2,2',4,4',5,5'-Hexachloro | 35065-27-1 | 1.19 x 10-4 | 0.00277 | 16.7 | 6.79 | 6.65 | 1.48 x 102 | 2.56 x 102 | -3.65 |
| 154 | 2,2',4,4',5,6'-Hexachloro | 60145-22-4 | NA | NA | 38.7 | 6.65 | 6.52 | 6.41 x 101 | 7.43 x 102 | -4.15 |
| 155 | 2,2',4,4',6,6'-Hexachloro | 33979-03-2 | 4.8 x 10-4 | 0.0055 | 85.2 | 6.35 | 6.22 | 2.91 x 101 | 2.79 x 103 | -4.80 |
| 156 | 2,3,3',4,4',5-Hexachloro | 38380-08-4 | NA | NA | 8.97 | 6.84 | 6.70 | 2.76 x 102 | 1.26 x 102 | -3.33 |
| 157 | 2,3,3',4,4',5'-Hexachloro | 69782-90-7 | NA | NA | 8.56 | 6.84 | 6.70 | 2.90 x 102 | 1.20 x 102 | -3.31 |
| 158 | 2,3,3',4,4',6-Hexachloro | 74472-42-7 | NA | NA | 16.7 | 6.72 | 6.58 | 1.48 x 102 | 2.90 x 102 | -3.72 |
| 159 | 2,3,3',4,5,5'-Hexachloro | 39635-35-3 | NA | NA | 15.8 | 6.89 | 6.75 | 1.57 x 102 | 2.02 x 102 | -3.52 |
| 160 | 2,3,3',4,5,6-Hexachloro | 41411-62-5 | NA | NA | 21.7 | 6.63 | 6.50 | 1.14 x 102 | 4.34 x 102 | -3.92 |
| 161 | 2,3,3',4,5',6-Hexachloro | 74472-43-8 | NA | NA | 28.9 | 6.76 | 6.62 | 8.58 x 101 | 4.66 x 102 | -3.92 |
| 162 | 2,3,3',4',5,5'-Hexachloro | 39635-34-2 | NA | NA | 13.3 | 6.89 | 6.75 | 1.86 x 102 | 1.71 x 102 | -3.45 |
| 163 | 2,3,3',4',5,6-Hexachloro | 74472-44-9 | NA | NA | 16.8 | 6.68 | 6.54 | 1.48 x 102 | 3.11 x 102 | -3.76 |
| 164 | 2,3,3',4',5',6-Hexachloro | 74472-45-0 | NA | NA | 17.9 | 6.70 | 6.56 | 1.38 x 102 | 3.22 x 102 | -3.77 |
| 165 | 2,3,3',5,5',6-Hexachloro | 74472-46-1 | NA | NA | 27.9 | 6.73 | 6.59 | 8.89 x 101 | 4.75 x 102 | -3.93 |
| 166 | 2,3,4,4',5,6-Hexachloro | 41411-63-6 | NA | NA | 18.7 | 6.63 | 6.50 | 1.33 x 102 | 3.72 x 102 | -3.86 |
| 167 | 2,3',4,4',5,5'-Hexachloro | 52663-72-6 | NA | NA | 11.1 | 6.92 | 6.78 | 2.23 x 102 | 1.35 x 102 | -3.34 |
| 168 | 2,3',4,4',5',6-Hexachloro | 59291-65-5 | NA | NA | 28.0 | 6.79 | 6.65 | 8.85 x 101 | 4.28 x 102 | -3.87 |
| 169 | 3,3',4,4',5,5'-Hexachloro | 32774-16-6 | NA | NA | 6.60 | 7.06 | 6.91 | 3.76 x 102 | 6.36 x 101 | -2.97 |
| 170 | 2,2',3,3',4,4',5-Heptachloro | 35065-30-6 | NA | NA | 8.85 | 7.09 | 6.94 | 2.80 x 102 | 1.76 x 102 | -3.07 |
| 171 | 2,2',3,3',4,4',6-Heptachloro | 52663-71-5 | 2.73 x 10-5 | 0.00506 | 17.5 | 6.96 | 6.82 | 1.42 x 102 | 4.24 x 102 | -3.50 |
| 172 | 2,2',3,3',4,5,5'-Heptachloro | 52663-74-8 | NA | NA | 12.1 | 7.14 | 6.99 | 2.05 x 102 | 2.21 x 102 | -3.16 |
| 173 | 2,2',3,3',4,5,6-Heptachloro | 68194-16-1 | NA | NA | 18.5 | 6.88 | 6.74 | 1.34 x 102 | 5.11 x 102 | -3.60 |
| 174 | 2,2',3,3',4,5,6'-Heptachloro | 38411-25-5 | NA | NA | 17.1 | 6.95 | 6.81 | 1.45 x 102 | 4.21 x 102 | -3.50 |
| 175 | 2,2',3,3',4,5',6-Heptachloro | 40186-70-7 | NA | NA | 22.6 | 7.01 | 6.87 | 1.10 x 102 | 5.02 x 102 | -3.56 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 176 | 2,2',3,3',4,6,6'-Heptachloro | 52663-65-7 | NA | NA | 30.1 | 6.65 | 6.52 | 8.24 x 101 | 1.20 x 103 | -4.04 |
| 177 | 2,2',3,3',4',5,6-Heptachloro | 52663-70-4 | NA | NA | 16.5 | 6.93 | 6.79 | 1.50 x 102 | 4.22 x 102 | -3.50 |
| 178 | 2,2',3,3',5,5',6-Heptachloro | 52663-67-9 | NA | NA | 21.6 | 6.98 | 6.84 | 1.15 x 102 | 5.06 x 102 | -3.57 |
| 179 | 2,2',3,3',5,6,6'-Heptachloro | 52663-64-6 | NA | NA | 27.9 | 6.63 | 6.50 | 8.89 x 101 | 1.15 x 103 | -4.03 |
| 180 | 2,2',3,4,4',5,5'-Heptachloro | 35065-29-3 | NA | NA | 10.9 | 7.17 | 7.02 | 2.27 x 102 | 1.90 x 102 | -3.08 |
| 181 | 2,2',3,4,4',5,6-Heptachloro | 74472-47-2 | NA | NA | 23.3 | 6.96 | 6.82 | 1.06 x 102 | 5.68 x 102 | -3.62 |
| 182 | 2,2',3,4,4',5,6'-Heptachloro | 60145-23-5 | NA | NA | 26.0 | 7.03 | 6.89 | 9.53 x 101 | 5.61 x 102 | -3.60 |
| 183 | 2,2',3,4,4',5',6-Heptachloro | 52663-69-1 | NA | NA | 20.4 | 7.03 | 6.89 | 1.22 x 102 | 4.38 x 102 | -3.49 |
| 184 | 2,2',3,4,4',6,6'-Heptachloro | 74472-48-3 | NA | NA | 46.4 | 6.73 | 6.59 | 5.34 x 101 | 1.64 x 103 | -4.15 |
| 185 | 2,2',3,4,5,5',6- Heptachloro | 52712-05-7 | NA | 0.00114 | 21.7 | 6.95 | 6.81 | 1.14 x 102 | 5.35 x 102 | -3.60 |
| 186 | 2,2',3,4,5,6,6'-Heptachloro | 74472-49-4 | NA | NA | 37.3 | 6.60 | 6.47 | 6.65 x 101 | 1.61 x 103 | -4.19 |
| 187 | 2,2',3,4',5,5',6-Heptachloro | 52663-68-0 | NA | NA | 20.5 | 7.01 | 6.87 | 1.21 x 102 | 4.57 x 102 | -3.52 |
| 188 | 2,2',3,4',5,6,6'-Heptachloro | 74487-85-7 | NA | NA | 44.9 | 6.71 | 6.57 | 5.52 x 101 | 1.65 x 103 | -4.16 |
| 189 | 2,3,3',4,4',5,5'-Heptachloro | 39635-31-9 | NA | NA | 6.74 | 7.30 | 7.15 | 3.68 x 102 | 9.47 x 101 | -2.74 |
| 190 | 2,3,3',4,4',5,6-Heptachloro | 41411-64-7 | NA | NA | 11.4 | 7.09 | 6.94 | 2.17 x 102 | 2.27 x 102 | -3.18 |
| 191 | 2,3,3',4,4',5',6-Heptachloro | 74472-50-7 | NA | NA | 13.5 | 7.17 | 7.02 | 1.84 x 102 | 2.34 x 102 | -3.18 |
| 192 | 2,3,3',4,5,5',6-Heptachloro | 74472-51-8 | NA | NA | 19.4 | 7.14 | 6.99 | 1.28 x 102 | 3.54 x 102 | -3.36 |
| 193 | 2,3,3',4',5,5',6-Heptachloro | 69782-91-8 | NA | NA | 13.6 | 7.14 | 6.99 | 1.82 x 102 | 2.49 x 102 | -3.21 |
| 194 | 2,2',3,3',4,4',5,5'-Octachloro | 35694-08-7 | NA | 4.7 x 10-4 | 6.79 | 7.55 | 7.39 | 3.65 x 102 | 1.32 x 102 | -2.51 |
| 195 | 2,2',3,3',4,4',5,6-Octachloro | 52663-78-2 | NA | NA | 12.0 | 7.34 | 7.19 | 2.07 x 102 | 3.20 x 102 | -2.95 |
| 196 | 2,2',3,3',4,4',5,6'-Octachloro | 42740-50-1 | NA | NA | 13.2 | 7.41 | 7.25 | 1.88 x 102 | 3.20 x 102 | -2.93 |
| 197 | 2,2',3,3',4,4',6,6'-Octachloro | 33091-17-7 | NA | NA | 25.7 | 7.11 | 6.96 | 9.65 x 101 | 9.78 x 102 | -3.52 |
| 198 | 2,2',3,3',4,5,5',6-Octachloro | 68194-17-2 | NA | NA | 15.6 | 7.39 | 7.24 | 1.59 x 102 | 3.83 x 102 | -3.02 |
| 199 | 2,2',3,3',4,5,5',6'-Octachloro | 52663-75-9 | NA | NA | 23.0 | 7.03 | 6.89 | 1.08 x 102 | 9.76 x 102 | -3.55 |
| 200 | 2,2',3,3',4,5,6,6'-Octachloro | 52663-73-7 | NA | NA | 24.4 | 7.09 | 6.94 | 1.02 x 102 | 9.52 x 102 | -3.51 |

Table D.2.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, Ha (Pa m3 mol-1) | log Kowb | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 201 | 2,2',3,3',4,5',6,6'-Octachloro | 40186-71-8 | NA | NA | 13.2 | 7.39 | 7.24 | 1.88 x 102 | 3.24 x 102 | -2.95 |
| 202 | 2,2',3,3',5,5',6,6'-Octachloro | 2136-99-4 | 2.66 x 10-5 | 7 x 10-4 | 22.6 | 7.06 | 6.91 | 1.10 x 102 | 9.29 x 102 | -3.51 |
| 203 | 2,2',3,4,4',5,5',6-Octachloro | 52663-76-0 | NA | NA | 14.2 | 7.41 | 7.25 | 1.75 x 102 | 3.44 x 102 | -2.96 |
| 204 | 2,2',3,4,4',5,6,6'-Octachloro | 74472-52-9 | NA | NA | 34.9 | 7.11 | 6.96 | 7.10 x 101 | 1.33 x 103 | -3.65 |
| 205 | 2,3,3',4,4',5,5',6-Octachloro | 74472-53-0 | NA | NA | 8.85 | 7.55 | 7.39 | 2.80 x 102 | 1.73 x 102 | -2.61 |
| 206 | 2,2',3,3',4,4',5,5',6-Nonachloro | 40186-72-9 | 1.96 x 10-7 | 2.37 x 10-4 | 8.85 | 7.80 | 7.63 | 2.80 x 102 | 2.32 x 102 | -2.41 |
| 207 | 2,2',3,3',4,4',5,6,6'-Nonachloro | 52663-79-3 | NA | NA | 17.1 | 7.49 | 7.33 | 1.45 x 102 | 6.95 x 102 | -2.96 |
| 208 | 2,2',3,3',4,5,5',6,6'-Nonachloro | 52663-77-1 | NA | 3.8 x 10-5 | 16.9 | 7.47 | 7.31 | 1.47 x 102 | 7.06 x 102 | -2.97 |
| 209 | 2,2',3,3',4,4',5,5',6,6'-Decachloro | 2051-24-3 | 5.02 x 10-8 | 2 x 10-6 | 11.4 | 7.88 | 7.71 | 2.17 x 102 | 4.85 x 102 | -2.41 |

aDunnivant et al. (1992).

bHansen et al. (1999).

cNA, not available

Table D.3.  Additional physicochemical properties of polychlorinated byphenyls (PCBs) at 298.2 K – Aroclor mixtures.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Aroclor 1016 | 12674-11-2 | 0.06-0.2 | 0.856-0.216 | 70-930 | 4.4-5.8 | 4.3-5.7 | 2.66 - 35.4 | 6.1 x 103 -9.4 x 103 | (-6.4) - (-6.7) |
| 2 | Aroclor 1221 | 111-042-82 | 0.89-2.0 | 3.07-26.0 | 77-290 | 4.1-4.7 | 4.0-4.6 | 8.54 - 32 | 3.0 x 103 -3.8 x 103 | -7.0 |
| 3 | Aroclor 1232 | 111-411-65 | 0.54 | 6.56-2.0 | 82-270 | 4.5-5.2 | 4.4-5.1 | 9.18 - 30.2 | 2.4 x 103 -3.7 x 103 | (-6.4) - (-6.6) |
| 4 | Aroclor 1242 | 534-692-19 | 0.05-0.13 | 0.383-2.87 | 45-130 | 4.5-5.8 | 4.4-5.7 | 19.1 - 55.1 | 9.4 x 102 -5.3 x 103 | (-5.5) - (-6.4) |
| 5 | Aroclor 1248 | 126-722-96 | 0.0085-0.11 | 0.347-1.74 | 24-63 | 5.8-6.3 | 5.7-6.2 | 39.3 - 103 | 3.7 x 102 -3.8 x 102 | (-4.7) - (-4.8) |
| 6 | Aroclor 1254 | 110-976-91 | 0.008-0.02 | 0.306-0.92 | 22-26 | 6.1-6.8 | 6.0-6.7 | 95.3 - 113 | 1.6 x 102 -4.9 x 102 | (-3.8) - (-4.5) |
| 7 | Aroclor 1260 | 110-968-25 | 0.0002-0.012 | 0.00806 -0.215 | 25-56 | 6.3-7.5 | 6.2-7.3 | 44.3 - 99.1 | 3.3 x 102 -1.1 x 103 | (-3.5) - (-4.3) |

Table D.4.  Addditional physicochemical properties of dioxins at 298.2 K.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dibenzo-p-dioxin | 262-23-4 | 0.055 | 4.70 | 11.7 | 4.30 | 4.23 | 2.12 x 102 | 7.81 x 101 | -5.98 |
| 2 | 1-Chlorodibenzo-p-dioxin | 39227-53-7 | 0.012 | 1.91 | 6.29 | 4.75 | 4.67 | 3.94 x 102 | 4.75 x 101 | -5.26 |
| 3 | 2-Chlorodibenzo-p-dioxin | 39227-54-8 | 0.017 | 1.35 | 12.6 | 5.00 | 4.91 | 1.97 x 102 | 4.93 x 101 | -5.32 |
| 4 | 2,3-Dichlorodibenzo-p-dioxin | 29446-15-9 | 3.9 x 10-4 | 0.059 | 6.62 | 5.60 | 5.49 | 3.74 x 102 | 2.14 x 101 | -4.44 |
| 5 | 2,7-Dichlorodibenzo-p-dioxin | 33857-26-0 | 1.2 x 10-4 | 0.0148 | 8.10 | 5.75 | 5.64 | 3.06 x 102 | 1.82 x 101 | -4.37 |
| 6 | 2,8-Dichlorodibenzo-p-dioxin | 38964-22-6 | 1.4 x 10-4 | 0.0660 | 2.12 | 5.60 | 5.49 | 1.17 x 103 | 6.84 | -3.94 |
| 7 | 1,2,4-Trichlorodibenzo-p-dioxin | 39227-58-2 | 1.0 x 10-4 | 0.0293 | 3.42 | 6.35 | 6.22 | 7.25 x 102 | 6.73 | -3.40 |
| 8 | 1,2,3,4-Tetrachlorodibenzo-p-dioxin | 30756-58-8 | 6.40 x 10-6 | 1.7 x 10-3 | 3.75 | 6.60 | 6.47 | 6.61 x 102 | 1.30 x 101 | -3.19 |
| 9 | 1,2,3,7-Tetrachlorodibenzo-p-dioxin | 67028-18-6 | 1.00 x 10-6 | 1.3 x 10-3 | 0.767 | 6.90 | 6.76 | 3.23 x 103 | 1.47 | -2.21 |
| 10 | 1,3,6,8-Tetrachlorodibenzo-p-dioxin | 30746-58-8 | 7.00 x 10-7 | 9.93 x 10-4 | 0.704 | 7.10 | 6.95 | 3.52 x 103 | 9.13 x 10-1 | -2.01 |
| 11 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin | 1746-01-6 | 2.00 x 10-7 | 6 x 10-5 | 3.34 | 6.80 | 6.66 | 7.42 x 102 | 7.84 | -2.94 |
| 12 | 1,2,3,4,7-Pentachlorodibenzo-p-dioxin | 39227-61-7 | 8.80 x 10-8 | 3.31 x 10-4 | 0.266 | 7.40 | 7.25 | 9.32 x 103 | 5.47 x 10-1 | -1.21 |
| 13 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin | 39227-26-8 | 5.10 x 10-9 | 1.13 x 10-5 | 0.451 | 7.80 | 7.63 | 5.50 x 103 | 1.20 | -1.11 |
| 14 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin | 35822-46-9 | 7.50 x 10-10 | 5.64 x 10-6 | 0.133 | 8.00 | 7.83 | 1.86 x 104 | 5.97 x 10-1 | -0.31 |
| 15 | Octachlorodibenzo-p-dioxin | 3268-87-9 | 1.10 x 10-10 | 1.61 x 10-7 | 0.684 | 8.20 | 8.02 | 3.62 x 103 | 4.94 | -0.81 |

Table D.5.  Additional physicochemical properties of dibenzofurans at 298.2 K.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dibenzofuran | 132-64-9 | 0.30 | 28.2 | 10.6 | 4.31 | 4.24 | 2.34 x 102 | 4.03 x 101 | -5.93 |
| 2 | 2,8-Dichlorodibenzofuran | 5409-83-6 | 3.9 x 10-4 | 0.0612 | 6.38 | 5.44 | 5.34 | 3.89 x 102 | 4.60 | -4.58 |
| 3 | 2,3,7,8-Tetrachlorodibenzofuran | 51207-31-9 | 2.00 x 10-6 | 1.37 x 10-3 | 1.46 | 6.1 | 5.98 | 1.70 x 103 | 9.67 | -3.28 |
| 4 | 2,3,4,7,8-Pentachlorodibenzofuran | 51207-31-4 | 3.50 x 10-7 | 6.93 x 10-4 | 0.505 | 6.5 | 6.37 | 4.91 x 103 | 3.99 | -2.41 |
| 5 | 1,2,3,4,7,8-Hexachlorodibenzofuran | 70658-26-9 | 3.20 x 10-8 | 2.20 x 10-5 | 1.45 | 7.0 | 6.86 | 1.71 x 103 | 1.11 x 101 | -2.41 |
| 6 | 1,2,3,6,7,8-Hexachlorodibenzofuran | 57117-44-9 | 3.50 x 10-8 | 4.72 x 10-5 | 0.741 | NAa | NA | 3.35 x 103 | NA | NA |
| 7 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran | 67462-39-4 | 4.70 x 10-9 | 3.30 x 10-6 | 1.43 | 7.4 | 7.25 | 1.73 x 103 | 1.27 x 101 | -2.01 |
| 8 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran | 55673-89-7 | 6.20 x 10-9 | NA | NA | NA | NA | NA | NA | NA |
| 9 | Octachlorodibenzofuran | 39001-02-0 | 5.0 x 10-10 | 2.61 x 10-6 | 0.191 | 8.0 | 7.83 | 1.30 x 104 | 1.42 | -0.51 |

aNA, not available

Table D.6.  Additional physicochemical properties of polycyclic organic matter (POM) at 298.2 K.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Indan | 496-11-7 | 197 | 846 | 233 | 3.33 | 3.29 | 1.06 x 101 | 1.62 x 103 | -8.25 |
| 2 | Naphthalene | 91-20-3 | 10.4 | 242 | 43.0 | 3.37 | 3.32 | 5.78 x 101 | 3.71 x 102 | -7.48 |
| 3 | 1-Methylnaphthalene | 90-12-0 | 8.84 | 197 | 44.9 | 3.87 | 3.81 | 5.52 x 101 | 4.00 x 102 | -7.00 |
| 4 | 2-Methylnaphthalene | 91-57-6 | 9 | 176 | 51.2 | 3.86 | 3.80 | 4.84 x 101 | 4.71 x 102 | -7.07 |
| 5 | 1,2-Dimethylnaphthalene | 573-98-8 | 0.87 | NAa | NA | 4.31 | 4.24 | NA | NA | NA |
| 6 | 1,3-Dimethylnaphthalene | 575-41-7 | NA | 51.2 | NA | 4.42 | 4.35 | NA | NA | NA |
| 7 | 1,4-Dimethylnaphthalene | 571-58-4 | 2.27 | 73.0 | 31.1 | 4.37 | 4.30 | 7.97 x 101 | 2.83 x 102 | -6.34 |
| 8 | 1,5-Dimethylnaphthalene | 571-61-9 | NA | 19.8 | NA | 4.38 | 4.31 | NA | NA | NA |
| 9 | 2,3-Dimethylnaphthalene | 581-40-8 | 1 | 16.0 | 62.5 | 4.40 | 4.33 | 3.97 x 101 | 5.23 x 102 | -6.61 |
| 10 | 2,6-Dimethylnaphthalene | 581-40-2 | 1.4 | 10.9 | 129 | 4.31 | 4.24 | 1.92 x 101 | 1.40 x 103 | -7.02 |
| 11 | 1-Ethylnaphthalene | 1127-76-0 | 2.51 | 64.7 | 38.8 | 4.40 | 4.33 | 6.39 x 101 | 3.25 x 102 | -6.40 |
| 12 | 2-Ethylnaphthalene | 939-27-5 | 4 | 51.2 | 78.1 | NA | NA | 3.17 x 101 | NA | NA |
| 13 | 1,4,5-Trimethylnaphthalene | 2131-41-1 | 0.681 | 11.9 | 57.1 | 5.00 | 4.91 | 4.34 x 101 | 3.90 x 102 | -5.97 |
| 14 | Biphenyl | 92-52-4 | 1.3 | 45.4 | 28.6 | 3.90 | 3.84 | 8.67 x 101 | 6.23 x 102 | -6.77 |
| 15 | 4-Methylbiphenyl | 644-08-6 | NA | 24.1 | NA | 4.63 | 4.55 | NA | NA | NA |
| 16 | 4,4'-Dimethylbiphenyl | 613-33-2 | NA | 0.960 | NA | 5.09 | 5.00 | NA | NA | NA |
| 17 | Diphenylmethane | 101-81-5 | 0.0885 | 95.1 | 0.931 | 4.14 | 4.07 | 2.66 x 103 | 3.76 x 101 | -5.05 |
| 18 | Bibenzyl | 103-29-7 | 0.406 | 24.0 | 16.9 | 4.70 | 4.62 | 1.47 x 102 | 5.19 x 102 | -5.74 |
| 19 | trans-Stilbene | 103-30-0 | 0.065 | 1.61 | 40.4 | 4.81 | 4.73 | 6.14 x 101 | 6.54 x 102 | -6.01 |
| 20 | Acenaphthene | 83-32-9 | 0.3 | 24.6 | 12.2 | 3.92 | 3.86 | 2.03 x 102 | 1.17 x 102 | -6.38 |
| 21 | Acenaphthylene | 208-96-8 | 0.9 | 107 | 8.40 | 4.00 | 3.94 | 2.95 x 102 | 3.59 x 101 | -6.14 |
| 22 | Fluorene | 86-73-7 | 0.09 | 11.4 | 7.87 | 4.18 | 4.11 | 3.15 x 102 | 9.56 x 101 | -5.93 |
| 23 | 1-Methylfluorene | 1730-37-6 | NA | 6.05 | NA | 4.97 | 4.88 | NA | NA | NA |
| 24 | Phenanthrene | 85-01-8 | 0.02 | 6.17 | 3.24 | 4.57 | 4.49 | 7.65 x 102 | 2.33 x 101 | -5.16 |
| 25 | 1-Methylphenanthrene | 832-69-6 | NA | 1.40 | NA | 5.14 | 5.05 | NA | NA | NA |
| 26 | Anthracene | 120-12-7 | 1 x 10-3 | 0.253 | 3.96 | 4.54 | 4.46 | 6.26 x 102 | 3.10 x 101 | -5.27 |
| 27 | 9-Methylanthracene | 779-02-2 | 2.4 x 10-3 | 1.36 | 1.77 | 5.07 | 4.98 | 1.40 x 103 | 1.33 x 101 | -4.39 |

Table D.6.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mmol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 2-Methylanthracene | 613-12-7 | NA | 0.156 | NA | 5.15 | 5.06 | NA | NA | NA |
| 29 | 9,10-Dimethylanthracene | 781-43-1 | 1.53 x 10-4 | 0.271 | 5.65 x 10-1 | 5.25 | 5.15 | 4.39 x 103 | 9.08 | -3.72 |
| 30 | Pyrene | 129-00-0 | 6 x 10-4 | 0.652 | 0.92 | 5.18 | 5.09 | 2.69 x 103 | 3.88 | -4.00 |
| 31 | Fluoranthene | 206-44-0 | 1.23 x 10-3 | 1.19 | 1.04 | 5.22 | 5.12 | 2.38 x 103 | 5.01 | -4.01 |
| 32 | Benzo[a]fluorene | 238-84-6 | NA | 0.208 | NA | 5.40 | 5.30 | NA | NA | NA |
| 33 | Benzo[b]fluorene | 243-17-4 | NA | 9.25 x 10-3 | NA | 5.75 | 5.64 | NA | NA | NA |
| 34 | Chrysene | 218-01-9 | 5.70 x 10-7 | 8.76 x 10-3 | 0.065 | 5.86 | 5.75 | 3.81 x 104 | 4.43 x 10-1 | -2.21 |
| 35 | Triphenylene | 217-59-4 | 2.30 x 10-6 | 0.188 | 0.012 | 5.49 | 5.39 | 2.07 x 105 | 1.79 x 10-1 | -1.81 |
| 36 | p-Terphenyl | 92-94-4 | 4.86 x 10-6 | 0.0782 | 6.21 x 10-2 | 6.03 | 5.91 | 3.99 x 104 | 8.61 x 10-1 | -2.01 |
| 37 | Naphthacene | 92-24-0 | 9.30 x 10-9 | 2.63 x 10-3 | 4 x 10-3 | 5.76 | 5.65 | 6.20 x 105 | 3.39 x 10-2 | -1.01 |
| 38 | Benz[a]anthracene | 56-55-3 | 2.80 x 10-5 | 0.0482 | 0.581 | 5.91 | 5.80 | 4.27 x 103 | 3.55 | -3.07 |
| 39 | Benzo[b]fluoranthene | 205-99-2 | 4.40 x 10-5 | 5.95 x 10-3 | 7.39 | 5.80 | 5.69 | 3.35 x 102 | 1.33 x 102 | -4.28 |
| 40 | Benzo[j]fluoranthene | 205-82-2 | NA | 9.9 x 10-3 | NA | 6.42 | 6.29 | NA | NA | NA |
| 41 | Benzo[k]fluoranthene | 207-08-9 | 5.20 x 10-8 | 3.17 x 10-3 | 0.016 | 6.00 | 5.88 | 1.55 x 105 | 1.95 x 10-1 | -1.41 |
| 42 | Benzo[a]pyrene | 50-32-8 | 7.00 x 10-7 | 0.0151 | 0.046 | 6.04 | 5.92 | 5.39 x 104 | 4.41 x 10-1 | -1.81 |
| 43 | Benzo[e]pyrene | 192-97-2 | 7.40 x 10-7 | 0.0159 | 0.020 | 6.44 | 6.31 | 1.24 x 105 | 8.55 x 10-2 | -1.11 |
| 44 | Perylene | 198-55-0 | 1.40 x 10-8 | 1.59 x 10-3 | 0.003 | 6.25 | 6.13 | 8.26 x 105 | 1.86 x 10-2 | -0.41 |
| 45 | 7,12-Dimethylbenz[a] anthracene | 57-97-6 | 3.84 x 10-7 | 0.195 | 1.97 x 10-3 | 6.00 | 5.88 | 1.26 x 106 | 6.96 x 10-2 | -0.51 |
| 46 | 9,10-Dimethylbenz[a] anthracene | 56-56-4 | 3.73 x 10-7 | 0.170 | 2.19 x 10-3 | 6.00 | 5.88 | 1.13 x 106 | 7.76 x 10-2 | -0.51 |
| 47 | 3-Methylcholanthrene | 56-49-5 | 1.03 x 10-6 | 7.08 x 10-3 | 0.145 | 6.42 | 6.29 | 1.71 x 104 | 2.73 | -1.91 |
| 48 | Benzo[ghi]perylene | 191-24-2 | 2.25 x 10-5b | 9.68 x 10-4 | 0.075 | 6.50 | 6.37 | 3.31 x 104 | 5.62 x 10-1 | -1.61 |
| 49 | Pentacene | 135-48-8 | 1.0 x 10-13 | NA | NA | 7.19 | 7.04 | NA | NA | NA |
| 50 | Dibenz[a,c]anthracene | 215-58-7 | 1.30 x 10-9 | 5.75 x 10-3 | 2.26 x 10-4 | 7.16 | 7.01 | 1.10 x 107 | 1.33 x 10-3 | 1.59 |
| 51 | Dibenz[a,h]anthracene | 53-70-3 | 3.70 x 10-10 | 2.16 x 10-3 | 1.71 x 10-4 | 6.75 | 6.61 | 1.45 x 107 | 2.09 x 10-3 | 1.29 |
| 52 | Dibenz[a,j]anthracene | 58-70-3 | NA | 0.0431 | NA | 7.15 | 7.00 | NA | NA | NA |
| 53 | Coronene | 191-07-1 | 2.0 x 10-10 | 4.66 x 10-4 | 4.29 x 10-4 | 6.75 | 6.61 | 5.78 x 106 | 3.82 x 10-3 | 0.89 |

aNA, not available

bVapor pressure of subcooled liquid.

Table D.7.  Additional physicochemical properties of coke oven emissions at 298.2 K – monoaromatic and phenolic compounds.

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ethylbenzene | 100-41-4 | 1270 | 1.43 | 887 | 3.13 | 2.96 | 2.79 | 1.66 x 104 | -9.03 |
| 2 | 1,2,3-Trimethylbenzene | 526-73-8 | 200 | 0.582 | 343 | 3.55 | 3.26 | 7.23 | 1.13 x 104 | -8.20 |
| 3 | 1,2,4-Trimethylbenzene | 95-63-6 | 270 | 0.474 | 569 | 3.60 | 3.37 | 4.36 | 1.30 x 104 | -8.37 |
| 4 | 1,3,5-Trimethylbenzene | 108-67-8 | 325 | 0.416 | 781 | 3.58 | 3.43 | 3.17 | 1.45 x 104 | -8.53 |
| 5 | n-Propylbenzene | 103-65-1 | 450 | 0.433 | 1040 | 3.69 | 3.43 | 2.38 | 1.94 x 104 | -8.54 |
| 6 | Isopropylbenzene | 98-82-8 | 610 | 0.416 | 1470 | 3.63 | 3.46 | 1.69 | 2.47 x 104 | -8.75 |
| 7 | 1-Ethyl-2-methylbenzene | 611-14-3 | 330 | 0.624 | 529 | 3.63 | 3.26 | 4.69 | 1.75 x 104 | -8.31 |
| 8 | 1-Ethyl-3-methylbenzene | 620-14-4 | 391 | NAa | NA | NA | NA | NA | NA | NA |
| 9 | 1-Ethyl-4-methylbenzene | 622-96-8 | 395 | 0.790 | 500 | 3.63 | 3.17 | 4.96 | 2.23 x 104 | -8.28 |
| 10 | 1-Isopropyl-4-methylbenzene | 99-87-6 | 204 | 0.253 | 805 | 4.10 | 3.63 | 3.08 | 3.32 x 104 | -8.02 |
| 11 | n-Butylbenzene | 104-51-8 | 137 | 0.103 | 1330 | 4.26 | 4.00 | 1.86 | 1.83 x 104 | -8.08 |
| 12 | Isobutylbenzene | 538-93-2 | 250 | 0.075 | 3320 | 4.01 | 4.16 | 7.47 x 10-1 | 2.84 x 104 | -8.73 |
| 13 | sec-Butylbenzene | 135-98-8 | 240 | 0.127 | 1890 | 3.99 | 3.93 | 1.31 | 3.21 x 104 | -8.51 |
| 14 | tert-Butylbenzene | 98-06-6 | 286 | 0.224 | 1280 | 4.11 | 3.70 | 1.94 | 4.26 x 104 | -8.21 |
| 15 | 1,2,3,4-Tetramethybenzene | 48-23-3 | 45 | NA | NA | 3.90 | 3.84 | NA | NA | NA |
| 16 | 1,2,3,5-Tetramethylbenzene | 527-53-7 | 62 | NA | NA | 4.04 | 3.98 | NA | NA | NA |
| 17 | 1,2,4,5-Tetramethylbenzene | 95-93-2 | 66 | 0.026 | 2540 | 4.10 | 4.03 | 9.76 x 10-1 | 3.19 x 104 | -8.52 |
| 18 | n-Pentylbenzene | 538-68-1 | 44 | 0.026 | 1690 | 4.90 | 4.81 | 1.47 | 9.69 x 103 | -7.54 |
| 19 | Pentamethylbenzene | 538-61-1 | 9.52 | 0.105 | 90.7 | NA | NA | 2.73 x 101 | NA | NA |
| 20 | n-Hexylbenzene | 1077-16-3 | 13.6 | 0.006 | 2170 | 5.52 | 5.42 | 1.14 | 9.98 x 103 | -7.03 |
| 21 | Hexamethylbenzene | 87-85-4 | 0.155 | 0.001 | 155 | 4.61 | 4.53 | 1.88 x 101 | 5.07 x 103 | -6.73 |
| 22 | 2,3-Dimethylphenol | 526-75-0 | 8.05b | 47.8 | 0.0540 | NA | NA | 4.59 x 104 | NA | NA |
| 23 | 2,4-Dimethylphenol | 105-67-9 | 13.0b | 70.0 | 0.182 | 2.35 | 2.33 | 1.36 x 104 | 6.28 x 101 | -6.13 |
| 24 | 2,5-Dimethylphenol | 105-67-9 | 10.7b | 25.3 | 0.135 | NA | NA | 1.84 x 104 | NA | NA |
| 25 | 2,6-Dimethylphenol | 576-26-1 | 32.8b | 49.6 | 0.383 | 2.36 | 2.34 | 6.47 x 103 | 1.28 x 102 | -6.44 |

Table D.7.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 3,4-Dimethylphenol | 95-65-8 | 2.24b | 40.6 | 0.0212 | 2.23 | 2.21 | 1.17 x 105 | 1.14 x 101 | -5.31 |
| 27 | 3,5-Dimethylphenol | 108-68-9 | 3.76b | 43.8 | 0.0322 | 2.35 | 2.33 | 7.70 x 104 | 1.11 x 101 | -5.37 |
| 28 | 2,3,5-Trimethylphenol | 697-82-5 | 2.43b | 6.37 | 0.0811 | NA | NA | 3.06 x 104 | NA | NA |
| 29 | 2,3,6-Trimethylphenol | 2416-94-6 | NA | NA | NA | NA | NA | NA | NA | NA |
| 30 | 2,4,5-Trimethylphenol | 496-78-6 | 7.73b | NA | NA | 2.67 | 2.64 | NA | NA | NA |
| 31 | 2,4,6-Trimethylphenol | 527-60-6 | 19.6b | 9.55 | 0.703 | 2.73 | 2.70 | 3.53 x 103 | 2.27 x 102 | -6.33 |
| 32 | 3,4,5-Trimethylphenol | 527-54-8 | 2.14b | 12.3 | 0.0264 | NA | NA | 9.39 x 104 | NA | NA |
| 33 | 3-Methyl-5-ethylphenol | NA | 1.18b | 18.4 | NA | NA | NA | NA | NA | NA |
| 34 | 2-Ethylphenol | 90-00-6 | 20.4b | NA | NA | 2.47 | 2.45 | NA | NA | NA |
| 35 | 3-Ethylphenol | 620-17-7 | 6.7b | NA | NA | 2.50 | 2.48 | NA | NA | NA |
| 36 | 4-Ethylphenol | 123-07-9 | 5b | NA | NA | 2.50 | 2.48 | NA | NA | NA |
| 37 | 2-Propylphenol | 644-35-0 | 11.2b | NA | NA | 2.93 | 2.90 | NA | NA | NA |
| 38 | 4-n-Propylphenol | 645-56-7 | 7.3b | NA | NA | 3.20 | 3.16 | NA | NA | NA |
| 39 | 2-Isopropylphenol | 88-69-7 | 12.5b | NA | NA | 2.88 | 2.85 | NA | NA | NA |
| 40 | 4-Isopropylphenol | 99-89-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 41 | 4-n-Butylphenol | 1638-22-8 | NA | 4.91 | NA | 3.65 | 3.60 | NA | NA | NA |
| 42 | 2-sec-Butylphenol | 89-72-5 | 5b | NA | NA | 2.80 | 2.77 | NA | NA | NA |
| 43 | 2-tert-Butylphenol | 88-18-6 | NA | NA | NA | 2.70 | 2.67 | NA | NA | NA |
| 44 | 3-tert-Butylphenol | 585-34-2 | NA | 16.5 | NA | 2.60 | 2.57 | NA | NA | NA |
| 45 | 4-sec-Butylphenol | 99-71-8 | 4b | 7.64 | 0.231 | 2.10 | 2.09 | 1.07 x 104 | 1.17 x 103 | -6.48 |
| 46 | 4-tert-Butylphenol | 98-54-4 | 1.24b | 4.62 | 0.0476 | 3.04 | 3.00 | 5.21 x 104 | 1.79 x 101 | -4.85 |
| 47 | 2-Methyl-5-tert-butylphenol | NA | 3.69b | 3.26 | NA | NA | NA | NA | NA | NA |
| 48 | 2,6-Di-sec-butylphenol | 128-39-2 | NA | NA | NA | 4.40 | 4.33 | NA | NA | NA |
| 49 | 3,5-Di-tert-butylphenol | 1138-52-9 | NA | NA | NA | NA | NA | NA | NA | NA |
| 50 | 2,6-Di-t-butyl-4-methylphenol | 128-37-0 | 0.0338b | NA | NA | 4.17 | 4.10 | NA | NA | NA |

Table D.7.  (Cont.).

| No. | Chemical | CAS No. | Vapor Pressure (Pa) | Aqueous Solubility (mol m-3) | Henry's Law constant, H (Pa m3 mol-1) | log Kow | log Kcw | Washout Ratio, Wg (unitless) | rcl (s cm-1) | log Kp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 3-Pentylphenol | NA | NA | NA | NA | 3.13 | 3.09 | NA | NA | NA |
| 52 | 4-Hexylphenol | 2446-69-7 | NA | 2.19 | NA | NA | NA | NA | NA | NA |
| 53 | 2-Heptylphenol | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 54 | 4-Heptylphenol | 1987-50-4 | NA | NA | NA | NA | NA | NA | NA | NA |
| 55 | 4-Octylphenol | 1806-26-4 | 0.071b | 0.1000 | 0.492 | 4.12 | 4.05 | 5.04 x 103 | 3.56 x 102 | -4.79 |
| 56 | 4-tert-Octylphenol | 140-66-9 | NA | NA | NA | 3.70 | 3.65 | NA | NA | NA |
| 57 | 4-Nonylphenol | 104-40-5 | 0.1b | 0.0432 | 1.57 | 4.48 | 4.40 | 1.58 x 103 | 1.08 x 103 | -4.93 |
| 58 | 1-Naphthol | 90-15-3 | 0.5b | 3.49 | 0.0285 | 2.84 | 2.81 | 8.70 x 104 | 2.69 | -4.83 |
| 59 | 2-Naphthol | 135-19-3 | 0.4b | 5.89 | 0.00746 | 2.70 | 2.67 | 3.32 x 105 | 1.16 | -4.39 |
| 60 | 2-Phenylphenol | 90-43-7 | 30b | 5.57 | 2.66 | 3.09 | 3.05 | 9.32 x 102 | 8.58 x 102 | -6.55 |
| 61 | 3-Phenylphenol | 588-51-8 | NA | NA | NA | NA | NA | NA | NA | NA |
| 62 | 4-Phenylphenol | 92-69-3 | 8.7b | 0.078 | 4.71 | 3.20 | 3.16 | 5.26 x 102 | 1.11 x 103 | -6.69 |
| 63 | 2-Allylphenol | 1745-81-9 | NA | NA | NA | 2.64 | 2.61 | NA | NA | NA |

aNA, not available

bVapor pressure of subcooled liquid.