Appendix E:

Range of Reported Values of H and log Kow for Individual Gaseous

Hazardous Air Pollutants and Mixtures

Table E.1.  Range of reported values of H and log Kow for individual hazardous air pollutants at 293-298 K.

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 1 | Acetaldehyde | 75-07-0 | 6.0 - 60 | 5.423 - 10.18 | -0.53 - 0.52 |
| 2 | Acetamide | 60-35-5 | NAc | 3.53 x 10-4 | -1.58 - (-1.09) |
| 3 | Acetonitrile | 75-05-8 | 1.9 - 3.5 | 2.033 - 3.501 | -0.54 - 0.06 |
| 4 | Acetophenone | 98-86-2 | 0.92 - 1.1 | 0.784 - 1.084 | 1.56 - 1.87 |
| 5 | 2-Acetylamino- fluorene | 53-96-3 | NA | NA | NA |
| 6 | Acrolein | 107-02-8 | 10 - 14 | 0.446, 12.36 | -0.10 - 0.101 |
| 7 | Acrylamide | 79-06-1 | NA | 1.44 x 10-4 | -1.24 - (-0.67) |
| 8 | Acrylic acid | 79-10-7 | 4.2 x 10-2 | 0.042 | 0.31 - 0.43 |
| 9 | Acrylonitrile | 107-13-1 | 9.2, 14 | 8.918 - 11.14 | -0.92 - 1.20 |
| 10 | Allyl chloride | 107-05-1 | 1.1 x 103, 9.2 x 102 | NA | NA |
| 11 | 4-Aminobiphenyl | 92-67-1 | NA | NA | NA |
| 12 | Aniline | 62-53-3 | NA | 12.16, 13778 | 0.78 - 1.34 |
| 13 | o-Anisidine | 90-04-0 | NA | NA | NA |
| 14 | Benzene | 71-43-2 | 4.6 x 102 - 8.4 x 102 | 441 - 740 | 1.56 - 2.69 |
| 15 | Benzidine | 92-87-5 | NA | 4.60 x 10-7, 3.93 x 10-6 | 1.34 - 2.40 |
| 16 | Benzotrichloride | 98-07-7 | NA | NA | NA |
| 17 | Benzyl chloride | 100-44-7 | 63 | NA | NA |
| 18 | Biphenyl | 92-52-4 | 28 - 84 | 11.55 - 124 | 3.16 - 4.52 |
| 19 | Bis(chloromethyl) ether | 542-88-1 | NA | 21.27 | -0.38, 2.40 |
| 20 | Bis(2-ethylhexyl) phthalate | 117-81-7 | NA | 0.0041 - 3.14 | 3.58 - 9.70 |
| 21 | Bromoform | 75-25-2 | 44 - 72 | 7.66 - 68.3 | 2.30 - 2.38 |
| 22 | 1,3-Butadiene | 106-99-0 | 6.3 x 103 - 7.2 x 103 | 6227 - 10820 | 1.56 - 1.99 |
| 23 | Caprolactan | 105-60-2 | NA | NA | NA |
| 24 | Captan | 133-06-2 | NA | 0.60 | 1.52 - 4.10 |
| 25 | Carbaryl | 63-25-2 | NA | 4.48 x 10-5 - 1.287 | 1.59 - 3.13 |

Table E.1.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 26 | Carbon disulfide | 75-15-0 | 1.3 x 103 - 2.3 x 103 | NA | NA |
| 27 | Carbon tetrachloride | 56-23-5 | 2.0 x 103 - 3.6 x 103 | 2000 - 3081 | 2.03 - 3.00 |
| 28 | Carbonyl sulfide | 463-58-1 | 3.1 x 103 - 6.8 x 103 | NA | NA |
| 29 | Catechol | 120-80-9 | 2.2 x 10-2 | 0.022 | 0.53 - 1.28 |
| 30 | Chloramben | 133-90-4 | NA | 0.274 | -2.64 - 1.46 |
| 31 | cis-Chlordane | 5103-71-9 | NA | 0.248 - 570 | 2.78 - 6.21 |
| 32 | trans-Chlordane | 5103-74-2 | NA | NA | NA |
| 33 | Chloroacetic acid | 79-11-8 | 9.2 x 10-4 | NA | 0.22 |
| 34 | 2-Chloroacetophenone | 532-27-4 | NA | NA | NA |
| 35 | Chlorobenzene | 108-90-7 | 3.2 x 102 - 4.6 x 102 | 263 - 545 | 2.13 - 3.18 |
| 36 | Chlorobenzilate | 510-15-6 | NA | NA | NA |
| 37 | Chloroform | 67-66-3 | 1.1 x 102 - 6.8 x 102 | 198.0 - 536.9 | 1.43 - 2.22 |
| 38 | Chloromethyl methyl ether | 107-30-2 | NA | NA | NA |
| 39 | Chloroprene | 126-99-8 | NA | 3242 | 2.03 |
| 40 | o-Cresol | 95-48-7 | 8.4 x 10-2 - 3.9 x 10-1 | 0.07 - 0.284 | 1.95 - 2.24 |
| 41 | m-Cresol | 108-39-4 | 7.2 x 10-2, 1.6 x 10-1 | 0.0203 - 0.1068 | 1.94 - 2.12 |
| 42 | p-Cresol | 106-44-5 | 4.1 x 10-2 - 1.9 x 10-1 | 0.0223 - 1.0820 | 1.62 - 2.20 |
| 43 | oCresyl acetate | 533-18-6 | NA | NA | NA |
| 44 | Cumene | 98-82-8 | 1.3 x 102 - 1.5 x 103 | 909 - 1521 | 3.23 - 3.66 |
| 45 | Diazomethane | 334-88-3 | NA | NA | NA |
| 46 | Dibenzofuran | 132-64-9 | NA | NA | 3.86 - 4.57 |
| 47 | 1,2-Dibromo-3-chloropropane | 96-12-8 | NA | NA | NA |
| 48 | 2,4-Dichloro-phenoxyacetic acid | 94-75-7 | NA | 1.39 x 10-5 - 0.55 | -1.36 - 4.88 1.57 - 3.00 |
| 49 | 1,1-Dichloro-2,2-bis(p-chlorophenyl)ethylene | 72-55-9 | NA | 0.78 - 370 | 4.28 - 6.96 |
| 50 | Dibutyl phthalate | 84-74-2 | NA | 0.0284 - 0.456 | 3.31 - 5.74 |

Table E.1.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 51 | 1,4-Dichlorobenzene | 106-46-7 | 64-4.6 x 102 | 119 - 744 | 3.14 - 3.90 |
| 52 | 3,3'-Dichlorobenzidine | 84-68-4 | NA | 0.0811 | 3.02 - 3.64 |
| 53 | Dichloroethyl ether | 111-44-4 | 2.2 | 1.320, 28.97 | 1.12 - 1.58 |
| 54 | trans-1,3-Dichloropropene | 10061-02-6 | 1.6 x 102-3.6 x 102 | 137 - 429 | 1.41 - 1.98 |
| 55 | Dichlorvos | 62-73-7 | NA | 0.097 - 0.194 | 1.16 - 2.29 |
| 56 | Diethanolamine | 111-42-2 | NA | 5.42 x 10-9 | -1.25 - (-1.43) |
| 57 | Diethyl sulfate | 64-67-5 | NA | NA | NA |
| 58 | Diethylene glycol monobutyl ether | 112-34-5 | NA | NA | NA |
| 59 | 3,3'-Dimethoxybenzidine | 119-90-4 | NA | NA | NA |
| 60 | 4-Dimethylamino- azobenzene | 60-11-7 | NA | NA | NA |
| 61 | N,N-Dimethylaniline | 121-69-7 | NA | 11.734 | 1.66 - 2.62 |
| 62 | 3,3'-Dimethylbenzidine | 119-93-7 | NA | NA | NA |
| 63 | Dimethylcarbamyl chloride | 79-44-7 | NA | NA | NA |
| 64 | N,N-Dimethylformamide | 68-12-2 | NA | NA | NA |
| 65 | 1,1-Dimethylhydrazine | 57-14-7 | NA | NA | NA |
| 66 | Dimethyl phthalate | 131-11-3 | 3.4 x 104 | 0.011 - 0.218 | 1.47 - 3.40 |
| 67 | Dimethyl sulfate | 77-78-1 | NA | NA | NA |
| 68 | 4,6-Dinitro-o-cresol | 534-52-1 | 2.2 x 10-4, 4.4 x 10-2 | 4.050, 43.22 | 2.12 - 2.85 |
| 69 | 2,4-Dinitrophenol | 51-28-5 | 8.4 x 10-3, 2.9 x 10-2 | 8.880 x 10-5-0.00355 | 1.18 - 1.92 |
| 70 | 2,4-Dinitrotoluene | 121-14-2 | 4.8 | 0.0160 - 0.4560 | 1.85 - 2.04 |
| 71 | 1,4-Dioxane | 123-91-1 | 0.46 - 0.72 | 0.431 - 1.564 | -0.42 - 0.01 |
| 72 | 1,2-Diphenylhydrazine | 122-66-7 | NA | 3.45 x 10-4 | 1.70 - 3.82 |
| 73 | Epichlorohydrin | 13403-37-7 | 3.6 | 3.375 | 0.30, 0.58 |
| 74 | 1,2-Epoxybutane | 106-88-7 | NA | NA | NA |
| 75 | Ethyl acrylate | 140-88-5 | NA | 34.041 | 0.66 - 1.33 |

Table E.1.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 76 | Ethylbenzene | 100-41-4 | 6.0 x 102 - 9.2 x 102 | 653 - 1001 | 2.68 - 3.43 |
| 77 | Ethyl carbamate | 51-79-6 | NA | NA | NA |
| 78 | Ethyl chloride | 75-00-3 | 2.0 x 102 - 1.5 x 103 | 860 - 1512 | 1.20 - 1.55 |
| 79 | Ethylene dibromide | 106-93-4 | 68 - 92 | 15.64 - 133.12 | NA |
| 80 | Ethylene dichloride | 107-06-2 | 92 - 1.6 x 102 | 92.00 - 143 | 1.30 - 1.79 |
| 81 | Ethylene glycol | 107-21-1 | 2.5 x 10-5, 6.0 x 10-3 | 0.006 - 5.81 x 10-6 | -2.02 - (-0.54) |
| 82 | Ethylene glycol monobutyl ether | 111-76-2 | NA | NA | NA |
| 83 | Ethyleneimine | 151-56-4 | NA | NA | NA |
| 84 | Ethylene oxide | 75-21-8 | NA | NA | NA |
| 85 | Ethylene thiourea | 96-45-7 | NA | NA | NA |
| 86 | Ethylidene dichloride | 75-34-3 | 5.1 x 102 - 7.8 x 102 | 421.4 - 594.9 | 1.78 - 1.92 |
| 87 | Formaldehyde | 50-00-0 | 7.2 x 10-3 - 3.4 x 10-2 | 0.0169 - 0.0331 | -0.75 - 0.35 |
| 88 | Heptachlor | 76-44-8 | NA | 17.8 - 233 | 3.87 - 5.73 |
| 89 | Hexachlorobenzene | 118-74-1 | 5.1 - 4.2 x 104 | 5.07 - 172 | 4.13 - 7.42 |
| 90 | Hexachlorobutadiene | 87-68-3 | 1.0 x 103 - 2.5 x 103 | 1044 - 2604 | 3.74 - 4.78 |
| 91 | (α)1,2,3,4,5,6-Hexachlorocyclohexane | 60291-32-9 | 7.8 x 10-1 | 0.43 - 2.16 | 3.80 - 4.44 |
| 92 | (γ)1,2,3,4,5,6-Hexachlorocyclohexane | 58-89-9 | 4.6 x 10-2 - 3.6 x 10-1 | 0.005 - 1.496 | 2.81 - 5.43 |
| 93 | Hexachloro- cyclopentadiene | 77-47-4 | NA | 1621 - 3668 | 3.99 - 5.51 |
| 94 | Hexachloroethane | 67-72-1 | 1.2 x 102 - 2.3 x 103 | 123.9 - 1302 | 2.73 - 4.62 |
| 95 | Hexamethylene diisocyanate | 822-06-0 | NA | NA | NA |
| 96 | Hexamethyl- phosphoramide | 680-31-9 | NA | NA | NA |
| 97 | Hexane | 110-54-3 | 1.0 x 105 - 1.8 x 105 | 50,586 - 190,000 | 2.90 - 4.25 |
| 98 | Hydrazine | 302-01-2 | NA | NA | NA |
| 99 | Hydroquinone | 123-31-9 | 3.9 x 10-6 - 6.0 x 10-6 | 3.89 x 10-6 - 5.91 x 10-6 | 0.47 - 1.28 |
| 100 | Isophorone | 78-59-1 | 0.60 | NA | NA |

Table E.1.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 101 | Maleic anhydride | 108-31-6 | NA | NA | NA |
| 102 | Methanol | 67-56-1 | 4.4 x 10-1 - 7.2 x 10-1 | 0.319 - 13.68 | -1.27 - (-0.25) |
| 103 | Methoxychlor | 72-43-5 | NA | 0.999, 1.60 | 3.31 - 5.67 |
| 104 | Methyl bromide | 74-83-9 | 5.3 x 102 - 2.3 x 104 | 533 - 20,262 | 1.00 - 1.19 |
| 105 | Methyl chloride | 74-87-3 | 7.8 x 102 - 3.5 x 103 | 669 - 4052 | 0.81 - 1.06 |
| 106 | Methyl chloroform | 71-55-6 | 4.6 x 102 - 3.8 x 103 | 399.9 - 3620 | 1.96 - 2.60 |
| 107 | Methyl ethyl ketone | 78-93-3 | 4.8 - 25 | 1.064 - 13.17 | -0.27 - 0.785 |
| 108 | Methylhydrazine | 60-34-4 | NA | NA | NA |
| 109 | Methyl iodide | 74-88-4 | 2.9 x 102 - 7.2 x 102 | 278.1 - 554.9 | 1.50 - 1.69 |
| 110 | Methyl isobutyl ketone | 108-10-1 | 19 - 46 | 9.523, 39.52 | 1.09 - 1.31 |
| 111 | Methyl isocyanate | 624-83-9 | NA | NA | NA |
| 112 | Methyl methacrylate | 80-62-6 | NA | 24.310 - 32.823 | 0.67 - 1.38 |
| 113 | Methyl tert-butyl ether | 1634-04-4 | 60, 63 | 59.46 - 305 | 0.94 - 1.30 |
| 114 | 4,4'-Methylenebis (2-chloroaniline) | 101-14-4 | NA | NA | NA |
| 115 | Methylene chloride | 75-09-2 | 84 - 3.3 x 102 | 173.0 - 451.1 | 1.01 - 1.63 |
| 116 | 4,4'-Methylenediphenyl diisocyanate | 101-68-8 | NA | NA | NA |
| 117 | 4,4'-Methylenedianiline | 101-77-9 | NA | NA | NA |
| 118 | Naphthalene | 91-20-3 | 42 - 1.3 x 102 | 29.2 - 123.95 | 3.01 - 4.70 |
| 119 | Nitrobenzene | 98-95-3 | 2.2 - 2.5 | 1.327 - 4.723 | 1.60 - 2.93 |
| 120 | 4-Nitrobiphenyl | 92-93-3 | NA | NA | NA |
| 121 | 4-Nitrophenol | 100-02-7 | 3.8 x 10-7 - 1.0 x 10-3 | 1.04 x 10-5 - 2.533 | 0.70 - 2.91 |
| 122 | 2-Nitropropane | 79-46-9 | 9.2, 12 | NA | NA |
| 123 | N-Nitroso-N-methylurea | 684-93-5 | NA | NA | NA |
| 124 | N-Nitrosodimethylamine | 62-75-9 | NA | 3.344 | -0.68 - 0.76 |
| 125 | N-Nitrosomorpholine | 59-89-2 | NA | NA | NA |

Table E.1.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 126 | Parathion | 56-38-2 | 1.2 x 10-1 | 0.012 - 0.120 | 2.15 - 4.00 |
| 127 | Pentachloro- nitrobenzene | 82-68-8 | NA | 0.3718 - 0.4812 | 4.22 - 6.0 |
| 128 | Pentachlorophenol | 87-86-5 | 2.5 x 10-3 - 9.2 | 0.0025 - 0.2840 | 2.50 - 5.86 |
| 129 | Phenol | 108-95-2 | 3.4 x 10-2 - 1.3 x 103 | 0.0274 - 1.0820 | 0.62 - 2.20 |
| 130 | p-Phenylenediamine | 106-50-3 | NA | NA | NA |
| 131 | Phosgene | 75-44-5 | NA | NA | NA |
| 132 | Phosphine | 7803-51-2 | 1.3 x 104 | NA | NA |
| 133 | Phthalic anhydride | 85-44-9 | NA | NA | NA |
| 134 | 1,3-Propane sultone | 1120-71-4 | NA | NA | NA |
| 135 | β -Propiolactone | 57-57-8 | NA | NA | NA |
| 136 | Propionaldehyde | 123-38-6 | 7.8 - 36 | 7.436 - 13.77 | 0.00 - 0.83 |
| 137 | Propoxur | 114-26-1 | NA | 1.98 x 10-6 - 0.1308 | 1.45 - 2.01 |
| 138 | 1,2-Dichloropropane | 26198-63-0 | 2.5 x 102 - 3.4 x 102 | 31.21 - 2261 | 1.99 - 2.30 |
| 139 | Propylene oxide | 75-56-9 | 19 | 8.653 | 0.03 - 0.23 |
| 140 | 1,2-Propylenimine | 75-55-8 | NA | NA | NA |
| 141 | Quinoline | 91-22-5 | 2.7 x 10-2 | 0.0253 - 0.1680 | 1.88 - 2.10 |
| 142 | Quinone | 106-51-4 | NA | NA | NA |
| 143 | Styrene | 100-42-5 | 2.7 x 102 - 3.5 x 102 | 233 - 527 | 2.76 - 3.39 |
| 144 | Styrene oxide | 96-09-3 | NA | NA | 1.43 - 1.84 |
| 145 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin | 1746-01-6 | NA | 0.0021 - 10.34 | 5.38 - 8.93 |
| 146 | 1,1,2,2-Tetrachloroethane | 79-34-5 | 34 - 56 | 25.33 - 48.6 | 2.39 - 3.27 |
| 147 | Tetrachloroethylene | 127-18-4 | 8.4 x 102 - 3.0 x 103 | 248 - 2940 | NA |
| 148 | Toluene | 108-88-3 | 4.8 x 102 - 7.8 x 102 | 518 - 825 | 1.83 - 3.06 |
| 149 | Toluene-2,4-diamine | 95-80-7 | NA | NA | NA |
| 150 | 2,4-Toluene diisocyanate | 584-84-9 | NA | NA | NA |

Table E.1.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 151 | o-Toluidine | 95-53-4 | NA | 0.095 | 1.29 - 1.63 |
| 152 | Toxaphene | 8001-35-2 | NA | 0.0238 - 6382 | 2.92 - 5.50 |
| 153 | 1,2,4-Trichlorobenzene | 120-82-1 | 1.4 x 102 - 3.8 x 102 | 101 - 435 | 3.34 - 4.40 |
| 154 | 1,1,2-Trichloroethane | 79-00-5 | 78 - 1.2 x 102 | 70.92 - 120 | 1.89 - 2.42 |
| 155 | Trichloroethylene | 79-01-6 | 4.2 x 102 - 1.4 x 103 | 119 - 1215 | 2.04 - 3.57 |
| 156 | 2,4,5-Trichlorophenol | 95-95-4 | NA | 0.521, 0.590 | 2.39 - 4.19 |
| 157 | 2,4,6-Trichlorophenol | 88-06-2 | NA | 0.405, 0.5687 | 2.67 - 4.03 |
| 158 | Triethylamine | 121-44-8 | 15 | NA | NA |
| 159 | Trifluralin | 1582-09-8 | NA | 4.02 - 16.61 | 3.0 - 5.34 |
| 160 | 2,2,4-Trimethylpentane | 540-84-1 | NA | 110,669 - 472,092 | 4.06 - 5.83 |
| 161 | Vinyl acetate | 108-05-4 | 60 | 48.74 | 0.21 - 0.73 |
| 162 | Vinyl bromide | 593-60-2 | NA | NA | NA |
| 163 | Vinyl chloride | 75-01-4 | 2.2 x 103 - 1.2 x 105 | 494.6 - 8246 | 0.60 - 1.39 |
| 164 | Vinylidene chloride | 75-35-4 | 1.5 x 103 - 1.9 x 104 | NA | NA |
| 165 | o-Xylene | 95-47-6 | 3.5 x 102 - 5.3 x 102 | 429 - 895 | 2.12 - 3.42 |
| 166 | m-Xylene | 108-38-3 | 6.0 x 102 - 7.8 x 102 | 506 - 1115 | 3.04 - 3.50 |
| 167 | p-Xylene | 106-42-3 | 4.4 x 102 - 8.4 x 102 | 506 - 1185 | 3.06 - 3.50 |

aSander (1999)

bMackay et al. (1992a and b, 1993, 1995)

cNA, not available

Table E.2.  Range of reported values of H and log Kow for polychlorinated byphenyls (PCB) congeners at 293-298 K.

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2-Monochloro | 2051-60-7 | NAe | 20.43 - 30.2 | 28.9 - 74.6 | 3.75 - 5.70 |
| 2 | 3-Monochloro | 2051-61-8 | NA | 29.0 | 14.49 - 75.5 | 4.34 - 4.95 |
| 3 | 4-Monochloro | 2051-62-9 | NA | 27.8 | 24.39 - 58.06 | 4.26 - 4.94 |
| 4 | 2,2'-Dichloro | 13029-08-8 | 23 - 56 | 23, 33.3 | 22.29 - 59.17 | 3.55 - 5.70 |
| 5 | 2,3-Dichloro | 16605-91-7 | NA | 23, 24.2 | 19.56 - 28.57 | 4.97 - 5.36 |
| 6 | 2,3'-Dichloro | 25569-80-6 | NA | 25, 33.1 | NA | NA |
| 7 | 2,4-Dichloro | 33284-50-3 | NA | 28, 38.2 | 24.78 - 96.66 | 4.67 - 5.30 |
| 8 | 2,4'-Dichloro | 34883-43-7 | NA | 20.3 - 30.7 | 22.29 - 96.7 | 4.47 - 5.51 |
| 9 | 2,5-Dichloro | 34883-39-1 | 34 - 44 | 32.8 | 20.1 - 39.31 | 4.67 - 5.46 |
| 10 | 2,6-Dichloro | 33146-45-1 | NA | 42.9 | 47.61 - 47.83 | 4.05 - 5.46 |
| 11 | 3,3'-Dichloro | 2050-67-1 | 14, 24 | 29.4 | 13.58 - 23.61 | 5.10 - 5.34 |
| 12 | 3,4-Dichloro | 2974-92-7 | 10, 21 | 14, 23.7 | 9.60 - 20.77 | 5.05 - 5.51 |
| 13 | 3,4'-Dichloro | 2974-90-5 | NA | 25.7 | NA | NA |
| 14 | 3,5-Dichloro | 34883-41-5 | NA | 42.6 | 16.72 - 49.55 | 5.10 - 5.41 |
| 15 | 4,4'-Dichloro | 2050-68-2 | 11 - 31 | 22.7 | 9.66 - 30.4 | 4.77 - 5.58 |
| 16 | 2,2',3-Trichloro | 38444-78-9 | NA | 20, 25.5 | 20.27 - 81.77 | 4.15 - 5.60 |
| 17 | 2,2',4-Trichloro | 37680-66-3 | NA | 37.8 | NA | NA |
| 18 | 2,2',5-Trichloro | 37680-65-2 | NA | 25.3 - 58.1 | 20.26 - 101.53 | 3.89 - 6.22 |
| 19 | 2,2',6-Trichloro | 38444-73-4 | NA | 23, 44.7 | NA | NA |
| 20 | 2,3,3'-Trichloro | 38444-84-7 | NA | 16, 22.0 | 16.21 - 82.0 | 4.99 - 5.60 |
| 21 | 2,3,4-Trichloro | 55702-46-0 | NA | 23.0 | 14.79 - 21.38 | 5.39 - 5.88 |
| 22 | 2,3,4'-Trichloro | 38444-85-8 | NA | 14, 19.4 | NA | NA |
| 23 | 2,3,5-Trichloro | 55720-44-0 | NA | 32.3 | NA | NA |
| 24 | 2,3,6-Trichloro | 58702-45-9 | NA | 22, 31.5 | NA | NA |
| 25 | 2,3',4-Trichloro | 55712-37-3 | NA | 32.0 | NA | NA |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 26 | 2,3',5-Trichloro | 38444-81-4 | 29, 34 | 20, 30.2 | 20.27 - 34.34 | 5.18 - 5.88 |
| 27 | 2,3',6-Trichloro | 38444-76-7 | NA | 41.0 | NA | NA |
| 28 | 2,4,4'-Trichloro | 7012-37-5 | 28 | 20.3 - 38.14 | 20.27 - 33.0 | 5.11 - 6.00 |
| 29 | 2,4,5-Trichloro | 15862-07-4 | NA | 20.3 - 37.89 | 20.27 - 27.05 | 5.22 - 6.25 |
| 30 | 2,4,6-Trichloro | 35693-92-6 | 0.72 - 68 | 58.0 | 49.51 - 65.76 | 5.44 - 6.17 |
| 31 | 2,4',5-Trichloro | 16606-02-3 | NA | 19, 27.8 | 19.25 - 94.13 | 5.11 - 6.22 |
| 32 | 2,4',6-Trichloro | 38444-77-8 | NA | 39.7 | NA | NA |
| 33 | 2',3,4-Trichloro | 38444-86-9 | NA | 24.3 | 15.20 - 43.67 | 5.46 - 6.10 |
| 34 | 2',3,5-Trichloro | 37680-68-5 | NA | 42.7 | NA | NA |
| 35 | 3,3',4-Trichloro | 37680-69-6 | NA | 18.2 | 8.13 - 22.49 | 5.53 - 5.82 |
| 36 | 3,3',5-Trichloro | 38444-87-0 | NA | 17, 34.1 | NA | NA |
| 37 | 3,4,4'-Trichloro | 38444-90-5 | NA | 10, 15.4 | 7.34 - 84.21 | 4.94 - 6.00 |
| 38 | 3,4,5-Trichloro | 53555-66-1 | NA | 23.5 | NA | NA |
| 39 | 3,4',5-Trichloro | 38444-88-1 | NA | 30.3 | NA | NA |
| 40 | 2,2',3,3'-Tetrachloro | 38444-93-8 | 12, 21 | 10, 18.5 | 10.13 - 21.94 | 4.54 - 6.67 |
| 41 | 2,2'3,4-Tetrachloro | 52663-59-9 | NA | 14, 24.8 | NA | NA |
| 42 | 2,2',3,4'-Tetrachloro | 36559-22-5 | NA | 14, 25.9 | NA | NA |
| 43 | 2,2',3,5-Tetrachloro | 70362-46-8 | NA | 33.9 | NA | NA |
| 44 | 2,2',3,5'-Tetrachloro | 41464-39-5 | NA | 23.3 - 32.8 | 19.15 - 79.28 | 4.79 - 6.67 |
| 45 | 2,2'3,6-Tetrachloro | 70362-45-7 | NA | 36.0 | NA | NA |
| 46 | 2,2',3,6'-Tetrachloro | 41464-47-5 | NA | 34.3 | NA | NA |
| 47 | 2,2',4,4'-Tetrachloro | 2437-79-8 | NA | 19, 37.3 | 17.38 - 44.48 | 5.20 - 7.13 |
| 48 | 2,2',4,5-Tetrachloro | 70362-47-9 | NA | 30.7 | NA | NA |
| 49 | 2,2',4,5'-Tetrachloro | 41464-40-8 | NA | 21, 35.8 | 20.27 - 47.72 | 5.20 - 6.88 |
| 50 | 2,2',4,6-Tetrachloro | 62796-65-8 | NA | 61.8 - 138 | 58.57 - 137.8 | 5.63 - 5.96 |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 51 | 2,2',4,6'-Tetrachloro | 65194-04-7 | NA | 51.7 | 49.04 - 101.2 | 5.51 - 5.96 |
| 52 | 2,2',5,5'-Tetrachloro | 35693-99-3 | 2.5 - 53 | 20 - 53.2 | 2.53 - 160 | 3.91 - 6.90 |
| 53 | 2,2',5,6'-Tetrachloro | 41464-41-9 | 26, 42 | 43.6 | 28.67 - 112.5 | 5.46 - 5.96 |
| 54 | 2,2',6,6'-Tetrachloro | 15968-05-5 | 15 - 190 | 20, 58.0 | 15.0 - 188.5 | 4.16 - 7.13 |
| 55 | 2,3,3',4-Tetrachloro | 74338-24-2 | NA | 18.5 | NA | NA |
| 56 | 2,3,3',4'-Tetrachloro | 41464-43-1 | NA | 15.3 | NA | NA |
| 57 | 2,3,3',5-Tetrachloro | 70424-67-8 | NA | 27.4 | NA | NA |
| 58 | 2,3,3',5'-Tetrachloro | 41464-49-7 | NA | 25.3 | NA | NA |
| 59 | 2,3,3',6-Tetrachloro | 74472-33-6 | NA | 30.8 | NA | NA |
| 60 | 2,3,4,4'-Tetrachloro | 33025-41-1 | NA | 15.5 | 10.34 - 84.20 | 5.33 - 6.25 |
| 61 | 2,3,4,5-Tetrachloro | 33284-53-6 | NA | 24.1 | 11.45 - 17.53 | 5.72 - 6.88 |
| 62 | 2,3,4,6-Tetrachloro | 54230-23-7 | NA | 21, 37.5 | NA | NA |
| 63 | 2,3,4',5-Tetrachloro | 74472-35-8 | NA | 24.6 | NA | NA |
| 64 | 2,3,4',6-Tetrachloro | 52663-58-8 | NA | 27.6 | NA | NA |
| 65 | 2,3,5,6-Tetrachloro | 33284-54-7 | NA | 34.1 | NA | NA |
| 66 | 2,3',4,4'-Tetrachloro | 32598-10-0 | NA | 13.7 - 84.2 | 13.68 - 84.20 | 5.45 - 6.67 |
| 67 | 2,3',4,5-Tetrachloro | 73575-53-8 | NA | 10, 23.7 | NA | NA |
| 68 | 2,3',4,5'-Tetrachloro | 73575-52-7 | NA | 38.2 | NA | NA |
| 69 | 2,3',4,6-Tetrachloro | 60233-24-1 | NA | 51.3 | NA | NA |
| 70 | 2,3',4',5-Tetrachloro | 32598-11-1 | NA | 10, 20.5 | 10.13 - 20.26 | 5.20 - 7.13 |
| 71 | 2,3',4',6-Tetrachloro | 41464-46-4 | NA | 31.8 | NA | NA |
| 72 | 2,4',5,5'-Tetrachloro | 41464-42-0 | NA | 36.7 | NA | NA |
| 73 | 2,3',5',6-Tetrachloro | 74338-23-1 | NA | 52.7 | NA | NA |
| 74 | 2,4,4',5-Tetrachloro | 32690-93-0 | NA | 10, 21.8 | NA | NA |
| 75 | 2,4,4',6-Tetrachloro | 32598-12-2 | NA | 47.1 | 42.25 - 55.32 | 5.87 - 6.05 |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 76 | 2',3,4,5-Tetrachloro | 70362-48-0 | NA | 24.2 | NA | NA |
| 77 | 3,3',4,4'-Tetrachloro | 32598-13-3 | 4.4, 9.2 | 4.37 - 16.2 | 1.72 - 9.52 | 5.27 - 6.77 |
| 78 | 3,3'4,5-Tetrachloro | 70362-49-1 | NA | 16.5 | NA | NA |
| 79 | 3,3',4,5'-Tetrachloro | 41464-48-6 | NA | 20.0 | NA | NA |
| 80 | 3,3',5,5'-Tetrachloro | 33284-52-5 | NA | 38.0 | 12.46 - 63.02 | 6.05 - 6.85 |
| 81 | 3,4,4',5-Tetrachloro | 70362-50-4 | NA | 14.5 | NA | NA |
| 82 | 2,2',3,3',4-Pentachloro | 52663-62-4 | NA | 14.8 | NA | NA |
| 83 | 2,2',3,3',5-Pentachloro | 60145-20-2 | NA | 21.5 | 16.62 - 26.65 | 6.24 - 6.26 |
| 84 | 2,2',3,3',6-Pentachloro | 52663-60-2 | NA | 25.5 | NA | NA |
| 85 | 2,2',3,4,4'-Pentachloro | 65510-45-4 | NA | 6.7, 19.5 | NA | NA |
| 86 | 2,2',3,4,5-Pentachloro | 55312-69-1 | NA | 24.1 | 17.23 - 84.1 | 6.20 - 7.49 |
| 87 | 2,2',3,4,5'-Pentachloro | 38380-02-8 | NA | 7.5 - 37.71 | 7.50 - 33.44 | 5.45 - 7.43 |
| 88 | 2,2',3,4,6-Pentachloro | 55215-17-3 | NA | 39.0 | 34.65 - 136.8 | 6.07 - 7.51 |
| 89 | 2,2',3,4,6'-Pentachloro | 73575-57-2 | NA | 30.2 | NA | NA |
| 90 | 2,2',3,4',5-Pentachloro | 68194-07-0 | NA | 29.8 | NA | NA |
| 91 | 2,2',3,4',6-Pentachloro | 58194-05-8 | NA | 35.1 | NA | NA |
| 92 | 2,2',3,5,5'-Pentachloro | 52663-61-3 | NA | 26.3 | NA | NA |
| 93 | 2,2',3,5,6-Pentachloro | 73575-56-1 | NA | 34.5 | NA | NA |
| 94 | 2,2',3,5,6'-Pentachloro | 73575-55-0 | NA | 39.7 | NA | NA |
| 95 | 2,2',3,5',6-Pentachloro | 38379-99-6 | NA | 12, 30.4 | 20.06 - 82.78 | 5.18 - 6.55 |
| 96 | 2,2',3,6,6'-Pentachloro | 73575-54-9 | NA | 41.6 | NA | NA |
| 97 | 2,2',3',4,5-Pentachloro | 41464-51-1 | NA | 7.5, 18.2 | NA | NA |
| 98 | 2,2',3',4,6-Pentachloro | 60233-25-2 | NA | 39.7 | NA | NA |
| 99 | 2,2',4,4',5-Pentachloro | 38380-01-7 | NA | 7.9, 25.3 | 7.90 - 30.50 | 6.11 - 7.21 |
| 100 | 2,2',4,4',6-Pentachloro | 39485-83-1 | NA | 57.0 | 62.62 - 97.27 | 5.5 - 6.44 |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 101 | 2,2'4,5,5'-Pentachloro | 37680-73-2 | 7.2-33 | 9.1 - 42.07 | 7.09 - 35.48 | 4.12 - 7.64 |
| 102 | 2,2',4,5,6'-Pentachloro | 68194-06-9 | NA | 9.1, 37.6 | NA | NA |
| 103 | 2,2',4,5',6-Pentachloro | 60145-21-3 | NA | 51.0 | NA | NA |
| 104 | 2,2',4,6,6'-Pentachloro | 56558-16-8 | 92, 180 | 66.0 - 185 | 90.9 - 185.4 | 5.37 - 6.40 |
| 105 | 2,3,3',4,4'-Pentachloro | 32598-14-4 | NA | 5.68 - 33.6 | NA | NA |
| 106 | 2,3,3',4,5-Pentachloro | 70424-69-0 | NA | 16.7 | NA | NA |
| 107 | 2,3,3',4',5-Pentachloro | 70424-68-9 | NA | 16.1 | NA | NA |
| 108 | 2,3,3',4,5'-Pentachloro | 70362-41-3 | NA | 17.8 | NA | NA |
| 109 | 2,3,3',4,6-Pentachloro | 74472-35-8 | NA | 28.6 | NA | NA |
| 110 | 2,3,3',4',6-Pentachloro | 38380-03-9 | NA | 19.9 | 10.74 - 37.48 | 6.20 - 6.53 |
| 111 | 2,3,3',5,5'-Pentachloro | 39635-32-0 | NA | 27.0 | NA | NA |
| 112 | 2,3,3',5,6-Pentachloro | 74472-36-9 | NA | 27.0 | NA | NA |
| 113 | 2,3,3',5',6-Pentachloro | 68194-10-5 | NA | 33.0 | NA | NA |
| 114 | 2,3,4,4',5-Pentachloro | 74472-37-0 | NA | 14.5 | NA | NA |
| 115 | 2,3,4,4',6-Pentachloro | 74472-38-1 | NA | 24.9 | NA | NA |
| 116 | 2,3,4,5,6-Pentachloro | 18259-05-7 | NA | 30.0 | 18.34 - 23.41 | 6.14 - 7.60 |
| 117 | 2,3,4',5,6-Pentachloro | 68194-11-6 | NA | 24.4 | NA | NA |
| 118 | 2,3'4,4',5-Pentachloro | 31508-00-6 | NA | 9.35 - 40.5 | NA | NA |
| 119 | 2,3',4,4',6-Pentachloro | 56558-17-9 | NA | 31.5 | NA | NA |
| 120 | 2,3',4,5,5'-Pentachloro | 68194-12-7 | NA | 5.7, 24.9 | NA | NA |
| 121 | 2,3',4,5',6-Pentachloro | 56558-18-0 | NA | 56.6 | NA | NA |
| 122 | 2',3,3',4,5-Pentachloro | 76842-07-4 | NA | 12.7 | NA | NA |
| 123 | 2',3,4,4',5-Pentachloro | 65510-44-3 | NA | 17.6 | NA | NA |
| 124 | 2',3,4,5,5'-Pentachloro | 70424-70-3 | NA | 17.3 | NA | NA |
| 125 | 2',3,4,5,6'-Pentachloro | 74472-39-2 | NA | 29.2 | NA | NA |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 126 | 3,3'4,4',5-Pentachloro | 57465-28-8 | NA | 2.78 - 21.02 | NA | NA |
| 127 | 3,3',4,5,5'-Pentachloro | 39635-33-1 | NA | 15.8 | NA | NA |
| 128 | 2,2'3,3'4,4'-Hexachloro | 38380-07-3 | 3.1-51 | 1.3 - 50.7 | 1.32 - 50.66 | 6.14 - 8.31 |
| 129 | 2,2',3,3',4,5-Hexachloro | 55215-18-4 | NA | 2.9, 14.2 | 2.94 - 39.52 | 6.50 - 8.26 |
| 130 | 2,2'3,3',4,5'-Hexachloro | 52663-66-8 | NA | 3.7, 15.4 | NA | NA |
| 131 | 2,2',3,3',4,6-Hexachloro | 61798-70-7 | NA | 4.0, 24.5 | NA | NA |
| 132 | 2,2',3,3',4,6'-Hexachloro | 38380-05-1 | NA | 4.5, 20.5 | NA | NA |
| 133 | 2,2'3,3'5,5'-Hexachloro | 35694-04-3 | NA | 20.6 | NA | NA |
| 134 | 2,2',3,3',5,6-Hexachloro | 52704-70-8 | NA | 5.0, 23.3 | 4.96 - 57.76 | 6.20 - 8.31 |
| 135 | 2,2',3,3',5,6'-Hexachloro | 52744-13-5 | NA | 5.7, 27.2 | NA | NA |
| 136 | 2,2',3,3',6,6'-Hexachloro | 38411-22-2 | NA | 8.9, 32.6 | 8.92 - 93.22 | 4.91 - 8.35 |
| 137 | 2,2',3,4,4',5-Hexachloro | 35694-06-5 | NA | 18.8 | NA | NA |
| 138 | 2,2',3,4,4',5'-Hexachloro | 35065-28-2 | NA | 2.1 - 48.6 | 2.13 - 69.0 | 6.50 - 7.44 |
| 139 | 2,2',3,4,4',6-Hexachloro | 56030-56-9 | NA | 33.3 | NA | NA |
| 140 | 2,2',3,4,4',6'-Hexachloro | 59291-64-4 | NA | 31.2 | NA | NA |
| 141 | 2,2',3,4,5,5'-Hexachloro | 52712-04-6 | NA | 2.3, 17.6 | NA | NA |
| 142 | 2,2',3,4,5,6-Hexachloro | 41411-61-4 | NA | 31.9 | NA | NA |
| 143 | 2,2',3,4,5,6'-Hexachloro | 68194-15-0 | NA | 29.8 | NA | NA |
| 144 | 2,2',3,4,5',6-Hexachloro | 68194-14-9 | NA | 30.0 | NA | NA |
| 145 | 2,2',3,4,6,6'-Hexachloro | 74472-40-5 | NA | 47.6 | NA | NA |
| 146 | 2,2',3,4',5,5'-Hexachloro | 51908-16-8 | NA | 2.5, 19.0 | NA | NA |
| 147 | 2,2',3,4',5,6-Hexachloro | 68194-13-8 | NA | 5.2, 32.0 | NA | NA |
| 148 | 2,2',3,4',5,6'-Hexachloro | 74472-42-7 | NA | 43.5 | NA | NA |
| 149 | 2,2',3,4',5',6-Hexachloro | 38380-04-0 | NA | 24.0 | NA | NA |
| 150 | 2,2',3,4',6,6'-Hexachloro | 68194-08-1 | NA | 51.3 | NA | NA |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 151 | 2,2',3,5,5',6-Hexachloro | 52663-63-5 | NA | 6.0, 28.7 | NA | NA |
| 152 | 2,2',3,5,6,6'-Hexachloro | 68194-09-2 | NA | 43.3 | NA | NA |
| 153 | 2,2',4,4',5,5'-Hexachloro | 35065-27-1 | NA | 2.3 - 52.8 | 2.33 - 224 | 6.34 - 8.35 |
| 154 | 2,2',4,4',5,6'-Hexachloro | 60145-22-4 | NA | 38.7 - 76.7 | NA | NA |
| 155 | 2,2',4,4',6,6'-Hexachloro | 33979-03-2 | 12-160 | 85.2 | 11.65 - 817.9 | 6.01 - 8.31 |
| 156 | 2,3,3',4,4',5-Hexachloro | 38380-08-4 | 6.3-36 | 8.97 | NA | NA |
| 157 | 2,3,3',4,4',5'-Hexachloro | 69782-90-7 | NA | 8.56 | NA | NA |
| 158 | 2,3,3',4,4',6-Hexachloro | 74472-42-7 | NA | 16.7 | NA | NA |
| 159 | 2,3,3',4,5,5'-Hexachloro | 39635-35-3 | NA | 2.0, 15.8 | NA | NA |
| 160 | 2,3,3',4,5,6-Hexachloro | 41411-62-5 | NA | 2.0, 21.7 | NA | NA |
| 161 | 2,3,3',4,5',6-Hexachloro | 74472-43-8 | NA | 28.9 | NA | NA |
| 162 | 2,3,3',4',5,5'-Hexachloro | 39635-34-2 | NA | 13.3 | NA | NA |
| 163 | 2,3,3',4',5,6-Hexachloro | 74472-44-9 | NA | 1.5, 16.8 | NA | NA |
| 164 | 2,3,3',4',5',6-Hexachloro | 74472-45-0 | NA | 17.9 | NA | NA |
| 165 | 2,3,3',5,5',6-Hexachloro | 74472-46-1 | NA | 2.9, 27.9 | NA | NA |
| 166 | 2,3,4,4',5,6-Hexachloro | 41411-63-6 | NA | 18.7 | NA | NA |
| 167 | 2,3',4,4',5,5'-Hexachloro | 52663-72-6 | NA | 11.1 | NA | NA |
| 168 | 2,3',4,4',5',6-Hexachloro | 59291-65-5 | NA | 28.0 | NA | NA |
| 169 | 3,3',4,4',5,5'-Hexachloro | 32774-16-6 | NA | 6.60 | 1.57 - 5.98 | 6.64 - 7.55 |
| 170 | 2,2',3,3',4,4',5-Heptachloro | 35065-30-6 | NA | 0.91 - 20.84 | 0.91 - 19.25 | 7.03 - 7.27 |
| 171 | 2,2',3,3',4,4',6-Heptachloro | 52663-71-5 | NA | 17.5 | 5.40 - 29.79 | 6.68 - 9.03 |
| 172 | 2,2',3,3',4,5,5'-Heptachloro | 52663-74-8 | NA | 1.3, 12.1 | NA | NA |
| 173 | 2,2',3,3',4,5,6-Heptachloro | 68194-16-1 | NA | 1.4, 18.5 | NA | NA |
| 174 | 2,2',3,3',4,5,6'-Heptachloro | 38411-25-5 | NA | 1.4, 17.1 | NA | NA |
| 175 | 2,2',3,3',4,5',6-Heptachloro | 40186-70-7 | NA | 22.6 | NA | NA |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 176 | 2,2',3,3',4,6,6'-Heptachloro | 52663-65-7 | NA | 30.1 | NA | NA |
| 177 | 2,2',3,3',4',5,6-Heptachloro | 52663-70-4 | NA | 16.5 | NA | NA |
| 178 | 2,2',3,3',5,5',6-Heptachloro | 52663-67-9 | NA | 2.3, 21.6 | NA | NA |
| 179 | 2,2',3,3',5,6,6'-Heptachloro | 52663-64-6 | NA | 2.4, 27.9 | NA | NA |
| 180 | 2,2',3,4,4',5,5'-Heptachloro | 35065-29-3 | NA | 1.01 - 37.0 | 1.013 - 102 | 6.70 - 7.36 |
| 181 | 2,2',3,4,4',5,6-Heptachloro | 74472-47-2 | NA | 23.3 | NA | NA |
| 182 | 2,2',3,4,4',5,6'-Heptachloro | 60145-23-5 | NA | 26.0 | NA | NA |
| 183 | 2,2',3,4,4',5',6-Heptachloro | 52663-69-1 | NA | 20.4 | NA | NA |
| 184 | 2,2',3,4,4',6,6'-Heptachloro | 74472-48-3 | NA | 46.4 | NA | NA |
| 185 | 2,2',3,4,5,5',6- Heptachloro | 52712-05-7 | NA | 1.6, 21.7 | 1.62 - 46.0 | 6.99 - 8.94 |
| 186 | 2,2',3,4,5,6,6'-Heptachloro | 74472-49-4 | NA | 37.3 | NA | NA |
| 187 | 2,2',3,4',5,5',6-Heptachloro | 52663-68-0 | NA | 20.5 - 62.2 | 8.41 - 42.15 | 6.92 - 7.19 |
| 188 | 2,2',3,4',5,6,6'-Heptachloro | 74487-85-7 | NA | 44.9 - 113 | NA | NA |
| 189 | 2,3,3',4,4',5,5'-Heptachloro | 39635-31-9 | NA | 6.74 | NA | NA |
| 190 | 2,3,3',4,4',5,6-Heptachloro | 41411-64-7 | NA | 11.4 | NA | NA |
| 191 | 2,3,3',4,4',5',6-Heptachloro | 74472-50-7 | NA | 13.5 | NA | NA |
| 192 | 2,3,3',4,5,5',6-Heptachloro | 74472-51-8 | NA | 19.4 | NA | NA |
| 193 | 2,3,3',4',5,5',6-Heptachloro | 69782-91-8 | NA | 13.6 | NA | NA |
| 194 | 2,2',3,3',4,4',5,5'-Octachloro | 35694-08-7 | NA | 1.0, 6.79 | 10.13 - 47.52 | 6.90 - 9.69 |
| 195 | 2,2',3,3',4,4',5,6-Octachloro | 52663-78-2 | NA | 1.1 - 14.13 | NA | NA |
| 196 | 2,2',3,3',4,4',5,6'-Octachloro | 42740-50-1 | NA | 1.0, 13.2 | NA | NA |
| 197 | 2,2',3,3',4,4',6,6'-Octachloro | 33091-17-7 | NA | 25.7 | NA | NA |
| 198 | 2,2',3,3',4,5,5',6-Octachloro | 68194-17-2 | NA | 1.4, 15.6 | NA | NA |
| 199 | 2,2',3,3',4,5,5',6'-Octachloro | 52663-75-9 | NA | 1.0, 23.0 | NA | NA |
| 200 | 2,2',3,3',4,5,6,6'-Octachloro | 52663-73-7 | NA | 24.4 | NA | NA |

Table E.2.  (Cont.).

| No. | Chemical | CAS No.c | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hd (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- | --- |
| 201 | 2,2',3,3',4,5',6,6'-Octachloro | 40186-71-8 | NA | 1.7 - 95.8 | NA | NA |
| 202 | 2,2',3,3',5,5',6,6'-Octachloro | 2136-99-4 | NA | 1.8, 22.6 | 1.82 - 75.79 | 7.10 - 9.77 |
| 203 | 2,2',3,4,4',5,5',6-Octachloro | 52663-76-0 | NA | 14.2 | NA | NA |
| 204 | 2,2',3,4,4',5,6,6'-Octachloro | 74472-52-9 | NA | 34.9 | NA | NA |
| 205 | 2,3,3',4,4',5,5',6-Octachloro | 74472-53-0 | NA | 8.85 | NA | NA |
| 206 | 2,2',3,3',4,4',5,5',6-Nonachloro | 40186-72-9 | NA | 8.85 | 27.66 | 7.20 - 10.44 |
| 207 | 2,2',3,3',4,4',5,6,6'-Nonachloro | 52663-79-3 | NA | 17.1 | 35.97 | 7.52 - 7.94 |
| 208 | 2,2',3,3',4,5,5',6,6'-Nonachloro | 52663-77-1 | NA | 16.9 | 32.53 | 7.60 - 10.45 |
| 209 | 2,2',3,3',4,4',5,5',6,6'-Decachloro | 2051-24-3 | NA | 11.4 | 12.46 - 100 | 7.85 - 11.20 |

aSander (1999)

bMackay et al. (1992a and b, 1993, 1995)

cChemical Abstract Service numbers from Erickson (1997)

dBrunner et al. (1990); Dunnivant et al. (1992); Bamford et al. (2000)

eNA, not available

Table E.3.  Range of reported values of H and log Kow for dioxins at 293-298 K.

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 1 | Dibenzo-p-dioxin | 262-23-4 | NAc | 12.29, 12.39 | 3.40 - 4.70 |
| 2 | 1-Chloro-dibenzo-p-dioxin | 39227-53-7 | NA | 8.38 | 4.20 - 5.74 |
| 3 | 2-Chloro-dibenzo-p-dioxin | 39227-54-8 | NA | 14.82 | 4.20 - 5.71 |
| 4 | 2,3-Dichloro-dibenzo-p-dioxin | 29446-15-9 | NA | 6.61 | 4.70 - 6.23 |
| 5 | 2,7-Dichloro-dibenzo-p-dioxin | 33857-26-0 | 6.0, 8.4 | 8.11 | 4.70 - 6.72 |
| 6 | 2,8-Dichloro-dibenzo-p-dioxin | 38964-22-6 | 3.6, 3.9 | 2.13 | 4.70 - 5.60 |
| 7 | 1,2,4-Trichlorodibenzo-p-dioxin | 39227-58-2 | NA | 3.84 | 5.10 - 7.77 |
| 8 | 1,2,3,4-Tetrachlorodibenzo-p-dioxin | 30756-58-8 | 2.0, 3.8 | 3.77 | 5.50 - 8.97 |
| 9 | 1,2,3,7-Tetrachlorodibenzo-p-dioxin | 67028-18-6 | NA | 0.77 | 5.50 - 8.81 |
| 10 | 1,3,6,8-Tetrachlorodibenzo-p-dioxin | 30746-58-8 | NA | 0.71, 6.90 | 5.50 - 9.43 |
| 11 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin | 1746-01-6 | NA | 0.0021-10.34 | 5.38 - 8.93 |
| 12 | 1,2,3,4,7-Pentachlorodibenzo-p-dioxin | 39227-61-7 | NA | 0.264 | 6.60 - 10.05 |
| 13 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin | 39227-26-8 | NA | 4.52 | 7.30 - 10.89 |
| 14 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin | 35822-46-9 | NA | 0.133 | 7.92 - 11.98 |
| 15 | Octachlorodibenzo-p-dioxin | 3268-87-9 | NA | 0.683 | 7.33 - 13.08 |

aSander (1999).

bMackay et al. (1992a and b, 1993, 1995)

cNA, not available

Table E.4.  Range of reported values of H and log Kow for dibenzofurans at 293-298 K.

| No. | Chemical | CAS No. | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- |
| 1 | Dibenzofuran | 132-64-9 | NAc | 3.86 - 4.57 |
| 2 | 2,8-Dichlorodibenzofuran | 5409-83-6 | NA | 4.91 - 6.16 |
| 3 | 2,3,7,8-Tetrachlorodibenzofuran | 51207-31-9 | 1.50, 1.70 | 5.82 - 7.70 |
| 4 | 2,3,4,7,8-Pentachlorodibenzofuran | 51207-31-4 | NA | 6.92 - 7.82 |
| 5 | 1,2,3,4,7,8-Hexachlorodibenzofuran | 70658-26-9 | NA | 7.70 |
| 6 | 1,2,3,6,7,8-Hexachlorodibenzofuran | 57117-44-9 | NA | NA |
| 7 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran | 67462-39-4 | NA | 7.90 - 9.25 |
| 8 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran | 55673-89-7 | NA | 6.90 |
| 9 | Octachlorodibenzofuran | 39001-02-0 | 0.10 | 7.05 - 13.93 |

bMackay et al. (1992a and b, 1993, 1995)

cNA, not available

Table E.5.  Range of reported values of H and log Kow for polycyclic organic matter (POM) at 293-298 K.

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 1 | Indan | 496-11-7 | NAc | NA | 3.30 - 3.57 |
| 2 | Naphthalene | 91-20-3 | 42 - 130 | 29.2 - 123.95 | 3.01 - 4.70 |
| 3 | 1-Methylnaphthalene | 90-12-0 | 26 - 44 | 16.0 - 45.0 | 3.86 - 5.08 |
| 4 | 2-Methylnaphthalene | 91-57-6 | 41 - 2.0 x 104 | 20.265 - 58.75 | 3.70 - 5.08 |
| 5 | 1,2-Dimethylnaphthalene | 573-98-8 | NA | NA | NA |
| 6 | 1,3-Dimethylnaphthalene | 575-41-7 | 72 | NA | 4.36 - 4.55 |
| 7 | 1,4-Dimethylnaphthalene | 571-58-4 | 51 | NA | 4.36 - 4.44 |
| 8 | 1,5-Dimethylnaphtalene | 571-61-9 | 63 | 35.5 - 36.3 | 4.35 - 4.44 |
| 9 | 2,3-Dimethylnaphthalene | 581-40-8 | 60 | 38.92 - 92.16 | 4.18 - 4.44 |
| 10 | 2,6-Dimethylnaphthalene | 581-40-2 | 120 | 6.53 | 4.31 - 4.44 |
| 11 | 1-Ethylnaphthalene | 1127-76-0 | 38 | 14.8 - 36.47 | 3.87 - 4.42 |
| 12 | 2-Ethylnaphthalene | 939-27-5 | 63 , 84 | 82.2 | 4.38 - 4.43 |
| 13 | 1,4,5-Trimethylnaphthalene | 2131-41-1 | NA | 23.50 | 4.79 - 4.94 |
| 14 | Biphenyl | 92-52-4 | 28 - 84 | NA | NA |
| 15 | 4-Methylbiphenyl | 644-08-6 | NA | NA | 4.63 - 4.91 |
| 16 | 4,4'-Dimethylbiphenyl | 613-33-2 | NA | NA | 5.09 - 5.40 |
| 17 | Diphenylmethane | 101-81-5 | NA | NA | 4.14 - 4.36 |
| 18 | Bibenzyl | 103-29-7 | NA | NA | 3.67 - 4.82 |
| 19 | trans-Stilbene | 103-30-0 | NA | NA | 4.53 - 4.81 |
| 20 | Acenaphthene | 83-32-9 | 7.8 - 160 | 6.45 - 24.79 | 3.32 - 4.49 |
| 21 | Acenaphthylene | 208-96-8 | NA | 8.40 - 146.9 | 3.55 - 4.08 |
| 22 | Fluorene | 86-73-7 | 8.4 - 12 | 1.90 - 33.4 | 3.91 - 4.47 |
| 23 | 1-Methylfluorene | 1730-37-6 | NA | NA | 4.63 - 4.97 |
| 24 | Phenanthrene | 85-01-8 | 2.6 - 11 | 2.06 - 22.9 | 3.60 - 5.92 |
| 25 | 1-Methylphenanthrene | 832-69-6 | NA | NA | 5.14 |

Table E.5.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 26 | Anthracene | 120-12-7 | 1.8 - 72 | 1.80 - 110 | 3.45 - 4.79 |
| 27 | 9-Methylanthracene | 779-02-2 | NA | NA | 4.61 - 5.65 |
| 28 | 2-Methylanthracene | 613-12-7 | NA | NA | 5.14 - 5.15 |
| 29 | 9,10-Dimethylanthracene | 781-43-1 | NA | NA | 5.25 - 5.67 |
| 30 | Pyrene | 129-00-0 | 1.1, 1.2 | 0.517 - 12.4 | 4.45 - 6.70 |
| 31 | Fluoranthene | 206-44-0 | 0.92, 220 | 0.659 - 220 | 4.70 - 6.50 |
| 32 | Benzo[a]fluorene | 238-84-6 | NA | NA | 5.32 - 5.75 |
| 33 | Benzo[b]fluorene | 243-17-4 | NA | NA | 5.27 - 5.75 |
| 34 | Chrysene | 218-01-9 | NA | 0.065 - 0.45 | 5.01 - 7.10 |
| 35 | Triphenylene | 217-59-4 | NA | NA | 5.20 - 7.10 |
| 36 | p-Terphenyl | 92-94-4 | NA | NA | 5.88 - 6.03 |
| 37 | Naphthacene | 92-24-0 | NA | NA | 5.26 - 6.02 |
| 38 | Benz[a]anthracene | 56-55-3 | NA | 0.1013 - 5.17 | 5.48 - 7.50 |
| 39 | Benzo[b]fluoranthene | 205-99-2 | 6.8 x 10-2 | 0.051 - 3.55 | 5.78 - 6.61 |
| 40 | Benzo[j]fluoranthene | 205-82-3 | NA | NA | 6.40 - 6.44 |
| 41 | Benzo[k]fluoranthene | 207-08-9 | 6.0 x 10-2 | 0.016 - 7.50 | 5.86 - 7.20 |
| 42 | Benzo[a]pyrene | 50-32-8 | 4.6 x 10-2, 630 | 0.034 - 0.810 | 5.78 - 8.50 |
| 43 | Benzo[e]pyrene | 192-97-2 | NA | NA | 6.44 - 7.40 |
| 44 | Perylene | 198-55-0 | NA | 0.440 | 4.80 - 6.53 |
| 45 | 7,12-Dimethylbenz[a] anthracene | 57-97-6 | NA | NA | 5.80 - 6.95 |
| 46 | 9,10-Dimethylbenz[a] anthracene | 56-56-4 | NA | NA | 6.93 |
| 47 | 3-Methylcholanthrene | 56-49-5 | NA | NA | 6.42 - 7.11 |
| 48 | Benzo[ghi]perylene | 191-24-2 | 3.4 x 10-2 | 0.0146 - 0.709 | 5.95 - 7.60 |
| 49 | Pentacene | 135-48-8 | NA | NA | 7.19 |
| 50 | Dibenz[a,c]anthracene | 215-58-7 | NA | NA | NA |
| 51 | Dibenz[a,h]anthracene | 53-70-3 | NA | 0.0074, 0.0076 | 5.80 - 7.19 |
| 52 | Dibenz[a,j]anthracene | 58-70-3 | NA | NA | 7.11 - 7.19 |
| 53 | Coronene | 191-07-1 | NA | NA | 5.40 - 8.20 |

aSander (1999).

bMackay et al. (1992a and b, 1993, 1995)

cNA, not available

Table E.6.  Range of reported values of H and log Kow for coke oven emissions at 293-298 K – monoaromatic and phenolic compounds.

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 1 | Ethylbenzene | 100-41-4 | 600 - 920 | 653 - 1001 | 2.68 - 3.43 |
| 2 | 1,2,3-Trimethylbenzene | 526-73-8 | 330, 380 | 323 - 441 | 3.31 - 3.81 |
| 3 | 1,2,4-Trimethylbenzene | 95-63-6 | 560 - 680 | 385 - 1242 | 3.31 - 3.82 |
| 4 | 1,3,5-Trimethylbenzene | 108-67-8 | 600 - 840 | 600 - 1459 | 3.41 - 4.32 |
| 5 | n-Propylbenzene | 103-65-1 | 720 - 1.1 x 103 | 700 - 1175 | 3.42 - 3.90 |
| 6 | Isopropylbenzene | 98-82-8 | 130 - 1.5 x 103 | 909 - 1521 | 3.23 - 3.66 |
| 7 | 1-Ethyl-2-methylbenzene | 611-14-3 | 420, 440 | 427 - 565 | 3.53 - 3.63 |
| 8 | 1-Ethyl-3-methylbenzene | 620-14-4 | NAc | NA | NA |
| 9 | 1-Ethyl-4-methylbenzene | 622-96-8 | 510 | 498 | 3.63 |
| 10 | 1-Isopropyl-4-methylbenzene | 99-87-6 | 780 | 800 | 3.45 - 4.14 |
| 11 | n-Butylbenzene | 104-51-8 | 1.3 x 103 | 1300 - 1692 | 3.18 - 4.60 |
| 12 | Isobutylbenzene | 538-93-2 | 3.3 x 103 | 1160 - 3300 | 4.01 |
| 13 | sec-Butylbenzene | 135-98-8 | 1.2 x 103, 1.4 x 103 | 1400 - 1838 | NA |
| 14 | tert-Butylbenzene | 98-06-6 | 1.2 x 103 | 1200 - 1300 | 4.01 - 4.11 |
| 15 | 1,2,3,4-Tetramethybenzene | 48-23-3 | NA | NA | 3.84 - 5.02 |
| 16 | 1,2,3,5-Tetramethylbenzene | 527-53-7 | NA | NA | 4.04 - 5.02 |
| 17 | 1,2,4,5-Tetramethylbenzene | 95-93-2 | 2.5 x 103, 2.6 x 103 | 2540, 2552 | 2.80 - 4.24 |
| 18 | n-Pentylbenzene | 538-68-1 | 600, 1.7 x 103 | 600, 617 | 4.56 - 4.90 |
| 19 | Pentamethylbenzene | 538-61-1 | NA | NA | 4.56 - 4.63 |
| 20 | n-Hexylbenzene | 1077-16-3 | 2.2 x 103 | 1977 | 5.24 - 5.52 |
| 21 | Hexamethylbenzene | 87-85-4 | NA | NA | 4.02 - 5.16 |
| 22 | 2,3-Dimethylphenol | 526-75-0 | NA | NA | NA |
| 23 | 2,4-Dimethylphenol | 105-67-9 | 0.25, 530 | 0.0638 - 1.722 | 1.99 - 3.20 |
| 24 | 2,5-Dimethylphenol | 105-67-9 | NA | NA | NA |
| 25 | 2,6-Dimethylphenol | 576-26-1 | NA | 0.0193 - 0.684 | 2.07 - 2.51 |

Table E.6.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 26 | 3,4-Dimethylphenol | 95-65-8 | NA | 0.00942 - 0.02120 | 2.23 - 2.47 |
| 27 | 3,5-Dimethylphenol | 108-68-9 | NA | NA | NA |
| 28 | 2,3,5-Trimethylphenol | 697-82-5 | NA | 0.404 | 3.06 |
| 29 | 2,3,6-Trimethylphenol | 2416-94-6 | NA | NA | NA |
| 30 | 2,4,5-Trimethylphenol | 496-78-6 | NA | NA | NA |
| 31 | 2,4,6-Trimethylphenol | 527-60-6 | NA | 0.2512 | 2.73 |
| 32 | 3,4,5-Trimethylphenol | 527-54-8 | NA | NA | NA |
| 33 | 3-Methyl-5-Ethylphenol | NA | NA | NA | NA |
| 34 | 2-Ethylphenol | 90-00-6 | NA | 0.174 | 2.46 - 2.64 |
| 35 | 3-Ethylphenol | 620-17-7 | NA | NA | NA |
| 36 | 4-Ethylphenol | 123-07-9 | NA | 0.132 | 2.12 - 2.81 |
| 37 | 2-Propylphenol | 644-35-0 | NA | NA | NA |
| 38 | 4-n-Propylphenol | 645-56-7 | NA | NA | NA |
| 39 | 2-Isopropylphenol | 88-69-7 | NA | NA | NA |
| 40 | 4-Isopropylphenol | 99-89-8 | NA | NA | NA |
| 41 | 4-n-Butylphenol | 1638-22-8 | NA | NA | NA |
| 42 | 2-sec-Butylphenol | 89-72-5 | NA | NA | NA |
| 43 | 2-tert-Butylphenol | 88-18-6 | NA | NA | NA |
| 44 | 3-tert-Butylphenol | 585-34-2 | NA | NA | NA |
| 45 | 4-sec-Butylphenol | 99-71-8 | NA | NA | NA |
| 46 | 4-tert-Butylphenol | 98-54-4 | 0.11 | 0.113 - 3.750 | 2.94 - 3.41 |
| 47 | 2-Methyl-5-tert-Butylphenol | NA | NA | NA | NA |
| 48 | 2,6-Di-sec-Butylphenol | 128-39-2 | NA | NA | NA |
| 49 | 3,5-Di-tert-Butylphenol | 1138-52-9 | NA | NA | NA |
| 50 | 2,6-Di-t-butyl-4-Methylphenol | 128-37-0 | NA | NA | NA |

Table E.6.  (Cont.).

| No. | Chemical | CAS No. | Henry's Law constant, Ha (Pa m3 mol-1) | Henry's Law constant, Hb (Pa m3 mol-1) | log Kowb |
| --- | --- | --- | --- | --- | --- |
| 51 | 3-Pentylphenol | NA | NA | NA | NA |
| 52 | 4-Hexylphenol | 2446-69-7 | NA | NA | NA |
| 53 | 2-Heptylphenol | NA | NA | NA | NA |
| 54 | 4-Heptylphenol | 1987-50-4 | NA | NA | NA |
| 55 | 4-Octylphenol | 1806-26-4 | NA | 0.4916 | 3.70, 4.12 |
| 56 | 4-tert-Octylphenol | 140-66-9 | NA | NA | NA |
| 57 | 4-Nonylphenol | 104-40-5 | NA | 1.5705 | 4.10 - 6.36 |
| 58 | 1-Naphthol | 90-15-3 | NA | 0.49 | 2.28 - 3.13 |
| 59 | 2-Naphthol | 135-19-3 | NA | 0.280 | 2.01 - 3.46 |
| 60 | 2-Phenylphenol | 90-43-7 | NA | NA | 3.09 - 3.36 |
| 61 | 3-Phenylphenol | 588-51-8 | NA | NA | NA |
| 62 | 4-Phenylphenol | 92-69-3 | NA | NA | 2.88 - 3.63 |
| 63 | 2-Allylphenol | 1745-81-9 | NA | NA | NA |

aSander (1999).

bMackay et al. (1992a and b, 1993, 1995)

cNA, not available