Appendix F:

Estimates of Gas-phase Lifetimes for Selected Substances

Table F.1.	Estimates of gas-phase lifetimes for selected substances.

| No. | Chemical | CAS No. | Gas-Phase Removal Process | kOH (cm3 molecule-1 s-1) | kNO3 (cm3 molecule-1 s-1) | kO3 (cm3 molecule-1 s-1) | Reported Gas-Phase Lifetime (days) | Calculated Gas-Phase Hal- Life (days) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Acetaldehyde | 75-07-0 | OH, photolysis | 15.8 x 10-12 | - | - | <1 | 1.06 |
| 6 | Acrolein | 107-02-8 | OH, photolysis, O3 | 19.9 x 10-12 | - | 3.0 x 10-14 | <1 | 32 sec |
| 9 | Acrylonitrile | 107-13-1 | OH | - | - | - | 1-5 to >5 | - |
| 10 | Allyl chloride | 107-05-1 |  | - | - | 1.60 x 10-18 | - | - |
| 12 | Aniline | 62-53-3 |  | - | - | 1.12 x 10-18 | - | - |
| 14 | Benzene | 71-43-2 | OH, NO3 | 1.23 x 10-12 | - | 1.7 x 10-22 | >5 | 14 |
| 17 | Benzyl chloride | 100-44-7 |  | - | - | <6 x 10-20 | - | - |
| 22 | 1,3-Butadiene | 106-99-0 | OH, O3 | 66.6 x 10-12 | - | 6.3 x 10-18 | <1 | 0.22 |
| 27 | Carbon tetrachloride | 56-23-5 | Photolysis (in stratosphere) | - | - | - | - | - |
| 37 | Chloroform | 67-66-3 | OH | - | - | - | >5 | - |
| 40 | o-Cresol | 95-48-7 |  | 4.2 x 10-11 | 1.37 x 10-11 | 2.6 x 10-19 | - | - |
| 41 | m-Cresol | 108-39-4 |  | 6.4 x 10-11 | 9.74 x 10-12 | 1.9 x 10-19 | - | - |
| 42 | p-Cresol | 106-44-5 |  | 4.7 x 10-11 | 1.07 x 10-11 | 4.7 x 10-19 | - | - |
| 44 | Cumene | 98-82-8 |  | 6.5 x 10-12 | - | - | - | - |
| 54 | 1,3-Dichloropropene | 542-75-6 | OH, O3 | - | - | - | <1 | - |
| 57 | Diethyl sulfate | 64-67-5 |  | 1.81 x 10-12 | - | - | - | - |
| 61 | N,N-Dimethylaniline | 121-69-7 |  | - | - | 9.1 x 10-18 | - | - |
| 67 | Dimethyl sulfate | 77-78-1 |  | <0.5 x 10-12 | - | - | - | - |
| 74 | 1,2-Epoxybutane | 106-88-7 |  | 1.91 x 10-12 | - | - | - | - |
| 76 | Ethylbenzene | 100-41-4 |  | 7.1 x 10-12 | - | - | - | - |
| 78 | Ethyl chloride | 75-00-3 |  | 40.8 x 10-14 | - | - | - | - |
| 79 | Ethylene dibromide | 106-93-4 |  | 22.5 x 10-14 | - | - | - | 74 |
| 80 | Ethylene dichloride | 107-06-2 |  | 26.4 x 10-14 | - | - | - | 63 |
| 84 | Ethylene oxide | 75-21-8 | OH | - | - | - | >5 | - |
| 87 | Formaldehyde | 50-00-0 | Photolysis, OH | 9.37 x 10-12 | - | - | <1 | 1.8 |
| 89 | Hexachlorobenzene | 118-74-1 | OH | - | - | - | >5 | - |
| 97 | Hexane | 110-54-3 | - | 5.61 x 10-12 | - | - | - | - |

Table F.1.  (Cont.).

| No. | Chemical | CAS No. | Gas-Phase Removal Process | kOH (cm3 molecule-1 s-1) | kNO3 (cm3 molecule-1 s-1) | kO3 (cm3 molecule-1 s-1) | Reported Gas-Phase Lifetime (days) | Calculated Gas-Phase Hal- Life (days) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 98 | Hydrazine | 302-01-2 | OH, O3 | - | - | - | <1 | - |
| 102 | Methanol | 67-56-1 |  | 0.944 x 10-12 | - | - | - | - |
| 105 | Methyl chloride | 74-87-3 |  | 5.3 x 10-14 | - | - | - | - |
| 106 | Methyl chloroform | 71-55-6 |  | 1.09 x 10-14 | - | - | - | - |
| 107 | Methyl ethyl ketone | 78-93-3 |  | 1.15 x 10-12 | - | - | - | - |
| 109 | Methyl iodide | 74-88-4 |  | 7.2 x 10-14 | - | - | - | - |
| 110 | Methyl isobutyl ketone | 108-10-1 |  | 14.1 x 10-12 | - | - | - | - |
| 113 | Methyl tert-butyl ether | 1634-04-4 |  | 2.94 x 10-12 | - | - | - | - |
| 115 | Methylene chloride | 75-09-2 | OH | 14.7 x 10-14 | ≤5.8 x 10-18 | - | >5 | 114 |
| 118 | Naphthalene | 91-20-3 |  | - | - | <3 x 10-19 | - | - |
| 119 | Nitrobenzene | 98-95-3 |  | - | - | <7 x 10-21 | - | - |
| 129 | Phenol | 108-95-2 |  | 2.63 x 10-11 | 3.92 x 10-12 | - | - | - |
| 131 | Phosgene | 75-44-5 |  | <0.1 x 10-14 | - | - | - | - |
| 136 | Propionaldehyde | 123-38-6 |  | 19.6 x 10-12 | - | - | - | - |
| 141 | Quinoline | 91-22-5 | OH | - | - | - | 1 to 5 | - |
| 143 | Styrene | 100-42-5 |  | 5.8 x 10-11 | 1.5 x 10-13 | 1.71 x 10-17 | - | - |
| 146 | 1,1,2,2-Tetrachloroethane | 79-34-5 | OH | 22.6 x 10-14 | - | - | >5 | 74 |
| 147 | Tetrachloroethylene | 127-18-4 | OH | 1.73 x 10-13 | <1.8 x 10-16 | ~2.8 x 10-20 | >5 | 97 |
| 148 | Toluene | 108-88-3 |  | 5.96 x 10-12 | 0.68 x 10-16 | 4.1 x 10-22 | - | 2.80 |
| 154 | 1,1,2-Trichloroethane | 79-00-5 |  | 19.6 x 10-14 | - | - | - | - |
| 155 | Trichloroethylene | 79-01-6 | OH | 1.76 x 10-12 | - | - | >5 | 9.5 |
| 160 | 2,2,4-Trimethylpentane | 540-84-1 |  | 3.59 x 10-12 | - | - | - | - |
| 163 | Vinyl chloride | 75-01-4 | OH, O3 | 7.55 x 10-12 | - | - | <1 to 1-5 | 2.2 |
| 164 | Vinylidene chloride | 75-35-4 |  | 11.2 x 10-12 | - | - | - | - |
| 165 | o-Xylene | 95-47-6 |  | 13.7 x 10-12 | 3.8 x 10-16 | 1.7 x 10-21 | - | 1.22 |
| 166 | m-Xylene | 108-38-3 |  | 23.6 x 10-12 | 2.33 x 10-16 | 8.5 x 10-22 | - | 0.71 |
| 167 | p-Xylene | 106-42-3 |  | 14.3 x 10-12 | 4.5 x 10-16 | 1.4 x 10-21 | - | 1.17 |