2016 Nonpoint Oil and Gas Emission Estimation Tool Version 1.0

Prepared for:

U.S. Environmental Protection Agency

U.S. Environmental Protection Agency

109 T.W. Alexander Drive

Mail Code C339-02

Research Triangle Park, NC 27711

Prepared by:

Eastern Research Group, Inc.

1600 Perimeter Park

Suite 200

Morrisville, NC 27560

December 31, 2018

EPA Contract No. EP-D-14-030

Delivery Order 00-57

1.0	Introduction	1

1.1	The National Emissions Inventory (NEI)	1

1.2	Nonpoint Oil and Gas Emission Estimation Tool	1

2.0	Background on Development of the Tool	7

2.1	Activity Data	8

2.1.1	HPDI and RIGDATA	11

2.1.2	State-Provided Data	15

2.1.3	State Oil and Gas Commission Websites	15

2.1.4	National Production Summary	16

2.2	Process Characterization Data	19

2.3	Updates Since 2014	22

3.0	Source Category Emission Estimation Methodologies	26

3.1	Artificial Lifts	27

3.2	Associated Gas Venting and Flaring	29

3.3	Condensate Tanks	32

3.4	Crude Oil Tanks	36

3.5	Dehydrators	40

3.6	Drilling Rigs	44

3.7	Fugitive Leaks	49

3.8	Gas-Actuated Pumps	52

3.9	Heaters	56

3.10	Hydraulic Fracturing Pumps	59

3.11	Lateral/Gathering Compressor Engines	62

3.12	Liquids Unloading	64

3.13	Loading	70

3.14	Mud Degassing	74

3.15	Pneumatic Devices	77

3.16	Produced Water Tanks	81

3.17	Well Completions	84

3.18	Wellhead Compressor Engines	88

4.0	Tool Nonpoint Oil and Gas Emissions Summary	91

Appendix A – Instructions for Using the EPA Nonpoint Oil and Gas Emission Estimation Tool, Exploration Module (12/1/2018)

Appendix B– Instructions for Using the EPA Nonpoint Oil and Gas Emission Estimation Tool, Production Module (12/1/2018)

Appendix C – US Oil and Gas Basins (Found in the “National Oil and Gas Tool Report Appendix C – Data Element Dictionary.xlsx” file)

Appendix D – Data Element Dictionary (Found in the “National Oil and Gas Tool Report Appendix D – US Oil and Gas Basins.xlsx” file)

LIST OF TABLES

Page

Table 1-1. SCC Listing	3

Table 2-1. Activity Parameters Needed to Estimate Emissions	8

Table 2-2. Activity Parameter Data Sources by State	9

Table 2-3. HPDI Data Coverage by State	11

Table 2-4. Oil and Gas Production by State	17

Table 2-5. Oil and Gas Basins Adjacent to CenSARA States	21

Table 2-6. Tool Data Sources by State and Source Type	23

Table 3-1. Emission Sources by Well Type	26

Table 3-2. Liquids Unloading Vent Rates from the U.S. GHG Inventory	68

Table 3-3. Default Liquids Unloading Vent Rates for the Tool	68

Table 3-4. National Default Emissions Factors for Mud Degassing by Mud Base	74

Table 3-5. Whole Gas Bleed Rates for Pneumatic Devices	79

Table 3-6. Pneumatic Device Counts for Oil and Gas Wells	80

Table 4-1. State-wide Tool Emissions Estimates	91

Table 4-2. Source Category Tool Emissions Estimates	92

LIST OF FIGURES

Page

Figure 2-1. Oil and Gas Basins Covered by the CenSARA Study	19

Figure 2-2. Oil and Gas Basins as Defined by the Geologic Provinces Published by the AAPG	20

Figure 3-1. Artificial Lift Engine	27

Figure 3-2. Liquid Storage Tanks	32

Figure 3-3. Permian Basin Tank Battery	37

Figure 3-4. Dehydrator	40

Figure 3-5. Drilling Rig	45

Figure 3-6. Flanges	49

Figure 3-7. Line Heater	57

Figure 3-8. Hydraulic Fracturing	60

Figure 3-9. Lateral Compressor Engine	62

Figure 3-10. Plunger Lifts	65

Figure 3-11. EIA Supply Region Map	67

Figure 3-12. Truck Loading Operations	71

Figure 3-13. Pneumatic Valve	78

Figure 3-14. Produced Water Tanks	81

Figure 3-15. Well Completion	84

Figure 3-16. Wellhead Compressor Engines	88

Figure 4-1. Tool Nonpoint Oil and Gas VOC Emissions	93

Figure 4-2. Tool Nonpoint Oil and Gas NOx Emissions	94

# Introduction

## The National Emissions Inventory (NEI)

The U.S. Environmental Protection Agency’s (EPA) Emission Inventory and Analysis Group (EIAG) produces the National Emission Inventory (NEI) for criteria and hazardous air pollutants (HAPs) every three years. The NEI is a comprehensive and detailed estimate of air emissions of both criteria and HAP from all air emissions sources, including both stationary (e.g. power plants and petroleum refineries) and mobile (e.g. automobiles and aircraft) sources. The NEI is prepared by the U.S. EPA based primarily upon emission estimates and emission model inputs provided by State, Local, and Tribal air agencies for sources in their jurisdictions, and supplemented by data developed by the U.S. EPA. These data are needed for a variety of reasons, including modeling demonstrations, regulatory analyses, and to produce the National Air Pollutant Emission Trends report.

Emissions from stationary sources can be divided into two sectors: point sources and nonpoint sources (nonpoint sources are sometimes referred to as area sources). The NEI point sources emissions inventory contains emissions estimates for sources that are individually inventoried and usually located at a fixed, stationary location, although portable sources such as some asphalt or rock crushing operations are also included. Point sources include large industrial facilities and electric power plants, but also increasingly include many smaller industrial and commercial facilities, such as dry cleaners and gas stations, which have traditionally been included as nonpoint sources.

The NEI nonpoint sources emissions inventory includes emission sources which individually are too small in magnitude or too numerous to inventory as individual point sources, and which can often be estimated more accurately as a single aggregate source for a County or Tribal area. Examples of nonpoint source categories are residential heating and consumer solvent use.

The 2011 and 2014 NEI and supporting documentation is available on-line at https://www.epa.gov/air-emissions-inventories/national-emissions-inventory-nei.

## Nonpoint Oil and Gas Emission Estimation Tool

Nonpoint source emissions from the oil and gas exploration and production sector have gained interest in recent years in the United States as drilling technology has allowed development of unconventional oil and gas plays in areas where there was previously no activity, or where activity had subsided after depletion of the conventional reserves. For example, the areas in and around the Barnett, Haynesville, and Eagle Ford Shales in Texas; the Marcellus Shale in Ohio, Pennsylvania, and West Virginia; and the Bakken Shale/Williston Basin in North Dakota and Montana have all experienced a rapid expansion in activity over the last twelve years. These are referred to as “unconventional” oil and gas plays as the resource must be stimulated through high-pressure, high-volume hydraulic fracturing to release the oil and gas trapped in the source formation (such as shale or tight sands). In this tool, these types of wells are assumed to be have been hydraulically fractured when completed, and emissions from the hydraulic fracturing pump engines are included as a discrete source type (see Section 3.10).

While the major emissions sources associated with oil and gas collection, processing, and distribution have traditionally been included in the NEI as point sources (e.g. gas processing plants, pipeline compressor stations, and refineries), the activities occurring “upstream” of these types of facilities have not been as well characterized in the NEI. In this report, upstream activities refer to emission units and processes associated with the exploration and drilling of oil and gas wells, and the equipment used at the wellsite to then extract the product from the well and deliver it “downstream” to a central collection point or processing facility. The types of unit processes found at upstream sites include separators, dehydrators, storage tanks, and compressor engines.

The NEI nonpoint oil and gas emissions inventory is primarily developed using data supplied to EPA by state air agencies. Where state data is not supplied to EPA, EPA populates the NEI with the best available data. In the case of nonpoint oil and gas emissions estimates, EPA has developed the 2016 Nonpoint Oil and Gas Emission Estimation Tool (the “tool”) described in this report to estimate emissions from this category. The tool is an Access database that utilizes county-level activity data (e.g. oil production and well counts), operational characteristics (types and sizes of equipment), and emission factors to estimate emissions.

The emission estimates generated by the tool are only used in the NEI if state data is not available. Where state data is available but does not include HAP, EPA estimates HAPs based on their ratios to VOC or PM in gas composition profiles, and adds them to the NEI. The HAP augmentation procedure is described in detail in the documentation for the 2014 NEI (https://www.epa.gov/air-emissions-inventories/2014-national-emissions-inventory-nei-data).

This report describes the technical approach used to develop the tool to characterize emissions from nonpoint oil and gas exploration and production sources for the year 2014. The tool generates estimates of emissions of oxides of nitrogen (NOx), volatile organic compounds (VOC), particulate matter (PM), carbon monoxide (CO), ammonia (NH3), sulfur dioxide (SO2), HAPs, and hydrogen sulfide (H2S) from upstream oil and gas production activities. Specific source categories included in the tool are:

- Artificial Lift Engines

- Associated Gas Venting

- Condensate Tanks

- Crude Oil Tanks

- Dehydrators

- Drilling Rigs

- Fugitive Emissions

- Gas-Actuated Pneumatic Pumps

- Heaters

- Hydraulic Fracturing Pumps

- Lateral Compressor Engines

- Liquids Unloading

- Hydrocarbon Liquids Loading

- Mud Degassing

- Pneumatic Devices

- Produced Water Tanks

- Well Completion Venting

- Wellhead Compressor Engines

- Flaring (when used to control emissions from the unit processes listed above)

Many of the source categories covered by the tool are further sub-divided into distinct source classification codes (SCCs) specific to either a well type (gas or oil), a sub-category of the broader equipment type (such as fugitive emissions from connectors and fugitive emissions from valves), or some other distinction. Table 1-1 presents a complete listing of the SCCs covered by the tool for each of the source categories listed above.

| Table 1-1. SCC Listing |  |  |
| --- | --- | --- |
| Source Category | SCC | SCC Description |
| Artificial Lift Engines | 2310000330 | Oil & Gas Expl & Prod /All Processes /Artificial Lift |
| Associated Gas Venting | 2310011000 | On Shore Crude Oil Production All Processes |
| Condensate Tanks | 2310021010 | On-Shore Gas Production /Storage Tanks: Condensate |
| Condensate Tanks | 2310023010 | On-Shore CBM Production /Storage Tanks: Condensate |
| Crude Oil Tanks | 2310010200 | Oil & Gas Expl & Prod /Crude Petroleum /Oil Well Tanks - Flashing & Standing/Working/Breathing |
| Dehydrators | 2310021400 | On-Shore Gas Production Dehydrators |
| Dehydrators | 2310023400 | Coal Bed Methane NG / Dehydrators |
| Drilling Rigs | 2310000220 | Oil And Gas Exploration Drill Rigs |
| Fugitive Emissions | 2310011501 | On-Shore Oil Production /Fugitives: Connectors |
| Fugitive Emissions | 2310011502 | On-Shore Oil Production /Fugitives: Flanges |
| Fugitive Emissions | 2310011503 | On-Shore Oil Production /Fugitives: Open Ended Lines |
| Fugitive Emissions | 2310011505 | On-Shore Oil Production /Fugitives: Valves |
| Fugitive Emissions | 2310021501 | On-Shore Gas Production /Fugitives: Connectors |
| Fugitive Emissions | 2310021502 | On-Shore Gas Production /Fugitives: Flanges |
| Fugitive Emissions | 2310021503 | On-Shore Gas Production /Fugitives: Open Ended Lines |
| Fugitive Emissions | 2310021505 | On-Shore Gas Production /Fugitives: Valves |
| Fugitive Emissions | 2310021506 | On-Shore Gas Production /Fugitives: Other |
| Fugitive Emissions | 2310023511 | On-Shore CBM Production /Fugitives: Connectors |
| Fugitive Emissions | 2310023512 | On-Shore CBM Production /Fugitives: Flanges |
| Fugitive Emissions | 2310023513 | On-Shore CBM Production /Fugitives: Open Ended Lines |
| Fugitive Emissions | 2310023515 | On-Shore CBM Production /Fugitives: Valves |
| Fugitive Emissions | 2310023516 | On-Shore CBM Production /Fugitives: Other |
| Gas-Actuated Pumps | 2310023310 | Coal Bed Methane NG / Pneumatic Pumps |
| Gas-Actuated Pumps | 2310111401 | On-Shore Oil Exploration /Oil Well Pneumatic Pumps |
| Gas-Actuated Pumps | 2310121401 | On-Shore Gas Exploration: Gas Well Pneumatic Pumps |
| Heaters | 2310010100 | On-Shore Oil Production /Heater Treater |
| Heaters | 2310021100 | On-Shore Gas Production /Gas Well Heaters |
| Heaters | 2310023100 | On-Shore CBM Production /CBM Well Heaters |
| Hydraulic Fracturing Pumps | 2310000660 | Oil & Gas Expl & Prod /All Processes /Hydraulic Fracturing Engines |
| Hydrocarbon Liquids Loading | 2310011201 | On-Shore Oil Production /Tank Truck/Railcar Loading: Crude Oil |
| Hydrocarbon Liquids Loading | 2310021030 | On-Shore Gas Production /Tank Truck/Railcar Loading: Condensate |
| Hydrocarbon Liquids Loading | 2310023030 | On-Shore CBM Production /Tank Truck/Railcar Loading: Condensate |
| Lateral Compressor Engines | 2310021251 | On-Shore Gas Production/Lateral Compressors 4 Cycle Lean Burn |
| Lateral Compressor Engines | 2310021351 | On-Shore Gas Production/Lateral Compressors 4 Cycle Rich Burn |
| Lateral Compressor Engines | 2310023251 | On-Shore CBM Production/Lateral Compressors 4 Cycle Lean Burn |
| Lateral Compressor Engines | 2310023351 | On-Shore CBM Production/Lateral Compressors 4 Cycle Rich Burn |
| Liquids Unloading | 2310021603 | On-Shore Gas Production / Gas Well Venting - Blowdowns |
| Liquids Unloading | 2310023603 | On-Shore CBM Production / CBM Well Venting - Blowdowns |
| Mud Degassing | 2310023606 | On-Shore CBM Exploration /Mud Degassing |
| Mud Degassing | 2310111100 | On-Shore Oil Exploration /Mud Degassing |
| Mud Degassing | 2310121100 | On-Shore Gas Exploration /Mud Degassing |
| Pneumatic Devices | 2310010300 | Oil Production Pneumatic Devices |
| Pneumatic Devices | 2310021300 | On-Shore Gas Production Pneumatic Devices |
| Pneumatic Devices | 2310023300 | On-Shore CBM Production Pneumatic Devices |
| Produced Water Tanks | 2310000550 | Produced Water |
| Well Completion Venting | 2310023600 | On-Shore CBM Exploration: CBM Well Completion: All Processes |
| Well Completion Venting | 2310111700 | On-Shore Oil Exploration: Oil Well Completion: All Processes |
| Well Completion Venting | 2310121700 | On-Shore Gas Exploration: Gas Well Completion: All Processes |
| Wellhead Compressor Engines | 2310021102 | On-Shore Gas Production /Natural Gas Fired 2Cycle Lean Burn Compressor Engines 50 To 499 HP |
| Wellhead Compressor Engines | 2310021202 | On-Shore Gas Production /Natural Gas Fired 4Cycle Lean Burn Compressor Engines 50 To 499 HP |
| Wellhead Compressor Engines | 2310021302 | On-Shore Gas Production /Natural Gas Fired 4Cycle Rich Burn Compressor Engines 50 To 499 HP |
| Wellhead Compressor Engines | 2310023102 | On-Shore CBM Production /CBM Fired 2Cycle Lean Burn Compressor Engines 50 To 499 HP |
| Wellhead Compressor Engines | 2310023202 | On-Shore CBM Production /CBM Fired 4Cycle Lean Burn Compressor Engines 50 To 499 HP |
| Wellhead Compressor Engines | 2310023302 | On-Shore CBM Production /CBM Fired 4Cycle Rich Burn Compressor Engines 50 To 499 HP |

It should be noted that these source categories do not represent a complete list of all emission sources or SCCs that may be found at upstream oil and gas exploration and production sites. However, the most significant nonpoint sources that contribute to emissions have been included. Sources that were not included due to limited data availability include: salt water injection engines, well pad construction equipment, workover equipment, water pumps at coalbed methane wells, and mobile sources. Associated on-road mobile sources operating in the field, such as service vehicles used during construction, drilling and production phases, may be included in some states’ mobile source emissions inventories but are not specifically included in the tool.

ERG developed the tool initially under EPA Contract No. EP-D-11-006, Work Assignment (WA) 2-05, followed on by subsequent WAs and this Delivery Order (DO). The purpose/objectives of the WAs/DO were the following:

1)	Develop a nonpoint methodology to estimate county-level emissions of criteria pollutants and HAP for the upstream oil and gas production sector for 2011, 2014, and 2016;

2)	Implement the methodology to develop county-level emissions inventories for this sector; and

3)	Develop a MS Access-based tool incorporating the methodologies and available information that may be used by EPA, states, and local agencies to develop state or region-specific emission inventories for the upstream oil and gas sector based on user supplied activity and emissions inputs.

The following describes how the information in this report is organized:

Section 2 – Background information on development of tool

Section 3 – Information on the methodology and emission estimation approach used for each source category

Section 4 – Summary of nonpoint oil and gas emission estimates generated by the tool

# Background on Development of the Tool

The tool was developed based on work that has been done in this sector over the last several years by various states, inter-governmental agencies, and EPA. These efforts include work done by the Texas Commission on Environmental Quality (TCEQ), the Western Regional Air Partnership (WRAP), and the Central States Air Resource Agencies (CenSARA) to develop improved nonpoint oil and gas emissions inventories.

In 2010, the seven CenSARA states (Texas, Louisiana, Oklahoma, Arkansas, Kansas, Missouri and Nebraska) had a combined oil production of approximately 611 million barrels and a combined gas production of 12.8 trillion cubic feet, representing 48% of total gas production and 31% of total oil production in the country, including both conventional and unconventional resource plays. As such, the CenSARA inventory effort covered a wide variety of processes and well types and was used as the starting point for the tool. In particular, the Excel-based emission estimation tool that was developed for the CenSARA study was used as the basis for initial development of the tool described in this report. Subsequent updates to the tool incorporated data from numerous additional sources, including the TCEQ and WRAP data mentioned above, related EPA inventory efforts, and data provided to EPA directly from state air agencies.

The basic methodology used to develop the CenSARA inventory was also used to develop the tool and consisted of the following steps:

- Compile activity data - Oil and gas activity data was obtained to include, but is not limited to, the number of active wells by well type, gas production and oil production, spud counts, feet drilled, and water production. The activity data for the tool was primarily obtained from DrillingInfo’s HPDI database, a commercial database that processes state-level oil and gas commission data into a comprehensive database of production statistics. Data used in this version of the tool is for the calendar year 2016 and is based on HPDI data as of June 2018. As described further in section 2.1, EPA uses other activity data that is not available in HPDI for certain states.

- Compile process characterization and emission factor data - To initially populate the tool, process characterization data and emissions factors from the CenSARA study were used for the counties in the CenSARA states, and an average of the data for the CenSARA basins were used for the remainder of the counties in the country. Under the CenSARA study, these data were developed or collected from a variety of sources including: 1) oil and gas operator surveys, 2) state minor source permit applications, and 3) literature review. Emission factors for combustion equipment has primarily been taken from AP-42. Much of the initial CenSARA process characterization data used to populate the tool database has since been replaced, as described below in more detail. For example, EPA GHG Reporting Program data (Subpart W) were used to develop default values for several categories, including condensate tanks, crude oil tanks, pneumatic devices, and heaters.

- Incorporate updated process characterization data - Several state and local air quality agencies and Regional Planning Organizations (RPOs) provided updates to replace the default CenSARA and EPA process characterization data. The tool database contains reference information identifying the source of all inputs into the estimates.

- Develop Access-based tool to house the inventory - A Microsoft Access-based tool that estimates 2016 nonpoint oil and gas emissions at the county level was then developed using the compiled activity and process characterization data. The tool has been programmed to be flexible and allow for user-specified inputs such that users may update activity and emissions data at the basin and/or county level for future use. Additional details on the tool and user’s instructions are included in Appendices A (Exploration Module) and B (Production Module).

## Activity Data

Activity data were obtained at the county level for the entire country to include the key activity parameters that affect emissions. These key activity factors include, but are not limited to, the number of active wells by well type, gas production and oil production by well type, spud count, estimated feet (depth) drilled by wellbore type, and water production by well type. Activity data for the 2016 base year were obtained from the HPDI database, RIGDATA, state oil and gas commission websites, and directly from state and local agencies involved in development and review of the tool.

Table 2-1 presents the activity data parameters used in the tool to calculate emissions.

Table 2-1. Activity Parameters Needed to Estimate Emissions

Data Parameter

Oil Production (barrels or BBL)

Natural Gas Production (thousand standard cubic feet or MCF)

CBM Production (thousand standard cubic feet or MCF)

Condensate Production (BBL)

Associated Gas Production (MCF)

Oil Well Counts

Natural Gas Well Counts

CBM Well Counts

Oil Well Completions (Conventional and Unconventional)

Natural Gas Well Completions (Conventional and Unconventional)

CBM Well Completions (Conventional and Unconventional)

Produced Water Production at Oil Wells (BBL)

Produced Water Production at Gas/CBM Wells (BBL)

Spud Counts (Vertical, Horizontal, Directional)

Feet Drilled (Vertical, Horizontal, Directional)

Table 2-2 presents the activity parameter data sources for each state for the data types identified in Table 2-1.

| Table 2-2. Activity Parameter Data Sources by State |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| State | Oil/ Associated Gas Production | Natural Gas/ Condensate Production | CBM Gas/ Condensate Production | Produced Water | Well Completions | Spud Counts/ Feet Drilled |
| Alabama | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Alaska | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission |
| Arizona | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission |
| Arkansas | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| California | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Colorado | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Florida | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI |
| Idaho | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission |
| Illinois | EIA/ Oil and Gas Commission | EIA/ Oil and Gas Commission | EIA/ Oil and Gas Commission | EIA/ Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission/ RIGDATA |
| Indiana | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission/ RIGDATA |
| Kansas | KSDEP | KSDEP | KSDEP | KSDEP | 2018 HPDI | KSDEP/ RIGDATA |
| Kentucky | Oil and Gas Commission | Oil and Gas Commission | Oil and Gas Commission | 2018 HPDI | Oil and Gas Commission | Oil and Gas Commission/ RIGDATA |
| Louisiana | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission/ RIGDATA |
| Maryland | EIA/ 2018 HPDI | EIA/ 2018 HPDI | EIA/ 2018 HPDI | EIA/ 2018 HPDI | 2018 HPDI | 2018 HPDI |
| Michigan | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission/ RIGDATA |
| Mississippi | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission/ RIGDATA |
| Missouri | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission |
| Montana | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Nebraska | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Nevada | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI and Oil and Gas Commission/ RIGDATA |
| New Mexico | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| New York | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ Oil and Gas Commission |
| North Dakota | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Ohio | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Oklahoma | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Oregon | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission |
| Pennsylvania | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission | 2018 HPDI | 2018 HPDI/ RIGDATA |
| South Dakota | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI |
| Tennessee | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | Oil and Gas Commission |
| Texas | TCEQ | TCEQ | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Utah | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |
| Virginia | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI |
| West Virginia | WVDEP | WVDEP | WVDEP | WVDEP/ 2018 HPDI | WVDEP | WVDEP |
| Wyoming | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI | 2018 HPDI/ RIGDATA |

### HPDI and RIGDATA

The primary data source for obtaining activity data was DrillingInfo’s HPDI database. This subscription-based information service extracts well-level data from state oil and gas commission websites and prepares it in a standardized format. As part of EPA’s Enforcement Activities, EPA has an annual subscription to DrillingInfo, allowing data downloads, or “refreshes,” to be obtained throughout the year. In accordance with the EPA’s licensing agreement, well-level data is proprietary, but derived products, such as aggregation at the county-level, are acceptable for public dissemination and use in the tool.

ERG extracted well identification (HPDIHeader) and production (HPDIProduction) information for onshore wells and leases. Table 2-3 provides details on the available data by state, as of the June 2018 refresh. Table 2-3 also includes the update frequency of the data by state and provides the date of the latest production data included in the June 2018 refresh.

| Table 2-3. HPDI Data Coverage by State |  |  |  |
| --- | --- | --- | --- |
| State Abbreviation | Production Group | Update Frequency | Latest Production Data |
| Alabama | Well | Monthly | March 2018 |
| Alaska | Well | Monthly | April 2018 |
| Arizona | Well | Monthly | January 2017 |
| Arkansas | Well | Monthly | March 2018 |
| California | Well | Monthly | February 2018 |
| Colorado | Well | Monthly | April 2018 |
| Florida | Well | Monthly | April 2018 |
| Kansas | Lease | Monthly | March 2018 |
| Kentucky | Well | Yearly | December 2015 |
| Louisiana | Well/Unita | Monthly | April 2018 |
| Maryland | Well | Semi-annually | December 2015 |
| Michigan | Lease | Monthly | February 2018 |
| Mississippi | Well | Monthly | March 2018 |
| Missouri | Well | Bi-Monthly | December 2017 |
| Montana | Well | Monthly | March 2018 |
| Nebraska | Well | Monthly | April 2018 |
| Nevada | Well | Bi-Monthly | March 2018 |
| New Mexico | Well | Monthly | March 2018 |
| New York | Well | Yearly | December 2016 |
| North Dakota | Well | Monthly | April 2018 |
| Ohio | Well | Monthlyb/Yearlyb | December 2017c/June 2018c |
| Oklahoma | Well | Monthly | March 2018 |
| Oregon | Well | Yearly | December 2017 |
| Pennsylvania | Well | Monthlyb/Yearlyb | December 2017d/April 2018d |
| South Dakota | Well | Yearly | April 2018 |
| Tennessee | Lease | Quarterly | December 2016 |
| Texas | Oil Lease/Gas Well | Twice monthly | April 2018 |
| Utah | Well | Monthly | April 2018 |
| Virginia | Well | Yearly | December 2017 |
| West Virginia | Well | Yearly | December 2016 |
| Wyoming | Well | Monthly | March 2018 |

a	Louisiana Department of Natural Resources defines a unit as the “surface area that encompasses part of or the entirety of a reservoir.”

b	For Ohio and Pennsylvania, production data for conventional wells are updated annually, while production data for unconventional wells are updated monthly.

c	For Ohio, production data for conventional wells are reported through December 2017 while production data for unconventional wells are reported through June 2018.

d	For Pennsylvania, production data for conventional wells are reported through December 2017 while production data for unconventional wells are reported through April 2018.

ERG imported all of the data from HPDI into an Oracle database for pre-processing. The Oracle database combines and processes all of the download files into one table of all production wells for the EPA Enforcement Universe Database. The processing steps are discussed below.

- Combine Monthly Production and Descriptive Information: For each entity, ERG combined the monthly production with the descriptive information (e.g., API number, lease name, location, operator, completion date, spud date, latest production date) from the HPDIHeader table to create the Wells table for the EPA Enforcement Universe Database.

- Remove Duplicate Wells: HPDI includes duplicate information for wells in some states because the data are stored by completion zone rather than at the well or lease level. Because all of the other descriptive data in HPDI are at the well or lease level, ERG combined duplicate API numbers (i.e., well bore identifiers) into a single record to avoid overcounting wells. ERG excluded the records with missing API numbers (i.e., API_NO is null) from this “remove duplicate well” step. This could result in some over counting of wells, but this should be minimal because a limited number of wells/leases did not have API numbers and there were a small percentage of duplicate wells identified.

- Create Updated Active Status Flag (ACTIVE_FLAG): ERG created an updated active status flag (ACTIVE_FLAG) per month using the latest production date (LAST_PROD_DATE) after determining that HPDI’s status flag (STATUS) was not always accurate as part of the 2011 version of the Universe Database.

- Create Monthly Production Flags: ERG created production flags to identify miscellaneous well types (e.g., injection, observation, abandoned, pressure maintenance, N/A) that have monthly oil and gas production in 2016 (PROD_01_16_FLAG through PROD_12_16). The production flag is “Yes” if the monthly oil or gas production is greater than zero.

- Assign Each Well as Oil, Gas, or CBM: Each well was reviewed to determine whether it should be labeled as an oil, gas, or CBM well. As such, the following hierarchy was used:

  - HPDI designations of CBM;

  - Wells that had 2016 oil production, but no 2016 natural gas production were assigned as “oil” wells;

  - Wells that had 2016 gas production, but no 2016 oil production were assigned as “gas” wells; and

  - Wells that had both 2016 oil and gas production were assigned “gas” if the ratio of gas to oil was greater than 100 MCF per barrel, and were assigned “oil” if the ratio of gas to oil was less than 100 MCF per barrel.

- Calculate Well Counts: Counting wells which produced oil, gas, or CBM and summing to the county-level will likely overestimate the number of wells which actually operated for an entire year because wells that operated for only one month would be essentially used as inputs for emission profiles assuming a year of operations for certain source categories. To account for this, monthly well counts were averaged to develop an annual average, and these averages were populated in the tool as:

  - NONPOINT_OIL_WELL_COUNT

  - NONPOINT_GAS_WELL_COUNT, and

  - NONPOINT_CBM_WELL COUNT

With the exception of “Oil (and Condensate) Production” and “Feet Drilled,” all of the data parameters shown in Table 2-1 are reported fields in HPDI. HPDI reports total hydrocarbon liquids production for each well, but does not distinguish between oil and condensate. As described above, each well was designated as either a gas well or an oil well. Liquid hydrocarbons produced at gas wells were then considered to be condensate, and liquid hydrocarbons produced at oil wells were considered to be oil.

Feet drilled and spud counts are needed for the Drilling and Mud Degassing source categories. While HPDI reports spud date and well depth for each well, that information is often lagging or may be incomplete at the time of the data retrieval. Thus, EPA developed an approach for utilizing the well-specific data from HPDI and state websites and from state-level “Well Starts” and “Feet Drilled” published by RIGDATA.4 The approach is as follows:

- RIGDATA published total “well starts” and “feet drilled” for 26 states:

| Alabama* | Illinois* | Michigan* | Nevada* | Oregon | Wyoming* |
| --- | --- | --- | --- | --- | --- |
| Alaska* | Indiana* | Mississippi* | New Mexico* | Pennsylvania* |  |
| Arkansas* | Kansas* | Missouri | North Dakota* | Texas* |  |
| California* | Kentucky* | Montana* | Ohio* | Utah* |  |
| Colorado* | Louisiana* | Nebraska* | Oklahoma* | West Virginia |  |

- Using the well-level spuds data from HPDI, 2016 total well depth were summed to the county-level for 23 of the 26 states (denoted by “*”). EPA then proportioned the county-level feet drilled totals to match the state totals using county-state proportions of spud counts.

- For the remaining 4 states that did not have 2016 spud counts or well depths in HPDI (Arizona, Idaho, Tennessee, and Virginia), EPA downloaded well information from the respective state websites.

- HPDI identified one additional state that reported drilling information that was not on the RIGDATA list (New York). EPA searched this state website to confirm that drilling occurred in 2016 and did not adjust the spud counts or estimated feet drilled.

- No drilling occurred in Florida, Maryland, and South Dakota in 2016.

### State-Provided Data

During the development of the 2016 oil and gas activity data, the following states provided county-level data:

- Kansas: Provided oil, gas, and CBM production, in addition to produced water;

- Texas: Provided oil and natural gas production; and

- West Virginia: Provided oil, gas, and CBM production, produced water, well completions, and drilling data.

### State Oil and Gas Commission Websites

While HPDI and state-submitted data (Kansas, Texas, and West Virginia) were the primary source for most oil, associated gas, natural gas, coalbed methane, condensate activity data, and drilling data for 2016, additional data were retrieved from oil and gas commission websites for Idaho, Illinois, Indiana, Kentucky, Missouri, Ohio, and Oregon. The 2016 production data for Kentucky and Indiana were obtained directly from Kentucky and Indiana oil and gas commission websites, respectively. Production data from Idaho and Missouri were also obtained from their respective state websites. Finally, 2016 state-level production data from the EIA annual reporting data were obtained for Illinois and Oregon, and were apportioned to the county-level by county-level well counts.

Produced water data were primarily available in HPDI for most of the states. When not reported in HPDI, information was obtained from the state’s oil and gas commission website (e.g., Pennsylvania). If produced water data were not available in either HPDI or the state’s oil and gas commission websites, then no emission estimates were generated for this source category (e.g., Oklahoma).

Well completions were primarily available in HPDI for most of the states. When not reported in HPDI, information was obtained from the state’s oil and gas commission website (e.g., Indiana). If well completions data were not available in either HPDI or the state’s oil and gas commission websites, then no emission estimates were generated for this source category (e.g., Florida).

As a result of this analysis, data from the following state oil and gas commission websites were used to compile the activity data in the tool:

- Alaska Oil and Gas Commission Data Extract: http://doa.alaska.gov/ogc/data.html

- Arizona Oil and Gas Conservation Commission: http://www.azogcc.az.gov/permits

- Idaho Oil and Gas Conservation Commission: https://ogcc.idaho.gov/wp-content/uploads/sites/50/2017/06/2016-production.pdf and https://ogcc.idaho.gov/active-drill-permits/

- Illinois: https://clearinghouse.isgs.illinois.edu/data/geology/location-points-isgs-wells-and-borings-database and https://isgs-oas.isgs.illinois.edu/reports/rwservlet?oil_permit_activity

- Indiana: https://www.in.gov/dnr/dnroil/files/og-Production_YTD.pdf and https://www.in.gov/dnr/dnroil/5447.htm

- Kentucky Geological Survey: https://kgs.uky.edu/kgsmap/OGProdPlot/OGProduction.asp and https://kgs.uky.edu/kygeode/services/oilgas/

- Louisiana Department of Natural Resources SONRIS Data Portal: http://sonlite.dnr.state.la.us/pls/apex/f?p=108:2700:9938680759893::NO:2700::::

- Michigan Department of Environmental Quality, Well Locations Database: https://www.michigan.gov/deq/0,4561,7-135-3311_4231-188295--,00.html

- Mississippi Oil and Gas Board Permit Info: http://gis.ogb.state.ms.us/MSOGBOnline/WebReportAccordion.aspx

- Missouri Department of Natural Resources: https://dnr.mo.gov/geology/geosrv/ogc/docs/oilandgasactivities-2016.pdf and https://dnr.mo.gov/geology/geosrv/ogc/ogc-permits/

- Nevada Division of Minerals: http://minerals.nv.gov/Programs/OG/OGPermits/

- New York Department of Environmental Conservation: https://www.dec.ny.gov/energy/30438.html

- Oregon Department of Geology and Mineral Industries: https://www.oregongeology.org/mlrr/oilgas-logs.htm

- Pennsylvania Office of Oil and Gas Management: http://www.depreportingservices.state.pa.us/ReportServer/Pages/ReportViewer.aspx?%2fOil_Gas%2fOil_Gas_Well_Waste

- Tennessee Department of Environment and Conservation: http://environment-online.state.tn.us:8080/pls/enf_reports/f?p=9034:34300:::NO:::

### National Production Summary

A summary of the resulting oil and gas 2016 production statistics by state is presented in Table 2-4. This includes key activity indicators such as natural gas production (associated gas, gas well, and coalbed methane gas), crude oil production, and condensate production. States not listed in Table 2-4 (e.g. Connecticut and North Carolina) did not have any oil or gas production in 2016.

| Table 2-4. Oil and Gas Production by State |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Oil Wells | Gas Wells | CBM Wells | Oil (BBL) | Associated Gas (MMCF) | Gas Well Gas (MMCF) | Condensate (BBL) | CBM Condensate (BBL) | CBM Gas (MMCF) |  |
| Alabama | 467 | 220 | 5,522 | 7,738,529 | 26,258 | 69,938 | 23,202 | 345,466 | 69,518 |  |
| Alaska | 1,724 | 198 | 0 | 177,550,165 | 2,930,180 | 317,652 | 1,619,044 | 0 | 0 |  |
| Arizona | 6 | 3 | 0 | 7,425 | 0 | 54 | 0 | 0 | 0 |  |
| Arkansas | 1,552 | 9,181 | 47 | 5,550,760 | 7,463 | 819,174 | 1,266 | 0 | 1,244 |  |
| California | 45,291 | 1,826 | 0 | 185,954,198 | 145,701 | 205,793 | 170,315 | 0 | 0 |  |
| Colorado | 18,854 | 17,586 | 5,088 | 114,000,512 | 693,308 | 1,063,247 | 1,281,526 | 260,829 | 369,082 |  |
| Florida | 59 | 3 | 0 | 1,930,197 | 20,160 | 756 | 3,408 | 0 | 0 |  |
| Idaho | 0 | 8 | 0 | 0 | 0 | 4,576 | 215,049 | 0 | 0 |  |
| Illinois | 46,653 | 1,176 | 198 | 8,633,483 | 0 | 1,868 | 0 | 0 | 315 |  |
| Indiana | 4,140 | 1,415 | 0 | 1,818,460 | 0 | 6,206 | 0 | 0 | 0 |  |
| Kansas | 52,869 | 18,235 | 5,175 | 37,938,284 | 38,544 | 223,338 | 290,681 | 48,247 | 23,201 |  |
| Kentucky | 10,421 | 18,751 | 14 | 2,594,784 | 0 | 163,040 | 0 | 0 | 125 |  |
| Louisiana | 18,284 | 14,546 | 6 | 54,674,668 | 234,585 | 1,493,036 | 1,197,869 | 5,514 | 83 |  |
| Maryland | 0 | 1 | 0 | 0 | 0 | 34 | 0 | 0 | 0 |  |
| Michigan | 4,565 | 9,851 | 0 | 6,119,891 | 12,598 | 88,934 | 4,628 | 0 | 0 |  |
| Mississippi | 1,759 | 1,367 | 0 | 20,388,447 | 17,356 | 207,060 | 61,533 | 0 | 0 |  |
| Missouri | 1,210 | 3 | 0 | 122,975 | 0 | 1 | 0 | 0 | 0 |  |
| Montana | 4,194 | 4,948 | 42 | 23,240,629 | 51,846 | 29,073 | 72 | 0 | 253 |  |
| Nebraska | 1,936 | 131 | 0 | 2,129,728 | 63 | 460 | 0 | 0 | 0 |  |
| Nevada | 47 | 0 | 0 | 276,727 | 3 | 0 | 0 | 0 | 0 |  |
| New Mexico | 22,547 | 18,913 | 5,534 | 146,031,433 | 574,092 | 532,425 | 940,202 | 26,252 | 260,266 |  |
| New York | 2,867 | 6,407 | 0 | 220,873 | 433 | 13,017 | 216 | 0 | 0 |  |
| North Dakota | 12,940 | 194 | 0 | 377,796,325 | 605,226 | 3,579 | 12,430 | 0 | 0 |  |
| Ohio | 22,498 | 22,612 | 7 | 20,716,844 | 304,998 | 1,133,575 | 904,181 | 0 | 2 |  |
| Oklahoma | 36,267 | 27,806 | 3,273 | 153,717,800 | 1,099,884 | 1,131,500 | 1,767,693 | 192,269 | 16,129 |  |
| Oregon | 0 | 13 | 0 | 0 | 0 | 797 | 0 | 0 | 0 |  |
| Pennsylvania | 19,583 | 57,173 | 458 | 5,431,511 | 148,547 | 5,061,915 | 848,173 | 0 | 8,306 |  |
| South Dakota | 153 | 44 | 0 | 1,399,859 | 9,225 | 940 | 6,810 | 0 | 0 |  |
| Tennessee | 628 | 791 | 0 | 253,740 | 319 | 3,270 | 1,034 | 0 | 0 |  |
| Texas | 177,743 | 101,868 | 535 | 1,013,025,977 | 2,468,026 | 5,701,082 | 145,028,288 | 370,363 | 8,356 |  |
| Utah | 5,702 | 5,096 | 903 | 29,867,298 | 103,388 | 221,276 | 661,665 | 0 | 40,604 |  |
| Virginia | 40 | 1,931 | 6,015 | 7,217 | 148 | 19,909 | 840 | 25 | 100,228 |  |
| West Virginia | 8,918 | 54,810 | 610 | 613,610 | 0 | 1,374,228 | 5,026,744 | 0 | 10,226 |  |
| Wyoming | 12,536 | 10,798 | 6,229 | 66,851,103 | 418,881 | 1,262,554 | 5,581,533 | 60,625 | 166,872 |  |
| Total | 536,453 | 407,905 | 39,656 | 2,466,603,451 | 9,911,231 | 21,154,308 | 165,648,402 | 1,309,590 | 1,074,811 |  |

## Process Characterization Data

As described in the CenSARA study, while activities can vary within a basin (e.g. both oil and gas operations), the geologically influenced characteristics of a specific basin (e.g. depth, pressure, presence of water, oil quality, gas composition) directly affect activity parameters that describe oil and gas operations within the basin boundaries, and in turn, influence emissions. A basin therefore represents a detailed but tractable geographic unit for development of emissions factors and other process characterization data for oil and gas nonpoint source emissions estimates.

In the CenSARA study, oil and gas nonpoint source emissions were estimated for each county within a discrete basin based on equipment characterization, activity data, and emission factors developed specifically for that basin. This equipment, activity, and emission factor data were obtained through industry surveys, a review of oil and gas datasets compiled by state and local agencies, and from existing studies.

Figure 2-1 below illustrates the 19 oil and gas basins included in the geographic scope of coverage of the CenSARA study.

Figure 2-1. Oil and Gas Basins Covered by the CenSARA Study

Nationally, the remainder of the country was sub-divided into oil and gas basins (as defined by the geologic provinces published by the American Association of Petroleum Geologists (AAPG)) as used under Subpart W of the Greenhouse Gas Reporting Program (GHGRP).

Using the AAPG definitions, the country is divided into 114 distinct oil and gas basins. Figure 2-2 below illustrates the geographic division of the country into oil and gas basins as defined by the geologic provinces published by the AAPG.

Figure 2-2. Oil and Gas Basins as Defined by the Geologic Provinces Published by the AAPG

The basic methodology employed in development of the CenSARA inventory was used to develop the national tool described in this report. However, during development and review of the tool by various stakeholders as part of the initial tool development for the 2011 NEI, it was determined that county-level resolution was needed to accommodate differing operational characteristics within a basin. Therefore, the tool currently resolves equipment characterization and activity data down to the county level.

For the CenSARA states, the input data from the CenSARA study have been used in the tool. For several oil and gas basins located in states adjacent to the CenSARA states, the AAPG basin definitions overlap into the CenSARA states. Therefore, CenSARA basin-specific data was used to initially populate the tool database for these basins. Table 2-5 identifies the basins adjacent to the CenSARA states where CenSARA basin-specific data was initially input into the tool.

| Table 2-5. Oil and Gas Basins Adjacent to CenSARA States |  |  |
| --- | --- | --- |
| AAPG Basin | Affected States | CenSARA basin |
| Anadarko Basin | CO | Anadarko Basin |
| Las Animas Arch | CO | Cambridge Arch-Central Kansas Uplift |
| Chadron Arch | SD | Cambridge Arch-Central Kansas Uplift |
| Denver Basin | CO, WY | Denver Basin |
| Forest City Basin | IA | Forest City Basin |
| Upper Mississippi Embayment | KY, MS, TN | Illinois Basin |
| Desha Basin | MS | Louisiana-Mississippi Salt Basins |
| Illinois Basin | IL, IN, KY | Illinois Basin |
| Palo Duro Basin | NM | Palo Duro Basin |
| Permian Basin | NM | Permian Basin |
| Orogrande Basin | NM | Permian Basin |
| Mid-Gulf Coast Basin | AL, FL, MS | Western Gulf |

Finally, for those basins falling entirely outside of the CenSARA states, national averages for equipment profiles and activity levels were developed based on the average of the surveyed basins within the CenSARA states. While this data were used to initially populate the input data for the tool database, many different state agencies, RPOs, and EPA supplied data that have been used in the current version of the tool.

For example, for certain source categories such as well completions and mud degassing, gas composition data developed by EPA for regulatory development purposes was used for the non-CenSARA basins (EPA, 2011). Gas composition profiles developed under this effort were used as default profiles for:

- Associated Gas Venting (Oil Wells)

- Fugitives (Gas Wells)

- Gas-Actuated Pumps (Gas Wells)

- Liquids Unloading (Gas Wells)

- Mud Degassing (Oil and Gas Wells)

- Pneumatic Devices (Gas Wells)

- Well Completions (Oil and Gas Wells)

Appendix C contains a comprehensive list of each county in the United States and the associated AAPG oil and gas basin name, and under the CenSARA inventory (if applicable). Appendix C also identifies what data was initially used to populate the tool database for each county. This was either data from a specific CenSARA basin for the CenSARA states (CENSARA_2012), data from a specific CenSARA basin for certain basins/counties adjacent to the CenSARA states as listed in Table 2-5 (CENSARA_EXTENSION), or nationally-averaged data from all CenSARA basins (CENSARA_AVG). While Appendix C identifies the initial reference for the data used to populate the tool database, numerous updates have been made to the tool since it was initially developed to incorporate EPA, state, and local data. The tool database contains specific references at the county level for each data element used in the emission estimation algorithms.

Table 2-6 provides a broad overview of the types of data currently found in the tool. The table indicates “S” for state supplied data, “D” for default CenSARA data, “E” for EPA data, or “R” for RPO data (e.g. CenSARA or WRAP). In many instances, a mix of these data types are used to estimate emissions for a single source category. In these cases, each type of data found in the tool is identified. These data sources were unchanged from the 2014 NEI.

## Updates Since 2014

The final version of the 2014 NEI Oil and Gas Tool was completed in June of 2017. Version 1 of the 2016 Oil and Gas Tool was completed in November of 2018. The primary updates made since finalization of the 2011 version of the tool include:

- Updated Activity Data. Oil and gas exploration and production activity data was updated to reflect 2016 as described in Section 2.1 using data from the HPDI database, RIGDATA, and various state oil and gas commission websites.

- Emission Factor Updates. Several drilling and hydraulic fracturing emission factors were updated using the MOVES model for 2016 nonroad engine conditions.

- Temperature Updates. Updated annual average temperature data by county using EPA’s 2016 Weather Research Factorization (WRF) model data (EPA, 2018x).

- Additional Pollutants. Additional HAPs were added to the Tool in preparation for the 2017 NEI oil and gas tool.

| Table 2-6. Tool Data Sources by State and Source Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Artificial Lifts | Associated Gas | Condensate Tanks | Crude Oil Tanks | Dehydrators | Drilling Rigs | Fugitive Leaks | Gas-actuated Pumps | Heaters | Hydraulic Fracturing Pumps | Lateral/Gathering Compressor Engines | Liquids Unloading | Loading | Mud Degassing | Pneumatic Devices | Produced Water Tanks | Well Completions | Wellhead Compressor Engines |  |
| AL | D, S | D, E, R | D, E, R, S | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| AK | D | B, D, E | D, E, S | D, E, S | D | D | B, D, E | B, D, E | D, E | D | D | D, E | D, E | D, E | B, D, E | D | B, D, E | D |  |
| AZ | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| AR | R | E, R | E, R | E, R | R | R | R | R | E, R | R | R | R | E, R | R | E, R | R | E, R | R |  |
| CA | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| CO | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| FL | D, S | D, E, R | D, E, R, S | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| ID | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| IL | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| IN | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| KS | R | E, R | E, R | E, R | R | R | R, S | R, S | E, R | R | R | R, S | E, R | R | E, R, S | R | E, R, S | R |  |
| KY | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| LA | R, S | E, R | E, R, S | E, R | R | R | R | R | E, R | R | R | R | E, R | R | E, R | R | E, R | R |  |
| MD | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| MI | D | D, E | D, E | D, E | D, S | D | D, E | D, E | D, E, S | D | D, S | D, E | D, E | D, E | D, E | D | D, E | D, S |  |
| MS | D, R, S | D, E, R | D, E, R, S | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| MO | R | E, R | E, R | E, R | R | R | R | R | E, R | R | R | R | E, R | R | E, R | R | E, R | R |  |
| MT | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E | D, E, R | D, R | D, E, R | D, R |  |
| NE | R | E, R | E, R | E, R | R | R | E, R | E, R | E, R | R | R | E, R | E, R | R | E, R | R | E, R | R |  |
| NV | D, S | D, E | D, E | D, E | D, S | D | D, E, S | D, E, S | D, E, S | D | D, S | D, E, S | D, E | D, E | D, E, S | D | D, E | D, S |  |
| NM | D, R, S | D, E, R, S | D, E, R, S | D, E, R, S | D, R, S | D, R, S | D, E, R, S | D, E, R, S | D, E, R | D, R, S | D, R | D, E, R, S | D, E, R | D, E, R, S | D, E, R, S | D, R | D, E, R, S | D, R, S |  |
| NY | D, S | D, E | D, E | D, E | D, S | D | D, E | D, E | D, E | D | D, S | D, E, S | D, E | D, E | D, E, S | D | D, E | D, S |  |
| ND | R | E, R | E, R | E, R | R | R | R | R | E, R | R | R | R | E, R | D, E | E, R | R | R | R |  |
| OH | D, S | D, E | D, E | D, E | D, S | D | D, E, S | D, E, S | D, E | D | D, S | D, E, S | D, E | D, E | D, E, S | D | D, E | D, S |  |
| OK | R, S | E, R | E, R, S | E, R | R | R | R | R | E, R | R | R | R | E, R | R | E, R | R | E, R | R |  |
| OR | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| PA | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| SD | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| TN | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R | D, R |  |
| TX | S | E, R | E, R, S | E, R | R, S | R | R, S | E, R | E, R | R, S | R, S | E, R | E, R | R, S | E, R, S | R | E, R, S | R, S |  |
| UT | D, R | D, E | D, E, R | D, E, R, S | D, E, R, S | D, R, S | D, E, R | D, E, R, S | D, E, R | D | D, R | D, E, R, S | D, E, R | D, E | D, E, R | D | D, E, R, S | D, R, S |  |
| VA | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D | D | D, E | D, E | D, E | D, E | D | D, E | D |  |
| WV | D | D, E, S | D, E, S | D, E | D, S | D | D, E | D, E, S | D, E | D | D | D, E, S | D, E, S | D, E | D, E | D | D, E, S | D |  |
| WY | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, R | D, R | D, E, R | D, E, R | D, E, R | D, E, R | D, R | D, E, R, S | D, R |  |

a D = Default data from CenSARA Study, E = EPA, R = RPO (CenSARA or WRAP), S = state, B = BOEM

# Source Category Emission Estimation Methodologies

Emissions for individual oil and gas nonpoint source categories were developed using a bottom-up approach that begins with developing mass emission rates for each pollutant based on an activity surrogate (e.g. tons per well, tons per barrel of oil, tons per feet drilled). These by-surrogate emission rates were then scaled to county-level emissions by multiplying the emission rates by the scaling surrogate or activity from a particular county (e.g. gas well counts, horizontal feet drilled, crude oil production, etc.).

Emissions calculations are performed within the Microsoft Access database. Data field names and definitions for calculation inputs are shown in Appendix D (Data Element Dictionary) in the same format and nomenclature as they appear in the database tool. Appendix D also provides the national “default” value for each variable (and reference) used in the calculations when state-supplied data is unavailable. Refer to the instructions included in Appendices A and B for details on how the database is organized.

The following sections describe emissions calculations for each source category; it is noted that some of these methodologies may apply to multiple SCCs and thus, are calculated separately in the tool. Example calculations are provided for each source category. The examples are provided for illustrative purposes only and may not match the totals calculated by the tool due to rounding or updates to any of the activity or emission factor inputs.

Table 3.1 below identifies the source categories associated with each type of well (oil or gas), and the primary activity parameter used as the basis to scale emissions up to the county level.

| Table 3-1. Emission Sources by Well Type |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category | Activity Basis | Oil | CBM | Gas |
| Artificial Lifts | Oil Well Count | Yes | No | No |
| Associated Gas | Oil Production | Yes | No | No |
| Condensate Tanks | Condensate Production | No | Yes | Yes |
| Crude Oil Tanks | Oil Production | Yes | No | No |
| Dehydrators | Gas, Associated Gas, and CBM Production; Gas and CBM Well Counts | No | Yes | Yes |
| Drill Rigs | Estimated Feet Drilled | Yes | Yes | Yes |
| Fugitive Leaks | Oil, Gas, and CBM Well Count | Yes | Yes | Yes |
| Gas-Actuated Pumps | Oil, Gas, and CBM Well Count | Yes | Yes | Yes |
| Heaters | Oil, Gas, and CBM Well Count | Yes | Yes | Yes |
| Hydraulic Fracturing Pumps | Horizontal Spud Count | Yes | Yes | Yes |
| Lateral/Gathering Compressor Engines | Gas and CBM Well Count | No | Yes | Yes |
| Liquids Unloading | Gas and CBM Well Count | No | Yes | Yes |
| Loading | Oil and Condensate Production | Yes | Yes | Yes |
| Mud Degassing | Spud Counts | Yes | Yes | Yes |
| Pneumatic Devices | Oil, Gas, and CBM Well Count | Yes | Yes | Yes |
| Produced Water Tanks | Produced Water Production | Yes | Yes | Yes |
| Well Completions | Completion Count | Yes | Yes | Yes |
| Wellhead Compressors | Gas and CBM Well Count | No | Yes | Yes |

## Artificial Lifts

Artificial lifts refer specifically to engines located at oil wells that provide lift to the liquids in a well up to the wellhead. Generally, artificial lift engines are small natural-gas fired engines. In the past decade there has been an increased use of electrified artificial lift engines powered by the grid; for this kind, emissions are assumed to be zero. Figure 3-1 shows a pump jack with an artificial lift engine (inset).

Figure 3-1. Artificial Lift Engine

The basic methodology for estimating emissions from a single non-electrified artificial lift engine is shown below:

Equation 1)

where:

Eengine are emissions from an artificial lift engine [ton/year/engine]

EFi is the emissions factor of pollutant i [g/hp-hr]

HP is the horsepower of the engine [hp]

LF is the load factor of the engine

tannual is the annual number of hours the engine is used [hr/yr]

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

Artificial lift engine emissions have been scaled up to the county level on the basis of oil well counts. The methodology for scaling up artificial lift engine emissions is shown below:

Equation 2)

where:

Eengine,TOTAL is the total emissions from artificial lift engines in a county [ton/yr]

n is the total number of artificial lift engines per well, generally equal to 1 (n=1) [engine/well]

Eengine is the total emissions from an artificial lift engine (as shown in Equation 1) [ton/yr/engine]

fpumpjack is the fraction of oil wells with artificial lift engines

FE is the fraction of artificial lift engines that are electric

# WOIL,TOTAL is the total number of oil wells in a county [wells]

Example Calculation for Artificial Lift:

Using the equations provided above, NOx emissions from artificial lift engines in Calhoun County, Arkansas were calculated as follows:

where:

E engine  = emissions from an artificial lift engine [ton/yr/engine]

EF = 8.24 [g/hp-hr]

HP = 77.5 [hp]

LF = 0.85 (load factor for the engine)

tannual = 8,000 [hr/yr]

907,185 [g/ton]

Therefore:

E engine  = 4.79 [ton/yr/engine]

Total NOx emissions from all artificial lift engines in Calhoun County can be evaluated as follows:

where:

Eengine,TOTAL is the total emissions from artificial lift engines in a county [ton/yr]

n = 1 [engine/well]

Eengine = 4.79 [ton/yr/engine]

fpumpjack = 0.95 (fraction of oil wells with artificial lift engines)

# FE = 0.965 (fraction of artificial lift engines that are electrified)

# WOIL,TOTAL = 18 [wells]

Therefore:

E engine, TOTAL = 2.86 [ton/yr]

## Associated Gas Venting and Flaring

This section refers to the practice of venting associated gas from oil wells which sometimes takes place when the well is not connected to a gas sales pipeline or when the amount of gas produced by the well is so limited that is not profitable for capture. In some areas of the country, this gas may be flared.

The calculation methodology for estimating county-wide emissions from associated gas venting is shown below in Equation 3:

Equation 3)

where:

E,assoc, gas,i is the county-wide emissions of pollutant i from associated gas venting [ton/yr]

P is atmospheric pressure [1 atm]

Qassoc,gas,i is the venting rate of associated gas per unit of oil production [MCF/bbl]

Poil is the annual county-wide oil production [bbl/yr]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the gas [g/mol]

T is the atmospheric temperature [298 K]

fi is the mass fraction of pollutant i in the associated gas

Fflare is the fraction of associated gas controlled with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Flaring emissions from associated gas controls

Emissions from flaring controls applied to associated gas are included in this source category. The methodology for estimating emissions from flaring of associated gas is described below:

Equation 4)

where:

Eflare,assoc,gas is the county-wide flaring emissions of pollutant i from vented associated gas [ton/yr]

EFi is the flaring emissions factor for pollutant i [lb/MMBtu]

Qassoc,gas is the volume of associated gas vented per barrel of oil produced [MCF/bbl]

F is the fraction of associated gas vent controlled with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

HV is the local heating value of the gas [BTU/SCF]

Poil is the annual county-wide oil production [bbl/yr]

2,000 is the unit conversion factor lbs/ton

The methodology for estimating SO2 emissions from flaring of associated head gas is shown below:

Equation 5)

where:

is the county-wide SO2 emissions from flaring of associated gas [ton/yr]

P is atmospheric pressure [1 atm]

Qassoc,gas is vented volume of associated gas per barrel of oil [MCF/bbl]

Poil is the annual county-wide oil production [bbl/yr]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the associated gas [g/mol]

T is the atmospheric temperature [298 K]

is the mass fraction of H2S in the associated gas

Fflare is the fraction of associated gas vents controlled by flare

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

County-wide emissions from associated gas venting and associated gas flaring are estimated directly from Equations 3-5. The sum of venting and flaring emissions by pollutant yield the total county-wide emissions from associated head gas that is not captured for sale.

Example Calculation for Associated Gas Venting:

Using the equations provided above, VOC emissions for associated gas venting in Columbia County, Arkansas were calculated as follows:

where:

Eassoc,gas is the county-wide emissions of VOC from associated gas venting [ton/bbl]

P = 1 [atm]

Qassoc,gas = 0.00365 [MCF/bbl]

Poil = 1,231,945 [bbl/yr]

R = 0.082 [L-atm/mol-K]

MWgas = 24.25 [g/mol]

T = 298 [K]

f = 0.262 (the mass fraction of VOC in the associated gas)

Fflare = 0 (the fraction of associated gas vent controlled with flares)

Ccaptured = 1.0 (capture efficiency expressed as fraction)

Cefficiency = 0.98 (control efficiency expressed as fraction)

3.5x10-5 [MCF/L]

907,185 [g/ton]

Therefore:

E assoc, gas = 36.82 [ton/yr]

Flaring emissions would be calculated similarly to the example given above for condensate tanks. In this case, since it is assumed that the fraction of associated gas controlled by flares is zero, there are no flare emissions.

## Condensate Tanks

Condensate storage tanks are considered a significant source of VOC emissions. Liquid storage tank losses are generated by flashing and by working and breathing processes, although generally the emissions are dominated by flashing losses. This analysis uses a combined-losses emissions factor and assumes that the gas compositions from both processes are identical. Figure 3-2 shows liquid storage tanks in the Barnett Shale.

Figure 3-2. Liquid Storage Tanks

The methodology for estimating condensate tank combined losses is shown below:

Equation 6)

where:

Econdensate,tanks,VOC is the VOC emissions per liquid unit throughput from condensate tanks [tons/bbl]

EFcondensate,tanks,VOC is the VOC emissions factor for combined losses from condensate tanks [lb-VOC/bbl]

Fflare is the fraction of condensate tanks with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

2,000 is the unit conversion factor lb/ton

The methodology for estimating condensate tank combined losses from other pollutants i in the gas is shown below:

Equation 7)		Econdensate, tanks,i= Econdensate,tanks,VOC ×weight fractioniweight fractionVOC

where:

Econdensate,tanks,i is the emissions of pollutant i per liquid unit throughput from condensate tanks [tons/bbl]

Econdensate,tanks,VOC is the VOC emissions per liquid unit throughput from condensate tanks [tons-VOC/bbl]

(weight fractioni/weight fractionvoc) is the mass-based weight fraction of pollutant i divided by the weight fraction of VOC in the gas

Flaring emissions from condensate tank controls

This source category includes any flaring emissions associated with controls applied to condensate tanks. The methodology for estimating emissions from flaring of condensate tank flash gas is described below:

Equation 8)

where:

Eflare,tank,i is the county-wide flaring emissions of pollutant i from condensate tank controls [ton/yr]

Pcondensate is the annual county-wide condensate production [bbl/yr]

Qcondensate,tank is the uncontrolled volume of tank losses vented per unit of condensate throughput [MCF/bbl]

Fflare is the fraction of condensate tanks with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

EFi is the flaring emissions factor for pollutant i [lb/MMBtu]

HV is the local heating value of the gas [BTU/SCF]

2,000 is the unit conversion factor lb/ton

1,000 is the unit conversion factor MCF/MMCF

The methodology for estimating SO2 emissions from flaring of oil and condensate flash gas is shown below:

Equation 9)

where:

is the county-wide SO2 flaring emissions from condensate tanks controls [ton/yr]

P is atmospheric pressure [1 atm]

Qcondensate,tank is the uncontrolled volume of tank losses vented per unit of condensate throughput [MCF/bbl]

Fflare is the fraction of condensate tanks with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

Pcondensate is the annual county-wide condensate production [bbl/yr]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the flash gas [g/mol]

T is the atmospheric temperature [298 K]

is the mass fraction of H2S in the flash gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

To estimate county-wide total controlled and uncontrolled condensate tank emissions, which includes venting and flaring, for each pollutant i, Equation 10 below is used:

Equation 10)

where:

Econdensate,tanks,TOTAL is the county-wide total emissions for pollutant i from condensate tanks [tons/yr]

Econdensate,tanks,i is the combined losses of pollutant i per liquid unit throughput from condensate tanks [tons/bbl]

Pcondensate is the annual county-wide condensate production [bbl/yr]

Ftank is the fraction of condensate directed to tanks [%]

Eflare,tanks,i is the county-wide flaring emissions of pollutant i from condensate tank controls [ton/yr]

Example Calculation for Condensate Tanks:

Using the equations provided above, VOC and SO2 emissions from condensate tank venting and flaring in Columbia County, Arkansas were calculated as follows:

Venting Emissions:

where:

Econdensate,tanks,VOC = the VOC emissions per liquid unit throughput from condensate tanks [tons/bbl]

EFcondensate,tanks,VOC = 3.60 [lb-VOC/bbl]

Fflare = 0.315 (fraction of condensate tanks with flares)

Ccaptured = 1.0 (capture efficiency expressed as fraction)

Cefficiency = 0.98 (control efficiency expressed as fraction)

2,000 [lb/ton]

Therefore:

Econdensate,tanks,VOC = 0.001244 [tons/bbl]

Flaring Emissions:

VOC emissions from flaring of condensate tank vapors may then be calculated as follows:

where:

Eflare,tank  is the county-wide flaring emissions of VOC from condensate tank controls [ton/yr]

Pcondensate = 275,892 [bbl/yr]

Qcondensate,tank = 0.037 [MCF/bbl]

Fflare = 0.315 (fraction of condensate tanks with flares)

Ccaptured = 1.0 (capture efficiency expressed as fraction)

Cefficiency = 0.98 (control efficiency expressed as fraction)

EF = 0.09 [lb/MMBtu]

HV = 2,597 [BTU/SCF]

2,000 [lb/ton]

1,000 (conversion factor)

Therefore:

Eflare,tank  = 0.37 [ton/yr]

Total VOC emissions from all condensate tanks in Columbia County can be evaluated as follows:

where

Econdensate,tanks,TOTAL is the county-wide total emissions of VOC from condensate tanks [ton/yr]

Econdensate,tanks,VOC = 0.0012 [tons/bbl]

Pcondensate  = 275,892 [bbl/yr]

Ftank = 1 (fraction directed to tanks)

Eflare,tanks  =  0.37 [ton/yr]

Therefore:

Econdensate,tanks,TOTAL = 343.6 [ton/yr]

## Crude Oil Tanks

Crude oil tanks are used to store liquid product at a well pad or central tank battery prior to transfer downstream to a refinery. Figure 3-3 shows a central tank battery (circled) in the Permian Basin adjacent to numerous well pads with pump jacks.

Crude oil tank emissions are generated by working and breathing processes. The methodology for estimating oil tank venting emissions is shown in Equations 11-12. This methodology is based on a combined working and breathing losses VOC emissions factor on a per unit throughput basis (mass emissions per barrel of oil).

Figure 3-3. Permian Basin Tank Battery

Equation 11)

where:

Eoil,tanks,VOC is the county-wide annual VOC venting losses from oil tanks [tons-VOC/yr]

Poil is the annual county-wide oil production [bbl/yr]

EFoil,tanks,VOC is the VOC emissions factor for total losses from oil tanks [lb-VOC/bbl]

Ftank  is the fraction of oil directed to tanks [%]

Fflare is the fraction of oil tanks with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

2,000 is the unit conversion factor lb/ton

The methodology for estimating crude oil tank losses from other pollutants i in the emissions is shown below:

Equation 12)		Eoil, tanks,i= Eoil,tanks,VOC ×weight fractioniweight fractionVOC

where:

Eoil,tanks,i  is the county-wide annual losses of pollutant i from oil tanks [tons/yr]

Eoil,tanks,VOC is the county-wide annual VOC venting losses from oil tanks [tons-VOC/yr]

(weight fractioni/weight fractionvoc) is the mass-based weight fraction of pollutant i divided by the weight fraction of VOC in the gas

Flaring emissions from oil tank controls

This source category includes any flaring emissions associated with controls applied to crude oil tanks. The methodology for estimating emissions from flaring of oil tank gas losses is described below:

Equation 13)

where:

Eflare,tank,i is the county-wide emissions from crude oil tank flaring [ton/yr]

Poil is the annual county-wide oil production [bbl/yr]

Qoil,tanks,flash is the volume of gas flared per unit of oil throughput [MCF/bbl]

Fflare is the fraction of oil tanks with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

EFi is the flaring emissions factor for pollutant i [lb/MMBtu]

HV is the local heating value of the gas [BTU/SCF]

1,000 is the unit conversion factor MCF/MMCF

2,000 is the unit conversion factor lb/ton

The methodology for estimating SO2 emissions from flaring of oil tank losses is shown below:

Equation 14)

where:

is the county-wide SO2 emissions from flaring controls in oil tanks [ton/yr]

P is atmospheric pressure [1 atm]

Qoil,tank,flash is the volume of gas vented per unit of oil throughput [MCF/bbl]

Fflare is the fraction of crude oil tanks with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

Poil is the annual county-wide oil production [bbl/yr]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the gas [g/mol]

T is the atmospheric temperature [298 K]

is the mass fraction of H2S in the gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

Equations 11-14 provide county-wide estimates directly using by-county oil production as a surrogate. The total county-wide emissions from crude oil tanks are the sum of flaring and crude tank working and breathing emissions (by-pollutant).

Example Calculation for Crude Oil Tanks:

Using the equations provided above, VOC emissions for crude oil tanks in Columbia County, Arkansas were calculated as follows:

where:

Eoil,tanks,VOC is the county-wide annual VOC venting losses from oil tanks [tons-VOC/yr]

Poil = 1,231,945 [bbl/yr]

EFoil,tanks,VOC = 0.287 [lb-VOC/bbl]

Ftank = 1 (fraction directed to tanks)

Fflare = 0 (fraction flared)

Ccaptured = 1.0 (capture efficiency expressed as fraction)

Cefficiency = 0.98 (control efficiency expressed as fraction)

2,000 [lb/ton]

Therefore:

E oil,tanks,VOC = 177 [tons-VOC/yr]

Flaring emissions are calculated similarly to the example given above for condensate tanks. In this case, since the fraction of crude oil tank vapors sent to flares is zero, there are no flare emissions.

## Dehydrators

This source category refers to wellhead dehydrator units. Dehydrator units are used to remove excess water from produced natural gas prior to delivery to the pipeline or to a gas processing plant. Two main sources of emissions are found in a dehydrator device: hydrocarbon emissions (including VOC and HAPs) are generated in the dehydrator still vent, and combustion emissions are generated in the dehydrator reboiler. In addition, if dehydrator still vents are controlled by flare, combustion emissions from flaring controls contribute to the total dehydrator emissions. Figure 3-4 shows a glycol dehydrator in the Barnett shale.

Figure 3-4. Dehydrator

The basic methodology for estimating county-wide emissions from dehydrator still vents is shown in Equation 15:

Equation 15)

where:

Estillvent,VOC is the county-wide VOC emissions from dehydrator still vents [ton/yr]

Pgas is the annual county-wide gas production [MCF/yr]

EFstillvent is the VOC emission factor for dehydrator still vent per unit of gas throughput [lb-VOC/MMCF]

Fflare is the fraction of dehydrator vents with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

2,000 is the unit conversion factor lb/ton

1,000 is the unit conversion factor MCF/MMCF

The methodology for estimating dehydrator still vent emissions from other pollutants i is shown below:

Equation 16)		Estillvent,i= Estillvent,VOC ×weight fractioniweight fractionVOC

where:

Estillvent,i is the county-wide emissions of pollutant i from dehydrator still vents [ton/yr]

Estillvent,VOC is the county-wide VOC emissions from dehydrator still vents [ton/yr]

(weight fractioni/weight fractionvoc) is the mass-based weight fraction of pollutant i divided by the weight fraction of VOC in the vented gas

The basic methodology for estimating emissions for the dehydrator reboiler is equivalent to that of a standard field heater:

Equation 17) 		Ereboiler,   i=N×EFi × Qreboiler× tannual × hcHV × 2,000×Wgas

where:

Ereboiler,i is the county-wide emissions from pollutant i from dehydrator reboilers [ton/yr]

N is the number of dehydrators per well [1/well]

EFi is the emission factor for pollutant i for natural gas-fired small boilers [lb/MMCF]

Qreboiler is the heater size [MMBtu/hr]

tannual is the annual hours of operation [hr]

hc is a heater cycling fraction of operating hours that the heater is firing

HV is the local natural gas heating value [Btulocal/SCF]

Wgas is the county-wide number of active gas wells in a particular year [well/yr]

2,000 is the unit conversion factor lb/ton

Flaring emissions from dehydrator venting controls

The methodology for estimating county-wide emissions from flaring of dehydrator still vent gas is described below:

Equation 18)

where:

Eflare,dehy,i is the county-wide emissions of pollutant i from dehydrator vent gas flaring [ton/yr]

Pgas is the annual county-wide gas production [MCF/yr]

Qdehydrator,vent is the volume of gas flared per unit of gas throughput in dehydrator [MCF vented/MMCF natural gas]

Fflare is the fraction of dehydrators with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

EFi is the flaring emissions factor for pollutant i [lb/MMBtu]

HV is the local heating value of the gas [BTU/SCF]

2,000 is the unit conversion factor lb/ton

106 is the unit conversion factor SCF/MMCF

The methodology for estimating SO2 emissions from flaring of dehydrator vent gas is shown below:

Equation 19)

where:

is the county-wide SO2 flaring emissions from flaring of dehydrator vent gas [ton/yr]

P is atmospheric pressure [1 atm]

Pgas is the annual county-wide gas production [MCF/yr]

Qdehydrator,vent is the volume of gas flared per unit of gas throughput [MCF vented/MMCF natural gas]

Fflare is the fraction of dehydrators with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the dehydrator venting gas [g/mol]

T is the atmospheric temperature [298 K]

is the mass fraction of H2S in the dehydrator venting gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

Equations 15-19 provide direct county-level estimates of pollutant emissions from dehydrator still vents, reboilers, and flaring controls. Emissions of the same pollutant each of these three sub-categories should be added together to arrive at total county-level dehydrator emissions (still vent + reboiler + flaring).

Example Calculation for Dehydrators:

Using the equations provided above, VOC emissions from the still vents and reboilers of dehydrators in Cleburne County, Arkansas were calculated as follows:

Still Vent emissions:

where:

Estillvent,VOC is the county-wide VOC emissions from dehydrator still vents [ton/yr]

Pgas = 139,458,888 [MCF/yr]

EFstillvent =  0.528 [lb-VOC/MMCF]

Fflare = 0 (fraction of dehydrator vents with flares)

Ccaptured = 1.0 (capture efficiency expressed as fraction)

Cefficiency = 0.98 (control efficiency expressed as fraction)

2,000 [lb/ton]

1,000 [MCF/MMCF]

Therefore:

Estillvent,VOC = 36.8 [ton/yr]

Flaring emissions are calculated similarly to the example given above for condensate tanks. In this case, since the fraction of still vent vapors sent to flares is zero, there are no flare emissions.

Reboiler emissions:

Ereboiler,   voc=N×EFVOC× Qreboiler× tannual × hcHV × 2,000×Wgas

where:

Ereboiler,VOC is the county-wide emissions of VOC from dehydrator reboilers [ton/yr]

N = 1 [per well]

EFVOC = 5.5 [lb/MMCF]

Qreboiler = 0.9875 [MMBtu/hr]

tannual = 8,672.5 [hr/yr]

hc =  1 (cycling fraction of operating hours that the heater is firing)

HV = 1,035 [Btulocal/SCF]

Wgas = 490 [wells]

2,000 [lb/ton]

Therefore:

Ereboiler,  VOC=1×5.5 × 0.9875× 8,672.5 × 1 1,035 ×2,000×490

Ereboiler,VOC = 11.15 [ton/yr]

Total VOC emissions from dehydrators in Cleburne County can be evaluated as follows:

Edehy,VOC = Estillvent,VOC + Ereboiler,VOC

Edehy,VOC = 36.8 [ton/yr] + 11.15 [ton/yr]

Edehy,VOC = 48.0 [ton/yr]

## Drilling Rigs

Drilling rig emissions come from three primary engine types: Draw works, Mud pumps and Generators. Each of these three engine types is used for differing periods of time throughout the drilling process and are likely to have different load factor and sizes. Each of the three engines is also likely to be of differing model years and hence Tier levels. Some drilling rigs operate with a set of large generator engines which provides electric power to the other prime movers of the rig – draw works and mud pumps; these type of rigs are referred to here as diesel-electric rigs. Figure 3-5 shows a drilling rig in the Barnett shale.

In order to account for variations in engine characteristics and their effect in final emissions, average emissions for each type of engine k (k=drawworks, mud pumps or generators) is estimated separately. In addition, operation parameters such as time and load factor may vary for vertical, directional, and horizontal wellbores; hence emissions are estimated separately for both drilling methods using equations 20 and 21. Directional wells are included with vertical wells for purposes of the calculation.

Figure 3-5. Drilling Rig

Emissions for a single engine of type k are estimated according to Equation 20:

Equation 20)			Eengine k, i = EFi×HPk×LFk×tevent×n907,185

where:

Eengine k,i are emissions of pollutant i from an engine type k [ton/spud]

EFi is the  emissions factor of pollutant i [g/hp-hr]

HPk is the  horsepower for an engine k in the county [hp]

LFk is the load factor of the engine k

tevent is the number of hours engine k is used [hr/spud]

n is the number of type-k engines in the typical drill rig

907,185 is the mass unit conversion [g/ton]

The emission factor for pollutant i, EFi, is an emissions factor derived from EPA’s MOVES model and based on the representative population of drilling engine of various tier levels in MOVES. The emissions factor for drill-rig equipment varies by horsepower range, and there are three possible horsepower bins applicable to the typical range of equipment sizes for drill rig engines. Hence, three sets of possible engine emissions factors (by HP) are used.

Emissions from a single drill rig EdrillrigTOTAL, i are estimated in Equation 21 as the sum of individual emissions from each drill rig engine as calculated with Equation 20 in [tons/spud]:

Equation 21)			 EdrillrigTOTAL, i=Eengine k, i

Two distinct drill-rig configurations may be found in various basins:

- Diesel-mechanical (D) drill rigs: in which all k engines are diesel-fueled

- Diesel-electric (DE) powered drill rigs: in which only the generator is powered by diesel and the draw works and mud pumps are electric (and thus do not have direct emissions associated with them)

Thus equations 20 and 21 will vary by these two configurations, and a set of input values for each the four combinations of vertical/horizontal wellbores and diesel/diesel-electric rigs must be applied.

Emissions from drill rigs correlate to the depth of the wellbore, which will vary between horizontal and vertical wellbores; thus emissions can be estimated on a “per foot drilled’ basis using the equation below.

Equation 22)   Edrilling, ivertical/horizontal= EdrillrigTOTAL, iD×(1-FDE)+EdrillrigTOTAL, iDE×FDEDspudverticalhorizontal

where

Edrilling, i is the total emissions for a horizontal or vertical spud per unit of feet drilled [tons/ft]

EdrillrigTOTAL, iD is the emissions from a single diesel-powered drill rig (from Equation 21) for a vertical or a horizontal spud [tons/spud]

FDE is the fraction of drill rigs that are diesel-electric

EdrillrigTOTAL, iDE is the emissions from a single diesel-electric drill rig (from Equation 21) for a vertical or a horizontal spud [tons/spud]

Dspud is the  depth of a vertical or horizontal spud [ft/spud]

Extrapolation to county-level emissions

Emissions per feet drilled are scaled to county-level drilling emissions according to Equation 23.

Equation 23)

Edrill,county-wide, i =Edrilling, ivertical×Dvertical+Edrilling, ihorizontal×Dhorizontal

where:

Edrill ,county-wide,i  is the total emissions of pollutant i from county-wide drilling activity [tons/yr]

Edrilling,i is the total emissions from drilling a single well [tons/ft]

Dvertical is the total depth drilled in the county for vertical wells in a particular year [ft/yr]

Dhorizontal is the total depth drilled in the county for horizontal wells in a particular year [ft/yr]

Example Calculation for Drill Rigs:

Drill rigs are classified as mechanical, or diesel electric. Mechanical rigs typically operate three types of engines during drilling: draw works engines (draw), mud pump engines (mud), and generator engines (gen). Diesel electric rigs are powered by a battery of diesel-electric generator engines. Wells are classified as vertical (a vertical wellbore), directional (a wellbore that is angled or deviates from vertical), and horizontal (after an initial vertical direction, the well is drilled horizontally). No vertical wells were drilled in Cleburne County, and there are no diesel electric rigs. Using the equations provided above, NOx emissions from drilling in Cleburne County, Arkansas were calculated as follows:

Emissions from a draw works engine during horizontal drilling:

Edraw works = EF×HP×LF×tevent×n907,185

where:

Edraw works  = are emissions of NOx from a draw works engine [ton/spud]

EF  = 4.258 [g/hp-hr]

HP = 557.5 [hp]

LF = 0.4 (load factor for the engine)

tevent = 200 [hr/spud]

n = 2 (number of draw work engines in the typical drill rig)

907,185 [g/ton]

Therefore:

Edraw works = 4.258×557.5×0.4×200×2907,185

Edraw works  = 0.42 [ton /spud]

Using similar methodology, emissions for mud pump and generator engines during horizontal drilling were calculated to yield:

Edraw works = 0.42 [ton /spud]

Emud pump = 0.90 [ton /spud]

Egenerator = 1.19 [ton /spud]

Total NOx emissions from all drill rig engines per spud can be evaluated as follows:

EdrillrigTOTAL=Eengine

EdrillrigTOTAL  = 2.51 [ton /spud]

Total NOx emissions on a per foot basis are then calculated using:

Edrillingvertical/horizontal= EdrillrigTOTALD×(1-FDE)+EdrillrigTOTALDE×FDEDspudverticalhorizontal

where

Edrilling is the total emissions for a horizontal or vertical spud per unit of feet drilled [tons/ft]

EdrillrigTOTAL,D = 2.51 [ton /spud]

FDE = 0 (fraction of drill rigs that are diesel-electric)

EdrillrigTOTAL,DE = 0 [ton /spud]

Dspud = 9,318.1 [ft/spud]

Therefore:

Edrilling, horizontal= 2.51×1-0+(0 0)9,318.1

Edrilling, horizontal  = 0.0002693 [ton /ft]

Finally, county-wide emissions may be calculated as follows:

Edrill,county-wide =Edrillingvertical×Dvertical+Edrillinghorizontal×Dhorizontal

where:

Edrill ,county-wide  is the total emissions of NOx from county-wide drilling activity [ton/yr]

Edrilling,vertical = 0 [tons/ft]

Dvertical  = 0 [ft/yr]

Edrilling,horizontal = 0.00002693 [tons/ft]

Dhorizontal = 596,026.5 [ft/yr]

Therefore:

Edrill,county-wide =0.00002693×596,026.5

Edrill ,county-wide = 160.55 [ton /yr]

## Fugitive Leaks

This source category refers to leaking emissions of produced gas that escape through well site and pipeline components such as connectors, flanges, open-ended lines, valves, and compressor wet seals. It must be noted that this source category refers only to fugitive emissions components located at the wellhead and that large transmission pipeline fugitives and other midstream fugitives sources are not part of this analysis. Figure 3-6 shows numerous flanges (circled) and a series of separators at a multi-well pad in the Marcellus shale.15

Figure 3-6. Flanges

Fugitive emissions for an individual typical well are estimated according to Equation 24:

Equation 24)

where:

Efugitive,j is the fugitive emissions for a single typical well for pollutant j [ton/yr/well]

EFi  is the emission factor of TOC for a single component i [kg/hr/component]

Ni is the total number of components of type i

tannual is the annual number of hours the well is in operation [8760 hr/yr]

Yj is the mass ratio of pollutant j to TOC in the vented gas

907.185 is the unit conversion factor kg/ton

In addition, fugitive leaks from wellhead compressor seals can be estimated from the following equations:

Equation 25)

where:

Ecompressor,fug,CH4 is the county-wide CH4 fugitive emissions from compressor seals  [ton/yr]

P is atmospheric pressure [1 atm]

Vvented is the volume of leaked gas per compressor [SCF/compressor/hour]

t is the annual hours of operation for wellhead compressors [hrs/yr]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the pollutant [g/mol]

T is the atmospheric temperature [298 K]

fwellhead is the fraction of wells with wellhead compressors

Nlateral is the number of gas wells served by a lateral compressor engine

Wgas is the county-wide number of gas wells

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

1,000 is the unit conversion factor SCF/MCF

To estimate emissions of other pollutants (VOC, H2S) the following equation may be used:

Equation 26)

where:

E compressor,fug,i is the county-wide compressor fugitive emissions for pollutant i [ton/yr]

EF compressor,fug,CH4  is the compressor fugitive emissions for CH4 [ton CH4/yr]

MWi is the molecular weight of pollutant i [lb/lb-mol]

MWCH4 is the molecular weight of CH4 [lb/lb-mol]

MCH4 is the mole percent of CH4 in the local gas [%]

Mi is the mole percent of pollutant in the local gas [%]

Extrapolation to county-level emissions

County-wide fugitive emissions from well-site piping components are estimated according to Equation 27:

Equation 27)

where:

Efugitive,TOTAL is the total fugitive emissions from well-site piping components in the county [ton/yr]

Efugitive,j is the fugitive emissions for a single well of pollutant j [ton/yr/well] (from Equation 24)

Nwell is the total number of active wells in the county [wells]

Total county-wide fugitive emissions are the sum of compressor seal emissions and component fugitive emissions.

Example Calculation for Fugitive Leaks:

Fugitive emissions at gas well and oil well sites occur from connectors, flanges, open-ended lines, compressor seals, and valves. Using the equations provided above, VOC emissions for fugitive leaks from valves at gas wells in Cleburne County, Arkansas were calculated as follows:

where:

Efugitive is the VOC emissions for a single gas well from valves [ton/yr/well]

EF  = 0.0045 [kg VOC/hr/valve]

N  = 12 [valves/well]

tannual = 8,760 [hr/yr]

Y = 0.036 [VOC to TOC ratio]

907.185 [kg/ton]

Therefore:

E fugitive = 0.0188 [ton/well]

Total VOC emissions from fugitive leaks from valves at gas wells in Cleburne County were calculated as follows:

where:

Efugitive,TOTAL is the total fugitive emissions from valves in Cleburne County [ton/yr]

Efugitive = 0.0188 [ton/yr/well]

Nwell = 490 [wells]

Therefore:

Efugitive,TOTAL = 9.21 [ton/yr]

## Gas-Actuated Pumps

Gas-actuated pumps refer to small gas-driven plunger pumps used at oil and gas production sites, to provide a constant supply of chemicals or lubricants to specific flow lines or equipment. These are regularly used in sites where electric power is unavailable. As part of their operation, gas-driven pumps vent part of the driving gas to the atmosphere, making them a VOC and CH4 emissions source. Two types of gas-actuated pumps were considered: Kimray pumps and chemical injection pumps (CIP). For oil wells only CIPs are assumed to be used. Annual vented gas rates per well from Kimray pumps are estimated following Equation 28:

Equation 28)

where:

Ekimray,CH4 is the per-well CH4 emissions from Kimray pumps [tons- CH4/well-yr]

EFCH4 is the CH4 emissions factor for a Kimray pump per unit throughput [SCF- CH4/MMCF]

Qkimray is the  gas pumped per well annually with Kimray pumps [MMCF/well-yr]

P is the atmospheric pressure [1 atm]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of CH4 [g/mol]

T is the atmospheric temperature [298 K]

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

1,000 is the unit conversion factor SCF/MCF

Emissions from CIPs are estimated based on Equation 29:

# Equation 29)

where:

ECIP,CH4 is the per-well CH4 emissions from CIP pumps [tons- CH4/well-yr]

EFCH4 is the CH4 emissions factor for a CIP pump [SCF- CH4/pump/day]

NCIP  is the number of CIPs per well [pump/well]

tCIP is the regular operation time for chemical injection pumps [hrs/yr]

P is the atmospheric pressure [1 atm]

R is the universal gas constant [0.082 L-atm/mol-K]

MWCH4 is the molecular weight of CH4 [g/mol]

T is the atmospheric temperature [298 K]

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

1,000 is the unit conversion factor SCF/MCF

To estimate emissions from other pollutants (VOC, CO2, H2S, HAPs) from Kimray and CIP pumps, the following equation may be used:

# Equation 30)

where:

E pump,i is the emissions for pollutant i per well from CIPs or Kimray Pumps [ton/well-yr]

Epump,CH4  is the CH4 emissions from CIPs or Kimray Pumps [ton CH4/well-yr] (from Equations 28 or 29)

MWi is the molecular weight of pollutant i [lb/lb-mol]

MWCH4 is the molecular weight of CH4 [lb/lb-mol]

MCH4 is the mole percent of CH4 in the local gas vented from the pump [%]

Mi is the mole percent of pollutant in the local gas vented from the pump [%]

Extrapolation to county-level emissions

To estimate county-wide annual emissions from gas-actuated pumps for each pollutant, the scaling surrogate used is well counts, according to Equation 31:

Equation 31)

EGAP,   i=(ECIP,   i+Ekimray, i) × Wgasgas wells+ ECIP,   i × Woiloil wells

where:

EGAP, i is the annual county-wide emissions for pollutant i from gas-actuated pumps [ton/yr]

ECIP, i is the emissions from chemical injection pumps per well type (gas or oil) [ton/yr-well]

Ekimray, i is the emissions from kimray pumps per well [ton/yr-well]

Wgas is the number of active gas wells in a particular county [wells]

Woil is the number of active oil wells in a particular county [wells]

Example Calculation for Gas-Actuated Pumps:

Using the equations provided above, VOC emissions for gas-actuated pumps in Cleburne County, Arkansas were calculated as follows:

Kimray Pumps

where:

Ekimray,CH4 is the per-well CH4 emissions from Kimray pumps at gas wells [tons-CH4/well-yr]

EFCH4 = 1,041 [SCF- CH4/MMCF]

Qkimray = 42.9 [MMCF/well-yr]

P = 1 [atm]

R = 0.082 [L-atm/mol-K]

MWCH4 = 16.04 [g/mol]

T = 298 [K]

907,185 [g/ton]

1,000 [SCF/MCF]

3.5x10-5 [MCF/L]

Therefore:

E kimray, CH4 = 0.923 [tons CH4/well/yr]

VOC emissions are then calculated using:

where:

E kimray is the emissions of VOC per well from Kimray Pumps [ton/well-yr]

EFkimray, CH4  = 0.923 [ton CH4/well-yr]

MWVOC = 52.1 [lb/lb-mol]

MWCH4 = 16.04 [lb/lb-mol]

MCH4 = 0.94 [percent CH4, expressed as a fraction]

MVOC = 0.01 [percent VOC, expressed as a fraction]

Therefore:

E kimray = 0.032 [ton/well-yr]

Chemical Injection Pumps

where:

ECIP, CH4 is the per-well CH4 emissions from CIP pumps at gas wells [tons- CH4/well-yr]

EFCH4 = 260 [SCF- CH4/pump/day]

NCIP = 0.142 [pump/well]

tCIP = 8,760 [hrs/yr]

P = 1 [atm]

R = 0.082 [L-atm/mol-K]

MWCH4 = 16.04 [g/mol]

T = 298 [K]

907,185 [g/ton]

1,000 [SCF/MCF]

3.5x10-5 [MCF/L]

Therefore:

E CIP, CH4 = 0.279 [tons CH4/well/yr]

Using the same methodology as above for Kimray pumps, VOC emissions from CIP pumps are estimated as:

E CIP =  0.011 [ton/well/yr]

Total VOC emissions from all gas-actuated pumps in Cleburne County can be evaluated as follows:

EGAP=(ECIP+Ekimray) × Wgasgas wells+ ECIP × Woiloil wells

where:

EGAP is the annual county-wide VOC emissions from gas-actuated pumps [ton/yr]

ECIP = 0.011 [ton/yr-well]

Ekimray = 0.032 [ton/yr-well]

Wgas  = 490 [wells]

Woil = 0 [wells]

Therefore:

EGAP=(0.011+0.032) × 490gas wells+ 0.011 × 0oil wells

E GAP  = 21.1 [ton/yr]

## Heaters

This category refers to natural gas-fired external combustors used in oil and gas production facilities to provide heat input to separators (separator heaters or heater treaters), to prevent the formation of hydrates during pressure reductions (line heaters), or to provide heat to tanks (tank heaters). This category does not refer to reboilers used in dehydrators as those emissions are captured in the dehydrator source category. Figure 3-7 shows a line heater at a natural gas well in the Marcellus shale.15

Figure 3-7. Line Heater

The basic methodology for estimating emissions for all pollutants except SO2 for a single heater is shown in Equation 32. Local fuel gas properties will vary between gas wells and oil wells; hence emissions are estimated separately for this category. Due to limited field data for this category, all other parameters unrelated to local gas composition were assumed to be the same for gas and oil wells.

Equation 32)

where:

Eheater is the emissions from a given heater [ton/yr]

EFheater is the emission factor for a heater for a given pollutant [lb/million SCF]

Qheater is the heater MMBTU/hr rating [MMBTUrated/hr]

tannual is the annual hours of operation [hr/yr]

hc is a heater cycling fraction to account for the fraction of operating hours that the heater is firing (if not available, hc=1)

HV is the local natural gas heating value [BTUlocal/SCF]

2,000 is the unit conversion factor lb/ton

The methodology for estimating SO2 emissions from heaters requires first estimating the mass of gas combusted in the heater, and then uses the mass fraction of H2S in the gas and the assumption that all H2S is converted to SO2. This methodology is described in Equation 33.

Equation 33)

where:

is the SO2 emissions from a given heater [ton-SO2/yr]

is the mass fraction of H2S in the gas

Qheater is the heater MMBTU/hr rating [MMBTUrated/hr]

tannual is the annual hours of operation [hr/yr]

hc is a heater cycling fraction to account for the fraction of operating hours that the heater is firing

HV  is the local natural gas heating value [MMBTUlocal/scf]

P is atmospheric pressure [1 atm]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the gas [g/mol]

T is the atmospheric temperature [298 K]

3.5x10-3 is the unit conversion factor SCF/L

907,185 is the unit conversion factor g/ton

1,000 is the unit conversion factor SCF/MCF

Extrapolation to county-level emissions

County-wide heater emissions are estimated by determining the typical number of heaters per well and scaling up by well count. This is shown in Equation 34:

Equation 34)

where:

Eheater,TOTAL is the total heater emissions in a county for a specific pollutant [ton/yr]

Eheater is the total emissions from a single heater for a specific pollutant [ton/yr]

Nheater is the typical number of heaters per well throughout in the county

WTOTAL is the total number of wells in the county

Example Calculation for Heaters - Gas:

Using the equations provided above, NOx emissions from heaters at gas wells in Cleburne County, Arkansas were calculated as follows:

where:

E heater = emissions from a single heater [ton /yr]

EF heater = 100 [lb NOx/MMCF]

Q heater = 0.61 [MMBtu/hr]

t annual = 8,760 [hr/yr]

hc = 1 (heater cycling fraction of operating hours that the heater is firing)

HV  = 1,035 [MMBtu/MMCF]

2,000 [lb/ton]

Therefore:

E heater = 0.258 [ton/heater/yr]

Total NOx emissions from all heaters in Cleburne County can be evaluated as follows:

where:

E heater, TOTAL = total emissions from heaters [ton/yr]

E heater = 0.258 [ton/heater/yr]

N heater = 0.5 [heaters/well]

WTOTAL = 490 [wells]

Therefore:

E heater, TOTAL = 63.21 [ton/yr]

## Hydraulic Fracturing Pumps

This category refers to equipment used in hydraulic fracturing practices during well completions and recompletions, generally related to unconventional oil and gas production such as shale gas and tight sands oil/gas. Engines used during hydraulic fracturing are generally large diesel-fueled pumps that can be a significant NOx emissions source. Figure 3-8 shows hydraulic fracturing of three wells in the Marcellus shale.15 The hydraulic fracturing pump engines are lined up on the red tractor trailer rigs.

Figure 3-8. Hydraulic Fracturing

Average emissions factors for hydraulic fracturing engines were derived from EPA’s MOVES model based on the oil equipment source category bin for nonroad engines. The basic methodology for estimating exhaust emissions from engines used at a hydraulic fracturing event is shown below:

Equation 35)

where:

Efracing,event is the exhaust emissions for pollutant i from a single fracing event [ton/event]

n is the number of engines used per fracing event

EFi is the emissions factor of pollutant i [g/hp-hr]

HP is the horsepower of the engine [hp]

LF is the load factor of the engine

Nstages is the number of stages per fracing event [stage/event]

tstage is the duration of the fracturing stage [hr/stage]

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

Fracing pump emissions can be scaled up to the county level on the basis of horizontal spuds. It is assumed that hydraulic fracturing is performed in all horizontal spuds and thus the methodology for scaling up fracturing pump engine emissions is based on this surrogate as shown below:

Equation 36)

where:

Efrac,pump,TOTAL is the total emissions from fracing pump engines in the county [ton/yr]

Nevents is the number of unconventional well completions in a particular year [spuds/yr]

Efracing,event is the total exhaust emissions from engines in a single fracing event [ton/event]

Example Calculation for Hydraulic Fracturing Pumps:

Using the equations provided above, NOx emissions from hydraulic fracturing pumps in Cleburne County, Arkansas were calculated as follows:

where:

E fracing,event,i = emissions from a single fracturing event [ton/event]

n = 8.5 [engines/event]

EF = 5.831 [g/hp-hr]

HP = 2,033 [hp]

LF = 0.688 (load factor for the engine)

N stages = 10.5 [stages/event]

t stage = 2.25 [hr/stage]

907,185 [g/ton]

Therefore:

E fracing = 1.81 [ton/event]

Total NOx emissions from all hydraulic fracturing pumps in Cleburne County can be evaluated as follows:

where:

E fracing, TOTAL = total emissions from hydraulic fracturing pumps in a county [ton/yr]

E fracing = 1.81 [ton/event]

N events = 133 [spuds/yr]

Therefore:

E fracing, TOTAL = 241 [ton/yr]

## Lateral/Gathering Compressor Engines

Lateral compressor engines are used to gather gas from multiple individual wells, generally serving groups of approximately 10 to 100 wells. These engines are generally medium size and larger than wellhead compressor engines, but often not large enough to trigger Title V or other major source permitting requirements. Lateral compressor engines were categorized into two main categories and thus emissions are estimated for each type of engine and consequently extrapolated to county-wide emissions. These categories of compressors are:

- Rich burn compressors

- Lean burn compressors

Figure 3-9 shows a large, lateral compressor engine operating in the Barnett shale.

Figure 3-9. Lateral Compressor Engine

The basic methodology for estimating emissions from lateral compressor engines is shown in Equation 37:

Equation 37)

where:

Eengine,type are emissions from a particular type (rich vs. lean) of compressor engine [ton/yr/engine]

EFi is the emissions factor of pollutant i [g/hp-hr] (note that this value may be differ between rich-burn vs. lean-burn engines)

HP is the horsepower of the engine [hp]

LF is the load factor of the engine

tannual is the annual number of hours the engine is used [hr/yr]

Fcontrolled is the fraction of lateral compressors of a particular type that are controlled

CFi is the control factor for controlled engines for pollutant i

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

County-level emissions are represented by a mix of the two types of lateral compressors. Single engine emissions are scaled to county level using the fraction (F) of these engine types to total engines, the fraction of wells served by lateral compressor engines, and the total gas well count in a county, according to equation below:

Equation 38)

where:

Eengine,TOTAL is the total emissions from compressor engines in a county [ton/yr]

Frich is the fraction of rich-burn lateral compressors in the county amongst all lateral compressors

Eengine,rich is the total emissions from a single rich burn compressor engine per Equation (37) [ton/yr]

Flean is the fraction of lean-burn lateral compressors in the county amongst all lateral compressors

Eengine,lean is the total emissions from a single lean burn compressor engine per Equation (37) [ton/yr]

Wgas is the total gas well count in a county

Nlateral is the number of gas wells served by a lateral compressor engine

Example Calculation for Rich-Burn Lateral Compressor:

Using the equations provided above, NOx emissions from rich-burn lateral compressor engines in Cleburne County, Arkansas were calculated as follows:

where:

E engine, rich = emissions from a rich-burn lateral compressor engine [ton/yr/engine]

EF  = 8.24 [g/hp-hr]

HP = 97.0 [hp]

LF = 0.74 (load factor for the engine)

tannual = 8,760 [hr/yr]

Fcontrolled = 0.44 (fraction controlled)

CF = 0.90 (control factor)

907,185 [g/ton]

Therefore:

E engine, rich  = 3.45 [ton/yr/engine]

Total NOx emissions from all rich-burn lateral compressor engines in Cleburne County can be evaluated as follows:

where:

E engine, rich, TOTAL  = total emissions from rich-burn lateral compressor engines in a county [ton/yr]

F rich = 0.490 (fraction of rich burn engines)

E engine, rich = 3.45 [ton/yr/engine]

W gas = 490 [wells]

N lateral = 32.05 (number of gas wells served by a lateral compressor engine)

Therefore:

E engine, rich, TOTAL  = 25.8 [ton/yr]

## Liquids Unloading

This source category refers to emissions from venting gas from gas wells to prevent liquid build-up in the well that could limit production. This practice is also commonly referred as “well blowdowns”. Vented gas from liquids unloading is a VOC emissions source. Some wells use plunger lifts for liquids unloading, which can also result in vented emissions. Liquids unloading emissions may be controlled by a combustion device such as a flare, or may also be controlled by a variety of devices and practices that reduce venting from the liquids unloading. Figure 3-10 shows 2 wells equipped with plunger lifts.

Figure 3-10. Plunger Lifts

Emissions from liquids unloading are based on the average venting volume per liquids unloading and the gas composition of the vented gas. The calculation methodology for estimating emissions from a single liquids unloading event is shown below in Equation 39:

Equation 39)

where:

Eliquids unloading,i is the emissions of pollutant i from a single liquids unloading event [ton/event]

P is atmospheric pressure [1 atm]

Vvented is the volume of vented gas per liquids unloading [MCF/event]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the gas [g/mol]

T is the atmospheric temperature [298 K]

fi is the mass fraction of pollutant i in the vented gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Emissions from flare controls for liquids unloading vents

In areas where flaring is used to control liquids unloading vents, the methodology for estimating flaring emissions is described below:

Equation 40)

where:

Eflare,liquids unloading is the county-wide flaring emissions of pollutant i for liquids unloading [ton/yr]

EFi is the flaring emissions factor for pollutant i [lb/MMBtu]

Vvented is the volume of vented gas per liquids unloading [MCF/event]

F is the fraction of well liquids unloading that are flared

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

HV is the local heating value of the gas [BTU/SCF]

Wgas is the county-wide number of active gas wells for a particular year [wells]

Nblowdown the number of annual blowdowns per well in the county [event/yr-well]

1,000 is the unit conversion factor MCF/MMCF

2,000 is the unit conversion factor lb/ton

The methodology for estimating SO2 emissions from flaring of liquids unloading gas is shown below:

Equation 41)

where:

is the county-wide SO2 flaring emissions from flaring of liquids unloading vent gas [ton/yr]

P is atmospheric pressure [1 atm]

Vvented is the volume of vented gas per liquids unloading [MCF/event]

Wgas is the county-wide number of gas wells [wells]

Nblowdown the number of annual blowdowns per well in the county [event/yr-well]

F is the fraction of liquids unloading with flares

Ccaptured  is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the liquids unloading gas [g/mol]

T is the atmospheric temperature [298 K]

is the mass fraction of H2S in the liquids unloading venting gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

The U.S. Inventory of Greenhouse Gas Emissions and Sinks (U.S. GHG Inventory) was updated in 2014 to reflect newly available data on emissions from liquids unloading. Specifically, EPA analyzed a report issued in September of 2012 by the American Petroleum Institute (API) and America’s Natural Gas Alliance (ANGA) entitled “Characterizing Pivotal Sources of Methane Emissions from Natural Gas Production”. Using data presented in the report, EPA developed updated vent rates (Vvented  in Equation 40) for liquids unloading activities based on U.S. Energy Information Administration (EIA) Supply Regions. Figure 3-11 below shows the six EIA Supply Regions used in the U.S. GHG Inventory.

Figure 3-11. EIA Supply Region Map

Table 3-2 below shows the vent rates (Vvented  in Equation 40) by EIA Supply Region for each venting scenario used in the U.S. GHG Inventory.

| Table 3-2. Liquids Unloading Vent Rates from the U.S. GHG Inventory |  |  |  |  |
| --- | --- | --- | --- | --- |
| EIA Supply Region | Wells Venting with Plunger Lift (%) | Wells Venting without Plunger Lift (%) | Vent Rate for Wells with Plunger Lift (scf/yr/well) a | Vent Rate for Wells without Plunger Lift (scf/yr/well) a |
| North East | 4.3 | 11.26 | 314,626 | 166,174 |
| Midcontinent | 2.33 | 4.14 | 1,379,958 | 230,199 |
| Rocky Mountain | 12.88 | 1.52 | 154,300 | 2,579,444 |
| South West | 3.32 | 19.47 | 3,547 | 96,748 |
| West Coast | 7.59 | 6.80 | 345,343 | 304,048 |
| Gulf Coast | 2.32 | 7.08 | 70,021 | 300,592 |

a	Whole gas vent rates.

In order to utilize this information within the structure and methodology used in the tool, a weighted vent rate was developed for all wells in a county. Calculation of a weighted vent rate was accomplished using the data in Table 3-2. For example, the updated default liquids unloading vent rate for the North East EIA Supply Region is calculated as follows (using the 2011 value of 153,773 wells in the North East as shown in Table 3-3):

= 32,421 (scf/yr/well)

Table 3-3 shows the resultant default vent rates used in the tool (data from the West Coast Region has been used for the State of Alaska). As these are annual vent rates, where this information is used in the tool, the frequency of liquids unloading venting has been set equal to one event per year. Additionally, as these rates reflect some level of control (through the use of plunger lifts), where this information is used in the tool, a value of “NA” is used for the control method, and no additional reduction from use of controls has been applied.

| Table 3-3. Default Liquids Unloading Vent Rates for the Tool |  |  |
| --- | --- | --- |
| EIA Supply Region | Gas Well Count | Default Vent Rate for all Wells (scf/yr/well) |
| North East | 153,773 | 32,421 |
| Midcontinent | 87,193 | 41,659 |
| Rocky Mountain | 58,285 | 59,047 |
| South West | 41,919 | 18,956 |
| West Coast | 1,516 | 46,884 |
| Gulf Coast | 71,629 | 22,913 |

Extrapolation to county-level emissions

The total county-level emissions from all liquids unloading are evaluated following Equation 42:

Equation 42)

where:

Eliquids unloading,TOTAL are the total county-wide emissions of pollutant i from liquids unloading [tons/yr]

Eliquids unloading,i are the liquids unloading emissions from a single liquids unloading event [tons/event]

Nblowdowns is the number of annual blowdowns per well in the county [event/yr-well]

Wgas is the total number of active gas wells in the county for a particular year [well]

Fcontrol,device is the fraction of liquids unloading in the county that were controlled

Cefficiency is the control efficiency of the control technology used (plunger lifts for example)

Example Calculation for Liquids Unloading:

Using the equations provided above, VOC emissions from liquids unloading in Cleburne County, Arkansas were calculated as follows:

where:

E liquidsunloading = emissions from a single liquids unloading event [ton/event]

P = 1 [atm]

V vented = 5.9375 [MSCF/event]

R = 0.082 [L-atm/mol-K]

MW gas = 17.3066 [g gas/mole gas]

T = 298 [K]

f = 0.03429 [VOC fraction]

3.5x10-5 [MCF/L]

907,185 [g/ton]

Therefore:

E liquidsunloading  = 0.004541 [ton/event]

In this example, liquids unloading emissions are controlled through the use of a Plunger lift, ESP, or Beam Pump.

Therefore, total VOC emissions from liquids unloading venting in Cleburne County were calculated as follows:

where:

Eliquidsunloading,TOTAL are the total county-wide emissions of VOC from blowdowns [ton/yr]

Eliquidsunloading = 0.004541 [ton/event]

Nblowdown = 64 [event/yr-well]

Wgas  = 490 [wells]

Fcontrol,device = 0.3769 (fraction controlled)

Cefficiency = 0.7063 (control efficiency expressed as fraction)

Therefore:

E liquidsunloading,TOTAL  = 104.5 [ton/yr]

Note that if liquids unloading emissions were controlled through the use of flares, flaring emissions would be calculated using equations 40 and 41.

## Loading

This category refers to loading losses that occur when transferring hydrocarbon liquids, crude oil or condensate, from storage tanks to cargo trucks. Figure 3-12 shows truck loading operations at a tank battery in Mississippi.

Figure 3-12. Truck Loading Operations

The emissions from loading operations will vary by the gas speciation of the working losses; hence emissions were calculated separately for each hydrocarbon liquid. Equations 43-46 may be used for both categories (SCCs). The loading loss rate is estimated following Equation 43:

Equation 43)

where:

L is the loading loss rate [lb/1,000gal]

S is the saturation factor taken from AP-42 default values based on operating mode (here assumed as submerged loading: dedicated normal service)

V is the true vapor pressure of the liquid loaded [psia]

MWgas is the molecular weight of the vapor [lb/lb-mole]

T is the temperature of the bulk liquid [oR]

VOC truck loading emissions are then estimated by Equation 44 which is dependent on the VOC fraction in the gas. When available, county-specific working/breathing gas compositions from condensate/crude oil storage tanks were used in Equations 44-46; however when county-level data was limited or unavailable, produced gas analyses were used to speciate emissions from each pollutant.

Equation 44)			 Eloading,   VOC=L1,000 ×Yvoc×422,000

where:

Eloading,VOC are the VOC tank loading emissions [ton-VOC/bbl]

L is the loading loss rate [lb/1,000gal]

YVOC is the weight fraction of VOC in the vapor in the liquid loaded

42 is a unit conversion [gal/bbl]

2,000 is a unit conversion [lbs/ton]

Emissions of other pollutants are calculated based on Equation 45:

Equation 45) 		Eloading,i= Eloading,VOC ×weight fractioniweight fractionVOC

# where:

E loading,i  is the total loading emissions of pollutant “i” per barrel of liquid [ton/bbl]

(weight fractioni/weight fractionvoc) is the mass-based weight fraction of pollutant i divided by the weight fraction of VOC in the gas

Extrapolation to county-level emissions

Annual emissions per pollutant i from condensate loading were scaled to county-level by annual condensate production per Equation 46:

Equation 46)		Etank loadout,   i=Eloading,   i ×  Pcondensate×Ftrucked

where:

Etank loadout, i is the annual county-level emissions for pollutant i from condensate tank load-out [ton/yr]

Eloading, i is the emissions for pollutant i from loading per barrel [ton/bbl]

P condensate is the total annual of barrels condensate produced county-wide [bbl/yr]

Ftrucked is the fraction of condensate production that is delivered by truck

Annual emissions per pollutant i from oil loading were scaled to county-level by annual oil production per Equation 47:

Equation 47)		Etank loadout, oil,   i=Eloading,   i ×  Poil ×Ftrucked

where:

Etank loadout, i is the annual county-level emissions for pollutant i from crude oil tank load-out [ton/yr]

Eloading, i is the emissions for pollutant i from loading per barrel [ton/bbl]

P oil is the total annual county-wide oil production [bbl/yr]

Ftrucked is the fraction of oil production that is delivered by truck

Example Calculation for Loading:

Using the equations provided above, VOC emissions for condensate loading in Columbia County, Arkansas were calculated as follows:

where:

L is the loading loss rate [lb/1,000gal]

S = 0.6 (based on submerged loading: dedicated normal service)

V = 5.12 [psia]

MWgas = 54.2 [lb/lb-mole]

T = 540 [oR]

Therefore:

L= 3.84 [lb/1,000gal]

Total VOC emissions from all condensate loading in Columbia County can be evaluated as follows:

Eloading,VOC=L1,000 ×Yvoc×422,000

where:

Eloading,VOC are the VOC tank loading emissions [ton-VOC/bbl]

L = 3.84 [lb/1,000gal]

YVOC = 0.933

42 [gal/bbl]

2,000 [lb/ton]

Therefore:

Eloading=3.841,000 ×0.933×422,000

Eloading = 0.0000752 [ton-VOC/bbl]

Annual emissions of VOC from condensate loading are then scaled to the county-level using:

Etank loadout=Eloading,VOC ×  Pcondensate×Ftrucked

where:

Etank loadout is the annual county-level emissions of VOC from condensate tank load-out [ton/yr]

Eloading,VOC = 0.0000752 [ton-VOC/bbl]

P condensate = 275,892 [bbl/yr]

Ftrucked = 1

Therefore:

Etank loadout=0.0000752 ×  275,892×1

Etank loadout = 20.76[ton/yr]

## Mud Degassing

Drilling mud degassing refers to the practice of extracting the entrained gas from the drilling mud once it is outside of the wellbore. During this process VOCs and CH4 (and other pollutants in the gas) are vented to the atmosphere. National default emissions factors for mud degassing are available from The Climate Registry Reporting Protocol as shown in Table 3-4:

| Table 3-4. National Default Emissions Factors for Mud Degassing by Mud Base |  |  |
| --- | --- | --- |
| Emission Source | Emission Factor Units | Emission Factor Units |
| Mud degassing – water-based mud | 881.84 lbs THC / drilling day | 0.2605 tonnes CH4/ drilling day |
| Mud degassing – oil-based mud | 198.41 lbs THC / drilling day | 0.0586 tonnes CH4/ drilling day |
| Mud degassing – synthetic mud | 198.41 lbs THC / drilling day | 0.0586 tonnes CH4/ drilling day |

Water-based mud emissions factors were assumed as a default conservative value, but this parameter may be updated in the tool. To account for the use of different mud bases within a region, the CH4 emissions factor may be estimated as a weighted average based on a usage fraction of each mud type within a county.

Applying the local-gas CH4 mass fraction to the mud degassing emission factors provides the site-representative emissions as shown in Equation 48. Because the mud entrained gas is the gas coming out directly from the wellbore during drilling, produced gas compositions by well type are used to characterize these emissions. Equations 48-49 are applicable to both oil and gas wells mud degassing emissions, however gas compositions and surrogate values (spuds) will vary for each well type.

Equation 48)

where:

E mudgas,CH4 is the mud degassing emissions for CH4 per spud [ton/spud]

Ndrill is the number of drilling days per spud [drilling days/spud]

EFmud,CH4  is the emissions factor for CH4 [tonne CH4/drilling days]

0.8385 is the mole percent of CH4 from the vented gas used to derive the emissions factor (EF)

MCH4 is the mole percent of CH4 in the local gas vented during mud degassing [percent, expressed as a fraction] (if county-specific CH4 emissions factor is used, M=0.8385)

1.102 is the conversion of tonnes to short tons

To estimate emissions from other pollutants in the vented gas Equation 49 may be used:

Equation 49)

where:

E mudgas,i is the mud degassing emissions for pollutant i per spud [ton/spud]

EFmudgas, CH4  is the vented emissions for CH4 [ton CH4/spud]

MWi is the molecular weight of pollutant i [lb/lb-mol]

MWCH4 is the molecular weight of CH4 [lb/lb-mol]

MCH4 is the mole percent of CH4 in the local gas vented during mud degassing [percent, expressed as a fraction]

Mi is the mole percent of pollutant in the local gas vented during mud degassing [percent, expressed as a fraction]

Extrapolation to county-level emissions

To estimate county-wide annual emissions, mud degassing emissions by spud are scaled with the county-wide count of drilling events (spuds), according to Equation 50:

Equation 50)			Emudgas,TOTAL   i=Emudgas,   i × Sspuds

where:

Emudgas,TOTAL,i is the annual county-wide emissions for pollutant i from mud degassing [ton/yr]

Emudgas, i is the emissions from mud degassing from a drilling event [ton/spud]

Sspuds is the number of wells drilled in a county for a particular year [spud/yr]

Example Calculation for Mud Degassing:

Using the equations provided above, VOC emissions for mud degassing in Cleburne County, Arkansas were calculated as follows:

where:

E mudgas, CH4 is the mud degassing emissions for CH4 per spud [ton/spud]

Ndrill = 20.22 [drilling days/spud]

EFmudgas, CH4  = 0.2605 [tonnes CH4/drilling days]

MCH4 = 0.94 [percent, expressed as a fraction]

0.8385 = [mole fraction CH4 used to derive emission factor]

1.102 [ton/tonnes]

Therefore:

E mudgas, CH4 = 6.51 [tons CH4/well/yr]

VOC emissions are then calculated using:

where:

E mudgas,VOC is the emissions of VOC per completion [ton/completion]

Emudgas,CH4  = 6.51 [ton CH4/well-yr]

MWVOC = 52.1 [lb/lb-mol]

MWCH4 = 16.04 [lb/lb-mol]

MCH4 = 0.94 [percent CH4, expressed as a fraction]

MVOC = 0.01 [percent VOC, expressed as a fraction]

Therefore:

E mudgas,VOC = 0.225 [ton/well-yr]

Total VOC emissions from all mud degassing in Cleburne County can be evaluated as follows:

Emudgas,TOTAL=Emudgas,VOC × Sspuds

where:

Emudgas,TOTAL is the annual county-wide VOC emissions from mud degassing [ton/yr]

Emudgas,VOC = 0.225 [ton/spud]

Sspuds = 133 [spud/yr]

Therefore:

Emudgas,TOTAL=0.225 × 133

Emudgas,TOTAL = 29.93 [ton/yr]

## Pneumatic Devices

Pneumatic devices are located at the well site and use high-pressure produced gas to produce mechanical motion. These devices are typically under operation throughout the year and they may or may not vent the working fluid during operation, making them a potentially significant source of VOC emissions. Figure 3-13 shows a pneumatic valve at a well in the Marcellus shale.15

Figure 3-13. Pneumatic Valve

The counts of pneumatic devices vary between oil and gas wells, thus emissions are estimated separately for both well types. Emissions from pneumatic devices vary by the bleed rate of the device. Here it is assumed that four configurations can be found in a typical well: high bleed, low bleed, intermittent and no bleed. Emissions for the first three types of device i must be estimated. The methodology for estimating the emissions from pneumatic devices for a particular type of well are shown in Equation 51:

Equation 51)

where:

Epneumatic,j is the total emissions of pollutant j from all pneumatic devices for a particular type of well (oil vs. gas) [ton/yr/well]

fj is the mass fraction of pollutant j in the vented gas (produced gas)

is the volumetric bleed rate from device i [SCF/hr/device]

Ni is the number of devices i found in a type of well (oil vs. gas) [devices/well]

tannual is the  number of hours per year that devices were operating [8760 hr/yr]

P is the atmospheric pressure [1 atm]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the gas [g/mol]

T is the atmospheric temperature [298 K]

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

1,000 is the unit conversion factor SCF/MCF

Extrapolation to county-level emissions

County-wide pneumatic device emissions for each well type are estimated according to Equation 52:

Equation 52)

where:

Epneumatic,TOTAL,j is the total pneumatic device emissions of pollutant j in the county [ton/yr]

Epneumatic,j is the pneumatic device emissions of pollutant j for a type of well (gas vs. oil) [ton/yr/well]

Wgas or oil is the total number of active gas (or oil) wells in the county [wells]

Total emissions from pneumatic devices will be the combination of county-wide emissions from each well type:

Equation 53)

Subpart W of the GHGRP prescribes bleed rates for low bleed, high bleed, and intermittent bleed devices that are to be used by reporters to estimate emissions. These rates, shown in Table 3-5 below, have been incorporated into the tool as default bleed rates for pneumatic devices used at oil and gas wells.

| Table 3-5. Whole Gas Bleed Rates for Pneumatic Devices |  |
| --- | --- |
| Onshore petroleum and natural gas production | Bleed Rate (scf/hour/component) |
| Low Bleed Pneumatic Devices | 1.39 |
| High Bleed Pneumatic Devices | 37.3 |
| Intermittent Bleed Pneumatic Devices | 13.5 |

The U.S. GHG Inventory utilizes per-well pneumatic device counts that are used in the tool. For gas wells, the total device counts in the U.S. GHG Inventory were used to derive default device counts by device type using the distribution between low, intermittent, and high bleed devices found in the CenSARA inventory and survey effort. The updated default device counts are shown in Table 3-6 below for each EIA Supply Region. (Note that for oil wells, the total device counts by device type will be updated in future inventories as EPA has identified a calculation error for the oil well device counts shown in Table 3-6.)

| Table 3-6. Pneumatic Device Counts for Oil and Gas Wells |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EIA Supply Region | Oil Well Device Counts |  |  | Gas Well Device Counts |  |  |
|  | Low Bleed | High Bleed | Intermittent Bleed | Low Bleed | High Bleed | Intermittent Bleed |
| North East | 0.495 | 0.267 | 0 | 0.144 | 0.222 | 0.120 |
| Midcontinent | 0.495 | 0.267 | 0 | 0.460 | 0.709 | 0.382 |
| Rocky Mountain | 0.495 | 0.267 | 0 | 0.434 | 0.669 | 0.360 |
| South West | 0.495 | 0.267 | 0 | 0.394 | 0.607 | 0.327 |
| West Coast | 0.495 | 0.267 | 0 | 0.297 | 0.458 | 0.247 |
| Gulf Coast | 0.495 | 0.267 | 0 | 0.206 | 0.318 | 0.171 |

Example Calculation for Pneumatic Devices:

Using the equations provided above, VOC emissions from low-bleed pneumatic devices located at gas wells in Cleburne County, Arkansas were calculated as follows:

where:

Epneumatic,VOC,well is the total emissions of VOC from low-bleed pneumatic devices [ton/yr/well]

f = 0.0342 [VOC fraction]

= 3.151 [SCF/hr/device]

N  = 0.99 [devices/well]

tannual = 8,760 [hr/yr]

P = 1 [atm]

R = 0.082 [L-atm/mol-K]

MWgas = 17.31 [g/mol]

T = 298 [K]

3.5x10-5 [MCF/L]

907,185 [g/ton]

1,000 [SCF/MCF]

Therefore:

Epneumatic,VOC,well = 0.021 [ton/yr/well]

VOC emissions from low-bleed pneumatic devices located at gas wells in Cleburne County can be evaluated as follows:

where:

Epneumatic,VOC,TOTAL is the total pneumatic device emissions of VOC from low-bleed pneumatic devices located at gas wells in Cleburne county [ton/yr]

Epneumatic,VOC,well = 0.021 [ton/yr/well]

Wgas = 490 [wells]

Therefore:

Epneumatic,VOC = 10.3 [ton/yr]

## Produced Water Tanks

Water tank emissions are generated by working and breathing processes from tanks used to store produced water. Figure 3-14 shows produced water tanks in the Barnett Shale.

Figure 3-14. Produced Water Tanks

Because information on oil and gas field handling of produced water is limited, emissions from this source were assumed uncontrolled. The methodology for estimating water tank emissions is shown below separately for gas wells and oil wells as water production and gas compositions for each well-type will differ:

Gas well water tanks

Equation 54)

where:

Ewater,gaswells,CH4 is the county-wide annual CH4 emissions from water tanks located at gas wells [tons/yr]

EFwater,tanks,CH4 is the emissions factor for CH4 from working/breathing losses from water tanks in gas well sites [lb/bbl]

Pwater,gas is the county-wide annual water production [bbl/yr] from gas wells

Ftank  is the fraction of produced water directed to tanks [%]

2,000 is the unit conversion factor lbs/ton

Oil well water tanks

Equation 55)

where:

Ewater,oil wells,CH4 is the county-wide annual CH4 emissions from water tanks located at oil wells [tons/yr]

EFwater,LPwells,CH4 is the emissions factor for CH4 from working/breathing losses from water tanks at low pressure oil wells (i.e. wells with artificial lifts) [lb/bbl]

EFwater,RPwells,CH4 is the emissions factor for CH4 from working/breathing losses from water tanks at regular pressure oil well sites [lb/bbl]

F is the fraction of water production from oil wells with artificial lifts

Ftank is the fraction of produced water directed to tanks [%]

Pwater,oil is the annual county-wide water production [bbl/yr] from oil wells

2,000 is the unit conversion factor lbs/ton

To estimate emissions of other pollutants in the losses from water tanks, the following equation may be used:

Equation 56)

where:

E water,wells,i is the water tank county-wide venting losses of pollutant i from water tanks at particular well type (oil or gas) [ton/yr]

EFwater,wells,CH4  is the water tank emissions for CH4 for a particular well type [ton CH4/yr]

MWi is the molecular weight of pollutant i [lb/lb-mol]

MWCH4 is the molecular weight of CH4 [lb/lb-mol]

MCH4 is the mole percent of CH4 in the water tanks gas (local produced gas) [%]

Mi is the mole percent of pollutant in the water tanks gas (local produced gas) [%]

Extrapolation to county-level emissions

County-wide emissions from produced water tanks are estimated directly from equations 55 through 57. The sum of oil wells and gas wells water tank emissions yield total county-wide emissions from water tanks.

Example Calculation for Produced Water Tanks:

Using the equations provided above, VOC emissions for produced water tanks in Columbia County, Arkansas were calculated as follows:

Venting emissions (CH4) from gas wells:

where:

Ewater,gaswell is the county-wide annual CH4 emissions from water tanks located at gas wells [ton/yr]

EFwater,tank = 0.11 [lb CH4/bbl]

Pwater,gas = 1,234,207 [bbl/yr]

Ftank = 1 [%]

2,000 [lb/ton]

Therefore:

E water, gaswell = 67.9 [tons CH4/yr]

VOC emissions are then calculated using:

where:

E water,gaswell,VOC is the emissions of VOC from produced water at gas wells [ton/yr]

EFwater,gaswell  = 67.9 [tons CH4/yr]

MWVOC = 59.5 [lb/lb-mol]

MWCH4 = 16.04 [lb/lb-mol]

MCH4 = 0.89 [percent CH4, expressed as a fraction]

MVOC = 0.04 [percent VOC, expressed as a fraction]

Therefore:

E water,gaswell,VOC = 11.32 [ton/yr]

## Well Completions

This category refers to emissions from well completions events, which includes initial completions and recompletions. Data provided in the HPDI database includes a count of annual well completions (combines initial and recompletions), thus county-wide emissions will be a combination of the two. However, well completions characteristics will vary by well type; hence emissions are estimated separately for gas well completions and oil well completions. Additionally, emissions are estimated separately for unconventional and conventional completions.

Figure 3-15 shows temporary storage tanks used to collect flowback fluids at an unconventional well completion in the Barnett Shale. Emissions are generated as gas entrained in the flowback fluid is emitted through open vents at the top of the tanks.

Figure 3-15. Well Completion

The calculation methodology for estimating emissions from a single, uncontrolled completion event is shown below in Equation 57. Emissions from well completions controlled by flaring or use of green completions are the calculated using equations 58 - 60 as described below.

Equation 57)

where:

Ecompletion,i is the uncontrolled emissions of pollutant i from a single completion event [ton/event]

P is atmospheric pressure [1 atm]

Qcompletion is the uncontrolled volume of gas generated per completion [MCF/event]

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the gas [g/mol]

T is the atmospheric temperature [298 K]

fi is the mass fraction of pollutant i in the completion venting gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Flaring emissions from well completion controls

The methodology for estimating flaring emissions from completion venting processes is described below:

Equation 58)

where:

Eflare,completion is the county-wide flaring emissions of pollutant i for well completions [ton/yr]

EFi is the flaring emissions factor for pollutant i [lb/MMBtu]

Qcompletion is the uncontrolled volume of gas generated per completion [MCF/event]

F is the fraction of well completions with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

HV is the local heating value of the gas [BTU/SCF]

WCcounty is the county-wide number of well completion events for a particular year [events/yr]

2,000 is the unit conversion factor lbs/ton

1,000 is the unit conversion factor MCF/MMCF

The methodology for estimating SO2 emissions from flaring of completion vent gas is shown below:

Equation 59)

where:

is the county-wide SO2 flaring emissions from flaring of completion vent gas [ton/yr]

P is atmospheric pressure [1 atm]

Qcompletion is the uncontrolled volume of gas generated per completion [MCF/event]

WCcounty is the county-wide number of well completion events for a particular year [events/yr]

F is the fraction of well completions with flares

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

R is the universal gas constant [0.082 L-atm/mol-K]

MWgas is the molecular weight of the completion venting gas [g/mol]

T is the atmospheric temperature [298 K]

is the mass fraction of H2S in the completion venting gas

3.5x10-5 is the unit conversion factor MCF/L

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

Controlled, county-wide emissions are obtained by scaling-up well completions by well type using the number of completion events by well type by year and accounting for any controls used. This is done by applying Equation 60:

Equation 60)

where:

Ecompletion,TOTAL are the total emissions county-wide of pollutant i from well completions [tons/yr]

Ecompletion,i are the completion emissions from a single completion event [tons/event]

WCcounty is the county-wide total completions events in a particular year [events/yr]

Fflare is the fraction of completions in the county controlled by flare

Ccaptured is the capture efficiency of the flare

Cefficiency is the control efficiency of the flare

Fgreen is the fraction of completions in the county that were controlled by green completion techniques

Eflare,completion,i is the county-wide flaring emissions from flaring of completion vent gas [ton/yr]

Example Calculation for Well Completions:

Using the equations provided above, VOC emissions from venting of controlled (accounting for both flaring and green completions) oil well completions in Columbia County, Arkansas were calculated as follows:

where:

Ecompletion is the uncontrolled emissions of VOC from a single completion event [ton/event]

P = 1 [atm]

Qcompletion = 226 [MCF/event]

R = 0.082 [L-atm/mol-K]

MWgas = 24.25 [g/mol]

T = 298 [K]

f = 0.26 [VOC fraction]

3.5x10-5 [MCF/L]

907,185 [g/ton]

Therefore:

E completion  = 1.84 [ton/event]

Well completion flaring emissions are calculated similarly to the example given above for condensate tanks. In this case, E flare, completion,VOC = 0.552 [ton/yr]

Total VOC emissions from well completion venting and flaring in Columbia County were calculated as follows:

where:

Ecompletion,TOTAL are the total emissions county-wide of VOC from well completions [tons/yr]

Ecompletion = 1.84 [tons/event]

WCcounty = 62 [events/yr]

Fflare = 0.833 (fraction flared)

Ccaptured = 0.898 (capture efficiency expressed as fraction)

Cefficiency = 0.98 (control efficiency expressed as fraction)

Fgreen = 0.167 (fraction green completions)

Eflare,completion  = 0.552 [ton/yr]

Therefore:

E completion,TOTAL = 11.95 [ton/yr]

## Wellhead Compressor Engines

Wellhead compressor engines are generally small natural-gas fired engines located at the well site and used to boost produced gas pressure from downhole pressure to the required pressure for delivery to a transmissions pipeline. The fractional usage of these engines will depend on the basin characteristics; hence for those basins that largely require wellhead compression, this may be a significant nonpoint source of NOx emissions. Figure 3-16 shows two wellhead compressor engines in the Barnett shale.

Figure 3-16. Wellhead Compressor Engines

Compressor engines found at a wellhead were categorized into two main categories in this analysis and thus emissions are estimated for each type of engine and consequently extrapolated to county-wide emissions. These categories of compressors are:

- Rich burn compressors

- Lean burn compressors

The basic methodology for estimating emissions from wellhead compressor engines is shown in Equation 61:

Equation 61)

where:

Eengine,type are emissions from a particular type (rich vs. lean) of compressor engine [ton/yr/engine]

EFi is the emissions factor of pollutant i [g/hp-hr] (note that this may be different for NOx emissions from rich-burn vs. lean-burn engines)

HP is the horsepower of the engine [hp]

LF is the load factor of the engine

tannual is the annual number of hours the engine is used [hr/yr]

Fcontrolled is the fraction of compressors of a particular type (rich vs. lean) that are controlled

CFi is the control factor for controlled engines for pollutant i

907,185 is the unit conversion factor g/ton

Extrapolation to county-level emissions

County-level emissions are made up of the combination of emissions from each type of wellhead compressor, rich burn and lean burn. Emissions are scaled to county level using the usage fraction (F) of each engine type against all other compressor engines, the fraction of wells with wellhead compressor engines, and the total gas well count in a county, according to equation below:

Equation 62)

where:

Eengine,TOTAL is the total emissions from compressor engines in a county [ton/yr]

Frich is the fraction of rich-burn wellhead compressors in the county amongst all wellhead compressors

Eengine,rich is the total emissions from a single rich burn compressor engine per Equation (61) [ton/yr]

# Flean is the fraction of lean-burn wellhead compressors in the county amongst all wellhead compressors

Eengine,lean is the total emissions from a single lean burn compressor engine per Equation (61) [ton/yr]

# Wgas is the total gas well count in a county

fwellhead is the fraction of all gas wells in the county with wellhead compressor engines

Example Calculation for Rich-Burn Wellhead Compressor:

Using the equations provided above, NOx emissions from rich-burn wellhead compressor engines in Cleburne County, Arkansas were calculated as follows:

where:

E engine, rich  = emissions from a rich-burn wellhead compressor engine [ton/yr/engine]

EF  = 8.24 [g/hp-hr]

HP = 105.5 [hp]

LF = 0.77 (load factor for the engine)

tannual = 8,370 [hr/yr]

Fcontrolled = 0.44 (fraction of engines controlled)

CF  = 0.90 (control factor)

907,185 [g/ton]

Therefore:

E engine, rich  = 3.73 [ton/yr/engine]

Total NOx emissions from all rich-burn wellhead compressor engines in Cleburne County can be evaluated as follows:

where:

E engine, rich, TOTAL  = total emissions from rich-burn compressor engines in a county [ton/yr]

F rich = 0.490 (fraction of rich burn engines)

E engine, rich  = 3.73 [ton/yr/engine]

W gas = 490 [wells]

f wellhead = 0.0845 (fraction of gas wells with compressor engines)

Therefore:

E engine, rich, TOTAL, = 75.7 [tons/NOx/yr]

# Tool Nonpoint Oil and Gas Emissions Summary

Table 4-1 presents a summary of nonpoint oil and gas emissions generated by the tool by state for 2016, including the District of Columbia, Puerto Rico, and the Virgin Islands.

| Table 4-1. State-wide Tool Emissions Estimates |  |  |  |  |
| --- | --- | --- | --- | --- |
| State | NOX (TPY) | VOCs (TPY) | CO (TPY) | Total HAP (TPY) |
| Alabama | 8,792 | 15,600 | 12,254 | 676 |
| Alaska | 2,090 | 25,211 | 2,936 | 10,858 |
| Arizona | 9 | 47 | 10 | 1 |
| Arkansas | 5,521 | 12,539 | 5,328 | 498 |
| California | 22,238 | 135,419 | 33,915 | 4,195 |
| Colorado | 28,227 | 73,729 | 36,274 | 7,169 |
| Florida | 20 | 916 | 32 | 25 |
| Idaho | 12 | 514 | 23 | 22 |
| Illinois | 23,478 | 117,342 | 35,508 | 638 |
| Indiana | 3,516 | 13,373 | 5,041 | 154 |
| Kansas | 49,832 | 89,595 | 72,720 | 2,894 |
| Kentucky | 15,903 | 40,564 | 23,098 | 1,161 |
| Louisiana | 28,442 | 57,592 | 39,870 | 4,436 |
| Maryland | 1 | 1 | 1 | 0 |
| Michigan | 11,665 | 22,656 | 16,576 | 809 |
| Mississippi | 2,806 | 11,382 | 3,893 | 353 |
| Missouri | 536 | 1,165 | 822 | 12 |
| Montana | 2,237 | 40,619 | 3,347 | 1,272 |
| Nebraska | 520 | 2,774 | 761 | 18 |
| Nevada | 3 | 169 | 3 | 2 |
| New Mexico | 33,143 | 180,794 | 47,903 | 3,512 |
| New York | 627 | 6,571 | 897 | 114 |
| North Dakota | 14,572 | 450,336 | 22,110 | 12,014 |
| Ohio | 1,668 | 15,559 | 1,352 | 303 |
| Oklahoma | 49,359 | 181,041 | 61,868 | 5,944 |
| Oregon | 15 | 32 | 21 | 3 |
| Pennsylvania | 45,684 | 139,974 | 65,218 | 19,215 |
| South Dakota | 98 | 2,282 | 174 | 79 |
| Tennessee | 757 | 2,058 | 1,102 | 44 |
| Texas | 179,392 | 1,136,375 | 234,086 | 27,644 |
| Utah | 8,300 | 78,696 | 12,591 | 2,441 |
| Virginia | 10,199 | 11,736 | 14,198 | 761 |
| West Virginia | 29,249 | 108,456 | 41,855 | 6,141 |
| Wyoming | 16,882 | 72,803 | 25,714 | 2,904 |
| Total | 595,790 | 3,047,920 | 821,501 | 116,312 |

While there is some variability in emissions due to regional and basin-specific factors such as the VOC weight percent in natural gas, in general, the relative magnitude of state-wide emissions is dependent on the level of oil and gas activity in each state. As shown in Table 4-1, the highest emissions occur in those states with the highest oil and gas production.

Table 4-2 presents a summary of national emissions for 2016 for each source category as calculated by the tool.

| Table 4-2. Source Category Tool Emissions Estimates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source Category | NOX (TPY) | VOCs (TPY) | CO (TPY) | Total HAP (TPY) |
| Artificial Lifts | 151,827 | 1,980 | 234,763 | 2,169 |
| Associated Gas | 315 | 153,652 | 1,665 | 908 |
| Condensate Tanks | 1,725 | 320,575 | 9,119 | 5,327 |
| Crude Oil Tanks | 2,323 | 1,009,088 | 12,278 | 26,109 |
| Dehydrators | 25,756 | 97,947 | 37,300 | 61,430 |
| Drill Rigs | 31,023 | 2,218 | 6,746 | 973 |
| Fugitives | 0 | 372,677 | 0 | 2,162 |
| Gas-Actuated Pumps | 0 | 185,648 | 0 | 1,475 |
| Heaters | 30,186 | 2,853 | 43,568 | 994 |
| Hydraulic Fracturing | 11,344 | 799 | 2,282 | 358 |
| Lateral/Gathering Compressor Engines | 128,667 | 3,837 | 182,064 | 3,182 |
| Liquids Unloading | 101 | 100,566 | 532 | 454 |
| Loading Emissions | 0 | 22,860 | 0 | 626 |
| Mud Degassing | 0 | 23,495 | 0 | 50 |
| Pneumatic Devices | 0 | 618,188 | 0 | 2,870 |
| Produced Water | 0 | 60,622 | 0 | 239 |
| Well Completions | 511 | 63,197 | 2,701 | 860 |
| Wellhead Compressor Engines | 212,011 | 7,719 | 288,483 | 6,127 |
| Total | 595,790 | 3,047,920 | 821,501 | 116,312 |

As Table 4-2 illustrates, NOx emissions are largely dominated by wellhead compressor emissions. This is particularly true for states with a large number of active gas wells. Other significant sources of NOx include lateral compressors, well-site heaters, and artificial lift engines. Pneumatic devices and crude oil storage tanks are the most significant source of VOC emissions in many states. Other key sources of VOC emission include condensate tanks, dehydrators, and fugitives (equipment leaks).

Figure 4-1 below shows 2016 tool nonpoint oil and gas VOC emissions for each county.

Figure 4-1. Tool Nonpoint Oil and Gas VOC Emissions

Figure 4-2 below shows 2016 tool nonpoint oil and gas NOx emissions for each county.

Figure 4-2. Tool Nonpoint Oil and Gas NOx Emissions

Appendix A – Instructions for Using the EPA Nonpoint Oil and Gas Emissions Estimation Tool, Exploration Module (12/1/2018)

Instructions for Using the EPA Nonpoint Oil and Gas Emissions Estimation Tool, Exploration Module (12/1/2018)

- Introduction

Under prior Work Assignments with U.S. EPA, Eastern Research Group, Inc. (ERG) was tasked to develop a tool that state, local, and tribal (SLT) agencies could use to develop a nonpoint source emission inventory for upstream oil and natural gas activities. To this end, ERG prepared the EPA Nonpoint Oil and Gas Emissions Estimation Tool for the 2016 base year to assist agencies in compiling, allocating, and adjusting upstream oil and natural gas activity data, and developing county-level nonpoint source emission estimates. The tool generated estimates for 54 source classification codes (SCCs) and 70 pollutants.

In support of a special 2016 oil and gas emissions inventory, U.S. EPA directed ERG to redesign the Tool for the 2016 baseyear. These instructions address use of the exploration module.

- MS-Access Databases

The Nonpoint Oil and Gas Emissions Estimation Tools were programmed in MS-Access. This platform offered several advantages, particularly in accessibility (software is available to most users), familiarity (MS-Access is used by most SLT agencies in preparation of Emission Inventory System (EIS) data files), and portability (the tool modules can be e-mailed as zipped files that are less than 25 MB each in size).

Included with the tool are the Nonpoint Emissions blank staging tables which are to be used for preparation of EIS data files.

- Tool Data Flow

The basic concept of the tool is to calculate the source category emissions using the activity data, emission factors, and basin factors. A conceptual flow is:

- Steps for Using the Oil and Natural Gas Tool for Exploration Sources to Generate Emissions

In this section, steps will be outlined to generate emissions from the Exploration sources.

Note: If the User will be editing an existing version of the database and wishes to reset the tool and regenerate the emissions, the following steps are recommended:

  - Click on the “Reset All Selections/Go to Step 1” button at the top of the Dashboard; and

  - Compact and Repair the database.

  - Preparation

Prior to running the tool, the User must properly link the data tables in the Nonpoint Emissions Staging Tables within the tool. To do this, follow the instructions below:

- Place both the “OIL_GAS_TOOL_2016_NEI_EXPLORATION_V1_0.accdb” and the “nonpoint_bridge_tool.accdb” database tables in the same directory. It is recommended that the User creates an “EPA_OIL_GAS” directory on their hard drive.

- Open the “OIL_GAS_TOOL_2016_NEI_EXPLORATION_V1_0.accdb” database. You will need to “Enable Content” if the message pops up.

- Click on the “LINK TO EIS STAGING TABLES” button, and a pop-up box will appear. Follow the instructions to link in the EIS Staging tables in the “noonpoint_bridge_tool.accdb” database (see figure below). If successfully linked, 11 tables will be linked.

- Once you have identified the location of the “nonpoint_bridge_tool.accdb” database to link, click on the “Link Tables” button. If successful, 11 tables will be linked. When finished click on the “When finished, Click here.” button.

- Click the “BEGIN (go to DASHBOARD VIEW)” button to go to the Dashboard View.

- In the Dashboard View, there are 10 tabs labeled Steps 1 through 10. The User will need to follow all ten steps in order to generate the emission estimates.

  - Steps to Generate Emissions

- Step 1 - Select the Geographic Level. In Step 1, the User selects the geographic-level of the emissions inventory based on interest. On this page, the User will see some of the Geographic Area Type maps, which include: EIA Supply Region; EPA Regional Offices; NEMS Regions; Ozone Attainment Status; Regional Planning Organization; or Subpart W Basin. Most Users will select the “STATE” view. When finished, click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 2.

- Step 2 – Select Specific Geographic Location. Click the “Step 2 – Select Specific Geographic Location” tab to continue. In Step 2, the User selects the specific geographic location of interest. The User may select more than specific location. When finished, click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 3.

- Step 3 – Select the Source Category Level. Click the “Step 3 – Select Source Category Level” tab to continue. In Step 3, the User can either pick to generate emission estimates for all oil and gas exploration source categories or individually select source categories. When finished, click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 4.

- Step 4 – Select Specific Source Category. Click the “Step 4 – Select Specific Source Category” tab to continue. In Step 4, the User can select the specific Source Categories to generate emission estimates. If in Step 3, the User selected “ALL OIL AND GAS EXPLORATION SOURCE CATEGORIES”, then all source categories will be checked. At this point, the User may choose to deselect certain source categories. When finished, click the “When finished, press here” button. A message box will appear instructing the User to proceed to Steps 5, 6, and 7 to review/edit the activity data, basin factors, and emission factors; or to proceed directly to Step 8 for Point Source Activity Adjustments.

If in Step 3, the User selected “SELECT OIL AND GAS EXPLORATION SOURCE CATEGORIES”, then no source categories will be checked. At this point, the User will select one or more source categories. When finished, click the “When finished, press here” button. A message box will appear instructing the User to proceed to Steps 5, 6, and 7 to review/edit the activity data, basin factors, and emission factors; or to proceed directly to Step 8 for Point Source Activity Adjustments.

- Step 5 – View/Edit County-Level Activity Data. Click the “Step 5 – View/Edit County-Level Activity Data” tab to continue. In Step 5, the User can view and edit the activity data that EPA has compiled for the geographic area and source categories selected.

To continue with this step, the User will need to pick an activity dataset to view/edit. If the “Drilling and Mud Degassing Activity” button is chosen, the User will then be asked to choose a well type.

Once the well type is selected, an Activity Data form will appear that the User can view or edit. To get to the next county, at the bottom of the screen is the record number. Use the triangle arrows to move through the counties.

# The User may also edit activity data in MS-Excel by using the “Import/Export Data…” button.

If the user elects to edit activity data in MS-Excel, after clicking the button, the data is then exported into MS-Excel as shown below.

A MS-Excel workbook will open when finished exporting. It is required that the User save this file to the hard drive for later upload. In the Excel file, the User can only edit the yellow shaded cells. When completed, simply save the file.

If data edits were made, then the User will need to go back to the Tool and click on the “Import/Export Data…” button to initiate importing the edited data file. After clicking, the Import/Export form will appear. The User will need to:

- Step 1 - Click on the “Import” tab,

- Step 2 - Click the “Select File” button

- Step 3 – Map to the location of the edited data, and click “OK”

- Step 4 – Click on the “Import from Excel” button

# The edited data is now imported into the Tool.

- Step 6 – View/Edit Basin Factors. Click the “Step 6 – View/Edit Basin Factors” tab to continue. In Step 6, the User can view and edit the basin factor data that EPA has compiled for the geographic area and source categories selected.

In the Basin Factors form, the User can view/edit the data. If the User updates values for one county in a basin, then all other counties in the basin and state can be updated by clicking on the “Click to apply these values to all other counties in the same basin for the state.” button. Additionally, the User can export and import data to MS-Excel similar to the procedure outlined in Step 5.

# Similarly, the User can view/edit the gas composition data for select categories.

- Step 7 – View/Edit Emission Factors. Click the “Step 7 – View/Edit Emission Factors” tab to continue. In Step 7, the User can view or edit the emission factors that are used to generate the emission estimates for the source categories selected.

Once a Source Category has been selected, the User can view or edit the emission factors. Remember to update the reference field (EMISSION_FACTOR_SOURCE) for any updated emission factors.

- Step 8 – Point Source Activity Adjustments. Click the “Step 8 – Point Source Activity Adjustments” tab to continue. After the activity data, basin factors, and emission factors have been reviewed and/or updated, the User may enter point source activity adjustments to account for emissions that are to be reported to the point sources emissions inventory. If the User does not have any point source activity adjustments, then they will need to click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 9.

Currently, ALL point source activity adjustments (e.g. county-level point source spud counts, county-level point source feet drilled, county-level well completions, etc.) are defaulted to zero (i.e., no point source activity adjustments).

It is encouraged that point source activity adjustments have priority over point source emission adjustments. Additionally, Users should pay careful attention to ensure that the point source activity data is entered in the same units as the nonpoint activity data. Users should refer to the “Nonpoint Source SCCs and Point Source SCCs Crosswalk” button to identify point source SCCs. After any point source activity adjustments have been made, proceed to Step 9.

- Step 9 – Point Source Emission Adjustments. Click the “Step 9 – Point Source Emission Adjustments” tab to continue. In Step 9, the User can make point source emission adjustments directly in the emission tables. Select a Source Category to open. If a User has no point source emissions adjustments, they may click on the “When finished, click here to complete this step” button.

Point source emission estimates are to be entered in the “POINT_EMISSIONS_TPY” field. It is important to note that if point source activity adjustments were made in Step 8, then point source emission adjustments should NOT be made in these tables for overlapping SCCs. Also, point source emission adjustments need to be entered as tons per year (TPY).

# After point source emission adjustments are made (if applicable), then the User should proceed to Step 10.

- Step 10 – Final Emissions. Click the “Step 10 – Final Emissions” tab to continue. In Step 10, the User can review the final emissions, update county-level activity data, emission factors, and basin factors that the User updated, retain point source activity and/or point source emission adjustments, or generate the Emission Inventory System (EIS) data tables.

Additional notes:

- In the EIS Staging Tables, the ControlApproach, ControlMeasure, ControlPollutant, Emissions, EmissionsProcess, Location, and ReportingPeriod are populated.

- EPA’s EIS Nonpoint Bridge Tool (included in the .zip file) can be used to generate the .xml file needed for EIS upload.

- If the User wishes to reset the tool, and regenerate the emissions, the following steps are recommended:

  - Click on the “Reset All Selections/Go to Step 1” button at the top of the Dashboard.

  - Compact and Repair the database.

- References cited for the original data in the Tool are found in the “Master References” tab.

Appendix B – Instructions for Using the EPA Nonpoint Oil and Gas Emissions Estimation Tool, Production Module (12/1/2018)

Instructions for Using the EPA Nonpoint Oil and Gas Emissions Estimation Tool, Production Module (12/1/2018)

- Introduction

Under prior Work Assignments with U.S. EPA, Eastern Research Group, Inc. (ERG) was tasked to develop a tool that state, local, and tribal (SLT) agencies could use to develop a nonpoint source emission inventory for upstream oil and natural gas activities. To this end, ERG prepared the EPA Nonpoint Oil and Gas Emissions Estimation Tool for the 2016 base year to assist agencies in compiling, allocating, and adjusting upstream oil and natural gas activity data, and developing county-level nonpoint source emission estimates. The tool generated estimates for 54 source classification codes (SCCs) and 70 pollutants.

In support of a special 2016 oil and gas emissions inventory, U.S. EPA directed ERG to redesign the Tool for the 2016 baseyear. These instructions address use of the production module.

- MS-Access Databases

The Nonpoint Oil and Gas Emissions Estimation Tools were programmed in MS-Access. This platform offered several advantages, particularly in accessibility (software is available to most users), familiarity (MS-Access is used by most SLT agencies in preparation of Emission Inventory System (EIS) data files), and portability (the tool modules can be e-mailed as zipped files that are less than 25 MB each in size).

Included with the tool are the Nonpoint Emissions blank staging tables which are to be used for preparation of EIS data files.

- Tool Data Flow

The basic concept of the tool is to calculate the source category emissions using the activity data, emission factors, and basin factors. A conceptual flow is:

- Steps for Using the Oil and Natural Gas Tool for Production Sources to Generate Emissions

In this section, steps will be outlined to generate emissions from the Production sources.

Note: If the User will be editing an existing version of the database and wishes to reset the tool and regenerate the emissions, the following steps are recommended:

  - Click on the “Reset All Selections/Go to Step 1” button at the top of the Dashboard; and

  - Compact and Repair the database

  - Preparation

Prior to running the tool, the User must properly link the data tables in the Nonpoint Emissions Staging Tables within the tool. To do this, follow the instructions below:

- Place both the “OIL_GAS_TOOL_2016_NEI_PRODUCTION_V1_0.accdb” and the “nonpoint_bridge_tool.accdb” database tables in the same directory. It is recommended that the User creates an “EPA_OIL_GAS” directory on their hard drive.

- Open the “OIL_GAS_TOOL_2016_NEI_PRODUCTION_V1_0.accdb” database. You will need to “Enable Macros” if the message pops up.

- Click on the “LINK TO EIS STAGING TABLES” button, and a pop-up box will appear. Follow the instructions to link in the EIS Staging tables in the “noonpoint_bridge_tool.accdb” database (see figure below). If successfully linked, 11 tables will be linked.

- Once you have identified the location of the “nonpoint_bridge_tool.accdb” database to link, click on the “Link Tables” button. If successful, 11 tables will be linked. When finished click on the “When finished, Click here.” button.

- Click the “BEGIN (go to DASHBOARD VIEW)” button to go to the Dashboard View.

- In the Dashboard View, there are 10 tabs labeled Steps 1 through 10. The User will need to follow all ten steps in order to generate the emission estimates.

  - Steps to Generate Emissions

- Step 1 - Select the Geographic Level. In Step 1, the User selects the geographic-level of the emissions inventory based on interest. On this page, the User will see some of the Geographic Area Type maps, which include: EIA Supply Region; EPA Regional Offices; NEMS Regions; Ozone Attainment Status; Regional Planning Organization; or Subpart W Basin. Most Users will select the “STATE” view. When finished, click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 2.

- Step 2 – Select Specific Geographic Location. Click the “Step 2 – Select Specific Geographic Location” tab to continue. In Step 2, the User selects the specific geographic location of interest. The User may select more than specific location. When finished, click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 3.

- Step 3 – Select the Source Category Level. Click the “Step 3 – Select Source Category Level” tab to continue. In Step 3, the User can either pick to generate emission estimates for all oil and gas production source categories or individually select source categories. When finished, click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 4.

- Step 4 – Select Specific Source Category. Click the “Step 4 – Select Specific Source Category” tab to continue. In Step 4, the User can select the specific Source Categories to generate emission estimates. If in Step 3, the User selected “ALL OIL AND GAS PRODUCTION SOURCE CATEGORIES”, then all source categories will be checked. At this point, the User may choose to deselect certain source categories. When finished, click the “When finished, press here” button. A message box will appear instructing the User to proceed to Steps 5, 6, and 7 to review/edit the activity data, basin factors, and emission factors; or to proceed directly to Step 8 for Point Source Activity Adjustments.

If in Step 3, the User selected “SELECT OIL AND GAS PRODUCTION SOURCE CATEGORIES”, then no source categories will be checked. At this point, the User will select one or more source categories. When finished, click the “When finished, press here” button. A message box will appear instructing the User to proceed to Steps 5, 6, and 7 to review/edit the activity data, basin factors, and emission factors; or to proceed directly to Step 8 for Point Source Activity Adjustments.

- Step 5 – View/Edit County-Level Activity Data. Click the “Step 5 – View/Edit County-Level Activity Data” tab to continue. In Step 5, the User can view and edit the activity data that EPA has compiled for the geographic area and source categories selected.

Once the county-level data set is selected, an Activity Data form will appear that the User can view or edit. To get to the next county, at the bottom of the screen is the record number. Use the triangle arrows to move through the counties.

# The User may also edit activity data in MS-Excel by using the “Import/Export Data…” button.

If the user elects to edit activity data in MS-Excel, after clicking the button, the data is then exported into MS-Excel as shown below.

A MS-Excel workbook will open when finished exporting. It is required that the User save this file to the hard drive for later upload. In the Excel file, the User can only edit the yellow shaded cells. When completed, simply save the file.

If data edits were made, then the User will need to go back to the Tool and click on the “Import/Export Data…” button to initiate importing the edited data file. After clicking, the Import/Export form will appear. The User will need to:

- Step 1 – Click on the “Import” tab

- Step 2 – Click the “Select File” button

- Step 3 – Map to the location of the edited data, and click “OK”

- Step 4 – Click on the “Import from Excel” button

The edited data is now imported into the Tool.

- Step 6 – View/Edit Basin Factors. Click the “Step 6 – View/Edit Basin Factors” tab to continue. In Step 6, the User can view and edit the basin factor data that EPA has compiled for the geographic area and source categories selected.

In the Basin Factors form, the User can view/edit the data. If the User updates values for one county in a basin, then all other counties in the basin and state can be updated by clicking on the “Click to apply these values to all other counties in the same basin for the state.” button.

# Similarly, the User can view/edit the gas composition data for select categories.

- Step 7 – View/Edit Emission Factors. Click the “Step 7 – View/Edit Emission Factors” tab to continue. In Step 7, the User can view or edit the emission factors that are used to generate the emission estimates for the source categories selected.

Once a Source Category has been selected, the User can view or edit the emission factors. Remember to update the reference field (EMISSION_FACTOR_SOURCE) for any updated emission factors.

- Step 8 – Point Source Activity Adjustments. Click the “Step 8 – Point Source Activity Adjustments” tab to continue. After the activity data, basin factors, and emission factors have been reviewed and/or updated, the User may enter point source activity adjustments to account for emissions that are to be reported to the point sources emissions inventory. If the User does not have any point source activity adjustments, then they will need to click the “When finished, click here to complete this step.” button. A message box will appear instructing the User to proceed to Step 9.

Currently, ALL point source activity adjustments (e.g. county-level point source well counts, county-level point source barrels of oil produced, etc.) are defaulted to zero (i.e., no point source activity adjustments).

It is encouraged that point source activity adjustments have priority over point source emission adjustments. Additionally, Users should pay careful attention to ensure that the point source activity data is entered in the same units as the nonpoint activity data. Users should refer to the “Nonpoint Source SCCs and Point Source SCCs Crosswalk” button to identify point source SCCs. After any point source activity adjustments have been made, proceed to Step 9.

- Step 9 – Point Source Emission Adjustments. Click the “Step 9 – Point Source Emission Adjustments” tab to continue. In Step 9, the User can make point source emission adjustments directly in the emission tables. Select a Source Category to open. If a User has no point source emissions adjustments, they may click on the “When finished, click here to complete this step” button.

Point source emission estimates are to be entered in the “POINT_EMISSIONS_TPY” field. It is important to note that if point source activity adjustments were made in Step 8, then point source emission adjustments should NOT be made in these tables for overlapping SCCs. Also, point source emission adjustments need to be entered as tons per year (TPY).

# After point source emission adjustments are made (if applicable), then the User should proceed to Step 10.

- Step 10 – Final Emissions. Click the “Step 10 – Final Emissions” tab to continue. In Step 10, the User can review the final emissions; update county-level activity data, emission factors, and/or basin factors they provided in Steps 5 through 7; or generate the Emission Inventory System (EIS) data tables.

# Summary screen (continued)

Additional notes:

- In the EIS Staging Tables, the ControlApproach, ControlMeasure, ControlPollutant, Emissions, EmissionsProcess, Location, and ReportingPeriod are populated.

- EPA’s EIS Nonpoint Bridge Tool (included in the .zip file) can be used to generate the .xml file needed for EIS upload.

- If the User wishes to reset the tool, and regenerate the emissions, the following steps are recommended:

  - Click on the “Reset All Selections/Go to Step 1” button at the top of the Dashboard.

  - Compact and Repair the database.

References cited for the original data in the Tool are found in the “Master References” tab.

Appendix C – US Oil and Gas Basins (Found in the “National Oil and Gas Tool Report Appendix C - Data Element Dictionary.xlsx” file)

Appendix D – Data Element Dictionary (Found in the “National Oil and Gas Tool Report Appendix D – US Oil and Gas Basins.xlsx” file)

Internet address:  http://www.eia.gov/

“DI Desktop Database powered by HPDI.” Accessed July 2018. Internet address: http://www.didesktop.com/

“Summary of Analysis of 2014 GHGRP Subpart W Data for Use in the 2014 NEI Nonpoint Oil and Gas Emission Estimation Tool”, Memorandum from Mike Pring and Stephen Treimel to Jennifer Snyder. November 24, 2015.

U.S. Well Starts By Depth Range, January 2016 through December 2016. Used by Permission and Approved for Publication by Jacqueline Hassan at RIGDATA (www.rigdata.com) in e-mail communication to Regi Oommen, Eastern Research Group, Inc. December 28, 2018.

HPDI assigns a unique number to each property (i.e., lease, well, unit) in the ENTITY_ID field.

API numbers are up to 14 digits long and are broken into four segments. The first two digits correspond to the state; the next three digits correspond to the county in the state. The next five digits are the unique well identifier for the county. The next two digits are for the directional side tracks (i.e., horizontal or directional drills that each have different bottom hole locations), with 00 representing the original well bore. The last two digits are the event sequence code that distinguish between original completion, reentries, recompletion, and hole deepenings. Some states do not assign directional side tracks or event sequence codes.

Duplicate wells in states with missing API numbers could be identified using the permit number, which should be unique for each well.

ERG found some wells with an “Active” STATUS had not produced in a number of years, while some wells with an “Inactive” STATUS had production data for 2010.

Texas production data sent via e-mail from Michael Ege/TCEQ to Jennifer Snyder/EPA on 9/19/2018

Kansas production data sent via e-mail from Lynn Deahl/KSDEP to Jennifer Snyder/EPA on 10/15/2018

West Virginia production and exploration data sent from Matt Kemper/WVDEP to Jennifer Snyder/EPA on 10/19/2018

ENVIRON International Company. Oil and Gas Emission Inventory Enhancement Project for CenSARA States. December 21, 2012. Internet address: www.censara.org/filedepot/folder/10

## U.S. EPA, 2013. Subpart W Basin and County Combinations. Internet address:

http://www.ccdsupport.com/confluence/display/help/Subpart+W+Basin+and+County+Combinations

## U.S. EPA, 2011. “Composition of Natural Gas for use in the Oil and Natural Gas Sector Rulemaking”, Memorandum from Heather P. Brown to Bruce Moore. July 28, 2011.

Personal Communication between Ms. Julie McDill, MARAMA, Ms. Megan Murphy, WVDEP, and Mr. Mike Pring, Eastern Research Group, Inc. January 24, 2014.

Google Earth, 2014. “Permian Basin Tank Battery.” 32°28’16.26" N and 102°49'26.40" W. November 14, 2011. March 25, 2014.

Artificial Lift R&D Council, 2014. Internet address: http://www.alrdc.org/production/

## U.S. EPA, 2013. Inventory of U.S. Greenhouse Gas Emissions and Sinks:  1990-2013. Internet address:

http://www.epa.gov/climatechange/ghgemissions/usinventoryreport.html

## Wilson, Darcy, Richard Billings, Regi Oommen, and Roger Chang, Eastern Research Group, Inc. Year 2005 Gulfwide Emission Inventory Study, U.S. Department of the Interior, Minerals Management Services, Gulf of Mexico OCS Region, New Orleans, December 2007, Section 5.2.10.

Based on gas content of 65.13 weight percent CH4, derived from sample data provided in the original source of the emission factors. Original sample data is as follows, in terms of mole%: 83.85% CH4, 5.41% C2H6, 6.12% C3H8, 3.21% C4H10, and 1.40% C5H12 (Wilson et al., 2007).